3GPP TSG-SA3 Meeting #86bis	S3-170725

# Sofia Antipolis, France, 6 – 10 Feb, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0603 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding BEST Service to TS 33.401 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, KPN, Juniper, Nokia |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | BEST_MTC_Sec |  |  |  |  |  |  | Date: |  |  | 2017-03-17 |
| Category: | B |  |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  |  | This CR adds the BEST Security Archetecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | This CR adds a new major clause that describes the BEST service, an annex defining the key derivation for BEST and an annex detailing the EMSDP used to transfer signalling and data messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | The BEST service not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 2, 3.1, 3.3, X (new), Annex A.19 (new section), 3 new Annexes |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  |  | TS 33.220 CR |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 33.102: "3G security; Security architecture".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[9]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[10] – [11]	Void.

[12]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[13]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18] – [20]	Void.

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[23]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Service principles".

[24]	3GPP TS 25.331: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification ".

[25]	3GPP TS 44.060: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol.

[26]	3GPP TS 23.122: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	3GPP TS 33.320: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[28]	(void)

[29]	ETSI TS  102 484 V10.0.0: "Smart Cards; Secure channel between a UICC and an end-point terminal".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 31.116 "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[32]	ETSI TS 102 221 V9.2.0: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[33]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[34]	RFC 4301: "Security Architecture for the Internet Protocol".

[35]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[36]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[37]	3GPP TS.33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[38]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[39]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[40]	3GPP TS 36.463: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[z]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[y]	3GPP TS 55.241: "Specification of the GIA4 integrity algorithm for GPRS; GIA4 specification"

[x]	3GPP TS 55.251: " Specification of the GEA5 encryption and GIA5 integrity algorithms for GPRS; GEA5 and GIA5 algorithm specification"

[w]	3GPP TS 35.205: " 3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General"

**** End of First Change ****

**** Second Change ****

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 33.102 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Security Management Entity: entity which receives the top-level keys in an access network from the HSS. For E-UTRAN access networks, the role of the ASME is assumed by the MME

Activation of security context: the process of taking into use a security context.

Authentication data: Data that is part of a security context or of authentication vectors.

BEST: Battery Efficient Security service for Cellular IoT

BEST Capable UE: A UE that is enabled for the BEST service

Chaining of KeNB: derivation of a new KeNB from another KeNB (i.e., at cell handover)

Current EPS security context: The security context which has been activated most recently. Note that a current EPS security context originating from either a mapped or native EPS security context may exist simultaneously with a native non-current EPS security context.

ECM-CONNECTED state: This is as defined in TS 23.401 [2]. The term ECM-CONNECTED state corresponds to the term EMM-CONNECTED mode used in TS 24.301 [9].

ECM-IDLE state: As defined in TS 23.401 [2]. The term ECM-IDLE state corresponds to the term EMM-IDLE mode used in TS 24.301 [9].

Enterprise Key: A secret key shared by the Enterprise Application Server and the UE for application in the BEST service

EPS-Authentication Vector: KASME, RAND, AUTN, XRES

EPS security context: A state that is established locally at the UE and a serving network domain. At both ends "EPS security context data" is stored, that consists of the EPS NAS security context, and the EPS AS security context.

NOTE 1:	An EPS security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

EPS AS security context: the cryptographic keys at AS level with their identifiers, the Next Hop parameter NH, the Next Hop Chaining Counter parameter NCC used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, counters used for replay protection and SCG Counter used as freshness input into S-KeNB derivations. Note that the EPS AS security context only exists when cryptographically protected radio bearers are established and is otherwise void.

NOTE 2: NH and NCC need to be stored also at the MME during connected mode.

EPS AS Secondary Cell security context: This context consists of the cryptographic keys for SeNB (KUPenc), the identifier of the selected AS SC level cryptographic algorithm and counters used for replay protection.

EPS NAS security context: This context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. In particular, separate pairs of NAS COUNT values are used for each EPS NAS security contexts, respectively. The distinction between native and mapped EPS security contexts also applies to EPS NAS security contexts. The EPS NAS security context is called "full" if it additionally contains the keys KNASint and KNASenc and the identifiers of the selected NAS integrity and encryption algorithms.

Full native EPS security context: A native EPS security context for which the EPS NAS security context is full according to the above definition. A full native EPS security context is either in state "current" or state "non-current".

Forward security: In the context of KeNB key derivation, forward security refers to the property that, for an eNB with knowledge of a KeNB, shared with a UE, it shall be computationally infeasible to predict any future KeNB, that will be used between the same UE and another eNB. More specifically, n hop forward security refers to the property that an eNB is unable to compute keys that will be used between a UE and another eNB to which the UE is connected after n or more handovers (n=1 or 2).

Legacy security context: A security context which has been established according to TS 33.102 [4].

Mapped security context: Security context created by converting the current security context in the source system to a security context for the target system in inter-system mobility, e.g., UMTS keys created from EPS keys. The EPS NAS security context of a mapped security context is full and current.

Native EPS security context: An EPS security context whose KASME was created by a run of EPS AKA.

Non-current EPS security context: A native EPS security context that is not the current one. A non-current EPS security context may be stored along with a current EPS security context in the UE and the MME. A non-current EPS security context does not contain an EPS AS security context. A non-current EPS security context is either of type "full native" or of type "partial native".

Partial native EPS security context: A partial native EPS security context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context. A partial native EPS security context is created by an EPS AKA, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

Re-derivation of NAS keys: derivation of new NAS keys from the same KASME but including different algorithms (and no freshness parameter)

Refresh of KeNB: derivation of a new KeNB from the same KASME and including a freshness parameter

Re-keying of KeNB: derivation of a new KeNB from a new KASME in ECM-CONNECTED (i.e., . to activate a partial native EPS security context, or to re-activate a non-current full EPS security context)

Re-keying of NAS keys: derivation of new NAS keys from a new KASME

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE. This includes capabilities for EPS AS and NAS, and includes capabilities for UTRAN and GERAN if these access types are supported by the UE.

UE EPS security capabilities: The UE security capabilities for EPS AS and NAS.

UE-to-HSE: UE to Home  Security Endpoint

UE-to-EAS: UE to Enterprise Application Server

User plane: Within the context of TS 33.401, this means the data path between UE and Serving Gateway that does NOT go via the MME.

(User) Data via MME: User Data sent to or from the UE that uses an RRC connection established using the Control Plane CIoT EPS optimisation specified in TS 23.401[2].

IOPS-capable eNB: an eNB that has the capability of IOPS mode operation, which provides local IP connectivity and Public Safety services to IOPS-enabled UEs via a Local EPC when the eNB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNBs in IOPS mode of operation use, instead of the Macro EPC, in order to support Public Safety services.

Macro EPC: the EPC which serves an eNB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNB(s)), local IP connectivity and Public Safety services to IOPS-enabled UEs in the absence of normal EPS.IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

**** End of Second Change ****

**** Third Change ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

EAS	Enterprise Application Server

ECM	EPS Connection Management

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

HSE	HPLMN Security Endpoint

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

**** End of Third Change ****

**** Fourth Change ****

UE

HSE

Session Initiation and Key Agreement (Control Plane)

EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt))

EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt)))

EMSDP Session Start Confirmation (Authentication result) (conditional)

Data Transfer

Data Transmission (User Plane)

EMSDP Manage Keys Request (Key ID) (opt)

EMSDP Manage Keys Response (Key ID, Key Agreement)

Session Key Management (Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination (Control Plane)

Message Reject  (Control Plane)

EMSDP Message Reject

Note:
- Bidirectional arrows indicate that the message can be initiated by either party.
- Arrows with dashed lines indicate optional messages.

EAS Session Start

UE

EAS

Session Initiation 
and Key Agreement 
(Control Plane)

EMSDP Session Request

EMSDP Session Start

EMSDP Session Start Confirmation

Data Transfer

Data Transmission 
(User Plane)

EMSDP Manage Keys Request

EMSDP Manage Keys Response

Session Key 
Management 
(Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination 
(Control Plane)

Message Reject  
(Control Plane)

EMSDP Message Reject

HSE

EAS Message Reject

EAS Session Request

EAS Session Registration

EAS Session Start Confirmation

EAS Manage Keys Request

EAS Manage Keys Response

EAS Session Terminate Request

EAS Session Terminate Response

With EMKS

Without EMKS

UE

HSE

1. EMSDP Session Request

EMKS

HSS

2. Keys required?

3. Authentication-Information-Request

4. Authentication-Information-Answer

d. Authentication-Information-Answer

c. Authentication-Information-Answer (to deliver authentication vectors – if required)

a. Authentication-Information-Request

b. Authentication-Information-Request (to request authentication vectors - if required)

Authentication vectors needed?

8. EMSDP Session Start

5. Calculate HSE Keys

EAS

6. EAS Session Reques

7. EAS Session Start

12. Calculate E2E Keys

9. EMSDP Session Start Confirmation (conditional).

11. Calculate UE Keys

10. EAS Session Start Confirmation

# X	Security Procedures for Battery Efficient Security for Low Throughput Devices (BEST)

## X.1	Introduction

This clause defines elements, protocols and procedures that enable battery efficient security for low throughput devices such as MTC devices.  The BEST service is a secure channel between a UE and a HSE, optimised for low throughput and high latency devices that are battery constrained.  The security is between the UE and either an element in the service provider home network (the HSE) or an element in the enterprise domain (the EAS). The design is modular and extensible so that it can be used to satify a wide range of use cases.

The following services are defined:

- BEST key agreement only service – This service is a battery efficient service for key agreement between a BEST compliant UE and the HSE or the EAS. The data for this service is provided by the application layer between the UE and the EAS and is out of scope of this specification.

- BEST user plane integrity protected service – This service is a battery efficient integrity protected user plane service for low throughput devices. This service includes the key agreement and includes integrity protected security over small data over NAS User Plane.  The data for this service can be either terminated in the HSE or in the EAS. Control messages are always terminated in the HSE.

- BEST user plane confidential service – This service is a battery efficient integrity and confidentiality protected user plane service for low throughput devices. The data for this service can be either terminated in the HSE or in the EAS. Control messages are always terminated in the HSE.

It may be possible for the UE to have concurrent BEST sessions.

## X.2	BEST Framework Service Description

### X.2.1	Architecture

Figure X.2.1-1: The architecture of the extended user plane protection service

The BEST service requires the following components:

- Home Security Environment (HSE) – This is the termination point in the home network that performs the following functions:

  - Terminating the control plane for BEST between the UE and the HSE

  - Terminating the secure communication for BEST between the UE and the HSE if UE-to-HSE security is selected.

  - Routing the user plane traffic for BEST between the UE and the Enterprise Application Server (EAS) if UE-to-EAS security is selected.

  - Anchor for BEST Key agreement only service. Exposes an interface for EAS to obtain MNO provided preshared key.

- End to Middle Key Server (EMKS) – This is an optional key server element that manages the key communication with the HSS (for quintets) and stores keys to reduce loading on the HSE and HSS. The EMKS has interfaces to the HSS (S6a) and the HSE (S6a).

The BEST service uses the following interfaces:

- S6a between the HSS and the HSE

- S6a between the HSS and EMKS

- S6a between the EMKS and the HSE

Annex W describes a candidate interface based on Restful HTTP for the communication between the HSE and ther EAS.

## X.3	Procedures between the UE and the HSE

### X.3.1	Overview of BEST procedures

To use the BEST service the UE shall first open a data session.  The UE may either use a locally stored IP address to locate the HSE or use a "BEST APN" where the traffic is directed by the PDN Gateway to the correct HSE for that UE.  Once a data session exists, the UE may initiate the BEST service.  It is up to the UE as to when it establishes the data session that is used for BEST and the bearer used.

The BEST service consists of 5 general processes between the UE and the HSE: session initiation and key agreement, key management, data transfer, session termination, and message rejection.  The details of the End to Middle Secure Data Protocol (EMSDP) used for the BEST service is detailed in Annex X.

When BEST user plane security services are used, UP data messages are between the UE and the HSE.

UE

HSE

Session Initiation and Key Agreement (Control Plane)

EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt))

EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt)))

EMSDP Session Start Confirmation (Authentication result) (conditional)

Data Transfer

Data Transmission (User Plane)

EMSDP Manage Keys Request (Key ID) (opt)

EMSDP Manage Keys Response (Key ID, Key Agreement)

Session Key Management (Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination (Control Plane)

Message Reject  (Control Plane)

EMSDP Message Reject

Note:
- Bidirectional arrows indicate that the message can be initiated by either party.
- Arrows with dashed lines indicate optional messages.

Figure X.3.1-1: Generalised BEST service flow

### X.3.2 BEST Session Initiation and Key Agreement

The UE shall initiate a BEST session using the EMSDP Session Request message following the establishment of the PDP Context.  To optimise the message flow for battery constrained devices, the EMSDP Session Response is combined with Session Key Agreement.

The EMSDP Session Request message shall include the UE Identity, BEST capabilities of the UE, the UE serving network and details of the enterprise service including the Enterprise server Id (EAS Id) that the BEST service is being used for.

The EMSDP Session Start message shall include an agreement on the BEST keys, the BEST service parameters and a checksum validating the previous EMSDP Session Request message.

The HSE shall determine the parameters for the BEST service.  The HSE may use the location information provided by the UE to determine whether aspects of the BEST service, such as cyphering, can be used in that location.

As a result of the key agreement exchange the UE and HSE calculate the keys to be used for future control plane and user plane security. In case of Key agreement only service, UE and HSE derive required intermediate key needed for application layer security at the enterprise layer.

To optimise the BEST service for battery constrained devices, confirmation of the BEST session start is not required.  The UE sending a UP message to the HSE is by itself is an implied confirmation.  However, if the BEST service is being used for key agreement only, the HSE shall require the UE to send EMSDP Session Start Confirmation by setting an an indicator in the EMSDP Session Response message.

### X.3.4 BEST Session Key Management

At any time during the session, either the UE or the HSE may trigger a re-negotiation of the keys being used for the BEST service using the Manage Keys Request and Response exchange.

### X.3.5 BEST Session Termination

At any time, either the UE or the HSE may terminate the current BEST session using the EMSDP Session Termination Request and Response message exchange.  Once terminated, all relevant keys and IDs shall be discarded and both the UE and HSE shall ignore further messages using that session ID, unless a session with that ID is re-established using the session initiation process.

### X.3.6 BEST Message Reject

Either the UE or the HSE may at any time respond with a EMSDP Message Reject message, upon which the recipient shall discard all relevant keys and IDs of the session, and both the UE and HSE shall ignore further messages using that session ID.

The EMSDP Message Reject is also used when the HSE needs to prompt a UE to initiate a new session using the Session Start message.  For example, if it receives a UP packet from the UE on a BEST session for which it aged out the context,

## X.4 Procedures between the HSE and the EAS

The messages exchanges between the HSE and the EAS are essentially a mirror of the ones between the UE and the HSE.  All control plane messages are terminated or initiated by the HSE.

When BEST user plane security services are used,  he UP data messages are end-to-end between the UE and the EAS.

Note:  The actual details and standardization of the HSE to EAS interfaces is out of scope for this release.

EAS Session Start

UE

EAS

Session Initiation 
and Key Agreement 
(Control Plane)

EMSDP Session Request

EMSDP Session Start

EMSDP Session Start Confirmation

Data Transfer

Data Transmission 
(User Plane)

EMSDP Manage Keys Request

EMSDP Manage Keys Response

Session Key 
Management 
(Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination 
(Control Plane)

Message Reject  
(Control Plane)

EMSDP Message Reject

HSE

EAS Message Reject

EAS Session Request

EAS Session Registration

EAS Session Start Confirmation

EAS Manage Keys Request

EAS Manage Keys Response

EAS Session Terminate Request

EAS Session Terminate Response

Figure X.4-1: Generalised BEST EAS service flow

### X.4.1 Registration

The EAS shall register for BEST service by sending the Session Registration message to the HSE. This results in a secure connection to be setup between the HSE and the EAS.

The BEST Session Registration message shall include the Enterprise server Id (EAS Id). The HSE sets up a session context for the EAS.

### X.4.2 Key Request

At the end of the Key agreement procedure, described in clause X.3.2, HSE forwards the required intermediate key to the EAS in the Session Request message. When BEST is used for Key agreement only, the required intermediate key is specifically derived for the enterprise as defined by key definition rules in clause A.y.1.3. The EAS responds with the Session Response message.

The EAS shall also be able to independently obtain the intermediate key from the HSE by sending Session Request message. This message shall include key identifier necessary for HSE to obtain the corresponding Intermediate Key (KInt_E2E). The HSE shall forward the required intermediate key to the EAS in the Session Response message. When BEST is used for Key agreement only, the required intermediate key is specifically derived for the enterprise as defined by key definition rules in clause A.y.1.3

### X.4.3 Key Management

The renegotiation of the intermediate key between the HSE and EAS is FFS.

## X.5 BEST Data Service

Once the BEST session is successfully initiated, the UE or the HSE or EAS may send UP data using the negotiated keys.

If the BEST UP session is set up between the UE and the HSE, then the UP packets are initiated or terminated by the HSE.  In this case, the low power optimized EMSDP protocol, as detailed in clause Annex x, is used.

If the BEST UP session is setup for end-to-end security to the EAS, then:

- The UP service may utilize the EMSDP protocol or another protocol with PSK, e.g., IKE/IPsec or (D)TLS.

- The key used by the UE to encrypt and decrypt data messages (when required) is KE2Eenc and to integrity protect the message is KE2Eint.

- The HSE passes the UP messages to the EAS after checking the message is formatted correctly and that it is a UP message.  Verification of any Key ID, counter, message integrity and deciphering is the responsibility of the EAS.

## X.6	Key Management

### X.6.1	Key Agreement and Refresh

#### X.6.1.1	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement is based on the 3GPP AKA mechanism detailed in TS 33.102 [4] and the AKA challenge is transported between the HSE and the UE as part of the BEST service detailed in clause X.3.2.

The EMDSP protocol has 7 Key IDs each with a separate keyset consisting of an integrity Key (KE2Mint), an encryption key (KE2Menc), optionally an End to End Intermediate Key (KInt_E2E) and optionally an End to End Pre Shared Key (Kint_PSK).

The End to End Intermediate Key (KInt_E2E) is used together with a local enterprise secret key to calculate KE2Eint and KE2Eenc when BEST User plane security services are used end to end between the UE and the EAS.

The End to End Pre Shared Key (Kint_PSK) is used together with a separate enterprise identifier to calculate KInt_EAS_PSK.  There can be many KInt_EAS_PSK derived from one Kint_PSK.

Figure X.6.1.1-1 shows the generic key agreement process:

With EMKS

Without EMKS

UE

HSE

1. EMSDP Session Request

EMKS

HSS

2. Keys required?

3. Authentication-Information-Request

4. Authentication-Information-Answer

d. Authentication-Information-Answer

c. Authentication-Information-Answer (to deliver authentication vectors – if required)

a. Authentication-Information-Request

b. Authentication-Information-Request (to request authentication vectors - if required)

Authentication vectors needed?

8. EMSDP Session Start

5. Calculate HSE Keys

EAS

6. EAS Session Reques

7. EAS Session Start

12. Calculate E2E Keys

9. EMSDP Session Start Confirmation (conditional).

11. Calculate UE Keys

10. EAS Session Start Confirmation

Figure X.6.1.1-1 Generic BEST key agreement process

The Key agreement steps are:

1.	EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt), location ID). The UE shall send the EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt) to set up a new BEST session.

2.	Keys required? - the HSE shall check to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the HSE has a valid set of keys for the indicated session and the UE ID is valid for that session then the HSE may start the BEST session without re-negotiating the keys (step 8).

-	If the UE ID is valid  for that HSE and the HSE does not have a valid set of keys for the indicated session or the HSE wishes to update the keys, then it shall first renegotiate the keys (steps 2 to 7) and then start the BEST session (step 8).

-	If the UE ID is not valid for that session ID or the UE does not support the level of service required by the HSE or the enterprise information is not valid for the HSE, then the HSE may reject the command.

3. Authentication-Information-Request over S6a interface – The HSE shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI.

4. Authentication-Information-Answer over S6a interface – The HSS uses the S6f interface to the HSE to return the requested authentication vectors.

a. Authentication-Information-Request over the S6a interface between HSE and EMKS – Where an EMKS is used, the HSE shall use the S6a interface to the EMKS to request an authentication vector using the UE IMSI.

b. Authentication-Information-Request over the S6a interface between EMKS and HSE – The EMKS shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI.

c. Authentication-Information-Answer over the S6a interface between EMKS and HSE – The HSS shall use the S6a interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the S6a interface between HSE and EMKS – The EMKS uses the S6a interface to the HSE to return the requested authentication vector.

5. Calculate HSE Keys -  See key derivation details in Annex Y.

6. The HSE may optionally send  "EAS Session Request" to the EAS over the SGi interface – The HSE shall use the SGi interface to the EAS to inform the Enterprise Server of the new UE-to-EAS session request and shall forward the intermediate E2E key (KInt_E2E)  to the Enterprise Server in case the intermediate E2E key (KInt_E2E) was calculated in the previous step.

7. The Enterprise Server sends a "EAS Session Accept" to the HSE – The Enterprise Server shall respond by sending the "UE-to-EAS Session Accept" message. This message may contain an EAS container.

8. EMSDP Session Response message -  The HSE shall send a EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt)).

9. EMSDP Session Start Confirmation - UE optionally, if requested in the Session Start, responds with an EMSDP Session Start Confirmation message.

10.  EAS Session Start Confirmation - The HSE may optionally send

11. Calculate UE Keys – See key derivation details in Annex Y.

12. Calculate E2E Keys – See key derivation details in Annex Y.

#### X.6.1.2	Usage of Keys

For UE-to-HSE BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the user plane and the signalling plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the user plane and the  Controlplane according to the agreed integrity protection algorithm.

For UE-to-EAS BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the control plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the control plane according to the agreed integrity protection algorithm.

-	The KE2Eenc shall be used by the UE and the EAS for the encryption of the user plane, according to the agreed encryption algorithm.

-	The KE2Eint shall be used by the UE and the EAS for the integrity protection of the user plane according to the agreed integrity protection algorithm.

For key-agreement only BEST service, the following key shall be used:

-	The KInt-PSK together with the Enterprise Application Server id shall be used by the HSE to generate EAS-specific pre-shared key (KInt_EAS_PSK). This is used for protection of the User plane as defined by the application layer security protocol used between the UE and the EAS.  The UE to EAS protocol in this case is out of scope of this specification.  Note: As the KInt_EAS_PSK is known to the HSE, the EAS and the UE will have to transform the key further to achieve end to end security.  This transformation is out of scope of this specification.

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the control plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the control plane according to the agreed integrity protection algorithm.

#### X.6.1.3	Key Setup for BEST session end point modification

The modification of a BEST UE-to-EAS session to a BEST UE-to-HSE session does not require any new calculations of keys.

The modification of a BEST UE-to-HSE session to a BEST UE-to-EAS session requires new keys to be calculated when no EAS keys are available in the EAS. Before sending the EMSDP Modification command the HSE does the following:

-	Checks with the EAS whether it has keys in storage

-	If not, obtains a EAS container from the EAS and forwards this to the UE in the EMSDP Modification command.

#### X.6.1.4	BEST Key Handling

The aging of keys and any counter window used are out of scope of this specification.

### X.6.2	BEST Key Hierarchy

#### X.6.2.1	Introduction

This clause describes the key hierarchies for BEST for the following options:

-	Best Key Hierarchy in case a separate BEST domain is used.

#### X.6.2.2	BEST Key Hierarchy for Separate BEST Domain

Figure 17.6.2.2-1: Key Hierarchy for Separate BEST Domain

The KInt_E2E, KInt_PSK and all of the keys derived from them are generated when indicated to do so in the BEST CP messaging.

**** End of Fourth Change ****

**** Fifth Change ****

A.y Derivation of BEST Keys

## A.y.1	BEST key derivation for separate BEST Domain

## A.y.1.1	Derivation of UE-to-HSE keys

The following input string shall be used when the UE and the HSE derive the BEST end2end Intermediate key KInt_E2E, BEST keys KE2Menc or KE2Mint from CK, IK:

-	FC = 0x60,

-	P0 = HSE id if supplied else NULL,

-	L0 = length of HSE id (i.e. 0x00 0x03 if HSE id supplied or 0x00 0x00 if not),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm id

-	L2 = length of algorithm id (i.e. 0x00 0x01)

Table A.y.1.1-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |
| BEST end2end Intermediate Key (KInt_E2E) | 0x03 |
| BEST end2end Intermediate Key (KInt_PSK) | 0x04 |

The input key shall be equal to the concatenation CK || IK of CK and IK.

## A.y.1.2	Derivation of UE-to-EAS keys

The following input string shall be used when the UE and the EAS derive the BEST User plane keys KE2Eenc or KE2Eint from KInt_E2E.:

-	FC = 0x61,

-	P0 = Enterprise Application Server id,

-	L0 = length of Enterprise Application Server id (i.e. 0x00 0x03),

-	P1 = algorithm id

-	L1 = length of algorithm id (i.e. 0x00 0x01)

Table A.y.1.2-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST e2e encryption key (KE2Eenc) | 0x01 |
| BEST e2e integrity Key (KE2Eint) | 0x02 |

The input key shall be equal to the concatenation KInt_E2E || Enterprise_Key of KInt_E2E  and Enterprise_Key. When the Enterprise container specifies no Enterprise key, the Enterprse_Key shall be set to zero (0..0).

Note: The Enterprise_Key provisioning is outside the scope of 3GPP.

## A.y.1.3	Derivation of EAS specific pre-shared key (KInt_EAS_psk) in HSE

The following input string shall be used when the UE and the HSE derive the pre shared key KInt_EAS_PSK from KInt_PSK

-	FC = 0x62,

-	P0 = Enterprise Application Server id,

-	L0 = length of Enterprise Application Server id (i.e. 0x00 0x03)

The input key shall be KInt_PSK, as derived in A.y.1.1.

**** End of Fifth Change ****

**** Sixth Change ****

EMSDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

IP

L2

L1

L1

S5

PDN GW

Transformation

GTP-U

UDP/IP

EMSDP

UDP/IP

IP

L2

L1

HSE

SGi

New-1

IP/UDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

L2

L1

L1

S5

PDN GW

EMSDP

GTP-U

L2

L1

HSE

SGi

New-1

EMSDP

IP/UDP

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Start Confirmation Message

5. Setup BEST Session

7. Setup BEST Session Response

9. Key derivation

6. EAS Key derivation

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Start Confirmation Message

5. Manage BEST Session

7. Manage BEST Session Response

9. Key derivation

6. EAS Key derivation

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. Setup BEST Session

7. Setup BEST Session Response

9. Key derivation

6. EAS Key derivation

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. Manage BEST Session

7. Manage BEST Session Response

9. Key derivation

6. EAS Key derivation

# Annex X (normative): End to Middle Secured Data Protocol (EMSDP)

## X.1	Introduction

The EMSDP protocol is used for the BEST service. This protocol runs between the UE and the HSE and has been optimised for efficient, secure transfer of both user plane and control plane data.

## X.2	EMSDP Protocol Description

### X.2.1	Data Stack

Figures X.2.1-1, X.2.1-2 and X.2.1-3 shows the data stacks for the EMSDP transfers (based on 3GPP TS 23.401 [2]) between the HSE and the UE.

EMSDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

IP

L2

L1

L1

S5

PDN GW

Transformation

GTP-U

UDP/IP

EMSDP

UDP/IP

IP

L2

L1

HSE

SGi

New-1

Figure X.2.1-1: Non-IP data stack for the EMSDP transfers over UP

NAS

PDCP

RLC

MAC

L1

UE

E-UTRAN

Relay

LTE - Uu

Serving GW

S1-MME

PDN GW

HSE

SGi

New-1

RLC

L1

MAC

PDCP

SCTP/IP

L2

L1

S1AP

EMSDP

IP

L2

L1

EMSDP

Relay

SCTP/IP

L1

L2

NAS

S1AP

MME

Relay

IP

L1

L2

UDP

GTP-U

GTP-U

IP

L2

L1

UDP

UDP

Relay

IP

L1

L2

IP

L2

L1

UDP

UDP

GTP-U

IP

L2

L1

GTP-U

UDP

S11

S5

Figure X.2.1-2: Non-IP data stack for the EMSDP transfers over NAS

If the UE selects the non-IP PDN data type as defined in clause 4.3.17.8. of  3GPP TS 23.401[2]. The PDN GW encapsulates the non-IP EMSDP payload in PtP (IP and UDP as detailed in clause 4.3.17.8.3.3.2 of  3GPP TS 23.401[2]) for the communication with the HSE.  In case of the EMSDP is in IP PDN, the PtP encapsulation is unnecessary, and the UE’s IP packet is directly forwarded to the HSE.  This is shown on Figure X.2.1-2.

IP/UDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

L2

L1

L1

S5

PDN GW

EMSDP

GTP-U

L2

L1

HSE

SGi

New-1

EMSDP

IP/UDP

Figure X.2.1-3: IP data stack for the EMSDP transfers

The dashed box indicates that for UE-to-EAS BEST sessions, the HSE may forward the EMSDP user plane unmodified to the EAS over the New-1 interface. The signalling plane always terminates in the HSE.

Note: It is up to the operator to decide the protocol between the HSE and the EAS. For example, an operator may agree to use XML over the New-1 interface. In that case, the HSE will have to encapsulate the user data in an agreed XML message.

### X.2.2	EMSDP general structure

This clause details a type 01 control plane EMSDP message and a type 01 user plane EMSDP message.

Figure X.2.2-1 shows the general structure of the EMSDP type 01 message:

| Control Plane type 01 message: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UP / SP Flag | RFU | Key ID | CP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |
| User Plane type 01 message: |  |  |  |  |  |  |  |  |
| UP / SP Flag | RFU | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |

Note 1: The length of these fields is indicated in the BEST service discovery from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure X.2.2-1: data stack for the EMSDP transfers

UP / CP Flag: 	This is a 1 bit field that when set to ‘1’ means that the data packet is a User Plane message and when set to ‘0’ means that the message is a control plane message.

RFU:	This is a 1 bit field that is reserved for future use. Set to ‘0’.

Key ID:	This is a 3 bit field that indicates the key being used for encryption and Integrity protection.  If no keys have been agreed between the HSE and the UE then this shall be ‘000’.

CP COUNTER:	This is a counter, used for control plane data messages, that is incremented every control plane message.  It is used to protect control plane data messages against replay attacks and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.

UP COUNTER:	This is a counter, used for user plain data messages, that is incremented every user plain data message.  It is used to protect user plain data message against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.    Its length is the value set in the BEST service discovery. For signalling messages sent outside of a BEST session, the Session ID length shall be 1 octet and the Session ID value shall be all 0’s.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST service discovery.  For an EMSDP session request command the MAC shall not be present.

Data length:	This holds the length of the following data in this data packet. Its length is set in the BEST service discovery.  This is not present if the data length is set to 0 in the BEST Service Discovery.

Data:	This is the data being transferred.

### X.2.3	EMSDP Counter Schemes

#### X.2.3.1	Optimised EMSDP counter scheme

The optimised EMSDP counter scheme has a 3 bit counter length followed by the counter value.  This allows the counter length to be changed on a per message basis.  The 3 bits are used to indicate the number of octets the counter is on, the value “000” is reserved for future use.  So for instance:

Counter value “1” is represented as “001 00000001”

Counter value “257” is represented as “010 00000001 00000001”

Counter values are rejected if the counter value is less than or equal to a valid counter value already received.  The HSE and the UE may also reject a message with a counter higher than a specific offset from the last valid counter value received.

The optimised EMSDP counter scheme is defined as scheme 01.

### X.2.4	EMSDP Integrity protection

The integrity protection algorithm to be used and the length of the MAC is indicated in the BEST service discovery.

Integrity protection is mandatory for all control plane and user plane messages except for the following control plane commands when no valid keyset is agreed between the UE and HSE:

-	an EMSDP session request command originating from the UE or the HSE.  For this command the MAC shall not be present.

For an EMSDP start session command,  originating from the HSE, if the Key Id for the message is the same as the Key Id indicated in the Key agreement TLV then the MAC shall be calculated using the new keys resulting from the authentication vectors in the Key agreement TLV.

For all other signalling plane and user plane data packets the MAC shall be computed as follows:

First the following fields are calculated (where needed for the chosen algorithm):

INPUT-I	set to the essage Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes.

COUNT-C	set to the essage Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes

M (GSM)	set to the length of message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for integrity protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for integrity protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be “0” for UE to HSE data packets and set to “1” for HSE to UE data packets.

BEARER	For control plane messages this shall be set to “00000” and for user plane messages this shall be set to “10101”

FRAMETYPE	For control plane messages this shall be set to “00” and for user plane messages this shall be set to “AA”.

KI128	This is the agreed integrity key value truncated to the lowest 128 bits.

IK	This is the agreed integrity key value truncated to the lowest 128 bits.

If GIA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [y] shall be used to generate the MAC value.  If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA4 function, then the MAC shall be truncated to the correct size from the right.

If GIA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [x] shall be used to generate the MAC value. If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA5 function, then the MAC shall be truncated to the correct size from the right.

If UIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [w] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If UIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.2.2 shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.2.3 shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the MAC-I value.  The MAC shall be set to the calculated MAC-I value truncated to the correct size from the right.

### X.2.5	EMSDP Encryption

The encryption protection algorithm to be used is indicated in the BEST service discovery.  If EEA0 is indicated then the message shall not be encrypted.

Encryption is mandatory for all control plane and user plane messages when an encryption algorithm other than EEA0 is indicated in the BEST service discovery, except for the following control plane commands when no valid keyset is agreed between the UE and HSE:

-	an EMSDP session request command originating from the UE or the HSE

-	an EMSDP start session command originating from the HSE

For all other messages the following encryption shall be applied the fields indicated in figure X.2.1-1: “data stack for the EMSDP transfers” to be encrypted.

First the following are computed (where relevant for the algorithm being used):

INPUT-I	set to the message Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes.

COUNT-C	set to the message Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes

M (GSM)	set to the length of message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for encryption protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for encryption protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be “0” for UE to HSE data packets and set to “1” for HSE to UE data packets.

BEARER	For signalling data packets this shall be set to “00000” and for user plane data packets this shall be set to “10101”

FRAMETYPE	For control plane messages this shall be set to “00” and for user plane messages this shall be set to “AA”.

KI128	This is the agreed encryption key value truncated to the lowest 128 bits.

CK	This is the agreed encryption key value truncated to the lowest 128 bits.

If GEA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [y] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If GEA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [x] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If UEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [w] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If UEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.1.2 shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.1.3 shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

### X.2.6	EMSDP Commands

#### X.2.6.1	Overview

The following EMSDP commands are proposed Table X.2.6.1-1:

Table X.2.6.1-1: EMSDP commands

| Code (Hex) | Command |
| --- | --- |
| 10 | EMSDP Session Request |
| 11 | EMSDP Session Response |
| 12 | EMSDP Session Start Confirmation |
| 20 | EMSDP Session Terminate Request |
| 21 | EMSDP Session Terminate Response |
| 30 | EMSDP Manage Keys Request |
| 31 | EMSDP Manage Keys Response |
| 80 | Message Reject |

##### X.2.6.1.1		EMSDP Session Request

The EMSDP Session Request command shall be used by the UE to trigger a new BEST session from the HSE.  This message shall includes an identification of the UE, an indication of its BEST support and the end enterprise service that this data is a part of.

This message is the EMSDP equivalent of the "BEST Session Request (UE ID, BEST capabilities, Enterprise information (opt))" detailed in clause X.

This message may be sent after a PDP context has been setup.

Note: the content of this message is used in the following EMSDP start session.

The cmd options for the EMSDP session request command are as follows:

Table X.2.6.1.1-1: Start EMSDP session request command options

| Name | M / C / O |
| --- | --- |
| IMSI TLV | M |
| BEST UE configuration TLV | M |
| Enterprise Setup Information Element TLV | O |
| Serving Network TLV | C |

IMSI TLV: This is a TLV that contains the IMSI as follows:

Table X.2.6.1.1-2: IMSI TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| IMSI TLV Tag | 1 byte | M | 01 |
| Length | 1 byte | M | Length of IMSI value (X) |
| IMSI value | X bytes | M | according to TS 24.008 [x]. |

BEST configuration TLV: This is a TLV that contains the BEST configuration details for the UE as follows:

Table X.2.6.1.1-3: BEST UE configuration TLV

| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BEST UE configuration TLV Tag = '02' |  |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |  |
| BEST release supported by the UE |  |  |  |  | BEST counter schemes supported by the UE | Reserved for future use (set to 000) |  |  |
| BEST protocols supported for control plane messages |  | Reserved for future use (set to 000) |  |  | BEST protocols supported for user plane messages | Reserved for future use (set to 000) |  |  |
| User data push to UE Supported | BEST encryption algorithm GEA4 supported | BEST encryption algorithm GEA5 supported | BEST encryption algorithm UEA1 supported | BEST encryption algorithm UEA2 supported | BEST encryption algorithm EEA0 supported | BEST encryption algorithm 128-EEA1 supported | BEST encryption algorithm 128-EEA2 supported |
| BEST encryption algorithm 128-EEA3 supported | BEST integrity algorithm GIA4 supported | BEST integrity algorithm GIA5 supported | BEST integrity algorithm UIA1 supported | BEST integrity algorithm UIA2 supported | BEST integrity algorithm 128-EIA1 supported | BEST integrity algorithm 128-EIA2 supported | BEST integrity algorithm 128-EIA3 supported |

Where:

- BEST release supported by the UE – an indicator the release of the BEST solution that the UE has been designed to.  If no release is indicated this means that the BEST service is not supported.

  - Value: This shall be a 4 bit field where "0000" = Rel.14 and "0001" to "1111" are RFU,

- BEST counter schemes supported by the UE – a flag for each supported scheme. At least one scheme must be supported and indicated.

  - Value: '1' – Optimised EMSDP counter scheme.

- BEST protocols supported for control plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

  - Value: '1' – EMSDP.

- BEST protocols supported for user plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

  - Value: '1' – EMSDP.

- User data push to UE Supported – a flag to indicate whether the UE supports user data pushed to the UE without a request for user data from the UE.

  - Values:

    - '1' = User data push to UE supported,

    - '0' = User data push to UE not supported.

- BEST encryption algorithms supported by the UE – a flag for each supported algorithm: GEA4, GEA5, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. EEA0 shall always be supported and means no encryption.

  - Values:

    - '1' = Algorithm supported,

    - '0' = Algorithm not supported.

- BEST integrity algorithms supported by the UE – a flag for each supported algorithm: GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3.

  - Values:

    - '1' = Algorithm supported,

    - '0' = Algorithm not supported.

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection as follows:

Table X.2.6.1.1-4: Enterprise Setup Information Element TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Enterprise Setup Information Element TLV Tag | 1 byte | M | 03 |
| Length | 1 byte | M | Length of IMSI value (X+1) |
| UE-to-EAS flag | 1 byte | M |  |
| Enterprise URL | X bytes | O |  |

UE-to-EAS flag: If set to 0 it indicates that the UE requests a UE-to-HSE BEST secure session. It set to 0 it means that the UE requests a UE-to-HSE BEST secure session.

Enterprise URL: The enterprise URL is used by the HSE to identify the enterprise and the service that the data belongs to.  These services are out of scope of this specification.

Serving network TLV: This is a TLV that contains information on the serving network.

Table X.2.6.1.1-5: Serving Network TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Enterprise Setup Information Element TLV Tag | 1 byte | M | 0B |
| Length | 1 byte | M | Length of IMSI value (3) |
| MCC/MNC | 3 bytes | M | MCC/MNC as defined in 3GPP TS 24.008 [x] |

Response:

If the HSE agrees to setup the session, it shall respond with an EMSDP start session command.

If the HSE does not agree to setup a BEST session, it may respond with a Request Rejected command.  This command may include the reason that the request has been rejected.

##### X.2.6.1.2		EMSDP Session Start

The EMSDP Session Start command is used by the HSE to setup a new BEST session.  This message shall contain information on the BEST service setup, key agreement details, a hash of the information sent by the UE in the prior EMSDP session request command and optionally, the HSE identity.

On recept of this command the UE shall:

- Perform a 3G security context authentication with the USIM using the RAND and AUTN combination from the Key Agreement TLV. If the USIM returns IK and CK values, the UE uses these keys and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause A.y.  If the USIM determines re-syncronisation is required and returns an AUTS then the UE sends a Request Rejected command containing the AUTS to the HSE.

- Verify that the UE supports the BEST service indicated in the BEST Service configuration TLV.

- Verify the received message format, the CP COUNTER value and the message MAC value.

- Verify that the MAC supplied in the MAC TLV matches the MAC that would be produced for the previous EMSDP session request message if the BEST configuration in the BEST Service configuration TLV had been applied using the integrity key calculated from the Key agreement TLV.

The Start new EMSDP session response command has the following cmd options:

Table X.2.6.1.2-1: EMSDP Session Start command options

| Name | M / C / O |
| --- | --- |
| BEST Service configuration TLV | M |
| Key agreement TLV | M |
| EMSDP session request MAC TLV | C |
| HSE Identity TLV | O |

BEST Service configuration TLV: The BEST Service configuration TLV sets the BEST service parameters to be used in this session as follows:

Table X.2.6.1.2-2: BEST UE configuration TLV

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BEST HSE configuration TLV Tag = '04' |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |
| BEST Service Activated | BEST encryption algorithm GEA4 to be used | BEST encryption algorithm GEA5 to be used | BEST encryption algorithm UEA1 to be used | BEST encryption algorithm UEA2 to be used | BEST encryption algorithm EEA0 to be used | BEST encryption algorithm 128-EEA1 to be used | BEST encryption algorithm 128-EEA2 to be used |
| BEST signalling plane protocol identifier |  |  |  |  |  |  |  |
| BEST user plane protocol identifier |  |  |  |  |  |  |  |
| BEST encryption algorithm 128-EEA3 to be used | BEST integrity algorithm GIA4 to be used | BEST integrity algorithm GIA5 to be used | BEST integrity algorithm UIA1 to be used | BEST integrity algorithm UIA2 to be used | BEST integrity algorithm 128-EIA1 to be used | BEST integrity algorithm 128-EIA2 to be used | BEST integrity algorithm 128-EIA3 to be used |
| Reserved for future use (set to 000) |  |  | Local BEST configuration management allowed | EMSDP session ID length |  |  |  |
| Reserved for future use (set to 00) | Use EAS UP Keys | EMSDP MAC length |  | Size of EMSDP Data Length |  |  |  |

- BEST Service Activated – a bit flag that when set instructs the UE to use the BEST service and when clear instructs the UE not to use the BEST service,

- BEST signalling plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 signalling plane EMSDP message. All other values are reserved for future use.

- BEST user plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 user plane EMSDP message. All other values are reserved for future use.

- BEST encryption algorithm to be used – 1 octet that is used to define which of the following algorithms to use for encryption: GEA0, GEA4, GEA5, UEA0, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. Only one algorithm from this list shall be indicated.  If the Visited network indicated that BEST encryption is restricted, then the HSE shall indicate EEA0.

- BEST integrity algorithm to be used – 1 octet that is used to define which one of the following algorithms to use for integrity:GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3). Only one algorithm from this list shall be indicated.

- BEST counter scheme to be used – a flag for each supported scheme.  4 bits that are used to define which one of the following counter schemes to use: "0001" = the optimised EMSDP counter scheme. All other values are reserved for future use. Only one counter scheme from this list is allowed to be indicated.

- Local BEST configuration management allowed – a flag to indicate that the software connected to the UE is allowed to manage the BEST service.

- EMSDP session ID length – 4 bits that indicates how many octets in the EMSDP data packet the EMSDP session ID will be on.

- New Session Required – 1 bit that indicates if a new session is required.  If this bit is set to 0 then the details agreed for the last session can be used and a new session is not required to be setup.

- EMSDP MAC length – 2 bits that indicates how many octets in the EMSDP data packet the integrity checksum (MAC) will be on, as follows: "00"=4 octets, "01"=8 octets, "10"=12 octets and "11"= 16 octets.  This value shall not be set to a size that is greater than MAC size produced by the chosen algorithm.

- Size of EMSDP Data Length – 4 bits that indicate how many octets are used for the EMSDP Data Length. "0000" is reserved for future use.

Any remaining bits are reserved for future use and are set to "0..0".

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [z] as follows:

Table X.2.6.1.2-3: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | 05 |
| Length | 1 byte | M | 1 or 36 |
| Additional Information | 1 Byte | M | Additional Keys to be generated |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [z] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [z] |

If the 'Length of the Key agreement' is set to 1 then this means use current keyset agreed for this KEY ID.  In this case the RAND IE and AUTN IE shall not be present.

The Additional information are as follows:

Table X.2.6.1.2-4: Additional information

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Confirm Authentication flag | RFU | RFU | RFU | RFU | Key ID |  |  |

b8: 	Confirm Authentication flag:

1 = Confirmation message required.

0 = Confirmation message not required.

b7 to b4: RFU (set to 0)

b3 to b1: Key ID

Key ID to be used for this keyset.

EMSDP session request MAC TLV:  The EMSDP session request MAC TLV shall be present if the previous command was an EMSDP session request message. Its contents are as follows:

Table X.2.6.1.2-5: EMSDP session request MAC TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| EMSDP session request MAC Tag | 1 byte | M | 06 |
| Length | 1 byte | M | Length of HSE Identity (X) |
| EMSDP session request MAC | X bytes | M | Result of MAC calculation on previous EMSDP session request message using current keys and BEST configuration in this message. |

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network. It is formatted as follows:

Table X.2.6.1.2-6: HSE Identity TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| HSE Identity Tag | 1 byte | M | 07 |
| Length | 1 byte | M | Length of HSE Identity ('04') |
| HSE Identity | 4 bytes | M | 4 octet numeric identifier for the HSE |

Response:

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE shall send an EMSDP start session confirmation message.

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE may send an EMSDP start session confirmation message.

If the message does not verify, then the UE shall respond with a Request Rejected command.  This command may include the reason that the request has been rejected.

If the USIM returns a AUTS as a result of the authentication, the the UE shall respond with a Request Rejected command with the reason "Authentication ReSync required" and including the AUTS.

##### X.2.6.1.3		EMSDP Session Start Confirmation message

The EMSDP start session infoirmation message is sent by the UE to confirm a previous EMSDP start session command.

This message is optional for the UE to send unless the "Confirmation message required" flag is set in the EMSDP start session command in which case this message shall be sent.

This command has the following cmd options:

Table X.2.6.1.3-1: EMSDP start session confirmation command options

| Name | M / C / O |
| --- | --- |
| AUTHENICATION RESPONSE TLV | M |

AUTHENICATION RESPONSE TLV: This TLV contains the authentication response for a successful authentication as follows:

Table X.2.6.1.3-2: AUTHENICATION RESPONSE TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| AUTHENICATION RESPONSE Tag | 1 byte | M | 08 |
| Length | 1 byte | M | Length of HSE Identity (X+1) |
| Key Information | 1 byte | M | Key identifier |
| RES | X bytes | M | As returned by the USIM. |

Where:

Key Information is coded:

Table X.2.6.1.3-3: Key Information information

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFU | RFU | RFU | RFU | RFU | Key ID |  |  |

b8 to b4: RFU (set to 0)

b3 to b1: Key ID

Key ID to be used for this keyset.

##### X.2.6.1.4		EMSDP Session Terminate Request and Response

The EMSDP Session Terminate Request may be sent by either the UE or the HSE to terminate an existing EMSDP session.

The sending party shall close their session on sending this command and the receiving party shall close the session upon reception of this command.  All future BEST User Plane messages and BEST control plane messages for the session indicated in the session ID shall be either refused or ignored.

This command has no cmd options.

##### X.2.6.1.5	EMSDP Manage Keys Request

The EMSDP manage Keys request command shall be sent by the UE to request the HSE to negotiate new key material.

If the BEST session requested includes cyphering then the UE shall send a Serving Network TLV (as defined in clause X.2.6.1.1).

##### X.2.6.1.6		EMSDP Manage Keys Response

The EMSDP manage Keys command is used by the HSE to agree new keys.

The EMSDP manage Keys command has the following cmd options:

Key agreement TLV: As detailed in clause X.2.6.1.2.

##### X.2.6.1.7		EMSDP Message Reject command

The Message Reject command may be used by either the UE or the HSE to reject messages, data or control plane.

The Message Reject command has the following cmd options:

Table X.2.6.1.7-1: Request Rejected command options

| Name | M / C / O |
| --- | --- |
| Rejection details TLV | C |
| AUTS TLV | C |

Where:

Rejection details TLV: Contains the rejection reason.  If the rejection reason is "Authentication ReSync required" then the Rejection details TLV and the AUTS TLV shall be present.  For all other reasons the rejection details TLV may be present.

Table X.2.6.1.7-2: Rejection details TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Rejection details Tag | 1 byte | M | 09 |
| Length | 1 byte | M | Length of HSE Identity (X+1) |
| Rejection reason | 1 byte | M | See below |

Rejection reason:

'00' = "Best session refused by the HSE"

'01' = "HSE not compatible with configuration request"

'02' = "UE not compatible with configuration request"

'03' = "HSE temporary error – try again later"

'04' = "Command Message error"

'05' = "Command message counter error"

'06 = "Authentication ReSync required"

'07' = "EMSDP session request MAC incorrect"

'08' = "Sesion ID not valid"

'09' = "Command not allowed"

Table X.2.6.1.7-3: AUTS TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Rejection details Tag | 1 byte | M | 0A |
| Length | 1 byte | M | Length of HSE Identity (X) |
| AUTS | 1 byte | M | AUTS as returned by the USIM |

### X.2.7	Procedures for BEST when using EMSDP

#### X.2.7.1	Introduction

This clause contains the procedures between the UE, HSE, HSS and EAS for the following BEST Services when using EMSDP:

- BEST key agreement only service in clause X.2.7.2

- BEST user plane integrity protected service in clause X.2.7.3

- BEST user plane confidential service in clause X.2.7.4

#### X.2.7.2	Procedures for BEST Key Agreement Only Service using EMSDP

Figure X.2.7.2-1 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to setup a BEST Session when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Start Confirmation Message

5. Setup BEST Session

7. Setup BEST Session Response

9. Key derivation

6. EAS Key derivation

Figure X.2.7.2-1: BEST Session Setup Procedure for BEST Key Agreement Only Service using EMSDP

In the above figure, steps 1-8 are mandatory and step 10 is optional depending on whether the HSE has set the 'Confirm Authentication' flag in the 'EMSDP Session Response' message. The contents of the respective commands are given in the respective clauses that detail the commands.

NOKIA: Do we also need to support a pull model where EAS will pull keys from HSS? In other words, instead of steps 5-7, EAS will pull the keys from HSS when UE establishes a communication session with it based on any application layer protocol?

Figure X.2.7.2-2 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to refresh the keys when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Start Confirmation Message

5. Manage BEST Session

7. Manage BEST Session Response

9. Key derivation

6. EAS Key derivation

Figure X.2.7.2-2: BEST Procedure for Key Refresh using EMSDP

In the above figure, steps 1-8 are mandatory and step 10 is optional depending on whether the HSE has set the 'Confirm Authentication' flag in the 'EMSDP Session Response' message. The contents of the respective commands are given in the respective clauses that detail the commands.

#### X.2.7.3	Procedures for BEST User Plane Integrity Protected Service using EMSDP

Figure X.2.7.3-1 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to setup a BEST Session when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. Setup BEST Session

7. Setup BEST Session Response

9. Key derivation

6. EAS Key derivation

Figure X.2.7.3-1: BEST Session Setup Procedure for BEST User Plane Integrity Only Service using EMSDP

In the above figure, steps 1-8 are mandatory. The contents of the respective commands are given in the respective clauses that detail the commands.

Figure X.2.7.3-2 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to refresh the keys when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. Manage BEST Session

7. Manage BEST Session Response

9. Key derivation

6. EAS Key derivation

Figure X.2.7.3-2: Best Session Setup Procedure for BEST Key Agreement Only Service using EMSDP

In the above figure, steps 1-8 are mandatory. The contents of the respective commands are given in the respective clauses that detail the commands.

#### X.2.7.4	Procedures for BEST User Plane Confidential Service using EMSDP

The procedures for the BEST User Plane Confidentiality Service are the same as the one detailed in clause X.7.4.3.

**** End of Sixth Change ****

**** Seventh Change ****

# Annex Y (informative): Structure of APN names for BEST

## Y.1 Introduction

Due to regulatory requirements, operators may have to disable the best service for UEs roaming in their network. A simple, but effective method is rely on the MME capability to filter specific APNs. For that reason, a BEST APN structure is proposed in this section. Operators may agree on a different method in bilateral requirements.

## Y.2 Structure of APN names for BEST

APNs that are used for the BEST service may reside in a specific subdomain of the operator. It is proposed to use the subdomain 'best' for this specific service.

Note 1: For example, if the operator uses operator.cc as a domain for APNs, the best servers should be in the domain 'best.operator.cc'. So 'apn.best.operator.cc' would be an APN that is best capable..

**** End of Seventh Change ****

**** Eighth Change ****

# Annex Z (normative): BEST Service Management

## Z.1	Ability to Enable and Disable the BEST service

The HSE may allow the enterprise to manage the BEST service.  The service shall allow enable / disable of the BEST service down to an individual UE level. It is anticipated that the interface for the management of the BEST service between the enterprise and the HSE will be out of scope for 3GPP.  However it is expected that such an interface would have the following abilities:

- a mechanism that allows the service to be removed for a specific subscriber at the network end (probably re-using  a service provisioning service).

- the ability for the enterprise to enable / disable the E2M security service for one or many of their endpoints through communication with the operators CIoT interface.  (This may be because other security mechanisms are being used).

- The ability for the home network to disable the BEST service due to non-payment or security concerns.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is not currently active with that UE then the changes shall take effect the next BEST session.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is currently active with that UE then the HSE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may allow the application connected to it to manage the BEST service if the Local BEST configuration management flag within the protocol configuration options of a ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST indicates this is allowed.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is not currently active then the changes shall take effect the next BEST session.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is currently active in that UE then the UE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may choose not to accept data that does not meet the defined security requirements.

When the BEST service is disabled the HSE and UE shall act as though the BEST service is not supported.

**** End of Eighth Change ****

**** Nineth Change ****

# Annex W (informative) HSE to EAS interface based on Restful HTTP

## W.1	Introduction

This annex contains example of a RESTful HTTP interface between the HSE and the EAS.

## W.2 Restful HTTP interface

It is described as follows:

-	TCP provides communication service at the transport layer

-	TLS provides security to the communication

-	HTTP based transport of XML data

-	XML documents used to embed specific datastructures, such as keys etc.

Figure A.2-1 illustrates the protocol stack of this reference point.

Figure: A.2-1 RESTful HTTPS between EMKS and EMSE

The HSE and the EAS act as both HTTP client and server. The underlying TCP connection permits bidirectional communication between the EAS and the HSE.

RESTful HTTP is used at the application delivery layer. The content type of the RESTful HTTP is XML.

The unsecured HTTP protocol shall be combined with TLS, as HTTPS, to provide confidentiality and integrity protection. Mutual authentication shall be enabled in TLS for authenticating and allowing only an authorized third party EMSE to access the EMKS. The profile for TLS implementation and usage shall follow the provisions given in 3GPP TS 33.310 [6], Annex E.

## W.2.1	Procedures over the RESTful HTTP reference point

Following procedures are supported over the RESTful HTTP reference point:

-	Initial registration by the EAS

-	Obtaining UE specific e2m_key from EAS

-	Deregistration by EAS

#### W.2.1.1 Initial registration by EMSE

The EAS registers with the HSE by sending an HTTP POST message to the HSE including its identifier (EAS_Id). The HSE establishes a session context for this EAS and returns session id in HTTP 201 CREATED.

#### W.2.1.2 Obtaining UE specific pre-shared key from the EAS

The EAS obtains UE specific pre-shared key from the HSE by sending an HTTP GET message including an identity of the UE and a key identifier received from the UE.

#### W.2.1.3 Deregistration by EAS

When the session needs to be terminated, EAS may send an HTTP DELETE message including the session ID to the HSE.

**** End of Nineth Change ****