3GPP TSG SA WG3 (Security) Meeting #87	    S3-171184

15-19 May 2017, Ljubljana

Source:	NEC

Title:	pCR to TR 33.899: Update of solution #4.6 on security mechanism for deployment scenario of option 3

Document for:	Approval

Agenda Item:	8.3.4

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes update of solution #4.6 on security mechanism for deployment scenario of option 3 (EN-DC) in the area of RAN security of TR 33.899.

#### Introduction

This pCR updates solution #4.6 “Security mechanism for deployment scenario of option 3” in TR 33.899 with the following five points and minor changes for expression. (1) Identifiers and names of new security algorithm for NR (New Radio), (2) Update of IEs (Information Element) (3) NR subscription information (4) Key derivation in Secondary gNB and (5) Addition of evaluation.

- Identifiers and names of new security algorithm for NR (New Radio)

4-bit identifiers and names for the algorithms are required for 5G as in clauses 5.1.3.2 and 5.1.4.2 of TS 33.401 for SAE/LTE. An example of the definition is shown below.

Encryption algorithms for NR in option 3, next generation system or 5GS

"00002"      NEA0            Null ciphering algorithm

"00012"      NEA1            Algorithm 1 (Same as 128-EEA1)

"00102"      NEA2            Algorithm 2 (Same as 128-EEA2)

"00112"      NEA3            Algorithm 3 (Same as 128-EEA3)

:

Integrity protection algorithms for NR in option 3, next generation system or 5GS

"00002"      NIA0            Null Integrity Protection algorithm

"00012"      NIA1            Algorithm 1 (Same as 128-EIA1)

"00102"      NIA2            Algorithm 2 (Same as 128-EIA2)

"00112"      NIA3            Algorithm 3 (Same as 128-EIA3)

:

- Update of IEs (Information Element)

In order to send security algorithms for NR as part of UE’s capability, IEs of “UE network capability” and “UE security capability” defined in section 9.9.3.34 and 9.9.3.36 of TS 24.301, respectively, should be updated by adding fields for the algorithms. Furthermore, one field of “NG-RAN” to show the UE has NR capability is also added to “UE network capability” IE to show whether the UE has NR capability or not.

- NR subscription information

HSS and MME store NR subscription information as “Access Restriction” defined in section 5.7.1 and 5.7.2 of TS 23.401, which has been updated by CR (S2-172397) in SA2. The NR subscription information is sent from MME to eNB with initial context setup request. The initial attach procedure shown in Figure 5.4.4.6.2-1 and the corresponding steps are updated with this information.

- Key derivation in secondary gNB

Since SRB (Signalling Radio Bearer) is established in secondary gNB for option 3, C-plane keys such as KRRCint and KRRCenc for integrity and confidentiality protection should also be derived as well as U-plane keys for DRB (Data Radio Bearer). This description is added to gNB addition procedure of the solution.

- Addition of evaluation

Evaluation mentions that the security mechanism for option 3 is provided based on the procedure for dual connectivity in E-UTRAN with UE’s access control to NR. It’s also mentioned that the security algorithms for NR should be defined in the existing specifications to be included in attach request and initial context setup request.

#### Proposal

***** Start of Change *****

#### 5.4.4.6	Solution #4.6: Security mechanism for deployment scenario of option 3

Editor’s Note: The SA3 study of security aspects of dual connectivity should be aligned with progress in the on-going study of dual connectivity in RAN2 and RAN3

##### 5.4.4.6.1	Introduction

This solution provides security mechanism for deployment scenario of option 3 to address key issue #4.3 "Security aspects for dual connectivity". During initial attach procedure, UE’s NR (New Radio) capability including security algorithms and NR subscription information are provided to MeNB. Then, MeNB authorizes the UE by checking the stored information before adding SgNB.

##### 5.4.4.6.2	Solution details

5.4.4.6.2.1	Security identifiers and names

Identifiers and names of security algorithms used for NR are defined as in section 5.1.3.2 and 5.1.4.2 of TS 33.401 as follows.

Encryption algorithms for NR in option 3, next generation system or 5GS

"00002"      NEA0            Null ciphering algorithm

"00012"      NEA1            Algorithm 1 (Same as 128-EEA1)

"00102"      NEA2            Algorithm 2 (Same as 128-EEA2)

"00112"      NEA3            Algorithm 3 (Same as 128-EEA3)

:

Integrity protection algorithms for NR in option 3, next generation system or 5GS

"00002"      NIA0            Null Integrity Protection algorithm

"00012"      NIA1            Algorithm 1 (Same as 128-EIA1)

"00102"      NIA2            Algorithm 2 (Same as 128-EIA2)

"00112"      NIA3            Algorithm 3 (Same as 128-EIA3)

:

In order to send security algorithms for NR as part of UE’s capability, IEs of “UE network capability” and “UE security capability” defined in section 9.9.3.34 and 9.9.3.36 of TS 24.301, respectively, should be updated by adding fields for the algorithms. Table 5.4.4.6.2-1 and 5.4.4.6.2-2 show examples for the update of the IEs. Furthermore, one field of “NG-RAN” to show the UE has NR capability is also added to “UE network capability” IE to show whether the UE has NR capability or not.

Table 5.4.4.6.2-1: Example of UE network capability IE

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE network capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE network capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| EEA0 |  | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 3 |  |
| EIA0 |  | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EIA7 |  | octet 4 |  |
| UEA0 |  | UEA1 |  | UEA2 |  | UEA3 |  | UEA4 |  | UEA5 |  | UEA6 |  | UEA7 |  | octet 5* |  |
| UCS2 |  | UIA1 |  | UIA2 |  | UIA3 |  | UIA4 |  | UIA5 |  | UIA6 |  | UIA7 |  | octet 6* |  |
| ProSe-dd |  | ProSe |  | H.245-ASH |  | ACC-CSFB |  | LPP |  | LCS |  | 1xSR VCC |  | NF |  | octet 7* |  |
| ePCO |  | HC-CP CIoT |  | ERw/oPDN |  | S1-U data |  | UP CIoT |  | CP CIoT |  | Prose-relay |  | ProSe-dc |  | octet 8* |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | NG-RAN |  | V2X PC5 |  | multipleDRB |  | octet 9* |  |
| NEA0 |  | NEA1 |  | NEA2 |  | NEA3 |  | NEA4 |  | NEA5 |  | NEA6 |  | NEA7 |  | octet 10* |  |
| NIA0 |  | NIA1 |  | NIA2 |  | NIA3 |  | NIA4 |  | NIA5 |  | NIA6 |  | NIA7 |  | octet 11* |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 12* -15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.4.4.6.2-2: Example of UE security capability IE

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE security capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE security capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| EEA0 |  | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 3 |  |
| EIA0 |  | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EIA7 |  | octet 4 |  |
| UEA0 |  | UEA1 |  | UEA2 |  | UEA3 |  | UEA4 |  | UEA5 |  | UEA6 |  | UEA7 |  | octet 5* |  |
| 0 spare |  | UIA1 |  | UIA2 |  | UIA3 |  | UIA4 |  | UIA5 |  | UIA6 |  | UIA7 |  | octet 6* |  |
| 0 spare |  | GEA1 |  | GEA2 |  | GEA3 |  | GEA4 |  | GEA5 |  | GEA6 |  | GEA7 |  | octet 7* |  |
| NEA0 |  | NEA1 |  | NEA2 |  | NEA3 |  | NEA4 |  | NEA5 |  | NEA6 |  | NEA7 |  | octet 8* |  |
| NIA0 |  | NIA1 |  | NIA2 |  | NIA3 |  | NIA4 |  | NIA5 |  | NIA6 |  | NIA7 |  | octet 9* |  |

5.4.4.6.2.2	Attach procedure

Procedure of initial attach for option 3 with security mechanism is shown in Figures 5.4.4.6.2.2-1.

The procedure is carried out with the following steps:

- Attach request with UE NR capability is sent from UE to eNB.

- eNB sends the attach request from UE to MME.

- MME stores UE’s capability and NR subscription information. The NR subscription information is stored as part of Access Restriction defined in TS 23.401, and it can be fetched from HSS if MME doesn’t have it.

- MME sends initial context setup request to eNB with UE security capabilities (both LTE and NR) and NR subscription information.

- eNB sends the RRC connection reconfiguration to UE.

- eNB stores the information on the UE capability and NR subscription received from MME.

Figure 5.4.4.6.2.2-1: Initial attach procedure for option 3

5.4.4.6.2.3	gNB addition procedure

Procedure of gNB addition for option 3 with security mechanism is shown in Figures 5.4.4.6.2.3-1.

Figure 5.4.4.6.2-2: gNB addition procedure for option 3

gNB addition procedure is carried out with the following steps:

- RRC connection is established between UE and MeNB.

- MeNB checks UE’s NR capability and NR subscription information that the UE is authorized to access gNR If UE doesn’t have the capability or access right to NR, MeNB doesn’t proceed SgNB addition procedure. If MeNB doesn’t have UE’s NR capability and/or NR subscription information, MeNB requests them to UE or MME.

- MeNB derives a key (S-KgNB) from KeNB used in SgNB for integrity and confidentiality protection.

- MeNB sends SgNB addition request to SgNB. The request includes S-KgNB and UE’s NR capability including security algorithms.

- SgNB decides security algorithms used for integrity and confidentiality protection based on the received UE’s capability information.

- SgNB derives security keys for integrity and confidentiality protection from S-KgNB. Separate keys are derived to protect SRB (Signalling Radio Bearer) and DRB (Data Radio Bearer).

- SgNB sends SgNB addition request acknowledge, which contains NR radio resource configuration including the decided security algorithms, to MeNB.

- MeNB sends RRC connection reconfig request including the NR radio resource configuration (e.g., the decided security algorithms) to UE.

- UE sends RRC connection reconfig response to MeNB.

- MeNB sends gNB Reconfiguration complete message to SgNB.

- Activation of encryption/decryption is carried out in UE.

- Activation of encryption/decryption is carried out in SgNB.

eNB

MME

UE

gNB

5. Attach response with UE capability check response

1. Attach request

(UE NR capabilities, …)

2. Attach request(UE capability check request, …)

3. UE capabilities and

NR subscription check

4. Attach response with UE capability check response

6. UE capabilities to be authorized to access NR are stored

Figure 5.4.4.6.2-1: Initial attach procedure for option 3

eNB

MME

UE

gNB

1. RRC connection establishment

Initial attach procedure

(UE req. algo./KDF IDs,

UE capability)

2. Check authorized UE capabilities to access

NR

3. KAN Derivation

4. gNB addition request

(KAN, UE req. algo./KDF IDs,

UE capability)

5. Decision

6. gNB addition response

(decided algo./KDF IDs)

7. RRC connection reconfig request

(decided algo./KDF IDs)

8. RRC connection reconfig response

9. gNB Reconfiguration complete

10. Activation of encryption /decryption

11. Activation of

encryption /decryption

##### 5.4.4.6.3	Evaluation

The solution allows to provide the same or better level of protection for SgNB compared to MeNB by checking the UE’s NR capability before establishing connection to SgNB. In addition, authorization of UE to access NR is checked using NR subscription information, which is provided from MME to MeNB.

### ***** End of Change *****