# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171832

# 7-11 August, 2017, Dali, China	revision of S3-17xabc

# Source:	Huawei, Hisilicon

# Title:	Interim Agreement to Key Issue #1.5 and #1.6

# Document for:	Approval

Agenda Item:	8.3.1

1	Decision/action requested

This contribution adds questions and interim agreements for Issues #1.5, #1.6

2	References

[1] 3GPP TS 23.502v0.4.0 (2017-6) Procedures for the 5G System; Stage 2

[2] 3GPP TS 33.401v15.0.0 (2017-6) 3GPP System Architecture Evolution (SAE); Security architecture (Release 15)

3	Rationale

SMS over NAS procedure has already been contained in clause 4.13.3 in TS 23.502 [1], and UE could send MO SMS to the network or receive MT SMS from the network. There are 6 procedures to transfer SMS over NAS, and most of them transfer the SMS in NAS uplink/downlink message. Thus, general NAS integrity and confidentiality mechanism shall be used to guarantee SMS integrity and confidentiality.

However, for MO SMS using one step approach in CM-IDLE in clause 4.13.3.4, SMS is transferred in the initial NAS message (after successful negotiation, so UE and AMF already have NAS security context). In LTE, for Control Plane CIoT EPS optimization solution, the uplink small data is transferred in control plane service request message. For integrity, the uplink small data could be guaranteed by the NAS integrity mechanism of control plane service request message. For confidentiality, the related solution can be found in clause 8.2 in TS 33.401 [2], the control plane service request is partially ciphered. Since the Control Plane CIoT EPS optimization solution in LTE is similar with the MO SMS using one step approach in CM-IDLE, so, general NAS integrity mechanism shall be used to guarantee SMS integrity, and the solution 1.48 which reuse the partial cipher mechanism in LTE shall be the baseline of confidentiality protection of this scenario.

4	Detailed proposal

**********************Begin of 1st changes********************************

E.1.5.X	Shall integrity for SMS over NAS be standardized in phase I?

E.1.5.X.1 Description of Question

Shall integrity for SMS be standardized in phase I?

E.1.5.X.2 Interim Agreement

Yes.

**********************End of 1st changes********************************

**********************Begin of 2nd changes********************************

E.1.6.X	Shall confidentiality for SMS over NAS be standardized in phase I?

E.1.6.X.1 Description of Question

Shall confidentiality for SMS be standardized in phase I?

E.1.6.X.2 Interim Agreement

Yes.

**********************End of 2nd changes********************************