# 3GPP TSG SA WG3 (Security) Meeting #89	S3-173112

# 27 November- 1 December, 2017, Reno (US)	resubmission of S3-172417

# Source:	Nokia, TIM

# Title:	Clause 5.2.9 Requirements for gNB DU-CU interfaces

# Document for:	approval

Agenda Item:	7.2.4

# 1	Decision/action requested

Approve the text changes

# 2	References

[1]	3GPP TS 33.501 v0.3.0

[2] 			3GPP TS 23.501 v1.1.0

[3] 			3GPP TS 38.470 v030

[4] 	           3GPP TS 38.472 v030

[5]	           3GPP TS 38.474 v010

# 3	Rationale

1. Current text in clause 5.2.9 mentions ‘gNB internal’ interface for split DU-CU implementations. The CU-DU interface is an open interface as per TS 38.470. The wording is corrected and a reference to TS 38.470 is inserted. Some general text has been added to mention the UP interface and the CP interface carrying two types of packets (F1 management + signaling bearer traffic transporting AS and NAS signalling packets).

2. Current requirements 2 and 3 refers to user plane traffic and is separately worded causing confusion, this is combined in to one requirement.

3. An Editor’s Note is added at the very beginning, since TS 38.470 has very little text now and is still evolving, TS 38.472 for F1-C and TS 38.474 for F1-U are also relevant specifications to be referenced.

4. A separate requirement added for the F1-C interface containing management traffic (by now only 3 messages for Reset, Setup and Error Indication of the F1-C). Since this type of traffic is different from the traffic transporting the RRC signalling packets), it cannot be hanlded in the same way. In particular the interface management traffic should be always protected in terms of authenticity and integrity and confidentiality, while the second one may not be required if over the air protection is enabled for signaling bearer traffic and the PDCP-C is terminated in the CU.

4. Reference to TS 38.470, TS 38.472, TS 38.474 added to Clause 2 References.

************************************* start of pCR  change1 *************************************

### 5.2.9	Requirements for gNB F1 interfaces

Editor’s Note: TS 38.470 for the F1 interface, TS 38.472 for the F1-C interface, TS 38.474 for F1-U are still evolving, detailed requirements if any are FFS.

Requirements given below apply to gNBs with split DU-CU implementions using F1 interface defined in TS 38.470[x]. Signalling traffic (i.e. both F1-C interface management traffic defined in TS 38.470[x] and F1-C signalling bearer defined in TS 38.472[xx] and user plane data may be sent on the F1 interface between a given DU and its CU.

1.	 gNB shall support confidentiality, integrity and anti replay protection for F1-C signalling bearer [xx].

2. 	F1-C interface management traffic defined in TS 38.470 [x] shall be integrity, confidentiality and anti-replay protected.

2.	gNB shall support confidentiality, integrity and anti replay protection on the gNB DU-CU F1-U interface[xxx] for user plane.

********************************************* end of pCR change 1 *********************************

******************************************** start of pCR change 2 *********************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System"

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[8]	3GPP TS 23.502: "Procedures for the 5G System"

[9]	3GPP TS 33.102: "3G security; Security architecture"

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture"

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses"

[12]	IETF RFC 5488: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')"

[13]	3GPP TS 24.301: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3"

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NG Radio Packet Data Convergence Protocol (PDCP)".

[x]	3GPP TS 38.470: “ NG-RAN; F1 General aspects and principles“.

[xx]	3GPP TS 38.472: “NG-RAN; F1 interface control plane protocol”.

[xxx] 	3GPP TS 38.474: “NG-RAN; F1 data transport”.

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

********************************* end of pCR change 2 ***************************************