# 3GPP TSG SA WG3 (Security) Meeting #89	S3-173436

# 27 November-1 December, 2017, Reno (US)	revision of S3-173278

# Source:	Orange, Gemalto, Oberthur Technologies, Morpho, CATT, KPN, Vodafone, Deutsche Telekom AG

# Title:	pCR- Updating 5.1.5 Subscriber privacy

# Document for:	Approval

Agenda Item:	7.2.14

# 1	Decision/action requested

This contribution clarifies where the calculation of the SUCI takes place.

# 2	References

[1]	3GPP TS 33.501

# 3	Rationale

In 5.1.5 subscriber privacy of TS 33.501, there are an editor’s note “Where the calculation of SUCI is done is FFS”. This pCR resolves this Editor’s Note.

This contribution is revision of contributions S3-173278, S3-173306, S3-173384.

# 4	Detailed proposal

***	BEGIN CHANGES	***

### 5.1.5	Subscriber privacy

Editor's Note: This subclause will contain general, high-level requirements on the UE related to at least the core network subscription identifiers (i.e., SUPI/SUCI and 5G-GUTI). Requirements should not unnecessarily delve into solutions.

UE shall support 5G-GUTI.

The SUPI should not be transferred in clear text over 5G RAN except routing information, e.g. Mobile Country Code (MCC) and Mobile Network Code (MNC).

The home network public key shall be stored on the tamper resistant secure hardware component. If the home network has not provisioned the public key in the tamper resistant secure hardware component, the SUPI protection in initial attach procedure is not provided.

Editor's Note: Whether the ME will choose the "null-scheme" if the tamper resistant secure hardware component does not include the home network public key is FFS. For example, whether the ME will choose the "null-scheme" if the public key is not stored in the USIM.

Editor's Note: It is FFS how the UE can connect to 5G network without concealing the SUPI.

.

Based on operator’s decision, indicated by the USIM, the calculation of the SUCI shall be performed either by the USIM or by the ME.

NOTE xx: If the indication is not present, the calculation is in the ME.

Provisioning, and updating the home network public key shall be in the control of the home network.

NOTE:	An example of the provisioning and updating of the home network public key is implemented using the Over the Air (OTA) mechanism. The provisioning and updating of the home network public key is out of the scope of the present document.

*** END OF CHANGES ***