3GPP TSG-SA WG3 Meeting #91 	S3-181970

# Belgrade (Serbia), 16-20 April          revision of S3-181496, merger of S3-181393, S3-181498, S3-181476, S3-181499, S3-181439,S3-181795,S3-181967

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0145 | rev | 5 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to Subscription identifier privacy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, NEC Corporation, NTT DOCOMO, Nokia, Qualcomm Incorporated, China Mobile, Apple, Deutsche Telekom, Sony, KPN, BT Group, Intel, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-04-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See S3-181393, S3-181498, S3-181476, S3-181499, S3-181439, S3-181795, S3-181967 |  |  |  |  |  |  |  |  |
| Summary of change: |  | S3-181393 (Ericsson): SUCI – readiness to protection schemes update in future Clause 6.12.2: addition of mechanism for protection scheme negotiation between USIM and ME S3-181498 (CATT): Clause 6.12.2 Wording and correction related to SUCI Clause 6.12.2: wording improvements and error corrections S3-181476 (NEC, NTT DOCOMO, Nokia): Resolving EN on sending SUCI in the Identifier Response message Clause 6.12.2: clarifies that SUCI shall be sent in the Identifier Response message in response to an Identifier Request message from the network, deletes the related editor’s note S3-181499 (QC): Limiting SUCI in Identifier Response Clause 6.12.4: A requirement is introduced so that UE implements a mechanism to limit the number of Identifier Requests that it will respond to. S3-181439 (NTT DOCOMO): Editorials to 33.501 ' Clauses 6.12.1, 6.12.2, 6.12.3, 6.12.4: editorials S3-181795(China Mobile):Remove the editor’s note in section 6.12.1 S3-181967(Apple, Deutsche Telekom, Sony, KPN, China Mobile, Qualcomm Incorporated, BT Group, Intel): When SUCI calculation is performed by ME, the home network operator shall provision in the USIM a list of protection schemes identifier, ordered according to priority. The list contains one or more protection schemes identifiers specified in Annex C. The ME shall read the ordered list of protection schemes identifiers that is provisioned in USIM by operator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.12.1, 6.12.2, 6.12.3, 6.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

### 6.12.1	Subscription permanent identifier

In the 5G system, the globally unique 5G subscription permanent identifier is called SUPI as defined in 3GPP TS 23.501 [2]. The SUCI is a privacy preserving identifier containing the concealed SUPI.

The SUPI is privacy protected over-the-air by using the SUCI which is described in clause 6.12.2. Handling of SUPI shall be done according to the requirements specified in clause 5 and details provided in clause 6.12.2.

Editor's Note: Privacy provisioning is FFS.

.

### 6.12.2	Subscription concealed identifier

The SUbscription Concealed Identifier, called SUCI, is a privacy preserving identifier containing the concealed SUPI.

The UE shall generate a SUCI using a protection scheme with the raw public key that was securely provisioned in control of the home network. The protection schemes shall be the ones specified in Annex C or the ones specified by the HPLMN.

The UE shall construct a scheme-input from the subscription identifier part of the SUPI, as specified by the protection scheme. The UE shall execute the protection scheme with the constructed scheme-input as input and take the output as the scheme-output.

The UE shall not conceal the home network identifier, e.g. Mobile Country Code (MCC) or Mobile Network Code (MNC).

The UE shall construct the SUCI with the following data fields:

-	The protection scheme identifier that represents a protection scheme specified in Annex C or a protection scheme specified by the HPLMN.

-	The home network public key identifier that represents a public key provisioned by the HPLMN. In case of null-scheme being used, this data field shall be set to null.

-	The home network identifier.

-	The scheme-output.

NOTE 1: 	The format of the SUPI protection scheme identifiers is defined in Annex C.

NOTE 2:	The identifier and the format of the scheme output are defined by the protection schemes in Annex C. In case non-null schemes, the freshness and randomness of the SUCI will be taken care of by the corresponding SUPI protection schemes

The UE shall include a SUCI only in the following 5G NAS messages:

-	if the UE is sending a Registration Request message of type "initial registration" to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include a SUCI to the Registration Request message, or

-	if the UE includes a 5G-GUTI when sending a Registration Request message of type "re-registration" to a PLMN and, in response, receives an Identity Request message, then the UE shall include a fresh SUCI in the Identity Response message (see clause 6.12.4).

NOTE: 	In response to the Identifier Request message, the UE never sends the SUPI.

-

The UE shall generate a SUCI using "null-scheme" only in the following cases:

-	if the UE is making an unauthenticated emergency session and it does not have a 5G-GUTI to the chosen PLMN, or

-	if the home network has configured "null-scheme" to be used, or

- 	if the home network has not provisioned the public key needed to generate a SUCI.

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give the ME any parameter for the calculation of the SUCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. The operator should use proprietary identifier for protection schemes if the operator chooses that the calculation of the SUCI shall be done in USIM.

If the operator’s decision is that ME shall calculate the SUCI, the home network operator shall provision in the USIM an ordered priority list of the protection scheme identifiers that the operator allows. The priority list of protection scheme identifiers in the USIM shall only contain protection scheme identifiers specified in Annex C, and the list may contain one or more protection schemes identifiers. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the home network public key, the home network public key identifier, and the list of protection scheme identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

The ME shall calculate the SUCI using the null-sheme if the home network public key or the priority list are not provisioned in the USIM.

NOTE:     The above feature is introduced since additional protection schemes could be specified in the future for a release newer than the ME release. In this case, the protection scheme selected by older MEs may not be the protection scheme with the highest priority in the list of the USIM.

### 6.12.3	Subscription temporary identifier

A new 5G-GUTI shall be sent to a UE only after a successful activation of NAS security. The 5G-GUTI is defined in TS 23.003 [19].

Upon receiving Registration Request message of type "initial registration" or "mobility registration update" from a UE, the AMF shall send a new 5G-GUTI to the UE in Registration Accept message.

Upon receiving Registration Request message of type "periodic registration update" from a UE, the AMF should send a new 5G-GUTI to the UE in Registration Accept message.

Upon receiving network triggered Service Request message from the UE (i.e., Service Request message sent by the UE in response to a Paging message), the AMF shall use a UE Configuration Update procedure to send a new 5G-GUTI to the UE. This UE Configuration Update procedure shall be used before the current NAS signalling connection is released, i.e., it need not be a part of the Service Request procedure because doing so delays the Service Request procedure.

NOTE 1:	It is left to implementation to re-assign 5G-GUTI more frequently than in cases mentioned above.

NOTE 2:	It is left to implementation to generate 5G-GUTI containing 5G-TMSI that uniquely identifies the UE within the AMF.

5G-TMSI generation should be following the best practices of unpredictable identifier generation.

### 6.12.4	Subscription identification procedure

The subscriber identification mechanism may be invoked by the serving network when the UE cannot be identified by means of a temporary identity (5G-GUTI). In particular, it should be used when the serving network cannot retrieve the SUPI based on the 5G-GUTI by which the subscriber identifies itself on the radio path.

The mechanism described in figure 6.12.4-1 allows the identification of a UE on the radio path by means of the SUCI.

Figure 6.12.4-1: Subscription identifier query

The mechanism is initiated by the AMF that requests the UE to send its SUCI.

The UE shall calculate a fresh SUCI from SUPI using the home network public key, and respond with Identifier Response carrying the SUCI.  The UE shall implement a mechanism to limit the frequency at which the UE responds with a fresh SUCI to an Identifier Request for a given 5G-GUTI.

NOTE 1:	If the UE is using any other scheme than the null-scheme, the SUCI does not reveal the SUPI.

AMF may initiate authentication with AUSF to receive SUPI as specified in clause 6.1.3.

In case the UE registers for Emergency Services and receives an Identifier Request, the UE shall use the null-scheme for generating the SUCI in the Identifier Response.

NOTE 2: 	Registration for Emergency does not provide subscription identifier confidentiality.

### 6.12.5	Subscription identifier de-concealing function (SIDF)

SIDF is responsible for de-concealing the SUPI from the SUCI. SIDF is using the private key part of the privacy related home network public/private key pair that is securely stored in the home operator's network. The de-concealment shall take place at the UDM. Access rights to the SIDF shall be defined, such that only a network element of the home network is allowed to request SIDF.

Editor's note: One UDM can comprise several UDM instances. How to find the right UDM instances is ffs keeping in mind that the SIDF needs to be invoked first.

**** END OF CHANGES **