3GPP TSG-SA WG3 Meeting #91Bis 	S3-181919

# La Jolla (US), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.501 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Protection scheme selection using legacy USIM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Protection scheme selection needs to be supported by legacy USIMs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds mechanism where the ME selects the protection scheme. This contribution contains the changes to clause 6.12.2. The companion contribution S3-181829 contains additional changes to clause 5.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SUPI privacy may not be supported when a legacy USIM is used, deployment of SUPI privacy is inhibited. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

** CHANGE#1 based on Clause 6.12.2 in S3-181496**

### 6.12.2	Subscription concealed identifier

The SUbscription Concealed Identifier, called SUCI, is a privacy preserving identifier containing the concealed SUPI.

The UE shall generate a SUCI using a protection scheme with the raw public key that was securely provisioned in control of the home network. The protection schemes shall be the ones specified in Annex C or the ones specified by the HPLMN.

The UE shall construct a scheme-input from the subscription identifier part of the SUPI, as specified by the protection scheme. The UE shall execute the protection scheme with the constructed scheme-input as input and take the output as the scheme-output.

The UE shall not conceal the home network identifier, e.g. Mobile Country Code (MCC) or Mobile Network Code (MNC).

The UE shall construct the SUCI with the following data fields:

-	The protection scheme identifier that represents a protection scheme specified in Annex C or a protection scheme specified by the HPLMN.

-	The home network public key identifier that represents a public key provisioned by the HPLMN. In case of null-scheme being used, this data field shall be set to null.

-	The home network identifier.

-	The scheme-output.

NOTE 1: 	The format of the SUPI protection scheme identifiers is defined in Annex C.

NOTE 2:	The identifier and the format of the scheme output are defined by the protection schemes in Annex C. In case non-null schemes, the freshness and randomness of the SUCI will be taken care of by the corresponding SUPI protection schemes

The UE shall include a SUCI only in the following 5G NAS messages:

-	if the UE is sending a Registration Request message of type "initial registration" to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include a SUCI to the Registration Request message, or

-	if the UE includes a 5G-GUTI when sending a Registration Request message of type "re-registration" to a PLMN and, in response, receives an Identity Request message, then the UE shall include a fresh SUCI in the Identity Response message (see clause 6.12.4).

NOTE: 	In response to the Identifier Request message, the UE never sends the SUPI.

-

The UE shall generate a SUCI using "null-scheme" only in the following cases:

-	if the UE is making an unauthenticated emergency session and it does not have a 5G-GUTI to the chosen PLMN, or

-	if the home network has configured "null-scheme" to be used, or

- 	if the home network has not provisioned the public key needed to generate a SUCI.

Based on home operator's decision, indicated by the USIM, the calculation of the SUCI shall be performed either by the USIM or by the ME. If the indication is not present, the calculation is in the ME.

Nokia comment: Using legacy USIM is possible using NULL-SCHEME. SUCI can  be calculated in the ME if the USIM doesn’t support it. But the issue is not just about SUCI calculation, whether it can be done in the USIM or ME . Upon receiving “Authentication- Request” from  AMF/SEAF ( i.e step 6 in the 5G AKA call flow), a legacy USIM can only generate LTE keys. The ME cannot generate these keys since it doesn’t have the root key, nor the root key can be transported out of the USIM. Hence for legacy USIM authentication to work a key mapping need to be done as shown below.

Secondly, the network elements AMF/SEAF and AUSF also need to be aware that the the UE used a legacy USIM and the UE doesn’t have the KAUSF and KSEAF. KASME can be mapped to KAMF. May be an Editor’s Note is needed to capture this and solve this in the next meeting. “ Editor’s Note: A legacy LTE USIM will not be able to generate keys such as KAUSF and KSEAF. Key generation and key mapping while using legacy USIM for authentication is FFS”.

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give the ME any parameter for the calculation of the SUCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. The operator should use proprietary identifier for protection schemes if the operator chooses that the calculation of the SUCI shall be done in USIM.

Nokia comment: The ME would have to differentiate between LTE Initial Attach Request and 5G Registration Request. Only for the 5G Registration Request the ME calculates the SUCI. For the LTE Initial Attach, the ME need to use the plain IMSI.

If the home network has not provisioned the public key in USIM, the SUPI protection in initial registration procedure is not provided. In this case, the null-scheme shall be used by the ME. Else, the home network operator shall also provision in the USIM a priority list of protection schemes that it allows, ordered according to its priority. In this case further, when SUCI calculation is performed by the ME, the following applies:

1>	If the USIM supports processing the priority list of protection schemes:

2>		The ME shall provide the protection schemes it supports to the USIM. Among the protection schemes supported by the ME, the USIM shall select the protection scheme which has the highest priority according to the ordered priority list provisioned by the home network operator. Further, the USIM shall indicate the selected protection scheme, i.e., the protection scheme identifier, to the ME.

1>	Else if the USIM supports only the storage and does not support processing the priority list of protection schemes:

2>	The ME shall read the priority list of protection schemes from the USIM. The ME shall select the protection scheme from its supported schemes that has the highest priority in the priority list obtained from the USIM.

1>	The ME shall further request the USIM to provide the SUCI calculation information, including the SUPI, the home network public key, and the home network public key identifier. If the selected protection scheme is the null-scheme, the USIM shall provide the SUPI. In case there is no indication from the USIM and the USIM provides only the SUPI to the ME, the ME shall calculate the SUCI using the null-scheme.

** END OF CHANGES **