3GPP TSG-SA WG3 Meeting #93 	S3-183480

# 12-16 November 2018, Spokane (US)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0457 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of N32-f key hierarchy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-09-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is based on approved Tdoc S3-183084 in last meeting. As there is no latest version of TS 33.501, S3-183084 is a psedou-CR but could not transform to CR automatically. So a related CR is proposed in this meeting. N32-f key hierarchy was specified in 13.2.4.4.1,where it was stated” each run of the N32-f key derivation creates two pairs of session keys and IV salts”. However, the defined N32_f key derivation function(N32-KDF (label, L)) contains the label for key separation, which means that each run of N32-KDF (label, L) can generate one session key or one IV salt. This results in an inconssitency between the statement and N32_f key derivation function. This pCR addresses this issue. Moreover, N32-Context-ID in the N32-KDF (label, L) is not defined anywhere. N32-Context-ID should be N32-f Context-ID to differentiate HTTP sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the sentence “Each run of the N32-f key derivation creates two pairs of session keys and IV salts.”. The sentence “Each run of N32-KDF (label, L) produces either one session key or one IV salt” is added.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** Begin of  changes ****

13.2.4.4.1	N32-f key hierarchy

The N32-f key hierarchy is based on the N32-f master key generated during the N32-c initial handshake by TLS key export. The N32-f key hierarchy consists of two pairs of session keys and two pairs of  IV salts, which are used in two different HTTP/2 sessions. In one Session the N32-c initiatior acts as the HTTP client and in the second the N32-c responder acts as the client.

If the exported master secret is reused to set up multiple HTTP sessions or to set up new HTTP sessions on stream ID exhaustion, a new, unique, N32-f Context-ID shall be generated to avoid key and IV re-use.

The master key is obtained from the TLS exporter. The export function takes 3 arguments: Label, Context, Length (in octets) of desired output. For the N32 Master key derivation, the label shall be "EXPORTER_3GPP_N32_MASTER", the Context shall be "" (the empty string) and the Length shall be 64.

Editor’s Note:  The exporter label for this usage should be registered with IANA

The N32 key derivation function N32-KDF is based on HKDF [62] and uses only the HKDF-Expand function as the initial key material has been generated securely:

N32-KDF (label, L) = HKDF-Expand (N32-f master key, "N32" || N32-f Context-ID || label, L),

where

-	label is a string used for key separation,

-	L is the length of output keying material in octets.

Each run of N32-KDF (label, L) produces either one session key or one IV salt.

There are two pairs of session keys and IV salts to be derived.

NOTE:	In AES-GCM re-use of one IV may reveal the integrity key (Joux’s Forbidden attack). The binding of session keys and IV salts to N32-f context-IDs and labels  is essential to protect against inadvertent use of the same key with a repeated IV.

The labels for the JWE keys are:

-	"parallel_request_key"

-	"parallel_response_key"

-	"reverse_request_key"

-	"reverse_response_key"

The keys derived with labels starting parallel are to be used for request/responses in an HTTP session with the N32-c initiating SEPP acting as the client (i.e. in parallel to the N32-c connection). The keys derived with the labels starting reverse are used for an HTTP session with the N32-c responding SEPP acting as the client.

To generate the IV salts, the length is 4 and the labels are:

-	"parallel_request_iv_salt"

-	"parallel_response_iv_salt"

-	"reverse_request_iv_salt"

-	"reverse_response_iv_salt"

The 96-bit nonce for AES_GCM shall be constructed as the concatenation of the IV salt (4 octets, 32-bits) and the sequence counter, SEQ, as defined in 8.2.1 of [63]. The sequence counter shall be a 64-bit unsigned integer that starts at zero and is incremented for each invocation of the encryption.  A different sequence counter shall be maintained for each IV salt.

Nonce = IV salt | SEQ

**** End of  changes ***