# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190172

# 28 January – 1 February 2019, Kochi (India)

# Source:	Huawei, Hisilicon

# Title:	New Conclusion: Small Data Transfer via NAS Signalling

# Document for:	Approval

Agenda Item:	8.6

1	Decision/action requested

This contribution addresses the Editor’s Notes in key issue#4 of TR 33.861.

2	References

[1]	3GPP TR 23.791, v.1.2.0, Study of Enablers for Network Automation for 5G

3	Rationale

It is concluded in TR 23.791 [1],

“

8.8	Key Issue 8: Performance improvement and supervision of mIoT terminals

8.8.1	General mode for mIoT terminals

It is concluded that Solution 8 and Solution 21 are used as the basis for normative work on how misused or hijacked UEs are recognized.

NOTE:      The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

”

Thus, SA2 has defined misbehaving UE. It is proposed to refer it to address the Editor’s Note in key issue #4.

4	Detailed proposal

**********Start of 1st CHANGE **********

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.724: "Study on Cellular IoT support and evolution for the 5G System".

[3]	3GPP TS 22.261:"Service requirements for next generation new services and markets".

[4]	3GPP TR 38.913: "Study on scenarios and requirements for next generation access technologies".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access (Release 16)".

[6]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications (Release 16)".

[7]				3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system (Release 15)".

[xx]	3GPP TR 23.791: “Study of Enablers for Network Automation for 5G (Release 16)”

********** END OF 1st CHANGES **********

**********Start of 2nd CHANGE **********

5.4	Key Issue #4: Signalling overload due to Malicious Applications on the UE

5.4.1	Introduction

A large number of UEs performing similar actions at the same time can easily lead to a signalling attack on the network. If such an attack persists and isn’t dealt with appropriately it brings a risk for other users of the network. As such, mitigating measures should be designed to protect the networks against such attacks.

For this key issue, it is assumed that the malicious behaviour on the UE is the result of an attacker having access to the application on the UE which it can instruct to make certain requests to the network. An attacker could have obtained this access through the over the top service and could for example instruct the UE to set up dedicated bearers or request access to certain network slices. Such procedures could lead to exhaustion of resources in both the radio and the core network, which would lead to denial of service or service degradation for other users of the network.

An important assumption of this key issue is that the UE part that is responsible for executing the radio instructions remains untouched. Also, it is assumed that the USIM is not compromised.

5.4.2	Security Threats

Denial of service due to signalling overload.

DDoS attacks towards the 5G core network include both large number of signal packets (e.g. NAS signalling messages) and user plane packets sent by compromised CIoT devices overloading the network. This kind of attack would lead to denial of service or at least throughput degradation caused by congestion to legitimate UEs whose traffic shares the same core network links. For example, access to the crucial core network functions (e.g. AMF) could be denied to normal UEs by one or more compromised UE(s)  repeatedly initiate(s) authentication procedures in a short period.

5.4.3	Potential security requirements

1) The 5GS shall be able to identify UEs that are operating normally and UEs that are misbehaving.

2) The 5GS shall support a secure mechanism to isolate the misbehaving UEs from the network.

3) The 5G network shall be able to protect itself against signalling overload from specific (e.g. malicious) UEs.

NOTE: Misbehaving UE is described in conclusion 8.8.1 of TR 23.791 [xx].

********** END OF 2nd CHANGES **********