# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190039

# 28 January – 1 February 2019, Kochi (India)

# Source:	GSMA

Title:	GTP Recovery Counter & GSN node behaviour

# Document for:	Information, Discussion

Agenda Item:	6.4

# Liaison Statement

| Liaison Statement Title: | GTP Recovery Counter & GSN node behaviour |
| --- | --- |
| Security Classification: | Confidential – Full, Rapporteur & Associate members |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| RIFS#73 | 28 November 2018 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| RIFS Doc 73_02 | 29th November 2018 | Jeremy Norris, Evolved Intellligence |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA Roaming and Interconnect Fraud and Security (RIFS) subgroup | - | GSMALiaisons@gsma.com |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | Cc: GSMA PACKET |
| External Recipients: | To: 3GPP CT4 , 3GPP SA3 Cc: 3GPP SA1, 3GPP CT3 |

# Potential risk of denial of service attack

During the development of of GTP security recommendations for GSMA members the GSMA Roaming and Interconnect Fraud and Security (RIFS) group has identified a possible attack that can result in a denial of service to subscribers. The attack is with respect to the GTP recovery counter, that can be manipulated in such a way that can result in a denial of service to subscribers. The attack type is described in the following table:

| Attack | Method | Attack Type | Attack Pre-conditions | Mitigation |
| --- | --- | --- | --- | --- |
| Denial of service on all subscribers on the same SGSN/SGW | An attacker can spoof any GTP-C message (e.g. Echo response) with increased Restart Counter and send it to the target node. The target node, since the counter has been increased, assumes a restart of the sending node and deletes all context assigned to this spoofed IP address. | Actively send malicious message | Target node IP reachability (e.g. access to the GRX/IPX, Internet exposure) | GSN nodes should not strictly follow this 3GPP functionality and thus not delete all contexts after restart counter indication to reduce the impact. Employ anti spoofing on the IP firewall, at least to protect spoofing of internal IPs and other explicitly not allowed IP addresses. Ensure the recovery counter is sequential and monitor counter values on GTP-C messages. |

RIFS plans to highlight this attack type and the associated risks to GSMA members. This attack is possible because within the 3GPP specification TS 23.007 section 10.1 Restart of the GGSN, when a restart condition is detected, then all UE context(s) are deleted, causing the associated denial of service.

RIFS also plans to highlight to its members that the recommended mitigation for the attack is to not delete all context(s). However, this contradicts the behaviour defined in TS 23.007.

# Action Required

GSMA RIFS would like to ask the 3GPP CT4 group to consider the attack and provide its view on the draft mitigation advice in the above table.

3GPP CT4 is invited to consider whether the specified behaviour in the 3GPP specification TS 23.007 (relevant extract copied below) should be changed to mitigate this vulnerability, in accordance with the RIFS draft recommendation.

When the SGSN detects a restart in a GGSN (see clause 18 "GTP-C based restart procedures") with which it has one or more PDP contexts activated, it shall deactivate all these PDP contexts and request the MS to reactivate them. When the SGSN detects a restart in a GGSN with which it has MBMS Bearer context(s) and/or MBMS UE context(s), it shall delete all these MBMS Bearer context(s) and/or MBMS UE context(s).

3GPP CT4 is invited to propose and advise RIFS of any other addtional security procedures for the protection of the Restart recovery counter within the GTP specification TS 23.007.

# Next Meetings

RIFS#74 call	9 Jan 2019

RIFS#75 call	13 Feb 2019

& calls at approximately 3-4 week intervals thereafter (dates TBC).