# 3GPP TSG SA WG3 (Security) Meeting #95bis 	S3-191869

# 24 – 28 Jun, 2019, Sapporo (US)

# Source:	Huawei, HiSilicon

# Title:	Enabling UE to detect FBS

# Document for:	Approval

Agenda Item:	8.9

# 1	Decision/action requested

This pCR proposes a solution to enable the UE not to connect to false base station during handover based on verification of SIBs hashes.

# 2	References

[1]	3GPP TR 33.809 V0.1.0, Study on 5G security enhancements against false base stations

[2]	3GPP TS 38.331 V15.4.0, Radio Resource Control (RRC)

# 3	Rationale

In solution #4, it’s described that the serving cell may use the hash verification result to decide whether or not to make an attempt to handover the UE to the reported neighbouring cell.

However, in the UE connected mode, the UE usually read the MIB of the neighbouring cell during the measurement procedure. Only when the the reportType is set to reportCGI, the UE need to read SIB1 of the dedicated cell, according to TS 38.331. Namely, the MR reported by the UE in connected mode can includes the hashes of MIB/SIB1 (just when the the reportType is set to reportCGI, it’s rare to happen).

If the false base station just copy the MIB and SIB1 of the real cell, but tamper the cell reselection information in SIB2, SIB3, SIB4 and SIB5, the UE would still handover to the false base station.

This proposal proposes that the target cell should provide the HASHes of all SIBs during the handover preparation phase, then UE can verify whether the SIBs (other SIBs) received during handover execution phase are true or false. If finding any received SIB is false, the UE should not connect to that cell and should leave the cell and try to re-establish with the old serving cell or other cells. The UE reports the handover result to the genuine base station.

# 4	Detailed proposal

**** Start of pCR to TR 33.809****

## 6.Y Solution #Y: Preventing UE to Connect to False Base Station Based on HASH Verification

### 6.Y.1	Introduction

This solution addresses the second security requirement in the following key issue:

-	Key issue #3: network detection of false base stations (5G system should be able to employ methods to prevent UEs from connecting to false base stations.).

According to TS 38.331 [2], the hash of MIB/SIBs can be reported by the UE in connected mode just includes the hash of MIB and small number of the SIB1. In solution 4, just the MIB/SIB1 hash verification result can be used to prevent the UE handover to the false base station.

This solution prevent the UE connect to false base station which copy the MIB and/or SIB1 of the real cell, but tamper the cell reselection information in SIB2, SIB3, SIB4,SIB5 and/or SIB1.

### 6.Y.2	Solution details

In RRC-CONNECTED, during the UE handover to a specific target cell, that the target cell should provide the HASHes of all SIBs during the preparation phase. When the UE executes the handover to the target cell, the UE monitor the SIBs of the target cell, and compute the HASHes of the received SIBs, compare that with those received from the target cell during the handover preparation. If finding any SIB received during handover execution fails, the UE should not connect to that cell and should leave the cell and try to re-establish with the old serving cell or other cells and report the handover result to the genuine base station.

### 6.Y.3	Evaluation

TBA.

**** END OF CHANGES ****