3GPP TSG-SA WG3 Meeting #95-Bis 	S3-192214

# Sapporo (Japan), 24-28 June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0022 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security aspects of CAPIF-7/7e reference points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF-Sec |  |  |  |  |  |  | Date: |  | 2019-06-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.122 specifies new reference points between AEF functions (topology hiding entity and AEF handling service APIs) and these are CAPIF-7 and CAPIF-7e. CAPIF-7 is between AEFs belonging to same trust domain and CAPIF-7e is between AEFs belonging to different trust domains. Security aspects for these reference points need to be specified. TS 23.122 also specifies that CAPIF-7 and CAPIF-7e are CAPIF-2 and CAPIF-2e compliant respectively. Taking the above into consideration, security requirements for CAPIF-7/7e are proposed. Security procedures for CAPIF-2 are applicable for CAPIF-7 reference point. To secure the communciation over CAPIF-7e reference point (AEFs between different trust domains), it is proposed to use N32 interface security that is used to secure communication between PLMNs. This avoids mutiple secure connections between AEF domains by leveraging the N32 security procedures specified in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses covering security requirements and procedures for CAPIF-7/7e reference points. Update References clause as to reflect the changes introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects of CAPIF-7/7e remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.X (New clause), 6.Y (New clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### ***** Start of Change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[5]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[6]	IETF RFC 7519: "JSON Web Token (JWT)".

[7]	IETF RFC 7515: "JSON Web Signature (JWS)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[X]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

#### ***** Next Change *****

## 4.X	Security requirements on the CAPIF-7/7e reference points

The security requirements for CAPIF-7/7e reference points are:

-	[CAPIF-SEC-4.X-a] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be integrity protected.

-	[CAPIF-SEC-4.X-b] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be protected from replay attacks.

-	[CAPIF-SEC-4.X-c] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be confidentiality protected.

-	[CAPIF-SEC-4.X-d] Privacy of the 3GPP user over the CAPIF-7 and CAPIF-7e reference points shall be protected.

-	[CAPIF-SEC-4.X-e] The API exposing function (destination AEF handling service API) shall determine whether AEF that is topology hiding entity, is authorized to access service API.

#### ***** Next Change *****

## 6.Y	Security procedures for CAPIF-7/7e reference points

To ensure security of the interfaces between API Exposing functions (Topology hiding entities and destination AEF handling service APIs), namely CAPIF-7 and CAPIF-7e:

- Security procedures specified for CAPIF-2 reference point shall be used for secure communication, authentication and authorization, between the AEFs belonging to same trust domain over CAPIF-7 reference point.

- N32 interface security between the SEPPs, as specified in TS 33.501 [X], shall be used for secure communication between the AEFs belonging to different trust domains over CAPIF-7e reference point.

Authorization, if required between AEFs belonging to different trust domains, shall be based on OAuth 2.0 [4], as specified for authorization between NFs belonging to different PLMNs in TS 33.501 [X].

#### ***** End of Change *****