# 3GPP TSG SA WG3 (Security) Meeting #96-Adhoc	S3-193754

# Chongqing (China), 14-18 October 2019	revision of S3-192360

# Source:	Qualcomm Incorporated

# Title: 	Evaluation of the shared key based MIB/SIB protection solution

# Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

This contribution proposes an evaluation text for solution #14: Shared key based MIB/SIB protection

# 2	References

[1]	 TR 33.809

# 3	Rationale

This paper proposes an evaluation text for the solution #14.

Even though this contribution was not agreed in the SA3 #95-bis meeting, we still believe the evaluation is technically correct.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR 33.809 [1].

***** START OF FIRST CHANGES *****

### 6.14.3 	Evaluation

This solution fulfils the potential security requirement of the KI #2 when the UE is in the RRC-Connected state.

This solution does not filful the potential security requirement of the KI #2 when the UE is in the RRC-Idle or the RRC-Inactive state.

Editor’s Note: How to protect the MIB/SIBs when AS security is not supported is FFS.

***** END OF FIRST CHANGES *****