Third Generation Partnership Project (3GPP™)

DRAFT Meeting Report
for
TSG SA WG3
meeting: 96AH

Chongqing, China, 14/10/2019 to 18/10/2019

Contents:

1	Opening of the meeting	3

2	Approval of Agenda and Meeting Objectives	3

3	IPR and Anti-Trust Law Reminder	3

4	Incoming LS	3

5	Studies and Work Areas (Rel-16)	5

5.1	Security Aspects of the 5G Service Based Architecture	5

5.1.1	Work Item (SBA_Sec)	5

5.1.2	Study (FS_SBA-Sec)	9

5.2	Authentication and key management for applications based on 3GPP credential in 5G	14

5.2.1	Work Item (AKMA)	14

5.2.2	Study (FS_AKMA)	16

5.3	Evolution of Cellular IoT security for the 5G System	23

5.3.1	Work Item (CIoT_sec_5G)	23

5.3.2	Study (FS_CIoT_sec_5G)	26

5.4	Security of the Wireless and Wireline Convergence for the 5G system architecture (5WWC_SEC)	33

5.5	5G security enhancement against false base stations (FS_5GFBS)	38

5.6	Security aspects of Enhancement of Network Slicing	55

5.6.1	Work Item (eNS_SEC)	55

5.6.2	Study (FS_eNS_SEC)	59

5.7	Security of the enhancement to the 5GC location services (eLCS_Sec)	68

5.8	Security for 5G URLLC (5G_URLLC_SEC)	69

5.9	Study on SECAM and SCAS for 3GPP virtualized network products (FS_VNP_SECAM_SCAS)	71

5.10	Security for 5GS Enhanced support of Vertical and LAN Services	77

5.10.1	Work Item (Vertical_LAN_SEC)	77

5.10.2	Study (FS_Vertical_LAN_SEC)	80

5.11	Study on LTKUP Detailed solutions (FS_LTKUP_Detail)	85

5.12	Study on User Plane Integrity Protection (FS_UP_IP_Sec)	85

5.13	Study on Security Impacts of Virtualisation (FS_SIV)	90

5.14	Study on authentication enhancements in 5GS (FS_AUTH_ENH)	96

5.15	Security for NR Integrated Access and Backhaul	102

5.15.1	Work Item (NR_IAB_Sec)	102

5.15.2	Study (FS_NR_IAB_Sec)	103

5.16	Study on Security Aspects of 3GPP support for Advanced V2X Services (FS_eV2X_Sec)	109

5.17	Study on storage and transport of the security parameters in a 5GC, that are used by the ARPF for Authentication (FS_5GC_sec_storage_transport)	121

6	Any Other Business	123

7	Close	126

Annex A: List of contribution documents	127

Annex B: Lists of liaisons	142

B1: Incoming liaison statements	142

B2: Outgoing liaison statements	142

Annex C: List of draft Technical Specifications and Reports	143

Annex D: List of participants	144

Annex E: List of future meetings	147

## 1	Opening of the meeting

## 2	Approval of Agenda and Meeting Objectives

S3-193300	Agenda

Type: agenda		For: (not specified)
					Source: WG Chair

Decision: 		The document was revised to S3-193681.

S3-193681	Agenda

Type: agenda		For: -
					Source: WG Chair

(Replaces S3-193300)

Decision: 		The document was approved.

## 3	IPR and Anti-Trust Law Reminder

## 4	Incoming LS

S3-193432	Discussion on Security of Multi-CU-UP connectivity

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was withdrawn.

S3-193433	LS reply to RAN WG3 LS on security for multi-CU-UP connectivity

Type: LS out		For: Approval
					to RAN3
					Source: CATT

Decision: 		The document was withdrawn.

S3-193661	LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: C4-193790

Discussion:

Nokia: no special security required

DCM: agree

Orange: if OTA keys go into new platform or stay in OTA platform. Put this in reply LS

Thales: keys should stay in OTA platform, only authorized NFs should be able to use this interface.

Nokia: noone wants to move the keys.

Orange: for operators without OTA gateways, where are the keys.

Decision: 		The document was replied to in S3-193682.

S3-193682	Reply to: LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update

Type: LS out		For: approval
					to CT4, cc CT1, CT6, SA2
					Source: Nokia

Decision: 		The document was approved.

S3-193670	N9HR Regulatory Obligations

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: S3i190548

Decision: 		The document was postponed.

S3-193680	LS on security consideration of performance measurement function protocol

Type: LS in		For: discussion
					Original outgoing LS: -, to -, cc -
					Source: C1-196940

Decision: 		The document was postponed.

S3-193683	LS on security consideration of performance measurement function protocol

Type: LS out		For: Approval
					to -
					Source: ZTE Corporation

Discussion:

Apple: UP IP is required, should be included in LS

CMCC: consensus is need integrity protection, does not mention other things, no other  security mechanism is needed

DCM: no new mechanism is required but including integrity protection is required.

Decision: 		The document was revised to S3-193816.

S3-193816	LS on security consideration of performance measurement function protocol

Type: LS out		For: Approval
					to -
					Source: ZTE Corporation

(Replaces S3-193683)

Discussion:

No agreement on whether  to include that exisiting mecchnaisms are sufficient

Decision: 		The document was noted.

S3-193628	Discussion Paper for Security of Performance Measurement Function Protocol

Type: discussion		For: Agreement
					Source: Apple Computer Trading Co. Ltd

Discussion:

Nokia: all UP is integrity protected between UE and BS, so do we need more protection

Apple: UP IP protection is good to have, but discussion is about whether, not how

Nokia: protocol needs protection, but no further protectino is required. It is already protected. But no reply is required in this meeting

Noamen: this was flagged as urgent

QC: no additional security required. disagree with Apple discussion paper

Apple: UP IP is optional. CT1 is only asking about problems, not about solution

QC: LS should say we don't need additional security

Huawei: no additional security is required

DCM: respond: UP IP will be sufficitent

E//: agree with QC that IP will not address all problems.

Apple: PMF protocol needs to be protected

Decision: 		The document was noted.

## 5	Studies and Work Areas (Rel-16)

### 5.1	Security Aspects of the 5G Service Based Architecture

#### 5.1.1	Work Item (SBA_Sec)

S3-193386	Update to 5G_eSBA WID

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Update to eSBA WID scope - remove inter-PLMN N9 security

Discussion:

There is a related WID in S3-193672 where there is some discussion on splitting of the work items

Needs to be re-submitted to Reno meeting.

Decision: 		The document was approved.

S3-193387	Security requirements for SeCoP

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security requirements for SeCoP

Discussion:

Huawei: Typo in title of 5.9.2.4 compared to abbreviations

It was agreed to use the same name as SA2 but different abbreviations

Huawei: What internal interface and why is it mentioned

DCM: For service mesh type deployment there will be internal interfaces - could EN be made into normative text.

Eri: Prefer to remove EN or re-word it.

DCM: Leave EN in now and deal with it at the next meeting

Eri: have other comments on the NOTE.

Decision: 		The document was revised to S3-193719.

S3-193719	Security requirements for SeCoP

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193387)

Decision: 		The document was approved.

S3-193428	Security requirements for UP Gateway Function

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security requirements for UP Gateway Function

Discussion:

There were some offline revisions suggested - first line of NOTE moving to the top of the clause

Eri: proxy requirement come from SA2 - does SA3 need to say

DCM: UPGF acts a a transparent function

Huawei: Remove last sentence of NOTE as not security related

Decision: 		The document was revised to S3-193720.

S3-193720	Security requirements for UP Gateway Function

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193428)

Decision: 		The document was approved.

S3-193388	Authentication and authorization between SeCoP and network functions

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication and authorization between SeCoP and network functions

Discussion:

Eri: last paragraph could be shortened

DCM: Concerned that it sounds like an option in that case

Huawei: There is no authorisation aspects in clause yet an EN should capture this

Decision: 		The document was revised to S3-193721.

S3-193721	Authentication and authorization between SeCoP and network functions

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193388)

Decision: 		The document was approved.

S3-193389	Authentication and authorization between SeCoPs

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication and authorization between SeCoPs

Discussion:

Eri: formating of bullets

Huawei: Authorisation EN is needed

Decision: 		The document was revised to S3-193722.

S3-193722	Authentication and authorization between SeCoPs

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193389)

Decision: 		The document was approved.

S3-193423	Protection of N9 interface in Inter-PLMN scenario

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell, Juniper

Abstract:

Protection of N9 interface

Discussion:

Nokia presents

Eri: Concerned that this includes N32 when it does not mean to

Agreed does not include N32 and taken offline for formulation.

Decision: 		The document was revised to S3-193723.

S3-193723	Protection of N9 interface in Inter-PLMN scenario

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell, Juniper

(Replaces S3-193423)

Decision: 		The document was approved.

S3-193622	Resource Level Authorization using Access Tokens

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

Huawei: Concened that there may be SA2 impacts and would like to postpone document.

Eri: How is the UDR impacted.

Huawei: EN on SA2 impact and UDR study may affect this.

Nok: Solution is overloading scope

DCM: Not clear on what is being mandated.

Decision: 		The document was noted.

S3-193624	Authorization using Access Tokens based on NF-Subtype

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

Nok: No key issue so should not have a solution until this is agreed

Huawei: Concerns raised for S3-193622 apply here.

Decision: 		The document was noted.

S3-193377	Security for Wireline access to 5G - General

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Add General clause for "Security for Wireline access to 5G"

Decision: 		The document was withdrawn.

S3-193378	Authentication for 5G-RG

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication for 5G-RG

Decision: 		The document was withdrawn.

#### 5.1.2	Study (FS_SBA-Sec)

S3-193614	LS on token-based authorization for indirect communication with delegated discovery (Scenario D)

Type: LS out		For: Approval
					to SA2
					Source: Ericsson

Discussion:

Some comments given in offline e-mail discussion

Nok: Update the question to SA2 and additional questions needed.

Decision: 		The document was revised to S3-193724.

S3-193724	LS on token-based authorization for indirect communication with delegated discovery (Scenario D)

Type: LS out		For: Approval
					to SA2
					Source: Ericsson

(Replaces S3-193614)

Decision: 		The document was approved.

S3-193678	Update to conclusion on Key issue #22: Authorization of NF service access in indirect communication

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Update to conclusion on Key issue #22: Authorization of NF service access in indirect communication

Discussion:

Wait for SA2 response LS to S3-193724 before progress can be made.

Decision: 		The document was noted.

S3-193616	Update to conclusion on Key issue #22: Authorization of NF service access in indirect communication

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

Wait for SA2 response LS to S3-193724 before progress can be made.

Decision: 		The document was noted.

S3-193617	Update to conclusion on Key issue #23: NF to NF authentication and authorization in Indirect communication

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

Kept open for offline discussions

DCM: authorization may not be so static.

Decision: 		The document was noted.

S3-193619	New solution for Key Issue #25 "Indirect communication in roaming scenarios"

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

Nokia: Sentence in intro about 'authentication between NFs in different networks is implicit by ….'

Huawei: Telescopic FQDN use requires an EN.

DCM: Got confused that SECOP is split across PLMNs - make it clear that SECOP does not span multiple PLMNs by adding a sentence.

Huwaei: Clarification needed in roaming scenarion that interface between SECOP and SEPP is a PLMN.

Decision: 		The document was revised to S3-193725.

S3-193725	New solution for Key Issue #25 "Indirect communication in roaming scenarios"

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

(Replaces S3-193619)

Decision: 		The document was approved.

S3-193620	Conclusion for Key Issue #25 "Indirect communication in roaming scenarios"

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

Huawei: Postpone due to EN on telescopic FQDN

Nok: OK to make progress even with that EN

DCM: Agree with Nokia and propose EN is added to conclusion - this was agreeable to meeting.

Decision: 		The document was revised to S3-193726.

S3-193726	Conclusion for Key Issue #25 "Indirect communication in roaming scenarios"

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

(Replaces S3-193620)

Decision: 		The document was approved.

S3-193445	eSBA: add conclusion on KI #24

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

Discussion:

Ericsson: Feel that response from SA2 is needed

Huawei: Feel that it can go forward with EN

Nok: Can go forward with EN

There is a competing conclusion in S3-193618.

Decision: 		The document was noted.

S3-193727	eSBA: add conclusion on KI #24

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

Decision: 		The document was withdrawn.

S3-193618	Conclusion of Key Issue #24 Service access authorization within a NF Set or a NF Service Set

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

DCM: support this as a conclusion.

Decision: 		The document was noted.

S3-193313	Resolving EN in TR33.855 6.18 N9 NDS/IP

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Juniper Networks, NTT DoCoMo, Ericsson

Discussion:

Huawei: Mentions that network slice can have different security requirements - this is not clear

Orange: Different security can be applied to different slices

DCM: A better formulation is needed

Juniper: Trying to resolve EN and it needs to be decided if some normative work needed.

Decision: 		The document was revised to S3-193728.

S3-193728	Resolving EN in TR33.855 6.18 N9 NDS/IP

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Juniper Networks, NTT DoCoMo, Ericsson

(Replaces S3-193313)

Decision: 		The document was approved.

S3-193613	Clarification on delegated subscribe-notify

Type: discussion		For: Endorsement
					33.855 v..
					Source: Ericsson

Decision: 		The document was noted.

S3-193447	Update on solution#15 in TR 33.855

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

Discussion:

Eri: evaluation does not contain disadvantages

Hua: Last paragraph contains impact

Taken for offline discussion of evaluation clause.

Decision: 		The document was revised to S3-193730.

S3-193730	Update on solution#15 in TR 33.855

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

(Replaces S3-193447)

Decision: 		The document was approved.

S3-193446	eSBA: add conclusion on KI #28

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

Discussion:

This is a competing concusion in S3-193621

Hua: There is missing verification of the call-back and hence the solution.

Eri: No way that this can be verified.

Decision: 		The document was noted.

S3-193621	Conclusion of Key Issue #28: Service access authorization in the delegated "Subscribe-Notify" scenarios

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193615	Implementation of agreed change from SA3#96

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

it is missing implementation from the last meeting.

Decision: 		The document was approved.

S3-193612	Clarification on identities in TLS and token-based authorization

Type: discussion		For: Endorsement
					33.855 v..
					Source: Ericsson

Discussion:

Huawei: Does not agree with proposal - need to know how to allocate instnace Id and certificate first

DCM: How would this work for scenario D? Does SECOP have NF instance IDs that it would use as transport IDs

Eri: Do you mean that SECOP case is not covered?

Decision: 		The document was noted.

S3-193626	Evaluation update for Solution #25

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Discussion:

Huawei: Do not agree with the proposed evaluation

Decision: 		The document was noted.

S3-193444	eSBA: add conclusion on KI #5

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Huawei, Hisilicon

Discussion:

There is a competing conclusion in S3-193444.

Decision: 		The document was noted.

S3-193625	Conclusion for Key Issue #5 "NF-NF Authorization"

Type: pCR		For: Approval
					33.855 v1.7.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193729	Draft TR 33.855

Type: draft TR		For: Approval
					33.855 v1.8.0
					Source: Ericsson

Decision: 		The document was approved.

### 5.2	Authentication and key management for applications based on 3GPP credential in 5G

#### 5.2.1	Work Item (AKMA)

S3-193582	skeleton of AKMA TS

Type: draft TS		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

Discussion:

Huawei: Prefer CMCC version

QC: OK to merge but think that there are good sections in Ericsson which should be kept.

Decision: 		The document was revised to S3-193769.

S3-193769	skeleton of AKMA TS

Type: draft TS		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell,Ericsson

(Replaces S3-193582)

Decision: 		The document was approved.

S3-193596	Skeleton of clause 4 and selected content for AKMA normative work

Type: pCR		For: Approval
					33.535 v0.0.0
					Source: Ericsson

Decision: 		The document was merged.

S3-193583	scope of AKMA TS

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

Discussion:

QC: Remove 3GPP and change to subscription credentials.

Orange: Reference the subscription credentials in TS 33.501 - align terminology with study.

QC: Remove the bullet list paragraph

Orange: Remove the 2nd sentence of first paragraph.

Decision: 		The document was revised to S3-193770.

S3-193770	scope of AKMA TS

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

(Replaces S3-193583)

Decision: 		The document was approved.

S3-193468	Propose AKMA architecture

Type: pCR		For: Approval
					33.535 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

There are competing proposals in S3-193584 and S3-193599

BT: Like S3-19599 as it looks more service based

Orange: We are defining a service that provides keys for use in an application

KPN: Why have the UDM in the picture?

Huawei: Need to make the high levl descision, e.g. local of anchor function, so issue with Ericsson's text on where a key is derived - prefer S3-193584 as the baseline for merger

Orange: No point having non-SBA interfaces as the core network is now servive based

Huwaei: Need UDM in picture

Orange: Do not see the point of not using a service based representation as that is now the normal

CMCC: Both representations are equivalent

Orange: Will object to non-service based version

Agreed to use UE and AF boxes - also AAnF will be the name of the AKMA anchor function

Orange: It is more complicated than the pictures shows as an AF outside 3GPP network may need to use an NEF to access AAnF?

Merged into 771.

Decision: 		The document was merged.

S3-193584	draft CR of AKMA architecture

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to S3-193771.

S3-193771	draft CR of AKMA architecture

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

(Replaces S3-193584)

Discussion:

Orange: disagree with ed note, object to collocation of AAnF with NEF

CMCC: was there before

QC: remove in parantheses?

Orange: no.

Decision: 		The document was revised to S3-193841.

S3-193841	draft CR of AKMA architecture

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: China Mobile, Nokia, Nokia Shanghai Bell

(Replaces S3-193771)

Decision: 		The document was approved.

S3-193599	AKMA architecture reference model

Type: pCR		For: Approval
					33.535 v0.0.0
					Source: Ericsson

Decision: 		The document was merged.

S3-193469	Propose AKMA key hierarchy

Type: pCR		For: Approval
					33.535 v0.0.1
					Source: Huawei, Hisilicon

Discussion:

Orange: Name of functions changes to AAnF and other fucntion to AF - key name should also change.

Decision: 		The document was merged.

S3-193600	AKMA Key hierarchy

Type: pCR		For: Approval
					33.535 v0.0.0
					Source: Ericsson

Discussion:

There is a competing proposal in S3-193469

Orange: Correct names based on agreements- text in S3-193469 is not correct as it talks about AKMA authentication

QC: Just show the derivation of K_AUSF and reference TS 33.401 for the rest of the derivations.

Decision: 		The document was revised to S3-193772.

S3-193772	AKMA Key hierarchy

Type: pCR		For: Approval
					33.535 v0.0.0
					Source: Ericsson

(Replaces S3-193600)

Decision: 		The document was approved.

S3-193817	Draft TS 33.535

Type: draft TS		For: Approval
					33.535 v0.1.0
					Source: China Mobile

Decision: 		The document was approved.

#### 5.2.2	Study (FS_AKMA)

S3-193441	AKMA: Resolving the EN in AKMA push

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Eri: need to go the UDM and not clear on what is meant by security context

Huawei: Will chck on UDR or UDM and change security context to K_AUSF.

Decision: 		The document was revised to S3-193761.

S3-193761	AKMA: Resolving the EN in AKMA push

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

(Replaces S3-193441)

Decision: 		The document was approved.

S3-193442	AKMA: adding the evaluation of solution #24

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Eri: Evaluation not in template

QC: When you have a K_AUSF then UE can connnect so need evaluation on why need to use PUSH for this case - also why do we need AKMA PUSH when GBA PUSH already exists

Huawei: AKMA is a new feature and needs its own PUSH

CMCC: Do we need an editor's note for the second point?

Huawei: Confirmed that the scope of GBA PUSH enhancements is network interfaces

QC: Important that understand if we need more than PUSH solution

QC: Don't want to have two solutions for PUSH?.

Decision: 		The document was revised to S3-193763.

S3-193763	AKMA: adding the evaluation of solution #24

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

(Replaces S3-193442)

Decision: 		The document was approved.

S3-193443	AKMA: add conclusion on KI #17

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193473	Evaluation for solution4

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

This was missed for earlier implementation of the TR.

Decision: 		The document was approved.

S3-193475	Implicite AKMA authenticaiton procedure

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Ericsson: this solution changes primary authentication and not sure if this is acceptable

QC: Do not understand why we are having an new solution.

Huawei: Can it derive the K_AKMA?

Eri: Changes the Registrations Accept

QC: do not understand why a different solution is needed when we have agreed solution.

Decision: 		The document was noted.

S3-193597	Solution #15 evaluation removal of EN

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Ericsson

Decision: 		The document was approved.

S3-193474	Evaluation for solution22

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Eri: Does not fit the template for evaluations

QC: Think first sentence should go away.

Decision: 		The document was revised to S3-193764.

S3-193764	Evaluation for solution22

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

(Replaces S3-193474)

Discussion:

Orange: don't agree with last two sentences

QC: conclusions made, doesn't matter

agreed as it is.

Decision: 		The document was approved.

S3-193595	Solution Key lifetimes

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Ericsson

Discussion:

QC: OK with proposal and analysis. Normative text on requirements in evaluation should go away.

Decision: 		The document was revised to S3-193765.

S3-193765	Solution Key lifetimes

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Ericsson

(Replaces S3-193595)

Decision: 		The document was approved.

S3-193591	Evaluation of solution#1

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Discussion:

Ericsson: Solution addresses a problem that is not actually for AKMA.

Decision: 		The document was approved.

S3-193587	Conclusion on key issue #6

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile, KPN

Discussion:

Eri: Do not agree with conclusion as this could be left applications

QC: Agree with ericsson comment

BT: Conclude on this issue now but either need it done properly or not at all

CMMC: This is a customer requirement

QC: This is in the scope of the application

ZTE: Think that this is an important requirement.

Decision: 		The document was noted.

S3-193340	Update of key issue #6

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: KPN

Abstract:

This contribution adds a security requirement to the key issue #6, Secure communication between UE and application server

Discussion:

Eri: Similar to the previous document - application can derive the keys

QC: Add a EN to capture the issue on whether the requirement can be left to application.

Decision: 		The document was revised to S3-193766.

S3-193766	Update of key issue #6

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: KPN

(Replaces S3-193340)

Decision: 		The document was approved.

S3-193598	Conclusions on Key management

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Ericsson

Discussion:

BT: This is important that application key outlives the root anchor. Supports the evaluation.

Decision: 		The document was approved.

S3-193590	Individual Evaluation of solution #6

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Decision: 		The document was approved.

S3-193470	Conclusion on 7.3

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Eri: maybe not possibel to conclude on this

CMCC: No been decided whether anchor function is standalone

KPN: Come a bit of out of the blue and not related to a solution - not ready to conclude.

Decision: 		The document was noted.

S3-193585	conclusion on key issue #2

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Discussion:

Eri: Key issue does not security theats or requirements - which messages are used

CMCC: To use NAS procedures between UE and 5GC

QC: Also struggle to see what the conclusion means.

Decision: 		The document was revised to S3-193767.

S3-193767	conclusion on key issue #2

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

(Replaces S3-193585)

Discussion:

Orange: AKMA application functino will be named to application function

E//: not consistent in the TR?

Orange: Note: AKMA application function is now application function in normative work.

Decision: 		The document was revised to S3-193842.

S3-193842	conclusion on key issue #2

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

(Replaces S3-193767)

Decision: 		The document was approved.

S3-193471	Conclusion on 7.4

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

There is an alternative conclusion in S3-193586

QC: not ready to make this conclusion as there are alternatives - hence need more analysis on how this should be done.

Decision: 		The document was merged.

S3-193586	Conclusion on key issue #5

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Discussion:

Huawei: Two conclusions are not competing

QC: Can make a conclusion on deriving identifier for K_AUSF

Huawei: In the TR there is a temporary identifier.

Decision: 		The document was revised to S3-193768.

S3-193768	Conclusion on key issue #5

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

(Replaces S3-193586)

Decision: 		The document was approved.

S3-193592	Evaluations of solution #7- #12

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Decision: 		The document was approved.

S3-193588	Conclusion on key issue #13

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Discussion:

QC: OK but do you expect changes to UE/UICC or keep it open

CMCC: keep it open.

Decision: 		The document was approved.

S3-193472	Propose new conlusion section

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: Huawei, Hisilicon

Discussion:

Eri: there is already a place for this conclusion

CMCC: Agree with ericsson.

Decision: 		The document was noted.

S3-193513	User identity privacy for GBA in 5GC

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: ZTE Corporation

Discussion:

CMCC: Don't have the GBA content in the TR

Eri: Maybe in scope of the other SID

Orange: Agree that could be treated in 5G GBA SID.

Decision: 		The document was noted.

S3-193589	Editorial Changes to TR 33.835

Type: pCR		For: Approval
					33.835 v1.0.1
					Source: China Mobile

Discussion:

QC: Editorial - think that the Voids can be removed as not under change control.

Decision: 		The document was approved.

S3-193762	draft TR 33.835

Type: draft TR		For: Approval
					33.835 v1.1.0
					Source: China Mobile

Decision: 		The document was approved.

### 5.3	Evolution of Cellular IoT security for the 5G System

#### 5.3.1	Work Item (CIoT_sec_5G)

S3-193660	LS on short MAC-I and ngKSI for 5G-CIoT

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: C1-195199

Decision: 		The document was replied to in S3-193715.

S3-193715	Reply to: LS on short MAC-I and ngKSI for 5G-CIoT

Type: LS out		For: approval
					to CT1
					Source: Ericsson

Decision: 		The document was approved.

S3-193658	Nokia comments on CT1 LS C1-195199

Type: discussion		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

S3-193668	Reply LS on LTE-M identification in 5GC

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: R3-194748

Decision: 		The document was noted.

S3-193435	Discussion on security context handling in fast RRC release

Type: discussion		For: (not specified)
					Source: LG Electronics

Discussion:

Intel: SA2 have not agreed to fast release, so should we discuss this here

QC: Not clear what the securiyt issue is

LG: Possible NCC mismatch

Nok: Agree with Intel that fast release is not yet standardiszed

Eri: Not clear that observation 1 is correct so maybe no security issue.

Decision: 		The document was noted.

S3-193569	CIOT: Discussion paper on ngKSI for NB-IoT in 5G-CIoT.

Type: discussion		For: Endorsement
					Source: Ericsson

Decision: 		The document was noted.

S3-193570	CIOT: Discussion paper on short MAC-I for NB-IoT in 5G-CIoT.

Type: discussion		For: Endorsement
					Source: Ericsson

Decision: 		The document was noted.

S3-193571	Draft LS reply on LS on short MAC-I and ngKSI for 5G-CIoT

Type: LS out		For: Approval
					to CT1
					Source: Ericsson

Discussion:

QC: first answer: make it shorter, not a security issue

DCM: if inteintegrity protection fails, then there is possibility of mismatch and of attack

QC: there can't be a mismatch on CPSR message

offline

QC: second question: shorter, simply say no. agree to not reduce reduced size MAC.

Decision: 		The document was revised to S3-193715.

S3-193562	DraftCR - Proposed skeleton for supporting 5G CIoT [Living Document]

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

Abstract:

Resubmission of S3-193052 agreed last meeting. Baseline for the CIoT normative work.

Decision: 		The document was revised to S3-193716.

S3-193716	DraftCR - Proposed skeleton for supporting 5G CIoT [Living Document]

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

(Replaces S3-193562)

Decision: 		The document was approved.

S3-193392	CIoT_ Modifications to draft CR

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell, Ericsson

Abstract:

Changes to resolve 'Data over NAS' terminology

Discussion:

Huawei: will provide definition next time

Nokia: slightly different title in 6.x.1

DCM: give different title to 6.x.1.

Decision: 		The document was revised to S3-193717.

S3-193717	CIoT_ Modifications to draft CR

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell, Ericsson

(Replaces S3-193392)

Decision: 		The document was approved.

S3-193484	Security handling for RRCConnectionRe-establishment Procedure for Control Plane Optimization for 5GS CIoT

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

Awaiting SA2 descion on truncating S-TMSI.

Decision: 		The document was noted.

S3-193559	DraftCR – RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

Abstract:

This draftCR is for the living document.

Discussion:

Awaiting SA2 descion on truncating S-TMSI.

Decision: 		The document was noted.

S3-193558	DraftCR – Control Plane optimized solution

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

Abstract:

This draftCR is for the living document.

Discussion:

Editor's note from this document should be merged into S3-193818 (update of S3-193485).

Decision: 		The document was merged.

S3-193560	DraftCR – Control Plane optimized solution

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

Abstract:

This draftCR is for the living document.

Decision: 		The document was withdrawn.

S3-193485	Security handling in Control Plane User Datafor Control Plane Optimization for 5GS CIoT

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

Spelling corrections and addition of EN from S3-193558

Nokia: An EN on FFS to clarify the encoding on CP optimisated messages.

Decision: 		The document was revised to S3-193818.

S3-193818	Security handling in Control Plane User Datafor Control Plane Optimization for 5GS CIoT

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193485)

Decision: 		The document was approved.

S3-193483	CR for DDoS mitigation caused by misbehaving CIoT UEs

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

Nok: don't need this

QC: agree with Nokia.

Decision: 		The document was noted.

S3-193561	DraftCR – NAS based redirection between 5GS and EPC

Type: other		For: Approval
					33.501 v..
					Source: Ericsson

Abstract:

This draftCR is for the living document.

Decision: 		The document was approved.

#### 5.3.2	Study (FS_CIoT_sec_5G)

S3-193367	LS on GUTI re-allocation

Type: LS out		For: Discussion
					to CT1
					Source: Qualcomm Incorporated

Discussion:

Nokia: Agree an important principle to maintain 5G-GUTI re-allocation

Eri: Agree but want to delay response as incoming LS is out of scope

There was general agreement on the principle that GUTI re-allocation should be supported.

Chair: There will be an LS out of the meeting but will need discussion on the process to follow due to original LS not being part of meeting

Result of discussion: send LS as new LS, not as reply.

Decision: 		The document was noted.

S3-193542	CIOT: Title correction solution 13

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193366	Key Issue on UE capability protection for CP optimization only CIoT UE

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Qualcomm Incorporated

Decision: 		The document was merged.

S3-193437	Protection of UE capability transfer for UEs without AS security

Type: pCR		For: (not specified)
					33.861 v1.3.0
					Source: Intel

Discussion:

No comments on the key issue and threats part of the documents.

Decision: 		The document was revised to S3-193704.

S3-193704	Protection of UE capability transfer for UEs without AS security

Type: pCR		For: -
					33.861 v1.3.0
					Source: Intel

(Replaces S3-193437)

Discussion:

E//: disagree with security requirements

QC: what is the proposal

E//: more about persistance of capabilities

QC: then it is a security problem

Nokia: agree with E//, so security is not as strong as other devices.

Decision: 		The document was revised to S3-193843.

S3-193843	Protection of UE capability transfer for UEs without AS security

Type: pCR		For: -
					33.861 v1.3.0
					Source: Intel

(Replaces S3-193704)

Decision: 		The document was approved.

S3-193497	New KI: Preventing mismatch for EDT with fast release

Type: discussion		For: (not specified)
					33.861 v..
					Source: LG Electronics France

Discussion:

Nok: Need to wait for SA2 work

Eri: Agree to wait for SA2 but still a key issue open on this topic

DCM: Concern that we need to wait for SA2 in all cases.

Decision: 		The document was noted.

S3-193545	CIOT: Proposed clarifications to KI#6

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193536	CIOT: Update of KI#12 description

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Discussion:

QC: Why key issue needed as initial NAS Security covers them?

BT: Thought we had agreement on protecting IEs by default

Nok: don’t feel they are a privacy threat

QC: don't need a key issue

There was general agreement that initial NAS security should be used to protect these IEs but it was not clear in the meeting that the tex in TS 33.401 covered this.

Decision: 		The document was noted.

S3-193548	CIOT: Update of Solution #14

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193493	Address EN in solution 23

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Huawei, Hisilicon

Discussion:

Nok: It seems strange to add the load to the surrounding base stations

Huawei: by sending mis-behaving UE ID to other base stations so the other base stations know about mis-behaving Ues

Eri: Update is OK but concerned about the use of the list in general

Nok: Making the update optional would improve the situation

DCM: RAN should be made to eNB/gNBs probably throughout the whole solution.

Decision: 		The document was noted.

S3-193537	CIOT: update to the evaluation of solution #24

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson, Qualcomm Incorporated

Decision: 		The document was approved.

S3-193438	Security solution for UE Capability Transfer for UE with no AS security.

Type: pCR		For: (not specified)
					33.861 v1.3.0
					Source: Intel

Discussion:

E//: key issue not approved yet.

Decision: 		The document was noted.

S3-193391	AMF verification of the UE radio capabilities for CP optimization only CIoT UE

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

S3-193390	Hash based UE capability protection for CP optimization only CIoT UE

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

S3-193541	CIOT: Discussion on DDoS solutions for KI#4 and KI#5

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson, Qualcomm Incorporated, Samsung

Decision: 		The document was noted.

S3-193539	CIOT: Conclusion on KI #4

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

Discussion:

Support: cosigners

Object: KPN, Huawei

DCM: same as KPN conclusion, except for SA2 part, send LS to SA2 instead.

Approved as LS has been sent

Decision: 		The document was approved.

S3-193482	Conclusion for Key Issue #4

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193341	Conclusion to Key Issue #4

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: KPN

Abstract:

This contribution proposes a conclusion to Key Issue #4 - Signalling overload due to Malicious Applications on the UE.

Discussion:

Support: cosigners

Object: KPN, Huawei

DCM: same as KPN conclusion, except for SA2 part, send LS to SA2 instead.

Approved as LS has been sent.

Decision: 		The document was noted.

S3-193494	Conclusion of KI#5

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193540	CIOT: Conclusion on KI #5

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

Discussion:

Nokia: solution 23 spreads load to other base stations

Huawei: NWDAF to gNB interface is missing

Huawei: remove first paragraph

DCM: if NWDAF doesnt provide the information, then this needs to be added to LS

Huawei: delete the sentence,

Decision: 		The document was revised to S3-193711.

S3-193711	CIOT: Conclusion on KI #5

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

(Replaces S3-193540)

Decision: 		The document was approved.

S3-193547	CIOT: Proposed_conclusion_KI_6

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193552	Conclusion on Key Issue#7

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Lenovo, Motorola Mobility

Abstract:

This document concludes on key issue #7 of TR 33.861

Discussion:

Nokia: explain "missing freedom of key derivation"

Lenovo: because there is only NCC in key derivation, the attacker can assume that NCC is incremented each time.

Decision: 		The document was merged.

S3-193544	CIOT: Proposed conclusion to KI#7

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Discussion:

Lenovo: add that stationary UE need more attention

DCM: don't understand that limitation, keep E// contribution

offline what to add for this.

Decision: 		The document was revised to S3-193712.

S3-193712	CIOT: Proposed conclusion to KI#7

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

(Replaces S3-193544)

Decision: 		The document was approved.

S3-193546	CIOT: Proposed_conclusion_KI_8

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Discussion:

DCM: delete last sentence.

Decision: 		The document was revised to S3-193713.

S3-193713	CIOT: Proposed_conclusion_KI_8

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

(Replaces S3-193546)

Decision: 		The document was approved.

S3-193543	CIOT: Proposed conclusion to KI#10

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193538	CIOT: recommendation to support solution #24 to solve KI #11

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson, Qualcomm Incorporated

Decision: 		The document was approved.

S3-193549	CIOT: Conclusion for KI#12

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

Discussion:

E// presents, propose to say: no normative work is required.

Decision: 		The document was revised to S3-193714.

S3-193714	CIOT: Conclusion for KI#12

Type: pCR		For: Approval
					33.861 v1.3.0
					Source: Ericsson

(Replaces S3-193549)

Decision: 		The document was approved.

S3-193838	LS on GUTI re-allocation

Type: LS out		For: Approval
					to CT1
					Source: Qualcomm

Decision: 		The document was approved.

S3-193710	LS on Signalling overload due to malicious Applications on UE

Type: LS out		For: Approval
					to -
					Source: KPN

Discussion:

BT: include security concerns and tracking

E//: no reference to the solutions in our TR

QC: remove the reference about normative texts

Huawei: it is no normative work *in SA3*.

Decision: 		The document was revised to S3-193844.

S3-193844	LS on Signalling overload due to malicious Applications on UE

Type: LS out		For: Approval
					to SA2
					Source: KPN

(Replaces S3-193710)

Decision: 		The document was approved.

S3-193703	draft TR 33.861

Type: draft TR		For: Approval
					33.861 v1.4.0
					Source: Ericsson

Decision: 		The document was approved.

### 5.4	Security of the Wireless and Wireline Convergence for the 5G system architecture (5WWC_SEC)

S3-193663	PEI for FN-RG Recommendation

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: BBF

Decision: 		The document was noted.

S3-193664	General Status of Work

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: BBF

Decision: 		The document was noted.

S3-193479	living doc for 5WWC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Decision: 		The document was revised to S3-193684.

S3-193684	living doc for 5WWC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193479)

Decision: 		The document was approved.

S3-193384	Security for trusted non-3GPP access – General

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security for trusted non-3GPP access – General

Discussion:

Lenovo: overlaps with 3550, merge general clause

QC: clarify that sentence 2 is about EAP 5G

Orange: nothing on 5G AKA in Lenovo contribution.

E//: last paragraph of Nokia is not needed, it is a comparison. Call flow is copied, so put ed note on step 8 that this needs more work (authentication) ed note: stating details of key hierarchy are FFS.

Huawei: second ed note not needed now.

Decision: 		The document was merged.

S3-193385	Security procedure for trusted non-3GPP access

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security procedure for trusted non-3GPP access

Discussion:

Orange: remove text from living document, remove stuff from Living document, iti is pCR to living document.

Decision: 		The document was merged.

S3-193550	Solution for trusted non-3GPP access

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Lenovo, Motorola Mobility

Abstract:

The agreed solution #1 from TR 33.807 on trusted non-3GPP access is incorporated into the skeleton Draft CR of S3-193053.

Decision: 		The document was revised to S3-193685.

S3-193685	Solution for trusted non-3GPP access

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Lenovo, Motorola Mobility

(Replaces S3-193550)

Discussion:

EN added for figure to be changed next time

Decision: 		The document was approved.

S3-193631	Trusted access key hierarchy

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

Huawei: this is not the agreement in TR, additional argument is required why is this required.

E//: makes mobility easier

Lenovo: what kind of mobility

E//: between TNAPs.

BT: What are TNAP keys for: different devices, or different versions (updates) of keys in the same ME.

Decision: 		The document was noted.

S3-193632	Trusted access key derivation

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

Huawei: related to 631

offline discussions, to be taken back next time.

Decision: 		The document was noted.

S3-193379	Security for Wireline access to 5G - General

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security for Wireline access to 5G - General

Discussion:

Orange: no roaming, put in as Note.

BT: GW owner and connected operator are same, for R17 something more practical is required.

Orange: use note from TR, which is useful

Nokia: should be covered in SA2 architecture

Huawei: sentence is needed here.

QC: Last sentence: remove "potential"

Hauwei: remove last paragraph

E//: remove reference to 33.501

E//: clarify FN-RG in third paragraph

QC: reference in first paragraph is xx, add TS 23.whatever.

Decision: 		The document was revised to S3-193686.

S3-193686	Security for Wireline access to 5G - General

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193379)

Decision: 		The document was approved.

S3-193380	Authentication for 5G-RG

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication for 5G-RG

Discussion:

Thales: add concealing the SUPI.

Decision: 		The document was merged.

S3-193480	Authentication for 5G-RG

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

BT: equipped with UICC, prevent removal and replacement

Huawei: not part of study, need requirement in study for next meeting

E//: remove Ipsec in step 12.

Decision: 		The document was revised to S3-193687.

S3-193687	Authentication for 5G-RG

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193480)

Decision: 		The document was approved.

S3-193381	Authentication for FN-RG

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication for FN-RG

Decision: 		The document was merged.

S3-193481	Authentication for FN-RG

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

QC: duplicate steps 6 in call flow

E//: AUSF skips authentication - doesn't perform auth

DCM: lack of shalls

E//: clarify that FN-RG is legacy

Huawei: this is already in general clause

QC: title should be about security procedures, because this is more general.

Decision: 		The document was revised to S3-193688.

S3-193688	Authentication for FN-RG

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193481)

Decision: 		The document was approved.

S3-193382	Authentication for the UE behind the RG

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Authentication for the UE behind the RG

Decision: 		The document was approved.

S3-193383	Security of the interface between W-5GAN and 5GC

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Security of the interface between W-5GAN and 5GC

Discussion:

Eri: list formatting

Eri: clarify of the N2 interface.

Decision: 		The document was revised to S3-193689.

S3-193689	Security of the interface between W-5GAN and 5GC

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193383)

Decision: 		The document was approved.

S3-193633	Subscriber privacy for wireline access

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193551	Solution on 5GC access from WLAN UEs that do not support NAS

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Lenovo, Motorola Mobility

Abstract:

The agreed Solution #6 from TR 33.807 on 5GC access from WLAN UEs that do not support NAS, is incorporated into the skeleton Draft CR of S3-193053.

Discussion:

Orange: questions whether the text is needed as they don't support authentication

Len: There are some inter-working functions

Orange: would like to see text that shows what is different to general authentication

Len: will work offline to produce that text

Thales: do not understand why use the term 'devices'

Len: usage follows 23.502.

Decision: 		The document was revised to S3-193690.

S3-193690	Solution on 5GC access from WLAN UEs that do not support NAS

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Lenovo, Motorola Mobility

(Replaces S3-193551)

Decision: 		The document was approved.

### 5.5	5G security enhancement against false base stations (FS_5GFBS)

S3-193602	Correction of conclusion for Key Issue #1

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Decision: 		The document was approved.

S3-193334	Address EN in solution #1

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Decision: 		The document was approved.

S3-193486	Merged Solution for RRC Reject Protection

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Samsung

Discussion:

Eri: Have concerns with the solution

QC: Also have concerns with this solution.

Decision: 		The document was noted.

S3-193487	Conclusion for Key Issue #1 for RRC Reject

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Samsung

Discussion:

Eri: no normative work needed

QC: Agree with Ericsson.

Decision: 		The document was noted.

S3-193488	Update on Protection of RRC Resume Request message

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

Discussion:

Eri: Simpler solution is available as needs to be updated as message content changes

Apple: This could be merged with Ericsson solution.

Decision: 		The document was approved.

S3-193574	Solution for Resumecause protection

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Samsung

Discussion:

Eri: Solution is not needed

Sam: In case of conjestion, then short-MAC-I may not be checked

Huawei: Disagree with this solution.

Decision: 		The document was noted.

S3-193603	New Solution: Protection of the whole RRCResumeRequest message

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Discussion:

Apple: support this solution

Huawei: missing disadvantage, like increase in length, or processing

E//: put in a ednote?

Huawei: straight forward: disadvantage: increase of processing timei

DCM: make this an ed note, because it is not clear that copying IEs adn running hash over a shorter message has overhead of copying

QC: mixed deployments between R15 and R16 in network and with UEs. Ed Note: the impact of working with legacy UEs and base stations are ffs

Orange: not have evaluation this time, because of ed notes.

remove evaluation, add ed note in solution details.

Decision: 		The document was revised to S3-193753.

S3-193753	New Solution: Protection of the whole RRCResumeRequest message

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

(Replaces S3-193603)

Decision: 		The document was approved.

S3-193489	Conclusion for Key Issue #1 for RRC Resume Request Protection

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

Discussion:

E//: competing solution

Huawei, then postpone.

Decision: 		The document was noted.

S3-193604	Way forward for KI#1 against tampering of RRCResumeRequest messages

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193502	Protection of UeapabilityInformation

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

Orange: this is concluded already?

Huawei: agree with Orange.

Decision: 		The document was noted.

S3-193360	Evaluation of the shared key based MIB/SIB protection solution

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

(Replaces S3-192938)

Discussion:

Apple: only agree with first two bullets

Huawei: support QC, ed note: how to protect MIB and SIB if AS security is not active is FFS

E//: not accept first statement, ecause requirement says should detect in all states

QC: first two paragraphs need to be read together

E//: disagree with last para

QC: add that verifiaction of hash by gNB?

Add ed note, remove last two paragraphs.

Decision: 		The document was revised to S3-193754.

S3-193754	Evaluation of the shared key based MIB/SIB protection solution

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

(Replaces S3-193360)

Decision: 		The document was approved.

S3-193500	Evaluation for solution#14

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

QC: disagree

E//: support apple

Huawei: support QC.

Decision: 		The document was noted.

S3-193575	Evaluation of solution#14 in TR 33.809

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Samsung

Discussion:

QC: disagree, first statement is false, because step 1 is only after AS security set up, it is to protect SIB messages, not against fake base stations

Apple: support Samsung

E//: same comment

Huawei: support QC

Orange: support QC.

Decision: 		The document was noted.

S3-193361	Shared key based MIB/SIBs integrity information provided by gNB

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

(Replaces S3-192936)

Discussion:

Samsung: what happens if message 8 is not received, what is UE behaviour?

QC: not described, UE knows this.

Samsung how does Ue know that?

QC: UE identifies mismatch, in step 7, then if message 8 does not arrive,

Samsung: without message 8, what does the UE do? incomplete solution

QC add ed note: UE behaviour when message 8 is not received is FFS

E//: disagree, doesn't cover UE in idle mode.

Huawei: support QC

Apple: note, because similar solution exists already

QC: differnt place of verification

Decision: 		The document was noted.

S3-193363	Evaluation on UE behavior on detection of false signature

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

Discussion:

E//: not agreeing with bit flip attack. In second paragraph, alter attack is in controlled environment, using different PCI and frequency, with signature that could be avoided.

Apple: first para should be moved into ky issue, second para is DoS, true for all MACs, disagree with third and fourth para as well

QC: not move to key issue description.

Decision: 		The document was noted.

S3-193364	Evaluation on signing key management

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: stronly support last paragraph, because when moving to new TA, the UE can suffer from MiTM attack.

E//: UE could get new keys before moving to new TA

Samsung: disagree with second evaluation, first also doesn't hold

Apple: also similar to Samsung

QC: agree that all signing solutions will not be pursued further, as no evaluations are going in.

Apple: problems where of technical nature.

Decision: 		The document was noted.

S3-193576	Updates to Solution#7 on obtaining accurate clock information

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Samsung

Decision: 		The document was approved.

S3-193577	Deletion of EN on Location update reject in Solution#7

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Samsung

Discussion:

Huawei: whem UE sends reg req message, this is not integrity protected. Add ed note or note this

Samsung: not part of solution,

QC: keep ed note and note this, as it doesn't explain

E//: support, but ok to note to make progress

Decision: 		The document was noted.

S3-193499	Update for Solution#7

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

QC: disagree, should be noted.

Huawei: support QC, when UE moves to new area, it is possible to connect to FBS wthout the UE noticing, that is a technical problem

Decision: 		The document was noted.

S3-193501	Update of Solution#11

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

Orange: major problem is provisioning, therefore it is not viable

orange: not linking the SUCI scheme

DCM: provisioning is essential

BT: not link to SUCI scheme to not make it more difficult to SUCI off the ground

Decision: 		The document was noted.

S3-193514	Assessment and evaluation of solution #9

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: ZTE Corporation

Discussion:

E//: SIB/MIB is matter of serving network, so shouldn't involve home network.

Orange: agree.

BT: support that, not using home public key.

Decision: 		The document was noted.

S3-193461	A solution to protect MIB/SIBs

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

Discussion:

E//: incomplete, doesn't support UE in Idle mode, disagree

Huawei: if NAS security is available, then use NAS security in IDLE

QC: E// comment is about evaluation

Apple, similar to previous, solution not needed.

Samsung: hash 2 details are missing

Huawei: key issue is not concluded, this is only for TR.

Decision: 		The document was noted.

S3-193610	New solution (SERSI – SERving network controlled SI signatures).

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Discussion:

QC: this is just duplicating existing solution, so note

Apple: support solution

Samsung: key difference is including PCI and freq in signature, therefore merge into exisiting solution

QC: fine in principle, but bring this to next meeting

chaiir: try to avoid in next meeting

QC: ok

make into pCR of solution 7, figure and explanation

Orange: what changes in the evalution

QC: keep evaluation out.

E//. then agree on somtething for next meeting

DCM: make pcr adding PCI and DLfreq are input to signature in solution details of solution 7, add ed note to evaluation that impact of new inputs is ffs.

this is agreed.

Decision: 		The document was revised to S3-193755.

S3-193755	New solution (SERSI – SERving network controlled SI signatures).

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

(Replaces S3-193610)

Discussion:

QC: add ed note that evaluation for soultion is FFS or move evaluation ed note to clause above

Orange: add FFS to evaluation

Decision: 		The document was revised to S3-193845.

S3-193845	New solution (SERSI – SERving network controlled SI signatures).

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

(Replaces S3-193755)

Decision: 		The document was approved.

S3-193490	Conclusion for Key Issue #2

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

Discussion:

Orange: disagree to have different soultions for NPN and for public network.

Decision: 		The document was noted.

S3-193498	Conclusin of key issue#2

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

QC: disagree, provide evaluation for each of the options, first evaluations need to be completed.

Orange: there are still some open security issues, type of paper is like discussion paper.

E// support Apple

DCM: not conclude this before provisioning is defined

Huawei: same comment.

Decision: 		The document was noted.

S3-193335	Enabling UE to detect FBS

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

E//: several problems: during handover, measurement is fast, but now SIB information needs to be collected, so introduces delay for handover, no problems are in connected mode, because no handover to FBS.

Huawei: this verification is not before handover, this is after

E//: how do you verify

Huawei: collection is in preparation phase

E//: still disagree.

Decision: 		The document was noted.

S3-193439	Security solution for UE to avoid connecting to the false base station during a handover procedure

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Intel

Discussion:

QC: contention free RA is already supported, then why not use that rather than new solution

intel: first time handover still succeeds is step 6.

E//: attacker can play number 5 preambel as well

Intel: This is in evaluation.

Decision: 		The document was noted.

S3-193331	Solution#4: resolving EN network verification of hashes of MIB/SIBs

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

QC: is thisi new solution?

Huawei: existing,

QC: sounds like different solution

Huawei: not different, using OAM for sending additional information

QC: not additional information

Huawei: resolves ed notes

E//: support this.

Decision: 		The document was approved.

S3-193332	Solution#4: Resolving EN Impact on UE power consumption

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

QC: disagree, not only creating hash, wait for RAN reply

E//: supoprt this contribution

QC: evaluation not true, impact on poser consumption may be larger.

Decision: 		The document was noted.

S3-193333	Solution #4: Details on hash algorithm used for MIB/SIB hashes.

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Decision: 		The document was revised to S3-193679.

S3-193679	Solution #4: Details on hash algorithm used for MIB/SIB hashes.

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

(Replaces S3-193333)

Discussion:

QC: why use HMAC when not using key

DCM: use SHA 256

E//: ok

rewrite to use SHA256.

Decision: 		The document was revised to S3-193756.

S3-193756	Solution #4: Details on hash algorithm used for MIB/SIB hashes.

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

(Replaces S3-193679)

Decision: 		The document was approved.

S3-193365	Evaluation on Enriched MR

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: disagree with evaluation, too subjective, limit to postmortem is not true,

E//: disagree to many paragraphs, some ok with changes

QC: discuss this offline.

Decision: 		The document was noted.

S3-193329	Resolve EN on signaling details of how UE hands over to FBS

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: if FBS operates in same frequency, will it detect it?

Huawei: this is not about solution, this is only background information

Nokia: ok

E//: disagree with solution, but this is ok.

Decision: 		The document was approved.

S3-193330	Resolve second and third EN in Solution #6

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: there is ed note on RAN2 feedback,

Huawei: the LS was sent sone time ago

Nokia: keep ed note until response is there.

QC: same comment

bring back 2nd ed note

Decision: 		The document was revised to S3-193757.

S3-193757	Resolve second and third EN in Solution #6

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

(Replaces S3-193330)

Decision: 		The document was approved.

S3-193336	preventing UE from reselecting to FBS

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

E//: no actions should be standarduzed, detection may not be 100% accurate, blacklisting creates new DoS, problems are with idle mode, not connected Ues.

Huawei: remove actions or ed note

E/: remove blacklisting

QC: add several ed notes:

Nokia: blacklisting in central, so nothing is left over

Decision: 		The document was noted.

S3-193758	preventing UE from reselecting to FBS

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

S3-193337	Avoiding UE from Suffering More MitM Attacks by Handover

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

E//: does this use blacklisting?

Huawei: yes, gNB collects and then informs UE

Nokia: how does network know the neighbors

Huawei: is that the assumption

QC: so what is different from normal handover?

Huawei: nothing

Nokia: what to do about man in the middle.

Decision: 		The document was noted.

S3-193338	Evaluation of solution #6

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Discussion:

E//: add that this solution does not mitigate against radio repeater attacks

QC: agree with E//

add sentence

QC add ed note further evaluation is ffs based on RAN2 feedback

DCM: repeater is not only an attack., there are also operator deployed repeaters

Nokia: but there could be a malicious repeater

QC: not may require signalling overhead, make "requires".

Decision: 		The document was revised to S3-193759.

S3-193759	Evaluation of solution #6

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

(Replaces S3-193338)

Decision: 		The document was approved.

S3-193491	Solution for Avoiding UE connecting to False Base Station during Conditional Handover

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

Discussion:

E//: same comments as previous, overhead, malicious radio repeater, sentence in evaluation

DCM: does it make sense to document when we know we are not doing anything about it?

E//: evaluation is there already, so doesn't need to be touched again

QC: same things from previous contribution

DCM: there is sentence about neww  signalling overhead

QC: copy sentence from previous contribution

Huawei: change sentence to ivnolves overhead

sentence: about radio repeaters, include ed note, say requires overhead.

Decision: 		The document was revised to S3-193760.

S3-193760	Solution for Avoiding UE connecting to False Base Station during Conditional Handover

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon

(Replaces S3-193491)

Decision: 		The document was approved.

S3-193492	LS to RAN1 on Clarification for parameters used to avoid UE connecting to the FBS

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was withdrawn.

S3-193605	Way forward for KI#3 with respect to handover to a False basestation

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193339	Conclustion for Key issue #3

Type: pCR		For: Approval
					33.809 v0.4.0
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

S3-193440	Conclusions on Key Issue #3

Type: pCR		For: (not specified)
					33.809 v0.6.0
					Source: Intel

Discussion:

E//: ok with detection, but not for handover

Huawei: ok, decide the rest after RAN2 feedback

QC: wait for RAN2 feedback and further evaluation

ZTE: wait for RAN2 feedback for all

DCM: wait for RAN2 feedback for detection, handover, no normative work

QC: support DCM

Orange: support DCM

Huawei: too early.

DCM: what are waiting for on handover, what would be the best outcome to supprt handover to FBS prevention

E//: LS is only about detection.

Decision: 		The document was noted.

S3-193606	Way forward for KI#4 on protection against SON poisoning

Type: discussion		For: Endorsement
					33.809 v..
					Source: Ericsson

Discussion:

Apple: support this way forward

E//: could send an LS to SA5 that measurements could be untrustworthy

Orange: not from this meeting, need to discuss internally

QC: we previously discussed about measurements reports, make it generic, not only false base station

Futurewei: what do we expect SA5 to do with this

E//: take this into account

Futurewei: how should they know that the report was falsified

Orange: it may be more generic than false base station, look more closely before deciding on what and whether to send

QC: keep LS generic if sent from next meeting, the 2nd proposal in presentation was to have no normative work

E//: not put in that proposal because of what SA5 could respond

Apple: can there be an agreement to send an LS, so bring LS directly, so we can focus on content.

Decision: 		The document was noted.

S3-193362	Evaluation against MitM false base station attacks

Type: other		For: Endorsement
					Source: Qualcomm Incorporated

(Replaces S3-192937)

Discussion:

DCM: is there a clear definition of counts as MitM attack

QC: stupid relay is not a relay, it s described in key issue 7

Apple: no requirements in key issue 7

Decision: 		The document was noted.

S3-193448	Resolving the ENs in solution #5

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Lenovo, Motorola Mobility

Discussion:

E//: not sure this attack works, as it is possible to mimic the environment

DCM: should be part of evaluation

E//: ed note: how the solution is resilient against GPS spoofing and falsifying location related radio environment is FFS

QC: second ed note was not addressed

Orange: prefer not to have last paragraph of evaluation

Apple: in 6.5.2 location granularity whould be as accurate as possible, add ed ntoe what kind of granularity is sufficient.

Huawei: location using here is TAI, no other granularity is required, regarding second ed note: solved in first para, because procedure will fail.

DCM: unclear how consent is solved.

Decision: 		The document was revised to S3-193835.

S3-193835	Resolving the ENs in solution #5

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Lenovo, Motorola Mobility

(Replaces S3-193448)

Discussion:

DCM bring back privacy ed note

Orange ed note: further evaluation is needed.

Decision: 		The document was revised to S3-193846.

S3-193846	Resolving the ENs in solution #5

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Lenovo, Motorola Mobility

(Replaces S3-193835)

Decision: 		The document was approved.

S3-193555	Update of Solution #15

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Lenovo, Motorola Mobility

Abstract:

This paper addresses the Editor’s Notes of solution #15

Discussion:

E//: how is this working reagrding NAS keys and serving network name

E//: NAS SMC integrity failure could be because of radio failure

Lenovo: that can't happen on NAS, maybe in PDCP

E//: ed note: whether NAS layer integrity protection could fail due to radio conditions

E//: ed note: are NAS SMC failures only due to causes mentioned in this solution  are FFS

QC: similar comment, second ed note has not been addressed

Lenovo: ok

QC:add ed note further evaluation is FFS

1 ed note: NAS failures, bring back ed note on error handling, ednote in evaluation further evaluation is ffs.

Decision: 		The document was revised to S3-193836.

S3-193836	Update of Solution #15

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Lenovo, Motorola Mobility

(Replaces S3-193555)

Decision: 		The document was approved.

S3-193449	Conclusion on KI#5 of TR 33.809

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Huawei, Hisilicon, Lenovo, Motorola Mobility

Decision: 		The document was noted.

S3-193607	Way forward for KI#5 mitigation against authentication relay.

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Decision: 		The document was noted.

S3-193608	Way forward for KI#6 about the resistance to radio jamming

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Discussion:

QC: add because it can't be addressed by SA3, include reason

no objections to the conclusion, just add the reason.

Decision: 		The document was revised to S3-193837.

S3-193837	Way forward for KI#6 about the resistance to radio jamming

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

(Replaces S3-193608)

Decision: 		The document was approved.

S3-193609	Way forward for KI#7 about the protection against Man-in-the-Middle false basestation attacks

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Ericsson

Discussion:

QC: need to evaluate solutions for this one

Decision: 		The document was noted.

S3-193503	5G paging security issue caused by false base station

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Discussion:

Orange: is this DoS or ID request attack? Not required a new threat not clear what is leaked

QC: paging identifiers are realloceted every time, so not needed

Apple: attacker could guess the 5GS TMSI

Nokia: all 5G S-TMSI can be discovered after 45h,

DCM: bigger problem are cross layer attacks, this is too small a problem to solve.

Decision: 		The document was noted.

S3-193504	solution for new key issue of 5G paging security issue caused by false base station

Type: pCR		For: Approval
					33.809 v0.6.0
					Source: Apple

Decision: 		The document was noted.

S3-193506	Meeting minutes of 5GFBS July conference call on July 18th

Type: discussion		For: Information
					33.809 v..
					Source: Apple

Decision: 		The document was noted.

S3-193507	Meeting minutes of 5GFBS August conference call on August 8th

Type: discussion		For: Information
					33.809 v..
					Source: Apple

Decision: 		The document was noted.

S3-193752	Draft TR 33.809

Type: draft TR		For: Approval
					33.809 v0.7.0
					Source: Apple

Decision: 		The document was approved.

### 5.6	Security aspects of Enhancement of Network Slicing

#### 5.6.1	Work Item (eNS_SEC)

S3-193394	draft CR for Security procedures for Network Slices

Type: other		For: Agreement
					33.501 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

draft CR with a structure and combined text for the clause for security procedures for network slices.

Discussion:

BT: is there going to be confusion if there is slice specific secondary authentication

E//: there is no such thing

Huawei: comments on structure and have own proposal, x.x.1 and x.x.2 are confusing, together with 401

discussion captured under 401

Decision: 		The document was revised to S3-193738.

S3-193738	draft CR for Security procedures for Network Slices

Type: other		For: Agreement
					33.501 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193394)

Discussion:

IDCC: the steps added in 12 and 13 in 673 are copied from SA2

Nokia: start with the non-conditional cases, there is no disagreement on 673 yet

Huawei: only keep first sentence of ed note.

BT: authorization should be removed from x.x.3

QC: baseline needs a lot of improvement, not agree this as working baseline, not endorse as living CR, agree on the skeleton

Nokia keep baseline up to ed note

QC: make clear that this is just a living document.

Decision: 		The document was revised to S3-193857.

S3-193857	draft CR for Security procedures for Network Slices

Type: other		For: Agreement
					33.501 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193738)

Discussion:

the group agrees that all text in clause x.x.3 may need to be revised.

Decision: 		The document was approved.

S3-193673	Comments to Draft for ‘proposed structure for network slice security procedures’ in S3-193394

Type: other		For: Approval
					Source: InterDigital Communications

Abstract:

This contribution proposes comments and corrections to Draft for ‘proposed structure for network slice security procedures’ in S3-193394, Steps 13 and 14 of the call flow description.

Discussion:

part of offline discussion

Nokia: is anybody objecting these clarification

E//: prefers to start with normal flow before looking at optimization

for offline discussion

Nokia: only up to step 5 has been discussed, see 738, so no decision on the steps in this doc.

Decision: 		The document was noted.

S3-193401	Skeloton for eNS

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: needs an overall clause before coming to slice specific authenticaiton

E//: prefer Nokia  skeleton

IDCC: prefer Nokia

BT: agree to general topic heading.

QC: agree

DCM: are there all three types of authentication possible

Nokia: no

QC: in principle, it is possible, but secondary auth is orthogonal

Nokia: remove authentication from title of x.x.2

TIM: if there is authorization in both, what is the difference?

QC: agree, remove authorization from x.x.3, make it security procedures for...

discussion on general clause captured under 402.

discussion on x.x.2

Huawei: problem with content, very confusing

orange: wording needs to be fixed, because it is on authorization.

QC: need to have prerequisite to access network, information coming down from UDM, some slices are automaticly authorized

Nokia: make this editor's note

QC: try in this meeting to get this done.

Chair: prefer to close

QC: ok, add editor's note: the content of this clause is to be reformulated to capture 3 aspects -> taken offline.

Decision: 		The document was noted.

S3-193402	Introduction to slice-specific authentication

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: merge this, not all is valid

DCM: reproduce text given by QC.

Nokia: this tdoc is for third para of 3394, so unrelated

QC: make this ed note, remove second sentence

Huawei: remove third sentence as well

Nokia: what is problem with second sentence

Thales: it is unclear which authenticatino is meant, so delete for now.

QC: ed note: relationship netween teh different types of authentication procedures need to be provided

comments on text of 402

E//: align terminology, mention AAA proxy, baseline ok, room for improvement

Nokia: specific updates to individual sentences

Nokia: not as separate subclause, but in beginning of x.x.3, remove last two sentences, reformulate last sentence of second paragraph: multiple methods are possible.

Decision: 		The document was merged.

S3-193403	EAP based slice-specific authentication procedure

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: each step descriptiion is different so merge is difficult

E//: even call flow is different

IDCC: missing the multiplie access, which has been added by SA2

Nokia: Nokia contribution is more correct

Huawei: multiple access is in text

Nokia: merge in offlien session

Huawei: in pleanry, how to handle AUSF:

Nokia: that depends on SA reply

-> use call flow of Nokia contribution as baseline, work offline

E//:  step 13, and 14 don't work in 394

Huawei: last two paragraphs of 394 should go away

Decision: 		The document was merged.

S3-193415	Discussion for EAP method negotiation

Type: discussion		For: Endorsement
					33.813 v..
					Source: Huawei, HiSilicon

Discussion:

QC: why is this needed, identity defines EAP method

Nokia: agree with QC

E//: agree with Nokia

Huawei: reference is required

Nokia: in primary it is known, so similar

Decision: 		The document was noted.

S3-193404	EAP method negotiation

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: disagree, UE says what it can and will do

Decision: 		The document was noted.

S3-193407	Security features for NSaaS

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

S3-193648	draft LS to SA2 on the procedure of Secondary authentication

Type: LS out		For: (not specified)
					to SA2
					Source: China Mobile Com. Corporation

Decision: 		The document was withdrawn.

#### 5.6.2	Study (FS_eNS_SEC)

S3-193405	Add Evaluation to solution 3

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

Nok: revise the sentence for clarity

Decision: 		The document was revised to S3-193731.

S3-193731	Add Evaluation to solution 3

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193405)

Decision: 		The document was approved.

S3-193406	Conclusion to KI#3

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

Nok: Not clear that this require normative work

Hua: Needs to be started that this data can be configured

Eri: Agree with Nokia that there is no need for normative work

QC: Leaning to agree with Nokia

Huawei: Should write an LS to SA5 to get feedback on the inclusion.

Decision: 		The document was noted.

S3-193408	Addressing EN in solution 6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

QC: replace new text by: "if user ID privacy is required, it is protected by the EAP method"

Huawei: this solution assumes privacy is required

Orange: the requirement is not there, yet.

Decision: 		The document was revised to S3-193734.

S3-193734	Addressing EN in solution 6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193408)

Decision: 		The document was approved.

S3-193409	Evaluating solution 6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: needs to be reworded to capture what is said in previous tdoc. Reformulate middle sentence: This solution relies on ID privacy protection by EAP framework. Delete third sentence

Huawei: third sentence is correct, just evaluation

QC: should be EAP method, not framework.

Decision: 		The document was revised to S3-193735.

S3-193735	Evaluating solution 6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193409)

Decision: 		The document was approved.

S3-193416	Overall evaluation of solutions addressing KI#4

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

Orange: good discussion, but not include in TR.

Huawei: what is the problem, to show what the rationale was

QC: adds no value, go straight to conclusion.

Decision: 		The document was noted.

S3-193410	Concluding KI#4

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

QC: not ready to accept yet, but say that EAP method should provide privacy

Nokia: none of the solutions requires normative work, so that should be the conclusion

Huawei: because some EAP methods don't provide privacy

Nokia: then it is a deployment issue

Orange: agree, it is a deployment issue, non ormative work is needed.

QC: need to make point that slice owner needs to pick the right EAP method.

Huawei: yes, specify note, activate NAS and AS security

Orange: is this just a Note

Huawei: but where to put note

It was agreed: There will be a Note in 33.501 adressing the user ID privacy for the slice specific authentication procedure.

Orange: change conclusion to "no normative work is needed".

Decision: 		The document was revised to S3-193736.

S3-193736	Concluding KI#4

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193410)

Decision: 		The document was approved.

S3-193411	On service request in solution 8

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: four solutions in the next documents, a lot of overhead in all of them , all are complicated, maybe have an offline for those four documents

Huawei: agree

DCM: make the proposed text as real text and not NOTE

Nok: Need to add EN on idle mode mobility

Huawei: This is not ralted to resolution of EN.

Decision: 		The document was revised to S3-193819.

S3-193819	On service request in solution 8

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193411)

Decision: 		The document was approved.

S3-193412	On Ng-RAN node change in solution 8

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Discussion:

ID: Isn't it a burden to have to give the whole set of UE specific T-S-NSSAIs

Huawei: There are different options

It was agreed to take the first EN from S3-193675 in the revision of this document.

Decision: 		The document was revised to S3-193820.

S3-193820	On Ng-RAN node change in solution 8

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

(Replaces S3-193412)

Decision: 		The document was approved.

S3-193675	Nokia comments on S3-193412

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Huawei does not agree with proposed Ens

Eri: OK with solution update and ENs going forward

Agreed to take only the first EN.

Decision: 		The document was noted.

S3-193354	Resolving editor’s note on refreshing the parameter in solution #10

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Qualcomm Incorporated

Discussion:

Nokia: feels ed note is only partly addressed, but ok

Decision: 		The document was approved.

S3-193355	Resolving editor’s note on relationship between S-NSSAI and the S-NSSAI identifiers in solution #10

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Qualcomm Incorporated

Discussion:

IDCC: this is beoming more confusing, encrypted identifier is pseudonym that is shorter, so more elaboration on what is the pseudonym, ed note relationship between S-NSSAI and S-NSSAI identifier needs elaboration

E//: are S-NSSAI idnetifier encrypted?

QC: yes, that is what is described

add ed note.

Decision: 		The document was revised to S3-193821.

S3-193821	Resolving editor’s note on relationship between S-NSSAI and the S-NSSAI identifiers in solution #10

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Qualcomm Incorporated

(Replaces S3-193355)

Decision: 		The document was approved.

S3-193395	eNS_Addition to evaluation of solution 10

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

QC: don't need to maintain the all this context, only maintain NSSAI identifier to NSSAI mapping, disagree

DCM: state that the NSSAI space is then reduced

IDCC: problem to understand  the solution

E//: share Nokias concerns.

Decision: 		The document was revised to S3-193822.

S3-193822	eNS_Addition to evaluation of solution 10

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell, Ericsson

(Replaces S3-193395)

Decision: 		The document was approved.

S3-193301	TR 33.813 - update for solution #11

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

Editor’s Note: it is FFS how Base Stations under one AMF learn Hash Values and route the NAS messages to a particular AMF. This may an impact on the solution evaluation.

Decision: 		The document was approved.

S3-193309	TR 33.813 - update for the evaluation for solution #11

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: InterDigital Communications

Abstract:

This contribution updates the evaluation for the Solution #11.

Discussion:

Nokia: 3396 is related

together with 3396

Nokia: remove word fully from evaluation in 309

QC: solution doesn't work well, so discuss this offline.

Decision: 		The document was revised to S3-193823.

S3-193823	TR 33.813 - update for the evaluation for solution #11

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: InterDigital Communications

(Replaces S3-193309)

Decision: 		The document was approved.

S3-193396	eNS_Addition of evaluation to solution 11

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

IDCC: on ed note: based on existing NGAP procedure, so should be clear enough. No per UE context required in RAN. All other parts of ed note also to be refuted

E//: support Nokia's ed note

Nokia: UE identity in AS layer, how is it learned

IDCC: from S-TMSI

Decision: 		The document was revised to S3-193824.

S3-193824	eNS_Addition of evaluation to solution 11

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell, Ericsson

(Replaces S3-193396)

Decision: 		The document was approved.

S3-193674	Comments to Evaluation for Solution 11 in S3-193396

Type: other		For: Approval
					Source: InterDigital Communications

Abstract:

This contribution proposes comments and corrections for Evaluation for Solution 11 in S3-193396.

Decision: 		The document was noted.

S3-193517	Resolving ENs of solution #12

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: ZTE Corporation

Discussion:

E//: keep first editor's note

Nokia: 676 is comments

together with 676

ZTE: this solution has similar to Huawei solution, why not only one ed note

Nokia: more ed notes are more helpful, but also ok to say solve idle mode mobillity

bring back first ed note, ed note on idle mode mobility to be added

Decision: 		The document was revised to S3-193825.

S3-193825	Resolving ENs of solution #12

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: ZTE Corporation

(Replaces S3-193517)

Decision: 		The document was approved.

S3-193676	Nokia comments on S3-193517

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

S3-193518	Evaluation of solution #12

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: ZTE Corporation

Discussion:

Nokia: to early to add evaluation without idle mode mobility.

Decision: 		The document was noted.

S3-193317	TR 33.813 – review and comparative evaluation of solution for NSSAI privacy

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: InterDigital Communications

Abstract:

This PCR attempts to aid SA3 in assessing solutions addressing “Key Issue #6 : Confidentiality protection of NSSAI and home control” by producing a solutions comparisons table.

Decision: 		The document was noted.

S3-193417	Overall evaluation of solutions addressing KI#6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

S3-193677	Nokia comments on S3-193417

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Nokia: prefer 417

Huawei: no time to work on this offline at this meeting, but agree with nokias comments

IDCC: some solutions need more details to be evaluated

Decision: 		The document was noted.

S3-193413	Conclusions to KI #6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

S3-193496	Conclusion on KI#6

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Ericsson, Nokia, Nokia Shanghai Bell

Discussion:

Nokia: propose to have call before next meeting on NSSAI privacy.

Decision: 		The document was noted.

S3-193553	Clarification on cancellation of rejected S-NSSAIs

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Lenovo, Motorola Mobility

Abstract:

This document provides additional clarifications on the cancellation of rejected S-NSSAIs.

Discussion:

Nokia: next tdoc, on same clauses.

Decision: 		The document was revised to S3-193737.

S3-193737	Clarification on cancellation of rejected S-NSSAIs

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Lenovo, Motorola Mobility

(Replaces S3-193553)

Discussion:

Nokia disagree with this interpretation of key issue, should be dynamic subscription re-enablement, so beyond the study item.

Lenovo: CT1 already added ed note.

Huawei: Nokia proposal seems agreeable

Decision: 		The document was noted.

S3-193397	KI#7 Revocation of rejected NSSAI

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Lenovo: cannot leave it to OAM

Huawei: this would be too restrictive

Nokia: there is no restriction on the mechanism

E//: in overload situation it is marked as pending, not as failed

Nokia: if a fraction of NSSAI attaches failes, then it should be possible to reauthenticate without primary auth

Lenovo: problem is only external AAA knows when to retry.

IDCC: Agree that problem is possible, but not for this group

DCM: how does the UE know the reason, load or OAM?

Nokia: error code

QC: agree with IDCC

Decision: 		The document was merged.

S3-193554	Solution on Cancellation of rejected S-NSSAIs

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Lenovo, Motorola Mobility

Abstract:

This paper provides a solution for rejected S-NSSAI revocation.

Decision: 		The document was noted.

S3-193398	Solution for KI#7 revocation of rejected NSSAI

Type: pCR		For: Approval
					33.813 v0.6.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

S3-193732	draft TR 33.813

Type: draft TR		For: Approval
					33.813 v0.7.0
					Source: Nokia

Decision: 		The document was approved.

S3-193733	LS on configuaration of security policy for NSaaS

Type: LS out		For: Approval
					to SA2
					Source: Huawei

Discussion:

Orange: no reason to send it.

Nokia: in SA2, there are many policy configuration mechanisms, let's not include yet another mechanism.

Decision: 		The document was noted.

S3-193809	Notes from evening session on eNSI

Type: report		For: Information
					Source: Qualcomm

Decision: 		The document was noted.

### 5.7	Security of the enhancement to the 5GC location services (eLCS_Sec)

S3-193434	Draft CR-living doucment for 5G_eLCS

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: CATT

Discussion:

Merged into S3-193702

Nok: Is feature optional to use by network or UE

Decision: 		The document was merged.

S3-193563	Draft CR as a living baseline for 5GS LCS normative work

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson LM

Abstract:

As a starting point, an initial baseline for normative work is proposed as a new clause in TS 33.501

Discussion:

Huawei: Prefer S3-193563 over S3-193434 - add an EN about bluetooth public name and public address is FFS and one about connection to false access point is FFS

BT: EN should be about corrupting data collection

DCM: Need an EN on linking names to network

Interdigital: If the data is collected without vefirication, then it has no value as it could be be faked to provide an attack.

QC: Text should be a new Annex.

Decision: 		The document was revised to S3-193702.

S3-193702	Draft CR as a living baseline for 5GS LCS normative work

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson LM

(Replaces S3-193563)

Discussion:

DCM: copy last two ed notes inito WLAN as well.

Decision: 		The document was revised to S3-193847.

S3-193847	Draft CR as a living baseline for 5GS LCS normative work

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson LM

(Replaces S3-193702)

Decision: 		The document was approved.

### 5.8	Security for 5G URLLC (5G_URLLC_SEC)

S3-193476	draftCR for URLLC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

Living document from last time

Decision: 		The document was revised to S3-193699.

S3-193699	draftCR for URLLC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193476)

Decision: 		The document was approved.

S3-193477	change introduction to align with URLLC architect

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

Eri: Propose that could move text out of dual connectivity

Huawei: Only one line on redundant bearer in SA2

QC: typo in wording

Nokia: Is two path mandatory?

Huwaei: It is not manadatory

DCM: Language should not be normative where it is SA2 described procedures.

Decision: 		The document was revised to S3-193700.

S3-193700	change introduction to align with URLLC architect

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

(Replaces S3-193477)

Decision: 		The document was approved.

S3-193478	clarification on security policy handling

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Huawei, Hisilicon

Discussion:

QC: Baseline text is wrong - first sentence of third paragraph is not needed

Eri: Have a similar concern

Decision: 		The document was merged.

S3-193627	Draft CR: UP security policy for URLLC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

Huawei: OK with 2nd modification - 1st modification adds redundant text

Nok: Agree with Huawei

DCM: consider the normative language in the document

Decision: 		The document was revised to S3-193701.

S3-193701	Draft CR: UP security policy for URLLC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

(Replaces S3-193627)

Discussion:

Nokia: disagree with preconfiguration, nonode should be preconfigured

Huawei: not agree with first change, this has a new comment by Nokia, prefer to note this time

Nokia: align with SA2, no point in putting some random statement, policy comes from SMF

E//: ed note to check with RAN2 internaly

revert back to plenary agreements

Decision: 		The document was revised to S3-193848.

S3-193848	Draft CR: UP security policy for URLLC

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

(Replaces S3-193701)

Decision: 		The document was approved.

### 5.9	Study on SECAM and SCAS for 3GPP virtualized network products (FS_VNP_SECAM_SCAS)

S3-193645	Meeting minutes of VNP_SECAM_SCAS conference call on 25th September

Type: discussion		For: Information
					33.818 v..
					Source: China Mobile

Decision: 		The document was noted.

S3-193649	Add Clarifications to the Scope of Accreditation for 3GPP VNPs

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.3 Scope of SECAM Accreditation for 3GPP virtualized network products.

Discussion:

CMCC: About EN, do we need to send an LS to the GSMA - maybe from next meeting

Eri: Why accreditation process does not need to be used

Nok: If consenus, then not needed as previously agreed

Huawei: Can remove accreditatio process part

Nok: Sentence the same in current SECAM specifications

Eri: GSMA is saying that ISO accreditation in needed

Taken offline to discuss that part only.

Nokia: ok to delete the added sentence in Scope

Sentence is to be removed "The same applies ... Desires".

Decision: 		The document was revised to S3-193849.

S3-193849	Add Clarifications to the Scope of Accreditation for 3GPP VNPs

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193649)

Decision: 		The document was approved.

S3-193650	Add Clarifications to Ultimate Output of SECAM Evaluation

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.4 Ultimate Output of SECAM Evaluation.

Discussion:

CMCC: Concerned specifications not known

Hua: Agree with concern

Hua: Believe the evidence is required and sentence on no evidence should not be added

BT: Thought that the idea was for vendor to get their equipment checked

CMCC: There are two ways that SECAM can work two ways

DCM: Agree that sentence should not be added

Decision: 		The document was revised to S3-193780.

S3-193780	Add Clarifications to Ultimate Output of SECAM Evaluation

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193650)

Decision: 		The document was approved.

S3-193651	Add Clarifications to 3GPP virtualized network product evaluation process

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.5 3GPP virtualized network product evaluation process.

Discussion:

BT: May want to consider this as a certification scheme due to Cybersecurity act - propose to change the sentence on 'out of scope of SECAM'.

Hua: first change change to may makes it optional which not sure is correct

Decision: 		The document was revised to S3-193782.

S3-193782	Add Clarifications to 3GPP virtualized network product evaluation process

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193651)

Decision: 		The document was approved.

S3-193652	Add Clarifications to Roles in SECAM for 3GPP virtualized network products

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.6 Roles in SECAM for 3GPP virtualized network products.

Discussion:

Huawei: oshould say other SDO than applicable in diagram

Nokia: discussion with CMCC: leave out other SDO

DCM: why remove other SDOs?

CMCC: other SDOs will provide requirements to 3GPP to integrate into SCAS

agreement: only change diagram.

E//: why is the half sentence added.

Nokia: complete sentence, GSMA is curerntly SECAM accreditation body.

Decision: 		The document was revised to S3-193783.

S3-193783	Add Clarifications to Roles in SECAM for 3GPP virtualized network products

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193652)

Decision: 		The document was approved.

S3-193653	Add Clarifications to SECAM Assurance Level for 3GPP VNPs

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.8 SECAM Assurance level for 3GPP virtualized network products.

Discussion:

Nokia: offline discussion with CMCC: SECAM assutance level completely differnt from ETSI NFV levels. Remove sentence in ed note.

BT: decide on a mapping, rather than on which level.

BT: the editor's note needs to be deleted or braought back. mapping is not possible, because NFV assumes starting from bottom (hardware), but SECAM doesn't do that. so delete references to NFV assurance levels.

second paragraph on clause  4.8.1 removed, delete editor's note in 4.8.2

Decision: 		The document was revised to S3-193784.

S3-193784	Add Clarifications to SECAM Assurance Level for 3GPP VNPs

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193653)

Decision: 		The document was approved.

S3-193655	Add Clarifications to Security Baseline for 3GPP VNPs

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 4.9 Security baseline for 3GPP virtualized network products.

Decision: 		The document was approved.

S3-193656	Add Clarifications to SCAS documents structure and content

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 5.1 Writing process overview and 5.2 SCAS documents structure and content.

Discussion:

BT: if the direction is to reduce the testing to teh virtual product, then this decision is too early.

Huawei: currently prefer to work directly in 33.926.

Nokia: the decision is not made

BT: but then they are identical for teh NFs to the self contained products.

DCM: note document?

CMCC: remove the contentious paragraph in 5.1

remove added paragraph in 5.1

Decision: 		The document was revised to S3-193785.

S3-193785	Add Clarifications to SCAS documents structure and content

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193656)

Decision: 		The document was approved.

S3-193638	Clarifying GVNP model of type 2

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Decision: 		The document was approved.

S3-193639	Adding description for Generic virtualized network product model of type 3

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Discussion:

BT: in hardware sectino, hardware is not necessarily COTS, maybe in telecoms it is remove that sentence

E//: add the same editors note as on type 1 and 2., add ed note which ETSI specifications are being referred to

Nokia: on hardware change description of picture: "hardware layer in addition to 3GPP", following sentence:VNF -> VNFC, add word "layer" in hardware.

Remove last sentence in clause x.4, ed note under teh diagram, and ed note for reference, + Nokia changes.

Nokia: more comments offline.

Decision: 		The document was revised to S3-193786.

S3-193786	Adding description for Generic virtualized network product model of type 3

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

(Replaces S3-193639)

Decision: 		The document was approved.

S3-193640	Adding Generic assets and threats of GVNP for type 1 into clause 5.2.3.b

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Discussion:

Nokia: ed ntoe about moving this to 900 series TR

CMCC: need to finalize this first, then move over

Nokia: ok

BT:  .b.2.3: say that there are two "relevant" interfaces, because more are defined; b.2.10, list identical to no virtual system, virtualization layer threats need to added, add editor's note

keep open for offline

Decision: 		The document was revised to S3-193787.

S3-193787	Adding Generic assets and threats of GVNP for type 1 into clause 5.2.3.b

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

(Replaces S3-193640)

Decision: 		The document was approved.

S3-193641	Adding Generic assets and threats of GVNP for type 2 into clause 5.2.3.c

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Discussion:

BT: this needs more work to defferentiate from nomral scas, but ok to put in.

E//: reference etc. as for 640

goes offline

Decision: 		The document was revised to S3-193831.

S3-193831	Adding Generic assets and threats of GVNP for type 2 into clause 5.2.3.c

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

(Replaces S3-193641)

Decision: 		The document was approved.

S3-193642	Adding Generic assets and threats of GVNP for type 3 into clause 5.2.3.d

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Decision: 		The document was revised to S3-193832.

S3-193832	Adding Generic assets and threats of GVNP for type 3 into clause 5.2.3.d

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

(Replaces S3-193642)

Decision: 		The document was approved.

S3-193643	Adding security requirements  into clause 5.2.4

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Discussion:

Nokia: in diagram, ETSI defines GR, GS not TR, TS; in 5.2.4.2. not define a new catalogue, but have it as extension

CMCC: can't change figure now, ed ntoe instead

BT: replace specified in 5.2.4.2 for identified, as TR doesn't specify

ed note to revise figure to change TR and TS to GR and GS for; rephrase first para of 5.2.4.2 to make it into extension instead of addition

Decision: 		The document was revised to S3-193833.

S3-193833	Adding security requirements  into clause 5.2.4

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

(Replaces S3-193643)

Decision: 		The document was approved.

S3-193657	Add Clarifications to generic virtualized network product model description

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes some clarifications to TR 33.818 clause 5.2.5 Generic virtualized network product model description. Two editor’s notes on mapping of ETSI NFV interface and 3GPP interface are resolved by adding more specific clarifications for the relevant interfaces.

Discussion:

CMCC: what is the reference [x]?

Nokia: will include missing reference

CMCC: name of local logical interface, change to execution interface

Nokia: maybe rename to execution environment interface

DCM: please provide definitino for next time

Decision: 		The document was revised to S3-193834.

S3-193834	Add Clarifications to generic virtualized network product model description

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193657)

Decision: 		The document was approved.

S3-193644	applying TR33.818 with new 3GPP_TS-TR_Template

Type: pCR		For: Approval
					33.818 v0.4.0
					Source: China Mobile

Decision: 		The document was approved.

S3-193781	Draft TR 33.818

Type: draft TR		For: Approval
					33.818 v0.5.0
					Source: China Mobile

Decision: 		The document was approved.

### 5.10	Security for 5GS Enhanced support of Vertical and LAN Services

#### 5.10.1	Work Item (Vertical_LAN_SEC)

S3-193662	LS on SUCI computation from an NSI

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: CP-192262

Decision: 		The document was postponed.

S3-193368	Reply LS on SUCI computation from an NSI

Type: LS out		For: Approval
					to CT, SA1, SA2, CT1, CT6, cc CT4, SA
					Source: Qualcomm Incorporated

Discussion:

DCM doesn't answer the question

QC: there are features that don't work

Thales: disagree with QC, currently nothing prevents the two identities

Orange: agree with LS, but need to add whether there are security concerns, Orange doesn't see any, respond from next meeting, but should be only one key.

Nokia: two identifiers for UE is ok?

Orange: yes, but need more time to study.

Decision: 		The document was noted.

S3-193353	Some corrections/clarification for non-public networks annex

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Qualcomm Incorporated

Decision: 		The document was merged.

S3-193512	CAG ID Privacy for non-public networks

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: ZTE Corporation

Discussion:

Nokia: depends on study item.

Decision: 		The document was noted.

S3-193527	CR CAG ID privacy

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

it depends on study item.

Decision: 		The document was noted.

S3-193528	Removal of ed.note on conformance tests

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was approved.

S3-193529	Editorial correction of format

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to S3-193705.

S3-193705	Editorial correction of format

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193529)

Decision: 		The document was approved.

S3-193531	CR Interworking between NPN and PLMN

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

E//: depends on result of study

Nokia: document in Annex, in SA2, or subclause somewhere else

QC: document separate fromm NPN

DCM: not in SA2

QC: clarify this is  about secondary authentication

tdoc name of revision had to be changed to reflect content

Decision: 		The document was revised to S3-193706.

S3-193706	CR Interworking between NPN and PLMN

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193531)

Decision: 		The document was approved.

S3-193532	CR Annex 5GLAN

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to S3-193707.

S3-193707	CR Annex 5GLAN

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193532)

Discussion:

Huawei: what is the authentication mechanisms

DCM: shall be used, not are

Huawei: formatting, should this be under same Annex

Nokia: no, same study: different topic

Huawei: ed note: which authentication mechanism

QC: not only auth mechanism, but also SMC, etc.ed note about all security procedures.

Nokia: delete second paragraph,

DCM: acronyms need to be added, refernce to IEEE TSN to be added

tdoc name of revision had to be changed to reflect content

Decision: 		The document was approved.

S3-193533	CR Annex TSC

Type: draftCR		For: Agreement
					33.501 v16.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Orange: nothing required in our TS.

Huawei: change NPN to SNPN

Thales: prefer not to add anything now.

Orange: also nothing is required for integrated NPNs.

Title should be(to reflect content of contribution): CR Interworking between NPN and PLMN

Decision: 		The document was noted.

#### 5.10.2	Study (FS_Vertical_LAN_SEC)

S3-193302	TR 33.819 – update of DH based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: the effectiveness of this solution against False Base Station attack is FFS.

•	Editor’s Note: how CAG ID is kept secret in case of changes of subscription and/or CAG IDs is FFS.

Decision: 		The document was approved.

S3-193303	TR 33.819 – update of Hash based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: the effectiveness of this solution against False Base Station attacks is FFS.

•	Editor’s Note: how CAG ID is kept secret in case of changes of subscription and/or CAG IDs is FFS.

•	Editor’s Note: resistance to offline/online dictionary attacks due to a low length of CAG ID is FFS.

Discussion:

Huawei: what is sufficient length ofCAG ID, where is it defined

IDCC: this is a suggested length

Huawei: length of CAG needsd to be defined ed note.

Nokia: fine as part of solution, because it says e.g.

QC: text against FBS is not making claims against active attacks.

IDCC: correct

Decision: 		The document was approved.

S3-193308	TR 33.819 – review and comparative evaluation of solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

Abstract:

This PCR attempts to aid SA3 in assessing solutions addressing Key Issue #6.2: CAG ID privacy by producing a solutions comparisons table.

Discussion:

need to wait for SA2

IDCC: propose to note all of the CAG conclusions.

Decision: 		The document was noted.

S3-193310	TR 33.819 – evaluation of DH based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation to the DH based solution for CAG ID privacy.

Discussion:

QC: after a FBS runs DH, it can run passive attacks,

IDCC that was not claimed

E//: mention explicitly that it doesn't address active attacks

QC: move that to first line

IDCC: requirements don't say both

QC: remove fully, add "against passive attacks..." to first sentence, combining last and first sentence

agreed on last QC comment

Decision: 		The document was revised to S3-193826.

S3-193826	TR 33.819 – evaluation of DH based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

(Replaces S3-193310)

Decision: 		The document was approved.

S3-193311	TR 33.819 – evaluation of Hash based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation to the Hash based solution for CAG ID privacy.

Discussion:

QC: does this against a passive attack from a UE in the same CAG

IDCC: insider attack

QC: not insider, include comment from 310

DCM: this solution does not protect against passive attacks by members of the CAG

QC changes and DCM comment agreed

Decision: 		The document was revised to S3-193827.

S3-193827	TR 33.819 – evaluation of Hash based solution for CAG ID privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: InterDigital Communications

(Replaces S3-193311)

Decision: 		The document was approved.

S3-193456	Adding the evaluation of solution #15

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

Discussion:

E//: support this

Nokia: it is modifying the message on the radio interface

Huawei: wait for RAN2 response on that

Nokia: this comment can come back next time

Decision: 		The document was approved.

S3-193457	Adding conclusion on KI #6.2

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193465	Solution for protection of time synchronization

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

Discussion:

Nokia: need more details, ed note details in line with SA2 specs is FFS

Decision: 		The document was revised to S3-193709.

S3-193709	Solution for protection of time synchronization

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

(Replaces S3-193465)

Decision: 		The document was approved.

S3-193466	Conclusion on protection of time synchronization

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

Discussion:

Nokia: postpone to next meeting.

Decision: 		The document was noted.

S3-193467	Remove the EN and add evaluation to solution 6

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, Hisilicon

Decision: 		The document was approved.

S3-193511	Conclusion of CAG ID Privacy

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: ZTE Corporation

Decision: 		The document was noted.

S3-193322	Editorial corrections for TR 33.819

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Decision: 		The document was approved.

S3-193325	Evaluation of solutions addressing Key Issue #6.2

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

S3-193523	TR 33.819 update

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

IDCC: don't waste time on CAG ID privacy, wait for SA2 progress.

Decision: 		The document was approved.

S3-193524	Status of TR

Type: discussion		For: Discussion
					33.819 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

S3-193525	CAG ID privacy solution considering RAN optimization

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was approved.

S3-193526	CAG ID privacy conclusion -v1

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

S3-193530	Conclusion to key issue 2.3

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Orange: no conclusion for this one, noted

BT is this related to multiple subscriptions

Decision: 		The document was noted.

S3-193593	Udpate to Solution #3

Type: pCR		For: (not specified)
					33.819 v1.2.0
					Source: Samsung

Discussion:

Nokia: what are various factors?

Samsung remove "based on various factors", add ed note that factors should be listed

Nokia: better  come back next time.

Samsung: give a few factors in this meeting

BT: concerned about dynamic factors in UDM.

Decision: 		The document was noted.

S3-193594	Key Issue #6.1 conclusion

Type: pCR		For: (not specified)
					33.819 v1.2.0
					Source: Samsung

Decision: 		The document was noted.

S3-193708	draft TR 33.819

Type: draft TR		For: Approval
					33.819 v1.3.0
					Source: Nokia

Decision: 		The document was approved.

### 5.11	Study on LTKUP Detailed solutions (FS_LTKUP_Detail)

### 5.12	Study on User Plane Integrity Protection (FS_UP_IP_Sec)

S3-193666	Reply to LS on Impersonation Attacks in 4G Networks

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: R2-1911819

Discussion:

Nok: should full rate by supported from Rel-16

QC: Full-rate can be supported for Rel-15 UEs

Samsung: Need non-full rate in Rel-16 for some Ues

QC: It is a product and not a standards issue

Orange: Need to specify full rate

Decision: 		The document was noted.

S3-193320	Proposal to solve ED notes in solution#4: Zero-overhead user plane integrity protection on the link layer

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Philips International B.V.

Abstract:

This contribution proposes solutions to the editor's notes in clause 6.4	Solution #4: Zero-overhead user plane integrity protection on the link layer. The proposal made in Wroclaw is extended here to work with multiple PDCP instances.

Discussion:

Nok and DCM: Not clear what truncated encryption means - explanation would be good in the text

Nok: First EN needs to be retained

Philips: Explained in new text

DCM: Replace EN1 as there is an impact by replacing hardware CRC with cryptgraphic function

CMCC: Add sentence on the length of MAC compared to 32 bits

QC: Security implications of just protecting the CRC needs to be further study needed

Decision: 		The document was revised to S3-193691.

S3-193691	Proposal to solve ED notes in solution#4: Zero-overhead user plane integrity protection on the link layer

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Philips International B.V.

(Replaces S3-193320)

Decision: 		The document was approved.

S3-193369	Proposed conclusion for 5G RAN connected to 5GC

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: Concern that there is a EN in solution so remove reference

QC: Ok

CMCC: Concern about the performance issues, e.g. some bearers may not require UP IP as performance is more important

QC: Proposal is as for solution 2 that UP IP is optional to use

CMCC: Propose to add a sentence on the performance

QC: Does this mean re-evaulation of existing Rel-15

Apple: Propose to remove the note - this was agreed.

DCM: Concerned about the impact on ng-eNB on this conclusion

Decision: 		The document was revised to S3-193698.

S3-193698	Proposed conclusion for 5G RAN connected to 5GC

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Qualcomm Incorporated

(Replaces S3-193369)

Decision: 		The document was approved.

S3-193393	Proposed Solution for key issue #6

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: Why only affects only ng-eNB

QC: Clarify that this is only for 5G core and not EPC - change the title to make this clear

Apple: 2nd paragraph in solution descirprion is not clear

QC: OK to remove that paragraph

Decision: 		The document was revised to S3-193692.

S3-193692	Proposed Solution for key issue #6

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Qualcomm Incorporated

(Replaces S3-193393)

Decision: 		The document was approved.

S3-193508	Evaluation for Solution#5 in UP IP

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Apple

Discussion:

CMMC: Do not agree with the figures given in evaluation

Apple: Point is that this solution cannot protect all the data packets when limited to 64 kbps

Decision: 		The document was merged.

S3-193509	Solution to key issue#5 in UP IP

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Apple

Discussion:

QC: Does not address attack - issue raised at the last meeting not resolved

DCM: Not sure that this works well as padding is at the end

Apple: Padding is not only at the end but also at the start

CMCC: Solution does not work without encryption

DCM: needs to adding padding to both beginning and end

Nok: Agree with the weaknesses of this proposal and the gain is too small

Taken offline for discussion

Decision: 		The document was revised to S3-193694.

S3-193694	Solution to key issue#5 in UP IP

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: Apple

(Replaces S3-193509)

Decision: 		The document was approved.

S3-193578	Delete the Evaluation for Solution 5 in TR 33.853

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: China Mobile

Discussion:

QC: PDCP is not aware whether traffic is IP or not IP but agree the sentence is not correct

Decision: 		The document was approved.

S3-193579	Resolving Editor’s Note in Solution #1

Type: pCR		For: (not specified)
					33.853 v0.5.0
					Source: Samsung

Discussion:

QC: Do not agree to delete EN as there are other attacks and also SIP signalling not a good example as this has its own protection

BT: Needs to consider all control plane protections and other sensitive traffic not just the listed ones

DCM: Need to clarify that all the protected data is less than 64Kbits

QC: OK if we note other data not protected

Nok: is this new capability

Samsung: exisiting capability

Decision: 		The document was revised to S3-193695.

S3-193695	Resolving Editor’s Note in Solution #1

Type: pCR		For: -
					33.853 v0.5.0
					Source: Samsung

(Replaces S3-193579)

Decision: 		The document was approved.

S3-193580	Add the Evaluation to Solution 5 in TR 33.853

Type: pCR		For: (not specified)
					33.853 v0.5.0
					Source: China Mobile

Discussion:

QC: All the parts of the evaluation need to be re-written

Nok: Need to detremoine if just header or the whole packets needs to be protected

QC: Agree with this but that there are many other issues as well

Decision: 		The document was revised to S3-193696.

S3-193696	Add the Evaluation to Solution 5 in TR 33.853

Type: pCR		For: -
					33.853 v0.5.0
					Source: China Mobile

(Replaces S3-193580)

Decision: 		The document was approved.

S3-193581	Conclusion to Key Issue #5

Type: pCR		For: (not specified)
					33.853 v0.5.0
					Source: Samsung

Discussion:

QC: does not agree with the conclusion.

Decision: 		The document was noted.

S3-193646	UPIP: update of solution #7 and addition of evaluation

Type: pCR		For: (not specified)
					33.853 v0.5.0
					Source: Ericsson

Discussion:

QC: Please the impacted elements to the evaluation and also an EN on consideration of mixed deployments where some elements are upgraded

Nok: Why do we need an indication if this mandatory for Rel-16 Ues

QC: There are multiple ways of indicating this capability of which this is one

Decision: 		The document was revised to S3-193697.

S3-193697	UPIP: update of solution #7 and addition of evaluation

Type: pCR		For: -
					33.853 v0.5.0
					Source: Ericsson

(Replaces S3-193646)

Decision: 		The document was approved.

S3-193659	pCR to TR33.853 on Migration paths for network deployment

Type: pCR		For: Approval
					33.853 v0.5.0
					Source: NTT DOCOMO INC.

Abstract:

It is proposed to add a new key issue that deals with how a network can introduce support for UP IP.

Discussion:

Apple: Don't believe the key issue is in scope of the TR

DCM: Ignoring this will mean that we will end up with something that will not be deployed

Nokia: Second point in particular had architectural impacts

DCM: Proposed text is an 'or' as there are multiple ways to do this

Huawei: Requirements are very solution specific - is it possible to reword them

BT: Supportive of this contribution as it covers the network constraints

Orange: Also supportive of this proposal

Apple: Are these really criteria for evaluation of each solution

DCM: The threats are raising security concerns

QC: Don't understand the requirements

Chair asked for who objects to the key issue: Phillips, Ericsson, Nokia & Huawei

Chair asked for who supports the key issue: Orange, BT, DCM & KPN.

Decision: 		The document was noted.

S3-193693	Draft TR 33.853

Type: draft TR		For: Approval
					33.853 v0.6.0
					Source: NTT DOCOMO INC.

Decision: 		The document was approved.

### 5.13	Study on Security Impacts of Virtualisation (FS_SIV)

S3-193318	FS_SIV TR 33.848 v030c

Type: pCR		For: Agreement
					33.848 v0.3.0
					Source: BT plc

Abstract:

Baseline TR output from conference calls for agreement to become v0.4.0.

Discussion:

Thales: Why is there a reference to SCASS

BT: Could be done by reference to specs inside 3GPP, specs outside 3GPP or by restricting the text cases

Thales: But where are the solutions?

BT: It is contribution driven and up to people to bring the solution for issue. Where it says 'inside SCASS' , it means that a text case in 3GPP is proposed outcome.

Thales: Concerned about the terms used - DSE

DCM: Propose reverting DSE chnage in 5.8.1 and adding an EN on generalising TPM/HSM and remove definition of DSE

Decision: 		The document was revised to S3-193749.

S3-193749	FS_SIV TR 33.848 v030c

Type: pCR		For: Agreement
					33.848 v0.3.0
					Source: BT plc

(Replaces S3-193318)

Decision: 		The document was approved.

S3-193328	TR 33.848 Scope Update

Type: pCR		For: Agreement
					33.848 v0.3.0
					Source: BT plc

Discussion:

DCM: Expand CNI abbreviation

Decision: 		The document was revised to S3-193751.

S3-193751	TR 33.848 Scope Update

Type: pCR		For: Agreement
					33.848 v0.3.0
					Source: BT plc

(Replaces S3-193328)

Decision: 		The document was approved.

S3-193371	TR 33.848 Security Requirements for Key Issue 17 (resubmission of S3-192559)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Decision: 		The document was merged.

S3-193372	TR 33.848 Security Threats and Requirements for Key Issue 16 (resubmission of S3-192558)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Decision: 		The document was revised to S3-193742.

S3-193742	TR 33.848 Security Threats and Requirements for Key Issue 16 (resubmission of S3-192558)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

(Replaces S3-193372)

Decision: 		The document was approved.

S3-193373	TR 33.848: Security Requirements for Key Issue 18 (resubmission of S3-192560)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Decision: 		The document was noted.

S3-193374	TR 33.848 Security Requirements for Key Issue 19 (resubmission of S3-192561)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Decision: 		The document was revised to S3-193773.

S3-193773	TR 33.848 Security Requirements for Key Issue 19 (resubmission of S3-192561)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

(Replaces S3-193374)

Decision: 		The document was noted.

S3-193375	TR 33.848 Security Threats and Requirements for Key Issue 21 (resubmission of S3-192562)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Decision: 		The document was merged.

S3-193376	TR 33.848 Annex – Virtualisation Security Questions (resubmission of S3-193089)

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: NCSC

Discussion:

Thales. Should not be a guideline that SA3 provides. Should be out of scope

DCM: make this in format of evidence to a requirement

BT: put this into document

Orange: some are SCAS related, others are more NESAS related, prefer not to put this into TC cyber

DCM: prefer to keep this here

Orange: should ask GSMA if they will deal with virtualization, send LS

Thales: ok to have this content, but this informative annex is the wrong way

Decision: 		The document was noted.

S3-193399	Threats and Requirements for Key Issue #8

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #8.

Decision: 		The document was revised to S3-193739.

S3-193739	Threats and Requirements for Key Issue #8

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193399)

Decision: 		The document was approved.

S3-193400	Threats and Requirements for Key Issue #13

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #13.

Decision: 		The document was revised to S3-193740.

S3-193740	Threats and Requirements for Key Issue #13

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193400)

Decision: 		The document was approved.

S3-193418	Threats and Requirements for Key Issue #17

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #17.

Decision: 		The document was revised to S3-193741.

S3-193741	Threats and Requirements for Key Issue #17

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193418)

Discussion:

Orange: avoid requirements on actor's, requirements on parts of infrastructure

BT: replace by 3GPP network

Orange: ok

DCM: still unclear

BT: then delete first requirement, remove "by the operator" from third  requirement

Nokia: add catalogue to third requirement

remove fist req, add catalogue to third, remove by the operator.

Decision: 		The document was revised to S3-193828.

S3-193828	Threats and Requirements for Key Issue #17

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193741)

Decision: 		The document was approved.

S3-193419	Threats and Requirements for Key Issue #18

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #18.

Decision: 		The document was revised to S3-193743.

S3-193743	Threats and Requirements for Key Issue #18

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193419)

Decision: 		The document was approved.

S3-193420	Threats and Requirements for Key Issue #20

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #20.

Decision: 		The document was revised to S3-193774.

S3-193774	Threats and Requirements for Key Issue #20

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193420)

Discussion:

DCM: protected by operators doesn't work

Orange: remove "by operator's" in requirement, no requirement on actors

update requirement

Decision: 		The document was revised to S3-193829.

S3-193829	Threats and Requirements for Key Issue #20

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193774)

Decision: 		The document was approved.

S3-193421	Threats and Requirements for Key Issue #21

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #21.

Decision: 		The document was revised to S3-193775.

S3-193775	Threats and Requirements for Key Issue #21

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193421)

Decision: 		The document was approved.

S3-193422	Threats and Requirements for Key Issue #22

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #22.

Decision: 		The document was approved.

S3-193424	Threats and Requirements for Key Issue #23

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #23.

Decision: 		The document was revised to S3-193776.

S3-193776	Threats and Requirements for Key Issue #23

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193424)

Decision: 		The document was approved.

S3-193425	Threats and Requirements for Key Issue #24

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This pCR proposes the threats and the requirements for key issue #24.

Decision: 		The document was revised to S3-193777.

S3-193777	Threats and Requirements for Key Issue #24

Type: pCR		For: Approval
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193425)

Decision: 		The document was approved.

S3-193427	Discussion on Categorization of the Key Issues

Type: pCR		For: Endorsement
					33.848 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This is a discussion paper. The proposal is to adopt the categorization of the key issues in TR 33.848, based on the proposed criterion.

Decision: 		The document was noted.

S3-193750	Draft TR 33.848

Type: draft TR		For: Approval
					33.848 v0.4.0
					Source: BT

Decision: 		The document was approved.

S3-193830	LS on virtualization security assurance

Type: LS out		For: Approval
					to GSMA SECAG
					Source: Orange

Decision: 		The document was approved.

### 5.14	Study on authentication enhancements in 5GS (FS_AUTH_ENH)

S3-193414	New key issue on PFS

Type: pCR		For: Approval
					33.819 v1.2.0
					Source: Huawei, HiSilicon

Discussion:

E//: similar proposal in 601

together with 601

Decision: 		The document was noted.

S3-193601	Existing authentication procedure lacking PFS property

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Ericsson

Discussion:

Orange: what is the problem

E//: PFS is the problem

CMCC: agree with E//, enhance the authentication, solve the issue if long term key is leaked, could revise auth protocol, or do something else to prevent long term key leakage

BT: balance between prevention, detection and response

Huawei: support E// paper, in security requirerment remove "if", or say provide forward security

DCM: need to see whether this is worth it considering the data is only protected over the air

Thales: decision already some meetings ago not to study

BT: need to document whether or not it is worth it.

Show of hands who supports this key issue: CMCC, BT, Aple, E//, Huawei, ZTE, Nokia; objects: Thales, Orange

Decision: 		The document was noted.

S3-193451	Resolving the ENs in KI#3.1

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

E//: still not aligned with the study

Huawei: explained in the paper

E//: this is not part of the objectives

Huawei: then keep first ed note

E//: ok

DCM: needs editorial rewrite

Nokia: when UE is deregistered, UE context is not cleaned up, is this the problem.

Huawei: could be a problem

Nokia : clean up on next deregistration, when there is error, there should be clean up in error case

QC: similar to Nokia, are we trying to route NAS traffic through teh home network

E//: more details on attack, bring back ed note

QC: couldn't the AMF not launch the same attack by overbilling

Nokia: add editor's note

TIM: too many additional editor's note being added, bring another proposal

Decision: 		The document was noted.

S3-193636	Discussion on the SUPI guessing attack

Type: discussion		For: Discussion
					33.846 v..
					Source: China Mobile

Decision: 		The document was noted.

S3-193637	Key issue to mitigate the SUCI guessing attacks in TR 33.846

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: China Mobile

Discussion:

DCM: how to deal with threats that we decided to be not relevant

Orange: agree, maybe remove threats and requirements

E//: same problem, keep threat, add editor's note

Nokia: can'T determine on individual SUPI

DCM: ed note: impact of threat is FFS

Orange: have threat: the attacker is able to determine whether a SUPI belongs to a given network, + ed note

ed note, Orange's sentence on threat, remove requirements - FFS

Decision: 		The document was revised to S3-193810.

S3-193810	Key issue to mitigate the SUCI guessing attacks in TR 33.846

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: China Mobile

(Replaces S3-193637)

Decision: 		The document was approved.

S3-193319	33:846: mitigation against linkability attack

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: THALES

Abstract:

33.846: mitigation against linkability attack

Discussion:

BT: does this need a new USIM, or can this be done with OTA

Thales: not OTA

BT: document that this means change of USIM

Orange: this solution requires change of the USIM

Nokia: step 5, and step 7 are authentication procedures, how is the USIM to know that ti needs to resynch failure

Thales: in case synch error, it will respond with a random, then the network wills start resynch procedure, USIM will know that is required

Nokia: there may be new threats

Thales: ok to add ed note

Nokia: ed note threat analysis for authentication resynch will need to be done.

Huawei: step 3 need a unified failure cause

Thales: UICC will see different causes, but for network and ME it will not be detected

Huawei: ed note: whether unified failure cause  is FFS

QC: if SUPI is known, this doesn't work against active attack. now this requires two passes for sync, after 3 failures the UE will block the network

sentence in evaluation, ed note on threat, on unified failure cause, offline for QC attack

Decision: 		The document was revised to S3-193811.

S3-193811	33:846: mitigation against linkability attack

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: THALES

(Replaces S3-193319)

Decision: 		The document was approved.

S3-193356	Using MACS to provide freshness for the protection of SQN during a re-synchronisation procedure in AKA

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-192921)

Discussion:

Orange: add to evaluation that USIM needs to be changed, no impact of visited network

Huawei:have to assume that MAC-S is trusted?

QC: just one extra input to AK

Thales: think solution is correct

ZTE: there may be problem of performance

Orange: no issue

Nokia: how feasible is algorithm update over OTA

Orange: added this sentence in eval

eval: this solution requires changing the USIM

Decision: 		The document was revised to S3-193812.

S3-193812	Using MACS to provide freshness for the protection of SQN during a re-synchronisation procedure in AKA

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193356)

Decision: 		The document was approved.

S3-193450	New solution for linkability attack

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

Orange: add two sentences: visited network is impacted, and not R15 compatible to evaluation

E//: needs  public key, another UE could impersonate the response, so not clear where failure message is coming from, unclear whether ME or USIM do calculation, impact of MAC failure message size is FFS: add ed note regarding this

BT: if SUCI protection is not there, then there is dependency

Thales: solution relies on availability of key  for SUCI, add ed note.

sentence in evaluation, (Orange, Thales), 3 ed notes (E//)

Decision: 		The document was revised to S3-193813.

S3-193813	New solution for linkability attack

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

(Replaces S3-193450)

Decision: 		The document was approved.

S3-193515	Structure RAND for authentication

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: ZTE Corporation

Discussion:

E//: as instances come and go in virtual environment start-up time is FFS

Orange: don't understand what are threats and requirements to be covered, so not include the solution

DCM: how does home network know that ME is capable of this solution

ZTE: doesn't know.

Decision: 		The document was noted.

S3-193635	Clarification to the usage of Kausf  for solution #2.2  in TR 33.846

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: China Mobile

Discussion:

Nokia: how can KAUSF work on the failed procedure.

Orange: if there is no previous KAUSF, then there is no protection, add ed note: security of using KAUSF=0 is FFS

Decision: 		The document was noted.

S3-193460	Address the EN and add evaluation in solution 2.3

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

E//: ed note in evalution: NAS procedure impact is FFS

Orange: statement: this solution impact the visisted network

ed note on NAS preocedure, sentence on visited network impact

Decision: 		The document was revised to S3-193814.

S3-193814	Address the EN and add evaluation in solution 2.3

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

(Replaces S3-193460)

Decision: 		The document was approved.

S3-193452	New solution for removing the authentication result from the UDM

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193459	solution on AUTS derivation to protect SQN

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

QC: there is no non-malleable block cipher for 48 bit,

Orange: impact on Milenage and TUAK

TIM: also new UICCs needs to be written

QC: no arrow down form AK in diagram

replace eval text, solution requires a major change of auth algorithms, and the examples defined in 3GPP (Milenage and TUAK), solution requires change of USIM

Decision: 		The document was revised to S3-193815.

S3-193815	solution on AUTS derivation to protect SQN

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: Huawei, Hisilicon

(Replaces S3-193459)

Decision: 		The document was approved.

S3-193516	Conclusion on linkability issues

Type: pCR		For: Approval
					33.846 v0.3.0
					Source: ZTE Corporation

Discussion:

Thales: too early for conclusions

Decision: 		The document was noted.

S3-193850	draft TR 33.846

Type: draft TR		For: Approval
					33.846 v0.4.0
					Source: Ericsson

Decision: 		The document was approved.

### 5.15	Security for NR Integrated Access and Backhaul

#### 5.15.1	Work Item (NR_IAB_Sec)

S3-193669	LS on the IAB-indication to core network

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: R3-194787

Decision: 		The document was noted.

S3-193564	[Draft CR]Solution for IAB Architecture

Type: draftCR		For: (not specified)
					33.501 v16.0.0
					Source: Samsung

Discussion:

BT: the IAB node needs to be protected like a gNB

Samsung: add in next meeting

together with 611

Decision: 		The document was revised to S3-193808.

S3-193808	[Draft CR]Solution for IAB Architecture

Type: draftCR		For: -
					33.501 v16.0.0
					Source: Samsung

(Replaces S3-193564)

Discussion:

will be living document for next meeting

Orange: needs to remain as skeleton only

Decision: 		The document was approved.

S3-193611	DraftCR –Security handling for IAB

Type: draftCR		For: Approval
					33.501 v16.0.0
					Source: Ericsson

Discussion:

QC: prefer Samsung proposal

Nokia: need some merger, start with Samsung proposal

Nokia: there is no IAB network, it is an IAB node

QC: diagram shows IAB architecture, not security architecture

QC: approach is model after SNPN, disagree with that approach, prefer the main clauses of 501 and 401

offline, merge into 808

Todor: no coontent from was taken from 611

Samsung: correct

Decision: 		The document was merged.

#### 5.15.2	Study (FS_NR_IAB_Sec)

S3-193565	Updates to Solution #2.1 on MT functionality

Type: pCR		For: (not specified)
					33.824 v0.4.0
					Source: Samsung

Discussion:

E//: is this copy paste from SA2

Samsung: yes

Huawei: Note how the IAB node… within UICC is not used,

Thales: why is the note there at all

Nokia: in 23.501, IoB node has gNB functionality, could be many platforms

QC: comment on paragraph above node, bacuse with EPC, there is no EAP support. should be treated as UE in 33.501. prefer the defined primary authentication methods.

Samsung: may be true for one kind of implementation

DCM: stick to existing authentication mechanisms

Nokia: need to be flexible,

Huawei: maybe treat IAB as NPN UE

Samsung: restrict option for EPC.

Decision: 		The document was revised to S3-193788.

S3-193788	Updates to Solution #2.1 on MT functionality

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193565)

Discussion:

Thales: remove the note, sentence above should be removed, if it is a PLMN, it should AKA

Samsung ok with removing note

E//: but keep EAP TLS above

Orange: against

supporting the note and para above: Nokia, E//, Huawei, Samsung; objecting: BT, Thales, Orange

Orange: removed text also related to teh discussion

Decision: 		The document was revised to S3-193851.

S3-193851	Updates to Solution #2.1 on MT functionality

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193788)

Discussion:

BT: without Note, it is ok

QC: ok with this, but add *subscription* credential in yellow marked text.

Decision: 		The document was revised to S3-193855.

S3-193855	Updates to Solution #2.1 on MT functionality

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193851)

Decision: 		The document was noted.

S3-193567	Evaluation of solution #2.1

Type: pCR		For: (not specified)
					33.824 v0.4.0
					Source: Samsung

Discussion:

DCM: ok, problem is only whether we restrict further

Decision: 		The document was revised to S3-193789.

S3-193789	Evaluation of solution #2.1

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193567)

Decision: 		The document was noted.

S3-193464	Evaluation of solution#2.1

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Huawei, Hisilicon

Discussion:

E//: ast sentence should go, looks like solution.

Decision: 		The document was merged.

S3-193568	Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (MT)

Type: pCR		For: (not specified)
					33.824 v0.4.0
					Source: Samsung

Discussion:

QC: solution 2.1 is open

DCM: be more specific what will be normative

Nokia: fine with the contribution as is

Chair only Note and para above is open of 2.1

Thales: same problem, keep open

Decision: 		The document was revised to S3-193790.

S3-193790	Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (MT)

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193568)

Decision: 		The document was noted.

S3-193357	F1 security establishment

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Qualcomm Incorporated

Discussion:

Nokia: DU setup is basically is a wireline interface. Defined solution may not scale

DCM: VPN is used in many corporates at scale

QC: this is a solution proposal, so what is the problem

Huawei: ed note what is the difference between wireless and wireline.

Nokia: solution can go in, but need to capture in evaluation

Decision: 		The document was approved.

S3-193358	Evaluation on Solution #3.1: F1 security context establishment

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Qualcomm Incorporated, Ericsson

Decision: 		The document was noted.

S3-193791	Evaluation on Solution #3.1: F1 security context establishment

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Qualcomm Incorporated, Ericsson

Decision: 		The document was withdrawn.

S3-193572	Evaluation of solution #3.1

Type: pCR		For: (not specified)
					33.824 v0.4.0
					Source: Samsung

Discussion:

QC: unclear where the requirement comes from.

E//: support QC, interface security should look the same for wireless and wireline

Nokia: prefer 572

QC: why one credential, how do you attach to management server.

Decision: 		The document was noted.

S3-193359	Conclusion on KI #4.1: F1 interface security

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Qualcomm Incorporated, Ericsson

Decision: 		The document was merged.

S3-193573	Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU)

Type: pCR		For: (not specified)
					33.824 v0.4.0
					Source: Samsung

Discussion:

DCM: think about platform security keys

Samsung: IAB has gNB requirements, so no platform security

Decision: 		The document was revised to S3-193792.

S3-193792	Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU)

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193573)

Discussion:

orange: from where to where IPsec is established

QC: delete last three sentences

Decision: 		The document was revised to S3-193856.

S3-193856	Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU)

Type: pCR		For: -
					33.824 v0.4.0
					Source: Samsung

(Replaces S3-193792)

Discussion:

E//: not ok

Decision: 		The document was noted.

S3-193462	Discussion on binding between USIM/UICC and IAB-node

Type: pCR		For: Discussion
					33.824 v0.4.0
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

S3-193463	Key issue on removal of UICC card in IAB node

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Huawei, Hisilicon

Discussion:

Nokia: agree with first requirement, but second requirement assumes removable UICC, should go away

BT: support second requirement staying, to remove

Huawei: competing 654

together with 654

Decision: 		The document was revised to S3-193793.

S3-193793	Key issue on removal of UICC card in IAB node

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Huawei, Hisilicon

(Replaces S3-193463)

Discussion:

BT: agrees

Orange: don't include primary authenticaiton

Decision: 		The document was revised to S3-193852.

S3-193852	Key issue on removal of UICC card in IAB node

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: Huawei, Hisilicon

(Replaces S3-193793)

Decision: 		The document was noted.

S3-193654	UICC removal from IAB-node

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: THALES

Abstract:

New Key Issue related to UICC removal from IAB-node

Discussion:

DCM: a bit like a solution, but threats are valid

QC: question whether threats exist in case same key is used

E//: first sentence sounds like IAB always conatins UICC

DCM: if no UICC is used, then key could be stolen in a different way

offline try to merge into 793

Decision: 		The document was merged.

S3-193519	New key issue on protection against Man-in-the-Middle false IAB node attacks

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: ZTE Corporation

Discussion:

Huawei: merge with 793

E//: not related, one has to break F1 security

Nokia: disagree with tdoc

QC: disagree with tdoc

Decision: 		The document was noted.

S3-193521	Editorial correction to TR 33.824

Type: pCR		For: Approval
					33.824 v0.4.0
					Source: ZTE Corporation

Decision: 		The document was approved.

S3-193807	draft TR 33.824

Type: draft TR		For: Approval
					33.824 v0.5.0
					Source: Samsung

Decision: 		The document was approved.

### 5.16	Study on Security Aspects of 3GPP support for Advanced V2X Services (FS_eV2X_Sec)

S3-193667	Reply LS on protection of PC5-RRC messages for sidelink unicast communication

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: R2-1911863

Decision: 		The document was noted.

S3-193665	LS on NR V2X Security for user plane data and PDCP SN size

Type: LS in		For: (not specified)
					Original outgoing LS: -, to -, cc -
					Source: R2-1911681

Decision: 		The document was replied to in S3-193854.

S3-193854	Reply to: LS on NR V2X Security for user plane data and PDCP SN size

Type: LS out		For: approval
					to RAN2
					Source: LG

Decision: 		The document was approved.

S3-193350	Discussion for a response LS on NR V2X Security for user plane data and PDCP SN size

Type: discussion		For: Endorsement
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

S3-193426	[draft] reply LS on NR V2X Security for user plane data and PDCP SN size

Type: LS out		For: (not specified)
					to RAN2
					Source: LG Electronics France

Decision: 		The document was revised to S3-193778.

S3-193778	[draft] reply LS on NR V2X Security for user plane data and PDCP SN size

Type: LS out		For: -
					to RAN2
					Source: LG Electronics France

(Replaces S3-193426)

Discussion:

QC: include key ID, confirm LCID are input to keystreams, in Q2, add key ID, 12 bit requires faster rekeying.

Decision: 		The document was revised to S3-193854.

S3-193304	33.836 - solution #1 update

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: Further explanation on the protection of exchanged messages is needed.

Decision: 		The document was approved.

S3-193312	33.836 – evaluation for the Solution #1

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation for the Solution #1.

Discussion:

QC: SA2 link layer is two roundtrip, this is 3 roundtrips, analysis needs to be done

IDCC: ed note how to address mismatch with SA?2

QC: ed note: securtiy comparison with SA2 solution for link layer update is FFS

DCM: remove and/or

ed note and and/or

Decision: 		The document was revised to S3-193795.

S3-193795	33.836 – evaluation for the Solution #1

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

(Replaces S3-193312)

Decision: 		The document was approved.

S3-193307	TR 33.836 solution #4 update

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: Adherence of this solution to 5G ProSe Architecture and Security to be clarified.

Decision: 		The document was approved.

S3-193316	TR 33.836 – evaluation of the solution #4

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation for the solution #4.

Discussion:

QC: this requires partial ciphering to direct SMC command, adds complexity: add editors note: compleity of adding partial ciphering to the direct security mode command compared to the benefit of the simultaneous updates is FFS.

Ed note

Decision: 		The document was revised to S3-193796.

S3-193796	TR 33.836 – evaluation of the solution #4

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

(Replaces S3-193316)

Decision: 		The document was approved.

S3-193345	Providing some analysis to solution #9 in TR 33.836

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

Intel: disagree with all of the evaluation: no regulatory requirements, credential provsioning KMS is responsible, there are options available, what has privacy to do with credentials, can be achieved by changing session keys, credentials are never exposed.

Decision: 		The document was noted.

S3-193429	Resolution of Editors notes for Key issue 1 and 8

Type: pCR		For: (not specified)
					33.836 v0.3.0
					Source: LG Electronics France

Decision: 		The document was approved.

S3-193305	TR 33.836 - update for solution #2

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: Adherence of this solution to 5G ProSe Architecture and Security to be clarified.

Discussion:

LG: this doesn't adhere to SA2 TS, There only initiating UE sends TCR, here the responding also sends TCR

QC: agree, but part of the solution, so put this into the evaluation. Add ed note

don't add note

Decision: 		The document was revised to S3-193797.

S3-193797	TR 33.836 - update for solution #2

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

(Replaces S3-193305)

Decision: 		The document was approved.

S3-193314	TR 33.836 - evaluation for the solution #2

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation for the solution #2.

Discussion:

QC: ed note, similar to other IDCC contribition. "Justification for the difference to SA2 call flow is FFS"

QC: remove last sentence

ed note, remove last sentence

Decision: 		The document was revised to S3-193798.

S3-193798	TR 33.836 - evaluation for the solution #2

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

(Replaces S3-193314)

Decision: 		The document was approved.

S3-193306	TR 33.836 - solution #3 update

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution addresses the following Editor Notes:

•	Editor’s Note: Adherence of this solution to 5G ProSe Architecture and Security to be clarified.

•	Editor’s Note: Differences between this solution and Solution #2 to be highlighted.

Decision: 		The document was approved.

S3-193315	TR 33.836 - evaluation for the solution #3

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: InterDigital Communications

Abstract:

This contribution adds evaluation for the solution #3.

Decision: 		The document was approved.

S3-193347	Resolving the editor’s note on RRC security establishment in solution #8

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

IDCC: Why does PC5 use the same algorithms

QC: because it has the same termination point

IDCC: make this clear

DCM: make note to shall

QC: ok

IDCC want reasoning

QC: part of evaluation

IDCC: keep as is.

No changes

Decision: 		The document was approved.

S3-193348	Resolving the editor’s note on operator knowledge of keys in solution #8

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

Intel: doesn't resolve teh issue

QC: if no confidentiality is provided, then LI is solved.

No changes

Decision: 		The document was approved.

S3-193346	Providing some analysis to solution #8 in TR 33.836

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

Intel: new key issue related to this

DCM: ed note (conditional: on accepting new key issue) evaluation regarding new key issue is FFS

IDCC: simiultaneous changes are needed, so this is against SA2

QC: this can only happen when you change the certificate when trying to change ID, so disagree with this statement

IDCC: editors note:conformance to SA2 TS23.287 clause 5.6.1.1 is FFS

LG: no relationship. QC is correct

IDCC: shouldn't be decoupled

DCM add ed note,

QC: privacy on two levels, application and on link. try to work offline on this as alternative to ed note

conditional ed note, resolution or ed note, clause heading 6.8.3

conditional ed note not necessary based on outcome of 520

Decision: 		The document was revised to S3-193799.

S3-193799	Providing some analysis to solution #8 in TR 33.836

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193346)

Decision: 		The document was approved.

S3-193349	Proposed solution for protecting the V2X PC5 traffic at the PDCP

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

LG: this is unicast, so is key ID required?

QC: in Uu, handover is used, do break then make, here you can whange key in active communication

no changes

Decision: 		The document was approved.

S3-193351	Solution for the activation of user plane security in NR PC5 unicast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: reuse activation, why define a different policy for UP ini PC5?

QC: add editor's note: details on the user plane activation policy is FFS

ed note

Decision: 		The document was revised to S3-193800.

S3-193800	Solution for the activation of user plane security in NR PC5 unicast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193351)

Decision: 		The document was approved.

S3-193454	New solution for PC5 layer key derivation using the 5G network keys

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

QC: how does the application derive the PC5 link key?

Huawei: several assumptions: UE is connected to network, has key to derive, not out of coverage

Intel: agree wirth QC

QC: accept the limitiation, why do this if there is a solution for out of coverage.Ed note: what happens on UE out of coverage?

Huawei: 6.8.2 gives this.limitation, and also the benefit af allowing the operator

QC: ed note: how the UE establishes PC5 security when out of coverage is FFS, what happens when the UEs are on different PLMNs

Huawei: described in step 2

QC, only one ed note is ok, then

QC: step 7, how is senidng the PC5 key securedin case of different PLMNs

ed note on out of coverage

Decision: 		The document was revised to S3-193801.

S3-193801	New solution for PC5 layer key derivation using the 5G network keys

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, Hisilicon

(Replaces S3-193454)

Decision: 		The document was approved.

S3-193352	Discussion on protection of messages for V2X PC5 unicast

Type: discussion		For: Endorsement
					Source: Qualcomm Incorporated

Discussion:

IDCC: support LS

QC one or two LSs.

Send LS based on the discussion paper

Decision: 		The document was noted.

S3-193342	Solution for privacy of link layer IDs in broadcast and groupcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

S3-193522	A solution against V2X UE tracking based on PC5 identifiers

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Ericsson LM

Discussion:

Lenovo: this is about L2 destiantion Ids?

QC: source Ids

QC: ok to approve this, and note 342

IDCC: what about privacy of groupcast? Put ed note, privacy of gorupcast is FFS

QC: that is attempted to solve in Lenovo contribution

IDCC: but that doesn't change the group ID

Lenovo: it does

no changes

Decision: 		The document was approved.

S3-193343	Conclusion on privacy for groupcast and broadcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

QC update to point to 522 soltuion

Huawei: is privacy only based on source id only, make this specific

add "for source layer 2 id"

Decision: 		The document was revised to S3-193803.

S3-193803	Conclusion on privacy for groupcast and broadcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193343)

Decision: 		The document was approved.

S3-193557	Identifier conversion in groupcast communication

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Lenovo, Motorola Mobility

Abstract:

This document addresses Key Issue #4 of TR 33.836.

Discussion:

Huawei: second last sentence first para, why in oder not to signal

Lenovo: so in order not to update

Huawei: so interval dT is fixed.problem with time synchronization

QC: remove evaluation line, basically application layer manages time interval

IDCC: use time for synchronization, use unprotected SIB or unprotected time source. add ed note on how to deal with spoofed time source is ffs

DCM: changing of group id may identify group members

BT: how to deal with impact on availability is ffs

remove evaluation, ednote from IDCC and  BT

Decision: 		The document was revised to S3-193804.

S3-193804	Identifier conversion in groupcast communication

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Lenovo, Motorola Mobility

(Replaces S3-193557)

Decision: 		The document was approved.

S3-193556	Update of solution #6

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Lenovo, Motorola Mobility

Abstract:

This document provides an update of solution #6 of TR 33.836.

Decision: 		The document was noted.

S3-193344	Proposed conclusion of security for groupcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

Huawei: if there is a key issue groupcast, then there should also be a key issue for broadcast

Lenovo: agree for broadcast, disagree with groupcast conclusion

QC: who supports work on groupcast.

Supporting no normative work for groupcast: QC, ZTE, LG, Huawei; objecting, Lenovo, IDCC

conclusion only about broadcast, include rationale

Decision: 		The document was revised to S3-193805.

S3-193805	Proposed conclusion of security for groupcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193344)

Discussion:

QC: keep both group and broadcast., there is a new split key issue on groupcast and broadcast

Huawei: no normative work for groupcast *for R16*

Huawei: change title to include broadcast

Decision: 		The document was revised to S3-193853.

S3-193853	Proposed conclusion of security for groupcast

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Qualcomm Incorporated

(Replaces S3-193805)

Decision: 		The document was approved.

S3-193430	 Resolution of Editors notes for Key issue 6

Type: pCR		For: (not specified)
					33.836 v0.3.0
					Source: LG Electronics France

Decision: 		The document was approved.

S3-193321	Adding evaluation to solution #7

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Decision: 		The document was approved.

S3-193323	Conclsion to Key Issue #7

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Decision: 		The document was approved.

S3-193327	New solution to minimize the impact on the application layer communication

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Discussion:

E//: how does it work while maintaining unicast connections

Huawei: see figure

no changes

Decision: 		The document was approved.

S3-193431	Conclusion for Key issue 10

Type: pCR		For: (not specified)
					33.836 v0.3.0
					Source: LG Electronics France

Decision: 		The document was revised to S3-193779.

S3-193779	Conclusion for Key issue 10

Type: pCR		For: -
					33.836 v0.3.0
					Source: LG Electronics France

(Replaces S3-193431)

Discussion:

Huawei: postpone to next meeting, SA still has some concern, also procedure defined in SA6

Decision: 		The document was noted.

S3-193453	New solution for UP security policy handling for PC5 and Uu interface

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, Hisilicon

Discussion:

QC: policy for PC5 and Uu should be sufficient, local policy should be enough.

Huawei: SA6 has defined a procedure for switching.

IDCC: path switching is part of 5G ProSe, wait for 5G ProSe

Huawei: TR is just study, so put it here as  key issue, and study next meeting

LG: include ed note to say we wait for SA2

Nokia: prefer to postpone this and wait for SA2 to define this.

Huawei: ok with ed note.

Decision: 		The document was noted.

S3-193324	New Key Issue on Security of broadcast eV2X messages over PC5

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Discussion:

E//: don't support this key issue

LG: also doesn'T support this

DCM: give reasoning, ok with no requirement

BT: problem with not including it, integrity is important

QC: on unicast the bearer may also be malicious

QC: inlcude rationale from discussion document in conclusion

offline whether required,

Decision: 		The document was revised to S3-193806.

S3-193806	New Key Issue on Security of broadcast eV2X messages over PC5

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

(Replaces S3-193324)

Decision: 		The document was approved.

S3-193436	Minimizing dependency on application layer security

Type: pCR		For: (not specified)
					33.836 v0.3.0
					Source: Intel

Discussion:

QC: privacy requirements will be driven by application

E//: supporting QC

Decision: 		The document was noted.

S3-193520	New key issue on security for multi-USIM communication over UU and PC5

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: ZTE Corporation

Discussion:

IDCC: there is no security component, not SA3 issue, threats missing

Nokia: this is not scope in our group

QC: not in scope here

Decision: 		The document was noted.

S3-193326	Editorial corrections for eV2X TR 33.836

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, HiSilicon

Discussion:

Nokia: sentence on broadcast is not editorial

QC: agree to this sentence

no changes

Decision: 		The document was approved.

S3-193495	New solution on KI #9 minimizing the impact of privacy protection mechanism in the application layer communication

Type: pCR		For: Approval
					33.836 v0.3.0
					Source: Huawei, Hisilicon

Decision: 		The document was withdrawn.

S3-193794	draft TR 33.836

Type: draft TR		For: Approval
					33.836 v0.4.0
					Source: LG

Decision: 		The document was approved.

S3-193802	LS on PC5S and PC5 RRC unicast message protection

Type: LS out		For: Approval
					to SA2,RAN2,CT1
					Source: Qualcomm

Decision: 		The document was approved.

### 5.17	Study on storage and transport of the security parameters in a 5GC, that are used by the ARPF for Authentication (FS_5GC_sec_storage_transport)

S3-193534	KI on Protection of authentication subscription data stored in UDR

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

KPN: Key issue is above transport and storage - better to split this into two key issues as only last sentence is about transfer

Thales: modifcation of data need to authorised as well

DCM: Needs security threat on confidentiality of data and also a threat on copying data to another subcription plus associated requirement

TI: Need to understand what is the sensitive data and subscription authentication data

KPN: Mention of transfer should also be deleted

Vdf: Support DCM new threat

Decision: 		The document was noted.

S3-193535	KI on Separation of authentication subscription data from subscription data

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

CMCC: More of an implementation issue rather than a security issue as too difficult to separate them

KPN: Support this key issue

BT: support this key issue - modify to make it "stored and managed" separately

CMMC: Feel that separtion is a soluiton and not a requirement

BT: Biggest threat is people who are runnung the the network should not get access to the more sensitive data

ID: Separate treatment of data is common

DCM: Reformulate the requirement to compartmentalize the the data rather than store it separately

TI: Propose EN that the used terms need to be defined

Hua: Support CMCC comments

Decision: 		The document was revised to S3-193747.

S3-193747	KI on Separation of authentication subscription data from subscription data

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

(Replaces S3-193535)

Decision: 		The document was approved.

S3-193746	KI on Protection of authentication subscription data stored in UDR

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

S3-193566	Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0

Type: draft TR		For: Approval
					33.845 v0.0.0
					Source: VODAFONE Group Plc

Abstract:

base document with content agreed in S3-192125

Discussion:

Introduction removed

Duplicate clauses removed

Decision: 		The document was revised to S3-193744.

S3-193744	Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0

Type: draft TR		For: Approval
					33.845 v0.0.0
					Source: VODAFONE Group Plc

(Replaces S3-193566)

Decision: 		The document was approved.

S3-193630	ARPF Deployment models

Type: pCR		For: Approval
					33.845 v0.0.0
					Source: Ericsson

Abstract:

This pCR proposes content for the TR 33.845 to describe the different deployment options for ARPF in relation to UDR, UDM and HSS/AuC

Discussion:

Nok: Do not think that the level of detail is needed for this TR

BT: Think it is useful so suggets putting in an Annex

KPN: Agree that is should go there

TI: Some content should go in but does not fit with skeleton

Decision: 		The document was noted.

S3-193634	Security Parameter Storage

Type: pCR		For: Approval
					33.845 v0.0.0
					Source: Ericsson

Abstract:

This pCR proposes content for the TR 33.845 to describe the different models for security parameter storage.

Discussion:

KPN: Confused about the limiting the storage in this contribution - move text in scenario C to model B and remove text about model A as out of scope

TI: realtionship between the various parameters need to be clarified

Nok: Clause 4.2.1 should not be included as it is the wrong part of TR

DCM: Part of 4.2.1 is wrong

Decision: 		The document was noted.

S3-193647	Privacy Aspects of ARPF deployment

Type: pCR		For: Approval
					33.845 v0.0.0
					Source: Ericsson

Abstract:

This pCR proposes content for the TR 33.845 to describe the security assumptions related to the support of the subscription privacy feature in relation to the deployment of ARPF and UDR.

Discussion:

Nok: remove last paragraph and shift 3rd paragraph to become 2nd paragraph

Decision: 		The document was revised to S3-193748.

S3-193748	Privacy Aspects of ARPF deployment

Type: pCR		For: Approval
					33.845 v0.0.0
					Source: Ericsson

(Replaces S3-193647)

Decision: 		The document was approved.

S3-193745	draft TR 33.845

Type: draft TR		For: Approval
					33.845 v0.1.0
					Source: Samsung

Decision: 		The document was approved.

## 6	Any Other Business

S3-193370	Security aspects of RLOS

Type: draftCR		For: Endorsement
					33.501 v16.0.0
					Source: Qualcomm Incorporated

Discussion:

QC: should be for information

DCM: out of scope

Decision: 		The document was noted.

S3-193455	New SID: Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC

Type: SID new		For: Approval
					Source: China Unicom, CAICT, China Telecom, Huawei, Hisilicon, ZTE

Discussion:

Orange: simplify objective by pointing to SA2 work,

Samsung: split this into two

Thales: don't know for UICC

QC: this is release 17, so not really for endorsement

CMCC: are there objections

QC: process needs to be adhered

Nokia: too early to start on this, wait more for SA2 progress

QC: have similar comment

feedback given, to be noted

Decision: 		The document was noted.

S3-193458	Discussion on potential solutions to security handling of AMF reallocation

Type: discussion		For: Endorsement
					33.501 v..
					Source: Huawei, Hisilicon

Discussion:

DCM: not in scope

Nokia: waitinig for reply LS from SA2

Huawei: decision is to do nothing in SA2

Decision: 		The document was noted.

S3-193505	Discussion on 5G UE privacy when connecting to EPC

Type: discussion		For: Agreement
					Source: Apple

Discussion:

DCM: if this is proposal for a new study item, the in scope, technically it will be very difficult

Nokia: will be very difficult to do this in all networks

QC: in 5G study, we looked at this in detail, but didn't get a working solution

Huawei: agree

Decision: 		The document was noted.

S3-193510	EAP-AKA privacy enhancement in non-3GPP access to EPS

Type: SID new		For: Approval
					Source: Apple

Decision: 		The document was revised to S3-193629.

S3-193623	TLS certificates for SBA: profile and provisioning

Type: discussion		For: Endorsement
					Source: Ericsson

Discussion:

CMCC: not to include provisioning in study

Huawei: without provisioning, the solution is already in eSBA

QC: is this also for RAN, or only core?

E//: this is core netowrk only

QC: useful to include gNB in scope for certificate enrolment

Nokia: profile needs to be defined, especially in virtual environment, profile in R16, provisioning in R17

BT: revocation and deprovisioning needs to included

Decision: 		The document was noted.

S3-193629	New SID on EPS AKA and EAP-AKA privacy enhancement in EPS

Type: SID new		For: Approval
					Source: Apple, Google, AT&T, Verizon UK Ltd, Accuris Networks, Charter Communications, Cablelabs, Article19, Sprint, Comcast, Broadcom

(Replaces S3-193510)

Discussion:

CMCC: does not support this

QC: is for R17, was already studied in R15, so why should be possible now

Nokia: shouldn't be started because it would waste time

BT: explicitly look at migration strategy and bid down prevention

Orange: in places talking about authentication method, talk about the access, from where to where should the IMSI be concealed.

Decision: 		The document was noted.

S3-193671	New WID: Security Aspects of PARLOS

Type: WID new		For: Discussion
					Source: SPRINT Corporation

Discussion:

orange: normative or informative

Sprint: no decision in the WID, but in TR

agreement: whether this is captured in an informative or normative annex is FFS.

E//: also wants to support, comment acronym

Supporting: E//; Samsung, T-Mobile

Decision: 		The document was endorsed.

S3-193672	New WID for User Plane Gateway Function for the Inter-PLMN Security

Type: WID new		For: Agreement
					Source: Juniper Networks

Discussion:

There was discussion on the location of the UPF, i.e whether it is co-located etc.

DCM: had discussion previously and concluded that it was different function as it doe not terminate the interface - coloaction is then a deployment option

Chair: ask if anyone is against the split

BT: would rather keep them together

Chair: explained that it was due to part of the work following SA2 and UP gateway part being standalone

Juniper: It is a clean separation of the work item

It is generally agreed that should be a split

Taken offline for discussion of an objective

Decision: 		The document was revised to S3-193718.

S3-193718	New WID for User Plane Gateway Function for the Inter-PLMN Security

Type: WID new		For: Agreement
					Source: Juniper Networks

(Replaces S3-193672)

Discussion:

Needs to be re-submitted fro agreement in Reno meeting

Decision: 		The document was endorsed.

S3-193839	SA3 meeting calendar

Type: other		For: Information
					Source: WG Chair

Discussion:

IDCC: SA#100 cadjacent to independence day weekend

Decision: 		The document was noted.

S3-193840	draft agenda SA3#97

Type: other		For: Information
					Source: WG Chair

Decision: 		The document was noted.

## 7	Close

The Chair thanks host, CF3, Huawei for the social event, vice chairmen and everyone for the hard work

## Annex A: List of contribution documents

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| S3-193300 | Agenda | WG Chair | revised |  | S3-193681 |
| S3-193301 | TR 33.813 - update for solution #11 | InterDigital Communications | approved |  |  |
| S3-193302 | TR 33.819 – update of DH based solution for CAG ID privacy | InterDigital Communications | approved |  |  |
| S3-193303 | TR 33.819 – update of Hash based solution for CAG ID privacy | InterDigital Communications | approved |  |  |
| S3-193304 | 33.836 - solution #1 update | InterDigital Communications | approved |  |  |
| S3-193305 | TR 33.836 - update for solution #2 | InterDigital Communications | revised |  | S3-193797 |
| S3-193306 | TR 33.836 - solution #3 update | InterDigital Communications | approved |  |  |
| S3-193307 | TR 33.836 solution #4 update | InterDigital Communications | approved |  |  |
| S3-193308 | TR 33.819 – review and comparative evaluation of solution for CAG ID privacy | InterDigital Communications | noted |  |  |
| S3-193309 | TR 33.813 - update for the evaluation for solution #11 | InterDigital Communications | revised |  | S3-193823 |
| S3-193310 | TR 33.819 – evaluation of DH based solution for CAG ID privacy | InterDigital Communications | revised |  | S3-193826 |
| S3-193311 | TR 33.819 – evaluation of Hash based solution for CAG ID privacy | InterDigital Communications | revised |  | S3-193827 |
| S3-193312 | 33.836 – evaluation for the Solution #1 | InterDigital Communications | revised |  | S3-193795 |
| S3-193313 | Resolving EN in TR33.855 6.18 N9 NDS/IP | Juniper Networks, NTT DoCoMo, Ericsson | revised |  | S3-193728 |
| S3-193314 | TR 33.836 - evaluation for the solution #2 | InterDigital Communications | revised |  | S3-193798 |
| S3-193315 | TR 33.836 - evaluation for the solution #3 | InterDigital Communications | approved |  |  |
| S3-193316 | TR 33.836 – evaluation of the solution #4 | InterDigital Communications | revised |  | S3-193796 |
| S3-193317 | TR 33.813 – review and comparative evaluation of solution for NSSAI privacy | InterDigital Communications | noted |  |  |
| S3-193318 | FS_SIV TR 33.848 v030c | BT plc | revised |  | S3-193749 |
| S3-193319 | 33:846: mitigation against linkability attack | THALES | revised |  | S3-193811 |
| S3-193320 | Proposal to solve ED notes in solution#4: Zero-overhead user plane integrity protection on the link layer | Philips International B.V. | revised |  | S3-193691 |
| S3-193321 | Adding evaluation to solution #7 | Huawei, HiSilicon | approved |  |  |
| S3-193322 | Editorial corrections for TR 33.819 | Huawei, HiSilicon | approved |  |  |
| S3-193323 | Conclsion to Key Issue #7 | Huawei, HiSilicon | approved |  |  |
| S3-193324 | New Key Issue on Security of broadcast eV2X messages over PC5 | Huawei, HiSilicon | revised |  | S3-193806 |
| S3-193325 | Evaluation of solutions addressing Key Issue #6.2 | Huawei, HiSilicon | noted |  |  |
| S3-193326 | Editorial corrections for eV2X TR 33.836 | Huawei, HiSilicon | approved |  |  |
| S3-193327 | New solution to minimize the impact on the application layer communication | Huawei, HiSilicon | approved |  |  |
| S3-193328 | TR 33.848 Scope Update | BT plc | revised |  | S3-193751 |
| S3-193329 | Resolve EN on signaling details of how UE hands over to FBS | Huawei, HiSilicon | approved |  |  |
| S3-193330 | Resolve second and third EN in Solution #6 | Huawei, HiSilicon | revised |  | S3-193757 |
| S3-193331 | Solution#4: resolving EN network verification of hashes of MIB/SIBs | Huawei, HiSilicon | approved |  |  |
| S3-193332 | Solution#4: Resolving EN Impact on UE power consumption | Huawei, HiSilicon | noted |  |  |
| S3-193333 | Solution #4: Details on hash algorithm used for MIB/SIB hashes. | Huawei, HiSilicon | revised |  | S3-193679 |
| S3-193334 | Address EN in solution #1 | Huawei, HiSilicon | approved |  |  |
| S3-193335 | Enabling UE to detect FBS | Huawei, HiSilicon | noted |  |  |
| S3-193336 | preventing UE from reselecting to FBS | Huawei, HiSilicon | noted |  | - |
| S3-193337 | Avoiding UE from Suffering More MitM Attacks by Handover | Huawei, HiSilicon | noted |  |  |
| S3-193338 | Evaluation of solution #6 | Huawei, HiSilicon | revised |  | S3-193759 |
| S3-193339 | Conclustion for Key issue #3 | Huawei, HiSilicon | noted |  |  |
| S3-193340 | Update of key issue #6 | KPN | revised |  | S3-193766 |
| S3-193341 | Conclusion to Key Issue #4 | KPN | noted |  |  |
| S3-193342 | Solution for privacy of link layer IDs in broadcast and groupcast | Qualcomm Incorporated | noted |  |  |
| S3-193343 | Conclusion on privacy for groupcast and broadcast | Qualcomm Incorporated | revised |  | S3-193803 |
| S3-193344 | Proposed conclusion of security for groupcast | Qualcomm Incorporated | revised |  | S3-193805 |
| S3-193345 | Providing some analysis to solution #9 in TR 33.836 | Qualcomm Incorporated | noted |  |  |
| S3-193346 | Providing some analysis to solution #8 in TR 33.836 | Qualcomm Incorporated | revised |  | S3-193799 |
| S3-193347 | Resolving the editor’s note on RRC security establishment in solution #8 | Qualcomm Incorporated | approved |  |  |
| S3-193348 | Resolving the editor’s note on operator knowledge of keys in solution #8 | Qualcomm Incorporated | approved |  |  |
| S3-193349 | Proposed solution for protecting the V2X PC5 traffic at the PDCP | Qualcomm Incorporated | approved |  |  |
| S3-193350 | Discussion for a response LS on NR V2X Security for user plane data and PDCP SN size | Qualcomm Incorporated | noted |  |  |
| S3-193351 | Solution for the activation of user plane security in NR PC5 unicast | Qualcomm Incorporated | revised |  | S3-193800 |
| S3-193352 | Discussion on protection of messages for V2X PC5 unicast | Qualcomm Incorporated | noted |  |  |
| S3-193353 | Some corrections/clarification for non-public networks annex | Qualcomm Incorporated | merged |  | S3-193705 |
| S3-193354 | Resolving editor’s note on refreshing the parameter in solution #10 | Qualcomm Incorporated | approved |  |  |
| S3-193355 | Resolving editor’s note on relationship between S-NSSAI and the S-NSSAI identifiers in solution #10 | Qualcomm Incorporated | revised |  | S3-193821 |
| S3-193356 | Using MACS to provide freshness for the protection of SQN during a re-synchronisation procedure in AKA | Qualcomm Incorporated | revised | S3-192921 | S3-193812 |
| S3-193357 | F1 security establishment | Qualcomm Incorporated | approved |  |  |
| S3-193358 | Evaluation on Solution #3.1: F1 security context establishment | Qualcomm Incorporated, Ericsson | noted |  | - |
| S3-193359 | Conclusion on KI #4.1: F1 interface security | Qualcomm Incorporated, Ericsson | merged |  | S3-193792 |
| S3-193360 | Evaluation of the shared key based MIB/SIB protection solution | Qualcomm Incorporated | revised | S3-192938 | S3-193754 |
| S3-193361 | Shared key based MIB/SIBs integrity information provided by gNB | Qualcomm Incorporated | noted | S3-192936 |  |
| S3-193362 | Evaluation against MitM false base station attacks | Qualcomm Incorporated | noted | S3-192937 |  |
| S3-193363 | Evaluation on UE behavior on detection of false signature | Qualcomm Incorporated | noted |  |  |
| S3-193364 | Evaluation on signing key management | Qualcomm Incorporated | noted |  |  |
| S3-193365 | Evaluation on Enriched MR | Qualcomm Incorporated | noted |  |  |
| S3-193366 | Key Issue on UE capability protection for CP optimization only CIoT UE | Qualcomm Incorporated | merged |  | S3-193704 |
| S3-193367 | LS on GUTI re-allocation | Qualcomm Incorporated | noted |  |  |
| S3-193368 | Reply LS on SUCI computation from an NSI | Qualcomm Incorporated | noted |  |  |
| S3-193369 | Proposed conclusion for 5G RAN connected to 5GC | Qualcomm Incorporated | revised |  | S3-193698 |
| S3-193370 | Security aspects of RLOS | Qualcomm Incorporated | noted |  |  |
| S3-193371 | TR 33.848 Security Requirements for Key Issue 17 (resubmission of S3-192559) | NCSC | merged |  | S3-193741 |
| S3-193372 | TR 33.848 Security Threats and Requirements for Key Issue 16 (resubmission of S3-192558) | NCSC | revised |  | S3-193742 |
| S3-193373 | TR 33.848: Security Requirements for Key Issue 18 (resubmission of S3-192560) | NCSC | noted |  |  |
| S3-193374 | TR 33.848 Security Requirements for Key Issue 19 (resubmission of S3-192561) | NCSC | revised |  | S3-193773 |
| S3-193375 | TR 33.848 Security Threats and Requirements for Key Issue 21 (resubmission of S3-192562) | NCSC | merged |  | S3-193775 |
| S3-193376 | TR 33.848 Annex – Virtualisation Security Questions (resubmission of S3-193089) | NCSC | noted |  |  |
| S3-193377 | Security for Wireline access to 5G - General | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| S3-193378 | Authentication for 5G-RG | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| S3-193379 | Security for Wireline access to 5G - General | Nokia, Nokia Shanghai Bell | revised |  | S3-193686 |
| S3-193380 | Authentication for 5G-RG | Nokia, Nokia Shanghai Bell | merged |  | S3-193687 |
| S3-193381 | Authentication for FN-RG | Nokia, Nokia Shanghai Bell | merged |  | S3-193688 |
| S3-193382 | Authentication for the UE behind the RG | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193383 | Security of the interface between W-5GAN and 5GC | Nokia, Nokia Shanghai Bell | revised |  | S3-193689 |
| S3-193384 | Security for trusted non-3GPP access – General | Nokia, Nokia Shanghai Bell | merged |  | S3-193685 |
| S3-193385 | Security procedure for trusted non-3GPP access | Nokia, Nokia Shanghai Bell | merged |  | S3-193685 |
| S3-193386 | Update to 5G_eSBA WID | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193387 | Security requirements for SeCoP | Nokia, Nokia Shanghai Bell | revised |  | S3-193719 |
| S3-193388 | Authentication and authorization between SeCoP and network functions | Nokia, Nokia Shanghai Bell | revised |  | S3-193721 |
| S3-193389 | Authentication and authorization between SeCoPs | Nokia, Nokia Shanghai Bell | revised |  | S3-193722 |
| S3-193390 | Hash based UE capability protection for CP optimization only CIoT UE | Qualcomm Incorporated | noted |  |  |
| S3-193391 | AMF verification of the UE radio capabilities for CP optimization only CIoT UE | Qualcomm Incorporated | noted |  |  |
| S3-193392 | CIoT_ Modifications to draft CR | Nokia, Nokia Shanghai Bell, Ericsson | revised |  | S3-193717 |
| S3-193393 | Proposed Solution for key issue #6 | Qualcomm Incorporated | revised |  | S3-193692 |
| S3-193394 | draft CR for Security procedures for Network Slices | Nokia, Nokia Shanghai Bell | revised |  | S3-193738 |
| S3-193395 | eNS_Addition to evaluation of solution 10 | Nokia, Nokia Shanghai Bell, Ericsson | revised |  | S3-193822 |
| S3-193396 | eNS_Addition of evaluation to solution 11 | Nokia, Nokia Shanghai Bell, Ericsson | revised |  | S3-193824 |
| S3-193397 | KI#7 Revocation of rejected NSSAI | Nokia, Nokia Shanghai Bell | merged |  | S3-193737 |
| S3-193398 | Solution for KI#7 revocation of rejected NSSAI | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193399 | Threats and Requirements for Key Issue #8 | Nokia, Nokia Shanghai Bell | revised |  | S3-193739 |
| S3-193400 | Threats and Requirements for Key Issue #13 | Nokia, Nokia Shanghai Bell | revised |  | S3-193740 |
| S3-193401 | Skeloton for eNS | Huawei, HiSilicon | noted |  |  |
| S3-193402 | Introduction to slice-specific authentication | Huawei, HiSilicon | merged |  | S3-193738 |
| S3-193403 | EAP based slice-specific authentication procedure | Huawei, HiSilicon | merged |  | S3-193738 |
| S3-193404 | EAP method negotiation | Huawei, HiSilicon | noted |  |  |
| S3-193405 | Add Evaluation to solution 3 | Huawei, HiSilicon | revised |  | S3-193731 |
| S3-193406 | Conclusion to KI#3 | Huawei, HiSilicon | noted |  |  |
| S3-193407 | Security features for NSaaS | Huawei, HiSilicon | noted |  |  |
| S3-193408 | Addressing EN in solution 6 | Huawei, HiSilicon | revised |  | S3-193734 |
| S3-193409 | Evaluating solution 6 | Huawei, HiSilicon | revised |  | S3-193735 |
| S3-193410 | Concluding KI#4 | Huawei, HiSilicon | revised |  | S3-193736 |
| S3-193411 | On service request in solution 8 | Huawei, HiSilicon | revised |  | S3-193819 |
| S3-193412 | On Ng-RAN node change in solution 8 | Huawei, HiSilicon | revised |  | S3-193820 |
| S3-193413 | Conclusions to KI #6 | Huawei, HiSilicon | noted |  |  |
| S3-193414 | New key issue on PFS | Huawei, HiSilicon | noted |  |  |
| S3-193415 | Discussion for EAP method negotiation | Huawei, HiSilicon | noted |  |  |
| S3-193416 | Overall evaluation of solutions addressing KI#4 | Huawei, HiSilicon | noted |  |  |
| S3-193417 | Overall evaluation of solutions addressing KI#6 | Huawei, HiSilicon | noted |  |  |
| S3-193418 | Threats and Requirements for Key Issue #17 | Nokia, Nokia Shanghai Bell | revised |  | S3-193741 |
| S3-193419 | Threats and Requirements for Key Issue #18 | Nokia, Nokia Shanghai Bell | revised |  | S3-193743 |
| S3-193420 | Threats and Requirements for Key Issue #20 | Nokia, Nokia Shanghai Bell | revised |  | S3-193774 |
| S3-193421 | Threats and Requirements for Key Issue #21 | Nokia, Nokia Shanghai Bell | revised |  | S3-193775 |
| S3-193422 | Threats and Requirements for Key Issue #22 | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193423 | Protection of N9 interface in Inter-PLMN scenario | Nokia, Nokia Shanghai Bell, Juniper | revised |  | S3-193723 |
| S3-193424 | Threats and Requirements for Key Issue #23 | Nokia, Nokia Shanghai Bell | revised |  | S3-193776 |
| S3-193425 | Threats and Requirements for Key Issue #24 | Nokia, Nokia Shanghai Bell | revised |  | S3-193777 |
| S3-193426 | [draft] reply LS on NR V2X Security for user plane data and PDCP SN size | LG Electronics France | revised |  | S3-193778 |
| S3-193427 | Discussion on Categorization of the Key Issues | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193428 | Security requirements for UP Gateway Function | Nokia, Nokia Shanghai Bell | revised |  | S3-193720 |
| S3-193429 | Resolution of Editors notes for Key issue 1 and 8 | LG Electronics France | approved |  |  |
| S3-193430 | Resolution of Editors notes for Key issue 6 | LG Electronics France | approved |  |  |
| S3-193431 | Conclusion for Key issue 10 | LG Electronics France | revised |  | S3-193779 |
| S3-193432 | Discussion on Security of Multi-CU-UP connectivity | CATT | withdrawn |  |  |
| S3-193433 | LS reply to RAN WG3 LS on security for multi-CU-UP connectivity | CATT | withdrawn |  |  |
| S3-193434 | Draft CR-living doucment for 5G_eLCS | CATT | merged |  | S3-193702 |
| S3-193435 | Discussion on security context handling in fast RRC release | LG Electronics | noted |  |  |
| S3-193436 | Minimizing dependency on application layer security | Intel | noted |  |  |
| S3-193437 | Protection of UE capability transfer for UEs without AS security | Intel | revised |  | S3-193704 |
| S3-193438 | Security solution for UE Capability Transfer for UE with no AS security. | Intel | noted |  |  |
| S3-193439 | Security solution for UE to avoid connecting to the false base station during a handover procedure | Intel | noted |  |  |
| S3-193440 | Conclusions on Key Issue #3 | Intel | noted |  |  |
| S3-193441 | AKMA: Resolving the EN in AKMA push | Huawei, Hisilicon | revised |  | S3-193761 |
| S3-193442 | AKMA: adding the evaluation of solution #24 | Huawei, Hisilicon | revised |  | S3-193763 |
| S3-193443 | AKMA: add conclusion on KI #17 | Huawei, Hisilicon | noted |  |  |
| S3-193444 | eSBA: add conclusion on KI #5 | Huawei, Hisilicon | noted |  |  |
| S3-193445 | eSBA: add conclusion on KI #24 | Huawei, Hisilicon | noted |  | - |
| S3-193446 | eSBA: add conclusion on KI #28 | Huawei, Hisilicon | noted |  |  |
| S3-193447 | Update on solution#15 in TR 33.855 | Huawei, Hisilicon | revised |  | S3-193730 |
| S3-193448 | Resolving the ENs in solution #5 | Huawei, Hisilicon, Lenovo, Motorola Mobility | revised |  | S3-193835 |
| S3-193449 | Conclusion on KI#5 of TR 33.809 | Huawei, Hisilicon, Lenovo, Motorola Mobility | noted |  |  |
| S3-193450 | New solution for linkability attack | Huawei, Hisilicon | revised |  | S3-193813 |
| S3-193451 | Resolving the ENs in KI#3.1 | Huawei, Hisilicon | noted |  |  |
| S3-193452 | New solution for removing the authentication result from the UDM | Huawei, Hisilicon | noted |  |  |
| S3-193453 | New solution for UP security policy handling for PC5 and Uu interface | Huawei, Hisilicon | noted |  |  |
| S3-193454 | New solution for PC5 layer key derivation using the 5G network keys | Huawei, Hisilicon | revised |  | S3-193801 |
| S3-193455 | New SID: Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | China Unicom, CAICT, China Telecom, Huawei, Hisilicon, ZTE | noted |  |  |
| S3-193456 | Adding the evaluation of solution #15 | Huawei, Hisilicon | approved |  |  |
| S3-193457 | Adding conclusion on KI #6.2 | Huawei, Hisilicon | noted |  |  |
| S3-193458 | Discussion on potential solutions to security handling of AMF reallocation | Huawei, Hisilicon | noted |  |  |
| S3-193459 | solution on AUTS derivation to protect SQN | Huawei, Hisilicon | revised |  | S3-193815 |
| S3-193460 | Address the EN and add evaluation in solution 2.3 | Huawei, Hisilicon | revised |  | S3-193814 |
| S3-193461 | A solution to protect MIB/SIBs | Huawei, Hisilicon | noted |  |  |
| S3-193462 | Discussion on binding between USIM/UICC and IAB-node | Huawei, Hisilicon | noted |  |  |
| S3-193463 | Key issue on removal of UICC card in IAB node | Huawei, Hisilicon | revised |  | S3-193793 |
| S3-193464 | Evaluation of solution#2.1 | Huawei, Hisilicon | merged |  | S3-193789 |
| S3-193465 | Solution for protection of time synchronization | Huawei, Hisilicon | revised |  | S3-193709 |
| S3-193466 | Conclusion on protection of time synchronization | Huawei, Hisilicon | noted |  |  |
| S3-193467 | Remove the EN and add evaluation to solution 6 | Huawei, Hisilicon | approved |  |  |
| S3-193468 | Propose AKMA architecture | Huawei, Hisilicon | merged |  | S3-193771 |
| S3-193469 | Propose AKMA key hierarchy | Huawei, Hisilicon | merged |  | S3-193772 |
| S3-193470 | Conclusion on 7.3 | Huawei, Hisilicon | noted |  |  |
| S3-193471 | Conclusion on 7.4 | Huawei, Hisilicon | merged |  | S3-193768 |
| S3-193472 | Propose new conlusion section | Huawei, Hisilicon | noted |  |  |
| S3-193473 | Evaluation for solution4 | Huawei, Hisilicon | approved |  |  |
| S3-193474 | Evaluation for solution22 | Huawei, Hisilicon | revised |  | S3-193764 |
| S3-193475 | Implicite AKMA authenticaiton procedure | Huawei, Hisilicon | noted |  |  |
| S3-193476 | draftCR for URLLC | Huawei, Hisilicon | revised |  | S3-193699 |
| S3-193477 | change introduction to align with URLLC architect | Huawei, Hisilicon | revised |  | S3-193700 |
| S3-193478 | clarification on security policy handling | Huawei, Hisilicon | merged |  | S3-193701 |
| S3-193479 | living doc for 5WWC | Huawei, Hisilicon | revised |  | S3-193684 |
| S3-193480 | Authentication for 5G-RG | Huawei, Hisilicon | revised |  | S3-193687 |
| S3-193481 | Authentication for FN-RG | Huawei, Hisilicon | revised |  | S3-193688 |
| S3-193482 | Conclusion for Key Issue #4 | Huawei, Hisilicon | noted |  |  |
| S3-193483 | CR for DDoS mitigation caused by misbehaving CIoT UEs | Huawei, Hisilicon | noted |  |  |
| S3-193484 | Security handling for RRCConnectionRe-establishment Procedure for Control Plane Optimization for 5GS CIoT | Huawei, Hisilicon | noted |  |  |
| S3-193485 | Security handling in Control Plane User Datafor Control Plane Optimization for 5GS CIoT | Huawei, Hisilicon | revised |  | S3-193818 |
| S3-193486 | Merged Solution for RRC Reject Protection | Huawei, Hisilicon, Samsung | noted |  |  |
| S3-193487 | Conclusion for Key Issue #1 for RRC Reject | Huawei, Hisilicon, Samsung | noted |  |  |
| S3-193488 | Update on Protection of RRC Resume Request message | Huawei, Hisilicon | approved |  |  |
| S3-193489 | Conclusion for Key Issue #1 for RRC Resume Request Protection | Huawei, Hisilicon | noted |  |  |
| S3-193490 | Conclusion for Key Issue #2 | Huawei, Hisilicon | noted |  |  |
| S3-193491 | Solution for Avoiding UE connecting to False Base Station during Conditional Handover | Huawei, Hisilicon | revised |  | S3-193760 |
| S3-193492 | LS to RAN1 on Clarification for parameters used to avoid UE connecting to the FBS | Huawei, Hisilicon | withdrawn |  |  |
| S3-193493 | Address EN in solution 23 | Huawei, Hisilicon | noted |  |  |
| S3-193494 | Conclusion of KI#5 | Huawei, Hisilicon | noted |  |  |
| S3-193495 | New solution on KI #9 minimizing the impact of privacy protection mechanism in the application layer communication | Huawei, Hisilicon | withdrawn |  |  |
| S3-193496 | Conclusion on KI#6 | Ericsson, Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193497 | New KI: Preventing mismatch for EDT with fast release | LG Electronics France | noted |  |  |
| S3-193498 | Conclusin of key issue#2 | Apple | noted |  |  |
| S3-193499 | Update for Solution#7 | Apple | noted |  |  |
| S3-193500 | Evaluation for solution#14 | Apple | noted |  |  |
| S3-193501 | Update of Solution#11 | Apple | noted |  |  |
| S3-193502 | Protection of UeapabilityInformation | Apple | noted |  |  |
| S3-193503 | 5G paging security issue caused by false base station | Apple | noted |  |  |
| S3-193504 | solution for new key issue of 5G paging security issue caused by false base station | Apple | noted |  |  |
| S3-193505 | Discussion on 5G UE privacy when connecting to EPC | Apple | noted |  |  |
| S3-193506 | Meeting minutes of 5GFBS July conference call on July 18th | Apple | noted |  |  |
| S3-193507 | Meeting minutes of 5GFBS August conference call on August 8th | Apple | noted |  |  |
| S3-193508 | Evaluation for Solution#5 in UP IP | Apple | merged |  | S3-193696 |
| S3-193509 | Solution to key issue#5 in UP IP | Apple | revised |  | S3-193694 |
| S3-193510 | EAP-AKA privacy enhancement in non-3GPP access to EPS | Apple | revised |  | S3-193629 |
| S3-193511 | Conclusion of CAG ID Privacy | ZTE Corporation | noted |  |  |
| S3-193512 | CAG ID Privacy for non-public networks | ZTE Corporation | noted |  |  |
| S3-193513 | User identity privacy for GBA in 5GC | ZTE Corporation | noted |  |  |
| S3-193514 | Assessment and evaluation of solution #9 | ZTE Corporation | noted |  |  |
| S3-193515 | Structure RAND for authentication | ZTE Corporation | noted |  |  |
| S3-193516 | Conclusion on linkability issues | ZTE Corporation | noted |  |  |
| S3-193517 | Resolving ENs of solution #12 | ZTE Corporation | revised |  | S3-193825 |
| S3-193518 | Evaluation of solution #12 | ZTE Corporation | noted |  |  |
| S3-193519 | New key issue on protection against Man-in-the-Middle false IAB node attacks | ZTE Corporation | noted |  |  |
| S3-193520 | New key issue on security for multi-USIM communication over UU and PC5 | ZTE Corporation | noted |  |  |
| S3-193521 | Editorial correction to TR 33.824 | ZTE Corporation | approved |  |  |
| S3-193522 | A solution against V2X UE tracking based on PC5 identifiers | Ericsson LM | approved |  |  |
| S3-193523 | TR 33.819 update | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193524 | Status of TR | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193525 | CAG ID privacy solution considering RAN optimization | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193526 | CAG ID privacy conclusion -v1 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193527 | CR CAG ID privacy | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193528 | Removal of ed.note on conformance tests | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193529 | Editorial correction of format | Nokia, Nokia Shanghai Bell | revised |  | S3-193705 |
| S3-193530 | Conclusion to key issue 2.3 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193531 | CR Interworking between NPN and PLMN | Nokia, Nokia Shanghai Bell | revised |  | S3-193706 |
| S3-193532 | CR Annex 5GLAN | Nokia, Nokia Shanghai Bell | revised |  | S3-193707 |
| S3-193533 | CR Annex TSC | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193534 | KI on Protection of authentication subscription data stored in UDR | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193535 | KI on Separation of authentication subscription data from subscription data | Nokia, Nokia Shanghai Bell | revised |  | S3-193747 |
| S3-193536 | CIOT: Update of KI#12 description | Ericsson | noted |  |  |
| S3-193537 | CIOT: update to the evaluation of solution #24 | Ericsson, Qualcomm Incorporated | approved |  |  |
| S3-193538 | CIOT: recommendation to support solution #24 to solve KI #11 | Ericsson, Qualcomm Incorporated | approved |  |  |
| S3-193539 | CIOT: Conclusion on KI #4 | Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung | approved |  |  |
| S3-193540 | CIOT: Conclusion on KI #5 | Ericsson Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung | revised |  | S3-193711 |
| S3-193541 | CIOT: Discussion on DDoS solutions for KI#4 and KI#5 | Ericsson, Qualcomm Incorporated, Samsung | noted |  |  |
| S3-193542 | CIOT: Title correction solution 13 | Ericsson | approved |  |  |
| S3-193543 | CIOT: Proposed conclusion to KI#10 | Ericsson | approved |  |  |
| S3-193544 | CIOT: Proposed conclusion to KI#7 | Ericsson | revised |  | S3-193712 |
| S3-193545 | CIOT: Proposed clarifications to KI#6 | Ericsson | approved |  |  |
| S3-193546 | CIOT: Proposed_conclusion_KI_8 | Ericsson | revised |  | S3-193713 |
| S3-193547 | CIOT: Proposed_conclusion_KI_6 | Ericsson | approved |  |  |
| S3-193548 | CIOT: Update of Solution #14 | Ericsson | noted |  |  |
| S3-193549 | CIOT: Conclusion for KI#12 | Ericsson | revised |  | S3-193714 |
| S3-193550 | Solution for trusted non-3GPP access | Lenovo, Motorola Mobility | revised |  | S3-193685 |
| S3-193551 | Solution on 5GC access from WLAN UEs that do not support NAS | Lenovo, Motorola Mobility | revised |  | S3-193690 |
| S3-193552 | Conclusion on Key Issue#7 | Lenovo, Motorola Mobility | merged |  | S3-193712 |
| S3-193553 | Clarification on cancellation of rejected S-NSSAIs | Lenovo, Motorola Mobility | revised |  | S3-193737 |
| S3-193554 | Solution on Cancellation of rejected S-NSSAIs | Lenovo, Motorola Mobility | noted |  |  |
| S3-193555 | Update of Solution #15 | Lenovo, Motorola Mobility | revised |  | S3-193836 |
| S3-193556 | Update of solution #6 | Lenovo, Motorola Mobility | noted |  |  |
| S3-193557 | Identifier conversion in groupcast communication | Lenovo, Motorola Mobility | revised |  | S3-193804 |
| S3-193558 | DraftCR – Control Plane optimized solution | Ericsson | merged |  | S3-193818 |
| S3-193559 | DraftCR – RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC | Ericsson | noted |  |  |
| S3-193560 | DraftCR – Control Plane optimized solution | Ericsson | withdrawn |  |  |
| S3-193561 | DraftCR – NAS based redirection between 5GS and EPC | Ericsson | approved |  |  |
| S3-193562 | DraftCR - Proposed skeleton for supporting 5G CIoT [Living Document] | Ericsson | revised |  | S3-193716 |
| S3-193563 | Draft CR as a living baseline for 5GS LCS normative work | Ericsson LM | revised |  | S3-193702 |
| S3-193564 | [Draft CR]Solution for IAB Architecture | Samsung | revised |  | S3-193808 |
| S3-193565 | Updates to Solution #2.1 on MT functionality | Samsung | revised |  | S3-193788 |
| S3-193566 | Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0 | VODAFONE Group Plc | revised |  | S3-193744 |
| S3-193567 | Evaluation of solution #2.1 | Samsung | revised |  | S3-193789 |
| S3-193568 | Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (MT) | Samsung | revised |  | S3-193790 |
| S3-193569 | CIOT: Discussion paper on ngKSI for NB-IoT in 5G-CIoT. | Ericsson | noted |  |  |
| S3-193570 | CIOT: Discussion paper on short MAC-I for NB-IoT in 5G-CIoT. | Ericsson | noted |  |  |
| S3-193571 | Draft LS reply on LS on short MAC-I and ngKSI for 5G-CIoT | Ericsson | revised |  | S3-193715 |
| S3-193572 | Evaluation of solution #3.1 | Samsung | noted |  |  |
| S3-193573 | Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU) | Samsung | revised |  | S3-193792 |
| S3-193574 | Solution for Resumecause protection | Samsung | noted |  |  |
| S3-193575 | Evaluation of solution#14 in TR 33.809 | Samsung | noted |  |  |
| S3-193576 | Updates to Solution#7 on obtaining accurate clock information | Samsung | approved |  |  |
| S3-193577 | Deletion of EN on Location update reject in Solution#7 | Samsung | noted |  |  |
| S3-193578 | Delete the Evaluation for Solution 5 in TR 33.853 | China Mobile | approved |  |  |
| S3-193579 | Resolving Editor’s Note in Solution #1 | Samsung | revised |  | S3-193695 |
| S3-193580 | Add the Evaluation to Solution 5 in TR 33.853 | China Mobile | revised |  | S3-193696 |
| S3-193581 | Conclusion to Key Issue #5 | Samsung | noted |  |  |
| S3-193582 | skeleton of AKMA TS | China Mobile, Nokia, Nokia Shanghai Bell | revised |  | S3-193769 |
| S3-193583 | scope of AKMA TS | China Mobile, Nokia, Nokia Shanghai Bell | revised |  | S3-193770 |
| S3-193584 | draft CR of AKMA architecture | China Mobile, Nokia, Nokia Shanghai Bell | revised |  | S3-193771 |
| S3-193585 | conclusion on key issue #2 | China Mobile | revised |  | S3-193767 |
| S3-193586 | Conclusion on key issue #5 | China Mobile | revised |  | S3-193768 |
| S3-193587 | Conclusion on key issue #6 | China Mobile, KPN | noted |  |  |
| S3-193588 | Conclusion on key issue #13 | China Mobile | approved |  |  |
| S3-193589 | Editorial Changes to TR 33.835 | China Mobile | approved |  |  |
| S3-193590 | Individual Evaluation of solution #6 | China Mobile | approved |  |  |
| S3-193591 | Evaluation of solution#1 | China Mobile | approved |  |  |
| S3-193592 | Evaluations of solution #7- #12 | China Mobile | approved |  |  |
| S3-193593 | Udpate to Solution #3 | Samsung | noted |  |  |
| S3-193594 | Key Issue #6.1 conclusion | Samsung | noted |  |  |
| S3-193595 | Solution Key lifetimes | Ericsson | revised |  | S3-193765 |
| S3-193596 | Skeleton of clause 4 and selected content for AKMA normative work | Ericsson | merged |  | S3-193769 |
| S3-193597 | Solution #15 evaluation removal of EN | Ericsson | approved |  |  |
| S3-193598 | Conclusions on Key management | Ericsson | approved |  |  |
| S3-193599 | AKMA architecture reference model | Ericsson | merged |  | S3-193771 |
| S3-193600 | AKMA Key hierarchy | Ericsson | revised |  | S3-193772 |
| S3-193601 | Existing authentication procedure lacking PFS property | Ericsson | noted |  |  |
| S3-193602 | Correction of conclusion for Key Issue #1 | Ericsson | approved |  |  |
| S3-193603 | New Solution: Protection of the whole RRCResumeRequest message | Ericsson | revised |  | S3-193753 |
| S3-193604 | Way forward for KI#1 against tampering of RRCResumeRequest messages | Ericsson | noted |  |  |
| S3-193605 | Way forward for KI#3 with respect to handover to a False basestation | Ericsson | noted |  |  |
| S3-193606 | Way forward for KI#4 on protection against SON poisoning | Ericsson | noted |  |  |
| S3-193607 | Way forward for KI#5 mitigation against authentication relay. | Ericsson | noted |  |  |
| S3-193608 | Way forward for KI#6 about the resistance to radio jamming | Ericsson | revised |  | S3-193837 |
| S3-193609 | Way forward for KI#7 about the protection against Man-in-the-Middle false basestation attacks | Ericsson | noted |  |  |
| S3-193610 | New solution (SERSI – SERving network controlled SI signatures). | Ericsson | revised |  | S3-193755 |
| S3-193611 | DraftCR –Security handling for IAB | Ericsson | merged |  | S3-193808 |
| S3-193612 | Clarification on identities in TLS and token-based authorization | Ericsson | noted |  |  |
| S3-193613 | Clarification on delegated subscribe-notify | Ericsson | noted |  |  |
| S3-193614 | LS on token-based authorization for indirect communication with delegated discovery (Scenario D) | Ericsson | revised |  | S3-193724 |
| S3-193615 | Implementation of agreed change from SA3#96 | Ericsson | approved |  |  |
| S3-193616 | Update to conclusion on Key issue #22: Authorization of NF service access in indirect communication | Ericsson | noted |  |  |
| S3-193617 | Update to conclusion on Key issue #23: NF to NF authentication and authorization in Indirect communication | Ericsson | noted |  |  |
| S3-193618 | Conclusion of Key Issue #24 Service access authorization within a NF Set or a NF Service Set | Ericsson | noted |  |  |
| S3-193619 | New solution for Key Issue #25 "Indirect communication in roaming scenarios" | Ericsson | revised |  | S3-193725 |
| S3-193620 | Conclusion for Key Issue #25 "Indirect communication in roaming scenarios" | Ericsson | revised |  | S3-193726 |
| S3-193621 | Conclusion of Key Issue #28: Service access authorization in the delegated "Subscribe-Notify" scenarios | Ericsson | noted |  |  |
| S3-193622 | Resource Level Authorization using Access Tokens | Ericsson | noted |  |  |
| S3-193623 | TLS certificates for SBA: profile and provisioning | Ericsson | noted |  |  |
| S3-193624 | Authorization using Access Tokens based on NF-Subtype | Ericsson | noted |  |  |
| S3-193625 | Conclusion for Key Issue #5 "NF-NF Authorization" | Ericsson | noted |  |  |
| S3-193626 | Evaluation update for Solution #25 | Ericsson | noted |  |  |
| S3-193627 | Draft CR: UP security policy for URLLC | Ericsson | revised |  | S3-193701 |
| S3-193628 | Discussion Paper for Security of Performance Measurement Function Protocol | Apple Computer Trading Co. Ltd | noted |  |  |
| S3-193629 | New SID on EPS AKA and EAP-AKA privacy enhancement in EPS | Apple, Google, AT&T, Verizon UK Ltd, Accuris Networks, Charter Communications, Cablelabs, Article19, Sprint, Comcast, Broadcom | noted | S3-193510 |  |
| S3-193630 | ARPF Deployment models | Ericsson | noted |  |  |
| S3-193631 | Trusted access key hierarchy | Ericsson | noted |  |  |
| S3-193632 | Trusted access key derivation | Ericsson | noted |  |  |
| S3-193633 | Subscriber privacy for wireline access | Ericsson | approved |  |  |
| S3-193634 | Security Parameter Storage | Ericsson | noted |  |  |
| S3-193635 | Clarification to the usage of Kausf for solution #2.2 in TR 33.846 | China Mobile | noted |  |  |
| S3-193636 | Discussion on the SUPI guessing attack | China Mobile | noted |  |  |
| S3-193637 | Key issue to mitigate the SUCI guessing attacks in TR 33.846 | China Mobile | revised |  | S3-193810 |
| S3-193638 | Clarifying GVNP model of type 2 | China Mobile | approved |  |  |
| S3-193639 | Adding description for Generic virtualized network product model of type 3 | China Mobile | revised |  | S3-193786 |
| S3-193640 | Adding Generic assets and threats of GVNP for type 1 into clause 5.2.3.b | China Mobile | revised |  | S3-193787 |
| S3-193641 | Adding Generic assets and threats of GVNP for type 2 into clause 5.2.3.c | China Mobile | revised |  | S3-193831 |
| S3-193642 | Adding Generic assets and threats of GVNP for type 3 into clause 5.2.3.d | China Mobile | revised |  | S3-193832 |
| S3-193643 | Adding security requirements into clause 5.2.4 | China Mobile | revised |  | S3-193833 |
| S3-193644 | applying TR33.818 with new 3GPP_TS-TR_Template | China Mobile | approved |  |  |
| S3-193645 | Meeting minutes of VNP_SECAM_SCAS conference call on 25th September | China Mobile | noted |  |  |
| S3-193646 | UPIP: update of solution #7 and addition of evaluation | Ericsson | revised |  | S3-193697 |
| S3-193647 | Privacy Aspects of ARPF deployment | Ericsson | revised |  | S3-193748 |
| S3-193648 | draft LS to SA2 on the procedure of Secondary authentication | China Mobile Com. Corporation | withdrawn |  |  |
| S3-193649 | Add Clarifications to the Scope of Accreditation for 3GPP VNPs | Nokia, Nokia Shanghai Bell | revised |  | S3-193849 |
| S3-193650 | Add Clarifications to Ultimate Output of SECAM Evaluation | Nokia, Nokia Shanghai Bell | revised |  | S3-193780 |
| S3-193651 | Add Clarifications to 3GPP virtualized network product evaluation process | Nokia, Nokia Shanghai Bell | revised |  | S3-193782 |
| S3-193652 | Add Clarifications to Roles in SECAM for 3GPP virtualized network products | Nokia, Nokia Shanghai Bell | revised |  | S3-193783 |
| S3-193653 | Add Clarifications to SECAM Assurance Level for 3GPP VNPs | Nokia, Nokia Shanghai Bell | revised |  | S3-193784 |
| S3-193654 | UICC removal from IAB-node | THALES | merged |  | S3-193793 |
| S3-193655 | Add Clarifications to Security Baseline for 3GPP VNPs | Nokia, Nokia Shanghai Bell | approved |  |  |
| S3-193656 | Add Clarifications to SCAS documents structure and content | Nokia, Nokia Shanghai Bell | revised |  | S3-193785 |
| S3-193657 | Add Clarifications to generic virtualized network product model description | Nokia, Nokia Shanghai Bell | revised |  | S3-193834 |
| S3-193658 | Nokia comments on CT1 LS C1-195199 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193659 | pCR to TR33.853 on Migration paths for network deployment | NTT DOCOMO INC. | noted |  |  |
| S3-193660 | LS on short MAC-I and ngKSI for 5G-CIoT | C1-195199 | replied to |  |  |
| S3-193661 | LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update | C4-193790 | replied to |  |  |
| S3-193662 | LS on SUCI computation from an NSI | CP-192262 | postponed |  |  |
| S3-193663 | PEI for FN-RG Recommendation | BBF | noted |  |  |
| S3-193664 | General Status of Work | BBF | noted |  |  |
| S3-193665 | LS on NR V2X Security for user plane data and PDCP SN size | R2-1911681 | replied to |  |  |
| S3-193666 | Reply to LS on Impersonation Attacks in 4G Networks | R2-1911819 | noted |  |  |
| S3-193667 | Reply LS on protection of PC5-RRC messages for sidelink unicast communication | R2-1911863 | noted |  |  |
| S3-193668 | Reply LS on LTE-M identification in 5GC | R3-194748 | noted |  |  |
| S3-193669 | LS on the IAB-indication to core network | R3-194787 | noted |  |  |
| S3-193670 | N9HR Regulatory Obligations | S3i190548 | postponed |  |  |
| S3-193671 | New WID: Security Aspects of PARLOS | SPRINT Corporation | endorsed |  |  |
| S3-193672 | New WID for User Plane Gateway Function for the Inter-PLMN Security | Juniper Networks | revised |  | S3-193718 |
| S3-193673 | Comments to Draft for ‘proposed structure for network slice security procedures’ in S3-193394 | InterDigital Communications | noted |  |  |
| S3-193674 | Comments to Evaluation for Solution 11 in S3-193396 | InterDigital Communications | noted |  |  |
| S3-193675 | Nokia comments on S3-193412 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193676 | Nokia comments on S3-193517 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193677 | Nokia comments on S3-193417 | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193678 | Update to conclusion on Key issue #22: Authorization of NF service access in indirect communication | Nokia, Nokia Shanghai Bell | noted |  |  |
| S3-193679 | Solution #4: Details on hash algorithm used for MIB/SIB hashes. | Huawei, HiSilicon | revised | S3-193333 | S3-193756 |
| S3-193680 | LS on security consideration of performance measurement function protocol | C1-196940 | postponed | - | - |
| S3-193681 | Agenda | WG Chair | approved | S3-193300 | - |
| S3-193682 | Reply to: LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update | Nokia | approved | - | - |
| S3-193683 | LS on security consideration of performance measurement function protocol | ZTE Corporation | revised | - | S3-193816 |
| S3-193684 | living doc for 5WWC | Huawei, Hisilicon | approved | S3-193479 | - |
| S3-193685 | Solution for trusted non-3GPP access | Lenovo, Motorola Mobility | approved | S3-193550 | - |
| S3-193686 | Security for Wireline access to 5G - General | Nokia, Nokia Shanghai Bell | approved | S3-193379 | - |
| S3-193687 | Authentication for 5G-RG | Huawei, Hisilicon | approved | S3-193480 | - |
| S3-193688 | Authentication for FN-RG | Huawei, Hisilicon | approved | S3-193481 | - |
| S3-193689 | Security of the interface between W-5GAN and 5GC | Nokia, Nokia Shanghai Bell | approved | S3-193383 | - |
| S3-193690 | Solution on 5GC access from WLAN UEs that do not support NAS | Lenovo, Motorola Mobility | approved | S3-193551 | - |
| S3-193691 | Proposal to solve ED notes in solution#4: Zero-overhead user plane integrity protection on the link layer | Philips International B.V. | approved | S3-193320 | - |
| S3-193692 | Proposed Solution for key issue #6 | Qualcomm Incorporated | approved | S3-193393 | - |
| S3-193693 | Draft TR 33.853 | NTT DOCOMO INC. | approved | - | - |
| S3-193694 | Solution to key issue#5 in UP IP | Apple | approved | S3-193509 | - |
| S3-193695 | Resolving Editor’s Note in Solution #1 | Samsung | approved | S3-193579 | - |
| S3-193696 | Add the Evaluation to Solution 5 in TR 33.853 | China Mobile | approved | S3-193580 | - |
| S3-193697 | UPIP: update of solution #7 and addition of evaluation | Ericsson | approved | S3-193646 | - |
| S3-193698 | Proposed conclusion for 5G RAN connected to 5GC | Qualcomm Incorporated | approved | S3-193369 | - |
| S3-193699 | draftCR for URLLC | Huawei, Hisilicon | approved | S3-193476 | - |
| S3-193700 | change introduction to align with URLLC architect | Huawei, Hisilicon | approved | S3-193477 | - |
| S3-193701 | Draft CR: UP security policy for URLLC | Ericsson | revised | S3-193627 | S3-193848 |
| S3-193702 | Draft CR as a living baseline for 5GS LCS normative work | Ericsson LM | revised | S3-193563 | S3-193847 |
| S3-193703 | draft TR 33.861 | Ericsson | approved | - | - |
| S3-193704 | Protection of UE capability transfer for UEs without AS security | Intel | revised | S3-193437 | S3-193843 |
| S3-193705 | Editorial correction of format | Nokia, Nokia Shanghai Bell | approved | S3-193529 | - |
| S3-193706 | CR Interworking between NPN and PLMN | Nokia, Nokia Shanghai Bell | approved | S3-193531 | - |
| S3-193707 | CR Annex 5GLAN | Nokia, Nokia Shanghai Bell | approved | S3-193532 | - |
| S3-193708 | draft TR 33.819 | Nokia | approved | - | - |
| S3-193709 | Solution for protection of time synchronization | Huawei, Hisilicon | approved | S3-193465 | - |
| S3-193710 | LS on Signalling overload due to malicious Applications on UE | KPN | revised | - | S3-193844 |
| S3-193711 | CIOT: Conclusion on KI #5 | Ericsson Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung | approved | S3-193540 | - |
| S3-193712 | CIOT: Proposed conclusion to KI#7 | Ericsson | approved | S3-193544 | - |
| S3-193713 | CIOT: Proposed_conclusion_KI_8 | Ericsson | approved | S3-193546 | - |
| S3-193714 | CIOT: Conclusion for KI#12 | Ericsson | approved | S3-193549 | - |
| S3-193715 | Reply to: LS on short MAC-I and ngKSI for 5G-CIoT | Ericsson | approved | - | - |
| S3-193716 | DraftCR - Proposed skeleton for supporting 5G CIoT [Living Document] | Ericsson | approved | S3-193562 | - |
| S3-193717 | CIoT_ Modifications to draft CR | Nokia, Nokia Shanghai Bell, Ericsson | approved | S3-193392 | - |
| S3-193718 | New WID for User Plane Gateway Function for the Inter-PLMN Security | Juniper Networks | endorsed | S3-193672 | - |
| S3-193719 | Security requirements for SeCoP | Nokia, Nokia Shanghai Bell | approved | S3-193387 | - |
| S3-193720 | Security requirements for UP Gateway Function | Nokia, Nokia Shanghai Bell | approved | S3-193428 | - |
| S3-193721 | Authentication and authorization between SeCoP and network functions | Nokia, Nokia Shanghai Bell | approved | S3-193388 | - |
| S3-193722 | Authentication and authorization between SeCoPs | Nokia, Nokia Shanghai Bell | approved | S3-193389 | - |
| S3-193723 | Protection of N9 interface in Inter-PLMN scenario | Nokia, Nokia Shanghai Bell, Juniper | approved | S3-193423 | - |
| S3-193724 | LS on token-based authorization for indirect communication with delegated discovery (Scenario D) | Ericsson | approved | S3-193614 | - |
| S3-193725 | New solution for Key Issue #25 "Indirect communication in roaming scenarios" | Ericsson | approved | S3-193619 | - |
| S3-193726 | Conclusion for Key Issue #25 "Indirect communication in roaming scenarios" | Ericsson | approved | S3-193620 | - |
| S3-193727 | eSBA: add conclusion on KI #24 | Huawei, Hisilicon | withdrawn | - | - |
| S3-193728 | Resolving EN in TR33.855 6.18 N9 NDS/IP | Juniper Networks, NTT DoCoMo, Ericsson | approved | S3-193313 | - |
| S3-193729 | Draft TR 33.855 | Ericsson | approved | - | - |
| S3-193730 | Update on solution#15 in TR 33.855 | Huawei, Hisilicon | approved | S3-193447 | - |
| S3-193731 | Add Evaluation to solution 3 | Huawei, HiSilicon | approved | S3-193405 | - |
| S3-193732 | draft TR 33.813 | Nokia | approved | - | - |
| S3-193733 | LS on configuaration of security policy for NSaaS | Huawei | noted | - | - |
| S3-193734 | Addressing EN in solution 6 | Huawei, HiSilicon | approved | S3-193408 | - |
| S3-193735 | Evaluating solution 6 | Huawei, HiSilicon | approved | S3-193409 | - |
| S3-193736 | Concluding KI#4 | Huawei, HiSilicon | approved | S3-193410 | - |
| S3-193737 | Clarification on cancellation of rejected S-NSSAIs | Lenovo, Motorola Mobility | noted | S3-193553 | - |
| S3-193738 | draft CR for Security procedures for Network Slices | Nokia, Nokia Shanghai Bell | revised | S3-193394 | S3-193857 |
| S3-193739 | Threats and Requirements for Key Issue #8 | Nokia, Nokia Shanghai Bell | approved | S3-193399 | - |
| S3-193740 | Threats and Requirements for Key Issue #13 | Nokia, Nokia Shanghai Bell | approved | S3-193400 | - |
| S3-193741 | Threats and Requirements for Key Issue #17 | Nokia, Nokia Shanghai Bell | revised | S3-193418 | S3-193828 |
| S3-193742 | TR 33.848 Security Threats and Requirements for Key Issue 16 (resubmission of S3-192558) | NCSC | approved | S3-193372 | - |
| S3-193743 | Threats and Requirements for Key Issue #18 | Nokia, Nokia Shanghai Bell | approved | S3-193419 | - |
| S3-193744 | Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0 | VODAFONE Group Plc | approved | S3-193566 | - |
| S3-193745 | draft TR 33.845 | Samsung | approved | - | - |
| S3-193746 | KI on Protection of authentication subscription data stored in UDR | Nokia, Nokia Shanghai Bell | withdrawn | - | - |
| S3-193747 | KI on Separation of authentication subscription data from subscription data | Nokia, Nokia Shanghai Bell | approved | S3-193535 | - |
| S3-193748 | Privacy Aspects of ARPF deployment | Ericsson | approved | S3-193647 | - |
| S3-193749 | FS_SIV TR 33.848 v030c | BT plc | approved | S3-193318 | - |
| S3-193750 | Draft TR 33.848 | BT | approved | - | - |
| S3-193751 | TR 33.848 Scope Update | BT plc | approved | S3-193328 | - |
| S3-193752 | Draft TR 33.809 | Apple | approved | - | - |
| S3-193753 | New Solution: Protection of the whole RRCResumeRequest message | Ericsson | approved | S3-193603 | - |
| S3-193754 | Evaluation of the shared key based MIB/SIB protection solution | Qualcomm Incorporated | approved | S3-193360 | - |
| S3-193755 | New solution (SERSI – SERving network controlled SI signatures). | Ericsson | revised | S3-193610 | S3-193845 |
| S3-193756 | Solution #4: Details on hash algorithm used for MIB/SIB hashes. | Huawei, HiSilicon | approved | S3-193679 | - |
| S3-193757 | Resolve second and third EN in Solution #6 | Huawei, HiSilicon | approved | S3-193330 | - |
| S3-193758 | preventing UE from reselecting to FBS | Huawei, HiSilicon | withdrawn | - | - |
| S3-193759 | Evaluation of solution #6 | Huawei, HiSilicon | approved | S3-193338 | - |
| S3-193760 | Solution for Avoiding UE connecting to False Base Station during Conditional Handover | Huawei, Hisilicon | approved | S3-193491 | - |
| S3-193761 | AKMA: Resolving the EN in AKMA push | Huawei, Hisilicon | approved | S3-193441 | - |
| S3-193762 | draft TR 33.835 | China Mobile | approved | - | - |
| S3-193763 | AKMA: adding the evaluation of solution #24 | Huawei, Hisilicon | approved | S3-193442 | - |
| S3-193764 | Evaluation for solution22 | Huawei, Hisilicon | approved | S3-193474 | - |
| S3-193765 | Solution Key lifetimes | Ericsson | approved | S3-193595 | - |
| S3-193766 | Update of key issue #6 | KPN | approved | S3-193340 | - |
| S3-193767 | conclusion on key issue #2 | China Mobile | revised | S3-193585 | S3-193842 |
| S3-193768 | Conclusion on key issue #5 | China Mobile | approved | S3-193586 | - |
| S3-193769 | skeleton of AKMA TS | China Mobile, Nokia, Nokia Shanghai Bell,Ericsson | approved | S3-193582 | - |
| S3-193770 | scope of AKMA TS | China Mobile, Nokia, Nokia Shanghai Bell | approved | S3-193583 | - |
| S3-193771 | draft CR of AKMA architecture | China Mobile, Nokia, Nokia Shanghai Bell | revised | S3-193584 | S3-193841 |
| S3-193772 | AKMA Key hierarchy | Ericsson | approved | S3-193600 | - |
| S3-193773 | TR 33.848 Security Requirements for Key Issue 19 (resubmission of S3-192561) | NCSC | noted | S3-193374 | - |
| S3-193774 | Threats and Requirements for Key Issue #20 | Nokia, Nokia Shanghai Bell | revised | S3-193420 | S3-193829 |
| S3-193775 | Threats and Requirements for Key Issue #21 | Nokia, Nokia Shanghai Bell | approved | S3-193421 | - |
| S3-193776 | Threats and Requirements for Key Issue #23 | Nokia, Nokia Shanghai Bell | approved | S3-193424 | - |
| S3-193777 | Threats and Requirements for Key Issue #24 | Nokia, Nokia Shanghai Bell | approved | S3-193425 | - |
| S3-193778 | [draft] reply LS on NR V2X Security for user plane data and PDCP SN size | LG Electronics France | revised | S3-193426 | S3-193854 |
| S3-193779 | Conclusion for Key issue 10 | LG Electronics France | noted | S3-193431 | - |
| S3-193780 | Add Clarifications to Ultimate Output of SECAM Evaluation | Nokia, Nokia Shanghai Bell | approved | S3-193650 | - |
| S3-193781 | Draft TR 33.818 | China Mobile | approved | - | - |
| S3-193782 | Add Clarifications to 3GPP virtualized network product evaluation process | Nokia, Nokia Shanghai Bell | approved | S3-193651 | - |
| S3-193783 | Add Clarifications to Roles in SECAM for 3GPP virtualized network products | Nokia, Nokia Shanghai Bell | approved | S3-193652 | - |
| S3-193784 | Add Clarifications to SECAM Assurance Level for 3GPP VNPs | Nokia, Nokia Shanghai Bell | approved | S3-193653 | - |
| S3-193785 | Add Clarifications to SCAS documents structure and content | Nokia, Nokia Shanghai Bell | approved | S3-193656 | - |
| S3-193786 | Adding description for Generic virtualized network product model of type 3 | China Mobile | approved | S3-193639 | - |
| S3-193787 | Adding Generic assets and threats of GVNP for type 1 into clause 5.2.3.b | China Mobile | approved | S3-193640 | - |
| S3-193788 | Updates to Solution #2.1 on MT functionality | Samsung | revised | S3-193565 | S3-193851 |
| S3-193789 | Evaluation of solution #2.1 | Samsung | noted | S3-193567 | - |
| S3-193790 | Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (MT) | Samsung | noted | S3-193568 | - |
| S3-193791 | Evaluation on Solution #3.1: F1 security context establishment | Qualcomm Incorporated, Ericsson | withdrawn | - | - |
| S3-193792 | Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU) | Samsung | revised | S3-193573 | S3-193856 |
| S3-193793 | Key issue on removal of UICC card in IAB node | Huawei, Hisilicon | revised | S3-193463 | S3-193852 |
| S3-193794 | draft TR 33.836 | LG | approved | - | - |
| S3-193795 | 33.836 – evaluation for the Solution #1 | InterDigital Communications | approved | S3-193312 | - |
| S3-193796 | TR 33.836 – evaluation of the solution #4 | InterDigital Communications | approved | S3-193316 | - |
| S3-193797 | TR 33.836 - update for solution #2 | InterDigital Communications | approved | S3-193305 | - |
| S3-193798 | TR 33.836 - evaluation for the solution #2 | InterDigital Communications | approved | S3-193314 | - |
| S3-193799 | Providing some analysis to solution #8 in TR 33.836 | Qualcomm Incorporated | approved | S3-193346 | - |
| S3-193800 | Solution for the activation of user plane security in NR PC5 unicast | Qualcomm Incorporated | approved | S3-193351 | - |
| S3-193801 | New solution for PC5 layer key derivation using the 5G network keys | Huawei, Hisilicon | approved | S3-193454 | - |
| S3-193802 | LS on PC5S and PC5 RRC unicast message protection | Qualcomm | approved | - | - |
| S3-193803 | Conclusion on privacy for groupcast and broadcast | Qualcomm Incorporated | approved | S3-193343 | - |
| S3-193804 | Identifier conversion in groupcast communication | Lenovo, Motorola Mobility | approved | S3-193557 | - |
| S3-193805 | Proposed conclusion of security for groupcast | Qualcomm Incorporated | revised | S3-193344 | S3-193853 |
| S3-193806 | New Key Issue on Security of broadcast eV2X messages over PC5 | Huawei, HiSilicon | approved | S3-193324 | - |
| S3-193807 | draft TR 33.824 | Samsung | approved | - | - |
| S3-193808 | [Draft CR]Solution for IAB Architecture | Samsung | approved | S3-193564 | - |
| S3-193809 | Notes from evening session on eNSI | Qualcomm | noted | - | - |
| S3-193810 | Key issue to mitigate the SUCI guessing attacks in TR 33.846 | China Mobile | approved | S3-193637 | - |
| S3-193811 | 33:846: mitigation against linkability attack | THALES | approved | S3-193319 | - |
| S3-193812 | Using MACS to provide freshness for the protection of SQN during a re-synchronisation procedure in AKA | Qualcomm Incorporated | approved | S3-193356 | - |
| S3-193813 | New solution for linkability attack | Huawei, Hisilicon | approved | S3-193450 | - |
| S3-193814 | Address the EN and add evaluation in solution 2.3 | Huawei, Hisilicon | approved | S3-193460 | - |
| S3-193815 | solution on AUTS derivation to protect SQN | Huawei, Hisilicon | approved | S3-193459 | - |
| S3-193816 | LS on security consideration of performance measurement function protocol | ZTE Corporation | noted | S3-193683 | - |
| S3-193817 | Draft TS 33.535 | China Mobile | approved | - | - |
| S3-193818 | Security handling in Control Plane User Datafor Control Plane Optimization for 5GS CIoT | Huawei, Hisilicon | approved | S3-193485 | - |
| S3-193819 | On service request in solution 8 | Huawei, HiSilicon | approved | S3-193411 | - |
| S3-193820 | On Ng-RAN node change in solution 8 | Huawei, HiSilicon | approved | S3-193412 | - |
| S3-193821 | Resolving editor’s note on relationship between S-NSSAI and the S-NSSAI identifiers in solution #10 | Qualcomm Incorporated | approved | S3-193355 | - |
| S3-193822 | eNS_Addition to evaluation of solution 10 | Nokia, Nokia Shanghai Bell, Ericsson | approved | S3-193395 | - |
| S3-193823 | TR 33.813 - update for the evaluation for solution #11 | InterDigital Communications | approved | S3-193309 | - |
| S3-193824 | eNS_Addition of evaluation to solution 11 | Nokia, Nokia Shanghai Bell, Ericsson | approved | S3-193396 | - |
| S3-193825 | Resolving ENs of solution #12 | ZTE Corporation | approved | S3-193517 | - |
| S3-193826 | TR 33.819 – evaluation of DH based solution for CAG ID privacy | InterDigital Communications | approved | S3-193310 | - |
| S3-193827 | TR 33.819 – evaluation of Hash based solution for CAG ID privacy | InterDigital Communications | approved | S3-193311 | - |
| S3-193828 | Threats and Requirements for Key Issue #17 | Nokia, Nokia Shanghai Bell | approved | S3-193741 | - |
| S3-193829 | Threats and Requirements for Key Issue #20 | Nokia, Nokia Shanghai Bell | approved | S3-193774 | - |
| S3-193830 | LS on virtualization security assurance | Orange | approved | - | - |
| S3-193831 | Adding Generic assets and threats of GVNP for type 2 into clause 5.2.3.c | China Mobile | approved | S3-193641 | - |
| S3-193832 | Adding Generic assets and threats of GVNP for type 3 into clause 5.2.3.d | China Mobile | approved | S3-193642 | - |
| S3-193833 | Adding security requirements into clause 5.2.4 | China Mobile | approved | S3-193643 | - |
| S3-193834 | Add Clarifications to generic virtualized network product model description | Nokia, Nokia Shanghai Bell | approved | S3-193657 | - |
| S3-193835 | Resolving the ENs in solution #5 | Huawei, Hisilicon, Lenovo, Motorola Mobility | revised | S3-193448 | S3-193846 |
| S3-193836 | Update of Solution #15 | Lenovo, Motorola Mobility | approved | S3-193555 | - |
| S3-193837 | Way forward for KI#6 about the resistance to radio jamming | Ericsson | approved | S3-193608 | - |
| S3-193838 | LS on GUTI re-allocation | Qualcomm | approved | - | - |
| S3-193839 | SA3 meeting calendar | WG Chair | noted | - | - |
| S3-193840 | draft agenda SA3#97 | WG Chair | noted | - | - |
| S3-193841 | draft CR of AKMA architecture | China Mobile, Nokia, Nokia Shanghai Bell | approved | S3-193771 | - |
| S3-193842 | conclusion on key issue #2 | China Mobile | approved | S3-193767 | - |
| S3-193843 | Protection of UE capability transfer for UEs without AS security | Intel | approved | S3-193704 | - |
| S3-193844 | LS on Signalling overload due to malicious Applications on UE | KPN | approved | S3-193710 | - |
| S3-193845 | New solution (SERSI – SERving network controlled SI signatures). | Ericsson | approved | S3-193755 | - |
| S3-193846 | Resolving the ENs in solution #5 | Huawei, Hisilicon, Lenovo, Motorola Mobility | approved | S3-193835 | - |
| S3-193847 | Draft CR as a living baseline for 5GS LCS normative work | Ericsson LM | approved | S3-193702 | - |
| S3-193848 | Draft CR: UP security policy for URLLC | Ericsson | approved | S3-193701 | - |
| S3-193849 | Add Clarifications to the Scope of Accreditation for 3GPP VNPs | Nokia, Nokia Shanghai Bell | approved | S3-193649 | - |
| S3-193850 | draft TR 33.846 | Ericsson | approved | - | - |
| S3-193851 | Updates to Solution #2.1 on MT functionality | Samsung | revised | S3-193788 | S3-193855 |
| S3-193852 | Key issue on removal of UICC card in IAB node | Huawei, Hisilicon | noted | S3-193793 | - |
| S3-193853 | Proposed conclusion of security for groupcast | Qualcomm Incorporated | approved | S3-193805 | - |
| S3-193854 | Reply to: LS on NR V2X Security for user plane data and PDCP SN size | LG | approved | - | - |
| S3-193855 | Updates to Solution #2.1 on MT functionality | Samsung | noted | S3-193851 | - |
| S3-193856 | Conclusion to Key Issue#2.1 on authentication and authorization of the IAB Node (gNB-DU) | Samsung | noted | S3-193792 | - |
| S3-193857 | draft CR for Security procedures for Network Slices | Nokia, Nokia Shanghai Bell | approved | S3-193738 | - |

## Annex B: Lists of liaisons

### B1: Incoming liaison statements

| Document | Original | Title | From | Decision | Reply TDoc |
| --- | --- | --- | --- | --- | --- |
| S3-193660 |  | LS on short MAC-I and ngKSI for 5G-CIoT | C1-195199 | replied to | S3-193715 |
| S3-193661 |  | LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update | C4-193790 | replied to | S3-193682 |
| S3-193662 |  | LS on SUCI computation from an NSI | CP-192262 | postponed | (none) |
| S3-193663 |  | PEI for FN-RG Recommendation | BBF | noted | (none) |
| S3-193664 |  | General Status of Work | BBF | noted | (none) |
| S3-193665 |  | LS on NR V2X Security for user plane data and PDCP SN size | R2-1911681 | replied to | S3-193854 |
| S3-193666 |  | Reply to LS on Impersonation Attacks in 4G Networks | R2-1911819 | noted | (none) |
| S3-193667 |  | Reply LS on protection of PC5-RRC messages for sidelink unicast communication | R2-1911863 | noted | (none) |
| S3-193668 |  | Reply LS on LTE-M identification in 5GC | R3-194748 | noted | (none) |
| S3-193669 |  | LS on the IAB-indication to core network | R3-194787 | noted | (none) |
| S3-193670 |  | N9HR Regulatory Obligations | S3i190548 | postponed | (none) |
| S3-193680 |  | LS on security consideration of performance measurement function protocol | C1-196940 | postponed | (none) |

### B2: Outgoing liaison statements

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| S3-193682 | Reply to: LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update | CT4 | CT1, CT6, SA2 | S3-193661 |
| S3-193715 | Reply to: LS on short MAC-I and ngKSI for 5G-CIoT | CT1 | - | S3-193660 |
| S3-193724 | LS on token-based authorization for indirect communication with delegated discovery (Scenario D) | SA2 | - | - |
| S3-193802 | LS on PC5S and PC5 RRC unicast message protection | SA2,RAN2,CT1 | - |  |
| S3-193830 | LS on virtualization security assurance | GSMA SECAG | - |  |
| S3-193838 | LS on GUTI re-allocation | CT1 | - |  |
| S3-193844 | LS on Signalling overload due to malicious Applications on UE | SA2 | - | - |
| S3-193854 | Reply to: LS on NR V2X Security for user plane data and PDCP SN size | RAN2 | - | S3-193665 |

## Annex C: List of draft Technical Specifications and Reports

| Document | Spec | vers | Doc title |
| --- | --- | --- | --- |
| S3-193566 | 33.845 | 0.0.0 | Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0 |
| S3-193582 | 33.535 | 0.0.1 | skeleton of AKMA TS |
| S3-193693 | 33.853 | 0.6.0 | Draft TR 33.853 |
| S3-193703 | 33.861 | 1.4.0 | draft TR 33.861 |
| S3-193708 | 33.819 | 1.3.0 | draft TR 33.819 |
| S3-193729 | 33.855 | 1.8.0 | Draft TR 33.855 |
| S3-193732 | 33.813 | 0.7.0 | draft TR 33.813 |
| S3-193744 | 33.845 | 0.0.0 | Draft TR 33.845 Storage of sensitive credentials in 5G systems v0.0.0 |
| S3-193745 | 33.845 | 0.1.0 | draft TR 33.845 |
| S3-193750 | 33.848 | 0.4.0 | Draft TR 33.848 |
| S3-193752 | 33.809 | 0.7.0 | Draft TR 33.809 |
| S3-193762 | 33.835 | 1.1.0 | draft TR 33.835 |
| S3-193769 | 33.535 | 0.0.1 | skeleton of AKMA TS |
| S3-193781 | 33.818 | 0.5.0 | Draft TR 33.818 |
| S3-193794 | 33.836 | 0.4.0 | draft TR 33.836 |
| S3-193807 | 33.824 | 0.5.0 | draft TR 33.824 |
| S3-193817 | 33.535 | 0.1.0 | Draft TS 33.535 |
| S3-193850 | 33.846 | 0.4.0 | draft TR 33.846 |

## Annex D: List of participants

| TITLE | Family Name | Given Name | Employer Organization | Employer Category Code | Organization Represented | Organization Represented Category Code |
| --- | --- | --- | --- | --- | --- | --- |
| Ms. | Andersdotter | Amelia | ARTICLE19 | ETSI | ARTICLE19 | ETSI |
| Mr. | Bakker | John-Luc | BlackBerry UK Limited | ETSI | BlackBerry UK Limited | ETSI |
| Dr. | Ben Henda | Noamen | Ericsson LM | ETSI | Nanjing Ericsson Panda Com Ltd | CCSA |
| Mr. | Bernsen | John | Philips International B.V. | ETSI | Philips International B.V. | ETSI |
| Mr. | Blanchard | Colin | BT plc | ETSI | BT plc | ETSI |
| Mr. | Brusilovsky | Alec | InterDigital, Inc. | ETSI | InterDigital Communications | ATIS |
| Mr. | Cano Soveri | Mirko | ETSI | ETSI | ETSI | ETSI |
| Mr. | Canterbury | Mark | Tencastle Limited |  | National Technical Assistance | ETSI |
| Mr. | Castagno | Mauro | TELECOM ITALIA S.p.A. | ETSI | TELECOM ITALIA S.p.A. | ETSI |
| Dr. | Chen | Chiung-jang | CHTTL | ETSI | CHTTL | ETSI |
| Mr. | Cichonski | Jeffrey | NIST | ATIS | NIST | ATIS |
| Dr. | Corbett | Cherita | Johns Hopkins University APL | ATIS | Johns Hopkins University APL | ATIS |
| Ms. | Deng | Juan | HuaWei Technologies Co., Ltd | CCSA | HUAWEI Technologies Japan K.K. | ARIB |
| Mr. | Doerr | Johannes | BMWi | ETSI | BMWi | ETSI |
| Mr. | Dolly | Martin | AT&T | ATIS | AT&T GNS Belgium SPRL | ETSI |
| Miss | Driscoll | Florence | NCSC | ETSI | NCSC | ETSI |
| Mr. | Ennesser | Fran?ois | Huawei Technologies France | ETSI | Huawei Technologies France | ETSI |
| Dr. | Escott | Adrian | Qualcomm CDMA Technologies | ETSI | Qualcomm CDMA Technologies | ETSI |
| Mr. | Evans | Tim P. | VODAFONE Group Plc | ETSI | Vodafone España SA | ETSI |
| Mr. | Everett | Jared | Johns Hopkins University APL | ATIS | Johns Hopkins University APL | ATIS |
| Mr. | Feng | Cheng | Datang Mobile Com. Equipment | CCSA | Datang Mobile Com. Equipment | CCSA |
| Mr. | Gamishev | Todor | Orange | ETSI | Orange | ETSI |
| Mr. | Gautam | Sheel Priya | Department of Telecom | TSDSI | Department of Telecom | TSDSI |
| Mr. | Goldberg | Martin | U.S. Department of Defense | ATIS | U.S. Department of Defense | ATIS |
| Dr. | Granboulan | Louis | Airbus DS SLC | ETSI | Airbus DS SLC | ETSI |
| Miss | Guo | Ivy | Apple (UK) Limited | ETSI | Apple Poland Sp. z.o.o. | ETSI |
| Mr. | Guo | Longhua | HUAWEI TECH. GmbH | ETSI | HiSilicon Technologies Co. Ltd | CCSA |
| Mr. | Gupta | Varini | Samsung R&D Institute India | TSDSI | Samsung R&D Institute UK | ETSI |
| Mr. | Hanhisalo | Markus | Ericsson LM | ETSI | L.M. Ericsson Limited | ETSI |
| Mr. | Heldenbrand | Rob | Hewlett-Packard Enterprise | ETSI | Hewlett-Packard Enterprise | ETSI |
| Miss | Huang | Xiaoting | China Mobile Com. Corporation | CCSA | China Mobile M2M Company Ltd. | CCSA |
| Miss | Jerichow | Anja | Nokia Germany | ETSI | Nokia Uk | ETSI |
| Dr. | Jost | Christine | Ericsson LM | ETSI | Ericsson-LG Co., LTD | TTA |
| Dr. | Keesmaat | Iko | TNO | ETSI | KPN N.V. | ETSI |
| Dr. | Kim | Joonwoong | LG Electronics France | ETSI | LG Electronics Polska | ETSI |
| Mr. | Kiss | Krisztian | Apple (UK) Limited | ETSI | Apple France | ETSI |
| Mr. | Kohalmi | Steve | Juniper Networks | ETSI | Juniper Networks | ETSI |
| Mr. | Kolekar | Abhijeet | Intel Corporation (UK) Ltd | ETSI | Intel Deutschland GmbH | ETSI |
| Mr. | Kumar | Lalith | Samsung R&D Institute India | TSDSI | Samsung Electronics GmbH | ETSI |
| Dr. | Kunz | Andreas | Motorola Mobility Germany GmbH | ETSI | Lenovo (Beijing) Ltd | CCSA |
| Mr. | Lazara | Dominic | Motorola Solutions UK Ltd. | ETSI | Motorola Solutions Danmark A/S | ETSI |
| Mr. | Leadbeater | Alex | BT plc | ETSI | BT plc | ETSI |
| Dr. | Lee | Soo Bum | Qualcomm Incorporated | ATIS | Qualcomm Wireless GmbH | ETSI |
| Mr. | Lee | Xiaoyang | CISA ECD | ATIS | CISA ECD | ATIS |
| Dr. | Lei | Zander (Zhongding) | HuaWei Technologies Co., Ltd | CCSA | Huawei Technologies Japan K.K. | TTC |
| Mr. | LI | HAIMING | China Mobile Com. Corporation | CCSA | China Mobile E-Commerce Co. | CCSA |
| Mr. | Li | He | Huawei Technologies Co. Ltd. | ETSI | Huawei Telecommunication India | TSDSI |
| Mr. | Li | Weiguang | Qihoo 360 | CCSA | Qihoo 360 | CCSA |
| Mr. | Li | Zhijun | ZTE Corporation | ETSI | ZXNE | CCSA |
| Mr. | Liao | Jun | China Mobile Com. Corporation | CCSA | China Mobile (Hangzhou) Inf. | CCSA |
| Mr. | Liu | Yuze | ZTE Corporation | CCSA | ZTE Wistron Telecom AB | ETSI |
| Mr. | Lu | Fei | ZTE Corporation | ETSI | ZTE Photonics | CCSA |
| Miss | Lu | Wei | Nokia Korea | TTA | Nokia Hungary | ETSI |
| Mr. | Mellqvist | Anders | Sony Europe Limited | ETSI | Sony Europe Limited | ETSI |
| Mr. | Migaldi | Scott | T-Mobile USA Inc. | ATIS | T-Mobile USA Inc. | ATIS |
| Dr. | Muhanna | Ahmad | Futurewei Technologies | ATIS | Futurewei Technologies | ATIS |
| Mr. | Nair | Suresh | Nokia Germany | ETSI | Nokia | ATIS |
| Mr. | Normann | Henrik Andreas | Ericsson LM | ETSI | Ericsson Telecomunicazioni SpA | ETSI |
| Mr. | Oishi | Tateo | Sony Europe B.V. | ETSI | Sony Corporation | ARIB |
| Mr. | Palanigounder | Anand | Qualcomm UK Ltd | ETSI | Qualcomm Finland RFFE Oy | ETSI |
| Mr. | Parambath Sasi | Nivedya | NEC Corporation | ETSI | NEC Telecom MODUS Ltd. | ETSI |
| Mr. | Pätzold | Thomas | Deutsche Telekom AG | ETSI | Deutsche Telekom AG | ETSI |
| Mrs. | Pauliac | Mireille | THALES | ETSI | THALES | ETSI |
| Mr. | PENG | Jin | ZTE Corporation | ETSI | Nubia Technology Co.,Ltd | CCSA |
| Mr. | Qi | Minpeng | China Mobile Com. Corporation | CCSA | China Mobile Group Device Co. | CCSA |
| Mr. | Rajadurai | Rajavelsamy | Samsung R&D Institute UK | ETSI | Samsung R&D Institute India | TSDSI |
| Mr. | RAMASWAMY | BABU SRINIVASA KUMAR | Department of Telecom | TSDSI | Department of Telecom | TSDSI |
| Mr. | Rezaki | Ali | Nokia Germany | ETSI | Nokia Poland | ETSI |
| Mrs. | Rong | Wu | Huawei Technologies Co. Ltd. | ETSI | Huawei Technologies Co. Ltd. | ETSI |
| Mr. | RV | ANIKETHAN | Samsung R&D Institute India | TSDSI | Samsung Electronics Co., Ltd | TTA |
| Mr. | Schroeder | Stefan | Deutsche Telekom AG | ETSI | Deutsche Telekom AG | ETSI |
| Mr. | Schumacher | Greg | SPRINT Corporation | ETSI | Sprint Corporation | ATIS |
| Mr. | Shah | Sapan | Samsung R&D Institute India | TSDSI | Harman GmbH | ETSI |
| Dr. | Shah | Yogendra | Perspecta Labs Inc. | ATIS | Perspecta Labs Inc. | ATIS |
| Mr. | Tangudu | Narendranath Durga | Samsung R&D Institute India | TSDSI | BEIJING SAMSUNG TELECOM R&D | CCSA |
| Mr. | Thewes | Simon | ZITiS | ETSI | ZITiS | ETSI |
| Mr. | Tiwari | Kundan | Samsung R&D Institute India | TSDSI | SAMSUNG R&D INSTITUTE JAPAN | ARIB |
| Ms. | Trakinat | Jean | T-Mobile USA Inc. | ATIS | T-Mobile USA Inc. | ATIS |
| Dr. | Tsiatsis | Vlasios | Ericsson LM | ETSI | Ericsson India Private Limited | TSDSI |
| Mr. | Vujcic | Dragan | IDEMIA | ETSI | IDEMIA | ETSI |
| Dr. | Wan | Tao | CableLabs | ETSI | CableLabs | ETSI |
| Miss | Wang | Haimei | CAICT | CCSA | CAICT | CCSA |
| Mr. | Whorlow | Colin | NCSC | ETSI | HOME OFFICE | ETSI |
| Mr. | Wong | Curt | Charter Communications, Inc | ATIS | Charter Communications, Inc | ATIS |
| Mr. | Wong | Marcus | Futurewei Technologies | ATIS | Futurewei Technologies | ATIS |
| Dr. | Wong | Stan | GSM Association | OTHER | GSM Association | OTHER |
| Mr. | Woodward | Tim | Motorola Solutions Danmark A/S | ETSI | Motorola Solutions Germany | ETSI |
| Miss | Xu | Hui | CATT | ETSI | CICT | CCSA |
| Miss | Yan | Xiaojian | ZTE Corporation | ETSI | ZONSON | CCSA |
| Mrs. | Yang | Hongmei | CAICT | CCSA | CAICT | CCSA |
| Mr. | Yoshizawa | Taka | NEC Europe Ltd | ETSI | NEC Corporation | ETSI |
| Dr. | Zhang | Bo | Huawei Technologies Co. Ltd. | ETSI | HuaWei Technologies Co., Ltd | CCSA |
| Miss | Zhang | Xiaowei | Deutsche Telekom AG | ETSI | Deutsche Telekom AG | ETSI |
| Ms. | ZHAO | BEI | China Mobile Com. Corporation | CCSA | CMDI | CCSA |
| Mr. | Zhou | Wei | CATT | CCSA | CATT | CCSA |
| Miss | Zhou | Xingyue | ZTE Corporation | ETSI | ShenZhen Zhongxing Shitong | CCSA |
| Dr. | Zugenmaier | Alf | NTT DOCOMO INC. | TTC | DOCOMO Communications Lab. | ETSI |

## Annex E: List of future meetings

| Title | Start date | End date (OP) | Town | Country | Reference |
| --- | --- | --- | --- | --- | --- |
| SA3#97 | 2019-11-18 | 2019-11-22 | Reno | US | S3-97 |
| SA3#98 | 2020-02-10 | 2020-02-14 | Ghuanzhou | CN | S3-98 |