3GPP TSG-SA3 Meeting #96	S3-192531

# Wroclaw, Poland, 26th Aug 2019 - 30th Aug 2019

# Source:	Juniper Networks

# Title:	Resolving EN in TR33.855 6.18 N9 NDS/IP

# Document for:	Approval

Agenda Item:	8.1 FS_SBA_Sec

## 1	Decision/action requested

It is expected that operators will create separate IPsec SAs with protection profiles specific for the requirements of each network slice.  The correlation of IPsec SAs to N9 network slice interfaces will be controlled by operator policy. Added text accordingly and deleted the EN.

## 2	Detailed proposal

*** First Change ***

## 6.18	Solution #18: Support NDS/IP on the inter-PLMN N9 interface

### 6.18.1	Introduction

For the protection of the non-SBA 5GC internal interfaces, such as N4 and N9, TS33.501 clause 9.9 already requires the use of NDS/IP.  For confidentiality, integrity, and replay protection of inter-PLMN UP, such as N9, NDS/IP shall also be used as specified in TS33.210.  This can be simply achieved by minor modifications of TS33.501 section 9.9, as shown in the highlighted text below.

### 6.18.2	Solution details

The following clause is a direct copy from TS33.501; with yellow highlights indicating the proposed changes.

9.9        Security mechanisms for non-SBA interfaces internal to the 5GC and between PLMNs

Interfaces internal to the 5G Core such as N4 and N9 and roaming interfaces between PLMNs such as N9, can be used to transport signalling data as well as privacy sensitive material, such as user and subscription data, or other parameters, such as security keys. Therefore, confidentiality and integrity protection are required.

For the protection of the non-SBA 5GC internal and inter-PLMN interfaces, NDS/IP shall be used as specified in [3], unless security is provided by other means, e.g. physical security.

Network slices can have different security requirements. Creation and transport of traffic from different slices on NDS/IP protected interfaces is up to operator policies.

Network slices can have different security requirements. Creation of NDS/IP interfaces commensurate with requirements of network slices and transport of traffic from different slices on NDS/IP protected interfaces is up to operator policies.

### 6.18.3	Evaluation

The above solution addresses all requirements of key issue #26.

*** End of Changes ***