3GPP TSG-SA3 Meeting #99e 	S3-201246-r2

# e-meeting, 11 -15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0833 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security entities at the perimeter of the 5G Core network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Juniper |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPGF |  |  |  |  |  | Date: |  |  | 15.5.2015 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 5G System architecture introduces security entities in the 5G Core network, but 2 are missing in the general section. Further it needs to be clarified that SEPP is for protection of control plane messages and IPUPS for protection of user plane messages. In addition, the NOTE in 4.2.2 including normative text is reformulated and the term “Inter PLMN” corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct term to “Inter-PLMN” - correction to general part of clause 4.2 including removal of hanging paragraph - Clarification that SEPP is for protection of control plane messages and IPUPS is for protection of user plane messages. - Adding SEPP and IPUPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclearity in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2., 4.2.1, 4.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******** START OF CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

Cell-ID	Cell Identity as used in TS 38.331 [22]

CHO	Conditional Handover

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

CP	Control Plane

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EMSK	Extended Master Session Key

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SECOP	Service COmmunication Proxy

NOTE: This abbreviation is used for disambiguation. 3GPP TS 23.501 [2] uses the abbreviation SCP.SEG	Security Gateway

SEPP	Security Edge Protection Proxy

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

**** NEXT CHANGE

## 4.2	Security at the perimeter of the 5G Core network

### 4.2.1	Security Edge Protection Proxy (SEPP)

The 5G System architecture introduces a Security Edge Protection Proxy (SEPP) as an entity sitting at the perimeter of the PLMN for protecting control plane messages.

The SEPP enforces inter-PLMN security on the N32 interface.

### 4.2.2	Inter-PLMN UP Security (IPUPS)

The 5G System architecture introduces Inter-PLMN UP Security (IPUPS) at the perimeter of the PLMN for protecting user plane messages.

The IPUPS is a functionality of the UPF that enforces GTP-U security on the N9 interface between UPFs of the visited and home PLMNs.

NOTE: 	IPUPS can be activated with other functionality in a UPF or activated in a UPF that is dedicated to be used for IPUPS functionality (see also TS 23.501 [2], clause 5.8.2.14).

******** END OF CHANGE