3GPP TSG-SA3 Meeting #99e 	S3-201046

# e-meeting, 11 – 15 May 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0070 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TLS 1.3 cipher suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptPr |  |  |  |  |  |  | Date: |  | 2020-04-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S3-200332 (CR 0065) added to the TLS 1.2 profile that only cipher suites with AEAD (e.g. GCM) and PFS (e.g. ECDHE, DHE) shall be supported. The same addition is necessary for the TLS 1.3 profile to align with TLS 1.2 profile. 3GPP should support "TOP SECRET" security and so recommend ecdsa_secp384r1_sha384. Typo "extention" instead of "extension". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text added that 3GPP nodes shall only support AEAD and PFS cipher suites in TLS 1.3 Text added to recommend ecdsa_secp384r1_sha384. Corrected typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Critical 3GPP systems might use non-recommended cipher suites leading to weaknesses |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***	BEGIN CHANGES	***

## 6.2.2	Profiling for TLS 1.3

TLS 1.3 shall support the following restrictions and extensions:

TLS cipher suites and Diffie-Hellman groups

-	The requirements given in section 9.1 of TLS 1.3 RFC 8446 [66] shall be followed. In addition:

-	Key exchange with secp384r1 should be supported.

-	Only cipher suites with AEAD (e.g. GCM) and PFS (e.g. ECDHE, DHE) shall be supported.

TLS signature schemes

-    ecdsa_secp384r1_sha384 should be supported.

TLS extensions

-	The requirements given in section 9.2 of TLS 1.3 RFC 8446 [66] shall be followed. In addition:

-	The OCSP Status extension (a.k.a. certificate status request), as defined in RFC 6066 [57] and RFC 8466 [66] should be supported.

***	END OF CHANGES	***