3GPP TSG-SA3LI, SA3#45LI   	SA3LI12_062r1

24-26 April 2012, Bratislava, Slovakia

Source:	Alcatel Lucent, Rogers Wireless

Title:	Evaluation of LI solutions for IMS Media Plane Security based on MIKEY-IBAKE

Document for:	Discussion and decision

Agenda Item:

Work Item / Release:	LI Rel-11

## Introduction

This contribution proposes text for evaluation of LI solutions for IMS media plane security based on MIKEY-IBAKE.

## Proposal

# 5	Candidate LI solutions for MIKEY-IBAKE based IMS media plane security

## 5.x Evaluation

In this section the proposed solutions are evaluated  against the LI requirements for encrypted services as specified in clause 5.7 of TS 33.106 [z].

It needs to be noted that the selected LI solution becomes an integral part of the MIKEY-IBAKE protocol in order to satisfy LI requirements, and testing of the UE ensuring compliance with these requirements may be necessary.

As previously described, for all four LI solutions the interception is activated once Law Enforcement Agency (LEA) provides the intercept request. This request in the LI methods described above is sent from Law Enforcement Management Function (LEMF) to the Administration Function (ADMF). The network provides access to the intercepted Content of Communications (CC) and the Intercept Related Information (IRI) of the mobile target and services related to the target (e.g. Call Forwarding) on behalf of Law Enforcement Agencies (LEAs). CC and IRI are sent from the Delivery Function to the LEMF. Therefore, lawful interception for encrypted services takes place as for an unencrypted communications (see clause 7, 7A and 12 of TS 33.107). In addition, delivery function can either decrypt the CC or can provide the decryption keys and any required associated information to the LEMF.

In all four solutions described above, the Interception is performed in such a manner that it cannot be detected by the Target or others. Call setup is not affected by an ongoing interception, thus there is no difference in latency during call setup or during communications compared to a non-intercepted communications. In addition, use of key exchange applications which provide a user key confirmation mechanisms are not prevented by the use of interception. Finally, given that interception of the target as described above affects neither call setup nor the call itself, in case interception fails during a call (or during call setup), the call is unaffected.

If the Communication Service Provider provides decryption of the communication, for all LI approaches described above the operator can choose where in the network this decryption is performed. Nevertheless, following this decryption, all IRI and CC is available in the Delivery Function and can be provided to the LEMF using handover mechanisms as per an unencrypted communication (see clause 7, 7A and 12 of TS 33.107).

All four LI solutions for MIKEY-IBAKE described above rely on information contained in SIP messages executed during the call setup with the Target of interception. Therefore, commencement of interception and decryption of an existing communication is possible if SIP messages executed during the call setup with the Target of interception as well as other session related information is available. If these SIP messages are no longer available, the forced or periodic rekeying could be executed.

Finally, as stated above if the key material and the associated information are available, for all four described solutions it is possible to retrospectively decrypt encrypted communications.

The table below summarizes the evaluation of the proposed LI approaches against the LI requirements for encrypted services specified in clause 5.7 of TS 33.106 [4].

| LI Approaches Requirements | Consolidated PRG | Split PRG | Consolidated Nonces | Split Nonces |
| --- | --- | --- | --- | --- |
| 1. When an encryption service is provided by the PLMN, lawful interception shall take place as for an unencrypted communications. | ✔ | ✔ | ✔ | ✔ |
| 1. a. In addition, encrypted communications shall be decrypted, or the decryption keys and any required associated information (e.g. roll over counters) shall be provided to the LEMF. | ✔ | ✔ | ✔ | ✔ |
| 1. b. For the specific case where a key server based solution is used, it is a national option for the operator to make keys and any associated information (e.g. roll over counters) directly available to the LEMF for the decryption of communications. | ✔ | ✔ | ✔ | ✔ |
| 2. Interception shall be performed in such a manner as to avoid detectability by the Target or others. | ✔ | ✔ | ✔ | ✔ |
| 2. a. There shall be no significant difference in latency during call setup or during communications compared to a non-intercepted communications. | ✔ | ✔ | ✔ | ✔ |
| 2. b. Interception of a Target shall not prevent the use of key exchange applications which provide a user key confirmation mechanism. | ✔ | ✔ | ✔ | ✔ |
| 2. c. Should interception fail during a call (or during call setup), the call shall be unaffected. | ✔ | ✔ | ✔ | ✔ |
| 3. Where the CSP provides decryption of the communication, it is the operator’s choice where in the network this decryption is performed. However, following decryption, all IRI and CC shall be provided to the LEMF using handover mechanisms as per an unencrypted communication. | ✔ | ✔ | ✔ | ✔ |
| 4. An encryption solution shall not prohibit commencement of Interception and decryption of an existing communication. | x* | x | x* | x* |
| 5. If key material and any associated information are available, it shall be possible to retrospectively decrypt encrypted communications. | ✔ | ✔ | ✔ | ✔ |

* Non-compliance with this requirement applies to all media plane security solutions, and is not specific to MIKEY-IBAKE. It may be possible to address this requirement through e.g.: rekeying.