Source:	TPS

Title:	Telepresence use case for the Living Document to study LI solutions for Mobile Cloud Services.

Document for:	Discussion and decision

Agenda Item:

Work Item / Release:	Cloud LI Living Document

## Introduction

This contribution proposes two revised Telepresence use cases for addition to the Living Document to study LI solutions for Mobile Cloud Services. This is a revision of SA3LI12_055 based on comments/discussions at the last meeting.

## Background

Telepresence is marketed as the next generation video teleconferencing service, which provides a level of realism, using high resolution video (e.g., 1080p) synchronized seamless with true fidelity sound over a high quality of service (e.g., little to no delay, delay variation/jitter, and packet loss) connection.  ATIS’s Cloud Services Forum (CSF) is working on a Draft Technical Report on Cloud Framework for Telepresence Services which includes the following statement:  “The telepresence service is a managed, hosted or premise based delivery of high definition video services.  A telepresence service is a component of the broader set of services enabling unified communications for an enterprise or consumer.  Unified communications refers to services such as presence, instant messaging, document and personal video sharing, telephony, unified inbox services such as voice mail, blogging, web conferencing and other elements of what it takes to provide a portfolio of collaborative services for an enterprise or consumer.”

3GPP, in TS 22.228, defines an IMS-based Telepresence as: “a conference with interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.”, where a conference is “an IP multimedia session with two or more participants. Each conference has a “conference focus”. A conference can be uniquely identified by a user. Examples for a conference could be a Telepresence or a multimedia game, in which the conference focus is located in a game server.”

Although SA3-LI has not yet discussed Telepresence with regards to specific LI requirements, 3GPP defines it as “a conference”, and “IMS conferencing” is covered in the 3GPP LI specifications.  Many cloud services are heavily on the information systems “side” of the spectrum; telepresence is a telecommunications service, which can be offered via “The Cloud”.

## Discussion

TSPs who offer telecommunications services, regardless of the network architecture used to deliver that service to the subscriber, must still be able to meet their (national/regional/local) legal obligations to support Law Enforcement.

The introduction of cloud architecture does not abrogate the TSP’s legal obligation.  “The Cloud” may provide a TSP with an opportunity to decide routing and control of communications with regards to existing intercept access points.  It may provide a TSP with implementation options which may impact either the availability or the veracity of communications or service logic information.  This contribution does not address carrier implementation options and their possible impact on availability for Law Enforcement or the level of trust of the information.

Questions that should be considered:

- Does “The Cloud” architecture impact the TSP’s ability to meet their legal obligations?

- Is the security of the LI solution compromised by the TSP’s cloud implementation?

  - Does a TSP’s implementation of cloud services allow for access by unauthorized personnel (e.g., personnel who do not have authorization to be aware of LI functionality or operations) or introduce vulnerabilities that may expose the unobtrusiveness of LI activities?

  - Does the TSP’s cloud services implementation provide knowledge or indications to any party to the communications?

  - If security mechanisms currently used (authentication, encryption, log files, etc.) are no longer viable for cloud services, the TSP still needs to meet the national/regional/local requirements for security.  TSP’s need to consider their responsibilities for audits and log files when implementing a cloud service architecture.

While it may be worthwhile holding these types of discussions in a standards forum, are these the types of questions that can be answered in technical specifications?

Telepresence is typically marketed as an “enterprise” type of communication, but that does not preclude the option of TSPs offering it to non-enterprise subscribers.  In the use case(s) described below the telepresence service discussed is a non-enterprise service offering.

The following outlines two Telepresence Service use cases. The first depicts when all the participants subscribe to their TSP’s Telepresence service.  The second is when the Telepresence service is not offered by the TSP(s) involved, but by a Third-Party (e.g., separate IMS provider). In this case, the participants may belong to the same or different TSPs.  For simplicity, all mobile participants are in their home network (i.e., roaming scenarios are not provided for discussion).

## **********************First addition******************************

- Telepresence use case 1:  TSP offers Telepresence and all participants are subscribers of the TSP.

  - Overview

This use case describes basic telepresence service.  A TSP offers Telepresence and all the participants are subscribers to that TSP.

  - Actors

The users are Jean (the subject of the lawful interception), and her two associates:  Greg and Peter.

McCloud is the mobile TSP/cloud service provider providing a Telepresence Service.

The Alleghany County Police department is the LEA authorized to perform the electronic surveillance.

  - Preconditions

Jean, Greg and Peter all are subscribers of the McCloud Telepresence services.

The Alleghany County Police Department has obtained the lawful authorization to perform electronic surveillance on Jean, who is suspected of involvement with some illegal activity.

The Allegany County Police Department has provided McCloud with this lawful authorization for assistance in intercepting Jean’s communications, which includes use of McCloud’s Telepresence service.

McCloud is able to deliver intercepted communications of its Telepresence Service to authorized LEAs.

Jean, Greg and Peter all are using UEs which are  active on the McCloud network.  However each has different video display capabilities.

  - Actions

    - Jean initiates Telepresence (conference) session with Greg and Peter.

    - Jean, not having extensive video administration experience, relies heavily on the McCloud Help Desk Service for assistance in using the service (e.g., to initiate and troubleshoot problems) while Peter and Greg join her in the Telepresence meeting.

    - Peter and Greg accept the invitation to join the Telepresence session and are added to the conference.

    - The McCloud surveillance facilities identify that a target of LI has initiated communication covered by the LI authorization and begins delivery of the communication to the LEA.

    - The Alleghany County Police Department begins receiving Jean’s intercepted communications (i.e., IRI and CC for all required services as identified in the lawful authorization).  The IRI and CC for the Telepresence service is delivered separately and the CC is the media that is sent to/from Jean (the Subject).

    - Jean, Peter, and Greg discuss the criminal activities they are planning, showing maps and pictures of the target crime venue (e.g., bank and surrounding streets for their “get-away”).  Greg shows a short silent movie clip of the bank guards to show their routine and guard positions.

    - McCloud’s Telepresence Service adapts the video delivery to each UE based on the UE capabilities and for network optimization.

    - Peter’s UE display is small and he is unable to see the important details of the video that Greg is sharing, since he is unable to zoom in for a closer view

    - The Telepresence session ends,

  - Results

    - Jean, Greg and Peter have finalized their plans for illegal activity and were unaware that any LI had occurred on their Telepresence session.

    - The Alleghany County Police Department received the IRI and CC of Jean’s communications.  Using that information in their investigations, they prevent a crime from occurring.

    - McCloud met their regulatory obligation to unobtrusively deliver communication to the authorized LEA.

  - LI Discussion/Challenges

    - McCloud has a legal obligation to provide LI for the Telepresence session.  Whether the video is provided to Law Enforcement is a national option.  At a minimum, the audio of this conference, as well as the IRI is required to be reported, as Telepresence is a conference per 3GPP definition.  The exact set of events and information is outside the scope of this use case (as SA3-LI has not yet discussed/agreed/defined LI for Telepresence).

    - The identities of the participants are known to McCloud, as they are all subscribers to McCloud’s Telepresence service.  The McCloud Telepresence Service has the service logic for Jean’s Telepresence session, the identities of the participants, and access to the media.

    - As with any other service lawfully intercepted, if McCloud provides encryption for the Telepresence Service, McCloud is responsible for either decrypting or providing the keys to law enforcement to decrypt.

## **********************Second addition******************************

- Telepresence use case 2:  Telepresence is offered by a Third Party provider.  Participants are subscribers of the same or different TSP(s).

  - Overview

This use case describes basic telepresence service.  The mobile TSP acts as a “cloud carrier” for the cloud provider’s Telepresence service.

  - Actors

The users are Jean (the subject of the lawful interception), and her two associates:  Gabor and Terry.

McCloud is the mobile TSP/cloud carrier.  Jean and Terry are subscribers of McCloud.

McCloud provides Jean with mobile broadband and voice services.

ExcellAlex Mobile is a mobile TSP.  Gabor is a subscriber of ExcellAlex Mobile.

TellyServ is a (third party) cloud service provider providing the Telepresence Service.  TellyServ is NOT offering Telepresence Service on McCloud’s behalf nor does TellyServ have any business relationship with McCloud.

Jean, Terry and Gabor all subscribe to TellyServ’s Telepresence Service.

The Maryland State Police department is the LEA authorized to perform the electronic surveillance.

  - Preconditions

The Maryland State Police has obtained legal authority to perform electronic surveillance on Jean, who is suspected of involvement in illegal activity.

The Maryland State Police has provided McCloud and TellyServ with this lawful authorization for assistance in intercepting Jean’s communications.

McCloud and TellyServ both are able to deliver intercepted communications to authorized LEAs.

The Maryland State Police do not provide ExcellAlex Mobile a lawful authorization as it is not providing a service to Jean (the Subject).

  - Jean and Terry are using UEs that are active on the McCloud network.  Gabor is using a UE that is active on ExcellAlex Mobile.  All three UEs (Jean’s, Terry’s and Gabor’s) are active with TellyServ to use the Telepresence Service.  Actions

    - Jean initiates a Telepresence (conference) session with Gabor and Terry.

    - Jean, not having extensive video administration experience, relies heavily on the TellyServ’s Help Desk Service for assistance in using the service (e.g., to set-up the meeting and troubleshoot problems) while Gabor and Terry join her in the Telepresence meeting.

    - Terry and Gabor accept Jean’s invitation to join the Telepresence session and are added to the conference.

    - McCloud surveillance functions identify that a target of LI has initiated a communication covered by their lawful authorization (i.e., broadband data access, SMS, and VoIP) and begin delivery of the communications to the LEA.  Note some of the TellyServ Telepresence communication is delivered as part of McCloud’s mobile broadband intercepted communications.

    - TellyServ surveillance functions identify that a target of LI has initiated a communication covered by their lawful authorization (i.e., Telepresence) and begin delivery of the communications to the LEA.

    - The Maryland State Police begin receiving Jean’s intercepted communications (i.e., IRI and CC) from both McCloud and TellyServ.

    - Jean, Terry and Gabor begin discussing criminal activities, showing maps and pictures of the target crime venue (e.g., bank and surrounding streets for their “get-away”).  Gabor shows a short silent movie clip of the bank guards to show their routine and guard positions.

    - The telepresence session ends.

  - Results

    - Jean, Terry and Gabor finalized their plans for illegal activity and were unaware that any LI had occurred on their Telepresence session.

    - Jean was also unaware of the LI on her broadband access.

    - The Maryland State Police received the IRI and CC of Jean’s communications. Using that information in their investigations, they prevent a crime from occurring.

    - McCloud met their regulatory obligation to unobtrusively deliver intercepted communications to the authorized LEA.

    - TellyServ met their regulatory obligation to unobtrusively deliver intercepted communications to the authorized LEA.

  - LI Discussion/Challenges

    - McCloud has a legal obligation to provide LI for the services that they offer the target.  In this use case, they do not offer the Telepresence Service, so they are not obligated to provide separate delivery of this service.  Since it is “available” in Jean’s packet data stream, it is delivered as part of McCloud’s packet data interception.  McCloud isolates and reports Jean’s intercepted voice, SMS, and packet data/broadband services per their LI solutions.  McCloud uses the identifiers that are available in their services and network.

    - TellyServ has the legal obligation to provide LI for the services that they offer the target.  In this use case, this is only the Telepresence Service.  They provide the CII and CC for the Telepresence service per their LI solution and using the identifiers that are used in the Telepresence Service.  TellyServ knows the identities of the participants; they have the service logic and access the media.

    - As with any other service lawfully intercepted, if TellyServ provides encryption for the Telepresence Service, TellyServ is responsible for either decrypting or providing the keys to law enforcement to decrypt.

## **********************End of Proposed Additions******************************

## Recommendation

Discuss and adopt the proposed use cases in a new use case section of the cloud services living document.

The document used is CSF-2011-158r13.  It is a draft document.

Release 12.0.0 of 3GPP TS 22.228, Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS); Stage 1, was used.