3GPP TSG-SA3-LI Meeting #51 	SA3LI12_118r1

# Burlington, VT., USA, 22-24, Oct 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 107 LTE IRI Packet Header Information Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (FBI, Ministère du Redressement Productif - France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To provide LTE IRI Packet Header Information reporting to support national requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new events defined to report per packet and summary reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12.1, 12.2.1.1, 12.2.1.2, new 12.2.3.X, 12.3.1.1, 12.3.1.2, new 12.3.3.X, 12.4.1.1, 12.4.1.2, new 12.4.3.X, 12.5.1.1, 12.5.1.2, new 12.5.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]	Void

[5]	Void

[6]	Void

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]	 VOID

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	 Void

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	Void

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[31]	3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[36]	IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ".

[a]	IETF RFC 791:  “Internet Protocol”.

[b]	IETF RFC 2460:  “Internet Protocol, Version 6 (IPv6) Specification”.

[c]	IEFT RFC 3697:  “IPv6 Flow Label Specification”.

*** NEXT MODIFICATION ***

## 12.1	LI functional architecture for EPS

In addition to the reference configurations applicable to PS interception, the following figures contain the reference configuration applicable for the lawful interception in the EPS nodes ([22], [23]):

LEMF

Mediation

Function

Delivery

Function 2

Mediation uuu

Function

ADMFuuu

X2

X1_2

X1_1

HI1

HI2

MME

Figure 12.1.1:	MME Intercept configuration

Figure 12.1.1a:	  Configuration for Intercept of HeNB

Figure 12.1.2:	HSS Intercept configuration

LEMF

Mediation

Function

Delivery

Function 2

Mediation

Function

ADMF

S

-

GW,

PDN

-

GW

X2

X3

X1_2

X1_1

X1_3

HI1

HI2

Mediation

Function

Delivery

Function 3

HI3

X2_3

Figure 12.1.3:	S-GW, PDN-GW Intercept configuration

The definition of the LI functional entities (ADMF, DF, MF, LEMF) and interfaces (X, HI) is the same as for 3G as given in chapter 4. Packet Header Information Reporting is a national option. For Packet Header Information Reporting, a S-GW/PDN-GW either isolates the relevant data and sends it to the DF2 or sends the packet stream to an MF for isolation and the MF provides the relevant data to the DF2.  The interface between the DF2 and MF (called X2_3) is beyond the scope of this specification.

Procedures for LI activation, deactivation and interrogation are the same as for 3G as given in chapter 5, provided that:

- the 3G ICE is replaced by the EPS node;

- the proper target identity applicable to EPS node is used.

When the SGSN is used as node in the Evolved Packet System, to support 2G/3G access and mobility between E-UTRAN and pre-E-UTRAN 3GPP radio access technologies, it is subjected to all the related PS requirements specified throughout this document.

When the ePDG and the AAA server are used as nodes in the Evolved Packet System, to support untrusted Non-3GPP IP Access, they are subjected to all the requirements specified in this document for the PDG and the AAA server for the case of I-WLAN interworking.

Figure 12.1.1a depicts how the HeNB location information is transferred from the HeNB location verifying node per TS 33.320 [34] to the DF2 via an Xv interface, in order to allow the DF2 to perform its functionality.  The public IP Address of the HeNB is provided to the HeNB location verifying node.  The manner that the HeNB location verifying node provides the DF2 with the HeNB location and HeNB IP Address is outside the scope of this document.  Additional information on HeNB interception is found in Clause X.

The target identities for 3GPP HeNB interception can be IMSI, MSISDN, IMEI, or ME Id.  Use of the HeNB ID or the CSG ID as a target identity is FFS.

*** NEXT MODIFICATION ***

#### 12.2.1.1		X2-interface

The following information needs to be transferred from the EPS nodes or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (IMSI, MSISDN, ME identity);

-	events and associated parameters as defined in clause 12.2.1.2 and 12.2.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	correlation number;

-	Quality of Service (QoS) information (if available);

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

For HeNB interception, the MME shall provide in addition the following:

-	HeNB Identity;

-	HeNB location.

HeNB location information needs to be transferred from the HeNB location verifying node to the DF2 in order to allow the DF2 to perform its functionality.

The EPS nodes detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the EPS nodes either directly to the DF2 or via a MF in order to allow the DF2 to perform its functionality.

The IRI should be sent to DF2 using a reliable transport mechanism.

*** NEXT MODIFICATION ***

#### 12.2.1.2		Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the EPC nodes or the HSS and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [22].

The following events are applicable to the MME:

-	Attach;

-	Detach;

- 	Tracking Area Update;

- 		UE requested PDN connectivity;

- 	UE Requested PDN disconnection;

- 	Start of interception with E-UTRAN attached UE.

The following events are applicable to the Serving GW and PDN GW:

-	Bearer activation (valid for both Default and Dedicated bearer);

-	Start of intercept with bearer active;

-	Bearer modification;

-	Bearer deactivation;

-	UE Requested Bearer Resource Modification;

-	Packet Data Header Information.

The following events are applicable to the HSS:

- Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. If interception is performed at the PDN GW, then Packet Header Information Reporting shall also be performed at the PDN GW and not at the Serving GW.

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed ME Id ME Id of the target subscriber (monitored subscriber); when it coincides with the IMEI, it shall be checked for each activation over the radio interface.

Event type Indicates which type of event is delivered: Attach, Detach, Tracking Area Update, UE requested PDN connectivity, UE Requested PDN disconnection, UE Requested Bearer Resource Modification, Bearer activation, Start of intercept with bearer active, Start of interception with E-UTRAN attached UE, Bearer deactivation, Bearer modification, Serving Evolved Packet System, Packet Data Header Information.

Event date Date of the event generation in the ICE.

Event time Time of the event generation in the ICE. Timestamp shall be generated relative to ICE internal clock.

PDN Type The parameter is applicable to the MME only and provides the IP version (IPv4, IPv4/IPv6, IPv6) requested by the UE.

PDN Address Allocation The parameter is applicable to the S-GW and PDN-GW; it provides the IP version (IPv4, IPv4/IPv6, IPv6) and IP address(es) allocated for the UE.

Protocol Configuration Options Are used to transfer parameters between the UE and the PDN-GW (e.g. Address Allocation Preference by DHCP).

Attach type Indicates the type of attach (may carry indication of handover in case of mobility with non-3GPP access).

Location Information Location Information is the Tracking Area Identity (TAI), TA List assigned to the UE, E-CGI and/or location area identity that is present at the node at the time of event record production. In case of Tracking Area Update event, the last visited TAI of the UE may be applicable.

PDN address(es) The UE IP address(es) for the PDN connection.

APN When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the Access Point Name used for the connection.

RAT type The Radio Access Type

APN-AMBR The Aggregate Maximum Bit Rate for the APN.

Handover indication Provides information from the GTPv2 protocol that the procedure is triggered as part of a handover.

Procedure Transaction Identifier Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE.

EPS bearer identity An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN. The EPS Bearer Identity is allocated by the MME.

Bearer activation/deactivation type Indicates the type of bearer being activated/deactivated, i.e. default or dedicated.

Linked EPS bearer identity Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer.

Initiator The initiator of the procedure, either the network, HeNB, or the UE.

Switch off indicator Indicates whether a detach procedure is due to a switch off situation or not.

Detach type Parameter sent by the network to the UE to indicate the type of detach.

Traffic Flow Template (TFT) The EPS bearer traffic flow template (TFT) is the collection of all packet filters associated with that EPS bearer.

Traffic Aggregate Description (TAD) The TAD consists of the description of the packet filter(s) for the traffic flow aggregate.

Serving MME address The address of the serving MME.

Old Location Information Location Information of the subscriber before Tracking Area Update.

Correlation Number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Failed attach reason Reason for failed attach of the target subscriber.

Failed bearer activation reason Reason for failed bearer activation for the target subscriber.

Failed Bearer Modification reason The reason for failure of Bearer Modification.

IAs The observed Interception Areas.

Bearer Deactivation cause The cause of deactivation of the PDP context.

EPS Bearer QoS This field indicates the Quality of Service associated with the Bearer procedure.

Request type Indicates the type of request in an UE requested PDN connectivity, i.e. initial request or handover.

CSG Identity Uniquely identifies a CSG within one PLMN. Note: Open HeNBs do not have associated CSGs.

CSG List Identifies the membership of a given CSG (i.e., CSG Identities and associated expiration data for the UEs).

HeNB Identity Uniquely identifies a HeNB (i.e., HeNB equipment ID and HeNB name).

HeNB IP Address The public IP address of the HeNB provided to the HeNB location verifying node

HeNB Location Reports the location of the HeNB used during location verification.

ISP Operator Identity Identifies the ISP through which the HeNB is connected to the SeGW (e.g., IP address).

Security Gateway IP Address The IP Address of the Security Gateway that terminates the tunnel from the HeNB.

Tunnel Protocol The tunnel protocol used between the HeNB and the SeGW.

Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

Destination Port Number The port number of the destination of the IP packet.

Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]).

Packet Count The number of packets detected and reported (for a particular summary period).

Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet.

Source Port Number The port number of the source of the IP packet.

Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

Summary Period Includes the dates and times of the first and last packets in a particular packet data interval.

Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported.

Editor’s note:  Identification of which HeNB IP Address is reported is needed.

*** NEXT MODIFICATION ***

### 12.2.3.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 12.2.3.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the S-GW/PDN-GW either directly to DF2 or via a MF, if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Location Information

IAs (if applicable)

Initiator

EPS bearer id

Handover indication

PDN Address Allocation

PDN address(es)

APN

Source IP Address

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 12.2.3.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within an EPS bearer,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 5-tuple of source/destination IP address/port number and the layer 4 protocol.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with an EPS bearer

- an interim report for a packet flow associated with an EPS bearer is to be reported

- end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via a MF for each packet flow if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Location Information

IAs (if applicable)

Initiator

EPS bearer id

Handover indication

PDN Address Allocation

PDN address(es)

APN

Care of address

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** NEXT MODIFICATION ***

#### 12.3.1.1	X2 interface

The following information needs to be transferred from the PDN-GW to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.3.1.2 and 12.3.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The PDN-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW either directly to the DF2 or via a MF in order to allow the DF2 to perform its functionality.

The IRI should be sent to DF2 using a reliable transport mechanism.

*** NEXT MODIFICATION ***

#### 12.3.1.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [23].

The following events are applicable to the PDN-GW:

-	PMIP Attach/tunnel activation;

-	PMIP Detach/tunnel deactivation;

-	PMIP session modification

- 	Start of interception with active PMIP tunnel;

-	PMIP PDN-GW initiated PDN-disconnection;

-	Packet Data Header Information.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMEI IMEI of the target subscriber (monitored subscriber)

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP Session modification, Start of interception with active PMIP tunnel, PMIP PDN-GW initiated PDN disconnection.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; it is set to a nonzero value in the case of registration; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the procedure is triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

PDN address(es) The UE IP address(es) for the PDN connection.

Revocation trigger Indicates the reason which triggered the PDN-GW initiated PDN-disconnection procedure

Serving Network Identifies the serving network the UE is attached to

DHCP v4 Address Allocation Indication Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE

Location Information Provides, if received from the PCRF, location information of the target subscriber.

Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

Destination Port Number The port number of the destination of the IP packet.

Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]).

Packet Count The number of packets detected and reported (for a particular summary period).

Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet.

Source Port Number The port number of the source of the IP packet.

Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

Summary Period Includes the dates and times of the first and last packets in a particular packet data interval.

Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported.

*** NEXT MODIFICATION ***

#### 12.3.3.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 12.2.3.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the PDN-GW either directly to DF2 or via a MF if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Additional Parameters

Serving Network

Handover indicator

DHCPv4 Address Allocation Indication

Location information

Source IP Address

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 12.2.3.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within an EPS bearer,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 5-tuple of source/destination IP address/port number and the layer 4 protocol.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with an EPS bearer

- an interim report for a packet flow associated with an EPS bearer is to be reported

- end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via DF3 for each packet flow if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Additional Parameters

Serving Network

Handover indicator

DHCPv4 Address Allocation Indication

Location information

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

Packet Summary Reason

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** NEXT MODIFICATION ***

#### 12.4.1.1	X2-interface

The following information needs to be transferred from the S-GW, PDN-GW or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.4.1.2 and 12.4.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The PDN-GW/S-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW/S-GW either directly to the DF2 or via a MF in order to allow the DF2 to perform its functionality.

The IRI should be sent to DF2 using a reliable transport mechanism.

*** NEXT MODIFICATION ***

#### 12.4.1.2		Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the S-GW, PDN-GW or the HSS and can be suppressed in the DF2.

The following events are applicable to the S-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

-	Start of interception with active PMIP tunnel;

-	Packet Data Header Information.

The following events are applicable to the PDN-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

-	Start of interception with active PMIP tunnel;

-	MIP registration/tunnel activation;

-	DSMIP registration/tunnel activation;

-	DSMIP session modification;

-	MIP deregistration/tunnel deactivation;

-	DSMIP deregistration/tunnel deactivation;

-	Start of interception with active MIP tunnel;

-	Start of interception with active DSMIP tunnel;

-	DSMIP HA Switch;

-	PMIP Resource Allocation Deactivation;

-	MIP Resource Allocation Deactivation;

-	Bearer activation;

- 	Bearer deactivation;

-	Bearer modification;

-	Start of interception with active bearer;

-	Packet Data Header Information.

NOTE: Bearer activation, bearer deactivation, bearer modification and start of interception with active bearer are applicable to trusted non-3GPP access when the GTP protocol is used over s2a interface as specified in 3GPP TS 23.402 [23].

The following event is applicable to the HSS:

-	Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. In case GTP protocol is used over s2a interface, elements from table 12.2.1.2 are included in the applicable events. If interception is performed at the PDN GW, then Packet Header Information Reporting shall also be performed at the PDN GW and not at the Serving GW.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed IMSI The IMSI of the target subscriber (monitored subscriber)

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP session modification, Start of interception with active PMIP tunnel, MIP registration/tunnel activation, DSMIP registration/tunnel activation, DSMIP session modification, MIP deregistration/tunnel deactivation, DSMIP deregistration/tunnel deactivation, Start of interception with active MIP tunnel, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, MIP Resource Allocation Deactivation, Serving Evolved Packet System, Packet Data Header Information.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Session modification failure reason Reason for a failure of a session modification attempt for the target subscriber

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

PDN address(es) The UE IP address(es) for the PDN connection.

Home address Contains the UE Home IP address.

Home Agent address Contains the IP address of the Home Agent.

Requested IPv6 Home Prefix The IPv6 Home Prefix requested by the UE.

IPv6 home prefix The IPv6 home prefix assigned by the PDN GW to the UE.

Care of Address The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

HSS/AAA address The address of the HSS/AAA triggering the PDN-GW reallocation.

Target PDN-GW address The address of the PDN-GW which the UE will be reallocated to.

Revocation trigger Contains the cause for the revocation procedure.

Foreign domain address The relevant IP address in the foreign domain.

Visited network identifier An identifier that allows the home network to identify the visited network [24]

Location Information Location information of the target subscriber, e.g. 3GPP2-BSID [26]. Provided if available from the PCRF.

Initiator The initiator of the procedure, either the network or the UE.

Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

Destination Port Number The port number of the destination of the IP packet.

Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]).

Packet Count The number of packets detected and reported (for a particular summary period).

Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet.

Source Port Number The port number of the source of the IP packet.

Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

Summary Period Includes the dates and times of the first and last packets in a particular packet data interval.

Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported.

*** NEXT MODIFICATION ***

### 12.4.3.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 12.4.3.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the S-GW/PDN-GW either directly to the DF2 or via a MF if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Serving Network

Home address

Care of Address

APN

Location information

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 12.4.3.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within an EPS bearer,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 5-tuple of source/destination IP address/port number and the layer 4 protocol.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with an EPS bearer

- an interim report for a packet flow associated with an EPS bearer is to be reported

- end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to the DF2 or via a MF for each packet flow if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Serving Network

Home address

Care of Address

APN

Location information

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** NEXT MODIFICATION ***

#### 12.5.1.1	X2-interface

The following information needs to be transferred from the PDN-GW or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.5.1.2 and 12.5.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The PDN-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW either directly to the DF2 or via a MF in order to allow the DF2 to perform its functionality.

The IRI should be sent to DF2 using a reliable transport mechanism.

*** NEXT MODIFICATION ***

#### 12.5.1.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW or the HSS and can be suppressed in the DF2.

The following events are applicable to the PDN-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

- 	Start of interception with active PMIP tunnel;

-	DSMIP registration/tunnel activation;

-	DSMIP deregistration/tunnel deactivation;

-	DSMIP session modification;

-	Start of interception with active DSMIP tunnel;

-	DSMIP HA Switch;

-	PMIP Resource Allocation Deactivation;

-	Packet Header Information.

The following event is applicable to the HSS:

-	Serving Evolved Packet System

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed IMSI The IMSI of the target subscriber (monitored subscriber).

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, Start of interception with active PMIP tunnel, DSMIP registration/tunnel activation, DSMIP deregistration/tunnel deactivation, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, Serving Evolved Packet System.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Session modification failure reason Reason for a failure of a session modification attempt for the target subscriber

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

Home Agent address Contains the IP address of the Home Agent.

Care of Address The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

HSS/AAA address The address of the HSS/AAA triggering the PDN-GW reallocation.

Target PDN-GW address The address of the PDN-GW which the UE will be reallocated to.

Revocation trigger Contains the cause for the revocation procedure.

Foreign domain address The relevant IP address in the foreign domain.

Visited network identifier An identifier that allows the home network to identify the visited network [24].

Requested IPv6 Home Prefix The IPv6 Home Prefix requested by the UE.

IPv6 home prefix The IPv6 home prefix assigned by the PDN GW to the UE.

Home address Contains the UE Home IP address.

Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

Destination Port Number The port number of the destination of the IP packet.

Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]).

Packet Count The number of packets detected and reported (for a particular summary period).

Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet.

Source Port Number The port number of the source of the IP packet.

Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

Summary Period Includes the dates and times of the first and last packets in a particular packet data interval.

Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported.

*** NEXT MODIFICATION ***

### 12.5.3.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 12.5.3.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the PDN-GW either directly to the DF2 or via a MF if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Serving Network

Home address

Care of Address

APN

Location information

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 12.5.3.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within an EPS bearer,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 5-tuple of source/destination IP address/port number and the layer 4 protocol.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with an EPS bearer

- an interim report for a packet flow associated with an EPS bearer is to be reported

- end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via DF3 for each packet flow if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Serving Network

Home address

Care of Address

APN

Location information

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** END OF MODIFICATIONS ***