3GPP TSG-SA3LI Meeting #51 	SA3LI13_124r1

# Burlington, VT., USA, 22-24 October 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 107 Separated VoIP Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (FBI) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To separate out VoIP intercept and delivery from other services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new clause X defining new events to report separated VoIP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications as some countries require per service interception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Add new Clause 12.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

Editor's note: 
References 4, 5 & 6 are not cited in the body of the present TS, and anyway they do not exist in this Release.

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122: "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	(void)

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2”

[31]	3GPP TS 22.220:  “Service Requirements for Home NodeBs and Home eNodeBs”.

[32]	3GPP TS 36.300:  Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2”.

[33]	3GPP TS 25.467:  “UTRAN architecture for 3G Home Node B (HNB); Stage 2”

[34]	3GPP TS 33.320:  “Security of Home Node B (HNB) / Home evolved Node B (HeNB)”.

[xx]	3GPP TS 24.229:   “IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3”

[yy]	3GPP TS 24.173:  “IMS multimedia telephony communication service and supplementary services; Stage 3”

*** NEXT MODIFICATION ***

# X	Separated VoIP Interception and Delivery

## X.1 Background

This clause describes a Separated VoIP (SEV) interception and delivery solution. SEV includes any type of VoIP service provided by a service provider including Voice over LTE (VoLTE). It is a national option to require SEV interceptions.   For SEV interceptions, the network operator is required to isolate and deliver the results of a VoIP intercept separate from all other on-going service(s) which the target is using.

SEV interception includes the capability to intercept and deliver VoIP IRI, and if authorized, CC.

The network elements that may be  involved with interception for VoIP include the a VoIP based call/application server such as a Multimedia Telephony Application Server (MMTEL AS), the HSS as well as other network elements that are necessary to intercept the IRI and CC for a VoIP session. The system of network elements are collectively referred to as a VoIP platform in this specification.  The  VoIP platform is involved in intercepting and delivering IRI for SEV sessions and related supplementary services.  The HLR/HSS is used to report a serving (evolved) packet system event.

Based on the lawful authorization and national requirements, a SEV intercept may need to be enabled.

Only the capabilities described in Clause X are to be reported for separated VoIP  interception and delivery (SEVID).  CC to be reported for a VoIP session is addressed in Clause X.3 (Editor’s note – may be addressed in another contribution).

## X.2 IRI for Separated VoIP Interception and Delivery

In addition to the reference configurations in Clause 4 applicable to UMTS and in Clause 12 applicable to EPS interception, the following figure contains the reference configuration applicable for the lawful interception in the VoIP Platform:

LEMF

Mediation

Function

Delivery

Function 2

Mediation uuu

Function

ADMFuuu

X2

X1_2

X1_1

HI1

HI2

VoIP Platform

Figure X.1:	VoIP Platform Intercept configuration

The MMTEL AS is defined in 4.1A.2 of 3GPP TS 24.173 [yy].

The following events are applicable to the SEVID:

Call Related Events:

-	Call Establishment;

-	Answer;

(If needed further discussion/consideration is required)

-	Release;

-  Media Modification and Address Reporting;

-  Subject Signal;

-  Network Signal.

-	Virtual Call Continuity

Supplementary Service Related Events:

-	Supplementary Service;

- Redirection;

Non Call Related Events:

-	Location Update; - Needs further study

-	Subscriber Controlled Input.  – Needs further study

-	IMS VoIP Registration/De-Registration

### X.2.1 IRI for Separated VoIP Interception and Delivery

This clause identifies the information to be reported for each event as viewed by the VoIP Platform.

Table 1: Information Elements for VoIP Event records

Observed MSISDN Target Identifier with the MSISDN of the target subscriber (monitored subscriber).

Observed IMSI Target Identifier with the IMSI of the target subscriber (monitored subscriber).

Observed IMEI Target Identifier with the IMEI of the target subscriber (monitored subscriber), It shall be checked for each call over the radio interface

SIP URI Target Identifier with the SIP URI of the target subscriber (monitored subscriber).

Tel URI Target Identifier with the Tel URI of the target subscriber (monitored subscriber).

Observed IMPU IMS Public User identity (IMPU) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

Observed IMPI IMS Private User identity (IMPI) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

event type Description which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Redirection, Release, Media Modification and Address Reporting, Subject Signal, Network Signal, Location update, Subscriber controlled input

event date Date of the event generation in the EPS Intercepting Node

event time Time of the event generation in the EPS Intercepting Node

Input address information Target dialled or signalled address information before address modification, IN-modification etc. May also include subsequent translation input

Connected party address Address of the answering party

Calling party address Address of the party originating the call.

Called party address Address of the party intended to receive the call.

call direction Information if the monitored subscriber is calling or called e.g. MOC/MTC or originating/ terminating In or/out

Correlation number Unique number for each call sent to the DF, to help the LEA, to have a correlation between each Call and the IRI

Network Element Identifier Unique identifier for the element reporting the ICE.

Location Information Location information is the service area identity and/or location area identity that is present at the EPS Intercepting Node at the time of event record production. In some traffic cases the available location information can be the one received from the MME, i.e. the Tracking Area Identity (TAI) and/or the E-UTRAN Cell Global Identification (ECGI) as specified in the 3GPP TS 23.272 [30].

basic service Information about Tele service or bearer service or media information.

Supplementary service Supplementary services used by the target e.g. CF, CW, ECT

Forwarded to address Forwarded to address at for a call diversion service

call release reason Call release reason of the target call

SCI Non call related Subscriber Controlled Input (SCI) which the MMTEL AS receives from the ME

Forked Calls Identifies the call legs and destinations for forked calls (e.g. forked INVITE)

Transit Carrier ID Identifies the transit carrier who will attempt to connect the call from the target to the called party.

Redirected From Party ID(s) Identifies the Party(ies) which invoked call redirection towards the target. This is provided if available.

Redirected To Party ID Identifies the Party to whom is being redirected by the target.

Visited System ID Identifies the system to which the call has been redirected, when a call to a target is redirected to another carrier and the system identity of that carrier is known. Examples include: “sip.provider.com” and “2025551234.”

Contact Address(es) An address(es) signalled by a target to the other party or vice versa.

Target Contact Address(es) An address(es) signalled by a target to the other party.

Other Party Contact Address(es) An address(es) signalled by the other party to the target.

Signalling Party ID Identifies the Target.

Signalled To Party Id Identifies the Other Party (e.g., result of any final translation).

User Signal Identifies the dialling or signalling by the intercept subject to control a feature or service.

Refer Identifies any SIP Refer information that may be available.

Network Signal Identifies the signal, tone or message initiated, generated or sent by the VoP network toward the target, or the signal, tone or message reported to the VoP network as having been initiated, generated or sent toward the target.

Request Identity Identifies an address registration or deregistration request within a system.

Address Registration Type Indicates whether an address registration, address deregistration, or both were detected.

Registering Party Identity Identifies the party for whom address registration, deregistration, or both, are being attempted

Requesting Party Identity Identifies the party requesting the address registration, deregistration, or both, when different from the Registering Party Identity.

Registrar Identity Identifies the registrar to which the address registration request, deregistration request, or both, are destined.

Request Address Information Identifies the address information attempted to be registered, deregistered, or both, when present.

Response Address Information Identifies the Address information included in the response to the attempt to register, deregister, or both register and deregister address information, when present.

Failure Reason Indicates the reason that an address registration, deregistration, or both, were unsuccessful, when the registration, deregistration, or both are unsuccessful.

Expiration Period Identifies the address-independent registration lifetime applicable to the registered addresses.

Visited Network ID For IMS roaming scenarios, identifies the visited IMS network information. Note that this is based on P-Visited-Network-Id header of the SIP REGISTER message.

NOTE 1:	In most cases, either the IMPU or IMPI may be available, but not necessarily both.

#### X.2.1.1	Call Related events

##### X.2.1.1.1	Call establishment

For call establishment a call establishment-event is generated. This event is generated at the beginning of a call when the VoIP Platform attempts to reach the subscriber. This information will be delivered to the DF2 if available:

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address information

call direction

Correlation number

Redirected From Party ID(s)

Network Element Identifier

Location Information

basic service

Supplementary service

Forked Calls

Transit Carrier ID

##### X.2.1.1.2	Answer

If the called party answers, an answer- event is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address

Connected party address

call direction

Correlation number

Redirected From Party ID(s)

Network Element Identifier

Location Information

basic service

Supplementary service

##### X.2.1.1.3	Supplementary Services

For supplementary services events are generated with the information regarding which supplementary service is used e.g. Call Forwarding (CF), Call Waiting (CW), Explicit Call Transfer (ECT), Multi Party (MPTY), Call Hold and information correlated to the service like the forwarded to number. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

Supplementary service

Forwarded to Address

##### X.2.1.1.5	Redirection

For the redirection of a target call, a redirection event with the following information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Calling Party Address

Called Party Address

call direction

Redirected From Party ID(s)

Redirected To Party ID

Transit Carrier ID

Correlation number

Network Element Identifier

Location Information

basic service – media information

Visited System ID

##### X.2.1.1.6	Release

For the release or failed attempt of a target call, a release event with the following information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Calling Party Address

Called Party Address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

call release reason

Contact Address(es)

##### X.2.1.1.7 	Media Modification and Address Reporting

For the release or failed attempt of a target call, a release event with the following information is generated, if this information would not already be reported by another event.. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

call direction

Correlation number

Network Element Identifier

basic service

Target Contact Address

Other Party Contact Address

##### X.2.1.1.8	Subject Signal

When an intercept subject sends a signal to the network, a subject signal events with the following information is generated, if this information would not already be reported by another event. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Signalling party ID

Signalled party ID

call direction

Correlation number – Only included if Network Signal is associated with a call.

Network Element Identifier

Input Address Information

User Signal

Refer

##### X.2.1.1.9	Network Signal

When a network signal is being sent to the intercept subject of a target call, a network signal event with the following information is generated, if this information would not already be reported by another event. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Signalled-to party ID

other party media information

Correlation number – Only included if Network Signal is associated with a call.

Network Element Identifier

Network Signal

Refer

#### X.2.1.2 Non Call Related events

##### X.2.1.2.1	Location update

For location updates a Location update-event is generated, with the new location information. This information will be delivered to the DF2 if available:

Observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

##### X.2.1.2.2	Subscriber Controlled Input (SCI)

SCI includes subscriber initiated changes in service activation and deactivation. SCI does not include any information available in the CC. For subscriber controlled inputs - a SCI-event is generated with information about the SCI. This information will be delivered to the DF2 if available:

observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

SCI

##### X.2.1.2.3	VoIP Service Registration/De-Registration

When the UE registers for VoIP service or deregisters, a VoIP service registration-event is generated, with the new registration information. This information will be delivered to the DF2 if available:

Observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

Request Identity

Address Registration Type

Registering Party Identity

Requesting Party Identity

Registrar Identity

Request Address Information

Response Address Information

Failure Reason

Expiration Period

Visited Network ID

### X.2.2 UMTS-Specific IRI for Separated VoIP Interception and Delivery

In addition to the IRI events to be reported from the VoIP Platform, this clause identified the events to be reported from UMTS specific network elements.

The following events are applicable to the HLR:

- Serving System.

HLR related events are defined in Clause 7.3.2.

#### X.2.2.1   UMTS Access-Oriented UMTS-Specific IRI for Separated VoIP Interception and Delivery

FFS.

### X.2.3 EPS-Specific IRI for Separated VoIP Interception and Delivery

In addition to the IRI events to be reported from the VoIP Platform, this clause identified the events to be reported from EPS specific network elements.

The following events are applicable to the HSS:

- Serving Evolved Packet System.

HSS related events are defined in Clause 12.2.3.8.

#### X.2.3.2   LTE Access-Oriented EPS-Specific IRI for Separated EPS VoIP Interception and Delivery

FFS.

### X.2.4 WLAN-I-Specific IRI for Separated VoIP Interception and Delivery

In addition to the IRI events to be reported from the VoIP Platform, this clause identifies the events to be reported from WLAN-I specific network elements.

The following events are applicable to the HLR:

- Serving System.

HLR related events are defined in Clause 7.3.2.

#### X.2.4.1   WLAN-I Access-Oriented WLAN-I-Specific IRI for Separated VoIP Interception and Delivery

FFS.

## X.3 Separated VoIP CC Interception and Delivery

### X.3.1 Triggering of VoIP CC Intercept

See SA3LI13_93 & _94.

### X.3.2 VoIP CC Intercept – Target using a UMTS Access

The following ICEs shall perform the CC interception and Delivery for VoIP when a target of interception is using a UMTS Access: SGSN, GGSN, Access Gateway, Media Gateway, and Interworking/Border Gateway.

The access method for the delivery of CC for Separated VoIP Interception and Delivery Product is based on duplication of VoIP CC packets without modification at the ICE. The duplicated packets with additional information in a header, as described in X.3.5, are sent to DF3 for further delivery to the LEA.

Two options are possible for isolating the VoIP CC:

- All CC packets related to the target are captured by the ICE and sent to the MF/DF which then isolates only those packets relating to the VoIP CC

- ICE isolates and forwards to the MF/DF only those packets relating to the VoIP CC.  Clause X.3.1 describes a triggering mechanism for realizing this option.

### X.3.3 VoIP CC Intercept – Target using an LTE Access Network

The following ICEs, as defined in SA3LI13_93 &_94, shall perform the CC interception and Delivery for VoIP when a target of interception is using a LTE Access: PDN-GW, Access Gateway, Media Gateway, and Interworking/Border Gateway.

The access method for the delivery of CC for Separated VoIP Interception and Delivery Product is based on duplication of VoIP CC packets without modification at the ICE. The duplicated packets with additional information in a header, as described in X.3.5, are sent to DF3 for further delivery to the LEA.

### X.3.2 VoIP CC Intercept – Target using a WLAN-I Access Network

The following ICEs shall perform the CC interception and Delivery for VoIP when a target of interception is using a WLAN-I Access: WAG, (e)PDG, Access Gateway, Media Gateway, and Interworking/Border Gateway.

The access method for the delivery of CC for Separated VoIP Interception and Delivery Product is based on duplication of VoIP CC packets without modification at the ICE. The duplicated packets with additional information in a header, as described in X.3.5, are sent to DF3 for further delivery to the LEA.

Two options are possible for isolating the VoIP CC:

- All cc packets related to the target are captured by the ICE and sent to the MF/DF which then isolates only those packets relating to the VoIP CC

- ICE isolates and forwards to the MF/DF only those packets relating to the VoIP CC.  Clause X.3.1 describes a triggering mechanism for realizing this option.

### X.3.5 ICE to MF Interface

In addition to the intercepted Separated VoIP content of communications, the following information needs to be transferred from the ICE performing VoIP cc interception to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

*** END OF MODIFICATIONS ***