3GPP TSG-SA3-LI Meeting #53 	SA3LI14_062r1

# New Orleans, USA, 29 April-1 May 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for HeNB (108) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2014-04-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To include support for LI of HeNB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Identifies events and event information to be reported to LE based on Clause 13 of 33.107 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5, 10.5.1.1, B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

## 10.5	IRI for evolved packet domain

The IRI will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a bearer/tunnel, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a bearer/tunnel);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data context information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are several different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF. The applicability of the events to specific access (E-UTRAN, trusted non-3GPP access, untrusted non-3GPP access) and network protocols (GTP/PMIP S5/S8 interface) is specified in [19]. Additional events and mapping with IRI Record type are applicable to EPS in case of interworking between SGSN and PDN-GW over Gn/Gp interface, as specified in this document for PS interception.

Table 10.5.1: Mapping between EPS Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| E-UTRAN attach | REPORT |
| E-UTRAN detach | REPORT |
| Bearer activation (successful) | BEGIN |
| Bearer modification | CONTINUE |
| UE Requested bearer resource modification | REPORT |
| Bearer activation (unsuccessful) | REPORT |
| Start of interception with active bearer | BEGIN or optionally CONTINUE |
| Bearer deactivation | END |
| UE requested PDN connectivity | REPORT |
| UE requested PDN disconnection | REPORT |
| Tracking Area update | REPORT |
| Serving Evolved Packet System | REPORT |
| PMIP attach/tunnel activation (successful) | BEGIN |
| PMIP attach/tunnel activation (unsuccessful) | REPORT |
| PMIP session modification | CONTINUE |
| PMIP detach/tunnel deactivation | END |
| Start of interception with active PMIP tunnel | BEGIN (or optionally CONTINUE) |
| PMIP PDN-GW initiated PDN disconnection | END |
| MIP registration/tunnel activation (successful) | BEGIN |
| MIP registration/tunnel activation (unsuccessful) | REPORT |
| MIP deregistration/tunnel deactivation | END |
| Start of interception with active MIP tunnel | BEGIN |
| DSMIP registration/tunnel activation (successful) | BEGIN |
| DSMIP registration/tunnel activation (unsuccessful) | REPORT |
| DSMIP session modification | CONTINUE |
| DSMIP deregistration/tunnel deactivation | END |
| Start of interception with active DSMIP tunnel | BEGIN |
| DSMIP HA Switch | REPORT |
| PMIP Resource Allocation Deactivation | END |
| MIP Resource Allocation Deactivation | END |
| Start of interception with E-UTRAN attached UE | REPORT |
| Packet Data Header Information | REPORT |

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the network nodes or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 10.5.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |  |
| --- | --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation (party-identiity) |  |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation (party-identity) |  |
| observed ME Id | Target Identifier with the ME Id of the target subscriber (monitored subscriber) | partyInformation (party-identity) |  |
| observed MN NAI | Target Identifier with the NAI of the target subscriber | partyInformation (party-identity) |  |
| event type | Description which type of event is delivered | ePSevent |  |
| event date | Date of the event generation in the node | Timestamp |  |
| event time | Time of the event generation in the node |  |  |
| access point name | When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the APN used for the PDN connection | aPN |  |
| APN-AMBR | Contains the Aggregate Maximum Bit Rate for the APN | aPN-AMBR |  |
| PDN type | Indicated the used IP version (IPv4, IPv6, IPv4/IPv6) | pDNAddressAllocation |  |
| PDN address allocation | Provides the IP version (IPv4, IPv6, IPv4/IPv6) and the IP address(es) allocated for the UE. | pDNAddressAllocation |  |
| Protocol Configuration Options | Are used to transfer parameters between the UE and the PDN-GW (e.g. address allocation preference by DHCP) | protConfigOptions |  |
| Attach type | Indicates the type of attach and may carry indication of handover in case of mobility with non-3GPP access. | attachType |  |
| RAT type | Radio Access Type | rATType |  |
| initiator | This field indicates whether the procedure is UE or network initiated. | initiator |  |
| Handover indication | Provides information that the procedure is triggered as part of a handover | handoverIndication, extendedHandoverIndication |  |
| Procedure Transaction Identifier | Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE | procedureTransactionId |  |
| EPS bearer identity | Identifies an EPS bearer for one UE accessing via E-UTRAN. It is allocated by the MME. | ePSBearerIdentity |  |
| Bearer activation/ deactivation type | Indicates the type of bearer being activated/deactivated, i.e. default or dedicated. | bearerActivationType, bearerDeactivationType |  |
| Linked EPS bearer identity | Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer. | linkedEPSBearerId |  |
| Switch off indicator | Indicates whether a detach procedure is due to a switch off situation or not. | detachType |  |
| Detach type | Parameter sent by the network to the UE to indicate the type of detach. | detachType |  |
| Traffic Flow Template (TFT) | Collection of all packet filters associated with the EPS bearer. | tFT |  |
| Traffic Aggregate Description (TAD) | Consists of the description of the packet filter(s) for the traffic flow aggregate. | trafficAggregateDescription |  |
| correlation number | Unique number for each target connection delivered to the LEMF, to help the LEA, to have a correlation between each target connection and the IRI. | ePSCorrelationNumber |  |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |  |
| location information | When authorized, this field provides the location information of the target that is present at the node at the time of event record production. | ePSlocationOfTheTarget |  |
| Old location information | Location information of the target subscriber before Tracking Area Update. | ePSlocationOfTheTarget |  |
| Failure reason | The reason for the failure or rejection of the Tracking Area Update | failedTAUReason |  |
| failed bearer activation reason | This field gives information about the reason for a failed bearer activation of the target subscriber. | failedBearerActivationReason |  |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target subscriber. | failedEUTRANAttachreason, status, code (depending on the protocol) |  |
| Session modification failure reason | This field gives information about the reason for a failed session modification attempt of the target subscriber | status |  |
| EPS bearer QOS | This field indicates the Quality of Service associated with the EPS bearer procedure. | ePSBearerqOS |  |
| bearer deactivation reason | This field gives information about the reason for bearer deactivation of the target subscriber. | bearerDeactivationCause |  |
| network identifier | Operator ID plus node address. | networkIdentifier |  |
| Failed Bearer Modification reason | The reason for failure of Bearer Modification | failedBearerModReason |  |
| ULI Timestamp | Indicates the time when the User Location Information was acquired. | uLITimestamp |
| Lifetime | Lifetime of the tunnel; it is set to a nonzero value in case of registration or lifetime extension; is set to zero in case of deregistration. | lifetime |  |
| Access technology type | Indicates the Radio Access Type | accessTechnologyType |  |
| UE address info | Includes one or more IP addresses allocated to the UE. | iPv6HomeNetworkPrefix, iPv4HomeAddress, iPv6careOfAddress, iPv4careOf Address |  |
| Additional parameters | Additional information provided by the UE, such as protocol configuration options | protConfigurationOption |  |
| serving MME address | Diameter Origin-Host and Origin-Realm of the serving MME. | servingMME-Address |  |
| Revocation trigger | Contains the reason which triggered a PDN-GW initiated PDN-disconnection (revocation) procedure. | revocationTrigger |  |
| Home Address | Contains the UE Home IP address | homeAddress |  |
| Home Agent Address | Contains the IP address of the Home Agent | homeAgentAddress |  |
| Requested IPv6 Home Prefix | The IPv6 Home Prefix requested by the UE. | requestedIPv6HomePrefix |  |
| Care of Address | The local IP address assigned to the UE by the Access Network. | careOfAddress |  |
| HSS/AAA address | The address of the HSS/AAA triggering a pDN-GW reallocation. | hSS-AAA-address |  |
| Target PDN-GW address | The address of the PDN-GW which the UE will be reallocated to. | targetPDN-GW-Address |  |
| Foreign domain address | The relevant IP address in the foreign domain. | foreignDomainAddress |  |
| Visited network identifier | An identifier that allows the home network to identify the visited network [53] | visitedNetworkId |  |
| DHCP v4 Address Allocation Indication | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE | dHCPv4AddressAllocationInd |  |
| Serving Network | Identifies, for E-UTRAN access, the serving network the UE is attached to | servingNetwork |  |
| Request type | Provides the type of UE requested PDN connectivity | requestType |  |
| Failed reason | Provides the failure cause for UE requested PDN connectivity | uEReqPDNConnFailReason |  |
| destination IP address | Identifies the destination IP address of a packet. | destinationIPAddress |  |
| destination port number | Identifies the destination port number of a packet | destinationPortNumber |  |
| source IP address | Identifies the source IP address of a packet. | sourceIPAddress |  |
| source port number | Identifies the source port number of a packet. | sourcePortNumber |  |
| transport protocol | Identifies the transport protocol (i.e., Protocol Field in IPv4 or Next Header Field in IPv6. | transportProtocol |  |
| flow label | The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]) | flowLabel |  |
| packet count | The number of packets detected and reported in a particular packet data summary report. | packetCount |  |
| packet size | The size of a packet (i.e., Total Length Field in IPv4 [a] or Payload Length Field in IPv6 [b]) | packetsize |  |
| packet direction | Identifies the direction of the intercepted packet (from subject or to subject) | packetDirection |  |
| packet header copy | Provides a copy of the packet headers including IP layer and next layer, and extensions, but excluding content. | packetHeaderCopy |  |
| summary period | Provides the period of time during which the packets of the summary report were sent or received by the subject. | summaryPeriod |  |
| sum of packet sizes | Sum of values in Total Length Fields in IPv4 packets or Payload Length Field in IPv6 packets. | sumOfPacketSizes |  |
| packet data summary reason | Provides the reason for a summary report. | packetDataSummaryReason |  |
| packet data summary | For each particular packet flow, identifies pertinent reporting information (e.g., source IP address, destination IP address, source port, destination port, transport protocol, packet count, time interval, sum of packet sizes) associated with the particular packet flow. | packetDataSummary |  |
| CSG Identity | Uniquely identifies a CSG within a PLMN. | csgIdentity |  |
| HeNB Identity | Identifies the HeNB providing access to a target UE. | heNBIdentity |  |
| HeNB IP address | Identifies the IP Address associated with an HeNB providing access to a target UE. | heNBiPAddress |  |
| HeNB Location | Identifies the location of an HeNB providing access to a target UE. | heNBLocation |  |
| Tunnel Protocol | Identifies the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. | tunnelProtocol |  |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

*** NEXT MODIFICATION ***

#### 10.5.1.1	REPORT record information

The REPORT record is used to report non-communication related subscriber actions (events) and for reporting unsuccessful packet-mode communication attempts. In addition, this record is also used to report some subscriber actions which may trigger communication attempts or modifications of an existing communication, when the communication attempt or the change of the existing communication itself is reported separately.

The REPORT record shall be triggered when:

-	the intercept subject's UE performs an E-UTRAN attach procedure (successful or unsuccessful) including via a HeNB;

-	the intercept subject's UE performs an E-UTRAN detach procedure including via a HeNB;

-	the intercept subject's UE is unsuccessful at performing an EPS bearer activation procedure;

-	the intercept subject’s UE performs an UE requested bearer resource modification;

-	the intercept subject's UE performs a tracking area update;

-	optionally when the intercept subject's UE leaves the old MME;

-	the intercept subject’s UE performs an UE requested PDN connectivity procedure;

-	the intercept subject’s UE performs an UE requested PDN disconnection procedure;

-	the intercept subject's UE is unsuccessful at performing a PMIP attach/tunnel activation procedure;

-	the intercept subject’s UE is unsuccessful at performing a MIP registration/tunnel activation procedure;

-	the intercept subject's UE is unsuccessful at performing a DSMIP registration/tunnel activation procedure;

-	optionally when the intercept subject's UE enters or leaves IA (FFS);

-	the intercept subject’s UE is ordered by the network to perform an home agent switch;

-	as a national option, a mobile terminal is authorized for service with another network operator or service provider;

-	the interception of a subject is started with E-UTRAN attached target. If there are more than one PDN connections then a REPORT record is generated per PDN connection.;

-	packet data header reporting is performed on an individual intercepted packet basis and a packet is detected as it is sent or received by the target for an EPS bearer/session.;

-	when packet data summary reporting is performed on an summary basis for an EPS bearer/session associated with a particular packet flow (defined as the combination of source IP address, destination IP address, source port, destination port, and protocol and for IPv6 also include the flow label) and:

-	the packet flow starts,

-	an interim packet summary report is to be provided, or

-	packet flow ends including the case where the EPS bearer/session is deactivated.

An interim packet summary report is triggered if:

-	the expiration of a configurable Summary Timer per intercept occurs.  The Summary Timer is configurable in units of seconds, or

-	a per-intercept configurable count threshold is reached.

Packet Header Information Reporting is reported either on a per-packet (i.e., non-summarised) basis or in a summary report.  These reports provide IRI associated with the packets detected. The packet header information related REPORT record is used to convey packet header information during an active EPS bearer/session.

NOTE:  in the case of IP Fragments, Packet Header Information on a 6-tuple basis may only be available on the first packet and subsequent packets may not include such information and therefore may not be reported.

Table 10.5.1.1.1: E-UTRAN Attach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide E-UTRAN Attach event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| failed attach reason | C | Provide information about the reason for failed attach attempt of the target subscriber. |
| PDN Type | C | Indicated the used IP version (IPv4, IPv6, IPv4/IPv6), including possible reason for modification by the network |
| APN | C | Provides the Access Point Name |
| Protocol Configuration Options | C | Provides information sent from the UE to the network |
| Attach type | C | Provides the type of attach |
| EPS bearer identity | C | When the attach is successful, provides the allocated EPS bearer identity. |
| CSG Identity | C | Provide if closed/hybrid HeNB is used in the UE attachment to the network |
| HeNB Identity | C | Provide information to identify the HeNB serving the intercept subject's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the intercept subject's UE used during location verification. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the intercept subject's UE. |
| Tunnel Protocol | C | Provide to identify the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. |

Table 10.5.1.1.2: E-UTRAN Detach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide E-UTRAN Detach event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's MS. |
| initiator | C | Provided to indicate whether the detach is UE or network initiated |
| Switch off indicator | C | Provided to indicate whether the detach is due to a switch off |
| Detach type | C | Sent by the network to the UE to indicate the type of detach |
| CSG Identity | C | Provide if closed/hybrid HeNB is used in the UE detachment from the network |
| HeNB Identity | C | Provide information to identify the HeNB serving the intercept subject's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the intercept subject's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the intercept subject's UE. |

Table 10.5.1.1.3: Bearer Activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| event type | C | Provide EPS Bearer Activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE; the parameter is applicable only for default bearer activation. |
| RAT type | C | Provide the Radio Access Type used by the target subscriber. |
| initiator | C | Provide to indicate whether the EPS bearer activation is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| failed bearer activation reason | C | Provide information about the reason for failed bearer activation attempts of the target subscriber. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. The parameter carries the requested EPS bearer QOS. |
| Bearer activation type | C | Provides information on default or dedicated bearer failed activation |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. The parameter carries the subscribed APN-AMBR. |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE |
| Procedure transaction identifier | C | Used to associate the EPS bearer activation attempt to other messages triggering the procedure. |
| Linked EPS bearer identity | C | Provides, in case of failed dedicated bearer activation attempt, the EPS bearer id of the associated default bearer; not applicable in case of default bearer activation attempt. |
| Traffic Flow Template TFT | C | The TFT associated to the dedicated bearer activation attempt; not applicable in case of default bearer activation attempt |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover |

Table 10.5.1.1.4: UE requested bearer resource modification REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested bearer resource modification event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| failed bearer modification reason | C | Provide information about the reason for failed UE requested bearer resource modification. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. |
| Procedure transaction identifier | C | Used to associate the UE requested bearer resource modification to other messages related to the procedure. |
| Linked EPS bearer identity | C | Provides the EPS bearer id of the associated default bearer. |
| EPS Bearer identity | C | Provides the EPS bearer id of the bearer which the request refers to. |
| Traffic Aggregate Description | C | Description of the packet filter(s) for the traffic flow aggregate |
| Protocol Configuration Options | C | Provide information about the protocol configuration options requested by the UE. |

Table 10.5.1.1.5: Tracking Area Update REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Tracking Area Update event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's MS. This parameter, in case of inter-MME TAU, will be sent only by the new MME. |
| old location information | O | Provide (only by the old MME), when authorized and if available, to identify the old location information for the intercept subject's MS. |
| Failure reason | C | Provide, in unsuccessful case, the reason for the failure or rejection of the TAU. |

In case of inter-MME TAU, Tracking Area Update REPORT Record shall be sent in the following cases:

-	when the intercept subject's UE moves to the new MME;

-	optionally when the intercept subject's UE leaves the old MME.

Table 10.5.1.1.6: UE requested PDN connectivity REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested PDN connectivity event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE (valid only for default bearer activation). |
| Request type | C | Indicates the type of request, i.e. initial request or handover |
| PDN type | C | Provide to describe the IP version requested by the target UE. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| failed reason | C | Provide information about the reason for failed procedure. |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE |
| EPS bearer identity | C | The identity of the allocated EPS bearer |
| HeNB Identity |  | C |  | Provide information to identify the HeNB serving the intercept subject's UE. |  |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the intercept subject's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the intercept subject's UE. |

Table 10.5.1.1.7: UE requested PDN disconnection REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested PDN disconnection event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| Linked EPS bearer identity | C | The identity of the default EPS bearer associated with the PDN connection being disconnected. |
| HeNB Identity |  | C |  | Provide information to identify the HeNB serving the intercept subject's UE. |  |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the intercept subject's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the intercept subject's UE. |

Table 10.5.1.1.8: PMIP Attach/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP Attach/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| Lifetime | C | The requested lifetime for the tunnel |
| Access technology type | C | Provide the radio access type |
| failed attach reason | C | Provide information about the reason for failed attach/tunnel activation attempt of the target subscriber. |
| Handover indicator | C | Provide information that the procedure is triggered as part of the handover |
| APN | C | Provide the Access Point Name |
| UE address info | C | Includes one or more addresses allocated to the UE |
| Additional parameters | C | Provide additional parameters sent by the UE. |
| Serving Network | C | Provide to identify the serving network the UE is attached to in case of E-UTRAN access and PMIP based S5/S8 interfaces. |
| DHCPv4 Address Allocation Indication | C | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE in case of E-UTRAN access and PMIP based S5/S8 interfaces |
| Location information | C | Provide, when authorized, to identify location information for the intercepted subject’s UE. |

Table 10.5.1.1.9: MIP registration/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide MIP registration/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| Lifetime | C | The requested lifetime for the tunnel |
| failed attach reason | C | Provide information about the reason for failed registration/tunnel activation attempt of the target subscriber. |
| Home Address | C | Provide the UE Home IP Address |
| Care of Address | C | The local IP address provided by the access network |
| Home Agent Address | C | Provide the Home Agent address |

Table 10.5.1.1.10: DSMIP registration/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide DSMIP registration/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| lifetime | C | The requested lifetime for the tunnel. |
| failed attach reason | C | Provide information about the reason for failed registration/tunnel activation attempt of the target subscriber. |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix. |
| Home address | C | Provide the assigned home address. |
| APN | C | Provides the Access Point Name. |
| Care of address | C | The local IP address provided by the access network. |

Table 10.5.1.1.11: DSMIP Home Agent Switch REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide DSMIP Home Agent Switch event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| HSS/AAA address | C | Provide the address of the HSS/AAA triggering the procedure |
| Target PDN-GW address | M | Provide the address of the new PDN-GW |

Table 10.5.1.1.12: Serving Evolved Packet System REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed ME Id |  |  |
| event type | C | Provide Serving Evolved Packet System event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HSS reporting the event. |
| lawful intercept identifier | M | Shall be provided. |
| Serving MME address | C | Provide the Diameter Origin-Host and the Diameter Origin-Realm of the serving MME (in case of E-UTRAN access). |
| Visited Network Identifier | C | An identifier that allows the home network to identify the visited network [53] |

Table 10.5.1.1.13: Start of interception with E-UTRAN attached UE REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide start of interception with PDN connection active |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE (valid only for default bearer activation). |
| PDN type | C | Provide to describe the IP version requested by the target UE. |
| EPS bearer identity | C | The identity of the default EPS bearer |
| lawful intercept identifier | M | Shall be provided. |
| CSG Identity |  | C |  | Provide if closed/hybrid HeNB is used in the UE attachment to the network |  |
| HeNB Identity | C | Provide information to identify the HeNB serving the intercept subject's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the intercept subject's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the intercept subject's UE. |
| Tunnel Protocol | C | Provide to identify the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. |

Table 10.5.1.1.14: Packet Data Header Information REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Packet Data Header Information event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| initiator | C | Provide to indicate whether the EPS bearer modification is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| Correlation number | M | Provide to uniquely identify the EPS bearer/tunnel delivered to the LEMF and to correlate IRI records with CC. |
| lifetime | C | The lifetime for the tunnel |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix |
| Home address | C | Provide the assigned home address |
| APN | C | Provides the Access Point Name |
| Care of address | C | The IP address provided by the access network |
| packet data header information | M | Shall be provided to identify the packet header information to be reported on a per-packet basis as defined in Table 10.5.1.1.15 or on a summary basis. For summary reporting includes one or more packet flow summaries where each packet flow summary is associated with a particular packet flow as defined in Table 10.5.1.1.16. |

Table 10.5.1.1.15: Contents of a per-packet packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | C | Provide when mapping packet header information to identify the source IP address for a particular packet flow. |
| source port number | C | Provide when mapping packet header information to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | C | Provide when mapping packet header information to Identify the destination IP address for a particular packet flow. |
| destination port number | C | Provide when mapping packet header information to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | C | Provide when mapping packet header information to identify the transport protocol (e.g., TCP) for a particular packet flow. |
| flow label | C | Provide when mapping packet header information for IPv6 only for a particular packet flow. |
| direction | M | Shall be provided. Identifies the direction of the packet (from subject or to subject). |
| packet size | C | Provide when mapping packet header information to convey the value contained in Total Length Fields of the IPv4 packets or the value contained in the Payload Length fields of the IPv6 packets. |
| packet data header copy | C | Provide when reporting a copy of the entire packet header information rather than mapping individual information. |

Table 10.5.1.1.16: Contents of a single summary flow packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | M | Shall be provided. Identifies the source IP address for a particular packet flow. |
| source port number | C | Provide to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | M | Shall be provided. Identifies the destination IP address for a particular packet flow. |
| destination port number | C | Provide to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | M | Identifies the transport protocol (e.g., TCP) for a particular packet flow. |
| flow label | C | Provide for IPv6 only for a particular packet flow. |
| summary period | M | Provides the period of time during which the packets of a particular packet flow of the summary report were sent or received by the subject and defined by specifying the time when the first packet and the last packet of the reporting period were detected. |
| packet count | M | Provides the number of packets detected for a particular packet flow. |
| sum of packet sizes | M | Provides the sum of values contained in Total Length Fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets. |
| packet data summary reason | M | Provides the reason for the report being delivered to the LEMF (i.e., timeout, count limit, end of session). |

*** NEXT MODIFICATION ***

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data UmtsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r12(12) version-5(5)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version10(10)}; -- Imported from TS 101 671v3.3.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r12(12) version-5(5)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- EpsIRIContent needs to be chosen.

-- EpsIRIContent includes events that correspond to EPS and UMTS/GPRS.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the EpsIRIContent may provide events that correspond to UMTS/GPRS as well.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target subscriber

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server within the calling (if server is originating) or called (if server is

-- terminating) party address parameters

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

-- this parameter provides GPRS Correlation number when the event corresponds to UMTS/GPRS.

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

mediaDecryption-info	       [42] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	       [43] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [44] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [45] OCTET STRING OPTIONAL,

sdpOffer                [46] OCTET STRING OPTIONAL,

sdpAnswer               [47] OCTET STRING OPTIONAL,

uLITimestamp            [48] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		   [49] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication         [50] MediaSecFailureIndication OPTIONAL,

csgIdentity				[51] OCTET STRING (SIZE (4)) OPTIONAL,  -- Octets are coded according

-- to 3GPP TS 23.003 [25].  The 27 bits specified in TS 23.003 shall be encoded as follows.

-- The most significant bit of the CSG Identity shall be encoded in the most significant

-- bit of the first octet of the octet string and the least significant bit coded in bit 6

-- of octet 4.

heNBIdentity			[52] OCTET STRING OPTIONAL, -- 4 or 6 octets are coded with the

-- with the HNB Unqiue Identity as specified in 3GPP TS 23.003 [25], Clause 4.10.

heNBiPAddress			[53] IPAddress,

heNBLocation			[54] HeNBLocation,

tunnelProtocol			[55] TunnelProtocol,

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRSorEPS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[29])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-uri					[9] OCTET STRING 	OPTIONAL,

-- See [67]

nai                    [10] OCTET STRING    OPTIONAL

-- NAI of the target, encoded in the same format as defined by [EPS stage 3 specs]

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

Location	::= SEQUENCE

{

e164-Number         [1] OCTET STRING (SIZE (1..25)) OPTIONAL,
        -- Coded in the same format as the ISUP location number (parameter
        -- field) of the ISUP (see EN 300 356 [30]).    
	globalCellID		[2] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

rAI					[4] Rai		OPTIONAL,

-- the Routeing Area Identifier in the current  SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] Sai	OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 – 3)

--			LAC		2 octets (no. 4 – 5)

--			SAC		2 octets (no. 6 – 7)

--			(according to 3GPP TS 25.413 [62])

...,

oldRAI				[8] Rai		OPTIONAL

-- the Routeing Area Identifier in the old SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used).

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

server			(1),

undefined-party	(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

EPSCorrelationNumber ::= OCTET STRING

-- In case of PS interception, the size will be in the range (8..20)

CorrelationValues ::= CHOICE {

iri-to-CC 	[0]		IRI-to-CC-Correlation, -- correlates IRI to Content(s)

iri-to-iri	[1]		IRI-to-IRI-Correlation, -- correlates IRI to IRI

both-IRI-CC	[2]		SEQUENCE { -- correlates IRI to IRI and IRI to Content(s)

iri-CC	[0]	IRI-to-CC-Correlation,

iri-IRI	[1]	IRI-to-IRI-Correlation}

}

IRI-to-CC-Correlation ::= SEQUENCE { -- correlates IRI to Content

cc 		[0] SET OF 	OCTET STRING,-- correlates IRI to multiple CCs

iri 	[1] 		OCTET STRING OPTIONAL

-- correlates IRI to CC with signaling

}

IRI-to-IRI-Correlation ::= OCTET STRING -- correlates IRI to IRI

EPSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

pDPContextModification					(13),

servingSystem							(14),

... ,

startOfInterceptionWithMSAttached		(15),

e-UTRANAttach							(16),

e-UTRANDetach                           (17),

bearerActivation                        (18),

startOfInterceptionWithActiveBearer     (19),

bearerModification                      (20),

bearerDeactivation                      (21),

uERequestedBearerResourceModification   (22),

uERequestedPDNConnectivity              (23),

uERequestedPDNDisconnection             (24),

trackingAreaUpdate                      (25),

servingEvolvedPacketSystem              (26),

pMIPAttachTunnelActivation              (27),

pMIPDetachTunnelDeactivation            (28),

startOfInterceptWithActivePMIPTunnel    (29),

pMIPPdnGwInitiatedPdnDisconnection      (30),

mIPRegistrationTunnelActivation         (31),

mIPDeregistrationTunnelDeactivation     (32),

startOfInterceptWithActiveMIPTunnel     (33),

dSMIPRegistrationTunnelActivation       (34),

dSMIPDeregistrationTunnelDeactivation   (35),

startOfInterceptWithActiveDsmipTunnel   (36),

dSMipHaSwitch                           (37),

pMIPResourceAllocationDeactivation      (38),

mIPResourceAllocationDeactivation       (39),

pMIPsessionModification                 (40),

startOfInterceptWithEUTRANAttachedUE    (41),

dSMIPSessionModification                (42) ,

packetDataHeaderInformation				(43)

}

-- see [19]

IMSevent ::= ENUMERATED

{

unfilteredSIPmessage (1),

-- This value indicates to LEMF that the whole SIP message is sent.

...,

sIPheaderOnly (2),

-- If warrant requires only IRI then specific content in a 'sIPMessage'

-- (e.g. 'Message', etc.) has been deleted before sending it to LEMF.

decryptionKeysAvailable (3) ,

-- This value indicates to LEMF that the IRI carries CC decryption keys for the session

-- under interception.

startOfInterceptionForIMSEstablishedSession  (4)

-- This value indicates to LEMF that the IRI carries information related to

-- interception started on an already established IMS session.

}

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

-- Include either Octets 3 and 4 of the Packet Data Protocol Address information element

-- of 3GPP TS 24.008 [9] or Octets 4 and 5 of the End User Address IE of 3GPP TS 29.060 [17].

-- when PDP-type is IPv4 or IPv6, the IP address is carried by parameter

-- pDP-address-allocated-to-the-target

-- when PDP-type is IPv4v6, the additional IP address is carried by parameter

-- additionalIPaddress

...,

nSAPI					[4] OCTET STRING (SIZE (1)) OPTIONAL,

-- Include either Octet 2 of the NSAPI IE of 3GPP TS 24.008 [9]

-- or Octet 2 of the NSAPI IE of 3GPP TS 29.060 [17].

additionalIPaddress		[5] DataNodeAddress OPTIONAL

}

GPRSOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the GMM cause value or the SM cause value, as defined in the

-- standard [9], without the IEI.

LDIevent ::= ENUMERATED

{

targetEntersIA 			(1),

targetLeavesIA			(2),

...

}

UmtsQos ::= CHOICE

{

qosMobileRadio [1] OCTET STRING,

-- The qosMobileRadio parameter shall be coded in accordance with the § 10.5.6.5 of

-- document [9] without the Quality of service IEI and Length of

-- quality of service IE (. That is, first

-- two octets carrying 'Quality of service IEI' and 'Length of quality of service

-- IE' shall be excluded).

qosGn [2] OCTET STRING

-- qosGn parameter shall be coded in accordance with § 7.7.34 of document [17]

}

EPS-GTPV2-SpecificParameters ::= SEQUENCE

{

pDNAddressAllocation                 [1]   OCTET STRING                       OPTIONAL,

aPN                                  [2]   OCTET STRING (SIZE (1..100))       OPTIONAL,

protConfigOptions                    [3]   ProtConfigOptions                  OPTIONAL,

attachType                           [4]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

ePSBearerIdentity                    [5]   OCTET STRING                       OPTIONAL,

detachType                           [6]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47], includes switch off indicator

rATType                              [7]   OCTET STRING (SIZE (1))            OPTIONAL,

failedBearerActivationReason         [8]   OCTET STRING (SIZE (1))            OPTIONAL,

ePSBearerQoS                         [9]   OCTET STRING                       OPTIONAL,

bearerActivationType                 [10]  TypeOfBearer                       OPTIONAL,

aPN-AMBR                             [11]  OCTET STRING                       OPTIONAL,

-- Only octets 5 onwards of AMBR IE from 3GPP TS 29.274 [46] shall be included.

procedureTransactionId               [12]  OCTET STRING                       OPTIONAL,

linkedEPSBearerId                    [13]  OCTET STRING                       OPTIONAL,

--The Linked EPS Bearer Identity shall be included and coded according to 3GPP TS 29.274 [46].

tFT                                  [14]  OCTET STRING                       OPTIONAL,

-- Only octets 3 onwards of TFT IE from 3GPP TS 24.008 [9] shall be included.

handoverIndication                   [15]  NULL                               OPTIONAL,

failedBearerModReason                [16]  OCTET STRING (SIZE (1))            OPTIONAL,

trafficAggregateDescription          [17]  OCTET STRING                       OPTIONAL,

failedTAUReason                      [18]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

failedEUTRANAttachReason             [19]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

servingMMEaddress                    [20]  OCTET STRING                       OPTIONAL,

-- Contains the data fields from the Diameter Origin-Host and Origin-Realm AVPs

-- as received in the HSS from the MME according to the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

bearerDeactivationType               [21]  TypeOfBearer                       OPTIONAL,

bearerDeactivationCause              [22]  OCTET STRING (SIZE (1))            OPTIONAL,
    ePSlocationOfTheTarget               [23]  EPSLocation                        OPTIONAL,
      -- the use of ePSLocationOfTheTarget is mutually exclusive with the use of locationOfTheTarget

-- ePSlocationOfTheTarget allows using the coding of the paramater according to SAE stage 3.

...,

pDNType                              [24]   OCTET STRING (SIZE (1))           OPTIONAL,

-- coded according to TS 24.301 [47]

requestType                          [25]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

uEReqPDNConnFailReason               [26]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

extendedHandoverIndication			 [27]  OCTET STRING (SIZE (1)) 			  OPTIONAL,

-- This parameter with value 1 indicates handover based on the flags in the TS 29.274 [46].

-- Otherwise set to the value 0.

-- The use of extendedHandoverIndication and handoverIndication parameters is

-- mutually exclusive and depends on the actual ASN.1 encoding method.

uLITimestamp                         [28]  OCTET STRING (SIZE (8))            OPTIONAL

}

-- All the parameters within EPS-GTPV2-SpecificParameters are coded as the corresponding IEs

-- without the octets containing type and length. Unless differently stated, they are coded

-- according to 3GPP TS 29.274 [46]; in this case the octet containing the instance

-- shall also be not included.

TypeOfBearer ::= ENUMERATED

{

defaultBearer          (1),
    dedicatedBearer        (2),

...

}

EPSLocation	::= SEQUENCE

{

userLocationInfo    [1] OCTET STRING (SIZE (1..39)) OPTIONAL,
        -- coded according to 3GPP TS 29.274 [46]; the type IE is not included

gsmLocation			[2] GSMLocation OPTIONAL,

umtsLocation		[3] UMTSLocation OPTIONAL,

olduserLocationInfo	[4] OCTET STRING (SIZE (1..39))	OPTIONAL,

-- coded in the same way as userLocationInfo

lastVisitedTAI	    [5] OCTET STRING (SIZE (1..5))	OPTIONAL,

-- the Tracking Area Identity is coded in accordance with the TAI field in 3GPP TS 29.274
        -- [46].

tAIlist			    [6] OCTET STRING (SIZE (7..97))	OPTIONAL,

-- the TAI List is coded acording to 3GPP TS 24.301 [47], without the TAI list IEI

...,

threeGPP2Bsid			[7] OCTET STRING (SIZE (1..12)) OPTIONAL

-- contains only the payload from the 3GPP2-BSID AVP described in the 3GPP TS 29.212 [56].

}

ProtConfigOptions ::= SEQUENCE

{

ueToNetwork                           [1]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

networkToUe                           [2]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

...

}

EPS-PMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]  INTEGER (0..65535)                 OPTIONAL,

accessTechnologyType                  [2]  OCTET STRING (SIZE (4))            OPTIONAL,

aPN                                   [3]  OCTET STRING (SIZE (1..100))       OPTIONAL,

iPv6HomeNetworkPrefix                 [4]  OCTET STRING (SIZE (20))           OPTIONAL,

protConfigurationOption               [5]  OCTET STRING                       OPTIONAL,

handoverIndication                    [6]  OCTET STRING (SIZE (4))            OPTIONAL,

status                                [7]  INTEGER (0..255)                   OPTIONAL,

revocationTrigger                     [8]  INTEGER (0..255)                   OPTIONAL,

iPv4HomeAddress                       [9]  OCTET STRING (SIZE (4))            OPTIONAL,

iPv6careOfAddress                     [10] OCTET STRING                       OPTIONAL,

iPv4careOfAddress                     [11] OCTET STRING                       OPTIONAL,

...,

servingNetwork                        [12] OCTET STRING (SIZE (3))            OPTIONAL,

dHCPv4AddressAllocationInd            [13] OCTET STRING (SIZE (1))            OPTIONAL,

ePSlocationOfTheTarget                [14] EPSLocation                        OPTIONAL

-- parameters coded according to 3GPP TS 29.275 [48] and RFCs specifically

-- referenced in it.

}

EPS-DSMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0..65535)                OPTIONAL,

requestedIPv6HomePrefix               [2]   OCTET STRING (SIZE (25))          OPTIONAL,

-- coded according to RFC 5026

homeAddress                           [3]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv4careOfAddress                     [4]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv6careOfAddress                     [5]   OCTET STRING (SIZE(16))           OPTIONAL,

aPN                                   [6]   OCTET STRING (SIZE (1..100))      OPTIONAL,

status                                [7]   INTEGER (0..255)                  OPTIONAL,

hSS-AAA-address                       [8]   OCTET STRING                      OPTIONAL,

targetPDN-GW-Address                  [9]   OCTET STRING                      OPTIONAL,

...

-- parameters coded according to 3GPP TS 24.303 [49] and RFCs specifically

-- referenced in it.

}

EPS-MIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0.. 65535)                  OPTIONAL,

homeAddress                           [2]   OCTET STRING (SIZE (4))           OPTIONAL,

careOfAddress                         [3]   OCTET STRING (SIZE (4))           OPTIONAL,

homeAgentAddress                      [4]   OCTET STRING (SIZE (4))           OPTIONAL,

code                                  [5]   INTEGER (0..255)                  OPTIONAL,

foreignDomainAddress                  [7]   OCTET STRING (SIZE (4))           OPTIONAL,

...

-- parameters coded according to 3GPP TS 29.279 [63] and RFCs specifically

-- referenced in it.

}

MediaDecryption-info ::= SEQUENCE OF CCKeyInfo

-- One or more key can be available for decryption, one for each media streams of the

-- intercepted session.

CCKeyInfo ::= SEQUENCE

{

cCCSID	 [1]	OCTET STRING,

-- the parameter uniquely mapping the key to the encrypted stream.

cCDecKey [2]	OCTET STRING,

cCSalt   [3]    OCTET STRING OPTIONAL,

-- The field reports the value from the CS_ID field in the ticket exchange headers as

-- defined in IETF RFC 6043 [61].

...

}

MediaSecFailureIndication  ::= ENUMERATED

{

genericFailure  (0),

...

}

PacketDataHeaderInformation ::= CHOICE

{

packetDataHeader		[1]	PacketDataHeader,

packetDataHeaderSummary	[2]	PacketDataHeaderSummary,

...

}

PacketDataHeader ::= CHOICE

{

packetDataHeaderMapped	[1]	PacketDataHeaderMapped,

packetDataHeaderCopy	[2]	PacketDataHeaderCopy,

...

}

PacketDataHeaderMapped ::= SEQUENCE OF

{

sourceIPAddress			[1] IPAddress	OPTIONAL,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress	OPTIONAL,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER  OPTIONAL,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

packetsize				[6] INTEGER OPTIONAL,

flowLabel				[7] INTEGER OPTIONAL,

packetCount				[8] INTEGER OPTIONAL,

direction				[9] TPDU-direction,

...

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

PacketDataHeaderCopy ::= SEQUENCE OF

{

direction				[1] TPDU-direction,

headerCopy				[2] OCTET STRING,	-- includes a copy of the packet header at the IP

-- network layer and above including extension headers, but excluding contents.

...

}

PacketDataSummary ::= SEQUENCE OF PacketFlowSummary

PacketFlowSummary ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the “Protocol” field and for IPv6 report “Next Header” field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

flowLabel				[6] INTEGER OPTIONAL,

summaryPeriod			[7] ReportInteval,

packetCount				[8] INTEGER,

sumOfPacketSizes		[9] INTEGER,

packetDataSummaryReason	[10] ReportReason,

...

}

ReportReason ::= ENUMERATED

{

timerExpired			[0],

countThresholdHit		[1],

pDPComtextDeactivated   [2],

pDPContextModification	[3],

other or unknown		[4],

...

}

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

END -- OF EpsHI2Operations

*** END OF MODIFICATIONS ***