3GPP TSG-SA3 Meeting #66	s3i170234

# Ljubljana, Slovenia, 25th Jul 2017 - 28th Jul 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0256 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Push to Talk over Cellular Service addition of PTC feature, part A. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-07-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of feature for Push to Talk over Cellular type services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of clause 7A.5 to add further clarity for PoC type services and a new cluase adding the new feature for Push to Talk over Cellular (PTC) type services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will not be able to meet regulatory requirements since LI requirments for PTC is not currently defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of FIRST MODIFICATION ***

## 7A.5	Push to talk over Cellular (PoC)

PoC is a service of the IMS Domain and interception is accomplished according the definitions in clause 7A.3. Interception of CC is available with the current implementations in the GSNs. This approach does not address regional regulatory requirements for Push to Talk over Cellular (PTC).

For separated interception and delivery of PTC the following apply. MS authentication is provided by the SIP/IP Core before any PTC type service is offered to the target.  PTC encompasses PoC as a commercial service and Mission Critical Push-To-Talk (MCPTT) communications services.  The IRI and CC events necessary for LI for PoC and MCPTT events are located in clause [Z].  For a description of the architectures for PoC read [Ref V] and for MCPTT read [Ref S]. Within this document the use of the term PTC will include PoC and MCPTT,  PTC will apply to both type of services. If PoC or MCPTT is specific mentioned in an event it is specific to the service for PoC or MCPTT, it would only apply to that service.

*** End of FIRST MODIFICATION ***

*** Start of SECOND MODIFICATION  - addition of new clause ***

Z	Invocation of Lawful Interception for Push to talk over Cellular services.

Z.0 	General

Figure Z.0.1 and Z.0.2 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the Push to Talk over Cellular type services. Push to Talk over Cellular (PTC) will be used to reference events or services that occur in the two different architectures unless specified otherwise e.g, MCPTT or PoC.

Figure Z.0.1: PoC/MCPTT Interception configuration

Figure Z.0.2 : PoC/MCPTT Shared Servers Interception configuration

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification. The delivery functions are used:

-	to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-	to distribute the intercept related information to the relevant LEA(s);

-	to distribute the intercept product to the relevant LEA(s).

For the delivery of CC and IRI the PoC and MCPTT servers and / or, per national option the PTC ICE provides correlation number and target identity to the DF2 and DF3 from the ICE which is used there in order to select the different LEAs where the product shall be delivered as depicted in figure Z.0.1. Figure Z.0.2 represents additional ICE requirements to capture other events that occur in the servers that support PTC services.  The following is a list of servers to support either type of PTC architecture, any combination of these servers could be deployed together to support both services if the SP choose to do so. These servers will require an ICE for LI of IRI for the events depicted in the following clauses.

MCPTT servers (Common services core):

- Group management, Configuration, Identity, Key, etc

PoC servers (Shared XDMS):

- Shared List, Group, Policy, Presence, NW PoC Box, Aggregation Proxy, External Media Content server, etc.

SIP/IP Core servers:

- Presence, NW pOc Box, etc

If interception has been activated for both parties of the PTC communication both CC and IRI shall be delivered for each party as separate intercept activity.

Z.1	Provision of IRI – PTC Service

- PoC and MCPTT use similar architectures for service delivery and LI shall primarily occur at the PoC or MCPTT server for that service.

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PTC session Activation, Start of intercept with PTC context active, PTC Context Deactivation, Serving System, Packet Data Header Information, and PTC events.

Other HLR, Non-Local ID targeting for PTC events (e.g. based on traffic analysis), LI LCS Client related and Serving System events reporting are national options.

Figure Z-3 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the Shared XDMS/MCPTT common core, LI LCS Client or the Home Location Register (HLR) sends the relevant data to the DF2.

Figure Z-3 shows a PTC transfer from the PoC/MCPTT node to the LEA. Quasi-parallel to the delivery from / to the mobile subscriber a PTC event, which contains the content and header of the PTC, is generated and sent via the Delivery Function 2 to the LEA in the same way as the Intercept Related Information. National regulations and warrant type determine if a PTC event shall contain only PTC header, or PTC header and PTC content. Non-Local ID targeting is optional and may require traffic analysis.

The IRI will be delivered to the LEA:

-	For a PTC, dependent on national requirements, delivery shall occur in the following cases:

-	when the PoC/MCPTT server receives the PTC session event from the target MS, or when the PoC/MCPTT server detects that an PTC session event is to the PTC Non-Local ID target.

-	when the Shared XDMS/MCPTT Common Core servers receive the PTC event from the target MS, or when the Shared XDMS/MCPTT Common Core servers detect that an PTC event is sent to the Non-Local ID target -

Figure Z-3: Provision of Intercept Related Information - PTC

## Z.2	Provision of Content – PTC Service

The access method for the delivering of PTC Intercept Product is based on duplication of packets without modification at PoC/MCPTT server. The duplicated packets with additional information in a header, as described in Z.1.3, are sent to DF3 for further delivery to the LEA. Figure Z-4: Configuration for interception of Content- PTC

Z3	Provision of Interception

Z.3.1 	X3-Interface

In addition to the intercepted content of communications, the following information needs to be transferred from the PTC ICE to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available) or the IAs in case of location dependent interception.

- In the case where the PTC ICE is performing interception of the content of communications, the target is handed off to another PTC server, the same PTC ICE continues to handle the content of communications, subject to roaming agreements. The PTC ICE shall continue to perform the interception of the content of communication.

Z.3.2	X2-interface

The following information needs to be transferred from the SIP Core, LI LCS Client or the HLR to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN or E. 164 number for Non-Local ID, IMSI, IMEI, SIP URI, Observed TEL URI, MCPTT);

-	events and associated parameters as defined in clauses Z.3.2 and 7.4 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	Correlation number;

-	Quality of Service (QoS) identifier;

-	Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

The PTC detects packets containing packet data header information in the communications path but the information needed for Packet Data Header Information reporting may need to be transferred from the PTC Server either directly to the DF2 or via another entity in order to allow the DF2 to perform its functionality.

Z.3.3 	Structure of the Events

The LI event messages sent to the DF2/DF3 is triggered by different PTC session related and non-call related events/reports. Details for each message are described in the following clauses.

- The following events are applicable to both the PoC and MCPTT service ICE:

- PTC Service Registration

- PTC Serving System

- PTC Session Initiation

- PTC Session Abandon

- PTC Session Start

- PTC Session End

- PTC Start of Interception

- PTC Pre-Established Session

- PTC Instant Personal Alert

- PTC Party Join

- PTC Party Drop

- PTC Party Hold Retrieve

- PTC Media Modification

- PTC Group Advertisement

- PTC Floor Control

- PTC Target Presence

- PTC Associate Presence

- PTC List Management

- PTC Access Policy

- Group call request (Client – Server)

- Group call request (Server – Server)

- Group call request (Server – Client)

- Group call response (Server – Client)

- Group call response (Server – Server)

- Group call response (Client – Server)

- Group call notify (Server – Client)

- Group call release request (Server – Client)

- Group call release request (Server – Server)

- Group call release response (Client – Server)

- Group call release response (Server – Server)

- Group Rejoin Request (Client – Server)

- Group Rejoin Response (Server – Client)

- Group Join Request (Client – Server)

- Group Join Response (Server – Client)

- Group Call Leave Request (Server – Client)

- Group Call Leave Response (Client – Server)

- Group Interrogate Request (Server – Server)

- Group Interrogate Response (Server – Server)

- The following events are applicable to the MCPTT service only:

- MCPTT emergency group call request

- MCPTT emergency group call response

- MCPTT emergency group call cancel request

- MCPTT emergency group call cancel response

- MCPTT emergency alert request

- MCPTT emergency alert response

- MCPTT emergency state cancel request

- MCPTT emergency state cancel response

- MCPTT imminent peril group call request

- MCPTT imminent peril group call response

- MCPTT imminent peril group call cancel request

- MCPTT imminent peril group call cancel response

A set of elements as shown below can be associated with the events above. The events trigger the transmission of the information from PTC server, LI LCS Client or HLR to DF2, perhaps via a MF in the case of Packet Data Header Information. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

*** End of Second MODIFICATION ***

*** End of MODIFICATIONS ***