3GPP TSG-SA WG3 LI Meeting #72bis	S3i190118

Dusseldorf (DE), Feb 26th – 28th 2019

Source:	NTAC

Title:	pCR to add ASN.1 module header

Document for:	Agreement

Agenda Item:	2

Work Item / Release:	FS_15LIS

Abstract of the contribution: This contribution proposes an ASN.1 module header, some necessary repairs to make the module compile (although other changes are also required), and a number of editorial improvements.

This contribution does the following

- Adds a module definition (will need to agree with Nokia’s contribution s3i190114)

- Applies “PL” style to the whole annex

- Removes the rogue non-ASN.1 text that had been copied across

- Corrects the SliceInformation structure to use the correct braces (“[]” not “()”)

- Removes duplicate definitions of SliceInformation, FiveGMMCause, AMFRegistrationResult and AMFRegistrationType

- Add missing “--” to comments in two of the SMF events

- Moves a number of comments from inside to outside the SEQUENCEs they refer to

- Tries to make consistent spacing between declarations, tabs etc.

This contribution doesn’t

- Supply any of the missing types (see s3i190116)

- Touch “Location” structures (see Roger’s contribution)

- Add the top-level PDU structures (see s3i190115)

FIRST CHANGE (Annex A)

Annex A (normative): Structure of the Internal Interface

TS33128HIOperations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) ts33128(19) r15(15) version0(0)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

UPFPDURecord ::= OCTET STRING

-- See clause 6.2.2.2.2 for details of this structure

XIRIAMFRegistrationMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.2 for details of this structu	registrationType			[1] AMFRegistrationType,

registrationResult			[2]	AMFRegistrationResult,

sliceInformation			[3]	SliceInformation			OPTIONAL,

supi						[4]	SUPI,

suci						[5] SUCI 						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI 						OPTIONAL,

guti						[8]	GUTI,

location					[9]	LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] UEEndpointAddress 			OPTIONAL

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

SliceInformation	::= SEQUENCE

{

allowedNSSAI					[1]	NSSAI			OPTIONAL,

configuredNSSAI					[2]	NSSAI			OPTIONAL,

rejectedNSSAI					[3]	RejectedNSSAI	OPTIONAL

}

-- See clause 6.2.2.2.3 for details of this structure

XIRIAMFDeregistrationMessage ::= SEQUENCE

{

deregistrationDirection	[1]	AMFDirection,

accessType				[2]	AMFAccessType,

supi					[3]	SUPI					OPTIONAL,

suci					[4]	SUCI					OPTIONAL,

pei						[5]	PEI						OPTIONAL,

gpsi					[6] GPSI					OPTIONAL,

guti					[7]	GUTI					OPTIONAL,

cause					[8]	FiveGMMCause			OPTIONAL,

location				[9] LocationInformation		OPTIONAL

}

AMFDirection	::=	ENUMERATED

{

networkInitiated 	(1),

ueInitiated			(2)

}

AMFAccessType	::=	ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPandNonThreeGPPAccess	(3)

}

FiveGMMCause	::= INTEGER (0..255)

-- See clause 6.2.2.2.4 for details of this structure

XIRIAMFLocationUpdateMessage ::= SEQUENCE

{

supi				[1]	SUPI,

suci				[2]	SUCI				OPTIONAL,

pei					[3]	PEI					OPTIONAL,

gpsi				[4] GPSI				OPTIONAL,

guti				[5]	GUTI				OPTIONAL,

location			[6] LocationInformation

}

-- See clause 6.2.2.2.5 for details of this structure

XIRIAMFStartOfInterceptionWithRegisteredUEMessage ::= SEQUENCE

{

registrationResult			[1]		AMFRegistrationResult,

registrationType			[2]		AMFRegistrationType			OPTIONAL,

sliceInformation	[3]		SliceInformation 	OPTIONAL,

supi						[4]		SUPI,

suci						[5] 	SUCI						OPTIONAL,

pei							[6] 	PEI						OPTIONAL,

gpsi						[7] 	GPSI						OPTIONAL,

guti						[8]		GUTI,

location					[9]		LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] 	UEEndpointAddress 			OPTIONAL,

timeOfRegistration			[11]	Timestamp 					OPTIONAL

}

-- See clause 6.2.2.2.6 for details of this structure

XIRIAMFUnsuccessfulProcedure ::= SEQUENCE

{

failedprocedureType			[1] FailedAMFProcedureType,

failurecause				[2]	AMFFailureCause,

requestedSliceInformation	[3]	NSSAI	OPTIONAL,

supi						[4]	SUPI						OPTIONAL,

suci						[5] SUCI						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI						OPTIONAL,

guti						[8]	GUTI						OPTIONAL,

location					[9]	LocationInformation 		OPTIONAL

}

FailedAMFProcedureType ::= ENUMERATED

{

registration            (1),

deregistration          (2),

locationUpdate          (3),

sms                     (4),

pDUSessionEstablishment (5)

}

AMFFailureCause	::= CHOICE

{

fiveGMMCause 	[1]		FiveGMMCause,

fiveGSMCause	[2]		FiveGSMCause

}

FiveGSMCause	::= INTEGER (0..255)

-- See clause 6.2.3.2.2 for details of this structure

XIRISMFPDUSessionEstablishmentMessage ::= SEQUENCE

{

supi					[0]		SUPI 							OPTIONAL,

supiUnauthenticated 	[1] 	SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] 	PEI 							OPTIONAL,

gpsi					[3] 	GPSI 							OPTIONAL,

pduSessionID			[4] 	PDUSessionID,

gtpTunnelID				[5] 	FTEID,

pduSessionType			[6]		PDUSessionType,

sNSSAI					[7] 	SNSSAI OPTIONAL,

ueEndpoint				[8] 	SEQUENCE OF UEEndpointAddress	OPTIONAL,

non3GPPAccessEndpoint 	[9] 	UEEndpointAddress 				OPTIONAL,

locationInformation		[10]	Location 						OPTIONAL,

dnn						[11] 	DNN,

amfID					[12] 	AMFID							OPTIONAL,

hSMFURI					[13] 	HSMFURI							OPTIONAL,

requestType				[14] 	FiveGSMRequestType,

accessType				[15] 	AMFAccessType 					OPTIONAL,

ratType					[16] 	RATType 						OPTIONAL,

smPDUDNRequest			[17] 	SMPDUDNRequest 					OPTIONAL

}

PDUsessionType ::= ENUMERATED

{

ipv4 			(1),

ipv6 			(2),

ipv4v6 			(3),

unstructured	(4),

ethernet		(5)

}

PDUSessionID ::= INTEGER (0..255)

SNSSAI ::= SEQUENCE

{

sliceServiceType 	[0] INTEGER (0..255),

sliceDifferentiator	[1]	OCTET STRING (SIZE(3)) OPTIONAL

}

FTEID ::= SEQUENCE

{

teid		[0] INTEGER (0.. 4294967295),

ipv4Address	[1] IPv4Address OPTIONAL,

ipv6Address	[2] IPv6Address OPTIONAL

}

UEEndpointAddress ::= CHOICE

{

ipv4Address		[1] IPv4Address,

ipv6Address		[2] IPv6Address,

ethernetAddress	[3] MACAddress

}

SMPDUDNRequest ::= OCTET STRING

DNN ::= UTF8STRING

SUPIUnauthenticatedIndication ::= BOOLEAN

AMFID ::= SEQUENCE

{

amfRegionID		[1] INTEGER (0..255),

amfSetID		[2] INTEGER (0..1023),

amfPointer		[3] INTEGER (0..63)

}

HSMFURI ::= UTF8String

RATType ::= ENUMERATED

{

nr(0),

eutra(1),

wlan(2),

virtual(3)

}

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(0),

existingPDUSession(1),

initialEmergencyRequest(2),

existingEmergencyPDUSession(3),

modificationRequest(4),

reserved(5)

}

-- See clause 6.2.3.2.3 for details of this structure

XIRISMFPDUSessionModificationMessage ::= SEQUENCE

{

supi					[0]	SUPI 							OPTIONAL,

supiUnauthenticated		[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

sNSSAI					[4] SNSSAI 							OPTIONAL,

non3GPPAccessEndpoint 	[5] UEEndpointAddress 				OPTIONAL,

locationInformation		[6]	Location 						OPTIONAL,

requestType				[7] FiveGSMRequestType,

accessType				[8] AMFAccessType 					OPTIONAL,

ratType					[9] RATType 						OPTIONAL

}

-- See clause 6.2.3.3.4 for details of this structure

XIRISMFPDUSessionReleaseMessage ::= SEQUENCE

{

supi				[0]	SUPI,

pei					[1] PEI 			OPTIONAL,

gpsi				[2] GPSI 			OPTIONAL,

pduSessionID		[3] PDUSessionID,

timeOfFirstPacket	[4] GeneralizedTime OPTIONAL,

timeOfLastPacket	[5] GeneralizedTime OPTIONAL,

uplinkVolume		[6] INTEGER 		OPTIONAL,

downlinkVolume		[7] INTEGER 		OPTIONAL,

locationInformation	[8]	Location 		OPTIONAL

}

-- See clause 6.2.3.3.5 for details of this structure

XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage ::= SEQUENCE

{

supi					[0]	SUPI 							OPTIONAL,

supiUnauthenticated 	[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

pduSessionID			[4] PDUSessionID,

gtpTunnelID				[5] FTEID,

pduSessionType			[6]	PDUSessionType,

sNSSAI					[7] SNSSAI 							OPTIONAL,

ueEndpoint				[8] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint 	[9] UEEndpointAddress 				OPTIONAL,

locationInformation		[10] Location 						OPTIONAL,

dnn						[11] DNN,

amfID					[12] AMFID							OPTIONAL,

hSMFURI					[13] HSMFURI						OPTIONAL,

requestType				[14] FiveGSMRequestType,

accessType				[15] AMFAccessType 					OPTIONAL,

ratType					[16] RATType 						OPTIONAL,

smPDUDNRequest			[17] SMPDUPDNRequest				OPTIONAL

}

-- See clause 6.2.5.3 for details of this structure

XIRISMSMessage ::= SEQUENCE

{

originatingSMSParty		[1] SMSParty,

terminatingSMSParty		[2] SMSParty,

direction				[3] SMSDirection,

transferStatus			[4] SMSTransferStatus,

otherMessage			[5] SMSOtherMessageIndication 	OPTIONAL,

location				[6]	LocationInformation 		OPTIONAL,

otherNFAddress			[7] SMSNFAddress 				OPTIONAL,

otherNFType				[8] SMSNFType 					OPTIONAL,

smsTPDUData				[9] SMSTPDUData					OPTIONAL

}

SMSParty ::= SEQUENCE

{

supi	[0] SUPI 	OPTIONAL,

pei		[1] PEI		OPTIONAL,

gpsi	[2] GPSI	OPTIONAL

}

SMSDirection ::= ENUMERATED

{

toTarget 	(0),

fromTarget 	(1)

}

SMSTransferStatus ::=  ENUMERATED

{

transferSucceeded 	(0),

transferFailed		(1),

undefined 			(2)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

ipAddress	[0] IPAddress,

e164Number	[1] E164Number
}

SMSNFType ::= ENUMERATED

{

sms-GMSC	(0),

iwMSC		(1),

sms-Router	(2)

}

SMSTPDUData ::= CHOICE

{

smsTPDU [0] SMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

XIRILALSReportMessage ::= SEQUENCE

{

sUPI						[1]	SUPI 					OPTIONAL,

pEI							[2] PEI 					OPTIONAL,

gPSI						[3] GPSI 					OPTIONAL,

location					[4]	LocationInformation		OPTIONAL,

rawMLPResponse				[5] RawMLPResponse 			OPTIONAL,

mLPErrorCode				[6] MLPErrorCode 			OPTIONAL

}

RawMLPResponse ::= UTF8String

MLPErrorCode ::= INTEGER (1..699)

END

END OF CHANGES