3GPP TSG-SA3 Meeting #79-LI-e-b	s3i200710

# Online, , 10th Nov 2020 - 12th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0140 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Provisioning for LI at the SMF/UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD, NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 33.128 is missing provisioning details for LI at the SMF/UPF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provisioning messages for LI at the SMF is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | National Regulations may not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of First Change ***

### 6.2.3.1	Provisioning over LI_X1

##### 6.2.3.1.C1	General

If the warrant is for IRI and CC, then the IRI-POI and the CC-TF in the SMF shall be provisioned in accordance with clause 6.2.3.1.C2, the MDF2 shall be provisioned in accordance with clause 6.2.3.1.C3, and the MDF3 shall be provisioned in accordance with clause 6.2.3.1.C4.

If the warrant is for IRI only, the IRI-POI in the SMF shall be provisioned in accordance with 6.2.3.1.C2 and the MDF2 shall be provisioned in accordance with clause 6.2.3.1.C3.

If approach 1 described in clause 6.2.3.9 is used for packet header reporting, the IRI-TF in the SMF shall be provisioned in accordance with clause 6.2.3.1.C2 and the MDF2 shall be provisioned in accordance with clause 6.2.3.1.C3.  If approach 2 described in clause 6.2.3.9 is used for packet header reporting, the CC-TF in the SMF shall be provisioned in accordance with clause 6.2.3.1.C2, the MDF2 shall be provisioned in accordance with clause 6.2.3.1.C3, and the MDF3 shall be provisioned in accordance with clause 6.2.3.1.C4.

##### 6.2.3.1.C2	Provisioning of the IRI-POI, IRI-TF and CC-TF in the SMF

The IRI-POI, IRI-TF and CC-TF present in the SMF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI/TF in the SMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

-	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.3.1-T1 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI, in the SMF.

Table 6.2.3-T1: ActivateTask message for SMF IRI-POI, CC-TF and IRI-TF

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. If the CC-TF or IRI-TF is also being tasked for the same interception, the same XID shall be used. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the paragraph above. | M |
| DeliveryType | For ActivateTask messages sent to the IRI-POI and IRI-TF, set to "X2Only". For ActivateTask messages sent to the CC-TF, set to “X3Only”. | M |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of ETSI TS 103 221-1 [7]. See Table 6.2.3-10. This field shall be present if the packet header reporting is required. Only permitted to be sent to the IRI-TF. | C |
| ListOfDIDs | Delivery endpoints of LI_X2 or LI_X3. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |

If packet header reporting is required, parameters specified in table 6.2.3-10: ActivatePDHReporting parameters shall be provided as the TaskDetailsExtensions/HeaderReporting field of the LI_X1 provisioning message.

##### 6.2.3.1.C3	Provisioning of the MDF2

The MDF2 listed as the delivery endpoint for xIRI generated by the IRI-POI in the SMF or the IRI-POI in the UPF shall be provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2. Table 6.2.3-T2 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF2.

The MDF2 shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

-	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.3-T2: ActivateTask message for MDF2

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the paragraph above. | M |
| DeliveryType | Set to “X2Only” or "X2andX3" as authorised by the warrant. (Ignored by the MDF2) | M |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of ETSI TS 103 221-1 [7]. See Table 6.2.3-10. This field shall be present if packet header reporting is required. | C |
| ListOfDIDs | Delivery endpoints of LI_HI2. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See Table 6.2.3-T3. | M |

Table 6.2.3-T3: Mediation Details for MDF2

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI2Only". | M |
| ListOfDIDs | Details of where to send the IRI for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScoping | Shall be included to Identify the service(s) and associated service-related delivery settings for this LIID. May include more than one instance of this parameter to allow for different combinations of subparameters associated with a single LIID. This parameter is defined in ETSI TS 103 221-1 [7], Annex C, Table C.2. | C |

##### 6.2.3.1.C4	Provisioning of the MDF3

The MDF3 listed as the delivery endpoint for the xCC generated by the CC-POI in the UPF shall be provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2. Table 6.2.3-T4 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF3.  If packet header reporting is authorised and approach 2 described in clause 6.2.3.5 is used, the endpoint for the MDF3 shall be the MDF2 over LI_MDF.

The MDF3 shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

-	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.3-T4: ActivateTask message for MDF3

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the paragraph above. | M |
| DeliveryType | Set to “X3Only” or "X2andX3" as authorised by the warrant. (Ignored by the MDF3) | M |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of ETSI TS 103 221-1 [7]. See Table 6.2.3-10. This field shall be present if packet header reporting is required. | C |
| ListOfDIDs | Delivery endpoints of LI_HI3 or LI_MDF. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See Table 6.2.3-T5. | M |

Table 6.2.3-T5: Mediation Details for MDF3

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI3Only". | M |
| ListOfDIDs | Details of where to send the CC for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScoping | Shall be included to Identify the service(s) and associated service-related delivery settings for this LIID. May include more than one instance of this parameter to allow for different combinations of subparameters associated with a single LIID. This parameter is defined in ETSI TS 103 221-1 [7], Annex C, Table C.2. | C |

*** End of All Changes ***