3GPP TSG-SA3 Meeting #82-LI-e-b	s3i210639

# Online, , 1st Sep 2021 - 3rd Sep 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0253 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to start of interception with registered UE record at the AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI(OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current table for the start of interception with registered UE states that the record should be populated with information provided at the initial registration. This information may not be available. This CR changes the table to say that the information should come from the UE context in the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the table in clause 6.2.2.2.5 to state that the information used to populate the record parameters comes from the UE context at the AMF rather than the initial registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required information may not be available, therefore, preventing LEAs from accress to pertinent target communication information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of First Change ***

##### 6.2.2.2.5	Start of interception with registered UE

The IRI-POI in the AMF shall generate an xIRI containing an AMFStartOfInterceptionWithRegisteredUE record when the IRI-POI present in the AMF detects that interception is activated on a UE that has already been registered in the 5GS (see clause 6.2.2.4 on identity privacy). A UE is considered already registered to the 5GS when the 5GMM state for the access type (3GPP NG-RAN or non-3GPP access) for that UE is 5GMM-REGISTERED. Therefore, the IRI-POI present in the AMF shall generate the xIRI AMFStartOfInterceptionWithRegisteredUE record when it detects that a new interception for a UE is activated (i.e. provisioned by the LIPF) and the 5G mobility management state for the access type (3GPP NG-RAN or non-3GPP access) within the AMF for that UE is 5GMM-REGISTERED. If the UE is registered over both 3GPP NG-RAN and non-3GPP access, the IRI-POI present in the AMF shall generate an xIRI containing an AMFStartOfInterceptionWithRegisteredUE record for each access type.

Table 6.2.2-4: Payload for AMFStartOfInterceptionWithRegisteredUE record

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationResult | Specifies the result of registration, see TS 24.501 [13], clause 9.11.3.6. The value of this parameter is derived from the information stored in the UE Context at the AMF, see TS 23.502 [4] clause 5.2.2.2.2. | M |
| registrationType | Specifies the type of registration, see TS 24.501 [13] clause 9.11.3.7, if available. The value of this parameter is derived from the information stored in the UE Context at the AMF, see TS 23.502 [4] clause 5.2.2.2.2. | C |
| slice | Provide, if available, one or more of the following: - allowed NSSAI (see TS 24.501 [13] clause 9.11.3.37). - configured NSSAI (see TS 24.501 [13] clause 9.11.3.37). The value of this parameter is derived from the information stored in the UE Context at the AMF, see TS 23.502 [4] clause 5.2.2.2.2. | C |
| sUPI | SUPI associated with the target UE Context at the AMF (see clause 6.2.2.4 and TS 23.502 [4] clause 5.2.2.2.2). | M |
| sUCI | SUCI used in the registration, if available. | C |
| pEI | PEI associated with the target UE context at the AMF, if available (see TS 23.502 [4] clause 5.2.2.2.2). | C |
| gPSI | GPSI associated with the target UE Context at the AMF, if available (see TS 23.502 [4] clause 5.2.2.2.2). | C |
| gUTI | Current 5G-GUTI associated with the target UE Context at the AMF (see TS 23.502 [4] clause 5.2.2.2.2). | M |
| location | Location information associated with the access type in the UE Context at the AMF (see TS 23.502 [4] clause 5.2.2.2.2), if available. Encoded as a userLocation parameter (location>locationInfo>userLocation) and, when Dual Connectivity is activated, as an additionalCellIDs parameter (location>locationInfo>additionalCellIDs), see Annex A. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, TNGF or TWIF, if available in the UE Context at the AMF (see TS 23.502 [4] clause 5.2.2.2.2). IP addresses are given as 4 octets (for IPv4) or 16 octets (for IPv6) with the most significant octet first (network byte order). | C |
| timeOfRegistration | Time at which the last registration occurred, if available. This is the time stamp when the REGISTRATION ACCEPT message was sent to the UE or (when applicable) when the REGISTRATION COMPLETE was received from the UE. Shall be given qualified with time zone information (i.e. as UTC or offset from UTC, not as local time). | C |
| fiveGSTAIList | List of tracking areas associated with the UE for the access type in the UE Context at the AMF (See TS 23.502 clause 5.2.2.2.2). | C |
| sMSoverNASIndicator | Indicates whether SMS over NAS is supported. Provide, if included in the UE Context at the AMF (See TS 23.502 clause 5.2.2.2.2). | C |
| oldGUTI | GUTI or 5G-GUTI, if provided in the REGISTRATION REQUEST message, see TS 24.501 [13] clause 5.5.1.2.2. | C |
| eMM5GRegStatus | UE Status, if available in the UE Context at the MME, see (TS 23.502 clause 5.2.2.2.2). | C |

The IRI-POI present in the AMF generating an xIRI containing an AMFStartOfInterceptionWithRegisteredUE record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

*** End of All Changes ***