3GPP TSG-SA3 Meeting #86-LI-e-a	s3i220350r3

# Online, , 13th Jul 2022 - 15th Jul 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0356 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Computing Aware UE stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2022-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some control plane events and information are missing without the following CR for Edge Computing based on Edge aware UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New IRI-PoI is introduced within Edge Enabler Server with related LI events based on 3GPP specs on Edge Computing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issues for CSPs that develop Edge Computing systems without LI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.X , Annex A, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | * | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | * | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | * | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/55 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/583d48e074b94b8b76682525f83c7f7273c107e2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i220311, s3i220327, s3i220350 |  |  |  |  |  |  |  |  |

#### **Start of First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

[8]	ETSI TS 103 221-2: "Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)".

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: "Numbering, addressing and identification ".

[20]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf.

[21]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[22]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[23]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[24]	3GPP TS 29.572: "Location Management Services; Stage 3".

[25]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[26]	IETF RFC 815: "IP datagram reassembly algorithms".

[27]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[28]	IETF RFC 793: "Transmission Control Protocol".

[29]	IETF RFC 768: "User Datagram Protocol".

[30]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[31]	IETF RFC 4960: "Stream Control Transmission Protocol".

[32]	IANA (www.iana.org): Assigned Internet Protocol Numbers, "Protocol Numbers".

[33]	IETF RFC 6437: "IPv6 Flow Label Specification".

[34]	IETF RFC 791: "Internet Protocol".

[35]	Open Geospatial Consortium OGC 05-010: "URNs of definitions in ogc namespace".

[36]	3GPP TS 33.107: "3G security; Lawful interception architecture and functions".

[37]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[38]	3GPP TS 36.413: "S1 Application Protocol (S1AP)".

[39]	OMA-TS-MMS_ENC-V1_3-20110913-A: "Multimedia Messaging Service Encapsulation Protocol".

[40]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[41]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IETF RFC 4566: "SDP: Session Description Protocol".

[44]	3GPP TS 24.193: "Stage 3: Access Traffic Steering, Switching and Splitting (ATSSS)".

[45]	3GPP TS 29.509: "5G System; Authentication Server Services; Stage 3".

[46]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[47]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[48]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[49]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[50]	3GPP TS 23.401 "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[51]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS), Stage 3".

[52]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[53]	3GPP TS 29.172 "Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface".

[54]	3GPP TS 29.171 "LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[55]	3GPP TS 24.379: "Mission Critical Push to Talk (MCPTT) call control; protocol specification".

[56]	OMA-TS-PoC-System_Description-V2_1-20110802-A: "OMA PoC System Description".

[57]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD); Stage 3".

[58]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[59]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs); Stage 3".

[60]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications".

[61]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[62]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[63]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[64]	3GPP TS 29.598: "5G System; Unstructured Data Storage Services; Stage3".

[65]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[66]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[67]	GSMA IR.88: "IR.88 LTE and EPC Roaming Guidelines".

[68]	GSMA NG.114 "IMS Profile for Voice, Video and Messaging over 5GS".

[69]	IETF RFC 8225: "PASSporT: Personal Assertion Token".

[70]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[71]	IETF RFC 8588: "Personal Assertion Token (PaSSporT) Extension for Signature-based Handling of Asserted information using toKENs (SHAKEN)".

[72]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[73]	IETF draft-ietf-stir-passport-rcd-12: "PASSporT Extension for Rich Call Data".

NOTE:	The above document cannot be formally referenced until it is published as an RFC.

[74]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP)and Session Description Protocol (SDP); Stage 3".

[75]	IANA Session Initiation Protocol (SIP) Parameters: https://www.iana.org/assignments/sip-parameters/sip-parameters.xhtml

[76]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[77]	3GPP TS 23.204: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[78]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[79]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[80]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[81]	IETF RFC 5438: "Instant Message Disposition Notification (IMDN)".

[82]	OMA-TS-CPM_System_Description-V2_2-20170926-C: "OMA Converged IP Messaging System Description".

[83]	IETF RFC 4566: "SDP: Session Description Protocol".

[84]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa) ".

[85]	3GPP TS 37.355: "LTE Positioning Protocol (LPP) ".

[86]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[87]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[88]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping".

[89]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[90]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[91]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[92]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[XX]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[YY]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification".

#### *** End of First Change ***

#### *** Start of Second Change ***

## 7.X	LI at EES

### 7.X.1	Provisioning over LI_X1

#### 7.X.1.1	Provisioning of IRI-POI in EES

The IRI-POI present in the EES is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the EES shall support the identifier types given in table 7.X.1-1.

Table 7.X.1-1: TargetIdentifier Types for Edge Computing

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |
| eECID | 3GPP | TargetIdentifierExtension | See XSD schema |

Table 7.X.1-2 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the EES.

If the IRI-POI in the EES receives an ActivateTask message and the ListOfServiceTypes parameter contains a ServiceType that is not supported, the IRI-POI in the EES shall reject the task with an appropriate error as described in ETSI TS 103 221-1 [7] clause 6.2.1.2.

Table 7.X.1-2: ActivateTask message for the IRI-POI in the EES

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One of the target identifiers listed in the paragraph above. | M |
| DeliveryType | Set to “X2Only”. | M |
| ListOfDIDs | Delivery endpoints for LI_X2 for the IRI-POI in the EES. These delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to the task activation. | M |
| ListOfServiceTypes | Shall be included when the task should only intercept specific CSP service types as described in clause 5.2.4. This parameter is defined in ETSI TS 103 221-1 [7], clause 6.2.1.2, table 4. | C |

#### 7.X.1.2	Provisioning of the MDF2

The MDF2 listed as the delivery endpoint over LI_X2 for xIRI generated by the EES shall be provisioned over LI_X1 by the LIPF.

The target identities listed in clause 7.X.1.1 shall apply for the provisioning of MDF2.

Table 7.X.1-3 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF2.

Table 7.X.1-3 ActivateTask message for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in clause 7.X.1.1. | M |
| DeliveryType | Set to “X2Only". (Ignored by the MDF2). | M |
| ListOfDIDs | Delivery endpoints of LI_HI2. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See table 7.X.1-3. | M |

Table 7.X.1-3: Mediation Details for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI2Only". | M |
| ListOfDIDs | Details of where to send the IRI for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScoping | Using the format defined in ETS TS 103 221 [7] include the service scoping as applicable to this LIID based on the service scoping listed above the table. | C |

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver IRI message for each additional warrant without receiving a corresponding xIRI.

### 7.X.2	Generation of xIRI at IRI-POI in EES over LI_X2

#### 7.X.2.1	General

The IRI-POI present in the EES shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.4, the details of which are described in the following clauses.

NOTE:	If GPSI is the target,  then xIRIs shall be generated only if the GPSI is available.

#### 7.X.2.2	EEC registration

The IRI-POI in the EES shall generate an xIRI containing an EESEECRegistration record when the IRI-POI present in the EES detects that an EEC has registered, updated its registration or deregistered. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES returns Eees_EECRegistration_Request response towards the EEC confirming the registration of the EEC for the target UE to the EES (as defined in TS 24.588 [XX] clause 5.2.2.2).

-	EES returns Eees_EECRegistration_Update response towards the EEC confirming the update of the registration information of the EEC for the target UE at the EES (as defined in TS 24.588 [XX] clause 5.2.2.3).

-	EES returns Eees_EECRegistration_Deregister response towards the EEC confirming the deregistration of the EEC for the target UE from a given EES (as defined in TS 24.588 [XX] clause 5.2.2.4).

Table 7.X.2-1: EESEECRegistration record

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationType | Types of registration. Possible values are: “Registration”, “Registration Update”, “Deregistration”. | M |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| aCProfiles | Profiles of application clients (ACs) for which the EEC provides edge enabling services, if available. See Table 7.X.2-2. | C |
| eECServiceContSupport | ACR (Application Context Relocation) scenarios supported by the EEC for service continuity if any. | C |
| expirationTime | Expiration time for the registration. If absent for registration types “Registration” and “Registration Update”, registration of EEC never expires. | C |
| eECContextID | Unique identifier of the EEC context if available. | C |
| srcEESID | Identifier of the EES providing the EEC context identifier, if available. | C |
| unfulfilledACProfiles | If requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include “unfulfilledAcProfiles” attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons, if available. | C |
| failureResponse | Cause information when the registration, registration update or deregistration has failed, if available. | C |

Table 7.X.2-2: Details of aCProfile parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| aCID | Application client identity. | M |
| aCType | Category or type of application client, if available. | C |
| aCSchedule | Expected operation schedule of the AC (e.g. time windows) if available. | C |
| expACGeoServArea | Expected location(s) of the hosting UE during the AC’s operation schedule, if available. | C |
| eASsInfo | List of EAS that serve the AC along with the service KPIs required by the AC if available. See Table 7.X.2-3. | C |
| aCServiceContSupport | ACR scenarios supported by the AC for service continuity, if any. | C |

Table 7.X.2-3: Details of eASInfo parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASId | EAS identity. | M |
| expectedSvcKPIs | KPIs expected in order for ACs to receive currently required services from the EAS. See Table 7.X.2-4. | C |
| minimumReqSvcKPIs | Minimum KPIs required in order for Acs to receive meaningful services from the EAS. See Table 7.X.2-4. | C |

Table 7.X.2-4: Details of expectedSvcKPIs and minimumReqSvcKPIs parameters

| Field name | Description | M/C/O |
| --- | --- | --- |
| connectionBandwidth | Required connection bandwidth in Kbit/s for the application. | C |
| requestRate | Request rate to be generated by the AC. | C |
| responseTime | Response time required for the server servicing the requests. | C |
| requestedAvailability | Percentage of time the server is required to be available for the AC’s use. | C |
| requestedCompute | Compute resources required by the AC. | C |
| requestedGraphCompute | Graphical compute resources required by the AC. | C |
| requestedMemory | Memory resources required by the AC. | C |
| requestedStorage | Storage resources required by the AC. | C |

#### 7.X.2.3	EAS discovery

The IRI-POI in the EES shall generate an xIRI containing an EESEASDiscovery record when the IRI-POI present in the EES detects that an EEC has requested a one-time EAS discovery information. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES returns Eees_EASDiscovery_Request response to the EEC containing a one-time EAS discovery information (as defined in TS 24.558 [XX] clause 5.3.2.2).

Table 7.X.2-5: EESEASDiscovery record

| Field name | Description | M/C/O |
| --- | --- | --- |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| eASDiscoveryFilter | Set of characteristics to determine required EASs, if available. | C |
| eECServiceContSupport | ACR scenarios supported by the EEC for service continuity if any. | C |
| locationInfo | Location information of the target UE, if available. | C |
| eASTargetDNAIs | Target DNAI information which can be associated with potential target EAS(s), if available. | C |
| discoveredEAS | List of discovered EAS(s), if available. | C |
| failureResponse | Cause information when the discovery request has failed, if available. | C |

Table 7.X.2-6: Details of eASDiscoveryFilter parameterendpoint

| Field name | Description | M/C/O |
| --- | --- | --- |
| aCCharacteristics | ACs for which a matching EAS is needed. See Table 7.X.2-2. | C |
| eASCharacteristics | Characteristics of required EASs. See Table 7.X.2-7. | C |
| NOTE: Either aCCharacteristics or eASCharacteristics shall be present. |  |  |

Table 7.X.2-7: Details of eASCharacteristics parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASID | EAS Identity, if available. | C |
| aSPID | Identity of the ASP which provides the EAS, if available. | C |
| eASType | EAS type or category, if available. | C |
| eASSchedule | Availability schedule of the EAS (e.g. time windows), if available. | C |
| eASProfile | Profiles of the discovered EASs, if available. See Table 7.X.2-8. | C |
| eASServiceArea | List of geographical and topological service areas that the EAS serves, if available. | C |
| eASServicePermLevel | Level of service permissions (e.g. trial, gold-class) supported by the EAS, if available. | C |

Table 7.X.2-8: Details of discoveredEAS parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASProfile | Profiles of the discovered EASs. See Table 7.X.2-9. | C |
| Lifetime | Time duration in seconds for which the EAS information is valid and supposed to be cached in the EEC. | C |

Table 7.X.2-9: Details of eASProfile parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASID | EAS Identity. | M |
| eASEndpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. | M |
| aCIDs | Identities of the AC(s) that can be served by the EAS, if available. | C |
| aSPID | Identity of the ASP which provides the EAS, if available. | C |
| eASType | EAS type or category, if available. | C |
| eASDescription | Human-readable description of the EAS. | C |
| eASSchedule | Availability schedule of the EAS (e.g. time windows), if available. | C |
| eASServiceArea | List of geographical and topological service areas that the EAS serves, if available. | C |
| eASServiceKPIs | Service characteristics provided by the EAS. | C |
| eASServicePermLevel | Level of service permissions (e.g. trial, gold-class) supported by the EAS, if available. | C |
| eASServiceFeatures | Service features e.g. single vs. multi-player gaming service supported by the EAS, if available. | C |
| eASServiceContSupport | ACR scenarios supported by the EAS for service continuity if any. | C |
| appLocs | List of DNAI(s) and the corresponding N6 traffic routing information/routing profile ID, associated with the EAS, if available. | C |
| eASStatus | EAS status (e.g. Enabled, Disabled etc.), if available. | C |

#### 7.X.2.4	EAS discovery subscription

The IRI-POI in the EES shall generate an xIRI containing an EESEASDiscoverySubscription record when the IRI-POI present in the EES detects that an EEC has requested to subscribe, update subscription and unsubscribe to EAS discovery information reporting. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES returns Eees_EASDiscovery_Subscribe response to the EEC confirming its subscription to EAS discovery information reporting (as defined in TS 24.558 [XX] clause 5.3.2.3).

-	EES returns Eees_EASDiscovery_UpdateSubscription response to the EEC confirming the update of its subscription at the EES for EAS discovery information reporting (as defined in TS 24.558 [XX] clause 5.3.2.5).

-	EES returns Eees_EASDiscovery_Unsubscribe response to the EEC confirming the deletion of an existing subscription at the EES to EAS discovery information reporting (as defined in TS 24.558 [XX] clause 5.3.2.6)

Table 7.X.2-10: EESEASDiscoverySubscription record

| Field name | Description | M/C/O |
| --- | --- | --- |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| subscriptionType | Subscription type, i.e., "Subscription”, “Subscription Update” or “Unsubscription”. | M |
| eASEventType | Event type for which the EEC shall be notified | M |
| eASDiscoveryFilter | Set of characteristics to determine required EASs, if available. | C |
| eASDynamicInfoFilter | List of EAS dynamic information required by the EEC per EAS, if available. See Table 7.X.2-11. | C |
| eECServiceContSupport | ACR scenarios supported by the EC for service continuity if any. | C |
| expirationTime | Expiration time for the subscription. If absent for subscription types “Subscription” and Subscription Update”, EAS discovery subscription from EEC never expires. | C |
| subscriptionId | Subscription identity, if available. | C |
| failureResponse | Cause information when the discovery request has failed, if available. | C |

Table 7.X.2-11: Details of eASDynamicInfoFilter parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASId | EAS identifier. | M |
| eASStatus | Notify if EAS status changed. | C |
| eASACIDs | Notify if list of AC identifiers changed. | C |
| eASDescription | Notify if EAS description changed. | C |
| eASEndpoint | Notify if EAS endpoint changed. | C |
| eASFeature | Notify if EAS feature changed. | C |
| eASSchedule | Notify if EAS schedule changed. | C |
| eASServiceArea | Notify if EAS service area changed. | C |
| eASServiceKPIs | Notify if EAS KPIs changed. | C |
| eASServiceContinuity | Notify if EAS supported ACR changed. | C |

#### 7.X.2.5	EAS discovery notification

The IRI-POI in the EES shall generate an xIRI containing an EESEASDiscoveryNotification record when the IRI-POI present in the EES detects that the EES has notified a previously subscribed EEC on EAS discovery information. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES receives an Eees_EASDiscovery_Notify response from an EEC confirming that the subscribed EEC has received EAS discovey information (as defined in TS 24.558 [XX] clause 5.3.2.4).

Note : Because it is HTTP-based transaction the notification request is always supposed to be received by the EEC which needs to answer wwith the reponse.

Table 7.X.2-12: EESEASDiscoveryNotification record

| Field name | Description | M/C/O |
| --- | --- | --- |
| subscriptionID | Identity of the individual subscription for which the notification is delivered. | M |
| eventType | Event type for which the notification is delivered; | M |
| discoveredEAS | List of EAS discovery information. See Table 7.X.2-8. | M |
| failureResponse | Cause information when the discovery notification request has failed, if available. | C |

#### 7.X.2.6	Application context relocation

The IRI-POI in the EES shall generate an xIRI containing an EESAppContextRelocation record when the IRI-POI present in the EES detects that an EEC has requested ACR (Application Context Relocation)f determination.or ACR initiation. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES returns Eees_AppContextRelocation_Determine response to the EEC confirming that EEC has carried out ACR determination (as defined in TS 24.558 [XX] clause 5.5.2.2). EEC requests that the EES evaluates if ACR is needed and subsequently initiate the ACR procedure if required.

-	EES returns Eees_AppContextRelocation_Initiate response to the EEC confirming that EEC has carried out ACR initiation (as defined in TS 24.558 [XX] clause 5.5.2.3). EEC requests initiation of an ACR procedure.

Table 7.X.2-13: EESAppContextRelocation record

| Field name | Value | M/C/O |
| --- | --- | --- |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| aCRDetermineReq | See Table 7.X.2-14. | C |
| aCRInitiateReq | See Table 7.X.2-15. | C |

Table 7.X.2-14: Details of aCRDetermineReq parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASID | Identifier of the EAS, if available. | C |
| aCID | Identifier of the AC, if available. | C |
| sEASEndpoint | Endpoint information of the selected S-EAS. | M |

Table 7.X.2-15: Details of aCRInitiateReq parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| eASID | Identifier of the EAS, if available. | C |
| aCID | Identifier of the AC, if available. | C |
| tEASEndpoint | Endpoint information of the T-EAS (target EAS). | M |
| sEASEndpoint | Endpoint information of the S-EAS (serving EAS). | C |
| previousTEASEndpoint | Endpoint information of the previous T-EAS. | C |
| routeReq | T-EAS's DNAI information and corresponding N6 traffic routing information and/or routing profile ID, if available. | C |

#### 7.X.2.7	Application context relocation subscription

The IRI-POI in the EES shall generate an xIRI containing an EESACRSubscription record when the IRI-POI present in the EES detects that an EEC has requested to subscribe, update subscription and unsubscribe to ACR events. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES returns Eees_ACREvents_Subscribe response to the EEC confirming its subscription for reporting of ACR information notification (as defined in TS 24.558 [XX] clause 5.4.2.2).

-	EES returns Eees_ACREvents_UpdateSubscription response to the EEC confirming the update of its subscription for reporting of ACR information notification (as defined in TS 24.558 [XX] clause 5.4.2.4).

-	EES returns Eees_ACREvents_Unsubscribe response to the EEC confirming the deletion of an existing subscription at the EES to ACR events (as defined in TS 24.558 [XX] clause 5.4.2.5)

Table 7.X.2-16: EESACRSubscription record

| Field name | Description | M/C/O |
| --- | --- | --- |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| subscriptionType | Subscription type, i.e., “Subscription”, “Subscription Update” or “Unsubscription”. | M |
| expirationTime | Expiration time for the subscription. If absent for subscription types “Subscription” and Subscription Update”, application context relocation subscription from EEC never expires. | C |
| eASIDs | List of identifier of the EASs, if available. | M |
| aCIDs | List of identifier of the ACs, if available. | C |
| eventIDs | Specifies the events for which EEC is subscribing. | M |
| subscriptionId | Subscription identity, if available | C |
| failureResponse | Cause information when the discovery request has failed, if available. | C |

#### 7.X.2.8	Application context relocation notification

The IRI-POI in the EES shall generate an xIRI containing an EESACRNotification record when the IRI-POI present in the EES detects that the EES has notify a previously subscribed EEC on EAS discovery information. The IRI-POI present in the EES shall generate the xIRI for the following events:

-	EES receives an Eees_ACREvents_Notify response from an EEC confirming that the subscribed EEC has received a notification of the ACR information events from EES (as defined in TS 24.558 [XX] clause 5.4.2.2).

Table 7.X.2-17: EESACRNotification record

| Field name | Description | M/C/O |
| --- | --- | --- |
| subscriptionID | Identity of the individual subscription for which the notification is delivered. | M |
| eASID | Identifier of the EAS. | M |
| eventIDs | Specifies the events for which notification is sent. | M |
| targetInfo | Details of the selected T-EAS and the T-EES. It is present when EventID indicates “TARGET_INFORMATION” event. | C |
| aCRRes | Indicates whether the ACR is successful or has failed. It is present when EventID indicates “ACR_COMPLETE” event. | C |
| failReason | Indicates the cause information for the failure when the ACRRes attribute is present and indicates failure, if available. | C |

#### 7.X.2.9	EEC context relocation

The IRI-POI in the EES shall generate an xIRI containing an EESEECContextRelocation record when the IRI-POI present in the EES detects that the EES has sent or received the EEC context relocation information. The IRI-POI present in the EES shall generate the xIRI for the following events according to its target EES (T-EES) or serving EES (S-EES) role:

-	T-EES receives an Eees_EECContextRelocation_Pull response from a S-EES upon request from the T-EES to S-EES (as defined in TS 29.558 [YY] clause 5.10.2.2).

-	T-EES sends an Eees_EECContextRelocation_Push response to a S-EES upon request from the S-EES to T-EES (as defined in TS 29.558 [YY] clause 5.10.2.3).

-	S-EES sends an Eees_EECContextRelocation_Pull response to a T-EES.

-	S-EES receives an Eees_EECContextRelocation_Push response from a T-EES.

NOTE : The term target EES is used in TS 29.558[YY] referring the new serving EES and the term serving EES is referring to the old serving EES.

Table 7.X.2-17: EESEECContextRelocation record

| Field name | Description |  | M/C/O |
| --- | --- | --- | --- |
| eECID | Unique identifier of the EEC. |  | M |
| gPSI | GPSI of the target UE, if available. |  | C |
| cntxtID | Unique identifier of the EEC context. |  | M |
| uELoc | Latest location information of the target UE, that is available at the EES, if available. |  | C |
| aCProfiles | Profiles of application clients (ACs) for which the EEC provides edge enabling services, if available. See Table 7.X.2-2. | C |  |

#### 7.X.2.10	Start of interception with registered EEC

The IRI-POI in the AES shall generate an xIRI containing an EESStartOfInterceptionWithRegisteredEEC record when the IRI-POI present in the EES detects that interception is activated on a UE which EEC has already registered with an EES.

Table 7.X.2-18: EESStartOfInterceptionWithRegisteredEEC record

| Field name | Description | M/C/O |
| --- | --- | --- |
| eECID | Unique identifier of the EEC. | M |
| gPSI | GPSI of the target UE, if available. | C |
| aCProfiles | Profiles of application clients (ACs) for which the EEC provides edge enabling services, if available. See Table 7.X.2-2. | C |
| eECServiceContSupport | ACR scenarios supported by the EEC for service continuity if any. | C |
| expirationTime | Expiration time for the registration. If absent for registration types “Registration” and “Registration Update”, registration of EEC never expires. | C |
| eECContextID | Unique identifier of the EEC context, if available. | C |
| srcEESID | Identifier of the EES providing the EEC context identifier, if available. | C |
| unfulfilledACProfiles | If requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include "unfulfilledAcProfiles" attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons, if available. | C |
| timeOfRegistration | Time at which the last registration occurred, if available. | C |

#### 7.X.2.11	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the EES, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the EES event was observed (i.e. the timestamp field of the xIRI).

The IRI type parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.X.2-18.

Table 7.X.2-19: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| EESEECRegistration | REPORT |
| EESEASDiscovery | REPORT |
| EESEASDiscoverySubscription | REPORT |
| EESEASDiscoveryNotification | REPORT |
| EESAppContextRelocation | REPORT |
| EESACRSubscription | REPORT |
| EESACRNotification | REPORT |
| EESEECContextRelocation | REPORT |
| EESStartOfInterceptionWithRegisteredEEC | REPORT |

The threeGPP33128DefinedIRI field (see ETSI TS 102 232-7 [10] clause 15) shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the EESStartOfInterceptionWithRegisteredEEC record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the EESStartOfInterceptionWithRegisteredEEC record is specified in table 7.X.2-18. The MDF2 shall generate and deliver the IRI message containing the EESStartOfInterceptionWithRegistered record for each of the registrations to the LEMF associated with the new warrant.

MDF2 delivers the IRI to the LEMF with GPSI as the target identity if and only if GPSI is present in the xIRI.

#### *** End of Second Change ***

#### *** Start of Third Change ***

Annex A (normative):
ASN.1 Schema for the Internal and External Interfaces

TS33128Payloads

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) ts33128(19) r18(18) version0(0)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

-- =============

-- Relative OIDs

-- =============

tS33128PayloadsOID          RELATIVE-OID ::= {threeGPP(4) ts33128(19) r18(18) version0(0)}

xIRIPayloadOID              RELATIVE-OID ::= {tS33128PayloadsOID xIRI(1)}

xCCPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID xCC(2)}

iRIPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID iRI(3)}

cCPayloadOID                RELATIVE-OID ::= {tS33128PayloadsOID cC(4)}

lINotificationPayloadOID    RELATIVE-OID ::= {tS33128PayloadsOID lINotification(5)}

-- ===============

-- X2 xIRI payload

-- ===============

XIRIPayload ::= SEQUENCE

{

xIRIPayloadOID      [1] RELATIVE-OID,

event               [2] XIRIEvent

}

XIRIEvent ::= CHOICE

{

-- Access and mobility related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulAMProcedure                             [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSMProcedure                             [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.1

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- tag 16 is reserved because there is no equivalent mDFCellSiteReport in XIRIEvent

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                       [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                               [55] UDMCancelLocationMessage,

-- SMS-related events continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssociation                            [62] AMFIdentifierAssociation,

mMEIdentifierAssociation                            [63] MMEIdentifierAssociation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

-- EPS Events, see clause 6.3

-- MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- AKMA key management events, see clause 7.9.1

aAnFAnchorKeyRegister                               [92] AAnFAnchorKeyRegister,

aAnFKAKMAApplicationKeyGet                          [93] AAnFKAKMAApplicationKeyGet,

aAnFStartOfInterceptWithEstablishedAKMAKeyMaterial  [94] AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial,

aAnFAKMAContextRemovalRecord                        [95] AAnFAKMAContextRemovalRecord,

aFAKMAApplicationKeyRefresh                         [96] AFAKMAApplicationKeyRefresh,

aFStartOfInterceptWithEstablishedAKMAApplicationKey [97] AFStartOfInterceptWithEstablishedAKMAApplicationKey,

aFAuxiliarySecurityParameterEstablishment           [98] AFAuxiliarySecurityParameterEstablishment,

aFApplicationKeyRemoval                             [99] AFApplicationKeyRemoval,

-- HR LI Events, see clause 7.10.3.3

n9HRPDUSessionInfo                                  [100] N9HRPDUSessionInfo,

s8HRBearerInfo                                      [101] S8HRBearerInfo,

-- Separated Location Reporting, see clause 7.3.4

separatedLocationReporting                          [102] SeparatedLocationReporting,

-- STIR SHAKEN and RCD/eCNAM Events, see clause 7.11.2

sTIRSHAKENSignatureGeneration                       [103] STIRSHAKENSignatureGeneration,

sTIRSHAKENSignatureValidation                       [104] STIRSHAKENSignatureValidation,

-- IMS events, see clause 7.12.4.2

iMSMessage                                          [105] IMSMessage,

startOfInterceptionForActiveIMSSession              [106] StartOfInterceptionForActiveIMSSession,

iMSCCUnavailable                                    [107] IMSCCUnavailable,

-- UDM events, see clause 7.2.2

uDMLocationInformationResult                        [108] UDMLocationInformationResult,

uDMUEInformationResponse                            [109] UDMUEInformationResponse,

uDMUEAuthenticationResponse                         [110] UDMUEAuthenticationResponse,

-- AMF events, see 6.2.2.2.8

positioningInfoTransfer                             [111] AMFPositioningInfoTransfer,

-- MME Events, see clause 6.3.2.2.8

mMEPositioningInfoTransfer                          [112] MMEPositioningInfoTransfer,

-- AMF events, see 6.2.2.2.9 continued from choice 5

aMFRANHandoverCommand                               [113] AMFRANHandoverCommand,

aMFRANHandoverRequest                               [114] AMFRANHandoverRequest,

-- EES events, see clause 7.X.2

eESEECRegistration                                  [201] EESEECRegistration,

eESEASDiscovery                                     [202] EESEASDiscovery,

eESEASDiscoverySubscription                         [203] EESEASDiscoverySubscription,

eESEASDiscoveryNotification                         [204] EESEASDiscoveryNotification,

eESAppContextRelocation                             [205] EESAppContextRelocation,

eESACRSubscription                                  [206] EESACRSubscription,

eESACRNotification                                  [207] EESACRNotification,

eESEECContextRelocation                             [208] EESEECContextRelocation,

eESStartOfInterceptionWithRegisteredEEC             [209] EESStartOfInterceptionWithRegisteredEEC

}

-- ==============

-- X3 xCC payload

-- ==============

-- No additional xCC payload definitions required in the present document.

-- ===============

-- HI2 IRI payload

-- ===============

IRIPayload ::= SEQUENCE

{

iRIPayloadOID       [1] RELATIVE-OID,

event               [2] IRIEvent,

targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

}

IRIEvent ::= CHOICE

{

-- Registration-related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulRegistrationProcedure                   [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSessionProcedure                        [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.1

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- MDF-related events, see clause 7.3.2

mDFCellSiteReport                                   [16] MDFCellSiteReport,

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                      [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                              [55] UDMCancelLocationMessage,

-- SMS-related events, continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssociation                           [62] AMFIdentifierAssociation,

mMEIdentifierAssociation                           [63] MMEIdentifierAssociation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2,

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

-- EPS Events, see clause 6.3

-- MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- AKMA key management events, see clause 7.9.1

aAnFAnchorKeyRegister                               [92] AAnFAnchorKeyRegister,

aAnFKAKMAApplicationKeyGet                          [93] AAnFKAKMAApplicationKeyGet,

aAnFStartOfInterceptWithEstablishedAKMAKeyMaterial  [94] AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial,

aAnFAKMAContextRemovalRecord                        [95] AAnFAKMAContextRemovalRecord,

aFAKMAApplicationKeyRefresh                         [96] AFAKMAApplicationKeyRefresh,

aFStartOfInterceptWithEstablishedAKMAApplicationKey [97] AFStartOfInterceptWithEstablishedAKMAApplicationKey,

aFAuxiliarySecurityParameterEstablishment           [98] AFAuxiliarySecurityParameterEstablishment,

aFApplicationKeyRemoval                             [99] AFApplicationKeyRemoval,

-- tag 100 is reserved because there is no equivalent n9HRPDUSessionInfo in IRIEvent.

-- tag 101 is reserved because there is no equivalent S8HRBearerInfo in IRIEvent.

-- Separated Location Reporting, see clause 7.3.4

separatedLocationReporting                          [102] SeparatedLocationReporting,

-- STIR SHAKEN and RCD/eCNAM Events, see clause 7.11.3

sTIRSHAKENSignatureGeneration                       [103] STIRSHAKENSignatureGeneration,

sTIRSHAKENSignatureValidation                       [104] STIRSHAKENSignatureValidation,

-- IMS events, see clause 7.11.4.2

iMSMessage                                          [105] IMSMessage,

startOfInterceptionForActiveIMSSession              [106] StartOfInterceptionForActiveIMSSession,

iMSCCUnavailable                                    [107] IMSCCUnavailable,

-- UDM events, see clause 7.2.2

uDMLocationInformationResultRecord                  [108] UDMLocationInformationResult,

uDMUEInformationResponse                            [109] UDMUEInformationResponse,

uDMUEAuthenticationResponse                         [110] UDMUEAuthenticationResponse,

-- AMF events, see 6.2.2.2.8

positioningInfoTransfer                             [111] AMFPositioningInfoTransfer,

-- MME Events, see clause 6.3.2.2.8

mMEPositioningInfoTransfer                          [112] MMEPositioningInfoTransfer,

-- AMF events, see 6.2.2.2.9 continued from choice 5

aMFRANHandoverCommand                               [113] AMFRANHandoverCommand,

aMFRANHandoverRequest                               [114] AMFRANHandoverRequest,

-- EES events, see clause 7.X.2

eESEECRegistration                                  [201] EESEECRegistration,

eESEASDiscovery                                     [202] EESEASDiscovery,

eESEASDiscoverySubscription                         [203] EESEASDiscoverySubscription,

eESEASDiscoveryNotification                         [204] EESEASDiscoveryNotification,

eESAppContextRelocation                             [205] EESAppContextRelocation,

eESACRSubscription                                  [206] EESACRSubscription,

eESACRNotification                                  [207] EESACRNotification,

eESEECContextRelocation                             [208] EESEECContextRelocation,

eESStartOfInterceptionWithRegisteredEEC             [209] EESStartOfInterceptionWithRegisteredEEC

}

IRITargetIdentifier ::= SEQUENCE

{

identifier                                          [1] TargetIdentifier,

provenance                                          [2] TargetIdentifierProvenance OPTIONAL

}

-- ==============

-- HI3 CC payload

-- ==============

CCPayload ::= SEQUENCE

{

cCPayloadOID         [1] RELATIVE-OID,

pDU                  [2] CCPDU

}

CCPDU ::= CHOICE

{

uPFCCPDU            [1] UPFCCPDU,

extendedUPFCCPDU    [2] ExtendedUPFCCPDU,

mMSCCPDU            [3] MMSCCPDU,

nIDDCCPDU           [4] NIDDCCPDU,

pTCCCPDU            [5] PTCCCPDU,

iMSCCPDU            [6] IMSCCPDU

}

-- ===========================

-- HI4 LI notification payload

-- ===========================

LINotificationPayload ::= SEQUENCE

{

lINotificationPayloadOID         [1] RELATIVE-OID,

notification                     [2] LINotificationMessage

}

LINotificationMessage ::= CHOICE

{

lINotification      [1] LINotification

}

-- =================

-- HR LI definitions

-- =================

N9HRPDUSessionInfo ::= SEQUENCE

{

sUPI                            [1] SUPI,

pEI                             [2] PEI OPTIONAL,

pDUSessionID                    [3] PDUSessionID,

location                        [4] Location OPTIONAL,

sNSSAI                          [5] SNSSAI OPTIONAL,

dNN                             [6] DNN OPTIONAL,

messageCause                    [7] N9HRMessageCause

}

S8HRBearerInfo ::= SEQUENCE

{

iMSI                            [1] IMSI,

iMEI                            [2] IMEI OPTIONAL,

bearerID                        [3] EPSBearerID,

linkedBearerID                  [4] EPSBearerID OPTIONAL,

location                        [5] Location OPTIONAL,

aPN                             [6] APN OPTIONAL,

sGWIPAddress                    [7] IPAddress OPTIONAL,

messageCause                    [8] S8HRMessageCause

}

-- ================

-- HR LI parameters

-- ================

N9HRMessageCause ::= ENUMERATED

{

pDUSessionEstablished(1),

pDUSessionModified(2),

pDUSessionReleased(3),

updatedLocationAvailable(4),

sMFChanged(5),

other(6),

hRLIEnabled(7)

}

S8HRMessageCause ::= ENUMERATED

{

bearerActivated(1),

bearerModified(2),

bearerDeleted(3),

pDNDisconnected(4),

updatedLocationAvailable(5),

sGWChanged(6),

other(7),

hRLIEnabled(8)

}

-- ==================

-- 5G NEF definitions

-- ==================

-- See clause 7.7.2.1.2 for details of this structure

NEFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

pDUSessionID          [3] PDUSessionID,

sNSSAI                [4] SNSSAI,

nEFID                 [5] NEFID,

dNN                   [6] DNN,

rDSSupport            [7] RDSSupport,

sMFID                 [8] SMFID,

aFID                  [9] AFID

}

-- See clause 7.7.2.1.3 for details of this structure

NEFPDUSessionModification ::= SEQUENCE

{

sUPI                         [1] SUPI,

gPSI                         [2] GPSI,

sNSSAI                       [3] SNSSAI,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

aFID                         [8] AFID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.7.2.1.4 for details of this structure

NEFPDUSessionRelease ::= SEQUENCE

{

sUPI                   [1] SUPI,

gPSI                   [2] GPSI,

pDUSessionID           [3] PDUSessionID,

timeOfFirstPacket      [4] Timestamp OPTIONAL,

timeOfLastPacket       [5] Timestamp OPTIONAL,

uplinkVolume           [6] INTEGER OPTIONAL,

downlinkVolume         [7] INTEGER OPTIONAL,

releaseCause           [8] NEFReleaseCause

}

-- See clause 7.7.2.1.5 for details of this structure

NEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] NEFFailureCause,

sUPI                         [2] SUPI,

gPSI                         [3] GPSI OPTIONAL,

pDUSessionID                 [4] PDUSessionID,

dNN                          [5] DNN OPTIONAL,

sNSSAI                       [6] SNSSAI OPTIONAL,

rDSDestinationPortNumber     [7] RDSPortNumber,

applicationID                [8] ApplicationID,

aFID                         [9] AFID

}

-- See clause 7.7.2.1.6 for details of this structure

NEFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI               [1] SUPI,

gPSI               [2] GPSI,

pDUSessionID       [3] PDUSessionID,

dNN                [4] DNN,

sNSSAI             [5] SNSSAI,

nEFID              [6] NEFID,

rDSSupport         [7] RDSSupport,

sMFID              [8] SMFID,

aFID               [9] AFID

}

-- See clause 7.7.3.1.1 for details of this structure

NEFDeviceTrigger ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID,

aFID                  [4] AFID,

triggerPayload        [5] TriggerPayload OPTIONAL,

validityPeriod        [6] INTEGER OPTIONAL,

priorityDT            [7] PriorityDT OPTIONAL,

sourcePortId          [8] PortNumber OPTIONAL,

destinationPortId     [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.2 for details of this structure

NEFDeviceTriggerReplace ::= SEQUENCE

{

sUPI                     [1] SUPI,

gPSI                     [2] GPSI,

triggerId                [3] TriggerID,

aFID                     [4] AFID,

triggerPayload           [5] TriggerPayload OPTIONAL,

validityPeriod           [6] INTEGER OPTIONAL,

priorityDT               [7] PriorityDT OPTIONAL,

sourcePortId             [8] PortNumber OPTIONAL,

destinationPortId        [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.3 for details of this structure

NEFDeviceTriggerCancellation ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID

}

-- See clause 7.7.3.1.4 for details of this structure

NEFDeviceTriggerReportNotify ::= SEQUENCE

{

sUPI                             [1] SUPI,

gPSI                             [2] GPSI,

triggerId                        [3] TriggerID,

deviceTriggerDeliveryResult      [4] DeviceTriggerDeliveryResult

}

-- See clause 7.7.4.1.1 for details of this structure

NEFMSISDNLessMOSMS ::= SEQUENCE

{

sUPI                      [1] SUPI,

gPSI                      [2] GPSI,

terminatingSMSParty       [3] AFID,

sMS                       [4] SMSTPDUData OPTIONAL,

sourcePort                [5] PortNumber OPTIONAL,

destinationPort           [6] PortNumber OPTIONAL

}

-- See clause 7.7.5.1.1 for details of this structure

NEFExpectedUEBehaviourUpdate ::= SEQUENCE

{

gPSI                                  [1] GPSI,

expectedUEMovingTrajectory            [2] SEQUENCE OF UMTLocationArea5G OPTIONAL,

stationaryIndication                  [3] StationaryIndication OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

batteryIndication                     [8] BatteryIndication OPTIONAL,

trafficProfile                        [9] TrafficProfile OPTIONAL,

expectedTimeAndDayOfWeekInTrajectory  [10] SEQUENCE OF UMTLocationArea5G OPTIONAL,

aFID                                  [11] AFID,

validityTime                          [12] Timestamp OPTIONAL

}

-- ==========================

-- Common SCEF/NEF parameters

-- ==========================

RDSSupport ::= BOOLEAN

RDSPortNumber ::= INTEGER (0..15)

RDSAction ::= ENUMERATED

{

reservePort(1),

releasePort(2)

}

SerializationFormat ::= ENUMERATED

{

xml(1),

json(2),

cbor(3)

}

ApplicationID ::= OCTET STRING

NIDDCCPDU ::= OCTET STRING

TriggerID ::= UTF8String

PriorityDT ::= ENUMERATED

{

noPriority(1),

priority(2)

}

TriggerPayload ::= OCTET STRING

DeviceTriggerDeliveryResult ::= ENUMERATED

{

success(1),

unknown(2),

failure(3),

triggered(4),

expired(5),

unconfirmed(6),

replaced(7),

terminate(8)

}

StationaryIndication ::= ENUMERATED

{

stationary(1),

mobile(2)

}

BatteryIndication ::= ENUMERATED

{

batteryRecharge(1),

batteryReplace(2),

batteryNoRecharge(3),

batteryNoReplace(4),

noBattery(5)

}

ScheduledCommunicationTime ::= SEQUENCE

{

days [1] SEQUENCE OF Daytime

}

UMTLocationArea5G ::= SEQUENCE

{

timeOfDay        [1] Daytime,

durationSec      [2] INTEGER,

location         [3] NRLocation

}

Daytime ::= SEQUENCE

{

daysOfWeek       [1] Day OPTIONAL,

timeOfDayStart   [2] Timestamp OPTIONAL,

timeOfDayEnd     [3] Timestamp OPTIONAL

}

Day ::= ENUMERATED

{

monday(1),

tuesday(2),

wednesday(3),

thursday(4),

friday(5),

saturday(6),

sunday(7)

}

TrafficProfile ::= ENUMERATED

{

singleTransUL(1),

singleTransDL(2),

dualTransULFirst(3),

dualTransDLFirst(4),

multiTrans(5)

}

ScheduledCommunicationType ::= ENUMERATED

{

downlinkOnly(1),

uplinkOnly(2),

bidirectional(3)

}

-- =================

-- 5G NEF parameters

-- =================

NEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

contextNotFound(3),

portNotFree(4),

portNotAssociatedWithSpecifiedApplication(5)

}

NEFReleaseCause ::= ENUMERATED

{

sMFRelease(1),

dNRelease(2),

uDMRelease(3),

cHFRelease(4),

localConfigurationPolicy(5),

unknownCause(6)

}

AFID ::= UTF8String

NEFID ::= UTF8String

-- ==================

-- SCEF definitions

-- ==================

-- See clause 7.8.2.1.2 for details of this structure

SCEFPDNConnectionEstablishment ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.2.1.3 for details of this structure

SCEFPDNConnectionUpdate ::= SEQUENCE

{

iMSI                         [1] IMSI OPTIONAL,

mSISDN                       [2] MSISDN OPTIONAL,

externalIdentifier           [3] NAI OPTIONAL,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

sCSASID                      [8] SCSASID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.8.2.1.4 for details of this structure

SCEFPDNConnectionRelease ::= SEQUENCE

{

iMSI                   [1] IMSI OPTIONAL,

mSISDN                 [2] MSISDN OPTIONAL,

externalIdentifier     [3] NAI OPTIONAL,

ePSBearerID            [4] EPSBearerID,

timeOfFirstPacket      [5] Timestamp OPTIONAL,

timeOfLastPacket       [6] Timestamp OPTIONAL,

uplinkVolume           [7] INTEGER OPTIONAL,

downlinkVolume         [8] INTEGER OPTIONAL,

releaseCause           [9] SCEFReleaseCause

}

-- See clause 7.8.2.1.5 for details of this structure

SCEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] SCEFFailureCause,

iMSI                         [2] IMSI OPTIONAL,

mSISDN                       [3] MSISDN OPTIONAL,

externalIdentifier           [4] NAI OPTIONAL,

ePSBearerID                  [5] EPSBearerID,

aPN                          [6] APN,

rDSDestinationPortNumber     [7] RDSPortNumber OPTIONAL,

applicationID                [8] ApplicationID OPTIONAL,

sCSASID                      [9] SCSASID

}

-- See clause 7.8.2.1.6 for details of this structure

SCEFStartOfInterceptionWithEstablishedPDNConnection ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.3.1.1 for details of this structure

SCEFDeviceTrigger ::= SEQUENCE

{

iMSI                  [1] IMSI,

mSISDN                [2] MSISDN,

externalIdentifier    [3] NAI,

triggerId             [4] TriggerID,

sCSASID               [5] SCSASID OPTIONAL,

triggerPayload        [6] TriggerPayload OPTIONAL,

validityPeriod        [7] INTEGER OPTIONAL,

priorityDT            [8] PriorityDT OPTIONAL,

sourcePortId          [9] PortNumber OPTIONAL,

destinationPortId     [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.2 for details of this structure

SCEFDeviceTriggerReplace ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID,

sCSASID                  [5] SCSASID OPTIONAL,

triggerPayload           [6] TriggerPayload OPTIONAL,

validityPeriod           [7] INTEGER OPTIONAL,

priorityDT               [8] PriorityDT OPTIONAL,

sourcePortId             [9] PortNumber OPTIONAL,

destinationPortId        [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.3 for details of this structure

SCEFDeviceTriggerCancellation ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID

}

-- See clause 7.8.3.1.4 for details of this structure

SCEFDeviceTriggerReportNotify ::= SEQUENCE

{

iMSI                             [1] IMSI OPTIONAL,

mSISDN                           [2] MSISDN OPTIONAL,

externalIdentifier               [3] NAI OPTIONAL,

triggerId                        [4] TriggerID,

deviceTriggerDeliveryResult      [5] DeviceTriggerDeliveryResult

}

-- See clause 7.8.4.1.1 for details of this structure

SCEFMSISDNLessMOSMS ::= SEQUENCE

{

iMSI                      [1] IMSI OPTIONAL,

mSISDN                    [2] MSISDN OPTIONAL,

externalIdentifie         [3] NAI OPTIONAL,

terminatingSMSParty       [4] SCSASID,

sMS                       [5] SMSTPDUData OPTIONAL,

sourcePort                [6] PortNumber OPTIONAL,

destinationPort           [7] PortNumber OPTIONAL

}

-- See clause 7.8.5.1.1 for details of this structure

SCEFCommunicationPatternUpdate ::= SEQUENCE

{

mSISDN                                [1] MSISDN OPTIONAL,

externalIdentifier                    [2] NAI OPTIONAL,

periodicCommunicationIndicator        [3] PeriodicCommunicationIndicator OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

stationaryIndication                  [8] StationaryIndication OPTIONAL,

batteryIndication                     [9] BatteryIndication OPTIONAL,

trafficProfile                        [10] TrafficProfile OPTIONAL,

expectedUEMovingTrajectory            [11] SEQUENCE OF UMTLocationArea5G OPTIONAL,

sCSASID                               [13] SCSASID,

validityTime                          [14] Timestamp OPTIONAL

}

-- =================

-- SCEF parameters

-- =================

SCEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

invalidEPSBearer(3),

operationNotAllowed(4),

portNotFree(5),

portNotAssociatedWithSpecifiedApplication(6)

}

SCEFReleaseCause ::= ENUMERATED

{

mMERelease(1),

dNRelease(2),

hSSRelease(3),

localConfigurationPolicy(4),

unknownCause(5)

}

SCSASID ::= UTF8String

SCEFID ::= UTF8String

PeriodicCommunicationIndicator ::= ENUMERATED

{

periodic(1),

nonPeriodic(2)

}

EPSBearerID ::= INTEGER (0..255)

APN ::= UTF8String

-- =======================

-- AKMA AAnF definitions

-- =======================

AAnFAnchorKeyRegister ::= SEQUENCE

{

aKID                  [1] NAI,

sUPI                  [2] SUPI,

kAKMA                 [3] KAKMA OPTIONAL

}

AAnFKAKMAApplicationKeyGet ::= SEQUENCE

{

type                  [1] KeyGetType,

aKID                  [2] NAI,

keyInfo               [3] AFKeyInfo

}

AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial ::= SEQUENCE

{

aKID                  [1] NAI,

kAKMA                 [2] KAKMA OPTIONAL,

aFKeyList             [3] SEQUENCE OF AFKeyInfo OPTIONAL

}

AAnFAKMAContextRemovalRecord ::= SEQUENCE

{

aKID                  [1] NAI,

nFID                  [2] NFID

}

-- ======================

-- AKMA common parameters

-- ======================

FQDN ::= UTF8String

NFID ::= UTF8String

UAProtocolID ::= OCTET STRING (SIZE(5))

AKMAAFID ::= SEQUENCE

{

aFFQDN                [1] FQDN,

uaProtocolID          [2] UAProtocolID

}

UAStarParams ::= CHOICE

{

tls12                 [1] TLS12UAStarParams,

generic               [2] GenericUAStarParams

}

GenericUAStarParams ::= SEQUENCE

{

genericClientParams [1] OCTET STRING,

genericServerParams [2] OCTET STRING

}

-- ===========================================

-- Specific UaStarParmas for TLS 1.2 (RFC5246)

-- ===========================================

TLSCipherType ::= ENUMERATED

{

stream(1),

block(2),

aead(3)

}

TLSCompressionAlgorithm ::= ENUMERATED

{

null(1),

deflate(2)

}

TLSPRFAlgorithm ::= ENUMERATED

{

rfc5246(1)

}

TLSCipherSuite ::= SEQUENCE (SIZE(2)) OF INTEGER (0..255)

TLS12UAStarParams ::= SEQUENCE

{

preMasterSecret       [1] OCTET STRING (SIZE(6)) OPTIONAL,

masterSecret          [2] OCTET STRING (SIZE(6)),

pRFAlgorithm          [3] TLSPRFAlgorithm,

cipherSuite           [4] TLSCipherSuite,

cipherType            [5] TLSCipherType,

encKeyLength          [6] INTEGER (0..255),

blockLength           [7] INTEGER (0..255),

fixedIVLength         [8] INTEGER (0..255),

recordIVLength        [9] INTEGER (0..255),

macLength             [10] INTEGER (0..255),

macKeyLength          [11] INTEGER (0..255),

compressionAlgorithm  [12] TLSCompressionAlgorithm,

clientRandom          [13] OCTET STRING (SIZE(4)),

serverRandom          [14] OCTET STRING (SIZE(4)),

clientSequenceNumber  [15] INTEGER,

serverSequenceNumber  [16] INTEGER,

sessionID             [17] OCTET STRING (SIZE(0..32)),

tLSExtensions         [18] OCTET STRING (SIZE(0..65535))

}

KAF ::= OCTET STRING

KAKMA ::= OCTET STRING

-- ====================

-- AKMA AAnF parameters

-- ====================

KeyGetType ::= ENUMERATED

{

internal(1),

external(2)

}

AFKeyInfo ::= SEQUENCE

{

aFID                 [1] AKMAAFID,

kAF                  [2] KAF,

kAFExpTime           [3] KAFExpiryTime

}

-- =======================

-- AKMA AF definitions

-- =======================

AFAKMAApplicationKeyRefresh ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

kAF                   [3] KAF,

uaStarParams          [4] UAStarParams OPTIONAL

}

AFStartOfInterceptWithEstablishedAKMAApplicationKey ::= SEQUENCE

{

aFID                  [1] FQDN,

aKID                  [2] NAI,

kAFParamList          [3] SEQUENCE OF AFSecurityParams

}

AFAuxiliarySecurityParameterEstablishment ::= SEQUENCE

{

aFSecurityParams      [1] AFSecurityParams

}

AFSecurityParams ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

kAF                   [3] KAF,

uaStarParams          [4] UAStarParams

}

AFApplicationKeyRemoval ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

removalCause          [3] AFKeyRemovalCause

}

-- ===================

-- AKMA AF parameters

-- ===================

KAFParams ::= SEQUENCE

{

aKID                 [1] NAI,

kAF                  [2] KAF,

kAFExpTime           [3] KAFExpiryTime,

uaStarParams         [4] UAStarParams

}

KAFExpiryTime ::= GeneralizedTime

AFKeyRemovalCause ::= ENUMERATED

{

unknown(1),

keyExpiry(2),

applicationSpecific(3)

}

-- ==================

-- 5G AMF definitions

-- ==================

-- See clause 6.2.2.2.2 for details of this structure

AMFRegistration ::= SEQUENCE

{

registrationType            [1] AMFRegistrationType,

registrationResult          [2] AMFRegistrationResult,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

fiveGSTAIList               [11] TAIList OPTIONAL,

sMSOverNasIndicator         [12] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [13] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [14] EMM5GMMStatus OPTIONAL,

nonIMEISVPEI                [15] NonIMEISVPEI OPTIONAL,

mACRestIndicator            [16] MACRestrictionIndicator OPTIONAL

}

-- See clause 6.2.2.2.3 for details of this structure

AMFDeregistration ::= SEQUENCE

{

deregistrationDirection     [1] AMFDirection,

accessType                  [2] AccessType,

sUPI                        [3] SUPI OPTIONAL,

sUCI                        [4] SUCI OPTIONAL,

pEI                         [5] PEI OPTIONAL,

gPSI                        [6] GPSI OPTIONAL,

gUTI                        [7] FiveGGUTI OPTIONAL,

cause                       [8] FiveGMMCause OPTIONAL,

location                    [9] Location OPTIONAL,

switchOffIndicator          [10] SwitchOffIndicator OPTIONAL,

reRegRequiredIndicator      [11] ReRegRequiredIndicator OPTIONAL

}

-- See clause 6.2.2.2.4 for details of this structure

AMFLocationUpdate ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location,

sMSOverNASIndicator         [7] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [8] EPS5GGUTI OPTIONAL

}

-- See clause 6.2.2.2.5 for details of this structure

AMFStartOfInterceptionWithRegisteredUE ::= SEQUENCE

{

registrationResult          [1] AMFRegistrationResult,

registrationType            [2] AMFRegistrationType OPTIONAL,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

timeOfRegistration          [11] Timestamp OPTIONAL,

fiveGSTAIList               [12] TAIList OPTIONAL,

sMSOverNASIndicator         [13] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [14] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [15] EMM5GMMStatus OPTIONAL

}

-- See clause 6.2.2.2.6 for details of this structure

AMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] AMFFailedProcedureType,

failureCause                [2] AMFFailureCause,

requestedSlice              [3] NSSAI OPTIONAL,

sUPI                        [4] SUPI OPTIONAL,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI OPTIONAL,

location                    [9] Location OPTIONAL

}

-- See clause 6.2.2.2.8 on for details of this structure

AMFPositioningInfoTransfer ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

nRPPaMessage                [6] OCTET STRING OPTIONAL,

lPPMessage                  [7] OCTET STRING OPTIONAL,

lcsCorrelationId            [8] UTF8String (SIZE(1..255))

}

-- See clause 6.2.2.2.9.2 for details of this structure

AMFRANHandoverCommand ::= SEQUENCE

{

userIdentifiers              [1] UserIdentifiers,

aMFUENGAPID                  [2] AMFUENGAPID,

rANUENGAPID                  [3] RANUENGAPID,

handoverType                 [4] HandoverType,

targetToSourceContainer      [5] RANTargetToSourceContainer

}

-- See clause 6.2.2.2.9.3 for details of this structure

AMFRANHandoverRequest ::= SEQUENCE

{

userIdentifiers                     [1] UserIdentifiers,

aMFUENGAPID                         [2] AMFUENGAPID,

rANUENGAPID                         [3] RANUENGAPID,

handoverType                        [4] HandoverType,

handoverCause                       [5] HandoverCause,

pDUSessionResourceInformation       [6] PDUSessionResourceInformation,

mobilityRestrictionList             [7] MobilityRestrictionList OPTIONAL,

locationReportingRequestType        [8] LocationReportingRequestType OPTIONAL,

targetToSourceContainer             [9] RANTargetToSourceContainer,

nPNAccessInformation                [10] NPNAccessInformation OPTIONAL,

sourceToTargetContainer             [11] RANSourceToTargetContainer

}

-- =================

-- 5G AMF parameters

-- =================

AMFID ::= SEQUENCE

{

aMFRegionID [1] AMFRegionID,

aMFSetID    [2] AMFSetID,

aMFPointer  [3] AMFPointer

}

AMFDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

AMFFailedProcedureType ::= ENUMERATED

{

registration(1),

sMS(2),

pDUSessionEstablishment(3)

}

AMFFailureCause ::= CHOICE

{

fiveGMMCause        [1] FiveGMMCause,

fiveGSMCause        [2] FiveGSMCause

}

AMFPointer ::= INTEGER (0..63)

AMFRegistrationResult ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPAndNonThreeGPPAccess(3)

}

AMFRegionID ::= INTEGER (0..255)

AMFRegistrationType ::= ENUMERATED

{

initial(1),

mobility(2),

periodic(3),

emergency(4)

}

AMFSetID ::= INTEGER (0..1023)

AMFUENGAPID ::= INTEGER (0..1099511627775)

-- ==================

-- 5G SMF definitions

-- ==================

-- See clause 6.2.3.2.2 for details of this structure

SMFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

uEEPSPDNConnection          [19] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo             [20] EPS5GSComboInfo OPTIONAL,

selectedDNN                 [21] DNN OPTIONAL,

servingNetwork              [22] SMFServingNetwork OPTIONAL,

oldPDUSessionID             [23] PDUSessionID OPTIONAL,

handoverState               [24] HandoverState OPTIONAL,

gTPTunnelInfo               [25] GTPTunnelInfo OPTIONAL,

pCCRules                    [26] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.3 for details of this structure

SMFPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL,

pDUSessionID                [11] PDUSessionID OPTIONAL,

ePS5GSComboInfo             [12] EPS5GSComboInfo OPTIONAL,

uEEndpoint                  [13] UEEndpointAddress OPTIONAL,

servingNetwork              [14] SMFServingNetwork OPTIONAL,

handoverState               [15] HandoverState OPTIONAL,

gTPTunnelInfo               [16] GTPTunnelInfo OPTIONAL,

pCCRules                    [17] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.4 for details of this structure

SMFPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL,

ePS5GSComboInfo             [11] EPS5GSComboInfo OPTIONAL,

nGAPCause                   [12] NGAPCauseInt OPTIONAL,

fiveGMMCause                [13] FiveGMMCause OPTIONAL,

pCCRuleIDs                  [14] PCCRuleIDSet OPTIONAL

}

-- See clause 6.2.3.2.5 for details of this structure

SMFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

timeOfSessionEstablishment  [19] Timestamp OPTIONAL,

ePS5GSComboInfo             [20] EPS5GSComboInfo OPTIONAL,

uEEPSPDNConnection          [21] UEEPSPDNConnection OPTIONAL,

servingNetwork              [22] SMFServingNetwork OPTIONAL,

gTPTunnelInfo               [23] GTPTunnelInfo OPTIONAL,

pCCRules                    [24] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.6 for details of this structure

SMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

initiator                   [3] Initiator,

requestedSlice              [4] NSSAI OPTIONAL,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

uEEndpoint                  [10] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [11] UEEndpointAddress OPTIONAL,

dNN                         [12] DNN OPTIONAL,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType OPTIONAL,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

location                    [19] Location OPTIONAL

}

-- See clause 6.2.3.2.8 for details of this structure

SMFPDUtoMAPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL,

pDUSessionID                [11] PDUSessionID,

requestIndication           [12] RequestIndication,

aTSSSContainer              [13] ATSSSContainer,

uEEndpoint                  [14] UEEndpointAddress OPTIONAL,

servingNetwork              [15] SMFServingNetwork OPTIONAL,

handoverState               [16] HandoverState OPTIONAL,

gTPTunnelInfo               [17] GTPTunnelInfo OPTIONAL

}

-- See clause 6.2.3.2.7.1 for details of this structure

SMFMAPDUSessionEstablishment ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

pDUSessionType              [6] PDUSessionType,

accessInfo                  [7] SEQUENCE OF AccessInfo,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [10] Location OPTIONAL,

dNN                         [11] DNN,

aMFID                       [12] AMFID OPTIONAL,

hSMFURI                     [13] HSMFURI OPTIONAL,

requestType                 [14] FiveGSMRequestType,

sMPDUDNRequest              [15] SMPDUDNRequest OPTIONAL,

servingNetwork              [16] SMFServingNetwork,

oldPDUSessionID             [17] PDUSessionID OPTIONAL,

mAUpgradeIndication         [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [20] SMFMAAcceptedIndication,

aTSSSContainer              [21] ATSSSContainer OPTIONAL,

uEEPSPDNConnection          [22] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo             [23] EPS5GSComboInfo OPTIONAL,

selectedDNN                 [24] DNN OPTIONAL,

handoverState               [25] HandoverState OPTIONAL,

pCCRules                    [26] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.7.2 for details of this structure

SMFMAPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

accessInfo                  [6] SEQUENCE OF AccessInfo OPTIONAL,

sNSSAI                      [7] SNSSAI OPTIONAL,

location                    [8] Location OPTIONAL,

requestType                 [9] FiveGSMRequestType OPTIONAL,

servingNetwork              [10] SMFServingNetwork,

oldPDUSessionID             [11] PDUSessionID OPTIONAL,

mAUpgradeIndication         [12] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [13] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [14] SMFMAAcceptedIndication,

aTSSSContainer              [15] ATSSSContainer OPTIONAL,

uEEPSPDNConnection          [16] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo             [17] EPS5GSComboInfo OPTIONAL,

handoverState               [18] HandoverState OPTIONAL,

pCCRules                    [19] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.7.3 for details of this structure

SMFMAPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL,

nGAPCause                   [11] NGAPCauseInt OPTIONAL,

fiveGMMCause                [12] FiveGMMCause OPTIONAL,

pCCRuleIDs                  [13] PCCRuleIDSet OPTIONAL

}

-- See clause 6.2.3.2.7.4 for details of this structure

SMFStartOfInterceptionWithEstablishedMAPDUSession ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

pDUSessionType              [6] PDUSessionType,

accessInfo                  [7] SEQUENCE OF AccessInfo,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [10] Location OPTIONAL,

dNN                         [11] DNN,

aMFID                       [12] AMFID OPTIONAL,

hSMFURI                     [13] HSMFURI OPTIONAL,

requestType                 [14] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest              [15] SMPDUDNRequest OPTIONAL,

servingNetwork              [16] SMFServingNetwork,

oldPDUSessionID             [17] PDUSessionID OPTIONAL,

mAUpgradeIndication         [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [20] SMFMAAcceptedIndication,

aTSSSContainer              [21] ATSSSContainer OPTIONAL,

ePS5GSComboInfo             [22] EPS5GSComboInfo OPTIONAL,

uEEPSPDNConnection          [23] UEEPSPDNConnection OPTIONAL,

pCCRules                    [24] PCCRuleSet OPTIONAL

}

-- See clause 6.2.3.2.7.5 for details of this structure

SMFMAUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

requestedSlice              [3] NSSAI OPTIONAL,

initiator                   [4] Initiator,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

accessInfo                  [10] SEQUENCE OF AccessInfo,

uEEndpoint                  [11] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [12] Location OPTIONAL,

dNN                         [13] DNN OPTIONAL,

aMFID                       [14] AMFID OPTIONAL,

hSMFURI                     [15] HSMFURI OPTIONAL,

requestType                 [16] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest              [17] SMPDUDNRequest OPTIONAL

}

-- =================

-- 5G SMF parameters

-- =================

SMFID ::= UTF8String

SMFFailedProcedureType ::= ENUMERATED

{

pDUSessionEstablishment(1),

pDUSessionModification(2),

pDUSessionRelease(3)

}

SMFServingNetwork ::= SEQUENCE

{

pLMNID  [1] PLMNID,

nID     [2] NID OPTIONAL

}

AccessInfo ::= SEQUENCE

{

accessType            [1] AccessType,

rATType               [2] RATType OPTIONAL,

gTPTunnelID           [3] FTEID,

non3GPPAccessEndpoint [4] UEEndpointAddress OPTIONAL,

establishmentStatus   [5] EstablishmentStatus,

aNTypeToReactivate    [6] AccessType OPTIONAL,

gTPTunnelInfo         [7] GTPTunnelInfo OPTIONAL

}

-- see Clause 6.1.2 of TS 24.193[44] for the details of the ATSSS container contents.

ATSSSContainer ::= OCTET STRING

DLRANTunnelInformation ::= SEQUENCE

{

dLQOSFlowTunnelInformation                    [1] QOSFlowTunnelInformation OPTIONAL,

additionalDLQOSFlowTunnelInformation          [2] QOSFlowTunnelInformationList OPTIONAL,

redundantDLQOSFlowTunnelInformation           [3] QOSFlowTunnelInformationList OPTIONAL,

additionalredundantDLQOSFlowTunnelInformation [4] QOSFlowTunnelInformationList OPTIONAL

}

EstablishmentStatus ::= ENUMERATED

{

established(0),

released(1)

}

FiveGSGTPTunnels ::= SEQUENCE

{

uLNGUUPTunnelInformation           [1] FTEID OPTIONAL,

additionalULNGUUPTunnelInformation [2] FTEIDList OPTIONAL,

dLRANTunnelInformation             [3] DLRANTunnelInformation OPTIONAL

}

FiveQI ::= INTEGER (0..255)

HandoverState ::= ENUMERATED

{

none(1),

preparing(2),

prepared(3),

completed(4),

cancelled(5)

}

NGAPCauseInt ::= SEQUENCE

{

group [1] NGAPCauseGroupInt,

value [2] NGAPCauseValueInt

}

-- Derived as described in TS 29.571 [17] clause 5.4.4.12

NGAPCauseGroupInt ::= INTEGER

NGAPCauseValueInt ::= INTEGER

SMFMAUpgradeIndication ::= BOOLEAN

-- Given in YAML encoding as defined in clause 6.1.6.2.31 of TS 29.502[16]

SMFEPSPDNCnxInfo ::= UTF8String

SMFMAAcceptedIndication ::= BOOLEAN

-- see Clause 6.1.6.3.8 of TS 29.502[16] for the details of this structure.

SMFErrorCodes ::= UTF8String

-- see Clause 6.1.6.3.2 of TS 29.502[16] for details of this structure.

UEEPSPDNConnection ::= OCTET STRING

-- see Clause 6.1.6.3.6 of TS 29.502[16] for the details of this structure.

RequestIndication ::= ENUMERATED

{

uEREQPDUSESMOD(0),

uEREQPDUSESREL(1),

pDUSESMOB(2),

nWREQPDUSESAUTH(3),

nWREQPDUSESMOD(4),

nWREQPDUSESREL(5),

eBIASSIGNMENTREQ(6),

rELDUETO5GANREQUEST(7)

}

QOSFlowTunnelInformation ::= SEQUENCE

{

uPTunnelInformation   [1] FTEID,

associatedQOSFlowList [2] QOSFlowLists

}

QOSFlowTunnelInformationList ::= SEQUENCE OF QOSFlowTunnelInformation

QOSFlowDescription ::= OCTET STRING

QOSFlowLists ::= SEQUENCE OF QOSFlowList

QOSFlowList ::= SEQUENCE

{

qFI                      [1] QFI,

qOSRules                 [2] QOSRules OPTIONAL,

eBI                      [3] EPSBearerID OPTIONAL,

qOSFlowDescription       [4] QOSFlowDescription OPTIONAL,

qOSFlowProfile           [5] QOSFlowProfile OPTIONAL,

associatedANType         [6] AccessType OPTIONAL,

defaultQOSRuleIndication [7] BOOLEAN OPTIONAL

}

QOSFlowProfile ::= SEQUENCE

{

fiveQI [1] FiveQI

}

QOSRules ::= OCTET STRING

-- See clauses 5.6.2.6-1 and 5.6.2.9-1 of TS 29.512 [89], clause table 5.6.2.5-1 of TS 29.508 [90] for the details of this structure

PCCRule ::= SEQUENCE

{

pCCRuleID                     [1] PCCRuleID OPTIONAL,

appId                         [2] UTF8String OPTIONAL,

flowInfos                     [3] FlowInformationSet OPTIONAL,

appReloc                      [4] BOOLEAN OPTIONAL,

simConnInd                    [5] BOOLEAN OPTIONAL,

simConnTerm                   [6] INTEGER OPTIONAL,

maxAllowedUpLat               [7] INTEGER OPTIONAL,

trafficRoutes                 [8] RouteToLocationSet,

trafficSteeringPolIdDl        [9] UTF8String OPTIONAL,

trafficSteeringPolIdUl        [10] UTF8String OPTIONAL,

sourceDNAI                    [11] DNAI OPTIONAL,

targetDNAI                    [12] DNAI OPTIONAL,

dNAIChangeType                [13] DNAIChangeType OPTIONAL,

sourceUEIPAddr                [14] IPAddress OPTIONAL,

targetUEIPAddr                [15] IPAddress OPTIONAL,

sourceTrafficRouting          [16] RouteToLocation OPTIONAL,

targetTrafficRouting          [17] RouteToLocation OPTIONAL,

eASIPReplaceInfos             [18] EASIPReplaceInfos OPTIONAL

}

-- See table 5.6.2.14-1 of TS 29.512 [89]

PCCRuleID ::= UTF8String

PCCRuleSet ::= SET OF PCCRule

PCCRuleIDSet ::= SET OF PCCRuleID

FlowInformationSet ::= SET OF FlowInformation

RouteToLocationSet ::= SET OF RouteToLocation

-- See table 5.6.2.14 of TS 29.512 [89]

FlowInformation ::= SEQUENCE

{

flowDescription    [1] FlowDescription OPTIONAL,

ethFlowDescription [2] EthFlowDescription OPTIONAL,

tosTrafficClass    [3] OCTET STRING (SIZE(2)) OPTIONAL,

spi                [4] OCTET STRING (SIZE(4)) OPTIONAL,

flowLabel          [5] OCTET STRING (SIZE(3)) OPTIONAL,

flowDirection      [6] FlowDirection OPTIONAL

}

-- See table 5.6.2.14 of TS 29.512 [89]

FlowDescription ::= SEQUENCE

{

sourceIPAddress       [1] IPAddressOrRangeOrAny,

destinationIPAddress  [2] IPAddressOrRangeOrAny,

sourcePortNumber      [3] PortNumber OPTIONAL,

destinationPortNumber [4] PortNumber OPTIONAL,

protocol              [5] NextLayerProtocolOrAny

}

IPAddressOrRangeOrAny ::= CHOICE

{

iPAddress      [1] IPAddress,

ipAddressRange [2] IPMask,

anyIPAddress   [3] AnyIPAddress

}

IPMask ::= SEQUENCE

{

fromIPAddress [1] IPAddress,

toIPAddress   [2] IPAddress

}

AnyIPAddress ::= ENUMERATED

{

any(1)

}

NextLayerProtocolOrAny ::= CHOICE

{

nextLayerProtocol    [1] NextLayerProtocol,

anyNextLayerProtocol [2] AnyNextLayerProtocol

}

AnyNextLayerProtocol ::= ENUMERATED

{

ip(1)

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

EthFlowDescription ::= SEQUENCE

{

destMacAddress    [1] MACAddress OPTIONAL,

ethType           [2] OCTET STRING (SIZE(2)),

fDesc             [3] FlowDescription OPTIONAL,

fDir              [4] FDir OPTIONAL,

sourceMacAddress  [5] MACAddress OPTIONAL,

vlanTags          [6] SET OF VLANTag,

srcMacAddrEnd     [7] MACAddress OPTIONAL,

destMacAddrEnd    [8] MACAddress OPTIONAL

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

FDir ::= ENUMERATED

{

downlink(1)

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

VLANTag ::= SEQUENCE

{

priority [1] BIT STRING (SIZE(3)),

cFI      [2] BIT STRING (SIZE(1)),

vLANID   [3] BIT STRING (SIZE(12))

}

-- See table 5.6.2.14 of TS 29.512 [89]

FlowDirection ::= ENUMERATED

{

downlinkOnly(1),

uplinkOnly(2),

dowlinkAndUplink(3)

}

-- See table 5.4.2.1 of TS 29.571 [17]

DNAIChangeType ::= ENUMERATED

{

early(1),

earlyAndLate(2),

late(3)

}

-- See table 5.6.2.15 of TS 29.571 [17]

RouteToLocation ::= SEQUENCE

{

dNAI            [1] DNAI,

routeInfo       [2] RouteInfo

}

-- See table 5.4.2.1 of TS 29.571 [17]

DNAI ::= UTF8String

-- See table 5.4.4.16 of TS 29.571 [17]

RouteInfo ::= SEQUENCE

{

iPAddressTunnelEndpoint       [1] IPAddress,

uDPPortNumberTunnelEndpoint   [2] PortNumber

}

-- See clause 4.1.4.2 of TS 29.512 [89]

EASIPReplaceInfos ::= SEQUENCE

{

sourceEASAddress [1] EASServerAddress,

targetEASAddress [2] EASServerAddress

}

-- See clause 4.1.4.2 of TS 29.512 [89]

EASServerAddress ::= SEQUENCE

{

iPAddress        [1]  IPAddress,

port             [2]  PortNumber

}

-- ======================

-- PGW-C + SMF Parameters

-- ======================

EPS5GSComboInfo ::= SEQUENCE

{

ePSInterworkingIndication [1] EPSInterworkingIndication,

ePSSubscriberIDs          [2] EPSSubscriberIDs,

ePSPDNCnxInfo             [3] EPSPDNCnxInfo OPTIONAL,

ePSBearerInfo             [4] EPSBearerInfo OPTIONAL

}

EPSInterworkingIndication ::= ENUMERATED

{

none(1),

withN26(2),

withoutN26(3),

iwkNon3GPP(4)

}

EPSSubscriberIDs ::= SEQUENCE

{

iMSI   [1] IMSI OPTIONAL,

mSISDN [2] MSISDN OPTIONAL,

iMEI   [3] IMEI OPTIONAL

}

EPSPDNCnxInfo ::= SEQUENCE

{

pGWS8ControlPlaneFTEID [1] FTEID,

linkedBearerID         [2] EPSBearerID OPTIONAL

}

EPSBearerInfo ::= SEQUENCE OF EPSBearers

EPSBearers ::= SEQUENCE

{

ePSBearerID         [1] EPSBearerID,

pGWS8UserPlaneFTEID [2] FTEID,

qCI                 [3] QCI

}

QCI ::= INTEGER (0..255)

GTPTunnelInfo ::= SEQUENCE

{

fiveGSGTPTunnels [1] FiveGSGTPTunnels OPTIONAL

}

-- ==================

-- 5G UPF definitions

-- ==================

UPFCCPDU ::= OCTET STRING

-- See clause 6.2.3.8 for the details of this structure

ExtendedUPFCCPDU ::= SEQUENCE

{

payload [1] UPFCCPDUPayload,

qFI     [2] QFI OPTIONAL

}

-- =================

-- 5G UPF parameters

-- =================

UPFCCPDUPayload ::= CHOICE

{

uPFIPCC           [1] OCTET STRING,

uPFEthernetCC     [2] OCTET STRING,

uPFUnstructuredCC [3] OCTET STRING

}

QFI ::= INTEGER (0..63)

-- ==================

-- 5G UDM definitions

-- ==================

UDMServingSystemMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

gUMMEI                      [5] GUMMEI OPTIONAL,

pLMNID                      [6] PLMNID OPTIONAL,

servingSystemMethod         [7] UDMServingSystemMethod,

serviceID                   [8] ServiceID OPTIONAL

}

UDMSubscriberRecordChangeMessage ::= SEQUENCE

{

sUPI                           [1] SUPI OPTIONAL,

pEI                            [2] PEI OPTIONAL,

gPSI                           [3] GPSI OPTIONAL,

oldPEI                         [4] PEI OPTIONAL,

oldSUPI                        [5] SUPI OPTIONAL,

oldGPSI                        [6] GPSI OPTIONAL,

oldserviceID                   [7] ServiceID OPTIONAL,

subscriberRecordChangeMethod   [8] UDMSubscriberRecordChangeMethod,

serviceID                      [9] ServiceID OPTIONAL

}

UDMCancelLocationMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

pLMNID                      [5] PLMNID OPTIONAL,

cancelLocationMethod        [6] UDMCancelLocationMethod

}

UDMLocationInformationResult ::= SEQUENCE

{

sUPI                     [1] SUPI,

pEI                      [2] PEI OPTIONAL,

gPSI                     [3] GPSI OPTIONAL,

locationInfoRequest      [4] UDMLocationInfoRequest,

vPLMNID                  [5] PLMNID OPTIONAL,

currentLocationIndicator [6] BOOLEAN OPTIONAL,

aMFInstanceID            [7] NFID OPTIONAL,

sMSFInstanceID           [8] NFID OPTIONAL,

location                 [9] Location OPTIONAL,

rATType                  [10] RATType OPTIONAL,

problemDetails           [11] UDMProblemDetails OPTIONAL

}

UDMUEInformationResponse ::= SEQUENCE

{

sUPI                        [1] SUPI,

tADSInfo                    [2] UEContextInfo OPTIONAL,

fiveGSUserStateInfo         [3] FiveGSUserStateInfo OPTIONAL,

fiveGSRVCCInfo              [4] FiveGSRVCCInfo OPTIONAL,

problemDetails              [5] UDMProblemDetails OPTIONAL

}

UDMUEAuthenticationResponse ::= SEQUENCE

{

sUPI                        [1] SUPI,

authenticationInfoRequest   [2] UDMAuthenticationInfoRequest,

aKMAIndicator               [3] BOOLEAN OPTIONAL,

problemDetails              [4] UDMProblemDetails OPTIONAL

}

-- =================

-- 5G UDM parameters

-- =================

UDMServingSystemMethod ::= ENUMERATED

{

amf3GPPAccessRegistration(0),

amfNon3GPPAccessRegistration(1),

unknown(2)

}

UDMSubscriberRecordChangeMethod ::= ENUMERATED

{

pEIChange(1),

sUPIChange(2),

gPSIChange(3),

uEDeprovisioning(4),

unknown(5),

serviceIDChange(6)

}

UDMCancelLocationMethod ::= ENUMERATED

{

aMF3GPPAccessDeregistration(1),

aMFNon3GPPAccessDeregistration(2),

uDMDeregistration(3),

unknown(4)

}

ServiceID ::= SEQUENCE

{

nSSAI                     [1] NSSAI OPTIONAL,

cAGID                     [2] SEQUENCE OF CAGID OPTIONAL

}

CAGID ::= UTF8String

UDMAuthenticationInfoRequest ::= SEQUENCE

{

infoRequestType    [1] UDMInfoRequestType,

rGAuthCtx          [2] SEQUENCE SIZE(1..MAX) OF SubscriberIdentifier,

authType           [3] PrimaryAuthenticationType,

servingNetworkName [4] PLMNID,

aUSFInstanceID     [5] NFID OPTIONAL,

cellCAGInfo        [6] CAGID OPTIONAL,

n5GCIndicator      [7] BOOLEAN OPTIONAL

}

UDMLocationInfoRequest ::= SEQUENCE

{

requested5GSLocation     [1] BOOLEAN OPTIONAL,

requestedCurrentLocation [2] BOOLEAN OPTIONAL,

requestedRATType         [3] BOOLEAN OPTIONAL,

requestedTimeZone        [4] BOOLEAN OPTIONAL,

requestedServingNode     [5] BOOLEAN OPTIONAL

}

UDMProblemDetails ::= SEQUENCE

{

cause        [1] UDMProblemDetailsCause OPTIONAL

}

UDMProblemDetailsCause ::= CHOICE

{

uDMDefinedCause       [1] UDMDefinedCause,

otherCause            [2] UDMProblemDetailsOtherCause

}

UDMDefinedCause ::= ENUMERATED

{

userNotFound(1),

dataNotFound(2),

contextNotFound(3),

subscriptionNotFound(4),

other(5)

}

UDMInfoRequestType ::= ENUMERATED

{

hSS(1),

aUSF(2),

other(3)

}

UDMProblemDetailsOtherCause ::= SEQUENCE

{

problemDetailsType   [1] UTF8String OPTIONAL,

title                [2] UTF8String OPTIONAL,

status               [3] INTEGER OPTIONAL,

detail               [4] UTF8String OPTIONAL,

instance             [5] UTF8String OPTIONAL,

cause                [6] UTF8String OPTIONAL,

uDMInvalidParameters [7] UDMInvalidParameters,

uDMSupportedFeatures [8] UTF8String

}

UDMInvalidParameters ::= SEQUENCE

{

parameter    [1] UTF8String OPTIONAL,

reason       [2] UTF8String OPTIONAL

}

-- ===================

-- 5G SMSF definitions

-- ===================

-- See clause 6.2.5.3 for details of this structure

SMSMessage ::= SEQUENCE

{

originatingSMSParty         [1] SMSParty,

terminatingSMSParty         [2] SMSParty,

direction                   [3] Direction,

linkTransferStatus          [4] SMSTransferStatus,

otherMessage                [5] SMSOtherMessageIndication OPTIONAL,

location                    [6] Location OPTIONAL,

peerNFAddress               [7] SMSNFAddress OPTIONAL,

peerNFType                  [8] SMSNFType OPTIONAL,

sMSTPDUData                 [9] SMSTPDUData OPTIONAL,

messageType                 [10] SMSMessageType OPTIONAL,

rPMessageReference          [11] SMSRPMessageReference OPTIONAL

}

SMSReport ::= SEQUENCE

{

location           [1] Location OPTIONAL,

sMSTPDUData        [2] SMSTPDUData,

messageType        [3] SMSMessageType,

rPMessageReference [4] SMSRPMessageReference

}

-- ==================

-- 5G SMSF parameters

-- ==================

SMSAddress ::= OCTET STRING(SIZE(2..12))

SMSMessageType ::= ENUMERATED

{

deliver(1),

deliverReportAck(2),

deliverReportError(3),

statusReport(4),

command(5),

submit(6),

submitReportAck(7),

submitReportError(8),

reserved(9)

}

SMSParty ::= SEQUENCE

{

sUPI        [1] SUPI OPTIONAL,

pEI         [2] PEI OPTIONAL,

gPSI        [3] GPSI OPTIONAL,

sMSAddress  [4] SMSAddress OPTIONAL

}

SMSTransferStatus ::= ENUMERATED

{

transferSucceeded(1),

transferFailed(2),

undefined(3)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

iPAddress   [1] IPAddress,

e164Number  [2] E164Number

}

SMSNFType ::= ENUMERATED

{

sMSGMSC(1),

iWMSC(2),

sMSRouter(3)

}

SMSRPMessageReference ::= INTEGER (0..255)

SMSTPDUData ::= CHOICE

{

sMSTPDU [1] SMSTPDU,

truncatedSMSTPDU [2] TruncatedSMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

TruncatedSMSTPDU ::= OCTET STRING (SIZE(1..130))

-- ===============

-- MMS definitions

-- ===============

MMSSend ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

dateTime            [3]  Timestamp,

originatingMMSParty [4]  MMSParty,

terminatingMMSParty [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients       [7]  SEQUENCE OF MMSParty OPTIONAL,

direction           [8]  MMSDirection,

subject             [9]  MMSSubject OPTIONAL,

messageClass        [10]  MMSMessageClass OPTIONAL,

expiry              [11] MMSExpiry,

desiredDeliveryTime [12] Timestamp OPTIONAL,

priority            [13] MMSPriority OPTIONAL,

senderVisibility    [14] BOOLEAN OPTIONAL,

deliveryReport      [15] BOOLEAN OPTIONAL,

readReport          [16] BOOLEAN OPTIONAL,

store               [17] BOOLEAN OPTIONAL,

state               [18] MMState OPTIONAL,

flags               [19] MMFlags OPTIONAL,

replyCharging       [20] MMSReplyCharging OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

adaptationAllowed   [26] MMSAdaptation OPTIONAL,

contentType         [27] MMSContentType,

responseStatus      [28] MMSResponseStatus,

responseStatusText  [29] UTF8String OPTIONAL,

messageID           [30] UTF8String

}

MMSSendByNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotification ::= SEQUENCE

{

transactionID           [1]  UTF8String,

version                 [2]  MMSVersion,

originatingMMSParty     [3]  MMSParty OPTIONAL,

direction               [4]  MMSDirection,

subject                 [5]  MMSSubject OPTIONAL,

deliveryReportRequested [6]  BOOLEAN OPTIONAL,

stored                  [7]  BOOLEAN OPTIONAL,

messageClass            [8]  MMSMessageClass,

priority                [9]  MMSPriority OPTIONAL,

messageSize             [10]  INTEGER,

expiry                  [11] MMSExpiry,

replyCharging           [12] MMSReplyCharging OPTIONAL

}

MMSSendToNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotificationResponse ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

direction     [3] MMSDirection,

status        [4] MMStatus,

reportAllowed [5] BOOLEAN OPTIONAL

}

MMSRetrieval ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

messageID           [3]  UTF8String,

dateTime            [4]  Timestamp,

originatingMMSParty [5]  MMSParty OPTIONAL,

previouslySentBy    [6]  MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [7]  Timestamp OPTIONAL,

terminatingMMSParty [8]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [9]  SEQUENCE OF MMSParty OPTIONAL,

direction           [10] MMSDirection,

subject             [11] MMSSubject OPTIONAL,

state               [12] MMState OPTIONAL,

flags               [13] MMFlags OPTIONAL,

messageClass        [14] MMSMessageClass OPTIONAL,

priority            [15] MMSPriority,

deliveryReport      [16] BOOLEAN OPTIONAL,

readReport          [17] BOOLEAN OPTIONAL,

replyCharging       [18] MMSReplyCharging OPTIONAL,

retrieveStatus      [19] MMSRetrieveStatus OPTIONAL,

retrieveStatusText  [20] UTF8String OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

replaceID           [26] UTF8String OPTIONAL,

contentType         [27] UTF8String OPTIONAL

}

MMSDeliveryAck ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

reportAllowed [3] BOOLEAN OPTIONAL,

status        [4] MMStatus,

direction     [5] MMSDirection

}

MMSForward ::= SEQUENCE

{

transactionID         [1]  UTF8String,

version               [2]  MMSVersion,

dateTime              [3]  Timestamp OPTIONAL,

originatingMMSParty   [4]  MMSParty,

terminatingMMSParty   [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients          [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients         [7]  SEQUENCE OF MMSParty OPTIONAL,

direction             [8]  MMSDirection,

expiry                [9]  MMSExpiry OPTIONAL,

desiredDeliveryTime   [10] Timestamp OPTIONAL,

deliveryReportAllowed [11] BOOLEAN OPTIONAL,

deliveryReport        [12] BOOLEAN OPTIONAL,

store                 [13] BOOLEAN OPTIONAL,

state                 [14] MMState OPTIONAL,

flags                 [15] MMFlags OPTIONAL,

contentLocationReq    [16] UTF8String,

replyCharging         [17] MMSReplyCharging OPTIONAL,

responseStatus        [18] MMSResponseStatus,

responseStatusText    [19] UTF8String  OPTIONAL,

messageID             [20] UTF8String OPTIONAL,

contentLocationConf   [21] UTF8String OPTIONAL,

storeStatus           [22] MMSStoreStatus OPTIONAL,

storeStatusText       [23] UTF8String OPTIONAL

}

MMSDeleteFromRelay ::= SEQUENCE

{

transactionID        [1] UTF8String,

version              [2] MMSVersion,

direction            [3] MMSDirection,

contentLocationReq   [4] SEQUENCE OF UTF8String,

contentLocationConf  [5] SEQUENCE OF UTF8String,

deleteResponseStatus [6] MMSDeleteResponseStatus,

deleteResponseText   [7] SEQUENCE OF UTF8String

}

MMSMBoxStore ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] UTF8String,

state               [5] MMState OPTIONAL,

flags               [6] MMFlags OPTIONAL,

contentLocationConf [7] UTF8String OPTIONAL,

storeStatus         [8] MMSStoreStatus,

storeStatusText     [9] UTF8String OPTIONAL

}

MMSMBoxUpload ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

direction           [3]  MMSDirection,

state               [4]  MMState OPTIONAL,

flags               [5]  MMFlags OPTIONAL,

contentType         [6]  UTF8String,

contentLocation     [7]  UTF8String OPTIONAL,

storeStatus         [8]  MMSStoreStatus,

storeStatusText     [9]  UTF8String OPTIONAL,

mMessages           [10] SEQUENCE OF MMBoxDescription

}

MMSMBoxDelete ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] SEQUENCE OF UTF8String,

contentLocationConf [5] SEQUENCE OF UTF8String OPTIONAL,

responseStatus      [6] MMSDeleteResponseStatus,

responseStatusText  [7] UTF8String OPTIONAL

}

MMSDeliveryReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

mMSDateTime         [4] Timestamp,

responseStatus      [5] MMSResponseStatus,

responseStatusText  [6] UTF8String OPTIONAL,

applicID            [7] UTF8String OPTIONAL,

replyApplicID       [8] UTF8String OPTIONAL,

auxApplicInfo       [9] UTF8String OPTIONAL

}

MMSDeliveryReportNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

mMSDateTime         [7]  Timestamp,

forwardToOriginator [8]  BOOLEAN OPTIONAL,

status              [9]  MMStatus,

statusExtension     [10] MMStatusExtension,

statusText          [11] MMStatusText,

applicID            [12] UTF8String OPTIONAL,

replyApplicID       [13] UTF8String OPTIONAL,

auxApplicInfo       [14] UTF8String OPTIONAL

}

MMSReadReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

mMSDateTime         [6] Timestamp,

readStatus          [7] MMSReadStatus,

applicID            [8] UTF8String OPTIONAL,

replyApplicID       [9] UTF8String OPTIONAL,

auxApplicInfo       [10] UTF8String OPTIONAL

}

MMSReadReportNonLocalTarget ::= SEQUENCE

{

version             [1] MMSVersion,

transactionID       [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

messageID           [6] UTF8String,

mMSDateTime         [7] Timestamp,

readStatus          [8] MMSReadStatus,

readStatusText      [9] MMSReadStatusText OPTIONAL,

applicID            [10] UTF8String OPTIONAL,

replyApplicID       [11] UTF8String OPTIONAL,

auxApplicInfo       [12] UTF8String OPTIONAL

}

MMSCancel ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

cancelID      [3] UTF8String,

direction     [4] MMSDirection

}

MMSMBoxViewRequest ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

totals          [9]  INTEGER OPTIONAL,

quotas          [10] MMSQuota OPTIONAL

}

MMSMBoxViewResponse ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

mMSTotals       [9]  BOOLEAN OPTIONAL,

mMSQuotas       [10] BOOLEAN OPTIONAL,

mMessages       [11] SEQUENCE OF MMBoxDescription

}

MMBoxDescription ::= SEQUENCE

{

contentLocation          [1]  UTF8String OPTIONAL,

messageID                [2]  UTF8String OPTIONAL,

state                    [3]  MMState OPTIONAL,

flags                    [4]  SEQUENCE OF MMFlags OPTIONAL,

dateTime                 [5]  Timestamp OPTIONAL,

originatingMMSParty      [6]  MMSParty OPTIONAL,

terminatingMMSParty      [7]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients             [8]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients            [9]  SEQUENCE OF MMSParty OPTIONAL,

messageClass             [10] MMSMessageClass OPTIONAL,

subject                  [11] MMSSubject OPTIONAL,

priority                 [12] MMSPriority OPTIONAL,

deliveryTime             [13] Timestamp OPTIONAL,

readReport               [14] BOOLEAN OPTIONAL,

messageSize              [15] INTEGER OPTIONAL,

replyCharging            [16] MMSReplyCharging OPTIONAL,

previouslySentBy         [17] MMSPreviouslySentBy OPTIONAL,

previouslySentByDateTime [18] Timestamp OPTIONAL,

contentType              [19] UTF8String OPTIONAL

}

-- =========

-- MMS CCPDU

-- =========

MMSCCPDU ::= SEQUENCE

{

version    [1] MMSVersion,

transactionID [2] UTF8String,

mMSContent    [3] OCTET STRING

}

-- ==============

-- MMS parameters

-- ==============

MMSAdaptation ::= SEQUENCE

{

allowed   [1] BOOLEAN,

overriden [2] BOOLEAN

}

MMSCancelStatus ::= ENUMERATED

{

cancelRequestSuccessfullyReceived(1),

cancelRequestCorrupted(2)

}

MMSContentClass ::= ENUMERATED

{

text(1),

imageBasic(2),

imageRich(3),

videoBasic(4),

videoRich(5),

megaPixel(6),

contentBasic(7),

contentRich(8)

}

MMSContentType ::= UTF8String

MMSDeleteResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSDirection ::= ENUMERATED

{

fromTarget(0),

toTarget(1)

}

MMSElementDescriptor ::= SEQUENCE

{

reference [1] UTF8String,

parameter [2] UTF8String     OPTIONAL,

value     [3] UTF8String     OPTIONAL

}

MMSExpiry ::= SEQUENCE

{

expiryPeriod [1] INTEGER,

periodFormat [2] MMSPeriodFormat

}

MMFlags ::= SEQUENCE

{

length     [1] INTEGER,

flag       [2] MMStateFlag,

flagString [3] UTF8String

}

MMSMessageClass ::= ENUMERATED

{

personal(1),

advertisement(2),

informational(3),

auto(4)

}

MMSParty ::= SEQUENCE

{

mMSPartyIDs [1] SEQUENCE OF MMSPartyID,

nonLocalID  [2] NonLocalID

}

MMSPartyID ::= CHOICE

{

e164Number   [1] E164Number,

emailAddress [2] EmailAddress,

iMSI         [3] IMSI,

iMPU         [4] IMPU,

iMPI         [5] IMPI,

sUPI         [6] SUPI,

gPSI         [7] GPSI

}

MMSPeriodFormat ::= ENUMERATED

{

absolute(1),

relative(2)

}

MMSPreviouslySent ::= SEQUENCE

{

previouslySentByParty [1] MMSParty,

sequenceNumber        [2] INTEGER,

previousSendDateTime  [3] Timestamp

}

MMSPreviouslySentBy ::= SEQUENCE OF MMSPreviouslySent

MMSPriority ::= ENUMERATED

{

low(1),

normal(2),

high(3)

}

MMSQuota ::= SEQUENCE

{

quota     [1] INTEGER,

quotaUnit [2] MMSQuotaUnit

}

MMSQuotaUnit ::= ENUMERATED

{

numMessages(1),

bytes(2)

}

MMSReadStatus ::= ENUMERATED

{

read(1),

deletedWithoutBeingRead(2)

}

MMSReadStatusText ::= UTF8String

MMSReplyCharging ::= ENUMERATED

{

requested(0),

requestedTextOnly(1),

accepted(2),

acceptedTextOnly(3)

}

MMSResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSRetrieveStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientMessageNotFound(3),

errorTransientNetworkProblem(4),

errorPermanentFailure(5),

errorPermanentServiceDenied(6),

errorPermanentMessageNotFound(7),

errorPermanentContentUnsupported(8)

}

MMSStoreStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientNetworkProblem(3),

errorPermanentFailure(4),

errorPermanentServiceDenied(5),

errorPermanentMessageFormatCorrupt(6),

errorPermanentMessageNotFound(7),

errorMMBoxFull(8)

}

MMState ::= ENUMERATED

{

draft(1),

sent(2),

new(3),

retrieved(4),

forwarded(5)

}

MMStateFlag ::= ENUMERATED

{

add(1),

remove(2),

filter(3)

}

MMStatus ::= ENUMERATED

{

expired(1),

retrieved(2),

rejected(3),

deferred(4),

unrecognized(5),

indeterminate(6),

forwarded(7),

unreachable(8)

}

MMStatusExtension ::= ENUMERATED

{

rejectionByMMSRecipient(0),

rejectionByOtherRS(1)

}

MMStatusText ::= UTF8String

MMSSubject ::= UTF8String

MMSVersion ::= SEQUENCE

{

majorVersion [1] INTEGER,

minorVersion [2] INTEGER

}

-- ==================

-- 5G PTC definitions

-- ==================

PTCRegistration  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

pTCRegistrationRequest        [3] PTCRegistrationRequest,

pTCRegistrationOutcome        [4] PTCRegistrationOutcome

}

PTCSessionInitiation  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL,

pTCHost                       [10] PTCTargetInformation OPTIONAL

}

PTCSessionAbandon  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

location                      [4] Location OPTIONAL,

pTCAbandonCause               [5] INTEGER

}

PTCSessionStart  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCHost                       [9] PTCTargetInformation OPTIONAL,

pTCBearerCapability           [10] UTF8String OPTIONAL

}

PTCSessionEnd  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCParticipants               [5] SEQUENCE OF PTCTargetInformation OPTIONAL,

location                      [6] Location OPTIONAL,

pTCSessionEndCause            [7] PTCSessionEndCause

}

PTCStartOfInterception  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

preEstSessionID               [3] PTCSessionInfo OPTIONAL,

pTCOriginatingID              [4] PTCTargetInformation,

pTCSessionInfo                [5] PTCSessionInfo OPTIONAL,

pTCHost                       [6] PTCTargetInformation OPTIONAL,

pTCParticipants               [7] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCMediaStreamAvail           [8] BOOLEAN OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL

}

PTCPreEstablishedSession  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

rTPSetting                    [3] RTPSetting,

pTCMediaCapability            [4] UTF8String,

pTCPreEstSessionID            [5] PTCSessionInfo,

pTCPreEstStatus               [6] PTCPreEstStatus,

pTCMediaStreamAvail           [7] BOOLEAN OPTIONAL,

location                      [8] Location OPTIONAL,

pTCFailureCode                [9] PTCFailureCode OPTIONAL

}

PTCInstantPersonalAlert  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCIPAPartyID                 [2] PTCTargetInformation,

pTCIPADirection               [3] Direction

}

PTCPartyJoin  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [5] MultipleParticipantPresenceStatus OPTIONAL,

pTCMediaStreamAvail           [6] BOOLEAN OPTIONAL,

pTCBearerCapability           [7] UTF8String OPTIONAL

}

PTCPartyDrop  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCPartyDrop                  [4] PTCTargetInformation,

pTCParticipantPresenceStatus  [5] PTCParticipantPresenceStatus OPTIONAL

}

PTCPartyHold  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCHoldID                     [5] SEQUENCE OF PTCTargetInformation,

pTCHoldRetrieveInd            [6] BOOLEAN

}

PTCMediaModification  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCMediaStreamAvail           [4] BOOLEAN OPTIONAL,

pTCBearerCapability           [5] UTF8String

}

PTCGroupAdvertisement  ::=SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCIDList                     [3] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCGroupAuthRule              [4] PTCGroupAuthRule OPTIONAL,

pTCGroupAdSender              [5] PTCTargetInformation,

pTCGroupNickname              [6] UTF8String OPTIONAL

}

PTCFloorControl  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessioninfo                [3] PTCSessionInfo,

pTCFloorActivity              [4] SEQUENCE OF PTCFloorActivity,

pTCFloorSpeakerID             [5] PTCTargetInformation OPTIONAL,

pTCMaxTBTime                  [6] INTEGER OPTIONAL,

pTCQueuedFloorControl         [7] BOOLEAN OPTIONAL,

pTCQueuedPosition             [8] INTEGER OPTIONAL,

pTCTalkBurstPriority          [9] PTCTBPriorityLevel OPTIONAL,

pTCTalkBurstReason            [10] PTCTBReasonCode OPTIONAL

}

PTCTargetPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCTargetPresenceStatus       [2] PTCParticipantPresenceStatus

}

PTCParticipantPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCParticipantPresenceStatus  [2] PTCParticipantPresenceStatus

}

PTCListManagement  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCListManagementType         [3] PTCListManagementType OPTIONAL,

pTCListManagementAction       [4] PTCListManagementAction OPTIONAL,

pTCListManagementFailure      [5] PTCListManagementFailure OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCIDList                     [7] SEQUENCE OF PTCIDList OPTIONAL,

pTCHost                       [8] PTCTargetInformation OPTIONAL

}

PTCAccessPolicy  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCAccessPolicyType           [3] PTCAccessPolicyType OPTIONAL,

pTCUserAccessPolicy           [4] PTCUserAccessPolicy OPTIONAL,

pTCGroupAuthRule              [5] PTCGroupAuthRule OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCAccessPolicyFailure        [7] PTCAccessPolicyFailure OPTIONAL

}

-- =========

-- PTC CCPDU

-- =========

PTCCCPDU ::= OCTET STRING

-- =================

-- 5G PTC parameters

-- =================

PTCRegistrationRequest  ::= ENUMERATED

{

register(1),

reRegister(2),

deRegister(3)

}

PTCRegistrationOutcome  ::= ENUMERATED

{

success(1),

failure(2)

}

PTCSessionEndCause  ::= ENUMERATED

{

initiaterLeavesSession(1),

definedParticipantLeaves(2),

numberOfParticipants(3),

sessionTimerExpired(4),

pTCSpeechInactive(5),

allMediaTypesInactive(6)

}

PTCTargetInformation  ::= SEQUENCE

{

identifiers                [1] SEQUENCE SIZE(1..MAX) OF PTCIdentifiers

}

PTCIdentifiers  ::= CHOICE

{

mCPTTID                    [1] UTF8String,

instanceIdentifierURN      [2] UTF8String,

pTCChatGroupID             [3] PTCChatGroupID,

iMPU                       [4] IMPU,

iMPI                       [5] IMPI

}

PTCSessionInfo  ::= SEQUENCE

{

pTCSessionURI              [1] UTF8String,

pTCSessionType             [2] PTCSessionType

}

PTCSessionType  ::= ENUMERATED

{

ondemand(1),

preEstablished(2),

adhoc(3),

prearranged(4),

groupSession(5)

}

MultipleParticipantPresenceStatus  ::= SEQUENCE OF PTCParticipantPresenceStatus

PTCParticipantPresenceStatus  ::= SEQUENCE

{

presenceID                 [1] PTCTargetInformation,

presenceType               [2] PTCPresenceType,

presenceStatus             [3] BOOLEAN

}

PTCPresenceType  ::= ENUMERATED

{

pTCClient(1),

pTCGroup(2)

}

PTCPreEstStatus  ::= ENUMERATED

{

established(1),

modified(2),

released(3)

}

RTPSetting  ::= SEQUENCE

{

iPAddress                  [1] IPAddress,

portNumber                 [2] PortNumber

}

PTCIDList  ::= SEQUENCE

{

pTCPartyID                 [1] PTCTargetInformation,

pTCChatGroupID             [2] PTCChatGroupID

}

PTCChatGroupID  ::= SEQUENCE

{

groupIdentity              [1] UTF8String

}

PTCFloorActivity  ::= ENUMERATED

{

tBCPRequest(1),

tBCPGranted(2),

tBCPDeny(3),

tBCPIdle(4),

tBCPTaken(5),

tBCPRevoke(6),

tBCPQueued(7),

tBCPRelease(8)

}

PTCTBPriorityLevel  ::= ENUMERATED

{

preEmptive(1),

highPriority(2),

normalPriority(3),

listenOnly(4)

}

PTCTBReasonCode  ::= ENUMERATED

{

noQueuingAllowed(1),

oneParticipantSession(2),

listenOnly(3),

exceededMaxDuration(4),

tBPrevented(5)

}

PTCListManagementType  ::= ENUMERATED

{

contactListManagementAttempt(1),

groupListManagementAttempt(2),

contactListManagementResult(3),

groupListManagementResult(4),

requestUnsuccessful(5)

}

PTCListManagementAction  ::= ENUMERATED

{

create(1),

modify(2),

retrieve(3),

delete(4),

notify(5)

}

PTCAccessPolicyType  ::= ENUMERATED

{

pTCUserAccessPolicyAttempt(1),

groupAuthorizationRulesAttempt(2),

pTCUserAccessPolicyQuery(3),

groupAuthorizationRulesQuery(4),

pTCUserAccessPolicyResult(5),

groupAuthorizationRulesResult(6),

requestUnsuccessful(7)

}

PTCUserAccessPolicy  ::= ENUMERATED

{

allowIncomingPTCSessionRequest(1),

blockIncomingPTCSessionRequest(2),

allowAutoAnswerMode(3),

allowOverrideManualAnswerMode(4)

}

PTCGroupAuthRule  ::= ENUMERATED

{

allowInitiatingPTCSession(1),

blockInitiatingPTCSession(2),

allowJoiningPTCSession(3),

blockJoiningPTCSession(4),

allowAddParticipants(5),

blockAddParticipants(6),

allowSubscriptionPTCSessionState(7),

blockSubscriptionPTCSessionState(8),

allowAnonymity(9),

forbidAnonymity(10)

}

PTCFailureCode  ::= ENUMERATED

{

sessionCannotBeEstablished(1),

sessionCannotBeModified(2)

}

PTCListManagementFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

PTCAccessPolicyFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

-- ===============

-- IMS definitions

-- ===============

-- See clause 7.12.4.2.1 for details of this structure

IMSMessage ::= SEQUENCE

{

payload               [1] IMSPayload,

sessionDirection      [2] SessionDirection,

voIPRoamingIndication [3] VoIPRoamingIndication OPTIONAL,

location              [6] Location OPTIONAL

}

-- See clause 7.12.4.2.2 for details of this structure

StartOfInterceptionForActiveIMSSession ::= SEQUENCE

{

originatingId         [1] SEQUENCE OF IMPU,

terminatingId         [2] IMPU,

sDPState              [3] SEQUENCE OF OCTET STRING OPTIONAL,

diversionIdentity     [4] IMPU OPTIONAL,

voIPRoamingIndication [5] VoIPRoamingIndication OPTIONAL,

location              [7] Location OPTIONAL

}

-- See clause 7.12.4.2.3 for the details.

IMSCCUnavailable ::= SEQUENCE

{

cCUnavailableReason   [1] UTF8String,

sDPState              [2] OCTET STRING OPTIONAL

}

-- =========

-- IMS CCPDU

-- =========

IMSCCPDU ::= SEQUENCE

{

payload [1] IMSCCPDUPayload,

sDPInfo [2] OCTET STRING OPTIONAL

}

IMSCCPDUPayload ::= OCTET STRING

-- ==============

-- IMS parameters

-- ==============

IMSPayload ::= CHOICE

{

encapsulatedSIPMessage            [1] SIPMessage

}

SIPMessage ::= SEQUENCE

{

iPSourceAddress       [1] IPAddress,

iPDestinationAddress  [2] IPAddress,

sIPContent            [3] OCTET STRING

}

VoIPRoamingIndication ::= ENUMERATED

{

roamingLBO(1),

roamingS8HR(2),

roamingN9HR(3)

}

SessionDirection ::= ENUMERATED

{

fromTarget(1),

toTarget(2),

combined(3),

indeterminate(4)

}

HeaderOnlyIndication ::= BOOLEAN

-- =================================

-- STIR/SHAKEN/RCD/eCNAM definitions

-- =================================

-- See clause 7.11.2.1.2 for details of this structure

STIRSHAKENSignatureGeneration ::= SEQUENCE

{

pASSporTs                 [1] SEQUENCE OF PASSporT,

encapsulatedSIPMessage    [2] SIPMessage OPTIONAL

}

-- See clause 7.11.2.1.3 for details of this structure

STIRSHAKENSignatureValidation ::= SEQUENCE

{

pASSporTs                 [1] SEQUENCE OF PASSporT OPTIONAL,

rCDTerminalDisplayInfo    [2] RCDDisplayInfo OPTIONAL,

eCNAMTerminalDisplayInfo  [3] ECNAMDisplayInfo OPTIONAL,

sHAKENValidationResult    [4] SHAKENValidationResult,

sHAKENFailureStatusCode   [5] SHAKENFailureStatusCode OPTIONAL,

encapsulatedSIPMessage    [6] SIPMessage OPTIONAL

}

-- ================================

-- STIR/SHAKEN/RCD/eCNAM parameters

-- ================================

PASSporT ::= SEQUENCE

{

pASSporTHeader    [1] PASSporTHeader,

pASSporTPayload   [2] PASSporTPayload,

pASSporTSignature [3] OCTET STRING

}

PASSporTHeader ::= SEQUENCE

{

type          [1] JWSTokenType,

algorithm     [2] UTF8String,

ppt           [3] UTF8String OPTIONAL,

x5u           [4] UTF8String

}

JWSTokenType ::= ENUMERATED

{

passport(1)

}

PASSporTPayload ::= SEQUENCE

{

issuedAtTime    [1] GeneralizedTime,

originator      [2] STIRSHAKENOriginator,

destination     [3] STIRSHAKENDestinations,

attestation     [4] Attestation,

origId          [5] UTF8String,

diversion       [6] STIRSHAKENDestination

}

STIRSHAKENOriginator ::= CHOICE

{

telephoneNumber [1] STIRSHAKENTN,

sTIRSHAKENURI   [2] UTF8String

}

STIRSHAKENDestinations ::= SEQUENCE OF STIRSHAKENDestination

STIRSHAKENDestination ::= CHOICE

{

telephoneNumber [1] STIRSHAKENTN,

sTIRSHAKENURI   [2] UTF8String

}

STIRSHAKENTN ::= CHOICE

{

mSISDN [1] MSISDN

}

Attestation ::= ENUMERATED

{

attestationA(1),

attestationB(2),

attestationC(3)

}

SHAKENValidationResult ::= ENUMERATED

{

tNValidationPassed(1),

tNValidationFailed(2),

noTNValidation(3)

}

SHAKENFailureStatusCode ::= INTEGER

ECNAMDisplayInfo ::= SEQUENCE

{

name           [1] UTF8String,

additionalInfo [2] OCTET STRING OPTIONAL

}

RCDDisplayInfo ::= SEQUENCE

{

name [1] UTF8String,

jcd  [2] OCTET STRING OPTIONAL,

jcl  [3] OCTET STRING OPTIONAL

}

-- =================

-- EES definitions

-- =================

-- See clause 7.X.2.2 for details of this structure

EESEECRegistration ::= SEQUENCE

{

registrationType         [1] RegistrationType,

eECID                    [2] UTF8String,

gPSI                     [3] GPSI OPTIONAL,

aCProfiles               [4] ACProfiles OPTIONAL,

eECServiceContSupport    [5] ACRScenarios OPTIONAL,

expirationTime           [6] Timestamp OPTIONAL,

eECContextID             [7] UTF8String OPTIONAL,

srcEESID                 [8] UTF8String OPTIONAL,

unfulfilledACProfiles    [9] UnfulfilledACProfiles OPTIONAL,

failureResponse          [10] FailureResponse OPTIONAL

}

-- See clause 7.X.2.3 for details of this structure

EESEASDiscovery ::= SEQUENCE

{

eECID                    [1] UTF8String,

gPSI                     [2] GPSI OPTIONAL,

eASDiscoveryFilter       [3] EASDiscoveryFilter OPTIONAL,

eECServiceContSupport    [4] ACRScenarios OPTIONAL,

uELocation               [5] Location OPTIONAL,

eASTargetDNAIs           [6] DNAIs OPTIONAL,

discoveredEAS            [7] DiscoveredEAS OPTIONAL,

failureResponse          [8] FailureResponse OPTIONAL

}

-- See clause 7.X.2.4 for details of this structure

EESEASDiscoverySubscription ::= SEQUENCE

{

eECID                    [1] UTF8String,

gPSI                     [2] GPSI OPTIONAL,

subscriptionType         [3] SubscriptionType,

eASEventType             [4] EASEventType,

eASDiscoveryFilter       [5] EASDiscoveryFilter OPTIONAL,

eASDynamicInfoFilter     [6] EASDynamicInfoFilter OPTIONAL,

eECServiceContSupport    [7] ACRScenarios OPTIONAL,

expirationTime           [8] Timestamp OPTIONAL,

subscriptionId           [9] UTF8String OPTIONAL,

failureResponse          [10] FailureResponse OPTIONAL

}

-- See clause 7.X.2.5 for details of this structure

EESEASDiscoveryNotification ::= SEQUENCE

{

subscriptionID     [1] UTF8String,

eventType          [2] EASEventType,

discoveredEAS      [3] DiscoveredEAS,

failureResponse    [4] FailureResponse OPTIONAL

}

-- See clause 7.X.2.6 for details of this structure

EESAppContextRelocation ::= SEQUENCE

{

eECID              [1] UTF8String,

gPSI               [2] GPSI OPTIONAL,

eESACRDetOrInit    [3] EESACRDetOrInit

}

EESACRDetOrInit ::= CHOICE

{

aCRDetermineReq    [1] ACRDetermineReq,

aCRInitiateReq     [2] ACRInitiateReq

}

ACRDetermineReq ::= SEQUENCE

{

eASID           [1] EASID OPTIONAL,

aCID            [2] ACID OPTIONAL,

sEASEndpoint    [3] EASEndpoint

}

ACRInitiateReq ::= SEQUENCE

{

eASID                   [1] EASID OPTIONAL,

aCID                    [2] ACID OPTIONAL,

tEASEndpoint            [3] EASEndpoint,

sEASEndpoint            [4] EASEndpoint OPTIONAL,

previousTEASEndpoint    [5] EASEndpoint OPTIONAL,

routeReq                [6] RouteToLocation OPTIONAL

}

-- See clause 7.X.2.7 for details of this structure

EESACRSubscription ::= SEQUENCE

{

eECID               [1] UTF8String,

gPSI                [2] GPSI OPTIONAL,

subscriptionType    [3] SubscriptionType,

expirationTime      [4] Timestamp OPTIONAL,

eASIDs              [5] EASIDs,

aCIDs               [6] ACIDs OPTIONAL,

eventIDs            [7] ACREventIDs OPTIONAL,

subscriptionId      [8] UTF8String OPTIONAL,

failureResponse     [9] FailureResponse OPTIONAL

}

-- See clause 7.X.2.8 for details of this structure

EESACRNotification ::= SEQUENCE

{

subscriptionID    [1] UTF8String,

eASID             [2] EASID,

eventID           [3] ACREventIDs,

targetInfo        [4] TargetInfo OPTIONAL,

aCRRes            [5] BOOLEAN OPTIONAL,

failReason        [6] UTF8String OPTIONAL

}

-- See clause 7.X.2.9 for details of this structure

EESEECContextRelocation ::= SEQUENCE

{

eECID           [1] UTF8String,

eECContextID    [2] UTF8String,

gPSI            [3] GPSI OPTIONAL,

uELoc           [4] Location OPTIONAL,

aCProfiles      [5] ACProfiles OPTIONAL

}

-- See clause 7.X.2.10 for details of this structure

EESStartOfInterceptionWithRegisteredEEC ::= SEQUENCE

{

eECID                    [1] UTF8String,

gPSI                     [2] GPSI OPTIONAL,

aCProfiles               [3] ACProfiles OPTIONAL,

eECServiceContSupport    [4] ACRScenarios OPTIONAL,

expirationTime           [5] Timestamp OPTIONAL,

eECContextID             [6] UTF8String OPTIONAL,

srcEESID                 [7] UTF8String OPTIONAL,

unfulfilledACProfiles    [8] UnfulfilledACProfiles OPTIONAL,

timeOfRegistration       [9] Timestamp OPTIONAL

}

-- ==============

-- EES parameters

-- ==============

RegistrationType ::= ENUMERATED

{

registration (1),

registrationUpdate(2),

deregistration(3)

}

ACProfiles ::= SET OF ACProfile

ACProfile ::= SEQUENCE

{

aCID                  [1] ACID,

aCType                [2] UTF8String OPTIONAL,

aCSchedule            [3] Daytime OPTIONAL,

expACGeoServArea      [4] Location OPTIONAL,

eASsInfo              [5] EASsInfo OPTIONAL,

aCServiceContSupport  [6] ACRScenarios OPTIONAL

}

ACID ::= UTF8String

ACRScenarios ::= SET OF ACRScenario

ACRScenario ::= ENUMERATED

{

eECInitiated(1),

eECExecutedViaSourceEES(2),

eECExecutedViaTargetEES(3),

sourceEASDecided(4),

sourceEESExecuted(5),

eELManagedACR(6)

}

UnfulfilledACProfiles ::= SET OF UnfulfilledACProfile

UnfulfilledACProfile ::= SEQUENCE

{

aCID      [1] ACID,

reason    [2] UnfulfilledACProfileReason

}

UnfulfilledACProfileReason ::= ENUMERATED

{

eASNotAvailable(1),

requirementsUnfulfilled(2)

}

EASID ::= UTF8String

EASsInfo ::= SET OF EASInfo

EASInfo ::= SEQUENCE

{

eASID                 [1] EASID,

expectedSvcKPIs       [2] ServiceKPIs OPTIONAL,

minimumReqSvcKPIs     [3] ServiceKPIs OPTIONAL

}

ServiceKPIs ::= SEQUENCE

{

connectionBandwidth      [1] INTEGER OPTIONAL,

requestRate              [2] INTEGER OPTIONAL,

responseTime             [3] INTEGER OPTIONAL,

requestedAvailability    [4] INTEGER OPTIONAL,

requestedCompute         [5] OCTET STRING OPTIONAL,

requestedGraphCompute    [6] OCTET STRING OPTIONAL,

requestedMemory          [7] OCTET STRING OPTIONAL,

requestedStorage         [8] OCTET STRING OPTIONAL

}

FailureResponse ::= ENUMERATED

{

error400(1),

error401(2),

error403(3),

error404(4),

error406(5),

error411(6),

error413(7),

error415(8),

error429(9),

error500(10),

error503(11)

}

EASDiscoveryFilter ::= CHOICE

{

aCsCharacteristics     [1] ACProfiles,

eASCharacteristics     [2] EASsCharacteristics

}

EASsCharacteristics ::= SET OF EASCharacteristics

EASCharacteristics ::= SEQUENCE

{

eASID                  [1] EASID OPTIONAL,

aSPID                  [2] UTF8String OPTIONAL,

eASType                [3] UTF8String OPTIONAL,

eASSchedule            [4] Daytime OPTIONAL,

eASProfile             [5] EASProfile OPTIONAL,

eASServiceArea         [6] Location OPTIONAL,

eASServicePermLevel    [7] UTF8String OPTIONAL,

eASServiceFeatures     [8] EASServiceFeatures OPTIONAL

}

DNAIs ::= SET OF DNAI

DiscoveredEAS ::= SEQUENCE

{

eASProfile    [1] EASProfile,

lifetime      [2] INTEGER OPTIONAL

}

EASProfile ::= SEQUENCE

{

eASID                  [1] EASID,

eASEndpoint            [2] EASEndpoint,

aCIDs                  [3] ACIDs OPTIONAL,

aSPID                  [4] UTF8String OPTIONAL,

eASType                [5] UTF8String OPTIONAL,

eASDescription         [6] UTF8String OPTIONAL,

eASSchedule            [7] Daytime OPTIONAL,

eASServiceArea         [8] Location OPTIONAL,

eASServiceKPIs         [9] ServiceKPIs OPTIONAL,

eASServicePermLevel    [10] UTF8String OPTIONAL,

eASServiceFeatures     [11] EASServiceFeatures OPTIONAL,

eASServiceContSupport  [12] ACRScenarios OPTIONAL,

appLocs                [13] RouteToLocations OPTIONAL,

eASStatus              [14] EASStatus OPTIONAL

}

EASStatus ::= ENUMERATED

{

enabled(1),

disabled(2)

}

EASEndpoint ::= SEQUENCE

{

fQDN             [1] FQDN OPTIONAL,

iPv4Addresses    [2] IPv4Addresses OPTIONAL,

iPv6Addresses    [3] IPv6Addresses OPTIONAL,

uRI              [4] UTF8String OPTIONAL

}

RouteToLocations ::= SET OF RouteToLocation

EASServiceFeatures ::= SET OF EASServiceFeature

EASServiceFeature ::= UTF8String

ACIDs ::= SET OF ACID

IPv4Addresses ::= SET OF IPv4Address

IPv6Addresses ::= SET OF IPv6Address

SubscriptionType ::= ENUMERATED

{

subscription(1),

subscriptionUpdate(2),

unsubscription(3)

}

EASEventType ::= ENUMERATED

{

eASAvailabilityChange(1),

eASDynamicInfoChange(2)

}

EASDynamicInfoFilter ::= SEQUENCE

{

eASId          [1] EASID,

eASStatus      [2] BOOLEAN,

eASAcIDs       [3] BOOLEAN,

eASDesc        [4] BOOLEAN,

eASPt          [5] BOOLEAN,

eASFeature     [6] BOOLEAN,

eASSchedule    [7] BOOLEAN,

eASSvcArea     [8] BOOLEAN,

eASSvcKpi      [9] BOOLEAN,

eASSvcCont     [10] BOOLEAN

}

EASIDs ::= SET OF EASID

ACREventIDs ::= ENUMERATED

{

targetInformation(1),

aCRComplete(2)

}

TargetInfo ::= SEQUENCE

{

discoveredEAS    [1] DiscoveredEAS,

targetEESInfo    [2] EDNConfigurationInfo OPTIONAL

}

EDNConfigurationInfo ::= SEQUENCE

{

eDNConnectionInfo    [1] EDNConnectionInfo,

eESsInfo             [2] EESsInfo,

lifetime             [3] INTEGER OPTIONAL

}

EDNConnectionInfo ::= SEQUENCE

{

dNN            [1] DNN OPTIONAL,

sNSSAI         [2] SNSSAI OPTIONAL,

serviceArea    [3] Location OPTIONAL

}

EESsInfo ::= SET OF EESInfo

EESInfo ::= SEQUENCE

{

eESID          [1] EESID,

eESEndpoint    [2] EESEndpoint,

eASIDs         [3] EASIDs OPTIONAL,

serviceArea    [4] Location OPTIONAL,

dNAIS          [5] DNAIs OPTIONAL

}

EESID ::= UTF8String

EESEndpoint ::= SEQUENCE

{

fQDN             [1] FQDN OPTIONAL,

iPv4Addresses    [2] IPv4Addresses OPTIONAL,

iPv6Addresses    [3] IPv6Addresses OPTIONAL,

uRI              [4] UTF8String OPTIONAL

}

-- ===================

-- 5G LALS definitions

-- ===================

LALSReport ::= SEQUENCE

{

sUPI                [1] SUPI OPTIONAL,

--  pEI                 [2] PEI OPTIONAL, deprecated in Release-16, do not re-use this tag number

gPSI                [3] GPSI OPTIONAL,

location            [4] Location OPTIONAL,

iMPU                [5] IMPU OPTIONAL,

iMSI                [7] IMSI OPTIONAL,

mSISDN              [8] MSISDN OPTIONAL

}

-- =====================

-- PDHR/PDSR definitions

-- =====================

PDHeaderReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

packetSize                  [9] INTEGER

}

PDSummaryReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

pDSRSummaryTrigger          [9] PDSRSummaryTrigger,

firstPacketTimestamp        [10] Timestamp,

lastPacketTimestamp         [11] Timestamp,

packetCount                 [12] INTEGER,

byteCount                   [13] INTEGER

}

-- ====================

-- PDHR/PDSR parameters

-- ====================

PDSRSummaryTrigger ::= ENUMERATED

{

timerExpiry(1),

packetCount(2),

byteCount(3),

startOfFlow(4),

endOfFlow(5)

}

-- ==================================

-- Identifier Association definitions

-- ==================================

AMFIdentifierAssociation ::= SEQUENCE

{

sUPI             [1] SUPI,

sUCI             [2] SUCI OPTIONAL,

pEI              [3] PEI OPTIONAL,

gPSI             [4] GPSI OPTIONAL,

gUTI             [5] FiveGGUTI,

location         [6] Location,

fiveGSTAIList    [7] TAIList OPTIONAL

}

MMEIdentifierAssociation ::= SEQUENCE

{

iMSI        [1] IMSI,

iMEI        [2] IMEI OPTIONAL,

mSISDN      [3] MSISDN OPTIONAL,

gUTI        [4] GUTI,

location    [5] Location,

tAIList     [6] TAIList OPTIONAL

}

-- =================================

-- Identifier Association parameters

-- =================================

MMEGroupID ::= OCTET STRING (SIZE(2))

MMECode ::= OCTET STRING (SIZE(1))

TMSI ::= OCTET STRING (SIZE(4))

-- ===================

-- EPS MME definitions

-- ===================

MMEAttach ::= SEQUENCE

{

attachType       [1] EPSAttachType,

attachResult     [2] EPSAttachResult,

iMSI             [3] IMSI,

iMEI             [4] IMEI OPTIONAL,

mSISDN           [5] MSISDN OPTIONAL,

gUTI             [6] GUTI OPTIONAL,

location         [7] Location OPTIONAL,

ePSTAIList       [8] TAIList OPTIONAL,

sMSServiceStatus [9] EPSSMSServiceStatus OPTIONAL,

oldGUTI          [10] GUTI OPTIONAL,

eMM5GRegStatus   [11] EMM5GMMStatus OPTIONAL

}

MMEDetach ::= SEQUENCE

{

detachDirection    [1] MMEDirection,

detachType         [2] EPSDetachType,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

cause              [7] EMMCause OPTIONAL,

location           [8] Location OPTIONAL,

switchOffIndicator [9] SwitchOffIndicator OPTIONAL

}

MMELocationUpdate ::= SEQUENCE

{

iMSI             [1] IMSI,

iMEI             [2] IMEI OPTIONAL,

mSISDN           [3] MSISDN OPTIONAL,

gUTI             [4] GUTI OPTIONAL,

location         [5] Location OPTIONAL,

oldGUTI          [6] GUTI OPTIONAL,

sMSServiceStatus [7] EPSSMSServiceStatus OPTIONAL

}

MMEStartOfInterceptionWithEPSAttachedUE ::= SEQUENCE

{

attachType         [1] EPSAttachType,

attachResult       [2] EPSAttachResult,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

location           [7] Location OPTIONAL,

ePSTAIList         [9] TAIList OPTIONAL,

sMSServiceStatus   [10] EPSSMSServiceStatus OPTIONAL,

eMM5GRegStatus     [12] EMM5GMMStatus OPTIONAL

}

MMEUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType [1] MMEFailedProcedureType,

failureCause        [2] MMEFailureCause,

iMSI                [3] IMSI OPTIONAL,

iMEI                [4] IMEI OPTIONAL,

mSISDN              [5] MSISDN OPTIONAL,

gUTI                [6] GUTI OPTIONAL,

location            [7] Location OPTIONAL

}

-- See clause 6.3.2.2.8 for details of this structure

MMEPositioningInfoTransfer ::= SEQUENCE

{

iMSI                [1] IMSI,

iMEI                [2] IMEI OPTIONAL,

mSISDN              [3] MSISDN OPTIONAL,

gUTI                [4] GUTI OPTIONAL,

lPPaMessage         [5] OCTET STRING OPTIONAL,

lPPMessage          [6] OCTET STRING OPTIONAL,

mMELCSCorrelationId [7] OCTET STRING (SIZE(4))

}

-- ==================

-- EPS MME parameters

-- ==================

EMMCause ::= INTEGER (0..255)

ESMCause ::= INTEGER (0..255)

EPSAttachType ::= ENUMERATED

{

ePSAttach(1),

combinedEPSIMSIAttach(2),

ePSRLOSAttach(3),

ePSEmergencyAttach(4),

reserved(5)

}

EPSAttachResult ::= ENUMERATED

{

ePSOnly(1),

combinedEPSIMSI(2)

}

EPSDetachType ::= ENUMERATED

{

ePSDetach(1),

iMSIDetach(2),

combinedEPSIMSIDetach(3),

reAttachRequired(4),

reAttachNotRequired(5),

reserved(6)

}

EPSSMSServiceStatus ::= ENUMERATED

{

sMSServicesNotAvailable(1),

sMSServicesNotAvailableInThisPLMN(2),

networkFailure(3),

congestion(4)

}

MMEDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

MMEFailedProcedureType ::= ENUMERATED

{

attachReject(1),

authenticationReject(2),

securityModeReject(3),

serviceReject(4),

trackingAreaUpdateReject(5),

activateDedicatedEPSBearerContextReject(6),

activateDefaultEPSBearerContextReject(7),

bearerResourceAllocationReject(8),

bearerResourceModificationReject(9),

modifyEPSBearerContectReject(10),

pDNConnectivityReject(11),

pDNDisconnectReject(12)

}

MMEFailureCause ::= CHOICE

{

eMMCause [1] EMMCause,

eSMCause [2] ESMCause

}

-- ===========================

-- LI Notification definitions

-- ===========================

LINotification ::= SEQUENCE

{

notificationType                    [1] LINotificationType,

appliedTargetID                     [2] TargetIdentifier OPTIONAL,

appliedDeliveryInformation          [3] SEQUENCE OF LIAppliedDeliveryInformation OPTIONAL,

appliedStartTime                    [4] Timestamp OPTIONAL,

appliedEndTime                      [5] Timestamp OPTIONAL

}

-- ==========================

-- LI Notification parameters

-- ==========================

LINotificationType ::= ENUMERATED

{

activation(1),

deactivation(2),

modification(3)

}

LIAppliedDeliveryInformation ::= SEQUENCE

{

hI2DeliveryIPAddress                [1] IPAddress OPTIONAL,

hI2DeliveryPortNumber               [2] PortNumber OPTIONAL,

hI3DeliveryIPAddress                [3] IPAddress OPTIONAL,

hI3DeliveryPortNumber               [4] PortNumber OPTIONAL

}

-- ===============

-- MDF definitions

-- ===============

MDFCellSiteReport ::= SEQUENCE OF CellInformation

-- ==============================

-- 5G EPS Interworking Parameters

-- ==============================

EMM5GMMStatus ::= SEQUENCE

{

eMMRegStatus  [1] EMMRegStatus OPTIONAL,

fiveGMMStatus [2] FiveGMMStatus OPTIONAL

}

EPS5GGUTI ::= CHOICE

{

gUTI      [1] GUTI,

fiveGGUTI [2] FiveGGUTI

}

EMMRegStatus ::= ENUMERATED

{

uEEMMRegistered(1),

uENotEMMRegistered(2)

}

FiveGMMStatus ::= ENUMERATED

{

uE5GMMRegistered(1),

uENot5GMMRegistered(2)

}

-- ========================================

-- Separated Location Reporting definitions

-- ========================================

SeparatedLocationReporting ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location,

non3GPPAccessEndpoint       [7] UEEndpointAddress OPTIONAL,

rATType                     [8] RATType OPTIONAL

}

-- =================

-- Common Parameters

-- =================

AccessType ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPandNonThreeGPPAccess(3)

}

AllowedNSSAI ::= SEQUENCE OF NSSAI

AllowedTACs ::= SEQUENCE (SIZE(1..MAX)) OF TAC

AreaOfInterest ::= SEQUENCE

{

areaOfInterestTAIList     [1] AreaOfInterestTAIList OPTIONAL,

areaOfInterestCellList    [2] AreaOfInterestCellList OPTIONAL,

areaOfInterestRANNodeList [3] AreaOfInterestRANNodeList OPTIONAL

}

AreaOfInterestCellList ::= SEQUENCE (SIZE(1..MAX)) OF NCGI

AreaOfInterestItem ::= SEQUENCE

{

areaOfInterest  [1] AreaOfInterest

}

AreaOfInterestRANNodeList ::= SEQUENCE (SIZE(1..MAX)) OF GlobalRANNodeID

AreaOfInterestTAIList ::= SEQUENCE (SIZE(1..MAX)) OF TAI

CellCAGList ::= SEQUENCE (SIZE(1..MAX)) OF CAGID

CauseMisc ::= ENUMERATED

{

controlProcessingOverload(1),

notEnoughUserPlaneProcessingResources(2),

hardwareFailure(3),

oMIntervention(4),

unknownPLMNOrSNPN(5),

unspecified(6)

}

CauseNas ::= ENUMERATED

{

normalRelease(1),

authenticationFailure(2),

deregister(3),

unspecified(4)

}

CauseProtocol ::= ENUMERATED

{

transferSyntaxError(1),

abstractSyntaxError-reject(2),

abstractSyntaxErrorIgnoreAndNotify(3),

messageNotCompatibleWithReceiverState(4),

semanticError(5),

abstractSyntaxErrorFalselyConstructedMessage(6),

unspecified(7)

}

CauseRadioNetwork ::= ENUMERATED

{

unspecified(1),

txnrelocoverallExpiry(2),

successfulHandover(3),

releaseDueToNGRANGeneratedReason(4),

releaseDueTo5gcGeneratedReason(5),

handoverCancelled(6),

partialHandover(7),

hoFailureInTarget5GCNGRANNodeOrTargetSystem(8),

hoTargetNotAllowed(9),

tNGRelocOverallExpiry(10),

tNGRelocPrepExpiry(11),

cellNotAvailable(12),

unknownTargetID(13),

noRadioResourcesAvailableInTargetCell(14),

unknownLocalUENGAPID(15),

inconsistentRemoteUENGAPID(16),

handoverDesirableForRadioReason(17),

timeCriticalHandover(18),

resourceOptimisationHandover(19),

reduceLoadInServingCell(20),

userInactivity(21),

radioConnectionWithUELost(22),

radioResourcesNotAvailable(23),

invalidQoSCombination(24),

failureInRadioInterfaceProcedure(25),

interactionWithOtherProcedure(26),

unknownPDUSessionID(27),

multiplePDUSessionIDInstances(29),

multipleQoSFlowIDInstances(30),

encryptionAndOrIntegrityProtectionAlgorithmsNotSupported(31),

nGIntraSystemHandoverTriggered(32),

nGInterSystemHandoverTriggered(33),

xNHandoverTriggered(34),

notSupported5QIValue(35),

uEContextTransfer(36),

iMSVoiceeEPSFallbackOrRATFallbackTriggered(37),

uPIntegrityProtectioNotPossible(38),

uPConfidentialityProtectionNotPossible(39),

sliceNotSupported(40),

uEInRRCInactiveStateNotReachable(41),

redirection(42),

resourcesNotAvailableForTheSlice(43),

uEMaxIntegrityProtectedDataRateReason(44),

releaseDueToCNDetectedMobility(45),

n26InterfaceNotAvailable(46),

releaseDueToPreemption(47),

multipleLocationReportingReferenceIDInstances(48),

rSNNotAvailableForTheUP(49),

nPMAccessDenied(50),

cAGOnlyAccessDenied(51),

insufficientUECapabilities(52)

}

CauseTransport ::= ENUMERATED

{

transportResourceUnavailable(1),

unspecified(2)

}

Direction ::= ENUMERATED

{

fromTarget(1),

toTarget(2)

}

DNN ::= UTF8String

E164Number ::= NumericString (SIZE(1..15))

EmailAddress ::= UTF8String

EquivalentPLMNs ::= SEQUENCE (SIZE(1..MAX)) OF PLMNID

EUI64 ::= OCTET STRING (SIZE(8))

FiveGGUTI ::= SEQUENCE

{

mCC         [1] MCC,

mNC         [2] MNC,

aMFRegionID [3] AMFRegionID,

aMFSetID    [4] AMFSetID,

aMFPointer  [5] AMFPointer,

fiveGTMSI   [6] FiveGTMSI

}

FiveGMMCause ::= INTEGER (0..255)

FiveGSSubscriberID ::= CHOICE

{

sUPI [1] SUPI,

sUCI [2] SUCI,

pEI  [3] PEI,

gPSI [4] GPSI

}

FiveGSSubscriberIDs ::= SEQUENCE

{

fiveGSSubscriberID [1] SEQUENCE SIZE(1..MAX) OF FiveGSSubscriberID

}

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(1),

existingPDUSession(2),

initialEmergencyRequest(3),

existingEmergencyPDUSession(4),

modificationRequest(5),

reserved(6),

mAPDURequest(7)

}

FiveGSMCause ::= INTEGER (0..255)

FiveGTMSI ::= INTEGER (0..4294967295)

FiveGSRVCCInfo ::= SEQUENCE

{

uE5GSRVCCCapability   [1] BOOLEAN,

sessionTransferNumber [2] UTF8String OPTIONAL,

correlationMSISDN     [3] MSISDN OPTIONAL

}

FiveGSUserStateInfo ::= SEQUENCE

{

fiveGSUserState [1] FiveGSUserState,

accessType      [2] AccessType

}

FiveGSUserState ::= ENUMERATED

{

deregistered(1),

registeredNotReachableForPaging(2),

registeredReachableForPaging(3),

connectedNotReachableForPaging(4),

connectedReachableForPaging(5),

notProvidedFromAMF(6)

}

ForbiddenAreaInformation ::= SEQUENCE

{

pLMNIdentity  [1] PLMNID,

forbiddenTACs [2] ForbiddenTACs

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..MAX)) OF TAC

FTEID ::= SEQUENCE

{

tEID        [1] INTEGER (0.. 4294967295),

iPv4Address [2] IPv4Address OPTIONAL,

iPv6Address [3] IPv6Address OPTIONAL

}

FTEIDList ::= SEQUENCE OF FTEID

GPSI ::= CHOICE

{

mSISDN      [1] MSISDN,

nAI         [2] NAI

}

GUAMI ::= SEQUENCE

{

aMFID       [1] AMFID,

pLMNID      [2] PLMNID

}

GUMMEI ::= SEQUENCE

{

mMEID       [1] MMEID,

mCC         [2] MCC,

mNC         [3] MNC

}

GUTI ::= SEQUENCE

{

mCC          [1] MCC,

mNC          [2] MNC,

mMEGroupID   [3] MMEGroupID,

mMECode      [4] MMECode,

mTMSI        [5] TMSI

}

HandoverCause ::= CHOICE

{

radioNetwork    [1] CauseRadioNetwork,

transport       [2] CauseTransport,

nas             [3] CauseNas,

protocol        [4] CauseProtocol,

misc            [5] CauseMisc

}

HandoverType ::= ENUMERATED

{

intra5GS(1),

fiveGStoEPS(2),

ePSto5GS(3),

fiveGStoUTRA(4)

}

HomeNetworkPublicKeyID ::= OCTET STRING

HSMFURI ::= UTF8String

IMEI ::= NumericString (SIZE(14))

IMEISV ::= NumericString (SIZE(16))

IMPI ::= NAI

IMPU ::= CHOICE

{

sIPURI [1] SIPURI,

tELURI [2] TELURI

}

IMSI ::= NumericString (SIZE(6..15))

Initiator ::= ENUMERATED

{

uE(1),

network(2),

unknown(3)

}

IPAddress ::= CHOICE

{

iPv4Address [1] IPv4Address,

iPv6Address [2] IPv6Address

}

IPv4Address ::= OCTET STRING (SIZE(4))

IPv6Address ::= OCTET STRING (SIZE(16))

IPv6FlowLabel ::= INTEGER(0..1048575)

LocationAreaOfInterestList  ::= SEQUENCE (SIZE(1..MAX)) OF AreaOfInterestItem

LocationEventType ::= ENUMERATED

{

direct(1),

changeOfServeCell(2),

uEPrescenceInAreaOfInterest(3),

stopChangeOfServeCell(4),

stopUEPresenceInAreaOfInterest(5),

cancelLocationReportingForTheUE(6)

}

LocationReportArea ::= ENUMERATED

{

cell(1)

}

LocationReportingRequestType ::= SEQUENCE

{

eventType           [1] LocationEventType,

reportArea          [2] LocationReportArea,

areaOfInterestList  [3] LocationAreaOfInterestList

}

MACAddress ::= OCTET STRING (SIZE(6))

MACRestrictionIndicator ::= ENUMERATED

{

noResrictions(1),

mACAddressNotUseableAsEquipmentIdentifier(2),

unknown(3)

}

MCC ::= NumericString (SIZE(3))

MNC ::= NumericString (SIZE(2..3))

MMEID ::= SEQUENCE

{

mMEGI       [1] MMEGI,

mMEC        [2] MMEC

}

MMEC ::= NumericString

MMEGI ::= NumericString

MobilityRestrictionList ::= SEQUENCE

{

servingPLMN               [1] PLMNID,

equivalentPLMNs           [2] EquivalentPLMNs OPTIONAL,

rATRestrictions           [3] RATRestrictions OPTIONAL,

forbiddenAreaInformation  [4] ForbiddenAreaInformation OPTIONAL,

serviceAreaInformation    [5] ServiceAreaInformation OPTIONAL

}

MSISDN ::= NumericString (SIZE(1..15))

NAI ::= UTF8String

NextLayerProtocol ::= INTEGER(0..255)

NonLocalID ::= ENUMERATED

{

local(1),

nonLocal(2)

}

NonIMEISVPEI ::= CHOICE

{

mACAddress [1] MACAddress

}

NPNAccessInformation ::= CHOICE

{

pNINPNAccessInformation [1] CellCAGList

}

NSSAI ::= SEQUENCE OF SNSSAI

PLMNID ::= SEQUENCE

{

mCC [1] MCC,

mNC [2] MNC

}

PLMNList ::= SEQUENCE (SIZE(1..MAX)) OF PLMNID

PDUSessionID ::= INTEGER (0..255)

PDUSessionResourceInformation ::= SEQUENCE

{

pDUSessionID              [1] PDUSessionID

}

PDUSessionType ::= ENUMERATED

{

iPv4(1),

iPv6(2),

iPv4v6(3),

unstructured(4),

ethernet(5)

}

PEI ::= CHOICE

{

iMEI        [1] IMEI,

iMEISV      [2] IMEISV,

mACAddress  [3] MACAddress,

eUI64       [4] EUI64

}

PortNumber ::= INTEGER (0..65535)

PrimaryAuthenticationType ::= ENUMERATED

{

eAPAKAPrime(1),

fiveGAKA(2),

eAPTLS(3),

none(4),

ePSAKA(5),

eAPAKA(6),

iMSAKA(7),

gBAAKA(8),

uMTSAKA(9)

}

ProtectionSchemeID ::= INTEGER (0..15)

RANUENGAPID ::= INTEGER (0..4294967295)

-- See clause 9.3.1.20 of TS 38.413 [23] for details

RANSourceToTargetContainer ::= OCTET STRING

-- See clause 9.3.1.21 of TS 38.413 [23] for details

RANTargetToSourceContainer ::= OCTET STRING

RATRestrictions ::= SEQUENCE (SIZE(1..MAX)) OF RATRestrictionItem

RATRestrictionInformation ::= BIT STRING (SIZE(8, ...))

RATRestrictionItem ::= SEQUENCE

{

pLMNIdentity               [1] PLMNID,

rATRestrictionInformation  [2] RATRestrictionInformation

}

RATType ::= ENUMERATED

{

nR(1),

eUTRA(2),

wLAN(3),

virtual(4),

nBIOT(5),

wireline(6),

wirelineCable(7),

wirelineBBF(8),

lTEM(9),

nRU(10),

eUTRAU(11),

trustedN3GA(12),

trustedWLAN(13),

uTRA(14),

gERA(15),

nRLEO(16),

nRMEO(17),

nRGEO(18),

nROTHERSAT(19),

nRREDCAP(20)

}

RejectedNSSAI ::= SEQUENCE OF RejectedSNSSAI

RejectedSNSSAI ::= SEQUENCE

{

causeValue  [1] RejectedSliceCauseValue,

sNSSAI      [2] SNSSAI

}

RejectedSliceCauseValue ::= INTEGER (0..255)

ReRegRequiredIndicator ::= ENUMERATED

{

reRegistrationRequired(1),

reRegistrationNotRequired(2)

}

RoutingIndicator ::= INTEGER (0..9999)

SchemeOutput ::= OCTET STRING

ServiceAreaInformation ::= SEQUENCE (SIZE(1..MAX)) OF ServiceAreaInfo

ServiceAreaInfo ::= SEQUENCE

{

pLMNIdentity    [1] PLMNID,

allowedTACs     [2] AllowedTACs OPTIONAL,

notAllowedTACs  [3] ForbiddenTACs OPTIONAL

}

SIPURI ::= UTF8String

Slice ::= SEQUENCE

{

allowedNSSAI        [1] NSSAI OPTIONAL,

configuredNSSAI     [2] NSSAI OPTIONAL,

rejectedNSSAI       [3] RejectedNSSAI OPTIONAL

}

SMPDUDNRequest ::= OCTET STRING

-- TS 24.501 [13], clause 9.11.3.6.1

SMSOverNASIndicator ::= ENUMERATED

{

sMSOverNASNotAllowed(1),

sMSOverNASAllowed(2)

}

SNSSAI ::= SEQUENCE

{

sliceServiceType    [1] INTEGER (0..255),

sliceDifferentiator [2] OCTET STRING (SIZE(3)) OPTIONAL

}

SubscriberIdentifier ::= CHOICE

{

sUCI   [1] SUCI,

sUPI   [2] SUPI

}

SUCI ::= SEQUENCE

{

mCC                         [1] MCC,

mNC                         [2] MNC,

routingIndicator            [3] RoutingIndicator,

protectionSchemeID          [4] ProtectionSchemeID,

homeNetworkPublicKeyID      [5] HomeNetworkPublicKeyID,

schemeOutput                [6] SchemeOutput,

routingIndicatorLength      [7] INTEGER (1..4) OPTIONAL

-- shall be included if different from the number of meaningful digits given

-- in routingIndicator

}

SUPI ::= CHOICE

{

iMSI        [1] IMSI,

nAI         [2] NAI

}

SUPIUnauthenticatedIndication ::= BOOLEAN

SwitchOffIndicator ::= ENUMERATED

{

normalDetach(1),

switchOff(2)

}

TargetIdentifier ::= CHOICE

{

sUPI                [1] SUPI,

iMSI                [2] IMSI,

pEI                 [3] PEI,

iMEI                [4] IMEI,

gPSI                [5] GPSI,

mSISDN              [6] MSISDN,

nAI                 [7] NAI,

iPv4Address         [8] IPv4Address,

iPv6Address         [9] IPv6Address,

ethernetAddress     [10] MACAddress

}

TargetIdentifierProvenance ::= ENUMERATED

{

lEAProvided(1),

observed(2),

matchedOn(3),

other(4)

}

TELURI ::= UTF8String

Timestamp ::= GeneralizedTime

UEContextInfo ::= SEQUENCE

{

supportVoPS         [1] BOOLEAN OPTIONAL,

supportVoPSNon3GPP  [2] BOOLEAN OPTIONAL,

lastActiveTime      [3] Timestamp OPTIONAL,

accessType          [4] AccessType OPTIONAL,

rATType             [5] RATType OPTIONAL

}

UEEndpointAddress ::= CHOICE

{

iPv4Address         [1] IPv4Address,

iPv6Address         [2] IPv6Address,

ethernetAddress     [3] MACAddress

}

UserIdentifiers ::= SEQUENCE

{

fiveGSSubscriberIDs [1] FiveGSSubscriberIDs OPTIONAL,

ePSSubscriberIDs    [2] EPSSubscriberIDs OPTIONAL

}

-- ===================

-- Location parameters

-- ===================

Location ::= SEQUENCE

{

locationInfo                [1] LocationInfo OPTIONAL,

positioningInfo             [2] PositioningInfo OPTIONAL,

locationPresenceReport      [3] LocationPresenceReport OPTIONAL,

ePSLocationInfo             [4] EPSLocationInfo OPTIONAL

}

CellSiteInformation ::= SEQUENCE

{

geographicalCoordinates     [1] GeographicalCoordinates,

azimuth                     [2] INTEGER (0..359) OPTIONAL,

operatorSpecificInformation [3] UTF8String OPTIONAL

}

-- TS 29.518 [22], clause 6.4.6.2.6

LocationInfo ::= SEQUENCE

{

userLocation                [1] UserLocation OPTIONAL,

currentLoc                  [2] BOOLEAN OPTIONAL,

geoInfo                     [3] GeographicArea OPTIONAL,

rATType                     [4] RATType OPTIONAL,

timeZone                    [5] TimeZone OPTIONAL,

additionalCellIDs           [6] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.7

UserLocation ::= SEQUENCE

{

eUTRALocation               [1] EUTRALocation OPTIONAL,

nRLocation                  [2] NRLocation OPTIONAL,

n3GALocation                [3] N3GALocation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.8

EUTRALocation ::= SEQUENCE

{

tAI                         [1] TAI,

eCGI                        [2] ECGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalNGENbID               [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL,

globalENbID                 [9] GlobalRANNodeID OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.9

NRLocation ::= SEQUENCE

{

tAI                         [1] TAI,

nCGI                        [2] NCGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.10

N3GALocation ::= SEQUENCE

{

tAI                         [1] TAI OPTIONAL,

n3IWFID                     [2] N3IWFIDNGAP OPTIONAL,

uEIPAddr                    [3] IPAddr OPTIONAL,

portNumber                  [4] INTEGER OPTIONAL,

tNAPID                      [5] TNAPID OPTIONAL,

tWAPID                      [6] TWAPID OPTIONAL,

hFCNodeID                   [7] HFCNodeID OPTIONAL,

gLI                         [8] GLI OPTIONAL,

w5GBANLineType              [9] W5GBANLineType OPTIONAL,

gCI                         [10] GCI OPTIONAL,

ageOfLocationInfo           [11] INTEGER OPTIONAL,

uELocationTimestamp         [12] Timestamp OPTIONAL,

protocol                    [13] TransportProtocol OPTIONAL

}

-- TS 38.413 [23], clause 9.3.2.4

IPAddr ::= SEQUENCE

{

iPv4Addr                    [1] IPv4Address OPTIONAL,

iPv6Addr                    [2] IPv6Address OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.28

GlobalRANNodeID ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

aNNodeID                    [2] ANNodeID,

nID                         [3] NID OPTIONAL

}

ANNodeID ::= CHOICE

{

n3IWFID [1] N3IWFIDSBI,

gNbID   [2] GNbID,

nGENbID [3] NGENbID,

eNbID   [4] ENbID,

wAGFID  [5] WAGFID,

tNGFID  [6] TNGFID

}

-- TS 38.413 [23], clause 9.3.1.6

GNbID ::= BIT STRING(SIZE(22..32))

-- TS 29.571 [17], clause 5.4.4.4

TAI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

tAC                         [2] TAC,

nID                         [3] NID OPTIONAL

}

CGI ::= SEQUENCE

{

lAI    [1] LAI,

cellID [2] CellID

}

LAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC

}

LAC ::= OCTET STRING (SIZE(2))

CellID ::= OCTET STRING (SIZE(2))

SAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC,

sAC    [3] SAC

}

SAC ::= OCTET STRING (SIZE(2))

-- TS 29.571 [17], clause 5.4.4.5

ECGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

eUTRACellID                 [2] EUTRACellID,

nID                         [3] NID OPTIONAL

}

TAIList ::= SEQUENCE OF TAI

-- TS 29.571 [17], clause 5.4.4.6

NCGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

nRCellID                    [2] NRCellID,

nID                         [3] NID OPTIONAL

}

RANCGI ::= CHOICE

{

eCGI                        [1] ECGI,

nCGI                        [2] NCGI

}

CellInformation ::= SEQUENCE

{

rANCGI                      [1] RANCGI,

cellSiteinformation         [2] CellSiteInformation OPTIONAL,

timeOfLocation              [3] Timestamp OPTIONAL

}

-- TS 38.413 [23], clause 9.3.1.57

N3IWFIDNGAP ::= BIT STRING (SIZE(16))

-- TS 29.571 [17], clause 5.4.4.28

N3IWFIDSBI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

TNGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

WAGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62

TNAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.64

TWAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

SSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

BSSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.36 and table 5.4.2-1

HFCNodeID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

GLI ::= OCTET STRING (SIZE(0..150))

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

GCI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.3.38

TransportProtocol ::= ENUMERATED

{

uDP(1),

tCP(2)

}

-- TS 29.571 [17], clause 5.4.4.10 and clause 5.4.3.33

W5GBANLineType ::= ENUMERATED

{

dSL(1),

pON(2)

}

-- TS 29.571 [17], table 5.4.2-1

TAC ::= OCTET STRING (SIZE(2..3))

-- TS 38.413 [23], clause 9.3.1.9

EUTRACellID ::= BIT STRING (SIZE(28))

-- TS 38.413 [23], clause 9.3.1.7

NRCellID ::= BIT STRING (SIZE(36))

-- TS 38.413 [23], clause 9.3.1.8

NGENbID ::= CHOICE

{

macroNGENbID                [1] BIT STRING (SIZE(20)),

shortMacroNGENbID           [2] BIT STRING (SIZE(18)),

longMacroNGENbID            [3] BIT STRING (SIZE(21))

}

-- TS 23.003 [19], clause 12.7.1 encoded as per TS 29.571 [17], clause 5.4.2

NID ::= UTF8String (SIZE(11))

-- TS 36.413 [38], clause 9.2.1.37

ENbID ::= CHOICE

{

macroENbID                  [1] BIT STRING (SIZE(20)),

homeENbID                   [2] BIT STRING (SIZE(28)),

shortMacroENbID             [3] BIT STRING (SIZE(18)),

longMacroENbID              [4] BIT STRING (SIZE(21))

}

-- TS 29.518 [22], clause 6.4.6.2.3

PositioningInfo ::= SEQUENCE

{

positionInfo                [1] LocationData OPTIONAL,

rawMLPResponse              [2] RawMLPResponse OPTIONAL

}

RawMLPResponse ::= CHOICE

{

-- The following parameter contains a copy of unparsed XML code of the

-- MLP response message, i.e. the entire XML document containing

-- a <slia> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.2) or

-- a <slirep> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.3) MLP message.

mLPPositionData             [1] UTF8String,

-- OMA MLP result id, defined in OMA-TS-MLP-V3_5-20181211-C [20], Clause 5.4

mLPErrorCode                [2] INTEGER (1..699)

}

-- TS 29.572 [24], clause 6.1.6.2.3

LocationData ::= SEQUENCE

{

locationEstimate            [1] GeographicArea,

accuracyFulfilmentIndicator [2] AccuracyFulfilmentIndicator OPTIONAL,

ageOfLocationEstimate       [3] AgeOfLocationEstimate OPTIONAL,

velocityEstimate            [4] VelocityEstimate OPTIONAL,

civicAddress                [5] CivicAddress OPTIONAL,

positioningDataList         [6] SET OF PositioningMethodAndUsage OPTIONAL,

gNSSPositioningDataList     [7] SET OF GNSSPositioningMethodAndUsage OPTIONAL,

eCGI                        [8] ECGI OPTIONAL,

nCGI                        [9] NCGI OPTIONAL,

altitude                    [10] Altitude OPTIONAL,

barometricPressure          [11] BarometricPressure OPTIONAL

}

-- TS 29.172 [53], table 6.2.2-2

EPSLocationInfo ::= SEQUENCE

{

locationData  [1] LocationData,

cGI           [2] CGI OPTIONAL,

sAI           [3] SAI OPTIONAL,

eSMLCCellInfo [4] ESMLCCellInfo OPTIONAL

}

-- TS 29.172 [53], clause 7.4.57

ESMLCCellInfo ::= SEQUENCE

{

eCGI          [1] ECGI,

cellPortionID [2] CellPortionID

}

-- TS 29.171 [54], clause 7.4.31

CellPortionID ::= INTEGER (0..4095)

-- TS 29.518 [22], clause 6.2.6.2.5

LocationPresenceReport ::= SEQUENCE

{

type                        [1] AMFEventType,

timestamp                   [2] Timestamp,

areaList                    [3] SET OF AMFEventArea OPTIONAL,

timeZone                    [4] TimeZone OPTIONAL,

accessTypes                 [5] SET OF AccessType OPTIONAL,

rMInfoList                  [6] SET OF RMInfo OPTIONAL,

cMInfoList                  [7] SET OF CMInfo OPTIONAL,

reachability                [8] UEReachability OPTIONAL,

location                    [9] UserLocation OPTIONAL,

additionalCellIDs           [10] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.3.3

AMFEventType ::= ENUMERATED

{

locationReport(1),

presenceInAOIReport(2)

}

-- TS 29.518 [22], clause 6.2.6.2.16

AMFEventArea ::= SEQUENCE

{

presenceInfo                [1] PresenceInfo OPTIONAL,

lADNInfo                    [2] LADNInfo OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.27

PresenceInfo ::= SEQUENCE

{

presenceState               [1] PresenceState OPTIONAL,

trackingAreaList            [2] SET OF TAI OPTIONAL,

eCGIList                    [3] SET OF ECGI OPTIONAL,

nCGIList                    [4] SET OF NCGI OPTIONAL,

globalRANNodeIDList         [5] SET OF GlobalRANNodeID OPTIONAL,

globalENbIDList             [6] SET OF GlobalRANNodeID OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.2.17

LADNInfo ::= SEQUENCE

{

lADN                        [1] UTF8String,

presence                    [2] PresenceState OPTIONAL

}

-- TS 29.571 [17], clause 5.4.3.20

PresenceState ::= ENUMERATED

{

inArea(1),

outOfArea(2),

unknown(3),

inactive(4)

}

-- TS 29.518 [22], clause 6.2.6.2.8

RMInfo ::= SEQUENCE

{

rMState                     [1] RMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.2.9

CMInfo ::= SEQUENCE

{

cMState                     [1] CMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.3.7

UEReachability ::= ENUMERATED

{

unreachable(1),

reachable(2),

regulatoryOnly(3)

}

-- TS 29.518 [22], clause 6.2.6.3.9

RMState ::= ENUMERATED

{

registered(1),

deregistered(2)

}

-- TS 29.518 [22], clause 6.2.6.3.10

CMState ::= ENUMERATED

{

idle(1),

connected(2)

}

-- TS 29.572 [24], clause 6.1.6.2.5

GeographicArea ::= CHOICE

{

point                       [1] Point,

pointUncertaintyCircle      [2] PointUncertaintyCircle,

pointUncertaintyEllipse     [3] PointUncertaintyEllipse,

polygon                     [4] Polygon,

pointAltitude               [5] PointAltitude,

pointAltitudeUncertainty    [6] PointAltitudeUncertainty,

ellipsoidArc                [7] EllipsoidArc

}

-- TS 29.572 [24], clause 6.1.6.3.12

AccuracyFulfilmentIndicator ::= ENUMERATED

{

requestedAccuracyFulfilled(1),

requestedAccuracyNotFulfilled(2)

}

-- TS 29.572 [24], clause 6.1.6.2.17

VelocityEstimate ::= CHOICE

{

horVelocity                         [1] HorizontalVelocity,

horWithVertVelocity                 [2] HorizontalWithVerticalVelocity,

horVelocityWithUncertainty          [3] HorizontalVelocityWithUncertainty,

horWithVertVelocityAndUncertainty   [4] HorizontalWithVerticalVelocityAndUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.14

CivicAddress ::= SEQUENCE

{

country                             [1] UTF8String,

a1                                  [2] UTF8String OPTIONAL,

a2                                  [3] UTF8String OPTIONAL,

a3                                  [4] UTF8String OPTIONAL,

a4                                  [5] UTF8String OPTIONAL,

a5                                  [6] UTF8String OPTIONAL,

a6                                  [7] UTF8String OPTIONAL,

prd                                 [8] UTF8String OPTIONAL,

pod                                 [9] UTF8String OPTIONAL,

sts                                 [10] UTF8String OPTIONAL,

hno                                 [11] UTF8String OPTIONAL,

hns                                 [12] UTF8String OPTIONAL,

lmk                                 [13] UTF8String OPTIONAL,

loc                                 [14] UTF8String OPTIONAL,

nam                                 [15] UTF8String OPTIONAL,

pc                                  [16] UTF8String OPTIONAL,

bld                                 [17] UTF8String OPTIONAL,

unit                                [18] UTF8String OPTIONAL,

flr                                 [19] UTF8String OPTIONAL,

room                                [20] UTF8String OPTIONAL,

plc                                 [21] UTF8String OPTIONAL,

pcn                                 [22] UTF8String OPTIONAL,

pobox                               [23] UTF8String OPTIONAL,

addcode                             [24] UTF8String OPTIONAL,

seat                                [25] UTF8String OPTIONAL,

rd                                  [26] UTF8String OPTIONAL,

rdsec                               [27] UTF8String OPTIONAL,

rdbr                                [28] UTF8String OPTIONAL,

rdsubbr                             [29] UTF8String OPTIONAL,

prm                                 [30] UTF8String OPTIONAL,

pom                                 [31] UTF8String OPTIONAL

}

-- TS 29.571 [17], clauses 5.4.4.62 and 5.4.4.64

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

CivicAddressBytes ::= OCTET STRING

-- TS 29.572 [24], clause 6.1.6.2.15

PositioningMethodAndUsage ::= SEQUENCE

{

method                              [1] PositioningMethod,

mode                                [2] PositioningMode,

usage                               [3] Usage,

methodCode                          [4] MethodCode OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.16

GNSSPositioningMethodAndUsage ::= SEQUENCE

{

mode                                [1] PositioningMode,

gNSS                                [2] GNSSID,

usage                               [3] Usage

}

-- TS 29.572 [24], clause 6.1.6.2.6

Point ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.7

PointUncertaintyCircle ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] Uncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.8

PointUncertaintyEllipse ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] UncertaintyEllipse,

confidence                          [3] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.9

Polygon ::= SEQUENCE

{

pointList                           [1] SET SIZE (3..15) OF GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.10

PointAltitude ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude

}

-- TS 29.572 [24], clause 6.1.6.2.11

PointAltitudeUncertainty ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude,

uncertaintyEllipse                  [3] UncertaintyEllipse,

uncertaintyAltitude                 [4] Uncertainty,

confidence                          [5] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.12

EllipsoidArc ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

innerRadius                         [2] InnerRadius,

uncertaintyRadius                   [3] Uncertainty,

offsetAngle                         [4] Angle,

includedAngle                       [5] Angle,

confidence                          [6] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.4

GeographicalCoordinates ::= SEQUENCE

{

latitude                            [1] UTF8String,

longitude                           [2] UTF8String,

mapDatumInformation                 [3] OGCURN OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.22

UncertaintyEllipse ::= SEQUENCE

{

semiMajor                           [1] Uncertainty,

semiMinor                           [2] Uncertainty,

orientationMajor                    [3] Orientation

}

-- TS 29.572 [24], clause 6.1.6.2.18

HorizontalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle

}

-- TS 29.572 [24], clause 6.1.6.2.19

HorizontalWithVerticalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection

}

-- TS 29.572 [24], clause 6.1.6.2.20

HorizontalVelocityWithUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

uncertainty                         [3] SpeedUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.21

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection,

hUncertainty                        [5] SpeedUncertainty,

vUncertainty                        [6] SpeedUncertainty

}

-- The following types are described in TS 29.572 [24], table 6.1.6.3.2-1

Altitude ::= UTF8String

Angle ::= INTEGER (0..360)

Uncertainty ::= INTEGER (0..127)

Orientation ::= INTEGER (0..180)

Confidence ::= INTEGER (0..100)

InnerRadius ::= INTEGER (0..327675)

AgeOfLocationEstimate ::= INTEGER (0..32767)

HorizontalSpeed ::= UTF8String

VerticalSpeed ::= UTF8String

SpeedUncertainty ::= UTF8String

BarometricPressure ::= INTEGER (30000..115000)

-- TS 29.572 [24], clause 6.1.6.3.13

VerticalDirection ::= ENUMERATED

{

upward(1),

downward(2)

}

-- TS 29.572 [24], clause 6.1.6.3.6

PositioningMethod ::= ENUMERATED

{

cellID(1),

eCID(2),

oTDOA(3),

barometricPressure(4),

wLAN(5),

bluetooth(6),

mBS(7),

motionSensor(8),

dLTDOA(9),

dLAOD(10),

multiRTT(11),

nRECID(12),

uLTDOA(13),

uLAOA(14),

networkSpecific(15)

}

-- TS 29.572 [24], clause 6.1.6.3.7

PositioningMode ::= ENUMERATED

{

uEBased(1),

uEAssisted(2),

conventional(3)

}

-- TS 29.572 [24], clause 6.1.6.3.8

GNSSID ::= ENUMERATED

{

gPS(1),

galileo(2),

sBAS(3),

modernizedGPS(4),

qZSS(5),

gLONASS(6),

bDS(7),

nAVIC(8)

}

-- TS 29.572 [24], clause 6.1.6.3.9

Usage ::= ENUMERATED

{

unsuccess(1),

successResultsNotUsed(2),

successResultsUsedToVerifyLocation(3),

successResultsUsedToGenerateLocation(4),

successMethodNotDetermined(5)

}

-- TS 29.571 [17], table 5.2.2-1

TimeZone ::= UTF8String

-- Open Geospatial Consortium URN [35]

OGCURN ::= UTF8String

-- TS 29.572 [24], clause 6.1.6.2.15

MethodCode ::= INTEGER (16..31)

END

#### *** End of Third Change ***

#### *** Start of Fourth Change ***

Annex C (normative):
XSD Schema for LI_X1 extensions

<?xml version="1.0" encoding="utf-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns="urn:3GPP:ns:li:3GPPX1Extensions:r17:v3"

xmlns:common="http://uri.etsi.org/03280/common/2017/07"

targetNamespace="urn:3GPP:ns:li:3GPPX1Extensions:r17:v3"

elementFormDefault="qualified">

<xs:import namespace="http://uri.etsi.org/03280/common/2017/07"/>

<xs:element name="X1Extensions" type="X1Extension"></xs:element>

<xs:element name="PTCLIX1TargetIdentifierExtensions" type="PTCLIX1TargetIdentifierExtensions"></xs:element>

<xs:complexType name="PTCLIX1TargetIdentifierExtensions">

<xs:sequence>

<xs:element name="PTCLIX1TargetIdentifier" type="PTCLIX1TargetIdentifier" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PTCLIX1TargetIdentifier">

<xs:choice>

<xs:element name="MCPTTID" type="MCPTTID"></xs:element>

<xs:element name="InstanceIdentifierURN" type="InstanceIdentifierURN"></xs:element>

<xs:element name="PTCChatGroupID" type="PTCChatGroupID"></xs:element>

</xs:choice>

</xs:complexType>

<xs:simpleType name="MCPTTID">

<xs:restriction base="xs:anyURI"></xs:restriction>

</xs:simpleType>

<xs:simpleType name="InstanceIdentifierURN">

<xs:restriction base="xs:anyURI"></xs:restriction>

</xs:simpleType>

<xs:simpleType name="PTCChatGroupID">

<xs:restriction base="xs:anyURI"></xs:restriction>

</xs:simpleType>

<xs:element name="UPFLIT3TargetIdentifierExtensions" type="UPFLIT3TargetIdentifierExtensions"></xs:element>

<xs:complexType name="UPFLIT3TargetIdentifierExtensions">

<xs:sequence>

<xs:element name="UPFLIT3TargetIdentifier" type="UPFLIT3TargetIdentifier" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="UPFLIT3TargetIdentifier">

<xs:choice>

<xs:element name="FSEID" type="FSEID"></xs:element>

<xs:element name="PDRID" type="xs:unsignedInt"></xs:element>

<xs:element name="QERID" type="xs:unsignedInt"></xs:element>

<xs:element name="NetworkInstance" type="xs:hexBinary"></xs:element>

<xs:element name="GTPTunnelDirection" type="GTPTunnelDirection"></xs:element>

<xs:element name="FTEID" type="FTEID"></xs:element>

</xs:choice>

</xs:complexType>

<xs:complexType name="FSEID">

<xs:sequence>

<xs:element name="SEID" type="xs:unsignedLong"></xs:element>

<xs:element name="IPv4Address" type="common:IPv4Address" minOccurs="0"></xs:element>

<xs:element name="IPv6Address" type="common:IPv6Address" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="FTEID">

<xs:sequence>

<xs:element name="TEID" type="xs:unsignedInt"></xs:element>

<xs:element name="IPv4Address" type="common:IPv4Address" minOccurs="0"></xs:element>

<xs:element name="IPv6Address" type="common:IPv6Address" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="GTPTunnelDirection">

<xs:restriction base="xs:string">

<xs:enumeration value="Outbound"></xs:enumeration>

<xs:enumeration value="Inbound"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:element name="IdentifierAssociationExtensions" type="IdentifierAssociationExtensions" ></xs:element>

<xs:complexType name="X1Extension">

<xs:choice>

<xs:element name="LALSLILCSTargetProvisioning" type="LALSLILCSTargetProvisioningExtensions"></xs:element>

<xs:element name="LALSLTFProvisioning" type="LALSLTFProvisioningExtensions"></xs:element>

<xs:element name="HeaderReporting" type="PDHRReportingExtensions"></xs:element>

<xs:element name="SMSFExtensions" type="SMSFProvisioningExtensions"></xs:element>

<xs:element name="IdentifierAssociation" type="IdentifierAssociationExtensions"></xs:element>

<xs:element name="SDP" type="SDP"></xs:element>

</xs:choice>

</xs:complexType>

<xs:complexType name="LALSLILCSTargetProvisioningExtensions">

<xs:sequence>

<xs:element name="PositioningServiceType" type="PositioningServiceType"></xs:element>

<xs:element name="PositioningPeriodicity" type="PositioningPeriodicity" minOccurs="0"></xs:element>

<xs:element name="PositioningParameters" type="PositioningParameters" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="PositioningServiceType">

<xs:restriction base="xs:string">

<xs:enumeration value="Immediate"></xs:enumeration>

<xs:enumeration value="Periodic"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PositioningPeriodicity">

<xs:restriction base="xs:nonNegativeInteger">

</xs:restriction>

</xs:simpleType>

<xs:complexType name="PositioningParameters">

<xs:sequence>

<xs:element name="RequestedLocationType" type="RequestedLocationType" minOccurs="0"></xs:element>

<xs:element name="RequestedResponseType" type="RequestedResponseType" minOccurs="0"></xs:element>

<xs:element name="MaxLocationAge" type="xs:nonNegativeInteger" minOccurs="0"></xs:element>

<xs:element name="ResponseTimingRequired" type="ResponseTimingRequired" minOccurs="0"></xs:element>

<xs:element name="ResponseTimer" type="xs:nonNegativeInteger" minOccurs="0"></xs:element>

<xs:element name="HorizontalAccuracy" type="NumberWithQOSClass" minOccurs="0"></xs:element>

<xs:element name="AltitudeAccuracy" type="NumberWithQOSClass" minOccurs="0"></xs:element>

<xs:element name="MotionStateRequest" type="EmptyElement" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="RequestedLocationType">

<xs:restriction base="xs:string">

<xs:enumeration value="CURRENT"></xs:enumeration>

<xs:enumeration value="CURRENT_OR_LAST"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RequestedResponseType">

<xs:restriction base="xs:string">

<xs:enumeration value="SYNC"></xs:enumeration>

<xs:enumeration value="ASYNC"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ResponseTimingRequired">

<xs:restriction base="xs:string">

<xs:enumeration value="NO_DELAY"></xs:enumeration>

<xs:enumeration value="LOW_DELAY"></xs:enumeration>

<xs:enumeration value="DELAY_TOL"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="NumberWithQOSClass">

<xs:simpleContent>

<xs:extension base="xs:nonNegativeInteger">

<xs:attribute name="qos_class" type="QOSClass"></xs:attribute>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="QOSClass">

<xs:restriction base="xs:string">

<xs:enumeration value="ASSURED"></xs:enumeration>

<xs:enumeration value="BEST_EFFORT"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="EmptyElement">

<xs:restriction base="xs:string">

<xs:enumeration value=""></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="LALSLTFProvisioningExtensions">

<xs:sequence>

<xs:element name="LILCSClientAddress" type="LILCSClientIPAddress"></xs:element>

<xs:element name="PositioningParameters" type="PositioningParameters" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="LILCSClientIPAddress">

<xs:sequence>

<xs:choice>

<xs:element name="IPv4Address" type="common:IPv4Address"/>

<xs:element name="IPv6Address" type="common:IPv6Address"/>

</xs:choice>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PDHRReportingExtensions">

<xs:sequence>

<xs:element name="PDHType" type="PDHType"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PDHType">

<xs:choice>

<xs:element name="PDHR" type="EmptyElement"></xs:element>

<xs:element name="PDSR" type="PDSRParameters"></xs:element>

</xs:choice>

</xs:complexType>

<xs:complexType name="PDSRParameters">

<xs:sequence>

<xs:element name="PDSRTriggerType" type="PDSRTriggerType" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PDSRTriggerType">

<xs:choice>

<xs:element name="TimerExpiry" type="TimerExpiryInSeconds"></xs:element>

<xs:element name="PacketCount" type="xs:nonNegativeInteger"></xs:element>

<xs:element name="ByteCount" type="xs:nonNegativeInteger"></xs:element>

</xs:choice>

</xs:complexType>

<xs:complexType name="SMSFProvisioningExtensions">

<xs:sequence>

<xs:element name="TruncateTPUserData" type="EmptyElement" minOccurs="0"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="TimerExpiryInSeconds">

<xs:restriction base="xs:nonNegativeInteger">

</xs:restriction>

</xs:simpleType>

<xs:complexType name="IdentifierAssociationExtensions">

<xs:sequence>

<xs:element name="IdentifierAssociationEventsGenerated" type="IdentifierAssociationEventsGenerated"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="IdentifierAssociationEventsGenerated">

<xs:restriction base="xs:string">

<xs:enumeration value="IdentifierAssociation"></xs:enumeration>

<xs:enumeration value="All"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:element name="IdentityAssociationTargetIdentifier" type="EmptyElement"></xs:element>

<xs:element name="AKMATargetIdentifier" type="AKMATargetIdentifier"></xs:element>

<xs:complexType name="AKMATargetIdentifier">

<xs:choice>

<xs:element name="AKID" type="common:NAI"></xs:element>

</xs:choice>

</xs:complexType>

<xs:element name="HR" type="EmptyElement"></xs:element>

<xs:element name="IMSSignaling" type="EmptyElement"></xs:element>

<xs:element name="HRLIT1TargetIdentifierExtensions" type="HRLIT1TargetIdentifierExtensions"></xs:element>

<xs:complexType name="HRLIT1TargetIdentifierExtensions">

<xs:sequence>

<xs:element name="HRLIT1TargetIdentifier" type="HRLIT1TargetIdentifier" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="HRLIT1TargetIdentifier">

<xs:choice>

<xs:element name="PDUSessionID" type="PDUSessionID"></xs:element>

<xs:element name="BearerID" type="BearerID"></xs:element>

<xs:element name="IMSVoiceMedia" type="EmptyElement"></xs:element>

</xs:choice>

</xs:complexType>

<xs:simpleType name="PDUSessionID">

<xs:restriction base="xs:unsignedInt">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="BearerID">

<xs:restriction base="xs:unsignedInt">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="RCSTargetIdentifierExtensions" type="RCSTargetIdentifierExtensions"></xs:element>

<xs:complexType name="RCSTargetIdentifierExtensions">

<xs:sequence>

<xs:element name="RCSTargetIdentifier" type="RCSTargetIdentifier" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="RCSTargetIdentifier">

<xs:choice>

<xs:element name="RCSContentURI" type="RCSContentURI"></xs:element>

</xs:choice>

</xs:complexType>

<xs:simpleType name="RCSContentURI">

<xs:restriction base="xs:anyURI"></xs:restriction>

</xs:simpleType>

<xs:element name="IMST3TargetIdentifierExtensions" type="IMST3TargetIdentifierExtensions"></xs:element>

<xs:complexType name="IMST3TargetIdentifierExtensions">

<xs:sequence>

<xs:element name="IMST3TargetIdentifierExtension" type="IMST3TargetIdentifierExtension" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IMST3TargetIdentifierExtension">

<xs:choice>

<xs:element name="H248ContextID" type="H248ContextID"></xs:element>

<xs:element name="PayloadDirectionAssignment" type="PayloadDirectionAssignment"></xs:element>

<xs:element name="TriggerScope" type="TriggerScope"></xs:element>

</xs:choice>

</xs:complexType>

<xs:simpleType name="PayloadDirectionAssignment">

<xs:restriction base="xs:string">

<xs:enumeration value="ToTarget"></xs:enumeration>

<xs:enumeration value="FromTarget"></xs:enumeration>

<xs:enumeration value="NotDetermined"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="H248ContextID">

<xs:restriction base="xs:integer">

<xs:minInclusive value="1"></xs:minInclusive>

<xs:maxInclusive value="4294967293"></xs:maxInclusive>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="TriggerScope">

<xs:restriction base="xs:string">

<xs:enumeration value="Unidirectional"></xs:enumeration>

<xs:enumeration value="Bidirectional"></xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="SDP">

<xs:sequence>

<xs:element name="SDPData" type="SDPData" minOccurs="1" maxOccurs="unbounded"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SDPData">

<xs:choice>

<xs:element name="LocalSDP" type="SDPInfo"></xs:element>

<xs:element name="RemoteSDP" type="SDPInfo"></xs:element>

</xs:choice>

</xs:complexType>

<xs:simpleType name="SDPInfo">

<xs:restriction base="xs:string">

</xs:restriction>

</xs:simpleType>

</xs:schema>

#### *** End of Fourth Change ***

#### *** End of Last Change ***