3GPP TSG-SA3 Meeting #88-LI-e-a	s3i230101

# Online, , 23rd Jan 2023 - 27th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0487 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the encoding of Uncertainty in Location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD, Rogers Communications Canada, Softel Systems) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2023-01-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The location structures that were used as the basis for the structures defined in TS 33.128 use two different formats for the Uncertainty value. TS 33.128 only defines one format and provides no information on how to translate the other format so that it can be accurately reported. This CR proposes a solution that both provides information on translating the Uncertainty value from one format to another and provides a new format to carry the Uncertainty represented as a float value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the existing Uncertainty type name to UncertaintyGAD and define a new UncertaintySBI type. Include comments with guidance for how to translate a float value into the 0-127 integer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of the Uncertainty parameter in the locations structures will remain undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !143 Commit hash: 0216e86d8599cd243985886fafb535fbc30400da TDocs S3i230053, S3i230054, S3i230056 (CRs 0485, 0486, 0488) are the release 15, 16 and 18 mirrors for this document. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i230055 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (ATTACHMENT TS33128Payloads.asn) ***

Annex A (normative):
ASN.1 Schema for the Internal and External Interfaces

Ts33128payloads

{itu-t(0) identified-organization(4) etsi(0) securitydomain(2) lawfulintercept(2) threegpp(4) ts33128(19) r17(17) version6(6)}

Definitions implicit tags extensibility implied ::=

Begin

-- =============

-- relative oids

-- =============

Ts33128payloadsoid          relative-oid ::= {threegpp(4) ts33128(19) r17(17) version6(6)}

Xiripayloadoid              relative-oid ::= {ts33128payloadsoid xiri(1)}

Xccpayloadoid               relative-oid ::= {ts33128payloadsoid xcc(2)}

Iripayloadoid               relative-oid ::= {ts33128payloadsoid iri(3)}

Ccpayloadoid                relative-oid ::= {ts33128payloadsoid cc(4)}

Linotificationpayloadoid    relative-oid ::= {ts33128payloadsoid linotification(5)}

-- ===============

-- x2 xiri payload

-- ===============

Xiripayload ::= sequence

{

xiripayloadoid      [1] relative-oid,

event               [2] xirievent

}

Xirievent ::= choice

{

-- access and mobility related events, see clause 6.2.2

registration                                        [1] amfregistration,

deregistration                                      [2] amfderegistration,

locationupdate                                      [3] amflocationupdate,

startofinterceptionwithregisteredue                 [4] amfstartofinterceptionwithregisteredue,

unsuccessfulamprocedure                             [5] amfunsuccessfulprocedure,

-- pdu session-related events, see clause 6.2.3

pdusessionestablishment                             [6] smfpdusessionestablishment,

pdusessionmodification                              [7] smfpdusessionmodification,

pdusessionrelease                                   [8] smfpdusessionrelease,

startofinterceptionwithestablishedpdusession        [9] smfstartofinterceptionwithestablishedpdusession,

unsuccessfulsmprocedure                             [10] smfunsuccessfulprocedure,

-- subscriber-management related events, see clause 7.2.2

servingsystemmessage                                [11] udmservingsystemmessage,

-- sms-related events, see clause 6.2.5, see also smsreport ([56] below)

smsmessage                                          [12] smsmessage,

-- lals-related events, see clause 7.3.1

lalsreport                                          [13] lalsreport,

-- pdhr/pdsr-related events, see clause 6.2.3.4.1

pdheaderreport                                      [14] pdheaderreport,

pdsummaryreport                                     [15] pdsummaryreport,

-- tag 16 is reserved because there is no equivalent mdfcellsitereport in xirievent

-- mms-related events, see clause 7.4.2

mmssend                                             [17] mmssend,

mmssendbynonlocaltarget                             [18] mmssendbynonlocaltarget,

mmsnotification                                     [19] mmsnotification,

mmssendtononlocaltarget                             [20] mmssendtononlocaltarget,

mmsnotificationresponse                             [21] mmsnotificationresponse,

mmsretrieval                                        [22] mmsretrieval,

mmsdeliveryack                                      [23] mmsdeliveryack,

mmsforward                                          [24] mmsforward,

mmsdeletefromrelay                                  [25] mmsdeletefromrelay,

mmsdeliveryreport                                   [26] mmsdeliveryreport,

mmsdeliveryreportnonlocaltarget                     [27] mmsdeliveryreportnonlocaltarget,

mmsreadreport                                       [28] mmsreadreport,

mmsreadreportnonlocaltarget                         [29] mmsreadreportnonlocaltarget,

mmscancel                                           [30] mmscancel,

mmsmboxstore                                        [31] mmsmboxstore,

mmsmboxupload                                       [32] mmsmboxupload,

mmsmboxdelete                                       [33] mmsmboxdelete,

mmsmboxviewrequest                                  [34] mmsmboxviewrequest,

mmsmboxviewresponse                                 [35] mmsmboxviewresponse,

-- ptc-related events, see clause 7.5.2

ptcregistration                                     [36] ptcregistration,

ptcsessioninitiation                                [37] ptcsessioninitiation,

ptcsessionabandon                                   [38] ptcsessionabandon,

ptcsessionstart                                     [39] ptcsessionstart,

ptcsessionend                                       [40] ptcsessionend,

ptcstartofinterception                              [41] ptcstartofinterception,

ptcpreestablishedsession                            [42] ptcpreestablishedsession,

ptcinstantpersonalalert                             [43] ptcinstantpersonalalert,

ptcpartyjoin                                        [44] ptcpartyjoin,

ptcpartydrop                                        [45] ptcpartydrop,

ptcpartyhold                                        [46] ptcpartyhold,

ptcmediamodification                                [47] ptcmediamodification,

ptcgroupadvertisement                               [48] ptcgroupadvertisement,

ptcfloorcontrol                                     [49] ptcfloorcontrol,

ptctargetpresence                                   [50] ptctargetpresence,

ptcparticipantpresence                              [51] ptcparticipantpresence,

ptclistmanagement                                   [52] ptclistmanagement,

ptcaccesspolicy                                     [53] ptcaccesspolicy,

-- more subscriber-management related events, see clause 7.2.2

subscriberrecordchangemessage                       [54] udmsubscriberrecordchangemessage,

cancellocationmessage                               [55] udmcancellocationmessage,

-- sms-related events continued from choice 12

smsreport                                           [56] smsreport,

-- ma pdu session-related events, see clause 6.2.3.2.7

smfmapdusessionestablishment                        [57] smfmapdusessionestablishment,

smfmapdusessionmodification                         [58] smfmapdusessionmodification,

smfmapdusessionrelease                              [59] smfmapdusessionrelease,

startofinterceptionwithestablishedmapdusession      [60] smfstartofinterceptionwithestablishedmapdusession,

unsuccessfulmasmprocedure                           [61] smfmaunsuccessfulprocedure,

-- identifier association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

amfidentifierassociation                            [62] amfidentifierassociation,

mmeidentifierassociation                            [63] mmeidentifierassociation,

-- pdu to ma pdu session-related events, see clause 6.2.3.2.8

smfpdutomapdusessionmodification                    [64] smfpdutomapdusessionmodification,

-- nef services related events, see clause 7.7.2

nefpdusessionestablishment                          [65] nefpdusessionestablishment,

nefpdusessionmodification                           [66] nefpdusessionmodification,

nefpdusessionrelease                                [67] nefpdusessionrelease,

nefunsuccessfulprocedure                            [68] nefunsuccessfulprocedure,

nefstartofinterceptionwithestablishedpdusession     [69] nefstartofinterceptionwithestablishedpdusession,

nefdevicetrigger                                    [70] nefdevicetrigger,

nefdevicetriggerreplace                             [71] nefdevicetriggerreplace,

nefdevicetriggercancellation                        [72] nefdevicetriggercancellation,

nefdevicetriggerreportnotify                        [73] nefdevicetriggerreportnotify,

nefmsisdnlessmosms                                  [74] nefmsisdnlessmosms,

nefexpecteduebehaviourupdate                        [75] nefexpecteduebehaviourupdate,

-- scef services related events, see clause 7.8.2

scefpdnconnectionestablishment                      [76] scefpdnconnectionestablishment,

scefpdnconnectionupdate                             [77] scefpdnconnectionupdate,

scefpdnconnectionrelease                            [78] scefpdnconnectionrelease,

scefunsuccessfulprocedure                           [79] scefunsuccessfulprocedure,

scefstartofinterceptionwithestablishedpdnconnection [80] scefstartofinterceptionwithestablishedpdnconnection,

scefdevicetrigger                                   [81] scefdevicetrigger,

scefdevicetriggerreplace                            [82] scefdevicetriggerreplace,

scefdevicetriggercancellation                       [83] scefdevicetriggercancellation,

scefdevicetriggerreportnotify                       [84] scefdevicetriggerreportnotify,

scefmsisdnlessmosms                                 [85] scefmsisdnlessmosms,

scefcommunicationpatternupdate                      [86] scefcommunicationpatternupdate,

-- eps events, see clause 6.3

-- mme events, see clause 6.3.2.2

mmeattach                                           [87] mmeattach,

mmedetach                                           [88] mmedetach,

mmelocationupdate                                   [89] mmelocationupdate,

mmestartofinterceptionwithepsattachedue             [90] mmestartofinterceptionwithepsattachedue,

mmeunsuccessfulprocedure                            [91] mmeunsuccessfulprocedure,

-- akma key management events, see clause 7.9.1

aanfanchorkeyregister                               [92] aanfanchorkeyregister,

aanfkakmaapplicationkeyget                          [93] aanfkakmaapplicationkeyget,

aanfstartofinterceptwithestablishedakmakeymaterial  [94] aanfstartofinterceptwithestablishedakmakeymaterial,

aanfakmacontextremovalrecord                        [95] aanfakmacontextremovalrecord,

afakmaapplicationkeyrefresh                         [96] afakmaapplicationkeyrefresh,

afstartofinterceptwithestablishedakmaapplicationkey [97] afstartofinterceptwithestablishedakmaapplicationkey,

afauxiliarysecurityparameterestablishment           [98] afauxiliarysecurityparameterestablishment,

afapplicationkeyremoval                             [99] afapplicationkeyremoval,

-- hr li events, see clause 7.10.3.3

n9hrpdusessioninfo                                  [100] n9hrpdusessioninfo,

s8hrbearerinfo                                      [101] s8hrbearerinfo,

-- separated location reporting, see clause 7.3.4

separatedlocationreporting                          [102] separatedlocationreporting,

-- stir shaken and rcd/ecnam events, see clause 7.11.2

stirshakensignaturegeneration                       [103] stirshakensignaturegeneration,

stirshakensignaturevalidation                       [104] stirshakensignaturevalidation,

-- ims events, see clause 7.12.4.2

imsmessage                                          [105] imsmessage,

startofinterceptionforactiveimssession              [106] startofinterceptionforactiveimssession,

imsccunavailable                                    [107] imsccunavailable,

-- udm events, see clause 7.2.2

udmlocationinformationresult                        [108] udmlocationinformationresult,

udmueinformationresponse                            [109] udmueinformationresponse,

udmueauthenticationresponse                         [110] udmueauthenticationresponse,

-- amf events, see 6.2.2.2.8

positioninginfotransfer                             [111] amfpositioninginfotransfer,

-- mme events, see clause 6.3.2.2.8

mmepositioninginfotransfer                          [112] mmepositioninginfotransfer,

-- tags 113 to 131 are not used in this version of the specification

-- amf events, see 6.2.2.2.9, continued from tag 111

amfueconfigurationupdate                            [132] amfueconfigurationupdate

}

-- ==============

-- x3 xcc payload

-- ==============

-- no additional xcc payload definitions required in the present document.

-- ===============

-- hi2 iri payload

-- ===============

Iripayload ::= sequence

{

iripayloadoid       [1] relative-oid,

event               [2] irievent,

targetidentifiers   [3] sequence of iritargetidentifier optional

}

Irievent ::= choice

{

-- registration-related events, see clause 6.2.2

registration                                        [1] amfregistration,

deregistration                                      [2] amfderegistration,

locationupdate                                      [3] amflocationupdate,

startofinterceptionwithregisteredue                 [4] amfstartofinterceptionwithregisteredue,

unsuccessfulregistrationprocedure                   [5] amfunsuccessfulprocedure,

-- pdu session-related events, see clause 6.2.3

pdusessionestablishment                             [6] smfpdusessionestablishment,

pdusessionmodification                              [7] smfpdusessionmodification,

pdusessionrelease                                   [8] smfpdusessionrelease,

startofinterceptionwithestablishedpdusession        [9] smfstartofinterceptionwithestablishedpdusession,

unsuccessfulsessionprocedure                        [10] smfunsuccessfulprocedure,

-- subscriber-management related events, see clause 7.2.2

servingsystemmessage                                [11] udmservingsystemmessage,

-- sms-related events, see clause 6.2.5, see also smsreport ([56] below)

smsmessage                                          [12] smsmessage,

-- lals-related events, see clause 7.3.1

lalsreport                                          [13] lalsreport,

-- pdhr/pdsr-related events, see clause 6.2.3.4.1

pdheaderreport                                      [14] pdheaderreport,

pdsummaryreport                                     [15] pdsummaryreport,

-- mdf-related events, see clause 7.3.2

mdfcellsitereport                                   [16] mdfcellsitereport,

-- mms-related events, see clause 7.4.2

mmssend                                             [17] mmssend,

mmssendbynonlocaltarget                             [18] mmssendbynonlocaltarget,

mmsnotification                                     [19] mmsnotification,

mmssendtononlocaltarget                             [20] mmssendtononlocaltarget,

mmsnotificationresponse                             [21] mmsnotificationresponse,

mmsretrieval                                        [22] mmsretrieval,

mmsdeliveryack                                      [23] mmsdeliveryack,

mmsforward                                          [24] mmsforward,

mmsdeletefromrelay                                  [25] mmsdeletefromrelay,

mmsdeliveryreport                                   [26] mmsdeliveryreport,

mmsdeliveryreportnonlocaltarget                     [27] mmsdeliveryreportnonlocaltarget,

mmsreadreport                                       [28] mmsreadreport,

mmsreadreportnonlocaltarget                         [29] mmsreadreportnonlocaltarget,

mmscancel                                           [30] mmscancel,

mmsmboxstore                                        [31] mmsmboxstore,

mmsmboxupload                                       [32] mmsmboxupload,

mmsmboxdelete                                       [33] mmsmboxdelete,

mmsmboxviewrequest                                  [34] mmsmboxviewrequest,

mmsmboxviewresponse                                 [35] mmsmboxviewresponse,

-- ptc-related events, see clause 7.5.2

ptcregistration                                     [36] ptcregistration,

ptcsessioninitiation                                [37] ptcsessioninitiation,

ptcsessionabandon                                   [38] ptcsessionabandon,

ptcsessionstart                                     [39] ptcsessionstart,

ptcsessionend                                       [40] ptcsessionend,

ptcstartofinterception                              [41] ptcstartofinterception,

ptcpreestablishedsession                            [42] ptcpreestablishedsession,

ptcinstantpersonalalert                             [43] ptcinstantpersonalalert,

ptcpartyjoin                                        [44] ptcpartyjoin,

ptcpartydrop                                        [45] ptcpartydrop,

ptcpartyhold                                        [46] ptcpartyhold,

ptcmediamodification                                [47] ptcmediamodification,

ptcgroupadvertisement                               [48] ptcgroupadvertisement,

ptcfloorcontrol                                     [49] ptcfloorcontrol,

ptctargetpresence                                   [50] ptctargetpresence,

ptcparticipantpresence                              [51] ptcparticipantpresence,

ptclistmanagement                                   [52] ptclistmanagement,

ptcaccesspolicy                                     [53] ptcaccesspolicy,

-- more subscriber-management related events, see clause 7.2.2

subscriberrecordchangemessage                      [54] udmsubscriberrecordchangemessage,

cancellocationmessage                              [55] udmcancellocationmessage,

-- sms-related events, continued from choice 12

smsreport                                           [56] smsreport,

-- ma pdu session-related events, see clause 6.2.3.2.7

smfmapdusessionestablishment                        [57] smfmapdusessionestablishment,

smfmapdusessionmodification                         [58] smfmapdusessionmodification,

smfmapdusessionrelease                              [59] smfmapdusessionrelease,

startofinterceptionwithestablishedmapdusession      [60] smfstartofinterceptionwithestablishedmapdusession,

unsuccessfulmasmprocedure                           [61] smfmaunsuccessfulprocedure,

-- identifier association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

amfidentifierassociation                           [62] amfidentifierassociation,

mmeidentifierassociation                           [63] mmeidentifierassociation,

-- pdu to ma pdu session-related events, see clause 6.2.3.2.8

smfpdutomapdusessionmodification                    [64] smfpdutomapdusessionmodification,

-- nef services related events, see clause 7.7.2,

nefpdusessionestablishment                          [65] nefpdusessionestablishment,

nefpdusessionmodification                           [66] nefpdusessionmodification,

nefpdusessionrelease                                [67] nefpdusessionrelease,

nefunsuccessfulprocedure                            [68] nefunsuccessfulprocedure,

nefstartofinterceptionwithestablishedpdusession     [69] nefstartofinterceptionwithestablishedpdusession,

nefdevicetrigger                                    [70] nefdevicetrigger,

nefdevicetriggerreplace                             [71] nefdevicetriggerreplace,

nefdevicetriggercancellation                        [72] nefdevicetriggercancellation,

nefdevicetriggerreportnotify                        [73] nefdevicetriggerreportnotify,

nefmsisdnlessmosms                                  [74] nefmsisdnlessmosms,

nefexpecteduebehaviourupdate                        [75] nefexpecteduebehaviourupdate,

-- scef services related events, see clause 7.8.2

scefpdnconnectionestablishment                      [76] scefpdnconnectionestablishment,

scefpdnconnectionupdate                             [77] scefpdnconnectionupdate,

scefpdnconnectionrelease                            [78] scefpdnconnectionrelease,

scefunsuccessfulprocedure                           [79] scefunsuccessfulprocedure,

scefstartofinterceptionwithestablishedpdnconnection [80] scefstartofinterceptionwithestablishedpdnconnection,

scefdevicetrigger                                   [81] scefdevicetrigger,

scefdevicetriggerreplace                            [82] scefdevicetriggerreplace,

scefdevicetriggercancellation                       [83] scefdevicetriggercancellation,

scefdevicetriggerreportnotify                       [84] scefdevicetriggerreportnotify,

scefmsisdnlessmosms                                 [85] scefmsisdnlessmosms,

scefcommunicationpatternupdate                      [86] scefcommunicationpatternupdate,

-- eps events, see clause 6.3

-- mme events, see clause 6.3.2.2

mmeattach                                           [87] mmeattach,

mmedetach                                           [88] mmedetach,

mmelocationupdate                                   [89] mmelocationupdate,

mmestartofinterceptionwithepsattachedue             [90] mmestartofinterceptionwithepsattachedue,

mmeunsuccessfulprocedure                            [91] mmeunsuccessfulprocedure,

-- akma key management events, see clause 7.9.1

aanfanchorkeyregister                               [92] aanfanchorkeyregister,

aanfkakmaapplicationkeyget                          [93] aanfkakmaapplicationkeyget,

aanfstartofinterceptwithestablishedakmakeymaterial  [94] aanfstartofinterceptwithestablishedakmakeymaterial,

aanfakmacontextremovalrecord                        [95] aanfakmacontextremovalrecord,

afakmaapplicationkeyrefresh                         [96] afakmaapplicationkeyrefresh,

afstartofinterceptwithestablishedakmaapplicationkey [97] afstartofinterceptwithestablishedakmaapplicationkey,

afauxiliarysecurityparameterestablishment           [98] afauxiliarysecurityparameterestablishment,

afapplicationkeyremoval                             [99] afapplicationkeyremoval,

-- tag 100 is reserved because there is no equivalent n9hrpdusessioninfo in irievent.

-- tag 101 is reserved because there is no equivalent s8hrbearerinfo in irievent.

-- separated location reporting, see clause 7.3.4

separatedlocationreporting                          [102] separatedlocationreporting,

-- stir shaken and rcd/ecnam events, see clause 7.11.3

stirshakensignaturegeneration                       [103] stirshakensignaturegeneration,

stirshakensignaturevalidation                       [104] stirshakensignaturevalidation,

-- ims events, see clause 7.11.4.2

imsmessage                                          [105] imsmessage,

startofinterceptionforactiveimssession              [106] startofinterceptionforactiveimssession,

imsccunavailable                                    [107] imsccunavailable,

-- udm events, see clause 7.2.2

udmlocationinformationresultrecord                  [108] udmlocationinformationresult,

udmueinformationresponse                            [109] udmueinformationresponse,

udmueauthenticationresponse                         [110] udmueauthenticationresponse,

-- amf events, see 6.2.2.2.8

positioninginfotransfer                             [111] amfpositioninginfotransfer,

-- mme events, see clause 6.3.2.2.8

mmepositioninginfotransfer                          [112] mmepositioninginfotransfer,

-- tags 113 to 131 are not used in this version of the specification

-- amf events, see 6.2.2.3, continued from tag 111

amfueconfigurationupdate                            [132] amfueconfigurationupdate

}

Iritargetidentifier ::= sequence

{

identifier                                          [1] targetidentifier,

provenance                                          [2] targetidentifierprovenance optional

}

-- ==============

-- hi3 cc payload

-- ==============

Ccpayload ::= sequence

{

ccpayloadoid         [1] relative-oid,

pdu                  [2] ccpdu

}

Ccpdu ::= choice

{

upfccpdu            [1] upfccpdu,

extendedupfccpdu    [2] extendedupfccpdu,

mmsccpdu            [3] mmsccpdu,

-- in rel-16 (threegpp(4) ts33128(19) r16(16) version9(9)),

-- tag 4 is ptcccpdu and tag 5 is not used.

-- rel-17 or newer decoders should decode tag 4 contents as ptcccpdu if

-- r16 is used in ccpayloadoid.

niddccpdu           [4] niddccpdu,

ptcccpdu            [5] ptcccpdu,

imsccpdu            [6] imsccpdu

}

-- ===========================

-- hi4 li notification payload

-- ===========================

Linotificationpayload ::= sequence

{

linotificationpayloadoid         [1] relative-oid,

notification                     [2] linotificationmessage

}

Linotificationmessage ::= choice

{

linotification      [1] linotification

}

-- =================

-- hr li definitions

-- =================

N9hrpdusessioninfo ::= sequence

{

supi                            [1] supi,

pei                             [2] pei optional,

pdusessionid                    [3] pdusessionid,

location                        [4] location optional,

snssai                          [5] snssai optional,

dnn                             [6] dnn optional,

messagecause                    [7] n9hrmessagecause

}

S8hrbearerinfo ::= sequence

{

imsi                            [1] imsi,

imei                            [2] imei optional,

bearerid                        [3] epsbearerid,

linkedbearerid                  [4] epsbearerid optional,

location                        [5] location optional,

apn                             [6] apn optional,

sgwipaddress                    [7] ipaddress optional,

messagecause                    [8] s8hrmessagecause

}

-- ================

-- hr li parameters

-- ================

N9hrmessagecause ::= enumerated

{

pdusessionestablished(1),

pdusessionmodified(2),

pdusessionreleased(3),

updatedlocationavailable(4),

smfchanged(5),

other(6),

hrlienabled(7)

}

S8hrmessagecause ::= enumerated

{

beareractivated(1),

bearermodified(2),

bearerdeleted(3),

pdndisconnected(4),

updatedlocationavailable(5),

sgwchanged(6),

other(7),

hrlienabled(8)

}

-- ==================

-- 5g nef definitions

-- ==================

-- see clause 7.7.2.1.2 for details of this structure

Nefpdusessionestablishment ::= sequence

{

supi                  [1] supi,

gpsi                  [2] gpsi,

pdusessionid          [3] pdusessionid,

snssai                [4] snssai,

nefid                 [5] nefid,

dnn                   [6] dnn,

rdssupport            [7] rdssupport,

smfid                 [8] smfid,

afid                  [9] afid

}

-- see clause 7.7.2.1.3 for details of this structure

Nefpdusessionmodification ::= sequence

{

supi                         [1] supi,

gpsi                         [2] gpsi,

snssai                       [3] snssai,

initiator                    [4] initiator,

rdssourceportnumber          [5] rdsportnumber optional,

rdsdestinationportnumber     [6] rdsportnumber optional,

applicationid                [7] applicationid optional,

afid                         [8] afid optional,

rdsaction                    [9] rdsaction optional,

serializationformat          [10] serializationformat optional

}

-- see clause 7.7.2.1.4 for details of this structure

Nefpdusessionrelease ::= sequence

{

supi                   [1] supi,

gpsi                   [2] gpsi,

pdusessionid           [3] pdusessionid,

timeoffirstpacket      [4] timestamp optional,

timeoflastpacket       [5] timestamp optional,

uplinkvolume           [6] integer optional,

downlinkvolume         [7] integer optional,

releasecause           [8] nefreleasecause

}

-- see clause 7.7.2.1.5 for details of this structure

Nefunsuccessfulprocedure ::= sequence

{

failurecause                 [1] neffailurecause,

supi                         [2] supi,

gpsi                         [3] gpsi optional,

pdusessionid                 [4] pdusessionid,

dnn                          [5] dnn optional,

snssai                       [6] snssai optional,

rdsdestinationportnumber     [7] rdsportnumber,

applicationid                [8] applicationid,

afid                         [9] afid

}

-- see clause 7.7.2.1.6 for details of this structure

Nefstartofinterceptionwithestablishedpdusession ::= sequence

{

supi               [1] supi,

gpsi               [2] gpsi,

pdusessionid       [3] pdusessionid,

dnn                [4] dnn,

snssai             [5] snssai,

nefid              [6] nefid,

rdssupport         [7] rdssupport,

smfid              [8] smfid,

afid               [9] afid

}

-- see clause 7.7.3.1.1 for details of this structure

Nefdevicetrigger ::= sequence

{

supi                  [1] supi,

gpsi                  [2] gpsi,

triggerid             [3] triggerid,

afid                  [4] afid,

triggerpayload        [5] triggerpayload optional,

validityperiod        [6] integer optional,

prioritydt            [7] prioritydt optional,

sourceportid          [8] portnumber optional,

destinationportid     [9] portnumber optional

}

-- see clause 7.7.3.1.2 for details of this structure

Nefdevicetriggerreplace ::= sequence

{

supi                     [1] supi,

gpsi                     [2] gpsi,

triggerid                [3] triggerid,

afid                     [4] afid,

triggerpayload           [5] triggerpayload optional,

validityperiod           [6] integer optional,

prioritydt               [7] prioritydt optional,

sourceportid             [8] portnumber optional,

destinationportid        [9] portnumber optional

}

-- see clause 7.7.3.1.3 for details of this structure

Nefdevicetriggercancellation ::= sequence

{

supi                  [1] supi,

gpsi                  [2] gpsi,

triggerid             [3] triggerid

}

-- see clause 7.7.3.1.4 for details of this structure

Nefdevicetriggerreportnotify ::= sequence

{

supi                             [1] supi,

gpsi                             [2] gpsi,

triggerid                        [3] triggerid,

devicetriggerdeliveryresult      [4] devicetriggerdeliveryresult

}

-- see clause 7.7.4.1.1 for details of this structure

Nefmsisdnlessmosms ::= sequence

{

supi                      [1] supi,

gpsi                      [2] gpsi,

terminatingsmsparty       [3] afid,

sms                       [4] smstpdudata optional,

sourceport                [5] portnumber optional,

destinationport           [6] portnumber optional

}

-- see clause 7.7.5.1.1 for details of this structure

Nefexpecteduebehaviourupdate ::= sequence

{

gpsi                                  [1] gpsi,

expecteduemovingtrajectory            [2] sequence of umtlocationarea5g optional,

stationaryindication                  [3] stationaryindication optional,

communicationdurationtime             [4] integer optional,

periodictime                          [5] integer optional,

scheduledcommunicationtime            [6] scheduledcommunicationtime optional,

scheduledcommunicationtype            [7] scheduledcommunicationtype optional,

batteryindication                     [8] batteryindication optional,

trafficprofile                        [9] trafficprofile optional,

expectedtimeanddayofweekintrajectory  [10] sequence of umtlocationarea5g optional,

afid                                  [11] afid,

validitytime                          [12] timestamp optional

}

-- ==========================

-- common scef/nef parameters

-- ==========================

Rdssupport ::= boolean

Rdsportnumber ::= integer (0..15)

Rdsaction ::= enumerated

{

reserveport(1),

releaseport(2)

}

Serializationformat ::= enumerated

{

xml(1),

json(2),

cbor(3)

}

Applicationid ::= octet string

Niddccpdu ::= octet string

Triggerid ::= utf8string

Prioritydt ::= enumerated

{

nopriority(1),

priority(2)

}

Triggerpayload ::= octet string

Devicetriggerdeliveryresult ::= enumerated

{

success(1),

unknown(2),

failure(3),

triggered(4),

expired(5),

unconfirmed(6),

replaced(7),

terminate(8)

}

Stationaryindication ::= enumerated

{

stationary(1),

mobile(2)

}

Batteryindication ::= enumerated

{

batteryrecharge(1),

batteryreplace(2),

batterynorecharge(3),

batterynoreplace(4),

nobattery(5)

}

Scheduledcommunicationtime ::= sequence

{

days [1] sequence of daytime

}

Umtlocationarea5g ::= sequence

{

timeofday        [1] daytime,

durationsec      [2] integer,

location         [3] nrlocation

}

Daytime ::= sequence

{

daysofweek       [1] day optional,

timeofdaystart   [2] timestamp optional,

timeofdayend     [3] timestamp optional

}

Day ::= enumerated

{

monday(1),

tuesday(2),

wednesday(3),

thursday(4),

friday(5),

saturday(6),

sunday(7)

}

Trafficprofile ::= enumerated

{

singletransul(1),

singletransdl(2),

dualtransulfirst(3),

dualtransdlfirst(4),

multitrans(5)

}

Scheduledcommunicationtype ::= enumerated

{

downlinkonly(1),

uplinkonly(2),

bidirectional(3)

}

-- =================

-- 5g nef parameters

-- =================

Neffailurecause ::= enumerated

{

userunknown(1),

niddconfigurationnotavailable(2),

contextnotfound(3),

portnotfree(4),

portnotassociatedwithspecifiedapplication(5)

}

Nefreleasecause ::= enumerated

{

smfrelease(1),

dnrelease(2),

udmrelease(3),

chfrelease(4),

localconfigurationpolicy(5),

unknowncause(6)

}

Afid ::= utf8string

Nefid ::= utf8string

-- ==================

-- scef definitions

-- ==================

-- see clause 7.8.2.1.2 for details of this structure

Scefpdnconnectionestablishment ::= sequence

{

imsi                  [1] imsi optional,

msisdn                [2] msisdn optional,

externalidentifier    [3] nai optional,

imei                  [4] imei optional,

epsbearerid           [5] epsbearerid,

scefid                [6] scefid,

apn                   [7] apn,

rdssupport            [8] rdssupport,

scsasid               [9] scsasid

}

-- see clause 7.8.2.1.3 for details of this structure

Scefpdnconnectionupdate ::= sequence

{

imsi                         [1] imsi optional,

msisdn                       [2] msisdn optional,

externalidentifier           [3] nai optional,

initiator                    [4] initiator,

rdssourceportnumber          [5] rdsportnumber optional,

rdsdestinationportnumber     [6] rdsportnumber optional,

applicationid                [7] applicationid optional,

scsasid                      [8] scsasid optional,

rdsaction                    [9] rdsaction optional,

serializationformat          [10] serializationformat optional

}

-- see clause 7.8.2.1.4 for details of this structure

Scefpdnconnectionrelease ::= sequence

{

imsi                   [1] imsi optional,

msisdn                 [2] msisdn optional,

externalidentifier     [3] nai optional,

epsbearerid            [4] epsbearerid,

timeoffirstpacket      [5] timestamp optional,

timeoflastpacket       [6] timestamp optional,

uplinkvolume           [7] integer optional,

downlinkvolume         [8] integer optional,

releasecause           [9] scefreleasecause

}

-- see clause 7.8.2.1.5 for details of this structure

Scefunsuccessfulprocedure ::= sequence

{

failurecause                 [1] sceffailurecause,

imsi                         [2] imsi optional,

msisdn                       [3] msisdn optional,

externalidentifier           [4] nai optional,

epsbearerid                  [5] epsbearerid,

apn                          [6] apn,

rdsdestinationportnumber     [7] rdsportnumber optional,

applicationid                [8] applicationid optional,

scsasid                      [9] scsasid

}

-- see clause 7.8.2.1.6 for details of this structure

Scefstartofinterceptionwithestablishedpdnconnection ::= sequence

{

imsi                  [1] imsi optional,

msisdn                [2] msisdn optional,

externalidentifier    [3] nai optional,

imei                  [4] imei optional,

epsbearerid           [5] epsbearerid,

scefid                [6] scefid,

apn                   [7] apn,

rdssupport            [8] rdssupport,

scsasid               [9] scsasid

}

-- see clause 7.8.3.1.1 for details of this structure

Scefdevicetrigger ::= sequence

{

imsi                  [1] imsi,

msisdn                [2] msisdn,

externalidentifier    [3] nai,

triggerid             [4] triggerid,

scsasid               [5] scsasid optional,

triggerpayload        [6] triggerpayload optional,

validityperiod        [7] integer optional,

prioritydt            [8] prioritydt optional,

sourceportid          [9] portnumber optional,

destinationportid     [10] portnumber optional

}

-- see clause 7.8.3.1.2 for details of this structure

Scefdevicetriggerreplace ::= sequence

{

imsi                     [1] imsi optional,

msisdn                   [2] msisdn optional,

externalidentifier       [3] nai optional,

triggerid                [4] triggerid,

scsasid                  [5] scsasid optional,

triggerpayload           [6] triggerpayload optional,

validityperiod           [7] integer optional,

prioritydt               [8] prioritydt optional,

sourceportid             [9] portnumber optional,

destinationportid        [10] portnumber optional

}

-- see clause 7.8.3.1.3 for details of this structure

Scefdevicetriggercancellation ::= sequence

{

imsi                     [1] imsi optional,

msisdn                   [2] msisdn optional,

externalidentifier       [3] nai optional,

triggerid                [4] triggerid

}

-- see clause 7.8.3.1.4 for details of this structure

Scefdevicetriggerreportnotify ::= sequence

{

imsi                             [1] imsi optional,

msisdn                           [2] msisdn optional,

externalidentifier               [3] nai optional,

triggerid                        [4] triggerid,

devicetriggerdeliveryresult      [5] devicetriggerdeliveryresult

}

-- see clause 7.8.4.1.1 for details of this structure

Scefmsisdnlessmosms ::= sequence

{

imsi                      [1] imsi optional,

msisdn                    [2] msisdn optional,

externalidentifie         [3] nai optional,

terminatingsmsparty       [4] scsasid,

sms                       [5] smstpdudata optional,

sourceport                [6] portnumber optional,

destinationport           [7] portnumber optional

}

-- see clause 7.8.5.1.1 for details of this structure

Scefcommunicationpatternupdate ::= sequence

{

msisdn                                [1] msisdn optional,

externalidentifier                    [2] nai optional,

periodiccommunicationindicator        [3] periodiccommunicationindicator optional,

communicationdurationtime             [4] integer optional,

periodictime                          [5] integer optional,

scheduledcommunicationtime            [6] scheduledcommunicationtime optional,

scheduledcommunicationtype            [7] scheduledcommunicationtype optional,

stationaryindication                  [8] stationaryindication optional,

batteryindication                     [9] batteryindication optional,

trafficprofile                        [10] trafficprofile optional,

expecteduemovingtrajectory            [11] sequence of umtlocationarea5g optional,

scsasid                               [13] scsasid,

validitytime                          [14] timestamp optional

}

-- =================

-- scef parameters

-- =================

Sceffailurecause ::= enumerated

{

userunknown(1),

niddconfigurationnotavailable(2),

invalidepsbearer(3),

operationnotallowed(4),

portnotfree(5),

portnotassociatedwithspecifiedapplication(6)

}

Scefreleasecause ::= enumerated

{

mmerelease(1),

dnrelease(2),

hssrelease(3),

localconfigurationpolicy(4),

unknowncause(5)

}

Scsasid ::= utf8string

Scefid ::= utf8string

Periodiccommunicationindicator ::= enumerated

{

periodic(1),

nonperiodic(2)

}

Epsbearerid ::= integer (0..255)

Apn ::= utf8string

-- =======================

-- akma aanf definitions

-- =======================

Aanfanchorkeyregister ::= sequence

{

akid                  [1] nai,

supi                  [2] supi,

kakma                 [3] kakma optional

}

Aanfkakmaapplicationkeyget ::= sequence

{

type                  [1] keygettype,

akid                  [2] nai,

keyinfo               [3] afkeyinfo

}

Aanfstartofinterceptwithestablishedakmakeymaterial ::= sequence

{

akid                  [1] nai,

kakma                 [2] kakma optional,

afkeylist             [3] sequence of afkeyinfo optional

}

Aanfakmacontextremovalrecord ::= sequence

{

akid                  [1] nai,

nfid                  [2] nfid

}

-- ======================

-- akma common parameters

-- ======================

Fqdn ::= utf8string

Nfid ::= utf8string

Uaprotocolid ::= octet string (size(5))

Akmaafid ::= sequence

{

affqdn                [1] fqdn,

uaprotocolid          [2] uaprotocolid

}

Uastarparams ::= choice

{

tls12                 [1] tls12uastarparams,

generic               [2] genericuastarparams

}

Genericuastarparams ::= sequence

{

genericclientparams [1] octet string,

genericserverparams [2] octet string

}

-- ===========================================

-- specific uastarparmas for tls 1.2 (rfc5246)

-- ===========================================

Tlsciphertype ::= enumerated

{

stream(1),

block(2),

aead(3)

}

Tlscompressionalgorithm ::= enumerated

{

null(1),

deflate(2)

}

Tlsprfalgorithm ::= enumerated

{

rfc5246(1)

}

Tlsciphersuite ::= sequence (size(2)) of integer (0..255)

Tls12uastarparams ::= sequence

{

premastersecret       [1] octet string (size(6)) optional,

mastersecret          [2] octet string (size(6)),

prfalgorithm          [3] tlsprfalgorithm,

ciphersuite           [4] tlsciphersuite,

ciphertype            [5] tlsciphertype,

enckeylength          [6] integer (0..255),

blocklength           [7] integer (0..255),

fixedivlength         [8] integer (0..255),

recordivlength        [9] integer (0..255),

maclength             [10] integer (0..255),

mackeylength          [11] integer (0..255),

compressionalgorithm  [12] tlscompressionalgorithm,

clientrandom          [13] octet string (size(4)),

serverrandom          [14] octet string (size(4)),

clientsequencenumber  [15] integer,

serversequencenumber  [16] integer,

sessionid             [17] octet string (size(0..32)),

tlsextensions         [18] octet string (size(0..65535))

}

Kaf ::= octet string

Kakma ::= octet string

-- ====================

-- akma aanf parameters

-- ====================

Keygettype ::= enumerated

{

internal(1),

external(2)

}

Afkeyinfo ::= sequence

{

afid                 [1] akmaafid,

kaf                  [2] kaf,

kafexptime           [3] kafexpirytime

}

-- =======================

-- akma af definitions

-- =======================

Afakmaapplicationkeyrefresh ::= sequence

{

afid                  [1] afid,

akid                  [2] nai,

kaf                   [3] kaf,

uastarparams          [4] uastarparams optional

}

Afstartofinterceptwithestablishedakmaapplicationkey ::= sequence

{

afid                  [1] fqdn,

akid                  [2] nai,

kafparamlist          [3] sequence of afsecurityparams

}

Afauxiliarysecurityparameterestablishment ::= sequence

{

afsecurityparams      [1] afsecurityparams

}

Afsecurityparams ::= sequence

{

afid                  [1] afid,

akid                  [2] nai,

kaf                   [3] kaf,

uastarparams          [4] uastarparams

}

Afapplicationkeyremoval ::= sequence

{

afid                  [1] afid,

akid                  [2] nai,

removalcause          [3] afkeyremovalcause

}

-- ===================

-- akma af parameters

-- ===================

Kafparams ::= sequence

{

akid                 [1] nai,

kaf                  [2] kaf,

kafexptime           [3] kafexpirytime,

uastarparams         [4] uastarparams

}

Kafexpirytime ::= generalizedtime

Afkeyremovalcause ::= enumerated

{

unknown(1),

keyexpiry(2),

applicationspecific(3)

}

-- ==================

-- 5g amf definitions

-- ==================

-- see clause 6.2.2.2.2 for details of this structure

Amfregistration ::= sequence

{

registrationtype            [1] amfregistrationtype,

registrationresult          [2] amfregistrationresult,

slice                       [3] slice optional,

supi                        [4] supi,

suci                        [5] suci optional,

pei                         [6] pei optional,

gpsi                        [7] gpsi optional,

guti                        [8] fivegguti,

location                    [9] location optional,

non3gppaccessendpoint       [10] ueendpointaddress optional,

fivegstailist               [11] tailist optional,

smsovernasindicator         [12] smsovernasindicator optional,

oldguti                     [13] eps5gguti optional,

emm5gregstatus              [14] emm5gmmstatus optional,

nonimeisvpei                [15] nonimeisvpei optional,

macrestindicator            [16] macrestrictionindicator optional,

pagingrestrictionindicator  [17] pagingrestrictionindicator optional

}

-- see clause 6.2.2.2.3 for details of this structure

Amfderegistration ::= sequence

{

deregistrationdirection     [1] amfdirection,

accesstype                  [2] accesstype,

supi                        [3] supi optional,

suci                        [4] suci optional,

pei                         [5] pei optional,

gpsi                        [6] gpsi optional,

guti                        [7] fivegguti optional,

cause                       [8] fivegmmcause optional,

location                    [9] location optional,

switchoffindicator          [10] switchoffindicator optional,

reregrequiredindicator      [11] reregrequiredindicator optional

}

-- see clause 6.2.2.2.4 for details of this structure

Amflocationupdate ::= sequence

{

supi                        [1] supi,

suci                        [2] suci optional,

pei                         [3] pei optional,

gpsi                        [4] gpsi optional,

guti                        [5] fivegguti optional,

location                    [6] location,

smsovernasindicator         [7] smsovernasindicator optional,

oldguti                     [8] eps5gguti optional

}

-- see clause 6.2.2.2.5 for details of this structure

Amfstartofinterceptionwithregisteredue ::= sequence

{

registrationresult          [1] amfregistrationresult,

registrationtype            [2] amfregistrationtype optional,

slice                       [3] slice optional,

supi                        [4] supi,

suci                        [5] suci optional,

pei                         [6] pei optional,

gpsi                        [7] gpsi optional,

guti                        [8] fivegguti,

location                    [9] location optional,

non3gppaccessendpoint       [10] ueendpointaddress optional,

timeofregistration          [11] timestamp optional,

fivegstailist               [12] tailist optional,

smsovernasindicator         [13] smsovernasindicator optional,

oldguti                     [14] eps5gguti optional,

emm5gregstatus              [15] emm5gmmstatus optional

}

-- see clause 6.2.2.2.6 for details of this structure

Amfunsuccessfulprocedure ::= sequence

{

failedproceduretype         [1] amffailedproceduretype,

failurecause                [2] amffailurecause,

requestedslice              [3] nssai optional,

supi                        [4] supi optional,

suci                        [5] suci optional,

pei                         [6] pei optional,

gpsi                        [7] gpsi optional,

guti                        [8] fivegguti optional,

location                    [9] location optional

}

-- see clause 6.2.2.2.8 on for details of this structure

Amfpositioninginfotransfer ::= sequence

{

supi                        [1] supi,

suci                        [2] suci optional,

pei                         [3] pei optional,

gpsi                        [4] gpsi optional,

guti                        [5] fivegguti optional,

nrppamessage                [6] octet string optional,

lppmessage                  [7] octet string optional,

lcscorrelationid            [8] utf8string (size(1..255))

}

--see clause 6.2.2.2.9 on for details of this structure

Amfueconfigurationupdate ::= sequence

{

useridentifiers     [1] useridentifiers,

guti                [2] guti,

oldguti             [3] eps5gguti optional,

fivegstailist       [4] tailist optional,

slice               [5] slice optional,

servicearealist     [6] servicearealist optional,

registrationresult  [7] amfregistrationresult optional,

smsovernasindicator [8] smsovernasindicator optional

}

-- =================

-- 5g amf parameters

-- =================

Amfid ::= sequence

{

amfregionid [1] amfregionid,

amfsetid    [2] amfsetid,

amfpointer  [3] amfpointer

}

Amfdirection ::= enumerated

{

networkinitiated(1),

ueinitiated(2)

}

Amffailedproceduretype ::= enumerated

{

registration(1),

sms(2),

pdusessionestablishment(3)

}

Amffailurecause ::= choice

{

fivegmmcause        [1] fivegmmcause,

fivegsmcause        [2] fivegsmcause

}

Amfpointer ::= integer (0..63)

Amfregistrationresult ::= enumerated

{

threegppaccess(1),

nonthreegppaccess(2),

threegppandnonthreegppaccess(3)

}

Amfregionid ::= integer (0..255)

Amfregistrationtype ::= enumerated

{

initial(1),

mobility(2),

periodic(3),

emergency(4),

snpnonboarding(5),

disastermobility(6),

disasterinitial(7)

}

Amfsetid ::= integer (0..1023)

-- ts 24.501 [13], clause 9.11.3.49

Servicearealist ::= octet string (size(4..112))

-- ==================

-- 5g smf definitions

-- ==================

-- see clause 6.2.3.2.2 for details of this structure

Smfpdusessionestablishment ::= sequence

{

supi                          [1] supi optional,

supiunauthenticated           [2] supiunauthenticatedindication optional,

pei                           [3] pei optional,

gpsi                          [4] gpsi optional,

pdusessionid                  [5] pdusessionid,

gtptunnelid                   [6] fteid,

pdusessiontype                [7] pdusessiontype,

snssai                        [8] snssai optional,

ueendpoint                    [9] sequence of ueendpointaddress optional,

non3gppaccessendpoint         [10] ueendpointaddress optional,

location                      [11] location optional,

dnn                           [12] dnn,

amfid                         [13] amfid optional,

hsmfuri                       [14] hsmfuri optional,

requesttype                   [15] fivegsmrequesttype,

accesstype                    [16] accesstype optional,

rattype                       [17] rattype optional,

smpdudnrequest                [18] smpdudnrequest optional,

ueepspdnconnection            [19] ueepspdnconnection optional,

eps5gscomboinfo               [20] eps5gscomboinfo optional,

selecteddnn                   [21] dnn optional,

servingnetwork                [22] smfservingnetwork optional,

oldpdusessionid               [23] pdusessionid optional,

handoverstate                 [24] handoverstate optional,

gtptunnelinfo                 [25] gtptunnelinfo optional,

pccrules                      [26] pccruleset optional,

epspdnconnectionestablishment [27] epspdnconnectionestablishment optional

}

-- see clause 6.2.3.2.3 for details of this structure

Smfpdusessionmodification ::= sequence

{

supi                        [1] supi optional,

supiunauthenticated         [2] supiunauthenticatedindication optional,

pei                         [3] pei optional,

gpsi                        [4] gpsi optional,

snssai                      [5] snssai optional,

non3gppaccessendpoint       [6] ueendpointaddress optional,

location                    [7] location optional,

requesttype                 [8] fivegsmrequesttype,

accesstype                  [9] accesstype optional,

rattype                     [10] rattype optional,

pdusessionid                [11] pdusessionid optional,

eps5gscomboinfo             [12] eps5gscomboinfo optional,

ueendpoint                  [13] ueendpointaddress optional,

servingnetwork              [14] smfservingnetwork optional,

handoverstate               [15] handoverstate optional,

gtptunnelinfo               [16] gtptunnelinfo optional,

pccrules                    [17] pccruleset optional,

epspdnconnectionmodification[18] epspdnconnectionmodification optional,

uppathchange                [19] uppathchange optional,

pfddataforapp               [20] pfddataforapp optional

}

-- see clause 6.2.3.2.4 for details of this structure

Smfpdusessionrelease ::= sequence

{

supi                        [1] supi,

pei                         [2] pei optional,

gpsi                        [3] gpsi optional,

pdusessionid                [4] pdusessionid,

timeoffirstpacket           [5] timestamp optional,

timeoflastpacket            [6] timestamp optional,

uplinkvolume                [7] integer optional,

downlinkvolume              [8] integer optional,

location                    [9] location optional,

cause                       [10] smferrorcodes optional,

eps5gscomboinfo             [11] eps5gscomboinfo optional,

ngapcause                   [12] ngapcauseint optional,

fivegmmcause                [13] fivegmmcause optional,

pccruleids                  [14] pccruleidset optional,

epspdnconnectionrelease     [15] epspdnconnectionrelease optional

}

-- see clause 6.2.3.2.5 for details of this structure

Smfstartofinterceptionwithestablishedpdusession ::= sequence

{

supi                                               [1] supi optional,

supiunauthenticated                                [2] supiunauthenticatedindication optional,

pei                                                [3] pei optional,

gpsi                                               [4] gpsi optional,

pdusessionid                                       [5] pdusessionid,

gtptunnelid                                        [6] fteid,

pdusessiontype                                     [7] pdusessiontype,

snssai                                             [8] snssai optional,

ueendpoint                                         [9] sequence of ueendpointaddress,

non3gppaccessendpoint                              [10] ueendpointaddress optional,

location                                           [11] location optional,

dnn                                                [12] dnn,

amfid                                              [13] amfid optional,

hsmfuri                                            [14] hsmfuri optional,

requesttype                                        [15] fivegsmrequesttype,

accesstype                                         [16] accesstype optional,

rattype                                            [17] rattype optional,

smpdudnrequest                                     [18] smpdudnrequest optional,

timeofsessionestablishment                         [19] timestamp optional,

eps5gscomboinfo                                    [20] eps5gscomboinfo optional,

ueepspdnconnection                                 [21] ueepspdnconnection optional,

servingnetwork                                     [22] smfservingnetwork optional,

gtptunnelinfo                                      [23] gtptunnelinfo optional,

pccrules                                           [24] pccruleset optional,

epsstartofinterceptionwithestablishedpdnconnection [25] epsstartofinterceptionwithestablishedpdnconnection optional,

pfddataforapps                                     [26] pfddataforapps optional

}

-- see clause 6.2.3.2.6 for details of this structure

Smfunsuccessfulprocedure ::= sequence

{

failedproceduretype         [1] smffailedproceduretype,

failurecause                [2] fivegsmcause,

initiator                   [3] initiator,

requestedslice              [4] nssai optional,

supi                        [5] supi optional,

supiunauthenticated         [6] supiunauthenticatedindication optional,

pei                         [7] pei optional,

gpsi                        [8] gpsi optional,

pdusessionid                [9] pdusessionid optional,

ueendpoint                  [10] sequence of ueendpointaddress optional,

non3gppaccessendpoint       [11] ueendpointaddress optional,

dnn                         [12] dnn optional,

amfid                       [13] amfid optional,

hsmfuri                     [14] hsmfuri optional,

requesttype                 [15] fivegsmrequesttype optional,

accesstype                  [16] accesstype optional,

rattype                     [17] rattype optional,

smpdudnrequest              [18] smpdudnrequest optional,

location                    [19] location optional

}

-- see clause 6.2.3.2.8 for details of this structure

Smfpdutomapdusessionmodification ::= sequence

{

supi                         [1] supi optional,

supiunauthenticated          [2] supiunauthenticatedindication optional,

pei                          [3] pei optional,

gpsi                         [4] gpsi optional,

snssai                       [5] snssai optional,

non3gppaccessendpoint        [6] ueendpointaddress optional,

location                     [7] location optional,

requesttype                  [8] fivegsmrequesttype,

accesstype                   [9] accesstype optional,

rattype                      [10] rattype optional,

pdusessionid                 [11] pdusessionid,

requestindication            [12] requestindication,

atssscontainer               [13] atssscontainer,

ueendpoint                   [14] ueendpointaddress optional,

servingnetwork               [15] smfservingnetwork optional,

handoverstate                [16] handoverstate optional,

gtptunnelinfo                [17] gtptunnelinfo optional,

epspdnconnectionmodification [18] epspdnconnectionmodification optional

}

-- see clause 6.2.3.2.7.1 for details of this structure

Smfmapdusessionestablishment ::= sequence

{

supi                          [1] supi optional,

supiunauthenticated           [2] supiunauthenticatedindication optional,

pei                           [3] pei optional,

gpsi                          [4] gpsi optional,

pdusessionid                  [5] pdusessionid,

pdusessiontype                [6] pdusessiontype,

accessinfo                    [7] sequence of accessinfo,

snssai                        [8] snssai optional,

ueendpoint                    [9] sequence of ueendpointaddress optional,

location                      [10] location optional,

dnn                           [11] dnn,

amfid                         [12] amfid optional,

hsmfuri                       [13] hsmfuri optional,

requesttype                   [14] fivegsmrequesttype,

smpdudnrequest                [15] smpdudnrequest optional,

servingnetwork                [16] smfservingnetwork,

oldpdusessionid               [17] pdusessionid optional,

maupgradeindication           [18] smfmaupgradeindication optional,

epspdncnxinfo                 [19] smfepspdncnxinfo optional,

maacceptedindication          [20] smfmaacceptedindication,

atssscontainer                [21] atssscontainer optional,

ueepspdnconnection            [22] ueepspdnconnection optional,

eps5gscomboinfo               [23] eps5gscomboinfo optional,

selecteddnn                   [24] dnn optional,

handoverstate                 [25] handoverstate optional,

pccrules                      [26] pccruleset optional,

epspdnconnectionestablishment [27] epspdnconnectionestablishment optional

}

-- see clause 6.2.3.2.7.2 for details of this structure

Smfmapdusessionmodification ::= sequence

{

supi                         [1] supi optional,

supiunauthenticated          [2] supiunauthenticatedindication optional,

pei                          [3] pei optional,

gpsi                         [4] gpsi optional,

pdusessionid                 [5] pdusessionid,

accessinfo                   [6] sequence of accessinfo optional,

snssai                       [7] snssai optional,

location                     [8] location optional,

requesttype                  [9] fivegsmrequesttype optional,

servingnetwork               [10] smfservingnetwork,

oldpdusessionid              [11] pdusessionid optional,

maupgradeindication          [12] smfmaupgradeindication optional,

epspdncnxinfo                [13] smfepspdncnxinfo optional,

maacceptedindication         [14] smfmaacceptedindication,

atssscontainer               [15] atssscontainer optional,

ueepspdnconnection           [16] ueepspdnconnection optional,

eps5gscomboinfo              [17] eps5gscomboinfo optional,

handoverstate                [18] handoverstate optional,

pccrules                     [19] pccruleset optional,

uppathchange                 [20] uppathchange optional,

pfddataforapp                [21] pfddataforapp optional,

epspdnconnectionmodification [22] epspdnconnectionmodification optional

}

-- see clause 6.2.3.2.7.3 for details of this structure

Smfmapdusessionrelease ::= sequence

{

supi                        [1] supi,

pei                         [2] pei optional,

gpsi                        [3] gpsi optional,

pdusessionid                [4] pdusessionid,

timeoffirstpacket           [5] timestamp optional,

timeoflastpacket            [6] timestamp optional,

uplinkvolume                [7] integer optional,

downlinkvolume              [8] integer optional,

location                    [9] location optional,

cause                       [10] smferrorcodes optional,

ngapcause                   [11] ngapcauseint optional,

fivegmmcause                [12] fivegmmcause optional,

pccruleids                  [13] pccruleidset optional,

epspdnconnectionrelease     [14] epspdnconnectionrelease optional

}

-- see clause 6.2.3.2.7.4 for details of this structure

Smfstartofinterceptionwithestablishedmapdusession ::= sequence

{

supi                                               [1] supi optional,

supiunauthenticated                                [2] supiunauthenticatedindication optional,

pei                                                [3] pei optional,

gpsi                                               [4] gpsi optional,

pdusessionid                                       [5] pdusessionid,

pdusessiontype                                     [6] pdusessiontype,

accessinfo                                         [7] sequence of accessinfo,

snssai                                             [8] snssai optional,

ueendpoint                                         [9] sequence of ueendpointaddress optional,

location                                           [10] location optional,

dnn                                                [11] dnn,

amfid                                              [12] amfid optional,

hsmfuri                                            [13] hsmfuri optional,

requesttype                                        [14] fivegsmrequesttype optional,

smpdudnrequest                                     [15] smpdudnrequest optional,

servingnetwork                                     [16] smfservingnetwork,

oldpdusessionid                                    [17] pdusessionid optional,

maupgradeindication                                [18] smfmaupgradeindication optional,

epspdncnxinfo                                      [19] smfepspdncnxinfo optional,

maacceptedindication                               [20] smfmaacceptedindication,

atssscontainer                                     [21] atssscontainer optional,

eps5gscomboinfo                                    [22] eps5gscomboinfo optional,

ueepspdnconnection                                 [23] ueepspdnconnection optional,

pccrules                                           [24] pccruleset optional,

pfddataforapps                                     [25] pfddataforapps optional,

epsstartofinterceptionwithestablishedpdnconnection [26] epsstartofinterceptionwithestablishedpdnconnection optional

}

-- see clause 6.2.3.2.7.5 for details of this structure

Smfmaunsuccessfulprocedure ::= sequence

{

failedproceduretype         [1] smffailedproceduretype,

failurecause                [2] fivegsmcause,

requestedslice              [3] nssai optional,

initiator                   [4] initiator,

supi                        [5] supi optional,

supiunauthenticated         [6] supiunauthenticatedindication optional,

pei                         [7] pei optional,

gpsi                        [8] gpsi optional,

pdusessionid                [9] pdusessionid optional,

accessinfo                  [10] sequence of accessinfo,

ueendpoint                  [11] sequence of ueendpointaddress optional,

location                    [12] location optional,

dnn                         [13] dnn optional,

amfid                       [14] amfid optional,

hsmfuri                     [15] hsmfuri optional,

requesttype                 [16] fivegsmrequesttype optional,

smpdudnrequest              [17] smpdudnrequest optional

}

-- =================

-- 5g smf parameters

-- =================

Smfid ::= utf8string

Smffailedproceduretype ::= enumerated

{

pdusessionestablishment(1),

pdusessionmodification(2),

pdusessionrelease(3)

}

Smfservingnetwork ::= sequence

{

plmnid  [1] plmnid,

nid     [2] nid optional

}

Accessinfo ::= sequence

{

accesstype            [1] accesstype,

rattype               [2] rattype optional,

gtptunnelid           [3] fteid,

non3gppaccessendpoint [4] ueendpointaddress optional,

establishmentstatus   [5] establishmentstatus,

antypetoreactivate    [6] accesstype optional,

gtptunnelinfo         [7] gtptunnelinfo optional

}

-- see clause 6.1.2 of ts 24.193[44] for the details of the atsss container contents.

Atssscontainer ::= octet string

Dlrantunnelinformation ::= sequence

{

dlqosflowtunnelinformation                    [1] qosflowtunnelinformation optional,

additionaldlqosflowtunnelinformation          [2] qosflowtunnelinformationlist optional,

redundantdlqosflowtunnelinformation           [3] qosflowtunnelinformationlist optional,

additionalredundantdlqosflowtunnelinformation [4] qosflowtunnelinformationlist optional

}

Establishmentstatus ::= enumerated

{

established(0),

released(1)

}

Fivegsgtptunnels ::= sequence

{

ulnguuptunnelinformation           [1] fteid optional,

additionalulnguuptunnelinformation [2] fteidlist optional,

dlrantunnelinformation             [3] dlrantunnelinformation optional

}

Fiveqi ::= integer (0..255)

Handoverstate ::= enumerated

{

none(1),

preparing(2),

prepared(3),

completed(4),

cancelled(5)

}

Ngapcauseint ::= sequence

{

group [1] ngapcausegroupint,

value [2] ngapcausevalueint

}

-- derived as described in ts 29.571 [17] clause 5.4.4.12

Ngapcausegroupint ::= integer

Ngapcausevalueint ::= integer

Smfmaupgradeindication ::= boolean

-- given in yaml encoding as defined in clause 6.1.6.2.31 of ts 29.502[16]

Smfepspdncnxinfo ::= utf8string

Smfmaacceptedindication ::= boolean

-- see clause 6.1.6.3.8 of ts 29.502[16] for the details of this structure.

Smferrorcodes ::= utf8string

-- see clause 6.1.6.3.2 of ts 29.502[16] for details of this structure.

Ueepspdnconnection ::= octet string

-- see clause 6.1.6.3.6 of ts 29.502[16] for the details of this structure.

Requestindication ::= enumerated

{

uereqpdusesmod(0),

uereqpdusesrel(1),

pdusesmob(2),

nwreqpdusesauth(3),

nwreqpdusesmod(4),

nwreqpdusesrel(5),

ebiassignmentreq(6),

reldueto5ganrequest(7)

}

Qosflowtunnelinformation ::= sequence

{

uptunnelinformation   [1] fteid,

associatedqosflowlist [2] qosflowlists

}

Qosflowtunnelinformationlist ::= sequence of qosflowtunnelinformation

Qosflowdescription ::= octet string

Qosflowlists ::= sequence of qosflowlist

Qosflowlist ::= sequence

{

qfi                      [1] qfi,

qosrules                 [2] qosrules optional,

ebi                      [3] epsbearerid optional,

qosflowdescription       [4] qosflowdescription optional,

qosflowprofile           [5] qosflowprofile optional,

associatedantype         [6] accesstype optional,

defaultqosruleindication [7] boolean optional

}

Qosflowprofile ::= sequence

{

fiveqi [1] fiveqi

}

Qosrules ::= octet string

-- see clauses 5.6.2.6-1 and 5.6.2.9-1 of ts 29.512 [89], clause table 5.6.2.5-1 of ts 29.508 [90] for the details of this structure

Pccrule ::= sequence

{

pccruleid                     [1] pccruleid optional,

appid                         [2] utf8string optional,

flowinfos                     [3] flowinformationset optional,

appreloc                      [4] boolean optional,

simconnind                    [5] boolean optional,

simconnterm                   [6] integer optional,

maxalloweduplat               [7] integer optional,

trafficroutes                 [8] routetolocationset,

trafficsteeringpoliddl        [9] utf8string optional,

trafficsteeringpolidul        [10] utf8string optional,

sourcednai                    [11] dnai optional,

targetdnai                    [12] dnai optional,

dnaichangetype                [13] dnaichangetype optional,

sourceueipaddr                [14] ipaddress optional,

targetueipaddr                [15] ipaddress optional,

sourcetrafficrouting          [16] routetolocation optional,

targettrafficrouting          [17] routetolocation optional,

easipreplaceinfos             [18] easipreplaceinfos optional

}

--see clause table 5.6.2.5-1 of ts 29.508 [90] for the details of this structure.

Uppathchange ::= sequence

{

sourcednai                    [1] dnai optional,

targetdnai                    [2] dnai optional,

dnaichangetype                [3] dnaichangetype optional,

sourceueipaddr                [4] ipaddress optional,

targetueipaddr                [5] ipaddress optional,

sourcetrafficrouting          [6] routetolocation optional,

targettrafficrouting          [7] routetolocation optional,

macaddress                    [8] macaddress optional

}

-- see table 5.6.2.14-1 of ts 29.512 [89]

Pccruleid ::= utf8string

Pccruleset ::= set of pccrule

Pccruleidset ::= set of pccruleid

Flowinformationset ::= set of flowinformation

Routetolocationset ::= set of routetolocation

-- see table 5.6.2.14 of ts 29.512 [89]

Flowinformation ::= sequence

{

flowdescription    [1] flowdescription optional,

ethflowdescription [2] ethflowdescription optional,

tostrafficclass    [3] octet string (size(2)) optional,

spi                [4] octet string (size(4)) optional,

flowlabel          [5] octet string (size(3)) optional,

flowdirection      [6] flowdirection optional

}

-- see table 5.6.2.14 of ts 29.512 [89]

Flowdescription ::= sequence

{

sourceipaddress       [1] ipaddressorrangeorany,

destinationipaddress  [2] ipaddressorrangeorany,

sourceportnumber      [3] portnumber optional,

destinationportnumber [4] portnumber optional,

protocol              [5] nextlayerprotocolorany

}

Ipaddressorrangeorany ::= choice

{

ipaddress      [1] ipaddress,

ipaddressrange [2] ipmask,

anyipaddress   [3] anyipaddress

}

Ipmask ::= sequence

{

fromipaddress [1] ipaddress,

toipaddress   [2] ipaddress

}

Anyipaddress ::= enumerated

{

any(1)

}

Nextlayerprotocolorany ::= choice

{

nextlayerprotocol    [1] nextlayerprotocol,

anynextlayerprotocol [2] anynextlayerprotocol

}

Anynextlayerprotocol ::= enumerated

{

ip(1)

}

-- see table 5.6.2.17-1 of ts 29.514 [91]

Ethflowdescription ::= sequence

{

destmacaddress    [1] macaddress optional,

ethtype           [2] octet string (size(2)),

fdesc             [3] flowdescription optional,

fdir              [4] fdir optional,

sourcemacaddress  [5] macaddress optional,

vlantags          [6] set of vlantag,

srcmacaddrend     [7] macaddress optional,

destmacaddrend    [8] macaddress optional

}

-- see table 5.6.2.17-1 of ts 29.514 [91]

Fdir ::= enumerated

{

downlink(1)

}

-- see table 5.6.2.17-1 of ts 29.514 [91]

Vlantag ::= sequence

{

priority [1] bit string (size(3)),

cfi      [2] bit string (size(1)),

vlanid   [3] bit string (size(12))

}

-- see table 5.6.2.14 of ts 29.512 [89]

Flowdirection ::= enumerated

{

downlinkonly(1),

uplinkonly(2),

dowlinkanduplink(3)

}

-- see table 5.4.2.1 of ts 29.571 [17]

Dnaichangetype ::= enumerated

{

early(1),

earlyandlate(2),

late(3)

}

-- see table 5.6.2.15 of ts 29.571 [17]

Routetolocation ::= sequence

{

dnai            [1] dnai,

routeinfo       [2] routeinfo

}

-- see table 5.4.2.1 of ts 29.571 [17]

Dnai ::= utf8string

-- see table 5.4.4.16 of ts 29.571 [17]

Routeinfo ::= sequence

{

ipaddresstunnelendpoint       [1] ipaddress,

udpportnumbertunnelendpoint   [2] portnumber

}

-- see clause 4.1.4.2 of ts 29.512 [89]

Easipreplaceinfos ::= sequence

{

sourceeasaddress [1] easserveraddress,

targeteasaddress [2] easserveraddress

}

-- see clause 4.1.4.2 of ts 29.512 [89]

Easserveraddress ::= sequence

{

ipaddress        [1]  ipaddress,

port             [2]  portnumber

}

-- ================================

-- pgw-c + smf pdnconnection events

-- ================================

Epspdnconnectionestablishment ::= sequence

{

epssubscriberids                   [1] epssubscriberids,

imsiunauthenticated                [2] imsiunauthenticatedindication optional,

defaultbearerid                    [3] epsbearerid,

gtptunnelinfo                      [4] gtptunnelinfo optional,

pdnconnectiontype                  [5] pdnconnectiontype,

ueendpoints                        [6] sequence of ueendpointaddress optional,

non3gppaccessendpoint              [7] ueendpointaddress optional,

location                           [8] location optional,

additionallocation                 [9] location optional,

apn                                [10] apn,

requesttype                        [11] epspdnconnectionrequesttype optional,

accesstype                         [12] accesstype optional,

rattype                            [13] rattype optional,

protocolconfigurationoptions       [14] pdnprotocolconfigurationoptions optional,

servingnetwork                     [15] smfservingnetwork optional,

smpdudnrequest                     [16] smpdudnrequest optional,

bearercontextscreated              [17] sequence of epsbearercontextcreated,

bearercontextsmarkedforremoval     [18] sequence of epsbearercontextforremoval optional,

indicationflags                    [19] pdnconnectionindicationflags optional,

handoverindication                 [20] pdnhandoverindication optional,

nbifomsupport                      [21] pdnnbifomsupport optional,

fivegsinterworkinginfo             [22] fivegsinterworkinginfo optional,

csrmfi                             [23] csrmfi optional,

restorationofpdnconnectionssupport [24] restorationofpdnconnectionssupport optional,

pgwchangeindication                [25] pgwchangeindication optional,

pgwrnsi                            [26] pgwrnsi optional

}

Epspdnconnectionmodification ::= sequence

{

epssubscriberids                   [1] epssubscriberids,

imsiunauthenticated                [2] imsiunauthenticatedindication optional,

defaultbearerid                    [3] epsbearerid,

gtptunnelinfo                      [4] gtptunnelinfo optional,

pdnconnectiontype                  [5] pdnconnectiontype,

ueendpoints                        [6] sequence of ueendpointaddress optional,

non3gppaccessendpoint              [7] ueendpointaddress optional,

location                           [8] location optional,

additionallocation                 [9] location optional,

apn                                [10] apn,

requesttype                        [11] epspdnconnectionrequesttype optional,

accesstype                         [12] accesstype optional,

rattype                            [13] rattype optional,

protocolconfigurationoptions       [14] pdnprotocolconfigurationoptions optional,

servingnetwork                     [15] smfservingnetwork optional,

smpdudnrequest                     [16] smpdudnrequest optional,

bearercontextscreated              [17] sequence of epsbearercontextcreated optional,

bearerconcextsmodified             [18] sequence of epsbearercontextmodified,

bearercontextsmarkedforremoval     [19] sequence of epsbearercontextforremoval optional,

bearersdeleted                     [20] sequence of epsbearersdeleted optional,

indicationflags                    [21] pdnconnectionindicationflags optional,

handoverindication                 [22] pdnhandoverindication optional,

nbifomsupport                      [23] pdnnbifomsupport optional,

fivegsinterworkinginfo             [24] fivegsinterworkinginfo optional,

csrmfi                             [25] csrmfi optional,

restorationofpdnconnectionssupport [26] restorationofpdnconnectionssupport optional,

pgwchangeindication                [27] pgwchangeindication optional,

pgwrnsi                            [28] pgwrnsi optional

}

Epspdnconnectionrelease ::= sequence

{

epssubscriberids    [1] epssubscriberids,

imsiunauthenticated [2] imsiunauthenticatedindication optional,

defaultbearerid     [3] epsbearerid,

location            [4] location optional,

gtptunnelinfo       [5] gtptunnelinfo optional,

rannascause         [6] epsrannascause optional,

pdnconnectiontype   [7] pdnconnectiontype,

indicationflags     [8] pdnconnectionindicationflags optional,

scopeindication     [9] epspdnconnectionreleasescopeindication optional,

bearersdeleted      [10] sequence of epsbearersdeleted optional

}

Epsstartofinterceptionwithestablishedpdnconnection ::= sequence

{

epssubscriberids                   [1] epssubscriberids,

imsiunauthenticated                [2] imsiunauthenticatedindication optional,

defaultbearerid                    [3] epsbearerid,

gtptunnelinfo                      [4] gtptunnelinfo optional,

pdnconnectiontype                  [5] pdnconnectiontype,

ueendpoints                        [6] sequence of ueendpointaddress optional,

non3gppaccessendpoint              [7] ueendpointaddress optional,

location                           [8] location optional,

additionallocation                 [9] location optional,

apn                                [10] apn,

requesttype                        [11] epspdnconnectionrequesttype optional,

accesstype                         [12] accesstype optional,

rattype                            [13] rattype optional,

protocolconfigurationoptions       [14] pdnprotocolconfigurationoptions optional,

servingnetwork                     [15] smfservingnetwork optional,

smpdudnrequest                     [16] smpdudnrequest optional,

bearercontexts                     [17] sequence of epsbearercontext

}

Pfddataforapps ::= set of pfddataforapp

Pfddataforapp ::= sequence

{

appid [1] utf8string,

pfds  [2] pfds

}

Pfds ::= set of pfd

-- see table 5.6.2.5-1 of ts 29.551 [94]

Pfd ::= sequence

{

pfdid                [1] utf8string,

pfdflowdescriptions  [2] pfdflowdescriptions,

urls                 [3] pfdurls,

domainnames          [4] domainnames,

dnprotocol           [5] dnprotocol

}

Pfdurls ::= set of utf8string

Pfdflowdescriptions ::= set of pfdflowdescription

Domainnames ::= set of utf8string

Pfdflowdescription ::= sequence

{

nextlayerprotocol [1] nextlayerprotocol,

serveripaddress   [2] ipaddress,

serverportnumber  [3] portnumber

}

-- see table 5.14.2.2.4-1 of ts 29.122 [63]

Dnprotocol ::= enumerated

{

dnsqname(1),

tlssni(2),

tlssan(3),

tlsscn(4)

}

-- ======================

-- pgw-c + smf parameters

-- ======================

Csrmfi ::= boolean

Eps5gscomboinfo ::= sequence

{

epsinterworkingindication [1] epsinterworkingindication,

epssubscriberids          [2] epssubscriberids,

epspdncnxinfo             [3] epspdncnxinfo optional,

epsbearerinfo             [4] epsbearerinfo optional

}

Epsinterworkingindication ::= enumerated

{

none(1),

withn26(2),

withoutn26(3),

iwknon3gpp(4)

}

Epssubscriberids ::= sequence

{

imsi   [1] imsi optional,

msisdn [2] msisdn optional,

imei   [3] imei optional

}

Epspdncnxinfo ::= sequence

{

pgws8controlplanefteid [1] fteid,

linkedbearerid         [2] epsbearerid optional

}

Epsbearerinfo ::= sequence of epsbearers

Epsbearers ::= sequence

{

epsbearerid         [1] epsbearerid,

pgws8userplanefteid [2] fteid,

qci                 [3] qci

}

Epsbearercontext ::= sequence

{

epsbearerid     [1] epsbearerid,

upgtptunnelinfo [2] gtptunnelinfo,

bearerqos       [3] epsbearerqos

}

Epsbearercontextcreated ::= sequence

{

epsbearerid                  [1] epsbearerid,

cause                        [2] epsbearercreationcausevalue,

gtptunnelinfo                [3] gtptunnelinfo optional,

bearerqos                    [4] epsbearerqos optional,

protocolconfigurationoptions [5] pdnprotocolconfigurationoptions optional

}

Epsbearercontextmodified ::= sequence

{

epsbearerid                  [1] epsbearerid,

cause                        [2] epsbearermodificationcausevalue,

gtptunnelinfo                [3] gtptunnelinfo optional,

bearerqos                    [4] epsbearerqos optional,

protocolconfigurationoptions [5] pdnprotocolconfigurationoptions optional

}

Epsbearersdeleted ::= sequence

{

linkedepsbearerid            [1] epsbearerid optional,

epsbearerids                 [2] sequence of epsbearerid optional,

protocolconfigurationoptions [3] pdnprotocolconfigurationoptions optional,

cause                        [4] epsbearerdeletioncausevalue optional,

deletebearerresponse         [5] epsdeletebearerresponse

}

Epsdeletebearerresponse ::= sequence

{

cause                        [1] epsbearerdeletioncausevalue,

linkedepsbearerid            [2] epsbearerid optional,

bearercontexts               [3] sequence of epsdeletebearercontext optional,

protocolconfigurationoptions [4] pdnprotocolconfigurationoptions optional

}

Epsdeletebearercontext ::= sequence

{

cause                        [1] epsbearerdeletioncausevalue,

epsbearerid                  [2] epsbearerid,

protocolconfigurationoptions [3] pdnprotocolconfigurationoptions optional,

rannascause                  [4] epsrannascause optional

}

Epsbearercontextforremoval ::= sequence

{

epsbearerid [1] epsbearerid,

cause       [2] epsbearerremovalcausevalue

}

Epsbearercreationcausevalue ::= integer (0..255)

Epsbearerdeletioncausevalue ::= integer (0..255)

Epsbearermodificationcausevalue ::= integer (0..255)

Epsbearerremovalcausevalue ::= integer (0..255)

Epsbearerqos ::= sequence

{

qci                       [1] qci optional,

maximumuplinkbitrate      [2] bitratebinkbps optional,

maximumdownlinkbitrate    [3] bitratebinkbps optional,

guaranteeduplinkbitrate   [4] bitratebinkbps optional,

guaranteeddownlinkbitrate [5] bitratebinkbps optional,

prioritylevel             [6] epsqospriority optional

}

Epsrannascause ::= octet string

Epsqospriority ::= integer (1..15)

Bitratebinkbps ::= octet string

Epsgtptunnels ::= sequence

{

controlplanesenderfteid  [1] fteid optional,

controlplanepgws5s8fteid [2] fteid optional,

s1uenodebfteid           [3] fteid optional,

s5s8sgwfteid             [4] fteid optional,

s5s8pgwfteid             [5] fteid optional,

s2buepdgfteid            [6] fteid optional,

s2auepdgfteid            [7] fteid optional

}

Epspdnconnectionrequesttype ::= enumerated

{

initialrequest(1),

handover(2),

rlos(3),

emergency(4),

handoverofemergencybearerservices(5),

reserved(6)

}

Epspdnconnectionreleasescopeindication ::= boolean

Fivegsinterworkinginfo ::= sequence

{

fivegsinterworkingindicator  [1] fivegsinterworkingindicator,

fivegsinterworkingwithoutn26 [2] fivegsinterworkingwithoutn26,

fivegcnotrestrictedsupport   [3] fivegcnotrestrictedsupport

}

Fivegsinterworkingindicator ::= boolean

Fivegsinterworkingwithoutn26 ::= boolean

Fivegcnotrestrictedsupport ::= boolean

Pdnconnectionindicationflags ::= octet string

Pdnhandoverindication ::= boolean

Pdnnbifomsupport ::= boolean

Pdnprotocolconfigurationoptions ::= sequence

{

requestpco   [1] pdnpco optional,

requestapco  [2] pdnpco optional,

requestepco  [3] pdnpco optional,

responsepco  [4] pdnpco optional,

responseapco [5] pdnpco optional,

responseepco [6] pdnpco optional

}

Pdnpco ::= octet string

Pgwchangeindication ::= boolean

Pgwrnsi ::= boolean

Qci ::= integer (0..255)

Gtptunnelinfo ::= sequence

{

fivegsgtptunnels [1] fivegsgtptunnels optional,

epsgtptunnels    [2] epsgtptunnels optional

}

Restorationofpdnconnectionssupport ::= boolean

-- ==================

-- 5g upf definitions

-- ==================

Upfccpdu ::= octet string

-- see clause 6.2.3.8 for the details of this structure

Extendedupfccpdu ::= sequence

{

payload [1] upfccpdupayload,

qfi     [2] qfi optional

}

-- =================

-- 5g upf parameters

-- =================

Upfccpdupayload ::= choice

{

upfipcc           [1] octet string,

upfethernetcc     [2] octet string,

upfunstructuredcc [3] octet string

}

Qfi ::= integer (0..63)

-- ==================

-- 5g udm definitions

-- ==================

Udmservingsystemmessage ::= sequence

{

supi                        [1] supi,

pei                         [2] pei optional,

gpsi                        [3] gpsi optional,

guami                       [4] guami optional,

gummei                      [5] gummei optional,

plmnid                      [6] plmnid optional,

servingsystemmethod         [7] udmservingsystemmethod,

serviceid                   [8] serviceid optional,

roamingindicator            [9] roamingindicator optional

}

Udmsubscriberrecordchangemessage ::= sequence

{

supi                           [1] supi optional,

pei                            [2] pei optional,

gpsi                           [3] gpsi optional,

oldpei                         [4] pei optional,

oldsupi                        [5] supi optional,

oldgpsi                        [6] gpsi optional,

oldserviceid                   [7] serviceid optional,

subscriberrecordchangemethod   [8] udmsubscriberrecordchangemethod,

serviceid                      [9] serviceid optional

}

Udmcancellocationmessage ::= sequence

{

supi                        [1] supi,

pei                         [2] pei optional,

gpsi                        [3] gpsi optional,

guami                       [4] guami optional,

plmnid                      [5] plmnid optional,

cancellocationmethod        [6] udmcancellocationmethod

}

Udmlocationinformationresult ::= sequence

{

supi                     [1] supi,

pei                      [2] pei optional,

gpsi                     [3] gpsi optional,

locationinforequest      [4] udmlocationinforequest,

vplmnid                  [5] plmnid optional,

currentlocationindicator [6] boolean optional,

amfinstanceid            [7] nfid optional,

smsfinstanceid           [8] nfid optional,

location                 [9] location optional,

rattype                  [10] rattype optional,

problemdetails           [11] udmproblemdetails optional

}

Udmueinformationresponse ::= sequence

{

supi                        [1] supi,

tadsinfo                    [2] uecontextinfo optional,

fivegsuserstateinfo         [3] fivegsuserstateinfo optional,

fivegsrvccinfo              [4] fivegsrvccinfo optional,

problemdetails              [5] udmproblemdetails optional

}

Udmueauthenticationresponse ::= sequence

{

supi                        [1] supi,

authenticationinforequest   [2] udmauthenticationinforequest,

akmaindicator               [3] boolean optional,

problemdetails              [4] udmproblemdetails optional

}

-- =================

-- 5g udm parameters

-- =================

Udmservingsystemmethod ::= enumerated

{

amf3gppaccessregistration(0),

amfnon3gppaccessregistration(1),

unknown(2)

}

Udmsubscriberrecordchangemethod ::= enumerated

{

peichange(1),

supichange(2),

gpsichange(3),

uedeprovisioning(4),

unknown(5),

serviceidchange(6)

}

Udmcancellocationmethod ::= enumerated

{

amf3gppaccessderegistration(1),

amfnon3gppaccessderegistration(2),

udmderegistration(3),

unknown(4)

}

Serviceid ::= sequence

{

nssai                     [1] nssai optional,

cagid                     [2] sequence of cagid optional

}

Cagid ::= utf8string

Udmauthenticationinforequest ::= sequence

{

inforequesttype    [1] udminforequesttype,

rgauthctx          [2] sequence size(1..max) of subscriberidentifier,

authtype           [3] primaryauthenticationtype,

servingnetworkname [4] plmnid,

ausfinstanceid     [5] nfid optional,

cellcaginfo        [6] cagid optional,

n5gcindicator      [7] boolean optional

}

Udmlocationinforequest ::= sequence

{

requested5gslocation     [1] boolean optional,

requestedcurrentlocation [2] boolean optional,

requestedrattype         [3] boolean optional,

requestedtimezone        [4] boolean optional,

requestedservingnode     [5] boolean optional

}

Udmproblemdetails ::= sequence

{

cause        [1] udmproblemdetailscause optional

}

Udmproblemdetailscause ::= choice

{

udmdefinedcause       [1] udmdefinedcause,

othercause            [2] udmproblemdetailsothercause

}

Udmdefinedcause ::= enumerated

{

usernotfound(1),

datanotfound(2),

contextnotfound(3),

subscriptionnotfound(4),

other(5)

}

Udminforequesttype ::= enumerated

{

hss(1),

ausf(2),

other(3)

}

Udmproblemdetailsothercause ::= sequence

{

problemdetailstype   [1] utf8string optional,

title                [2] utf8string optional,

status               [3] integer optional,

detail               [4] utf8string optional,

instance             [5] utf8string optional,

cause                [6] utf8string optional,

udminvalidparameters [7] udminvalidparameters,

udmsupportedfeatures [8] utf8string

}

Udminvalidparameters ::= sequence

{

parameter    [1] utf8string optional,

reason       [2] utf8string optional

}

Roamingindicator ::= boolean

-- ===================

-- 5g smsf definitions

-- ===================

-- see clause 6.2.5.3 for details of this structure

Smsmessage ::= sequence

{

originatingsmsparty         [1] smsparty,

terminatingsmsparty         [2] smsparty,

direction                   [3] direction,

linktransferstatus          [4] smstransferstatus,

othermessage                [5] smsothermessageindication optional,

location                    [6] location optional,

peernfaddress               [7] smsnfaddress optional,

peernftype                  [8] smsnftype optional,

smstpdudata                 [9] smstpdudata optional,

messagetype                 [10] smsmessagetype optional,

rpmessagereference          [11] smsrpmessagereference optional

}

Smsreport ::= sequence

{

location           [1] location optional,

smstpdudata        [2] smstpdudata,

messagetype        [3] smsmessagetype,

rpmessagereference [4] smsrpmessagereference

}

-- ==================

-- 5g smsf parameters

-- ==================

Smsaddress ::= octet string(size(2..12))

Smsmessagetype ::= enumerated

{

deliver(1),

deliverreportack(2),

deliverreporterror(3),

statusreport(4),

command(5),

submit(6),

submitreportack(7),

submitreporterror(8),

reserved(9)

}

Smsparty ::= sequence

{

supi        [1] supi optional,

pei         [2] pei optional,

gpsi        [3] gpsi optional,

smsaddress  [4] smsaddress optional

}

Smstransferstatus ::= enumerated

{

transfersucceeded(1),

transferfailed(2),

undefined(3)

}

Smsothermessageindication ::= boolean

Smsnfaddress ::= choice

{

ipaddress   [1] ipaddress,

e164number  [2] e164number

}

Smsnftype ::= enumerated

{

smsgmsc(1),

iwmsc(2),

smsrouter(3)

}

Smsrpmessagereference ::= integer (0..255)

Smstpdudata ::= choice

{

smstpdu [1] smstpdu,

truncatedsmstpdu [2] truncatedsmstpdu

}

Smstpdu ::= octet string (size(1..270))

Truncatedsmstpdu ::= octet string (size(1..130))

-- ===============

-- mms definitions

-- ===============

Mmssend ::= sequence

{

transactionid       [1]  utf8string,

version             [2]  mmsversion,

datetime            [3]  timestamp,

originatingmmsparty [4]  mmsparty,

terminatingmmsparty [5]  sequence of mmsparty optional,

ccrecipients        [6]  sequence of mmsparty optional,

bccrecipients       [7]  sequence of mmsparty optional,

direction           [8]  mmsdirection,

subject             [9]  mmssubject optional,

messageclass        [10]  mmsmessageclass optional,

expiry              [11] mmsexpiry,

desireddeliverytime [12] timestamp optional,

priority            [13] mmspriority optional,

sendervisibility    [14] boolean optional,

deliveryreport      [15] boolean optional,

readreport          [16] boolean optional,

store               [17] boolean optional,

state               [18] mmstate optional,

flags               [19] mmflags optional,

replycharging       [20] mmsreplycharging optional,

applicid            [21] utf8string optional,

replyapplicid       [22] utf8string optional,

auxapplicinfo       [23] utf8string optional,

contentclass        [24] mmscontentclass optional,

drmcontent          [25] boolean optional,

adaptationallowed   [26] mmsadaptation optional,

contenttype         [27] mmscontenttype,

responsestatus      [28] mmsresponsestatus,

responsestatustext  [29] utf8string optional,

messageid           [30] utf8string

}

Mmssendbynonlocaltarget ::= sequence

{

version             [1]  mmsversion,

transactionid       [2]  utf8string,

messageid           [3]  utf8string,

terminatingmmsparty [4]  sequence of mmsparty,

originatingmmsparty [5]  mmsparty,

direction           [6]  mmsdirection,

contenttype         [7]  mmscontenttype,

messageclass        [8]  mmsmessageclass optional,

datetime            [9]  timestamp,

expiry              [10] mmsexpiry optional,

deliveryreport      [11] boolean optional,

priority            [12] mmspriority optional,

sendervisibility    [13] boolean optional,

readreport          [14] boolean optional,

subject             [15] mmssubject optional,

forwardcount        [16] integer optional,

previouslysentby    [17] mmspreviouslysentby optional,

prevsentbydatetime  [18] timestamp optional,

applicid            [19] utf8string optional,

replyapplicid       [20] utf8string optional,

auxapplicinfo       [21] utf8string optional,

contentclass        [22] mmscontentclass optional,

drmcontent          [23] boolean optional,

adaptationallowed   [24] mmsadaptation optional

}

Mmsnotification ::= sequence

{

transactionid           [1]  utf8string,

version                 [2]  mmsversion,

originatingmmsparty     [3]  mmsparty optional,

direction               [4]  mmsdirection,

subject                 [5]  mmssubject optional,

deliveryreportrequested [6]  boolean optional,

stored                  [7]  boolean optional,

messageclass            [8]  mmsmessageclass,

priority                [9]  mmspriority optional,

messagesize             [10]  integer,

expiry                  [11] mmsexpiry,

replycharging           [12] mmsreplycharging optional

}

Mmssendtononlocaltarget ::= sequence

{

version             [1]  mmsversion,

transactionid       [2]  utf8string,

messageid           [3]  utf8string,

terminatingmmsparty [4]  sequence of mmsparty,

originatingmmsparty [5]  mmsparty,

direction           [6]  mmsdirection,

contenttype         [7]  mmscontenttype,

messageclass        [8]  mmsmessageclass optional,

datetime            [9]  timestamp,

expiry              [10] mmsexpiry optional,

deliveryreport      [11] boolean optional,

priority            [12] mmspriority optional,

sendervisibility    [13] boolean optional,

readreport          [14] boolean optional,

subject             [15] mmssubject optional,

forwardcount        [16] integer optional,

previouslysentby    [17] mmspreviouslysentby optional,

prevsentbydatetime  [18] timestamp optional,

applicid            [19] utf8string optional,

replyapplicid       [20] utf8string optional,

auxapplicinfo       [21] utf8string optional,

contentclass        [22] mmscontentclass optional,

drmcontent          [23] boolean optional,

adaptationallowed   [24] mmsadaptation optional

}

Mmsnotificationresponse ::= sequence

{

transactionid [1] utf8string,

version       [2] mmsversion,

direction     [3] mmsdirection,

status        [4] mmstatus,

reportallowed [5] boolean optional

}

Mmsretrieval ::= sequence

{

transactionid       [1]  utf8string,

version             [2]  mmsversion,

messageid           [3]  utf8string,

datetime            [4]  timestamp,

originatingmmsparty [5]  mmsparty optional,

previouslysentby    [6]  mmspreviouslysentby optional,

prevsentbydatetime  [7]  timestamp optional,

terminatingmmsparty [8]  sequence of mmsparty optional,

ccrecipients        [9]  sequence of mmsparty optional,

direction           [10] mmsdirection,

subject             [11] mmssubject optional,

state               [12] mmstate optional,

flags               [13] mmflags optional,

messageclass        [14] mmsmessageclass optional,

priority            [15] mmspriority,

deliveryreport      [16] boolean optional,

readreport          [17] boolean optional,

replycharging       [18] mmsreplycharging optional,

retrievestatus      [19] mmsretrievestatus optional,

retrievestatustext  [20] utf8string optional,

applicid            [21] utf8string optional,

replyapplicid       [22] utf8string optional,

auxapplicinfo       [23] utf8string optional,

contentclass        [24] mmscontentclass optional,

drmcontent          [25] boolean optional,

replaceid           [26] utf8string optional,

contenttype         [27] utf8string optional

}

Mmsdeliveryack ::= sequence

{

transactionid [1] utf8string,

version       [2] mmsversion,

reportallowed [3] boolean optional,

status        [4] mmstatus,

direction     [5] mmsdirection

}

Mmsforward ::= sequence

{

transactionid         [1]  utf8string,

version               [2]  mmsversion,

datetime              [3]  timestamp optional,

originatingmmsparty   [4]  mmsparty,

terminatingmmsparty   [5]  sequence of mmsparty optional,

ccrecipients          [6]  sequence of mmsparty optional,

bccrecipients         [7]  sequence of mmsparty optional,

direction             [8]  mmsdirection,

expiry                [9]  mmsexpiry optional,

desireddeliverytime   [10] timestamp optional,

deliveryreportallowed [11] boolean optional,

deliveryreport        [12] boolean optional,

store                 [13] boolean optional,

state                 [14] mmstate optional,

flags                 [15] mmflags optional,

contentlocationreq    [16] utf8string,

replycharging         [17] mmsreplycharging optional,

responsestatus        [18] mmsresponsestatus,

responsestatustext    [19] utf8string  optional,

messageid             [20] utf8string optional,

contentlocationconf   [21] utf8string optional,

storestatus           [22] mmsstorestatus optional,

storestatustext       [23] utf8string optional

}

Mmsdeletefromrelay ::= sequence

{

transactionid        [1] utf8string,

version              [2] mmsversion,

direction            [3] mmsdirection,

contentlocationreq   [4] sequence of utf8string,

contentlocationconf  [5] sequence of utf8string,

deleteresponsestatus [6] mmsdeleteresponsestatus,

deleteresponsetext   [7] sequence of utf8string

}

Mmsmboxstore ::= sequence

{

transactionid       [1] utf8string,

version             [2] mmsversion,

direction           [3] mmsdirection,

contentlocationreq  [4] utf8string,

state               [5] mmstate optional,

flags               [6] mmflags optional,

contentlocationconf [7] utf8string optional,

storestatus         [8] mmsstorestatus,

storestatustext     [9] utf8string optional

}

Mmsmboxupload ::= sequence

{

transactionid       [1]  utf8string,

version             [2]  mmsversion,

direction           [3]  mmsdirection,

state               [4]  mmstate optional,

flags               [5]  mmflags optional,

contenttype         [6]  utf8string,

contentlocation     [7]  utf8string optional,

storestatus         [8]  mmsstorestatus,

storestatustext     [9]  utf8string optional,

mmessages           [10] sequence of mmboxdescription

}

Mmsmboxdelete ::= sequence

{

transactionid       [1] utf8string,

version             [2] mmsversion,

direction           [3] mmsdirection,

contentlocationreq  [4] sequence of utf8string,

contentlocationconf [5] sequence of utf8string optional,

responsestatus      [6] mmsdeleteresponsestatus,

responsestatustext  [7] utf8string optional

}

Mmsdeliveryreport ::= sequence

{

version             [1] mmsversion,

messageid           [2] utf8string,

terminatingmmsparty [3] sequence of mmsparty,

mmsdatetime         [4] timestamp,

responsestatus      [5] mmsresponsestatus,

responsestatustext  [6] utf8string optional,

applicid            [7] utf8string optional,

replyapplicid       [8] utf8string optional,

auxapplicinfo       [9] utf8string optional

}

Mmsdeliveryreportnonlocaltarget ::= sequence

{

version             [1]  mmsversion,

transactionid       [2]  utf8string,

messageid           [3]  utf8string,

terminatingmmsparty [4]  sequence of mmsparty,

originatingmmsparty [5]  mmsparty,

direction           [6]  mmsdirection,

mmsdatetime         [7]  timestamp,

forwardtooriginator [8]  boolean optional,

status              [9]  mmstatus,

statusextension     [10] mmstatusextension,

statustext          [11] mmstatustext,

applicid            [12] utf8string optional,

replyapplicid       [13] utf8string optional,

auxapplicinfo       [14] utf8string optional

}

Mmsreadreport ::= sequence

{

version             [1] mmsversion,

messageid           [2] utf8string,

terminatingmmsparty [3] sequence of mmsparty,

originatingmmsparty [4] sequence of mmsparty,

direction           [5] mmsdirection,

mmsdatetime         [6] timestamp,

readstatus          [7] mmsreadstatus,

applicid            [8] utf8string optional,

replyapplicid       [9] utf8string optional,

auxapplicinfo       [10] utf8string optional

}

Mmsreadreportnonlocaltarget ::= sequence

{

version             [1] mmsversion,

transactionid       [2] utf8string,

terminatingmmsparty [3] sequence of mmsparty,

originatingmmsparty [4] sequence of mmsparty,

direction           [5] mmsdirection,

messageid           [6] utf8string,

mmsdatetime         [7] timestamp,

readstatus          [8] mmsreadstatus,

readstatustext      [9] mmsreadstatustext optional,

applicid            [10] utf8string optional,

replyapplicid       [11] utf8string optional,

auxapplicinfo       [12] utf8string optional

}

Mmscancel ::= sequence

{

transactionid [1] utf8string,

version       [2] mmsversion,

cancelid      [3] utf8string,

direction     [4] mmsdirection

}

Mmsmboxviewrequest ::= sequence

{

transactionid   [1]  utf8string,

version         [2]  mmsversion,

contentlocation [3]  utf8string optional,

state           [4]  sequence of mmstate optional,

flags           [5]  sequence of mmflags optional,

start           [6]  integer optional,

limit           [7]  integer optional,

attributes      [8]  sequence of utf8string optional,

totals          [9]  integer optional,

quotas          [10] mmsquota optional

}

Mmsmboxviewresponse ::= sequence

{

transactionid   [1]  utf8string,

version         [2]  mmsversion,

contentlocation [3]  utf8string optional,

state           [4]  sequence of mmstate optional,

flags           [5]  sequence of mmflags optional,

start           [6]  integer optional,

limit           [7]  integer optional,

attributes      [8]  sequence of utf8string optional,

mmstotals       [9]  boolean optional,

mmsquotas       [10] boolean optional,

mmessages       [11] sequence of mmboxdescription

}

Mmboxdescription ::= sequence

{

contentlocation          [1]  utf8string optional,

messageid                [2]  utf8string optional,

state                    [3]  mmstate optional,

flags                    [4]  sequence of mmflags optional,

datetime                 [5]  timestamp optional,

originatingmmsparty      [6]  mmsparty optional,

terminatingmmsparty      [7]  sequence of mmsparty optional,

ccrecipients             [8]  sequence of mmsparty optional,

bccrecipients            [9]  sequence of mmsparty optional,

messageclass             [10] mmsmessageclass optional,

subject                  [11] mmssubject optional,

priority                 [12] mmspriority optional,

deliverytime             [13] timestamp optional,

readreport               [14] boolean optional,

messagesize              [15] integer optional,

replycharging            [16] mmsreplycharging optional,

previouslysentby         [17] mmspreviouslysentby optional,

previouslysentbydatetime [18] timestamp optional,

contenttype              [19] utf8string optional

}

-- =========

-- mms ccpdu

-- =========

Mmsccpdu ::= sequence

{

version    [1] mmsversion,

transactionid [2] utf8string,

mmscontent    [3] octet string

}

-- ==============

-- mms parameters

-- ==============

Mmsadaptation ::= sequence

{

allowed   [1] boolean,

overriden [2] boolean

}

Mmscancelstatus ::= enumerated

{

cancelrequestsuccessfullyreceived(1),

cancelrequestcorrupted(2)

}

Mmscontentclass ::= enumerated

{

text(1),

imagebasic(2),

imagerich(3),

videobasic(4),

videorich(5),

megapixel(6),

contentbasic(7),

contentrich(8)

}

Mmscontenttype ::= utf8string

Mmsdeleteresponsestatus ::= enumerated

{

ok(1),

errorunspecified(2),

errorservicedenied(3),

errormessageformatcorrupt(4),

errorsendingaddressunresolved(5),

errormessagenotfound(6),

errornetworkproblem(7),

errorcontentnotaccepted(8),

errorunsupportedmessage(9),

errortransientfailure(10),

errortransientsendingaddressunresolved(11),

errortransientmessagenotfound(12),

errortransientnetworkproblem(13),

errortransientpartialsuccess(14),

errorpermanentfailure(15),

errorpermanentservicedenied(16),

errorpermanentmessageformatcorrupt(17),

errorpermanentsendingaddressunresolved(18),

errorpermanentmessagenotfound(19),

errorpermanentcontentnotaccepted(20),

errorpermanentreplycharginglimitationsnotmet(21),

errorpermanentreplychargingrequestnotaccepted(22),

errorpermanentreplychargingforwardingdenied(23),

errorpermanentreplychargingnotsupported(24),

errorpermanentaddresshidingnotsupported(25),

errorpermanentlackofprepaid(26)

}

Mmsdirection ::= enumerated

{

fromtarget(0),

totarget(1)

}

Mmselementdescriptor ::= sequence

{

reference [1] utf8string,

parameter [2] utf8string     optional,

value     [3] utf8string     optional

}

Mmsexpiry ::= sequence

{

expiryperiod [1] integer,

periodformat [2] mmsperiodformat

}

Mmflags ::= sequence

{

length     [1] integer,

flag       [2] mmstateflag,

flagstring [3] utf8string

}

Mmsmessageclass ::= enumerated

{

personal(1),

advertisement(2),

informational(3),

auto(4)

}

Mmsparty ::= sequence

{

mmspartyids [1] sequence of mmspartyid,

nonlocalid  [2] nonlocalid

}

Mmspartyid ::= choice

{

e164number   [1] e164number,

emailaddress [2] emailaddress,

imsi         [3] imsi,

impu         [4] impu,

impi         [5] impi,

supi         [6] supi,

gpsi         [7] gpsi

}

Mmsperiodformat ::= enumerated

{

absolute(1),

relative(2)

}

Mmspreviouslysent ::= sequence

{

previouslysentbyparty [1] mmsparty,

sequencenumber        [2] integer,

previoussenddatetime  [3] timestamp

}

Mmspreviouslysentby ::= sequence of mmspreviouslysent

Mmspriority ::= enumerated

{

low(1),

normal(2),

high(3)

}

Mmsquota ::= sequence

{

quota     [1] integer,

quotaunit [2] mmsquotaunit

}

Mmsquotaunit ::= enumerated

{

nummessages(1),

bytes(2)

}

Mmsreadstatus ::= enumerated

{

read(1),

deletedwithoutbeingread(2)

}

Mmsreadstatustext ::= utf8string

Mmsreplycharging ::= enumerated

{

requested(0),

requestedtextonly(1),

accepted(2),

acceptedtextonly(3)

}

Mmsresponsestatus ::= enumerated

{

ok(1),

errorunspecified(2),

errorservicedenied(3),

errormessageformatcorrupt(4),

errorsendingaddressunresolved(5),

errormessagenotfound(6),

errornetworkproblem(7),

errorcontentnotaccepted(8),

errorunsupportedmessage(9),

errortransientfailure(10),

errortransientsendingaddressunresolved(11),

errortransientmessagenotfound(12),

errortransientnetworkproblem(13),

errortransientpartialsuccess(14),

errorpermanentfailure(15),

errorpermanentservicedenied(16),

errorpermanentmessageformatcorrupt(17),

errorpermanentsendingaddressunresolved(18),

errorpermanentmessagenotfound(19),

errorpermanentcontentnotaccepted(20),

errorpermanentreplycharginglimitationsnotmet(21),

errorpermanentreplychargingrequestnotaccepted(22),

errorpermanentreplychargingforwardingdenied(23),

errorpermanentreplychargingnotsupported(24),

errorpermanentaddresshidingnotsupported(25),

errorpermanentlackofprepaid(26)

}

Mmsretrievestatus ::= enumerated

{

success(1),

errortransientfailure(2),

errortransientmessagenotfound(3),

errortransientnetworkproblem(4),

errorpermanentfailure(5),

errorpermanentservicedenied(6),

errorpermanentmessagenotfound(7),

errorpermanentcontentunsupported(8)

}

Mmsstorestatus ::= enumerated

{

success(1),

errortransientfailure(2),

errortransientnetworkproblem(3),

errorpermanentfailure(4),

errorpermanentservicedenied(5),

errorpermanentmessageformatcorrupt(6),

errorpermanentmessagenotfound(7),

errormmboxfull(8)

}

Mmstate ::= enumerated

{

draft(1),

sent(2),

new(3),

retrieved(4),

forwarded(5)

}

Mmstateflag ::= enumerated

{

add(1),

remove(2),

filter(3)

}

Mmstatus ::= enumerated

{

expired(1),

retrieved(2),

rejected(3),

deferred(4),

unrecognized(5),

indeterminate(6),

forwarded(7),

unreachable(8)

}

Mmstatusextension ::= enumerated

{

rejectionbymmsrecipient(0),

rejectionbyotherrs(1)

}

Mmstatustext ::= utf8string

Mmssubject ::= utf8string

Mmsversion ::= sequence

{

majorversion [1] integer,

minorversion [2] integer

}

-- ==================

-- 5g ptc definitions

-- ==================

Ptcregistration  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcserveruri                  [2] utf8string,

ptcregistrationrequest        [3] ptcregistrationrequest,

ptcregistrationoutcome        [4] ptcregistrationoutcome

}

Ptcsessioninitiation  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcserveruri                  [3] utf8string,

ptcsessioninfo                [4] ptcsessioninfo,

ptcoriginatingid              [5] ptctargetinformation,

ptcparticipants               [6] sequence of ptctargetinformation optional,

ptcparticipantpresencestatus  [7] multipleparticipantpresencestatus optional,

location                      [8] location optional,

ptcbearercapability           [9] utf8string optional,

ptchost                       [10] ptctargetinformation optional

}

Ptcsessionabandon  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

location                      [4] location optional,

ptcabandoncause               [5] integer

}

Ptcsessionstart  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcserveruri                  [3] utf8string,

ptcsessioninfo                [4] ptcsessioninfo,

ptcoriginatingid              [5] ptctargetinformation,

ptcparticipants               [6] sequence of ptctargetinformation optional,

ptcparticipantpresencestatus  [7] multipleparticipantpresencestatus optional,

location                      [8] location optional,

ptchost                       [9] ptctargetinformation optional,

ptcbearercapability           [10] utf8string optional

}

Ptcsessionend  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcserveruri                  [3] utf8string,

ptcsessioninfo                [4] ptcsessioninfo,

ptcparticipants               [5] sequence of ptctargetinformation optional,

location                      [6] location optional,

ptcsessionendcause            [7] ptcsessionendcause

}

Ptcstartofinterception  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

preestsessionid               [3] ptcsessioninfo optional,

ptcoriginatingid              [4] ptctargetinformation,

ptcsessioninfo                [5] ptcsessioninfo optional,

ptchost                       [6] ptctargetinformation optional,

ptcparticipants               [7] sequence of ptctargetinformation optional,

ptcmediastreamavail           [8] boolean optional,

ptcbearercapability           [9] utf8string optional

}

Ptcpreestablishedsession  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcserveruri                  [2] utf8string,

rtpsetting                    [3] rtpsetting,

ptcmediacapability            [4] utf8string,

ptcpreestsessionid            [5] ptcsessioninfo,

ptcpreeststatus               [6] ptcpreeststatus,

ptcmediastreamavail           [7] boolean optional,

location                      [8] location optional,

ptcfailurecode                [9] ptcfailurecode optional

}

Ptcinstantpersonalalert  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcipapartyid                 [2] ptctargetinformation,

ptcipadirection               [3] direction

}

Ptcpartyjoin  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

ptcparticipants               [4] sequence of ptctargetinformation optional,

ptcparticipantpresencestatus  [5] multipleparticipantpresencestatus optional,

ptcmediastreamavail           [6] boolean optional,

ptcbearercapability           [7] utf8string optional

}

Ptcpartydrop  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

ptcpartydrop                  [4] ptctargetinformation,

ptcparticipantpresencestatus  [5] ptcparticipantpresencestatus optional

}

Ptcpartyhold  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

ptcparticipants               [4] sequence of ptctargetinformation optional,

ptcholdid                     [5] sequence of ptctargetinformation,

ptcholdretrieveind            [6] boolean

}

Ptcmediamodification  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

ptcmediastreamavail           [4] boolean optional,

ptcbearercapability           [5] utf8string

}

Ptcgroupadvertisement  ::=sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcidlist                     [3] sequence of ptctargetinformation optional,

ptcgroupauthrule              [4] ptcgroupauthrule optional,

ptcgroupadsender              [5] ptctargetinformation,

ptcgroupnickname              [6] utf8string optional

}

Ptcfloorcontrol  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcsessioninfo                [3] ptcsessioninfo,

ptcflooractivity              [4] sequence of ptcflooractivity,

ptcfloorspeakerid             [5] ptctargetinformation optional,

ptcmaxtbtime                  [6] integer optional,

ptcqueuedfloorcontrol         [7] boolean optional,

ptcqueuedposition             [8] integer optional,

ptctalkburstpriority          [9] ptctbprioritylevel optional,

ptctalkburstreason            [10] ptctbreasoncode optional

}

Ptctargetpresence  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptctargetpresencestatus       [2] ptcparticipantpresencestatus

}

Ptcparticipantpresence  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcparticipantpresencestatus  [2] ptcparticipantpresencestatus

}

Ptclistmanagement  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptclistmanagementtype         [3] ptclistmanagementtype optional,

ptclistmanagementaction       [4] ptclistmanagementaction optional,

ptclistmanagementfailure      [5] ptclistmanagementfailure optional,

ptccontactid                  [6] ptctargetinformation optional,

ptcidlist                     [7] sequence of ptcidlist optional,

ptchost                       [8] ptctargetinformation optional

}

Ptcaccesspolicy  ::= sequence

{

ptctargetinformation          [1] ptctargetinformation,

ptcdirection                  [2] direction,

ptcaccesspolicytype           [3] ptcaccesspolicytype optional,

ptcuseraccesspolicy           [4] ptcuseraccesspolicy optional,

ptcgroupauthrule              [5] ptcgroupauthrule optional,

ptccontactid                  [6] ptctargetinformation optional,

ptcaccesspolicyfailure        [7] ptcaccesspolicyfailure optional

}

-- =========

-- ptc ccpdu

-- =========

Ptcccpdu ::= octet string

-- =================

-- 5g ptc parameters

-- =================

Ptcregistrationrequest  ::= enumerated

{

register(1),

reregister(2),

deregister(3)

}

Ptcregistrationoutcome  ::= enumerated

{

success(1),

failure(2)

}

Ptcsessionendcause  ::= enumerated

{

initiaterleavessession(1),

definedparticipantleaves(2),

numberofparticipants(3),

sessiontimerexpired(4),

ptcspeechinactive(5),

allmediatypesinactive(6)

}

Ptctargetinformation  ::= sequence

{

identifiers                [1] sequence size(1..max) of ptcidentifiers

}

Ptcidentifiers  ::= choice

{

mcpttid                    [1] utf8string,

instanceidentifierurn      [2] utf8string,

ptcchatgroupid             [3] ptcchatgroupid,

impu                       [4] impu,

impi                       [5] impi

}

Ptcsessioninfo  ::= sequence

{

ptcsessionuri              [1] utf8string,

ptcsessiontype             [2] ptcsessiontype

}

Ptcsessiontype  ::= enumerated

{

ondemand(1),

preestablished(2),

adhoc(3),

prearranged(4),

groupsession(5)

}

Multipleparticipantpresencestatus  ::= sequence of ptcparticipantpresencestatus

Ptcparticipantpresencestatus  ::= sequence

{

presenceid                 [1] ptctargetinformation,

presencetype               [2] ptcpresencetype,

presencestatus             [3] boolean

}

Ptcpresencetype  ::= enumerated

{

ptcclient(1),

ptcgroup(2)

}

Ptcpreeststatus  ::= enumerated

{

established(1),

modified(2),

released(3)

}

Rtpsetting  ::= sequence

{

ipaddress                  [1] ipaddress,

portnumber                 [2] portnumber

}

Ptcidlist  ::= sequence

{

ptcpartyid                 [1] ptctargetinformation,

ptcchatgroupid             [2] ptcchatgroupid

}

Ptcchatgroupid  ::= sequence

{

groupidentity              [1] utf8string

}

Ptcflooractivity  ::= enumerated

{

tbcprequest(1),

tbcpgranted(2),

tbcpdeny(3),

tbcpidle(4),

tbcptaken(5),

tbcprevoke(6),

tbcpqueued(7),

tbcprelease(8)

}

Ptctbprioritylevel  ::= enumerated

{

preemptive(1),

highpriority(2),

normalpriority(3),

listenonly(4)

}

Ptctbreasoncode  ::= enumerated

{

noqueuingallowed(1),

oneparticipantsession(2),

listenonly(3),

exceededmaxduration(4),

tbprevented(5)

}

Ptclistmanagementtype  ::= enumerated

{

contactlistmanagementattempt(1),

grouplistmanagementattempt(2),

contactlistmanagementresult(3),

grouplistmanagementresult(4),

requestunsuccessful(5)

}

Ptclistmanagementaction  ::= enumerated

{

create(1),

modify(2),

retrieve(3),

delete(4),

notify(5)

}

Ptcaccesspolicytype  ::= enumerated

{

ptcuseraccesspolicyattempt(1),

groupauthorizationrulesattempt(2),

ptcuseraccesspolicyquery(3),

groupauthorizationrulesquery(4),

ptcuseraccesspolicyresult(5),

groupauthorizationrulesresult(6),

requestunsuccessful(7)

}

Ptcuseraccesspolicy  ::= enumerated

{

allowincomingptcsessionrequest(1),

blockincomingptcsessionrequest(2),

allowautoanswermode(3),

allowoverridemanualanswermode(4)

}

Ptcgroupauthrule  ::= enumerated

{

allowinitiatingptcsession(1),

blockinitiatingptcsession(2),

allowjoiningptcsession(3),

blockjoiningptcsession(4),

allowaddparticipants(5),

blockaddparticipants(6),

allowsubscriptionptcsessionstate(7),

blocksubscriptionptcsessionstate(8),

allowanonymity(9),

forbidanonymity(10)

}

Ptcfailurecode  ::= enumerated

{

sessioncannotbeestablished(1),

sessioncannotbemodified(2)

}

Ptclistmanagementfailure  ::= enumerated

{

requestunsuccessful(1),

requestunknown(2)

}

Ptcaccesspolicyfailure  ::= enumerated

{

requestunsuccessful(1),

requestunknown(2)

}

-- ===============

-- ims definitions

-- ===============

-- see clause 7.12.4.2.1 for details of this structure

Imsmessage ::= sequence

{

payload               [1] imspayload,

sessiondirection      [2] sessiondirection,

voiproamingindication [3] voiproamingindication optional,

location              [6] location optional

}

-- see clause 7.12.4.2.2 for details of this structure

Startofinterceptionforactiveimssession ::= sequence

{

originatingid         [1] sequence of impu,

terminatingid         [2] impu,

sdpstate              [3] sequence of octet string optional,

diversionidentity     [4] impu optional,

voiproamingindication [5] voiproamingindication optional,

location              [7] location optional

}

-- see clause 7.12.4.2.3 for the details.

Imsccunavailable ::= sequence

{

ccunavailablereason   [1] utf8string,

sdpstate              [2] octet string optional

}

-- =========

-- ims ccpdu

-- =========

Imsccpdu ::= sequence

{

payload [1] imsccpdupayload,

sdpinfo [2] octet string optional

}

Imsccpdupayload ::= octet string

-- ==============

-- ims parameters

-- ==============

Imspayload ::= choice

{

encapsulatedsipmessage            [1] sipmessage

}

Sipmessage ::= sequence

{

ipsourceaddress       [1] ipaddress,

ipdestinationaddress  [2] ipaddress,

sipcontent            [3] octet string

}

Voiproamingindication ::= enumerated

{

roaminglbo(1),

roamings8hr(2),

roamingn9hr(3)

}

Sessiondirection ::= enumerated

{

fromtarget(1),

totarget(2),

combined(3),

indeterminate(4)

}

Headeronlyindication ::= boolean

-- =================================

-- stir/shaken/rcd/ecnam definitions

-- =================================

-- see clause 7.11.2.1.2 for details of this structure

Stirshakensignaturegeneration ::= sequence

{

passports                 [1] sequence of passport,

encapsulatedsipmessage    [2] sipmessage optional

}

-- see clause 7.11.2.1.3 for details of this structure

Stirshakensignaturevalidation ::= sequence

{

passports                 [1] sequence of passport optional,

rcdterminaldisplayinfo    [2] rcddisplayinfo optional,

ecnamterminaldisplayinfo  [3] ecnamdisplayinfo optional,

shakenvalidationresult    [4] shakenvalidationresult,

shakenfailurestatuscode   [5] shakenfailurestatuscode optional,

encapsulatedsipmessage    [6] sipmessage optional

}

-- ================================

-- stir/shaken/rcd/ecnam parameters

-- ================================

Passport ::= sequence

{

passportheader    [1] passportheader,

passportpayload   [2] passportpayload,

passportsignature [3] octet string

}

Passportheader ::= sequence

{

type          [1] jwstokentype,

algorithm     [2] utf8string,

ppt           [3] utf8string optional,

x5u           [4] utf8string

}

Jwstokentype ::= enumerated

{

passport(1)

}

Passportpayload ::= sequence

{

issuedattime    [1] generalizedtime,

originator      [2] stirshakenoriginator,

destination     [3] stirshakendestinations,

attestation     [4] attestation,

origid          [5] utf8string,

diversion       [6] stirshakendestination

}

Stirshakenoriginator ::= choice

{

telephonenumber [1] stirshakentn,

stirshakenuri   [2] utf8string

}

Stirshakendestinations ::= sequence of stirshakendestination

Stirshakendestination ::= choice

{

telephonenumber [1] stirshakentn,

stirshakenuri   [2] utf8string

}

Stirshakentn ::= choice

{

msisdn [1] msisdn

}

Attestation ::= enumerated

{

attestationa(1),

attestationb(2),

attestationc(3)

}

Shakenvalidationresult ::= enumerated

{

tnvalidationpassed(1),

tnvalidationfailed(2),

notnvalidation(3)

}

Shakenfailurestatuscode ::= integer

Ecnamdisplayinfo ::= sequence

{

name           [1] utf8string,

additionalinfo [2] octet string optional

}

Rcddisplayinfo ::= sequence

{

name [1] utf8string,

jcd  [2] octet string optional,

jcl  [3] octet string optional

}

-- ===================

-- 5g lals definitions

-- ===================

Lalsreport ::= sequence

{

supi                [1] supi optional,

--  pei                 [2] pei optional, deprecated in release-16, do not re-use this tag number

gpsi                [3] gpsi optional,

location            [4] location optional,

impu                [5] impu optional,

imsi                [7] imsi optional,

msisdn              [8] msisdn optional

}

-- =====================

-- pdhr/pdsr definitions

-- =====================

Pdheaderreport ::= sequence

{

pdusessionid                [1] pdusessionid,

sourceipaddress             [2] ipaddress,

sourceport                  [3] portnumber optional,

destinationipaddress        [4] ipaddress,

destinationport             [5] portnumber optional,

nextlayerprotocol           [6] nextlayerprotocol,

ipv6flowlabel               [7] ipv6flowlabel optional,

direction                   [8] direction,

packetsize                  [9] integer

}

Pdsummaryreport ::= sequence

{

pdusessionid                [1] pdusessionid,

sourceipaddress             [2] ipaddress,

sourceport                  [3] portnumber optional,

destinationipaddress        [4] ipaddress,

destinationport             [5] portnumber optional,

nextlayerprotocol           [6] nextlayerprotocol,

ipv6flowlabel               [7] ipv6flowlabel optional,

direction                   [8] direction,

pdsrsummarytrigger          [9] pdsrsummarytrigger,

firstpackettimestamp        [10] timestamp,

lastpackettimestamp         [11] timestamp,

packetcount                 [12] integer,

bytecount                   [13] integer,

persessiontrigger           [14] boolean

}

-- ====================

-- pdhr/pdsr parameters

-- ====================

Pdsrsummarytrigger ::= enumerated

{

timerexpiry(1),

packetcount(2),

bytecount(3),

startofflow(4),

endofflow(5)

}

-- ==================================

-- identifier association definitions

-- ==================================

Amfidentifierassociation ::= sequence

{

supi             [1] supi,

suci             [2] suci optional,

pei              [3] pei optional,

gpsi             [4] gpsi optional,

guti             [5] fivegguti,

location         [6] location,

fivegstailist    [7] tailist optional

}

Mmeidentifierassociation ::= sequence

{

imsi        [1] imsi,

imei        [2] imei optional,

msisdn      [3] msisdn optional,

guti        [4] guti,

location    [5] location,

tailist     [6] tailist optional

}

-- =================================

-- identifier association parameters

-- =================================

Mmegroupid ::= octet string (size(2))

Mmecode ::= octet string (size(1))

Tmsi ::= octet string (size(4))

-- ===================

-- eps mme definitions

-- ===================

Mmeattach ::= sequence

{

attachtype       [1] epsattachtype,

attachresult     [2] epsattachresult,

imsi             [3] imsi,

imei             [4] imei optional,

msisdn           [5] msisdn optional,

guti             [6] guti optional,

location         [7] location optional,

epstailist       [8] tailist optional,

smsservicestatus [9] epssmsservicestatus optional,

oldguti          [10] guti optional,

emm5gregstatus   [11] emm5gmmstatus optional

}

Mmedetach ::= sequence

{

detachdirection    [1] mmedirection,

detachtype         [2] epsdetachtype,

imsi               [3] imsi,

imei               [4] imei optional,

msisdn             [5] msisdn optional,

guti               [6] guti optional,

cause              [7] emmcause optional,

location           [8] location optional,

switchoffindicator [9] switchoffindicator optional

}

Mmelocationupdate ::= sequence

{

imsi             [1] imsi,

imei             [2] imei optional,

msisdn           [3] msisdn optional,

guti             [4] guti optional,

location         [5] location optional,

oldguti          [6] guti optional,

smsservicestatus [7] epssmsservicestatus optional

}

Mmestartofinterceptionwithepsattachedue ::= sequence

{

attachtype         [1] epsattachtype,

attachresult       [2] epsattachresult,

imsi               [3] imsi,

imei               [4] imei optional,

msisdn             [5] msisdn optional,

guti               [6] guti optional,

location           [7] location optional,

epstailist         [9] tailist optional,

smsservicestatus   [10] epssmsservicestatus optional,

emm5gregstatus     [12] emm5gmmstatus optional

}

Mmeunsuccessfulprocedure ::= sequence

{

failedproceduretype [1] mmefailedproceduretype,

failurecause        [2] mmefailurecause,

imsi                [3] imsi optional,

imei                [4] imei optional,

msisdn              [5] msisdn optional,

guti                [6] guti optional,

location            [7] location optional

}

-- see clause 6.3.2.2.8 for details of this structure

Mmepositioninginfotransfer ::= sequence

{

imsi                [1] imsi,

imei                [2] imei optional,

msisdn              [3] msisdn optional,

guti                [4] guti optional,

lppamessage         [5] octet string optional,

lppmessage          [6] octet string optional,

mmelcscorrelationid [7] octet string (size(4))

}

-- ==================

-- eps mme parameters

-- ==================

Emmcause ::= integer (0..255)

Esmcause ::= integer (0..255)

Epsattachtype ::= enumerated

{

epsattach(1),

combinedepsimsiattach(2),

epsrlosattach(3),

epsemergencyattach(4),

reserved(5)

}

Epsattachresult ::= enumerated

{

epsonly(1),

combinedepsimsi(2)

}

Epsdetachtype ::= enumerated

{

epsdetach(1),

imsidetach(2),

combinedepsimsidetach(3),

reattachrequired(4),

reattachnotrequired(5),

reserved(6)

}

Epssmsservicestatus ::= enumerated

{

smsservicesnotavailable(1),

smsservicesnotavailableinthisplmn(2),

networkfailure(3),

congestion(4)

}

Mmedirection ::= enumerated

{

networkinitiated(1),

ueinitiated(2)

}

Mmefailedproceduretype ::= enumerated

{

attachreject(1),

authenticationreject(2),

securitymodereject(3),

servicereject(4),

trackingareaupdatereject(5),

activatededicatedepsbearercontextreject(6),

activatedefaultepsbearercontextreject(7),

bearerresourceallocationreject(8),

bearerresourcemodificationreject(9),

modifyepsbearercontectreject(10),

pdnconnectivityreject(11),

pdndisconnectreject(12)

}

Mmefailurecause ::= choice

{

emmcause [1] emmcause,

esmcause [2] esmcause

}

-- ===========================

-- li notification definitions

-- ===========================

Linotification ::= sequence

{

notificationtype                    [1] linotificationtype,

appliedtargetid                     [2] targetidentifier optional,

applieddeliveryinformation          [3] sequence of liapplieddeliveryinformation optional,

appliedstarttime                    [4] timestamp optional,

appliedendtime                      [5] timestamp optional

}

-- ==========================

-- li notification parameters

-- ==========================

Linotificationtype ::= enumerated

{

activation(1),

deactivation(2),

modification(3)

}

Liapplieddeliveryinformation ::= sequence

{

hi2deliveryipaddress                [1] ipaddress optional,

hi2deliveryportnumber               [2] portnumber optional,

hi3deliveryipaddress                [3] ipaddress optional,

hi3deliveryportnumber               [4] portnumber optional

}

-- ===============

-- mdf definitions

-- ===============

Mdfcellsitereport ::= sequence of cellinformation

-- ==============================

-- 5g eps interworking parameters

-- ==============================

Emm5gmmstatus ::= sequence

{

emmregstatus  [1] emmregstatus optional,

fivegmmstatus [2] fivegmmstatus optional

}

Eps5gguti ::= choice

{

guti      [1] guti,

fivegguti [2] fivegguti

}

Emmregstatus ::= enumerated

{

ueemmregistered(1),

uenotemmregistered(2)

}

Fivegmmstatus ::= enumerated

{

ue5gmmregistered(1),

uenot5gmmregistered(2)

}

-- ========================================

-- separated location reporting definitions

-- ========================================

Separatedlocationreporting ::= sequence

{

supi                        [1] supi,

suci                        [2] suci optional,

pei                         [3] pei optional,

gpsi                        [4] gpsi optional,

guti                        [5] fivegguti optional,

location                    [6] location,

non3gppaccessendpoint       [7] ueendpointaddress optional,

rattype                     [8] rattype optional

}

-- =================

-- common parameters

-- =================

Accesstype ::= enumerated

{

threegppaccess(1),

nonthreegppaccess(2),

threegppandnonthreegppaccess(3)

}

Direction ::= enumerated

{

fromtarget(1),

totarget(2)

}

Dnn ::= utf8string

E164number ::= numericstring (size(1..15))

Emailaddress ::= utf8string

Eui64 ::= octet string (size(8))

Fivegguti ::= sequence

{

mcc         [1] mcc,

mnc         [2] mnc,

amfregionid [3] amfregionid,

amfsetid    [4] amfsetid,

amfpointer  [5] amfpointer,

fivegtmsi   [6] fivegtmsi

}

Fivegssubscriberid ::= choice

{

supi [1] supi,

suci [2] suci,

pei  [3] pei,

gpsi [4] gpsi

}

Fivegssubscriberids ::= sequence

{

fivegssubscriberid [1] sequence size(1..max) of fivegssubscriberid

}

Fivegmmcause ::= integer (0..255)

Fivegsmrequesttype ::= enumerated

{

initialrequest(1),

existingpdusession(2),

initialemergencyrequest(3),

existingemergencypdusession(4),

modificationrequest(5),

reserved(6),

mapdurequest(7)

}

Fivegsmcause ::= integer (0..255)

Fivegtmsi ::= integer (0..4294967295)

Fivegsrvccinfo ::= sequence

{

ue5gsrvcccapability   [1] boolean,

sessiontransfernumber [2] utf8string optional,

correlationmsisdn     [3] msisdn optional

}

Fivegsuserstateinfo ::= sequence

{

fivegsuserstate [1] fivegsuserstate,

accesstype      [2] accesstype

}

Fivegsuserstate ::= enumerated

{

deregistered(1),

registerednotreachableforpaging(2),

registeredreachableforpaging(3),

connectednotreachableforpaging(4),

connectedreachableforpaging(5),

notprovidedfromamf(6)

}

Fteid ::= sequence

{

teid        [1] integer (0.. 4294967295),

ipv4address [2] ipv4address optional,

ipv6address [3] ipv6address optional

}

Fteidlist ::= sequence of fteid

Gpsi ::= choice

{

msisdn      [1] msisdn,

nai         [2] nai

}

Guami ::= sequence

{

amfid       [1] amfid,

plmnid      [2] plmnid

}

Gummei ::= sequence

{

mmeid       [1] mmeid,

mcc         [2] mcc,

mnc         [3] mnc

}

Guti ::= sequence

{

mcc          [1] mcc,

mnc          [2] mnc,

mmegroupid   [3] mmegroupid,

mmecode      [4] mmecode,

mtmsi        [5] tmsi

}

Homenetworkpublickeyid ::= octet string

Hsmfuri ::= utf8string

Imei ::= numericstring (size(14))

Imeisv ::= numericstring (size(16))

Impi ::= nai

Impu ::= choice

{

sipuri [1] sipuri,

teluri [2] teluri

}

Imsi ::= numericstring (size(6..15))

Imsiunauthenticatedindication ::= boolean

Initiator ::= enumerated

{

ue(1),

network(2),

unknown(3)

}

Ipaddress ::= choice

{

ipv4address [1] ipv4address,

ipv6address [2] ipv6address

}

Ipv4address ::= octet string (size(4))

Ipv6address ::= octet string (size(16))

Ipv6flowlabel ::= integer(0..1048575)

Macaddress ::= octet string (size(6))

Macrestrictionindicator ::= enumerated

{

noresrictions(1),

macaddressnotuseableasequipmentidentifier(2),

unknown(3)

}

Mcc ::= numericstring (size(3))

Mnc ::= numericstring (size(2..3))

Mmeid ::= sequence

{

mmegi       [1] mmegi,

mmec        [2] mmec

}

Mmec ::= numericstring

Mmegi ::= numericstring

Msisdn ::= numericstring (size(1..15))

Nai ::= utf8string

Nextlayerprotocol ::= integer(0..255)

Nonlocalid ::= enumerated

{

local(1),

nonlocal(2)

}

Nonimeisvpei ::= choice

{

macaddress [1] macaddress,

eui64      [2] eui64

}

Nssai ::= sequence of snssai

Pagingrestrictionindicator ::= octet string (size(1..33))

Plmnid ::= sequence

{

mcc [1] mcc,

mnc [2] mnc

}

Pdnconnectiontype ::= enumerated

{

ipv4(1),

ipv6(2),

ipv4v6(3),

nonip(4),

ethernet(5)

}

Pdusessionid ::= integer (0..255)

Pdusessiontype ::= enumerated

{

ipv4(1),

ipv6(2),

ipv4v6(3),

unstructured(4),

ethernet(5)

}

Pei ::= choice

{

imei        [1] imei,

imeisv      [2] imeisv,

macaddress  [3] macaddress,

eui64       [4] eui64

}

Portnumber ::= integer (0..65535)

Primaryauthenticationtype ::= enumerated

{

eapakaprime(1),

fivegaka(2),

eaptls(3),

none(4),

epsaka(5),

eapaka(6),

imsaka(7),

gbaaka(8),

umtsaka(9)

}

Protectionschemeid ::= integer (0..15)

Rattype ::= enumerated

{

nr(1),

eutra(2),

wlan(3),

virtual(4),

nbiot(5),

wireline(6),

wirelinecable(7),

wirelinebbf(8),

ltem(9),

nru(10),

eutrau(11),

trustedn3ga(12),

trustedwlan(13),

utra(14),

gera(15),

nrleo(16),

nrmeo(17),

nrgeo(18),

nrothersat(19),

nrredcap(20)

}

Rejectednssai ::= sequence of rejectedsnssai

Rejectedsnssai ::= sequence

{

causevalue  [1] rejectedslicecausevalue,

snssai      [2] snssai

}

Rejectedslicecausevalue ::= integer (0..255)

Reregrequiredindicator ::= enumerated

{

reregistrationrequired(1),

reregistrationnotrequired(2)

}

Routingindicator ::= integer (0..9999)

Schemeoutput ::= octet string

Sipuri ::= utf8string

Slice ::= sequence

{

allowednssai        [1] nssai optional,

configurednssai     [2] nssai optional,

rejectednssai       [3] rejectednssai optional

}

Smpdudnrequest ::= octet string

-- ts 24.501 [13], clause 9.11.3.6.1

Smsovernasindicator ::= enumerated

{

smsovernasnotallowed(1),

smsovernasallowed(2)

}

Snssai ::= sequence

{

sliceservicetype    [1] integer (0..255),

slicedifferentiator [2] octet string (size(3)) optional

}

Subscriberidentifier ::= choice

{

suci   [1] suci,

supi   [2] supi

}

Suci ::= sequence

{

mcc                         [1] mcc,

mnc                         [2] mnc,

routingindicator            [3] routingindicator,

protectionschemeid          [4] protectionschemeid,

homenetworkpublickeyid      [5] homenetworkpublickeyid,

schemeoutput                [6] schemeoutput,

routingindicatorlength      [7] integer (1..4) optional

-- shall be included if different from the number of meaningful digits given

-- in routingindicator

}

Supi ::= choice

{

imsi        [1] imsi,

nai         [2] nai

}

Supiunauthenticatedindication ::= boolean

Switchoffindicator ::= enumerated

{

normaldetach(1),

switchoff(2)

}

Targetidentifier ::= choice

{

supi                   [1] supi,

imsi                   [2] imsi,

pei                    [3] pei,

imei                   [4] imei,

gpsi                   [5] gpsi,

msisdn                 [6] msisdn,

nai                    [7] nai,

ipv4address            [8] ipv4address,

ipv6address            [9] ipv6address,

ethernetaddress        [10] macaddress,

impu                   [11] impu,

impi                   [12] impi,

e164number             [13] e164number,

emailaddress           [14] emailaddress,

mcpttid                [15] utf8string,

instanceidentifierurn  [16] utf8string,

ptcchatgroupid         [17] ptcchatgroupid

}

Targetidentifierprovenance ::= enumerated

{

leaprovided(1),

observed(2),

matchedon(3),

other(4)

}

Teluri ::= utf8string

Timestamp ::= generalizedtime

Uecontextinfo ::= sequence

{

supportvops         [1] boolean optional,

supportvopsnon3gpp  [2] boolean optional,

lastactivetime      [3] timestamp optional,

accesstype          [4] accesstype optional,

rattype             [5] rattype optional

}

Ueendpointaddress ::= choice

{

ipv4address         [1] ipv4address,

ipv6address         [2] ipv6address,

ethernetaddress     [3] macaddress

}

Useridentifiers ::= sequence

{

fivegssubscriberids [1] fivegssubscriberids optional,

epssubscriberids    [2] epssubscriberids optional

}

-- ===================

-- location parameters

-- ===================

Location ::= sequence

{

locationinfo                [1] locationinfo optional,

positioninginfo             [2] positioninginfo optional,

locationpresencereport      [3] locationpresencereport optional,

epslocationinfo             [4] epslocationinfo optional

}

Cellsiteinformation ::= sequence

{

geographicalcoordinates     [1] geographicalcoordinates,

azimuth                     [2] integer (0..359) optional,

operatorspecificinformation [3] utf8string optional

}

-- ts 29.518 [22], clause 6.4.6.2.6

Locationinfo ::= sequence

{

userlocation                [1] userlocation optional,

currentloc                  [2] boolean optional,

geoinfo                     [3] geographicarea optional,

rattype                     [4] rattype optional,

timezone                    [5] timezone optional,

additionalcellids           [6] sequence of cellinformation optional

}

-- ts 29.571 [17], clause 5.4.4.7

Userlocation ::= sequence

{

eutralocation               [1] eutralocation optional,

nrlocation                  [2] nrlocation optional,

n3galocation                [3] n3galocation optional

}

-- ts 29.571 [17], clause 5.4.4.8

Eutralocation ::= sequence

{

tai                         [1] tai,

ecgi                        [2] ecgi,

ageoflocationinfo           [3] integer optional,

uelocationtimestamp         [4] timestamp optional,

geographicalinformation     [5] utf8string optional,

geodeticinformation         [6] utf8string optional,

globalngenbid               [7] globalrannodeid optional,

cellsiteinformation         [8] cellsiteinformation optional,

globalenbid                 [9] globalrannodeid optional

}

-- ts 29.571 [17], clause 5.4.4.9

Nrlocation ::= sequence

{

tai                         [1] tai,

ncgi                        [2] ncgi,

ageoflocationinfo           [3] integer optional,

uelocationtimestamp         [4] timestamp optional,

geographicalinformation     [5] utf8string optional,

geodeticinformation         [6] utf8string optional,

globalgnbid                 [7] globalrannodeid optional,

cellsiteinformation         [8] cellsiteinformation optional

}

-- ts 29.571 [17], clause 5.4.4.10

N3galocation ::= sequence

{

tai                         [1] tai optional,

n3iwfid                     [2] n3iwfidngap optional,

ueipaddr                    [3] ipaddr optional,

portnumber                  [4] integer optional,

tnapid                      [5] tnapid optional,

twapid                      [6] twapid optional,

hfcnodeid                   [7] hfcnodeid optional,

gli                         [8] gli optional,

w5gbanlinetype              [9] w5gbanlinetype optional,

gci                         [10] gci optional,

ageoflocationinfo           [11] integer optional,

uelocationtimestamp         [12] timestamp optional,

protocol                    [13] transportprotocol optional

}

-- ts 38.413 [23], clause 9.3.2.4

Ipaddr ::= sequence

{

ipv4addr                    [1] ipv4address optional,

ipv6addr                    [2] ipv6address optional

}

-- ts 29.571 [17], clause 5.4.4.28

Globalrannodeid ::= sequence

{

plmnid                      [1] plmnid,

annodeid                    [2] annodeid,

nid                         [3] nid optional

}

Annodeid ::= choice

{

n3iwfid [1] n3iwfidsbi,

gnbid   [2] gnbid,

ngenbid [3] ngenbid,

enbid   [4] enbid,

wagfid  [5] wagfid,

tngfid  [6] tngfid

}

-- ts 38.413 [23], clause 9.3.1.6

Gnbid ::= bit string(size(22..32))

-- ts 29.571 [17], clause 5.4.4.4

Tai ::= sequence

{

plmnid                      [1] plmnid,

tac                         [2] tac,

nid                         [3] nid optional

}

Cgi ::= sequence

{

lai    [1] lai,

cellid [2] cellid

}

Lai ::= sequence

{

plmnid [1] plmnid,

lac    [2] lac

}

Lac ::= octet string (size(2))

Cellid ::= octet string (size(2))

Sai ::= sequence

{

plmnid [1] plmnid,

lac    [2] lac,

sac    [3] sac

}

Sac ::= octet string (size(2))

-- ts 29.571 [17], clause 5.4.4.5

Ecgi ::= sequence

{

plmnid                      [1] plmnid,

eutracellid                 [2] eutracellid,

nid                         [3] nid optional

}

Tailist ::= sequence of tai

-- ts 29.571 [17], clause 5.4.4.6

Ncgi ::= sequence

{

plmnid                      [1] plmnid,

nrcellid                    [2] nrcellid,

nid                         [3] nid optional

}

Rancgi ::= choice

{

ecgi                        [1] ecgi,

ncgi                        [2] ncgi

}

Cellinformation ::= sequence

{

rancgi                      [1] rancgi,

cellsiteinformation         [2] cellsiteinformation optional,

timeoflocation              [3] timestamp optional

}

-- ts 38.413 [23], clause 9.3.1.57

N3iwfidngap ::= bit string (size(16))

-- ts 29.571 [17], clause 5.4.4.28

N3iwfidsbi ::= utf8string

-- ts 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

Tngfid ::= utf8string

-- ts 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

Wagfid ::= utf8string

-- ts 29.571 [17], clause 5.4.4.62

Tnapid ::= sequence

{

ssid         [1] ssid optional,

bssid        [2] bssid optional,

civicaddress [3] civicaddressbytes optional

}

-- ts 29.571 [17], clause 5.4.4.64

Twapid ::= sequence

{

ssid         [1] ssid optional,

bssid        [2] bssid optional,

civicaddress [3] civicaddressbytes optional

}

-- ts 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

Ssid ::= utf8string

-- ts 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

Bssid ::= utf8string

-- ts 29.571 [17], clause 5.4.4.36 and table 5.4.2-1

Hfcnodeid ::= utf8string

-- ts 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

-- contains the original binary data i.e. Value of the yaml field after base64 encoding is removed

Gli ::= octet string (size(0..150))

-- ts 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

Gci ::= utf8string

-- ts 29.571 [17], clause 5.4.4.10 and table 5.4.3.38

Transportprotocol ::= enumerated

{

udp(1),

tcp(2)

}

-- ts 29.571 [17], clause 5.4.4.10 and clause 5.4.3.33

W5gbanlinetype ::= enumerated

{

dsl(1),

pon(2)

}

-- ts 29.571 [17], table 5.4.2-1

Tac ::= octet string (size(2..3))

-- ts 38.413 [23], clause 9.3.1.9

Eutracellid ::= bit string (size(28))

-- ts 38.413 [23], clause 9.3.1.7

Nrcellid ::= bit string (size(36))

-- ts 38.413 [23], clause 9.3.1.8

Ngenbid ::= choice

{

macrongenbid                [1] bit string (size(20)),

shortmacrongenbid           [2] bit string (size(18)),

longmacrongenbid            [3] bit string (size(21))

}

-- ts 23.003 [19], clause 12.7.1 encoded as per ts 29.571 [17], clause 5.4.2

Nid ::= utf8string (size(11))

-- ts 36.413 [38], clause 9.2.1.37

Enbid ::= choice

{

macroenbid                  [1] bit string (size(20)),

homeenbid                   [2] bit string (size(28)),

shortmacroenbid             [3] bit string (size(18)),

longmacroenbid              [4] bit string (size(21))

}

-- ts 29.518 [22], clause 6.4.6.2.3

Positioninginfo ::= sequence

{

positioninfo                [1] locationdata optional,

rawmlpresponse              [2] rawmlpresponse optional

}

Rawmlpresponse ::= choice

{

-- the following parameter contains a copy of unparsed xml code of the

-- mlp response message, i.e. The entire xml document containing

-- a <slia> (described in oma-ts-mlp-v3_5-20181211-c [20], clause 5.2.3.2.2) or

-- a <slirep> (described in oma-ts-mlp-v3_5-20181211-c [20], clause 5.2.3.2.3) mlp message.

mlppositiondata             [1] utf8string,

-- oma mlp result id, defined in oma-ts-mlp-v3_5-20181211-c [20], clause 5.4

mlperrorcode                [2] integer (1..699)

}

-- ts 29.572 [24], clause 6.1.6.2.3

Locationdata ::= sequence

{

locationestimate            [1] geographicarea,

accuracyfulfilmentindicator [2] accuracyfulfilmentindicator optional,

ageoflocationestimate       [3] ageoflocationestimate optional,

velocityestimate            [4] velocityestimate optional,

civicaddress                [5] civicaddress optional,

positioningdatalist         [6] set of positioningmethodandusage optional,

gnsspositioningdatalist     [7] set of gnsspositioningmethodandusage optional,

ecgi                        [8] ecgi optional,

ncgi                        [9] ncgi optional,

altitude                    [10] altitude optional,

barometricpressure          [11] barometricpressure optional

}

-- ts 29.172 [53], table 6.2.2-2

Epslocationinfo ::= sequence

{

locationdata  [1] locationdata,

cgi           [2] cgi optional,

sai           [3] sai optional,

esmlccellinfo [4] esmlccellinfo optional

}

-- ts 29.172 [53], clause 7.4.57

Esmlccellinfo ::= sequence

{

ecgi          [1] ecgi,

cellportionid [2] cellportionid

}

-- ts 29.171 [54], clause 7.4.31

Cellportionid ::= integer (0..4095)

-- ts 29.518 [22], clause 6.2.6.2.5

Locationpresencereport ::= sequence

{

type                        [1] amfeventtype,

timestamp                   [2] timestamp,

arealist                    [3] set of amfeventarea optional,

timezone                    [4] timezone optional,

accesstypes                 [5] set of accesstype optional,

rminfolist                  [6] set of rminfo optional,

cminfolist                  [7] set of cminfo optional,

reachability                [8] uereachability optional,

location                    [9] userlocation optional,

additionalcellids           [10] sequence of cellinformation optional

}

-- ts 29.518 [22], clause 6.2.6.3.3

Amfeventtype ::= enumerated

{

locationreport(1),

presenceinaoireport(2)

}

-- ts 29.518 [22], clause 6.2.6.2.16

Amfeventarea ::= sequence

{

presenceinfo                [1] presenceinfo optional,

ladninfo                    [2] ladninfo optional

}

-- ts 29.571 [17], clause 5.4.4.27

Presenceinfo ::= sequence

{

presencestate               [1] presencestate optional,

trackingarealist            [2] set of tai optional,

ecgilist                    [3] set of ecgi optional,

ncgilist                    [4] set of ncgi optional,

globalrannodeidlist         [5] set of globalrannodeid optional,

globalenbidlist             [6] set of globalrannodeid optional

}

-- ts 29.518 [22], clause 6.2.6.2.17

Ladninfo ::= sequence

{

ladn                        [1] utf8string,

presence                    [2] presencestate optional

}

-- ts 29.571 [17], clause 5.4.3.20

Presencestate ::= enumerated

{

inarea(1),

outofarea(2),

unknown(3),

inactive(4)

}

-- ts 29.518 [22], clause 6.2.6.2.8

Rminfo ::= sequence

{

rmstate                     [1] rmstate,

accesstype                  [2] accesstype

}

-- ts 29.518 [22], clause 6.2.6.2.9

Cminfo ::= sequence

{

cmstate                     [1] cmstate,

accesstype                  [2] accesstype

}

-- ts 29.518 [22], clause 6.2.6.3.7

Uereachability ::= enumerated

{

unreachable(1),

reachable(2),

regulatoryonly(3)

}

-- ts 29.518 [22], clause 6.2.6.3.9

Rmstate ::= enumerated

{

registered(1),

deregistered(2)

}

-- ts 29.518 [22], clause 6.2.6.3.10

Cmstate ::= enumerated

{

idle(1),

connected(2)

}

-- ts 29.572 [24], clause 6.1.6.2.5

Geographicarea ::= choice

{

point                       [1] point,

pointuncertaintycircle      [2] pointuncertaintycircle,

pointuncertaintyellipse     [3] pointuncertaintyellipse,

polygon                     [4] polygon,

pointaltitude               [5] pointaltitude,

pointaltitudeuncertainty    [6] pointaltitudeuncertainty,

ellipsoidarc                [7] ellipsoidarc

}

-- ts 29.572 [24], clause 6.1.6.3.12

Accuracyfulfilmentindicator ::= enumerated

{

requestedaccuracyfulfilled(1),

requestedaccuracynotfulfilled(2)

}

-- ts 29.572 [24], clause 6.1.6.2.17

Velocityestimate ::= choice

{

horvelocity                         [1] horizontalvelocity,

horwithvertvelocity                 [2] horizontalwithverticalvelocity,

horvelocitywithuncertainty          [3] horizontalvelocitywithuncertainty,

horwithvertvelocityanduncertainty   [4] horizontalwithverticalvelocityanduncertainty

}

-- ts 29.572 [24], clause 6.1.6.2.14

Civicaddress ::= sequence

{

country                             [1] utf8string,

a1                                  [2] utf8string optional,

a2                                  [3] utf8string optional,

a3                                  [4] utf8string optional,

a4                                  [5] utf8string optional,

a5                                  [6] utf8string optional,

a6                                  [7] utf8string optional,

prd                                 [8] utf8string optional,

pod                                 [9] utf8string optional,

sts                                 [10] utf8string optional,

hno                                 [11] utf8string optional,

hns                                 [12] utf8string optional,

lmk                                 [13] utf8string optional,

loc                                 [14] utf8string optional,

nam                                 [15] utf8string optional,

pc                                  [16] utf8string optional,

bld                                 [17] utf8string optional,

unit                                [18] utf8string optional,

flr                                 [19] utf8string optional,

room                                [20] utf8string optional,

plc                                 [21] utf8string optional,

pcn                                 [22] utf8string optional,

pobox                               [23] utf8string optional,

addcode                             [24] utf8string optional,

seat                                [25] utf8string optional,

rd                                  [26] utf8string optional,

rdsec                               [27] utf8string optional,

rdbr                                [28] utf8string optional,

rdsubbr                             [29] utf8string optional,

prm                                 [30] utf8string optional,

pom                                 [31] utf8string optional

}

-- ts 29.571 [17], clauses 5.4.4.62 and 5.4.4.64

-- contains the original binary data i.e. Value of the yaml field after base64 encoding is removed

Civicaddressbytes ::= octet string

-- ts 29.572 [24], clause 6.1.6.2.15

Positioningmethodandusage ::= sequence

{

method                              [1] positioningmethod,

mode                                [2] positioningmode,

usage                               [3] usage,

methodcode                          [4] methodcode optional

}

-- ts 29.572 [24], clause 6.1.6.2.16

Gnsspositioningmethodandusage ::= sequence

{

mode                                [1] positioningmode,

gnss                                [2] gnssid,

usage                               [3] usage

}

-- ts 29.572 [24], clause 6.1.6.2.6

Point ::= sequence

{

geographicalcoordinates             [1] geographicalcoordinates

}

-- ts 29.572 [24], clause 6.1.6.2.7

Pointuncertaintycircle ::= sequence

{

geographicalcoordinates             [1] geographicalcoordinates,

-- the uncertainty parameter has been deprecated and shall be set to 0.

-- the uncertaintysbi parameter shall be used instead.

uncertainty                         [2] uncertainty,

uncertaintysbi                      [3] uncertaintysbi optional

}

-- ts 29.572 [24], clause 6.1.6.2.8

Pointuncertaintyellipse ::= sequence

{

geographicalcoordinates             [1] geographicalcoordinates,

uncertainty                         [2] uncertaintyellipse,

confidence                          [3] confidence

}

-- ts 29.572 [24], clause 6.1.6.2.9

Polygon ::= sequence

{

pointlist                           [1] set size (3..15) of geographicalcoordinates

}

-- ts 29.572 [24], clause 6.1.6.2.10

Pointaltitude ::= sequence

{

point                               [1] geographicalcoordinates,

altitude                            [2] altitude

}

-- ts 29.572 [24], clause 6.1.6.2.11

Pointaltitudeuncertainty ::= sequence

{

point                               [1] geographicalcoordinates,

altitude                            [2] altitude,

uncertaintyellipse                  [3] uncertaintyellipse,

-- the uncertaintyaltitude parameter has been deprecated and shall be set to 0.

-- the uncertaintyaltitudesbi parameter shall be used instead.

uncertaintyaltitude                 [4] uncertainty,

confidence                          [5] confidence,

uncertaintyaltitudesbi              [6] uncertaintysbi optional

}

-- ts 29.572 [24], clause 6.1.6.2.12

Ellipsoidarc ::= sequence

{

point                               [1] geographicalcoordinates,

innerradius                         [2] innerradius,

-- the uncertaintyradius parameter has been deprecated and shall be set to 0.

-- the uncertaintyradiussbi parameter shall be used instead.

uncertaintyradius                   [3] uncertainty,

offsetangle                         [4] angle,

includedangle                       [5] angle,

confidence                          [6] confidence,

uncertaintyradiussbi                [7] uncertaintysbi optional

}

-- ts 29.572 [24], clause 6.1.6.2.4

Geographicalcoordinates ::= sequence

{

latitude                            [1] utf8string,

longitude                           [2] utf8string,

mapdatuminformation                 [3] ogcurn optional

}

-- ts 29.572 [24], clause 6.1.6.2.22

Uncertaintyellipse ::= sequence

{

-- the semimajor parameter has been deprecated and shall be set to 0.

-- the semimajorsbi parameter shall be used instead.

semimajor                           [1] uncertainty,

-- the semiminor parameter has been deprecated and shall be set to 0.

-- the semiminorsbi parameter shall be used instead.

semiminor                           [2] uncertainty,

orientationmajor                    [3] orientation,

semimajorsbi                        [4] uncertaintysbi optional,

semiminorsbi                        [5] uncertaintysbi optional

}

-- ts 29.572 [24], clause 6.1.6.2.18

Horizontalvelocity ::= sequence

{

hspeed                              [1] horizontalspeed,

bearing                             [2] angle

}

-- ts 29.572 [24], clause 6.1.6.2.19

Horizontalwithverticalvelocity ::= sequence

{

hspeed                              [1] horizontalspeed,

bearing                             [2] angle,

vspeed                              [3] verticalspeed,

vdirection                          [4] verticaldirection

}

-- ts 29.572 [24], clause 6.1.6.2.20

Horizontalvelocitywithuncertainty ::= sequence

{

hspeed                              [1] horizontalspeed,

bearing                             [2] angle,

uncertainty                         [3] speeduncertainty

}

-- ts 29.572 [24], clause 6.1.6.2.21

Horizontalwithverticalvelocityanduncertainty ::= sequence

{

hspeed                              [1] horizontalspeed,

bearing                             [2] angle,

vspeed                              [3] verticalspeed,

vdirection                          [4] verticaldirection,

huncertainty                        [5] speeduncertainty,

vuncertainty                        [6] speeduncertainty

}

-- the following types are described in ts 29.572 [24], table 6.1.6.3.2-1

Altitude ::= utf8string

Angle ::= integer (0..360)

Uncertaintysbi ::= utf8string

Orientation ::= integer (0..180)

Confidence ::= integer (0..100)

Innerradius ::= integer (0..327675)

Ageoflocationestimate ::= integer (0..32767)

Horizontalspeed ::= utf8string

Verticalspeed ::= utf8string

Speeduncertainty ::= utf8string

Barometricpressure ::= integer (30000..115000)

Uncertainty ::= integer (0..127)

-- ts 29.572 [24], clause 6.1.6.3.13

Verticaldirection ::= enumerated

{

upward(1),

downward(2)

}

-- ts 29.572 [24], clause 6.1.6.3.6

Positioningmethod ::= enumerated

{

cellid(1),

ecid(2),

otdoa(3),

barometricpressure(4),

wlan(5),

bluetooth(6),

mbs(7),

motionsensor(8),

dltdoa(9),

dlaod(10),

multirtt(11),

nrecid(12),

ultdoa(13),

ulaoa(14),

networkspecific(15)

}

-- ts 29.572 [24], clause 6.1.6.3.7

Positioningmode ::= enumerated

{

uebased(1),

ueassisted(2),

conventional(3)

}

-- ts 29.572 [24], clause 6.1.6.3.8

Gnssid ::= enumerated

{

gps(1),

galileo(2),

sbas(3),

modernizedgps(4),

qzss(5),

glonass(6),

bds(7),

navic(8)

}

-- ts 29.572 [24], clause 6.1.6.3.9

Usage ::= enumerated

{

unsuccess(1),

successresultsnotused(2),

successresultsusedtoverifylocation(3),

successresultsusedtogeneratelocation(4),

successmethodnotdetermined(5)

}

-- ts 29.571 [17], table 5.2.2-1

Timezone ::= utf8string

-- open geospatial consortium urn [35]

Ogcurn ::= utf8string

-- ts 29.572 [24], clause 6.1.6.2.15

Methodcode ::= integer (16..31)

End

## **** END OF ALL CHANGES ***