| 3GPP TR 33.928 V0.0.1 (2023-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Security; ADMF Logic for Provisioning Lawful Interception (LI); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for Styles	8

4.1	Heading Styles	8

4.2	Other common styles	8

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

Annex <B>: <Informative annex title for a Technical Report>	15

Annex <C> (informative): Bibliography	16

Annex <D> (informative): Index	17

Annex <X> (informative): Change history	18

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

# Introduction

The LI technical specifications (TS 33.126 [2], TS 33.127 [3], TS 33.128 [4]) contain the normative part of the LI requirements and the technical report TR 33.929 [5] contains additional information as an implementation guidance for LI. The ADMF that receives the warrant information from the Law Enforcement Agencies has the task of provisioning the LI functions present in various Network Functions (NFs) of the serving CSP network. Upon provisioning, the LI is activated in those NFs, and accordingly, the LI functions within those NFs monitor the target’s communications and provide the LI as required by the warrant.

The scope of the NFs that provide the LI functions within the CSP network is determined based on various factors such as LI service type, CSP deployment choice, scope of LI as authorized in the warrant. The present document provides the logic used within the ADMF in provisioning the LI functions considering those points.

# 1	Scope

The present document provides ADMF provisioning logic for LI in association with the LI functions defined in TS 33.126 [2], TS 33.127 [3] and TS 33.128 [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.126: "Lawful Interception Requirements".

[3]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[4]	3GPP TS 33.128: "Lawful Interception (LI) Protocol and Procedures".

[5]	3GPP TS 33.929: "Lawful Interception (LI) Implementation Guidance".

[6]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	ADMF and Provisioning

## 4.1	Overview

According to the LI model defined in ETSI TS 103 221-1 [6], the ADMF as an administration function establishes and manages the Lawful Interception (LI). In doing so, the ADMF performs the target provisioning at various Network Elements (NEs) using the X1 protocol as defined in ETSI TS 103 221-1 [6].

Within the LI architecture model defined in TS 33.127 [3] and TS 33.128 [4], the ADMF has two sub-functions referred to as Lawful Interception Control Function (LICF) and Lawful Interception Provisioning Function (LIPF). The LICF receives the warrant information from the LEA over LI_HI1 interface. The LIPF performs the provisioning of all LI functions within various NFs of CSP network including the MDF2 and MDF3. See figure 4-1 below for an overview:

Figure 4.1-1: LIPF in ADMF provisioning of NEs

With respect to the LI model of ETSI 103 221-1 [6], the LIPF plays the role of “ADMF” (as defined in ETSI 103 221-1 [6], and the LI functions (within the NFs), MDF2, and MDF3 play the role of “NE” (as defined in ETSI 103 221-1 [6]).

The present document focusses on LIPF provisioning logic of LI functions, MDF2, MDF3 over the LI_X1. Henceforth, the term LIF logic is used in the present document. See clause 5 for details of LIPF logic.

## 4.2	General

Remark 1: A separate box is used to represent each of the NF in which an LI function is provisioned by the LIPF. In the illustration shown below in figure 4.2-1, P-CSCF and MGCF are two NFs and are represented by two separate boxes.

Figure 4.2-1: Separate box for each NF that has the LI function

Remark 2: The LI function present within a NF and provisioned by the LIPF is represented within the parenthesis. In the illustration shown in figure 4.2-2, the IRI-POI in P-CSCF and IRI-POI in MGCF are provisioned by the LIPF.

Figure 4.2-2: LI function as  applicable to the NF

Remark 3: The possible target identities that are applicable to the LI function present in a NF are represented within another parenthesis that begin with "Target Id:". In the illustration shown in figure 4.2-3, the possible target identities for an IRI-POI in P-CSCF and IRI-POI in MGCF are PEI (IMEI only), IMEI, IMPU and IMPI.

Figure 4.2-3: Possible target identifiers are in parentheses

Remark 4: Some of the flow-charts have a callout description shown next to the provisioned box. The text within the callout description provides a hint on the conditions that would enable the LI function to provide the interception. When such a condition for the interception is obvious, no such call-out description is provided. In the illustration shown in figure 4.2-4, no callout description is given for the IRI-POI of P-CSCF implying no further clarity is needed. The callout description for the IRI-POI in MGCF is to hint that the IRI-POI in an MGCF is used only when an IMS session is redirected to a CS domain (see TR 33.929 [5]).

Figure 4.2-4: Call out next to the NF describe the conditionality of interception

The conditions given inside the call-out description are outside the scope of LIPF logic. However, the LIPF logic is aware of the condition in an overall scheme of things.

Remark 5: Most of the flow-charts have follow up tables that identify the scope of NF domain in providing the LI functions.

In the illustration shown in table 4.2-1:

P-CSCF has CC-TF in a non-roaming case, has IRI-POI (for non-emergency services only) and CC-TF in VPLMN with LBO and CC-TF (for emergency services only) in VPLMN with HR (home-routed).

MGCF has CC-TF in a non-roaming case, CC-TF in HPLMN with roaming (both LBO and HR) when an incoming session is redirected over a CS domain.

IMS-AGW has CC-POI when P-CSCF has the CC-TF and IM-MGW has the CC-POI whenever the MGCF has the CC-TF.

Note that the table is to be used as an aid to understand the LIPF logic and it is outside the scope of LIPF logic. However, the LIPF logic is aware of the condition in an overall scheme of things.

Table 4.2-1: Scope of NF domain that provide the LI functions

| NFs with LI function | Non-roaming | Roaming with LBO |  | Roaming with HR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VPLMN | HPLMN | VPLMN | HPLMN |  |
| P-CSCF | n/a | IRI-POI (NOTE 1) | n/a | n/a | n/a |  |
| P-CSCF | CC-TF | CC-TF | n/a | CC-TF (NOTE 2) | n/a |  |
| IMS-AGW | CC-POI | CC-POI | n/a | CC-POI (NOTE 2) | n/a |  |
| MGCF (NOTE 3) | CC-TF | n/a | CC-TF | n/a | CC-TF |  |
| IM-MGW (NOTE 3) | CC-POI | n/a | CC-POI | n/a | CC-POI |  |

NOTE 1:	For non-emergency sessions only.

NOTE 2:	For emergency sessions only.

NOTE 3:	Only when an incoming session to a target is redirected over a CS domain.

# 5	LIPF logic

## 5.1	Background

According to TS 33.126 [2] clause 6.4, the CSP is expected to only deliver Interception Product relating to specific CSP services which are specified implicitly or explicitly in the warrant. In other words, the CSP is expected to perform the interception only for the services indicated in the warrant.

NOTE:	The term "interception" used in this annex refers to the step that involves actual capturing and then delivery of the Intercept Product to the LEMF.

This clause considers the following possibilities in the analysis:

-	The intended target may have subscribed to only a specific service and in this case, by default, the interception would apply only to such service when specified in the warrant. The CSP network would provide the interception as and when the service is accessed by the target.

-	The intended target may have subscribed to multiple services and in this case, the interception would have to be done based on the service type(s) specified in the warrant as and when CSP network detects that such services are accessed by the target.

-	A NF may be involved in providing only a particular service and in this case, by default, the interception performed by the POI present in that NF would apply to such service when specified in the warrant.

-	A NF may be involved in providing multiple services and in this case, the interception performed by the POI present in that NF would have to be based on the service type applicable to the warrant.

-	There may be multiple warrants with differing service types active on a target, in this case, all applicable services would have to be intercepted at the POIs, and the MDFs would have to then deliver Interception Product based on the service type (s) applicable to the warrant.

In supporting the above scenarios, as per clause 4.4 of TS 33.128 [4], the LIPF will have to provision the POIs, TFs and the MDF2/MDF3 according to the CSP service type(s) applicable to a warrant.

To cover all the scenarios mentioned above, the service type may have to be part of LI provisioning data sent to the MDFs. Whether a service type will have to be provisioned to the POIs and TFs as an indication will depend on the services provided by the NFs that have such POIs and TFs.

In addition to the CSP service type, a few other factors present in the warrant may influence the LIPF logic in provisioning the POIs, TFs and MDF2/MDF3. Few examples are:

-	Delivery type.

-	LALS triggering.

-	CSP deployment options.

-	The target type (local Vs non-local ID).

For the target non-local ID, Voice and Messaging type of services are supported in the present document. In this case, the other party communicating with the target non-local ID happens to access the service provided by the CSP.

This clause illustrates the LIPF logic through a series of flow-charts in provisioning the POIs and the TFs. The provisioning aspect of MDF2/MDF3 are not shown.

## 5.2	Governing scenarios

With respect to the interception performed within the CSP network, there are five scenarios:

1. The target (or party communicating with a target non-local ID) is non-roaming.

2. The target (or party communicating with a target non-local ID) is outbound roaming with HR.

3. The target (or party communicating with a target non-local ID) is outbound roaming with LBO.

4. The target (or party communicating with a target non-local ID) is inbound roaming with HR.

5. The target (or party communicating with a target non-local ID) is inbound roaming with LBO.

The scenario 4 is also referred to as N9HR or S8HR, depending on whether the packet core is 5GC or EPC. As indicated clause 5.1, a target can be a non-local ID only when the service type is Voice or Messaging.

The same NF that provides an LI function may be present in one or more of the above scenarios. The LIPF logic, even though may not be aware of the roaming nature of a target, will have to accommodate the above five scenarios while provisioning the LI functions.

## 5.3	Top-level LIPF provisioning logic

The flow-chart in figure 5.3-1 shows a top-level logic within the LIPF to branch off into separate processes according to the service type defined in the present document.

Figure 5.3-1: Top-level view of LIPF logic in handling the service type

Based on the LI functionality defined in the present document:

-	For the service type of Data, it is assumed that the NFs in the packet core network are involved and hence, provide the IRI and CC interception.

-	For the service type of Voice, it is assumed that the NFs in the IMS domain are involved and hence, provide the IRI and CC interception.

-	For the service type of Messaging (that includes SMS and MMS), the NFs in the packet core network, IMS or MMS Proxy Relay are involved and hence, provide the IRI and CC interception. The interception of SMS has only the IRIs.

-	For the service type of PTC, the PTC Server is involved and hence, provides the IRI and CC interception.

-	For the service type of LALS, the LI-LCS Client provides the IRI interception, and the CC interception does not apply to LALS.

The UDM and HSS are also the NFs that have the IRI-POI and the provisioning of IRI-POI in UDM and HSS is independent of the service type indicated in the warrant as long as the target is not indicated as a non-local ID.

## 5.4	Data

### 5.4.1	Scope of interception

For the service type of Data, the NFs present in the packet core network provide the LI functions. This annex illustrates the LIPF logic for 5GC and EPC as the two packet core networks.

The interception of service type of Data includes:

-	Delivery of IRI, or CC based on the delivery type indicated in the warrant.

-	When required, the delivery of packet data header reporting.

-	When required, the delivery of LALS reports based on the LALS triggering.

The CSP may have differing implementation options for the packet data header reporting and LALS triggering.

In the case of EPC, the CSP may also have differing deployment options in choosing the NFs (SGW-based Vs PGW-based) that provide the interception.

### 5.4.2	Top-level view

When the target identifier is one or more of IMSI, IMEI, MSISDN, the LI functions in EPC are provisioned. When the target identifier is one or more variants of SUPI, PEI, GPSI, the LI functions in 5GC are provisioned.

Figure 5.4-1 provides the top-level view of LIPF logic for the service type of Data.

Figure 5.4-1: Top-level view of LIPF logic for the service type Data

Within figure 5.4-1, PEI collectively represents PEIIMEI and PEIIMEISV. Likewise, SUPI represents SUPIIMSI and SUPINAI whereas GPSI represents GPSIMISDN and GPSINAI.

### 5.4.3	5GC

#### 5.4.3.1	The flow-chart

Figure 5.4-2 shows the LIPF logic in provisioning the LI functions for the 5GC for the service type of Data.

Figure 5.4-2: LIPF logic for the service type Data in 5GC

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs and the IRI-TFs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

#### 5.4.3.2	Interception

##### 5.4.3.2.1	PDHR

There are two deployment options for the packet data header reporting. It is expected that the CSP implements one of the two deployment options.

In PDHR option 1, the IRI-POI in the UPF (based on a trigger from IRI-TF present in the SMF) generates the xIRI. Accordingly, the IRI-TFs in SMFs are provisioned.

In PDHR option 2, the CC-POI present in the UPF (based on a trigger from CC-TF present in the SMF) delivers the UP packets as xCC to the MDF3, which in turn, forwards the same to the MDF2 and MDF2 would then generate the IRI messages from the received xCC. Accordingly, the CC-TFs in SMFs, and the PDHR handling function in MDF3 are provisioned.

##### 5.4.3.2.2	LALS triggering

There are two deployment options for LALS triggering. It is expected that the CSP implements one of the two deployment options.

In LALS triggering option 1, the LTF present in the host NF that has the associated IRI-POI triggers the LI-LCS Client. Accordingly, the LTFs in AMFs and SMFs are provisioned.

In LALS triggering option 2, the LTF present in the MDF2 triggers the LI-LCS Client. Accordingly, the LTF in MDF2 is provisioned.

##### 5.4.3.2.3	UDM

The UDM is expected to provide the IRI-POI functions in the HPLMN only.

##### 5.4.3.2.4	Summary

Table 5.4-1 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Data in the 5GC for various scenarios.

Table 5.4-1: Scope of NF domain in 5GC providing the LI functions

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| UDM |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AMF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SMF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | IRI-POI |
| SMF |  |  | CC-TF | CC-TF | n/a | CC-TF | CC-TF |
| UPF |  |  | CC-POI | CC-POI | n/a | CC-POI | CC-POI |
| PDHR | Option 1 | SMF | IRI-TF | IRI-TF | n/a | IRI-TF | IRI-TF |
|  |  | UPF | IRI-POI | IRI-POI | n/a | IRI-POI | IRI-POI |
|  | Option 2 | SMF | CC-TF | CC-TF | n/a | CC-TF | CC-TF |
|  |  | UPF | CC-POI | CC-POI | n/a | CC-POI | CC-POI |
|  |  | MDF3 | PDHR | PDHR | n/a | PDHR | PDHR |
| LALS triggering | Option 1 | AMF | LTF | LTF | n/a | LTF | n/a |
|  |  | SMF | LTF | LTF | n/a | LTF | LTF |
|  | Option 2 | MDF2 | LTF | LTF | n/a | LTF | LTF |

NOTE 1:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 2:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 3:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

### 5.4.4	EPC

#### 5.4.4.1		The flow-chart

Figure 5.4-3 shows the LIPF logic in determining the host NFs in EPC that have the LI functions for the service type of Data.

Figure 5.4-3: LIPF logic for the service type Data in EPC

For the delivery type of IRI + CC, the IRI-POIs, the CC-TFs and the CC-POIs (when EPC is deployed without CUPS) are provisioned. For the delivery type of IRI, the IRI-POIs and the IRI-TFs (when EPC is deployed with CUPS) are provisioned. For the delivery type of CC, the CC-TFs and the CC-POIs (when EPC is deployed without CUPS) are provisioned.

For the LI within the EPC, the CSP may deploy either an SGW-based interception or a PGW-based interception. The LIPF logic in supporting the two deployment options is illustrated in figure 5.4-4.

Figure 5.4-4: Two deployment options in EPC

The LIPF includes a parameter while provisioning the SGW/SGW-C and PGW/PGW-C.

To PGW/PGW-C, with the SGW based deployment option:

-	Provide LI functions only for the targets that are outbound roaming with HR.

SGW/SGW-C, with the PGW based deployment option:

-	Provide LI functions only for the targets that are inbound roaming with HR.

The above parameter is needed to avoid both SGW/SGW-C and PGW/PGW-C end up performing the interceptions for the same packet data session.

#### 5.4.4.2	Interception

##### 5.4.4.2.1	PDHR

There are two deployment options for the packet data header reporting. It is expected that the CSP implements one of the two deployment options.

In PDHR option 1, the IRI-POI present in the SGW-U/PGW-U (based on a trigger from IRI-TF present in the SGW-C/PGW-C) or the IRI-POI present in the SGW/PGW generates the xIRI. Accordingly, the IRI-TFs in SGW-C/PGW-C and IRI-POI in SGW/PGW are provisioned.

In PDHR option-2, the CC-POI present in the SGW-U/PGW-U (based on a trigger from CC-TF present in the SGW-C/PGW-C) or the CC-POI present in the SGW/PGW delivers the UP packets as xCC to the MDF3, which in turn, forward the same to the MDF2, and MDF2 then would generate the IRI messages from the received xCC. Accordingly, the CC-TF in SGW-C/PGW-C, CC-POI in SGW/PGW and the PDHR handling function in MDF3 are provisioned.

##### 5.4.4.2.2	LALS triggering

There are two deployment options for LALS triggering. It is expected that the CSP implements one of the two deployment options.

In LALS triggering option 1, the LTF present in the host NF that has the associated IRI-POI triggers the LI-LCS client. Accordingly, the LTFs in MME, SGW, PGW, SGW-C and PGW-C (when EPC is deployed with CUPS) are provisioned.

In LALS triggering option 2, the LTF present in the MDF2 triggers the LI-LCS Client. Accordingly, the LTF in MDF2 is provisioned.

##### 5.4.4.2.3	SGW/PGW deployment options

For the IRI/CC generation, two deployment options are considered:

-	SGW-based.

-	PGW-based.

The CSP expected to implement one of the two options.

When SGW/SGW-C is provisioned, if the deployment option is PGW-based approach, then the SGW/SGW-C does not perform an interception unless the target is inbound roaming with HR. Similarly, when the PGW/PGW-C is provisioned, if the deployment option is SGW-based approach, then the PGW/PGW does not perform the interception unless the target is outbound roaming with HR.

##### 5.4.4.2.4	HSS

The HSS is expected to provide the IRI-POI functions in the HPLMN only.

##### 5.4.4.2.5	Summary

Table 5.4-2 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Data in the EPC for an SGW-based deployment option.

| Table 5.4-2: Scope of NF domain in EPC providing the LI functions (SGW-based deployment)NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MME |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | CC-POI | CC-POI | n/a | CC-POI | n/a |
| PGW |  |  | n/a | n/a | n/a | n/a | IRI-POI |
| PGW |  |  | n/a | n/a | n/a | n/a | CC-POI |
| SGW-C |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW-C |  |  | CC-TF | CC-TF | n/a | CC-TF | n/a |
| SGW-U |  |  | CC-POI | CC-POI | n/a | CC-POI | n/a |
| PGW-C |  |  | n/a | n/a | n/a | n/a | IRI-POI |
| PGW-C |  |  | n/a | n/a | n/a | n/a | CC-TF |
| PGW-U |  |  | n/a | n/a | n/a | n/a | CC-POI |
| PDHR | Option 1 | SGW | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
|  |  | SGW-C | IRI-TF | IRI-TF | n/a | IRI-TF | n/a |
|  |  | SGW-U | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | IRI-POI |
|  |  | PGW-C | n/a | n/a | n/a | n/a | IRI-TF |
|  |  | PGW-U | n/a | n/a | n/a | n/a | IRI-POI |
|  | Option 2 | SGW | CC-POI | CC-POI | n/a | CC-POI | n/a |
|  |  | SGW-C | CC-TF | CC-TF | n/a | CC-TF | n/a |
|  |  | SGW-U | CC-POI | CC-POI | n/a | CC-POI | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | CC-POI |
|  |  | PGW-C | n/a | n/a | n/a | n/a | CC-TF |
|  |  | PGW-U | n/a | n/a | n/a | n/a | CC-POI |
|  |  | MDF3 | PDHR | PDHR | n/a | PDHR | PDHR |
| LALS triggering | Option 1 | SGW | LTF | LTF | n/a | LTF | n/a |
|  |  | SGW-C | LTF | LTF | n/a | LTF | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | LTF |
|  |  | PGW-C | n/a | n/a | n/a | n/a | LTF |
|  | Option 2 | MDF2 | LTF | LTF | n/a | LTF | LTF |

Table 5.4-3 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Data in the EPC for a PGW-based deployment option.

Table 5.4-3: Scope of NF domain in EPC providing the LI functions (PGW-based deployment)

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MME |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| SGW |  |  | n/a | n/a | n/a | CC-POI | n/a |
| PGW |  |  | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
| PGW |  |  | CC-POI | CC-POI | n/a | n/a | CC-POI |
| SGW-C |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| SGW-C |  |  | n/a | n/a | n/a | CC-TF | n/a |
| SGW-U |  |  | n/a | n/a | n/a | CC-POI | n/a |
| PGW-C |  |  | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
| PGW-C |  |  | CC-TF | CC-TF | n/a | n/a | CC-TF |
| PGW-U |  |  | CC-POI | CC-POI | n/a | n/a | CC-POI |
| PDHR | Option 1 | SGW | n/a | n/a | n/a | IRI-POI | n/a |
|  |  | SGW-C | n/a | n/a | n/a | IRI-TF | n/a |
|  |  | SGW-U | n/a | n/a | n/a | IRI-POI | n/a |
|  |  | PGW | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
|  |  | PGW-C | IRI-TF | IRF-TF | n/a | n/a | IRI-TF |
|  |  | PGW-U | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
|  | Option 2 | SGW | n/a | n/a | n/a | CC-POI | n/a |
|  |  | SGW-C | n/a | n/a | n/a | CC-TF | n/a |
|  |  | SGW-U | n/a | n/a | n/a | CC-POI | n/a |
|  |  | PGW | CC-POI | CC-POI | n/a | n/a | CC-POI |
|  |  | PGW-C | CC-TF | CC-TF | n/a | n/a | CC-TF |
|  |  | PGW-U | CC-POI | CC-POI | n/a | n/a | CC-POI |
|  |  | MDF3 | PDHR | PDHR | n/a | PDHR | PDHR |
| LALS triggering | Option 1 | SGW | n/a | n/a | n/a | LTF | n/a |
|  |  | SGW-C | n/a | n/a | n/a | LTF | n/a |
|  |  | PGW | LTF | LTF | n/a | n/a | LTF |
|  |  | PGW-C | LTF | LTF | n/a | n/a | LTF |
|  | Option 2 | MDF2 | LTF | LTF | n/a | LTF | LTF |

NOTE 1:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 2:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 3:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

## 5.5	Voice

### 5.5.1	Scope of interception

For the service type of Voice, the NFs present in IMS domain provide the LI functions except for the case of inbound roaming targets with home routed roaming architecture.

A target can be a subscriber of the CSP, an inbound roamer or a non-local ID. In the case where a target is a non-local ID, the party communicating with the target can be non-roaming, inbound roamer or outbound roamer. When a target is non-local ID, provisioning of HSS and LTF are not applicable.

The interception of service type of Voice includes:

-	Delivery of IRI, or CC based on the delivery type indicated in the warrant.

-	When required, the delivery of LALS reports based on the LALS triggering.

-	Whether a target is non-local ID.

-	IMS services such as redirection, conferencing, application of media such as music or announcements.

The CSP may have differing implementation options for the interception of IRI and CC, for local as well as non-local ID targets. The CSP may also have differing implementation options for LALS triggering.

The CSP may have either an LBO based roaming architecture or a home-routed based roaming architecture.

### 5.5.2	Initial configuration for N9HR/S8HR

To support the N9HR/S8HR (i.e. scenario 4 listed in clause 5.2), the LIPF will have to configure the BBIFF/BBIFF-C present in the packet core network. This applies to all inbound roaming users using the IMS service.

Figure 5.5-1 provides the LIPF logic for N9HR/S8HR that is applicable to all inbounding roaming users using IMS services.

Figure 5.5-1: LIPF logic for initial configuration for N9HR/S8HR

When the packet core network is 5GC, the BBIFF-C function is provided by the SMF and when the packet core network is EPC, the BBIFF-C/BBIFF function is provided by the SGW-C/SGW.

### 5.5.3	Top level LIPF logic for service type voice

When a target ID is an IMPU, the warrant may specify whether a target is non-local ID. The host NFs that provide the LI functions when the target is non-local ID can be different from the host NFs that provide the LI functions when the target is not a non-local ID. The provisioning of IRI-POI in HSS and provisioning of LTF in support of LALS triggering applies only when the target is not a non-local ID.

Figure 5.5-2 provides the top-level view of LIPF logic for the service type of Voice.

Figure 5.5-2: Top level view of LIPF logic for service type of Voice

Within figure 5.5-2, PEI (IMEI only) represents PEIIMEI. The target identity in the IMPI format may contain a value derived from a SUPI or an IMSI. The target identity in the IMPU format containing a SIP URI or TEL URI may contain a value derived from a GPSI, MSISDN, an E.164 number, or IMSI. Only IMPU is used for target non-local ID.

The LMISF-IRI is provisioned to support the interception of inbound roaming target (or party communicating with a target non-local ID target is inbound roaming) with HR that uses IMS services. The LMISF-IRI is expected to provide the LI functions only for non-emergency services.

### 5.5.4	LIPF logic for targets that are not non-local ID

#### 5.5.4.1	The flowchart

Figures 5.5-3, 5.5-4, 5.5-5 and 5.5-6 show the LIPF logic for the service type of Voice when the target is not a non-local ID.

Figure 5.5-3: Top level LIPF logic for service type of Voice when target is not a non-local ID

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

Figure 5.5-4 shows the LIPF logic for the provisioning of IRI-POIs and figure 5.5-5 shows the LIPF logic for the provisioning of CC-TFs.

Figure 5.5-4: LIPF logic for delivery type of IRI for service type of Voice when target is not a non-local ID

The S-CSCF, E-CSCF, P-CSCF, IBCF, MGCF and AS (in figure 5.5-4) provide IRI-POI functions under certain conditions as noted within the illustration. To prevent those IRI-POIs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning. If STIR/SHAKEN is required to be intercepted in the network, then the provisioning of IRI-POIs in IBCF and AS include the parameter ReportDiversionPASSporTInfo if the target identity is IMPU. Additional STIR/SHAKEN related provisioning is illustrated in figure 5.5-7.

Figure 5.5-5: LIPF logic for delivery type of CC for service type of Voice when target is not a non-local ID

The IBCF, MGCF and AS (in figure 5.5-5) provide CC-TF functions under certain conditions as noted within the illustration. To prevent those CC-TFs from triggering the CC-POIs when not supposed to, the LIPF may have to include a parameter during the provisioning.

Figure 5.5-6 illustrates the LIPF logic for LALS triggering.

Figure 5.5-6: LIPF logic for LALS triggering for the service type of Voice

In LALS triggering option 1, the host NF that provides the LTF is same as the NF that provides the IRI-POI functions. And therefore, the host NFs that provide the LTF can be different based on the IMS deployment options.

The P-CSCF, IBCF and LMISF-IRI (in figure 5.5-6) provide LTF under certain conditions as noted within the illustration. Under special scenario, the S-CSCF may provide the IRI-POI for emergency services (instead of E-CSCF) as specified in TS 33.127 [3]. To prevent those LTFs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

Figure 5.5-7 illustrates the LIPF logic for additional STIR/SHAKEN related provisioning.

Figure 5.5-7: LIPF logic for additional STIR/SHAKEN related provisioning

For STIR/SHAKEN related reporting, IBCF, AS, P-CSCF (VPLMN with LBO), LMISF-IRI (VPLMN with HR) provide the IRI-POI functions. The LIPF logic shown in figure 5.5-7 is additional logic required to support the LI for STIR/SHAKEN.

As illustrated in figure 5.5-4, the IRI-POIs in P-CSCF, LMISF-IRI and AS are provisioned as part IMS-based voice LI. Likewise, the IRI-POI in IBCF is also provisioned for IMS-based voice LI when the alternate option is used.

When the STIR/SHAKEN is required to be intercepted in the network, the IRI-POIs in IBCF and AS are to be provisioned with ReportDiversionPASSporTInfo value when the target identity is IMPU.

#### 5.5.4.2	Interception

##### 5.5.4.2.1	IMS deployment

There are two deployment options for IMS for intercepting the service type of Voice (TS 33.127 [3]):

-	Default.

-	Alternate option.

It is expected that the CSP implements one of the two deployment options.

The conditions under which IRI-POI or CC-TF functions have to provisioned are illustrated within the drawing and are further clarified in table 5.5-1 and 5.5-2.

##### 5.5.4.2.2	LALS triggering

There are two deployment options for LALS triggering. It is expected that the CSP implements one of the two deployment options.

In LALS triggering option 1, the LTF present in the host NF that has the associated IRI-POI triggers the LI-LCS Client. In LALS triggering option 2, the LTF presents in the MDF2 triggers the LI-LCS Client.

##### 5.5.4.2.3	Summary

Table 5.5-1 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Default.

Table 5.5-1: Scope of NF domain in IMS providing the LI functions with Default option

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 6, NOTE 12) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 7) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| MRFP (NOTE 7) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| S-CSCF (NOTE 8) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| E-CSCF (NOTE 8) |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| P-CSCF |  |  | n/a | IRI-POI (NOTE 1) | n/a | n/a | n/a |
| P-CSCF |  |  | CC-TF | CC-TF | n/a | CC-TF (NOTE 2) | n/a |
| IMS-AGW |  |  | CC-POI | CC-POI | n/a | CC-POI (NOTE 2) | n/a |
| MGCF (NOTE 3) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| IM-MGW (NOTE 3) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| IBCF (NOTE 14) |  |  | IRI-POI | IRI-POI | IRI-POI | IRI-POI | IRI-POI |
| IBCF (NOTE 4) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| TrGW (NOTE 4) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| LMISF-IRI (NOTE 1) |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC (NOTE 1) |  |  | n/a | n/a | n/a | CC-POI | n/a |
| LALS triggering | Option 1 | S-CSCF | LTF | n/a | LTF | n/a | LTF |
|  |  | E-CSCF | LTF | LTF | n/a | LTF | n/a |
|  |  | P-CSCF | n/a | LTF (NOTE 1) | n/a | n/a | n/a |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE 1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

Table 5.5-2 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Alternate option.

Table 5.5-2: Scope of NF domain in IMS providing the LI functions with Alternate option

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 6, NOTE 12) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 7) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| MRFP (NOTE 7) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| P-CSCF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI (NOTE 2) | IRI-POI |
| P-CSCF |  |  | CC-TF | CC-TF | n/a | CC-TF (NOTE 2) | CC-TF |
| IMS-AGW |  |  | CC-POI | CC-POI | n/a | CC-POI (NOTE 2) | CC-POI |
| MGCF (NOTE 3) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MGCF (NOTE 3) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| IM-MGW (NOTE 3) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| IBCF |  |  | IRI-POI (NOTE 13) | IRI-POI (NOTE 2, 14) | IRI-POI (NOTE 13) | IRI-POI (NOTE 2, 14) | IRI-POI (NOTE 13) |
| IBCF (NOTE 4) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| TrGW (NOTE4) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| LMISF-IRI (NOTE 1) |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC (NOTE 1) |  |  | n/a | n/a | n/a | CC-POI | n/a |
| LALS Triggering | Option 1 | P-CSCF | LTF | LTF | n/a | LTF (NOTE 2) | LTF |
|  |  | IBCF | n/a | n/a | LTF (NOTE 5) | n/a | n/a |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE 1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

NOTE 1:	For non-emergency sessions only.

NOTE 2:	For emergency sessions only.

NOTE 3:	Only when an incoming session to a target is redirected over a CS domain.

NOTE 4:	Only when target is outbound roaming or when an incoming session to a target is redirected over an IP domain, or to an outbound roaming party with LBO.

NOTE 5:	Only when the target is outbound roaming without a redirection.

NOTE 6:	When the interception of conferencing services is required.

NOTE 7:	When the content interception of conferencing, or application of music/ is required.

NOTE 8:	For IMS emergency sessions in fixed networks when the S-CSCF is on the signalling path, S-CSCF may optionally (instead of E-CSCF) provide the IRI-POI functions with the default option.

NOTE 9:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 10:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 11:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

NOTE 12:	When the interception of STIR/SHAKEN is required.

NOTE 13:	Only when target is outbound roaming or when an incoming session to a target is redirected over an IP domain, or to an outbound roaming party with LBO, or when the interception of STIR/SHAKEN is required.

NOTE 14:	Only when the interception of STIR/SHAKEN is required.

##### 5.5.4.2.4	STIR/SHAKEN

Since the IRI-POI in AS, in support of LI for special services such as conferencing, is always provisioned, the LIPF logic has to ensure that when the STIR/SHAKEN is required to be intercepted and the target Id is IMPU, the ReportDiversionPASSporTInfo is included as part of that provisioning.

Likewise, the IRI-POI in IBCF is also provisioned except for the case when the default option for IMS LI is deployed. The diagram shown in figure 5.5-7 illustrates that when STIR/SHAKEN is required to be intercepted and the target Id is IMPU, the IRI-POI in IBCF is provisioned even with the default option of IMS LI with ReportDiversionPASSporTInfo is included.

In general, when the STIR/SHAKEN is required to be intercepted in the network, and the target Id is IMPU, the IRI-POIs in AS and the IBCF are provisioned with ReportDiversionPASSporTInfo parameter included.

The diagram shown in figure 5.5-8 below illustrates the LI provisioning just from STIR/SHAKEN perspective. However, from an overall provisioning perspective, it is embedded within the LIPF logic of IMS LI provisioning as illustrated in clause 5.5.4.1.

Figure 5.5-8: Localized LI provisioning view from STIR/SHAKEN perspective

The inclusion of ReportDiversionPASSporTInfo for provisioning of IRI-POI in P-CSCF and LMISF-IRI is not required.

Table 5.5-3 shows the NFs that will have to provide the STIR/SHAKEN LI (signing) for various scenarios and the table 5.5-4 shows the NFs that will have to provide the STIR/SHAKEN LI (verification) for various scenarios.

The signing for STIR/SHAKEN happens in the HPLMN except for the emergency sessions it can also happen in the VPLMN. In these tables the indicated scenarios are from the perspective of target.

Table 5.5-3: Scope of NF domain in IMS providing the LI functions for STIR/SHAKEN (signing)

| Scenario |  | HPLMN |  | VPLMN |
| --- | --- | --- | --- | --- |
|  |  | CSP choice AS | CSP choice is IBCF |  |
| Emergency call |  | IBCF | IBCF | IBCF |
| RCD present |  | AS | AS | n/a |
| Intra-CSP session signing/verification required |  | AS | AS | n/a |
| Intra-CSP session signing/verification not required | Intra-CSP session | n/a | n/a | n/a |
|  | Inter-CSP session | AS | IBCF | n/a |

Table 5.5-4: Scope of NF domain in IMS providing the LI functions for STIR/SHAKEN (verification)

| Scenario |  | HPLMN |  | VPLMN |
| --- | --- | --- | --- | --- |
|  |  | CSP choice AS | CSP choice is IBCF |  |
| Emergency callback |  | AS | IBCF | See NOTE 1 |
| Inbound roaming with LBO |  | n/a | n/a | P-CSCF |
| Inbound roaming with Home-Routed |  | n/a | n/a | LMISF-IRI |
| Intra-CSP session signing/verification required |  | AS | AS | See NOTE 1 |
| Intra-CSP session signing/verification not required | Intra-CSP session | AS (see NOTE 2) | AS (see NOTE 2) | See NOTE 1 |
|  | Inter-CSP session | AS | IBCF (see NOTE 3) | See NOTE 1 |

NOTE 1: 	Same as in the row for inbound roaming (LBO) and inbound roaming (HR).

NOTE 2: 	This is the case where the redirection happens with the outgoing SIP INVITE containing the validation result and the REQUEST URI is a target identity (see TS 33.128 [4], clause 7.11.2.3). The AS may or may not interact with the Verification AS.

NOTE 3:	The IRI-POI is in IBCF. The IRI-POI can be in AS for the special redirection case depicted in NOTE 2 (see TS 33.128 [4], clause 7.11.2.3).

The indicated CSP choice is applicable when the signing/verification of only inter-CSP session is required. The CSP choice for signing and verification need not be the same.

### 5.5.5	LIPF logic for targets that are non-local ID

#### 5.5.5.1	The flowchart

Figures 5.5-8, 5.5-9 and 5.5-10 show the LIPF logic for the service type of Voice when the target is a non-local ID.

Figure 5.5-8: LIPF logic for service type of Voice when target is a non-local ID

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

Figure 5.5-9 shows the LIPF logic for the provisioning of IRI-POIs and figure 5.5-10 shows the LIPF logic for the provisioning of CC-TFs.

Figure 5.5-9: LIPF logic for delivery type of IRI for service type of Voice when target is a non-local ID

The P-CSCF, IBCF and MGCF (in figure 5.5-9) provide IRI-POI functions under certain conditions as noted within the illustration. To prevent those IRI-POIs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

If STIR/SHAKEN is required to be intercepted in the network, the provisioning of IRI-POI in IBCF include the parameter ReportDiversionPASSporTInfo. Additional STIR/SHAKEN related provisioning for target non-local ID is illustrated in figure 5.5-11.

Figure 5.5-10: LIPF logic for delivery type of CC for service type of Voice when target is a non-local ID

The P-CSCF, IBCF and MGCF (in figure 5.5-10) provide CC-TF functions under certain conditions as noted within the illustration. To prevent those CC-TFs from triggering the CC-POI when not supposed to, the LIPF may have to include a parameter during the provisioning.

Figure 5.5-11 illustrates the LIPF logic for additional STIR/SHAKEN related provisioning for target non-local ID.

Figure 5.5-11: LIPF logic for additional STIR/SHAKEN related provisioning for target non-local ID

For STIR/SHAKEN related reporting IBCF, AS, P-CSCF (VPLMN with LBO), LMISF-IRI-POI (VPLMN with HR) provide the IRI-POI functions. The LIPF logic shown in figure 5.5-11 is additional logic required to support the STIR/SHAKEN when the target is non-local ID.

As illustrated in figure 5.5-9, the IRI-POIs in P-CSCF, LMISF-IRI are provisioned as part IMS-based voice LI. Likewise, the IRI-POI in IBCF is also provisioned for IMS-based voice LI when default option is used in HPLMN, or when alternate options are used in both HPLMN and VPLMN.

When the interception of STIR/SHAKEN is required in the network, the IRI-POIs in IBCF and AS are to be provisioned with ReportDiversionPASSporTInfo value.

#### 5.5.5.2	Interception

##### 5.5.5.2.1	IMS deployment

There are two deployment options for IMS for intercepting the service type of Voice (TS 33.127 [3]):

-	Default.

-	Alternate option.

It is expected that the CSP implements one of the two deployment options.

The conditions under which IRI-POI or CC-TF functions have to provided are illustrated within the drawing and are further clarified in table 5.5-5 and 5.5-6. The LIPF may have to indicate to the IRI-POI and CC-TF the condition in which they have to provide the respective functions.

##### 5.5.5.2.2	Summary

Table 5.5-5 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Default in HPLMN.

Table 5.5-5: Scope of NF domain in IMS providing the LI functions with Default in HPLMN

| NF with LI function | Non-roaming | Roaming with LBO |  |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VPLMN |  | HPLMN | VPLMN | HPLMN |
|  |  | Default | Alternate |  |  |  |
| P-CSCF | n/a | IRI-POI | n/a | n/a | n/a | n/a |
| P-CSCF | n/a | CC-TF | n/a | n/a | n/a | n/a |
| IMS-AGW | n/a | CC-POI | n/a | n/a | n/a | n/a |
| AS (NOTE 5) | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | CC-TF | n/a | n/a | CC-TF | n/a | CC-TF |
| IM-MGW | CC-POI | n/a | n/a | CC-POI | n/a | CC-POI |
| IBCF | IRI-POI | IRI-POI (NOTE 5) | IRI-POI | IRI-POI | IRI-POI (NOTE5) | IRI-POI |
| IBCF | CC-TF | n/a | CC-TF | CC-TF | n/a | CC-TF |
| TrGW | CC-POI | n/a | CC-POI | CC-POI | n/a | CC-POI |
| LMISF-IRI | n/a | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC | n/a | n/a | n/a | n/a | CC-POI | n/a |

Table 5.5-6 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Alternate option in HPLMN.

Table 5.5-6: Scope of NF domain in IMS providing the LI functions with Alternate option in HPLMN

| NF with LI function | Non-roaming | Roaming with LBO |  |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VPLMN |  | HPLMN | VPLMN | HPLMN |
|  |  | Default | Alternate |  |  |  |
| P-CSCF | n/a | IRI-POI | n/a | n/a | n/a | n/a |
| P-CSCF | n/a | CC-TF | n/a | n/a | n/a | n/a |
| IMS-AGW | n/a | CC-POI | n/a | n/a | n/a | n/a |
| AS (NOTE 5) | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| S-CSCF | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | CC-TF | n/a | n/a | CC-TF | n/a | CC-TF |
| IM-MGW | CC-POI | n/a | n/a | CC-POI | n/a | CC-POI |
| IBCF (NOTE 5) | IRI-POI | IRI-POI | IRI-POI | IRI-POI | IRI-POI | I-IRI-POI |
| IBCF | CC-TF | n/a | CC-TF | CC-TF | n/a | CC-TF |
| TrGW | CC-POI | n/a | CC-POI | CC-POI | n/a | CC-POI |
| LMISF-IRI | n/a | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC | n/a | n/a | n/a | n/a | CC-POI | n/a |

NOTE 1:	In tables 5.5-5 and 5.5-6, the use of the phrase non-roaming/roaming applies to the party communicating with the target non-local ID.

NOTE 2:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 3:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 4:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

NOTE 5:	Only when the interception of STIR/SHAKEN is required.

##### 5.5.5.2.3	STIR/SHAKEN

When the target is non-local ID, the IRI-POI in AS is not provisioned for IMS-LI. The IRI-POI in IBCF is provisioned except for the case where the alternate option is deployed in the HPLMN for IMS LI (target non-local ID) and default option is used for IMS-LI in the VPLMN (target non-local ID).

The diagram shown in figure 5.5-11 illustrates that when the interception of STIR/SHAKEN is required in the network, and the target Id is IMPU, and the target is non-local ID, the IBCF is provisioned even when the alternate option for IMS LI is used in the HPLMN and default option is used for IMS LI in the VPLMN.

The diagram shown in figure 5.5-7 (clause 5.5.4.1) illustrates the LI provisioning just from STIR/SHAKEN perspective. However, from an overall provisioning perspective for target non-local ID, it is embedded within the LIPF logic of IMS LI provisioning as illustrated in clause 5.5.5.1.

Table 5.5-7 shows the NFs that will have to provide the STIR/SHAKEN LI (signing) for various scenarios and table 5.5-8 shows the NFs that will have to provide the STIR/SHAKEN LI (verification) for various scenarios.

The signing for STIR/SHAKEN happens in the HPLMN. In these tables the indicated scenarios are from the perspective of the party communicating with the target non-local ID.

Table 5.5-7: Scope of NF domain in IMS providing the LI functions for STIR/SHAKEN (signing)

| Scenario |  | CSP choice AS | CSP choice is IBCF |
| --- | --- | --- | --- |
| RCD present |  | AS | AS |
| Intra-CSP session |  | n/a | n/a |
| Inter-CSP session | Intra-CSP session signing/verification is required | AS | AS |
|  | Intra-CSP session signing/verification not required | AS | IBCF |

Table 5.5-8: Scope of NF domain in IMS providing the LI functions for STIR/SHAKEN (verification)

| Scenario |  | HPLMN |  | VPLMN |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CSP choice AS | CSP choice is IBCF | Default | Alternate |
| Inbound roaming with LBO |  | n/a | n/a | P-CSCF | IBCF |
| Inbound roaming with Home-Routed |  | n/a | n/a | LMISF-IRI | LMISF-IRI |
| Intra-CSP session |  | n/a | n/a | n/a | n/a |
| Inter-CSP session | Intra-CSP session signing/verification is required | AS | AS | See NOTE | See NOTE |
|  | Intra-CSP session signing/verification not required | AS | IBCF | See NOTE | See NOTE |

NOTE:	Same as in the rows for inbound roaming (LBO) and inbound roaming (HR).

The indicated CSP choice is applicable when the signing/verification of only inter-CSP session is required. The CSP choice for signing and verification need not be the same.

## 5.6	Messaging

### 5.6.1	Scope of interception

The illustrations shown in the annex for the service type Messaging includes:

-	MMS.

-	SMS.

The interception for the MMS is done by the IRI-POI and CC-POI present in the MMS Proxy Relay. The interception for the SMS is done by the IRI-POI present in the SMSF and the MME (when MME provides the SMS service) and the IMS domain for SMS over IMS.

A target can be a subscriber of the CSP, an inbound roamer or a non-local ID. In the case where a target is a non-local ID, the party communicating with the target can be non-roaming, inbound roamer or outbound roamer. When a target is non-local ID, provisioning of HSS and LTF are not applicable.

For N9HR/S8HR, the LI functions for the inbound roamers are provided in the LMISF-IRI. To support the interception in LMISF-IRI, the initial configuration for N9HR/S8HR will have to be done as illustrated in clause 5.5.2.

The interception of service type of Messaging includes:

-	Delivery of IRI or CC based on the delivery type indicated in the warrant (the delivery of CC is applicable to MMS only).

-	Whether a target is non-local ID.

-	When required, the delivery of LALS reports based on the LALS triggering (applicable to SMS only).

In view of SMS over IMS, the CSP may have either an LBO based roaming architecture or a home-routed based roaming architecture. The CSP may have differing implementation options for LALS triggering.

Table 5.6-1 shows the target identities that are applicable to different type of SMS use.

Table 5.6-1: Target IDs as applicable to the interception of service type Messaging

| SMS method | GPSI | SUPI | PEI | MSISDN | IMSI | IMEI | IMPU | IMPI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MMS | YES | n/a | n/a | YES | n/a | n/a | n/a | n/a |
| MMS (target non-local ID) | YES | n/a | n/a | YES | n/a | n/a | n/a | n/a |
| SMS over IMS (local) | YES (NOTE) | As IMPI | As IMEI | YES (NOTE) | As IMPI | YES | YES | YES |
| SMS over IMS (non-local ID) | YES | n/a | n/a | YES | n/a | n/a | YES | n/a |
| SMS over 5GS | YES | YES | YES | As GPSI | As SUPI | As PEI | n/a | n/a |
| SMS over 5GS (target non-local ID) | YES | n/a | n/a | As GPSI | n/a | n/a | n/a | n/a |
| SMS over EPS | n/a | n/a | n/a | YES | YES | YES | n/a | n/a |
| SMS over EPS (target non-local ID) | n/a | n/a | n/a | YES | n/a | n/a | n/a | n/a |

The target identity PEI collectively represents PEIIMEI and PEIIMEISV. Likewise, SUPI represents SUPIIMSI and SUPINAI whereas GPSI represents GPSIMISDN and GPSINAI. The target identity in the IMPI format may contain a value derived from a SUPI or an IMSI. The target identity in the IMPU format containing a SIP URI or TEL URI may contain a value derived from a GPSI, MSISDN, an E.164 number, or IMSI.

NOTE:	The GPSI and MSISDN may also be the target IDs as an IMPU.

A part of LIPF logic is based on the target identity applicability shown in table 5.6-1.

### 5.6.2	LIPF logic for service type messaging

#### 5.6.2.1	Flowcharts

Figure 5.6-1 provides the top-level view of LIPF logic for the service type of Messaging.

Figure 5.6-1: Top-level view of LIPF logic for service type of Messaging

The IRI-POI in HSS, UDM and LMISF-IRI are provisioned only when a target is not a non-local ID.

Figure 5.6-2 shows the LIPF logic for service type Messaging with SMS over IMS.

Figure 5.6-2: LIPF logic for service type of Messaging for SMS over IMS

The P-CSCF (in figure 5.6-2) provides IRI-POI functions under certain conditions as noted within the illustration. To prevent the IRI-POI in P-CSCF from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

Figure 5.6-3 shows the LIPF logic for LALS triggering with service type of Messaging.

Figure 5.6-3: LIPF logic for LALS triggering for the service type of Messaging

The P-CSCF (in figure 5.6-3) provides LTF functions under certain conditions as noted within the illustration. To prevent LTF in P-CSCF from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

#### 5.6.2.2	Interception

##### 5.6.2.2.1	IMS deployment

There are two deployment options for IMS for intercepting the service type of Messaging for SMS over IMS:

-	Default.

-	Alternate option.

It is expected that the CSP implements one of the two deployment options.

The conditions under which IRI-POI functions have to be provisioned are illustrated within the drawing and are further clarified in tables from 5.6-3 to 5.6-6.

##### 5.6.2.2.2	LALS triggering

There are two deployment options for LALS triggering. It is expected that the CSP implements one of the two deployment options.

In LALS triggering option 1, the LTF present in the host NF that has the associated IRI-POI triggers the LI-LCS Client. In LALS triggering option 2, the LTF presents in the MDF2 triggers the LI-LCS Client.

##### 5.6.2.2.3	Summary

Table 5.6-2 provides the scope of NF domain that provides the IRI-POI/CC-POI/LTF functions for the service type of Messaging.

Table 5.6-2: Scope of NF domain providing the LI functions for service type Messaging

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| MMS Proxy Relay |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MMS Proxy Relay |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| SMSF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| MME (with SMS service) |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| IMS NFs |  |  | See table 5.6-3, table 5.6-4, table 5.6.5 and table 5.6.6. |  |  |  |  |
| LALS triggering | Option 1 | SMSF | LTF | LTF | n/a | LTF | n/a |
|  |  | MME | LTF | LTF | n/a | LTF | n/a |
|  |  | IMS NFs | See table 5.6-3 and table 5.6.4 |  |  |  |  |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

Table 5.6-3 provides the scope of NF domain in the IMS that provides the IRI-POI/LTF for SMS over IMS when the target is not a non-local ID with default IMS deployment option.

Table 5.6-3: Scope of NF domain in IMS providing the LI functions for SMS over IMS (local ID) with Default

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| P-CSCF |  |  | n/a | IRI-POI (NOTE 1) | n/a | n/a | n/a |
| E-CSCF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| S-CSCF |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| LMISF-IRI |  |  | n/a | n/a | n/a | IRI-POI (NOTE 1) | n/a |
| LALS triggering | Option 1 | P-CSCF | n/a | LTF (NOTE 1) | n/a | n/a | n/a |
|  |  | E-CSCF | n/a | LTF | n/a | LTF | n/a |
|  |  | S-CSCF | LTF | n/a | LTF | n/a | LTF |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

Table 5.6-4 provides the scope of NF domain in the IMS that provides the IRI-POI/LTF for SMS over IMS when the target is not a non-local ID with Alternate IMS deployment option.

Table 5.6-4: Scope of NF domain in IMS providing the LI functions for SMS over IMS (local ID) with Alternate option

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| P-CSCF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI (NOTE 2) | IRI-POI |
| IBCF |  |  | n/a | n/a | IRI-POI | n/a | n/a |
| LMISF-IRI |  |  | n/a | n/a | n/a | IRI-POI (NOTE 1) | n/a |
| LALS triggering | Option 1 | P-CSCF | LTF | LTF | n/a | LTF (NOTE 2) | n/a |
|  |  | IBCF | n/a | n/a | LTF | n/a | n/a |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE 1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

Table 5.6-5 provides the scope of NF domain in the IMS that provides the IRI-POI/LTF for SMS over IMS when the target is a non-local ID with default IMS deployment option.

Table 5.6-5: Scope of NF domain in IMS providing the LI functions for SMS over IMS (non-local ID) with Default

| NFs with LI function | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| P-CSCF | n/a | IRI-POI | n/a | n/a | n/a |
| S-CSCF | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| LMISF-IRI | n/a | n/a | n/a | IRI-POI | n/a |

Table 5.6-6 provides the scope of NF domain in the IMS that provides the IRI-POI/LTF for SMS over IMS when the target is a non-local ID with Alternate IMS deployment option.

Table 5.6-6: Scope of NF domain in IMS providing the LI functions for SMS over IMS (non-local ID) with Alternate option

| NFs with LI function | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| P-CSCF | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
| IBCF | n/a | n/a | IRI-POI | n/a | n/a |
| LMISF-IRI | n/a | n/a | n/a | IRI-POI | n/a |

NOTE 1:	Only for SMS over IMS not involving the emergency centre.

NOTE 2:	Only for SMS over IMS to emergency centres.

NOTE 3:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 4:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions

NOTE 5:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

## 5.7	PTC

### 5.7.1	Scope of interception

The illustrations shown in this clause are for the service type PTC.

The interception of service type of PTC is done by the IRI-POI and CC-POI present in the PTC server and includes delivery of IRI or CC based on the delivery type indicated in the warrant.

The following target identifiers are applicable to the service type of PTC for provisioning the IRI-POI/CC-POI at the PTC Server:

-	IMPU.

-	IMPI.

-	MCPTT ID.

-	Instant ID URN.

-	Chat group ID.

The following target identifiers are applicable to the service type of PTC for provisioning the IRI-POI at the HSS:

-	IMPU.

-	IMPI.

The target identity in the IMPU format may contain a SIP URI, TEL URI.

### 5.7.2	LIPF logic for service type of PTC

Figure 5.7-1 illustrates the LIPF logic for the provisioning of IRI-POI/CC-POI in the PTC server.

Figure 5.7-1: LIPF logic for provisioning the IRI-POI/CC-POI in PTC Server for the service type of PTC

When the Chat Group ID is used as the target ID, the PTC server happens to intercept when the indicated chat group ID is used for a PTC chat session.

## 5.8	LALS

### 5.8.1	Scope of interception

The illustrations shown in this clause are for the service type LALS.

The interception of service type of LALS includes the delivery of target’s location information either immediate (at the time of provisioning) or periodic by the LI-LCS Client.

### 5.8.2	LIPF logic for service type of LALS

Figure 5.8-1 illustrates the LIPF logic for the provisioning of LI-LCS Client.

Figure 5.8-1: LIPF logic the provisioning of LI-LCS Client for the service type of LALS

Annex <C> (informative):
Bibliography

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | SA3LI#89 | S3i230xxx |  |  |  | Initial Version | 0.0.1 |