3GPP TSG-SA3 Meeting #90-LI	s3i230419

# Prague, Czech Republic, 27th Jun 2023 - 30th Jun 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0557 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UDM Authentication Response modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2023-06-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Needed a method of reporting AAA authentication to assist with identifying the access method of the target. Additionally, identifying remote server information for SNPN access was needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added fields to the UDMUEAuthenticationResponse record to indicate AAA authentication and addressing information for SNPN onboarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM Authentication reporting would not include AAA authentication information or remote server identifying information for SNPN access. CSPs may not be able to fulfill LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3.8, Annex A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Merge Request: !195 Commit Hash: 311d63195f609dd7f15b5cb4ad391718cb40b99a |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i230393 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ***

##### 7.2.2.3.8	UE Authentication response

The IRI-POI in the UDM shall generate an xIRI containing the UDMUEAuthenticationResponse record when it detects the following events:

-	When the UDM receives the AuthenticationInfoRequest from the AUSF as part of Nudm_UEAuthentication service operation (see TS 29.503 [25], clause 6.3.6.2.2) and the UDM sends the AuthenticationInfoResult to the AUSF as part of the Nudm_UEAuthentication service operation (see TS 29.503 [25], clause 6.3.6.2.3).

-	When the UDM receives the HSSAuthenticationInfoRequest from the HSS as part of the Nudm_UEAuthentication service operation (see TS 29.503 [25], clause 6.3.6.2.10) and the UDM sends the HSSAuthenticationInfoResult to the AUSF as part of the Nudm_UEAuthentication service operation (see TS 29.503 [25], clause 6.3.6.2.11).

When a target UE registers from both 3GPP and non-3GPP access, two separate xIRIs each containing the UDMUEAuthentication report record may be generated by the IRI-POI in the UDM.

Table 7.2.2.3.8-1: Payload for UDMUEAuthenticationResponse record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| sUPI | SUPI | 1 | SUPI currently associated with the target UE, see TS 29.571 [17]. | M |
| authenticationInfoRequest | UDMAuthenticationInfoRequest | 1 | Indicateds information provided in the UEAuthenticationInfoRequest. See Table 7.2.2.3.8-2 for details of payload. | M |
| aKMAIndicator | BOOLEAN | 0..1 | Indicates whether AKMA keys are needed for the UE, shall be included if AKMA keys are requested in the AuthenticationInfoRequest. | C |
| problemDetails | UDMProblemDetails | 0..1 | Shall Indicate reason for AuthenticationInfoResultfailure. Shall be included if failure occurs. See TS 29.571 [17], clause 5.2.4.1. | C |
| authAAA | BOOLEAN | 0..1 | Boolean value that indicates whether authentication is required to be perfromed using AAA as sent in the UEAuthenticationInfoResult. Included when present in the AuthenticationInfoResult. See TS 29.503 [25], clause 6.3.6.2.3. | C |
| pvsInfo | ServerAddressingInfoList | 0..1 | Provides remote provisioning server information when the PLMN is used for target UE SNPN onboarding. Include when known at the NF See TS 29.503 [25] clause 6.3.6.2.3. | C |

Table 7.2.2.3.8-2: Payload for UDMAuthenticationInfoRequest parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| infoRequestType | UDMInfoRequestType, | 1 | Indicates whether the AuthenticationInfoRequest was sent by the HSS, AUSF or other. | M |
| rGAuthCtx | SEQUENCE (SIZE (1..MAX)) OF SubscriberIdentifier | 1..MAX | Contains the UE ID (i.e. SUPI, SUCI) provided in the authentication indication, at least one shall be present. | M |
| authType | PrimaryAuthenticationType | 1 | Indicates the authentication method provided by the HSS or AUSF in the AuthenticationInfoRequest. | M |
| servingNetworkName | PLMNID | 1 | Serving network name. See TS 33.501 [11] clause 6.1.1.4. | M |
| aUSFInstanceID | NFID | 0..1 | Identifies the AUSF instance which generated the AuthenticationInformatoinRequest. Shall be included if known. | C |
| cellCAGInfo | CAGID | 0..1 | Provides CAG cell information (e.g. CAGId) if UE is attempting registration from a CAG. | C |
| n5GCIndicator | BOOLEAN | 0..1 | Boolean value that indicates whether the device is a N5GC device. Include if provided in the AuthenticationInfoRequest. | C |

## **** END OF MAIN DOCUMENTS CHANGES ***

## **** START OF ATTACHMENT CHANGE ***

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -2700,7 +2700,9 @@ UDMUEAuthenticationResponse ::= SEQUENCE

2700	2700	    sUPI                        [1] SUPI,

2701	2701	    authenticationInfoRequest   [2] UDMAuthenticationInfoRequest,

2702	2702	    aKMAIndicator               [3] BOOLEAN OPTIONAL,

2703	-	    problemDetails              [4] UDMProblemDetails OPTIONAL

-	2703	    problemDetails              [4] UDMProblemDetails OPTIONAL,

-	2704	    authAAA                     [5] BOOLEAN OPTIONAL,

-	2705	    pvsInfo                     [6] ServerAddressingInfoList OPTIONAL

2704	2706	}

2705	2707

2706	2708	UDMStartOfInterceptionWithRegisteredTarget ::= SEQUENCE

@@ -2710,6 +2712,17 @@ UDMStartOfInterceptionWithRegisteredTarget ::= SEQUENCE

2710	2712	    uDMSubscriptionDataSets     [3] SBIType

2711	2713	}

2712	2714

-	2715	ServerAddressingInfoList ::= SEQUENCE (SIZE(1..MAX)) OF ServerAddressingInfo

-	2716

-	2717	ServerAddressingInfo ::= SEQUENCE

-	2718	{

-	2719	    iPv4Address [1] IPv4Address OPTIONAL,

-	2720	    iPv6Address [2] IPv6Address OPTIONAL,

-	2721	    fQDNList    [3] FQDNList OPTIONAL

-	2722	}

-	2723

-	2724	FQDNList ::= SEQUENCE (SIZE(1..MAX)) OF FQDN

-	2725

2713	2726	-- =================

2714	2727	-- 5G UDM parameters

2715	2728	-- =================

## **** END OF ATTACHMENT CHANGES ***

## **** END OF ALL CHANGES ***