3GPP TSG-SA3 Meeting #91-LI	s3i230594

# Sydney, Australia, 24th Oct 2023 - 27th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0595 | rev | 0 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing parameters to SUCI definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The currently defined format for the SUCI is based on a format that was deprecated in TS 23.003 version 15.5.0. The current format for SUCI cannot report SUCIs based on SUPIs in the form of an NAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes were made to the format of the SUCI defined in the ASN.1: - Addition of a conditional SUPI type field, with the assumption being that if it is absent, the supi type is zero - Addition of a conditional Home Network Identifier field, which may be omitted if supi type is zero and the MCC/MNC fields populated instead. - Explain that if the supi type is not zero, the MCC/MNC can be populated with any value and ignored by the LEMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SUCI will not be able to be reported if the SUCI was generated based on a SUPI in the form of an NAI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Attachment TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !228 Commit hash: 44e4cce36424f3d8e30cf1a1685e51ef2717a960 CRs 592, 593, and 594 are mirrors of this CR. Tables for the ASN.1 in this CR may be found in CR 0572. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i230571 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -5576,6 +5576,8 @@ HandoverType ::= ENUMERATED

5576	5576		    fiveGStoUTRA(4)

5577	5577		}

5578	5578

5579	+	HomeNetworkIdentifier ::= UTF8String

5580	+

5579	5581		HomeNetworkPublicKeyID ::= OCTET STRING

5580	5582

5581	5583		HSMFURI ::= UTF8String

@@ -5890,9 +5892,11 @@ SUCI ::= SEQUENCE

5890	5892		    protectionSchemeID          [4] ProtectionSchemeID,

5891	5893		    homeNetworkPublicKeyID      [5] HomeNetworkPublicKeyID,

5892	5894		    schemeOutput                [6] SchemeOutput,

5893		-	    routingIndicatorLength      [7] INTEGER (1..4) OPTIONAL

5894		-	       -- shall be included if different from the number of meaningful digits given

5895		-	       -- in routingIndicator

5895	+	    -- shall be included if different from the number of meaningful digits given

5896	+	    -- in routingIndicator

5897	+	    routingIndicatorLength      [7] INTEGER (1..4) OPTIONAL,

5898	+	    sUPIType                    [8] SUPIType OPTIONAL,

5899	+	    homeNetworkIdentifier       [9] HomeNetworkIdentifier OPTIONAL

5896	5900		}

5897	5901

5898	5902		SUPI ::= CHOICE

@@ -5903,6 +5907,8 @@ SUPI ::= CHOICE

5903	5907

5904	5908		SUPIUnauthenticatedIndication ::= BOOLEAN

5905	5909

5910	+	SUPIType ::= INTEGER (0..7)

5911	+

5906	5912		SupportedTAList ::= SEQUENCE (SIZE(1..MAX)) OF TAItem

5907	5913

5908	5914		SwitchOffIndicator ::= ENUMERATED

## **** END OF ALL CHANGES ****