3GPP SA3LI#91	S3i230604

# Sydney; October 24-27, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0577 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RCS: Fixing a few suspected editorial errors in stage 3 text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell, AT&T) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Few editorial errors are suspected in the RCS related text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Suspected editorials are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RCS stage 3 text is not consistent within the same document. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.13.3.4.3.1, 7.13.3.4.4.1, 7.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i230524 |  |  |  |  |  |  |  |  |

### ** First Change **

###### 7.13.3.4.3.1	RCSSessionModification record

The IRI-POI in the RCS Server shall generate an RCSSessionModification record when the IRI-POI in the RCS Server detects any of the following:

-	A request is sent to request the next leg of a SIP Session or a response is received establishing a SIP Session for the transfer of a Large Message Mode CPM Standalone message or a CPM 1-to-1 Chat Session.

-	Apreviously established SIP session for the transfer of a Large Message Mode CPM Standalone message to or from a target has been modified (see clause 7.13.3.4.3.2).

-	A CPM 1-to-1 Chat Session established for the target's communications has been modified (see clause 7.13.3.4.3.3).

Table 7.13.3.4.3-1: Payload for RCSSessionModification record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| rCSTargetIdentities | SEQUENCE OF RCSIdentity | 1..MAX | RCS target identities. All identities associated to the target known at the POI shall be included. | M |
| conversationID | RCSConversationID | 1 | Set to the value of the Conversion-ID header in the original SIP INVITE request. | M |
| contributionID | RCSContributionID | 1 | Set to the value of the Contribution-ID header in the original SIP INVITE request. | M |
| inReplyToContributionID | RCSContributionID | 0..1 | InReplyTo-Contribution-ID identifying the Contribution-ID of the CPM Standalone Message, CPM File Transfer or CPM Session that is being replied to (see OMA-TS-CPM_Conversation_Function [109] clause 5.3). Shall be included if the InReplyTo-Contribution-ID header field is present for the message being reported. | C |
| sessionReplaces | RCSContributionID | 0..1 | The Contribution-ID present in the Session-Replaces header of the SIP INVITE identifying the Contribution-ID of the CPM 1-to-1 Chat Session that is being replaced to (see OMA-TS-CPM_Conversation_Function [109] clause 5.3). Shall be included if the Session-Replaces header field is present for the message being reported. | C |
| rCSSessionType | RCSSessionType | 1 | Indicates the type of RCSSession. | M |
| sessionDirection | Direction | 1 | Shall be provided to identify the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| sessionEndpoints | RCSSessionEndpoints | 1 | Indicates whether the session continues through the server or is terminated at the server. | M |
| rCSSIPSessionMessage | RCSSIPSessionMessage | 1 | Shall contain the SIP message that triggered the xIRI, an indication of whether the the establishment or removal of a leg has been attempted or completed. | M |
| location | Location | 0..1 | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

### ** Next Change **

###### 7.13.3.4.4.1	RCSSessionRelease record

The IRI-POI in the RCS Server shall generate an RCSSessionRelease record when the IRI-POI in the RCS Server detects any of the following:

-	A SIP Session for the transfer of a Large Message Mode CPM Standalone message to or from a target has been released (see clause 7.13.3.4.4.2).

-	A CPM 1-to-1 Chat Session established for the target's communications has been released (see clause 7.13.3.4.4.3).

Table 7.13.3.4.4-1: Payload for RCSSessionRelease record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| rCSTargetIdentities | SEQUENCE OF RCSIdentity | 1..MAX | RCS target identities. All identities associated to the target known at the POI shall be included. | M |
| conversationID | RCSConversationID | 1 | Set to the value of the Conversion-ID header in the original SIP INVITE request. | M |
| contributionID | RCSContributionID | 1 | Set to the value of the Contribution-ID header in the original SIP INVITE request. | M |
| rCSSessionType | RCSSessionType | 1 | Indicates the type of RCSSession. | M |
| sessionDirection | Direction | 1 | Shall be provided to identify the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| sessionEndpoints | RCSSessionEndpoints | 1 | Indicates whether the session continued through the server or is terminated at the server. | M |
| rCSSIPSessionMessage | RCSSIPSessionMessage | 1 | Shall contain the SIP message that triggered the xIRI, an indication of whether the the establishment or removal of a leg has been attempted or completed. | M |
| location | Location | 0..1 | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

### ** Next Change **

#### 7.13.4	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the RCS server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the RCS server event was observed (i.e. the timestamp field of the xIRI).

Tables 7.13.4-1 shows the IRI type (see ETSI TS 102 232-1 [9] clause 5.2.10) to be used for each record type.

Table 7.13.4-1: IRI type for messages

| Record type | IRI Type |
| --- | --- |
| RCSRegistration | REPORT |
| RCSCapabilityDiscovery | REPORT |
| RCSMessage | REPORT |
| RCSSessionEstablishmentAttempt | REPORT |
| RCSSessionModification | REPORT |
| RCSSessionRelease | REPORT |

### ** End of all Changes **