3GPP TSG-SA3 Meeting #91-LI-e-b	s3i230711

# Online, , 27th Nov 2023 - 28th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0601 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution for the delivery of RCS CC from the CC-POI in the RCS Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution provides details on the generation and delivery of xCC from the CC-POI in the RCS Server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds details for the generation and delivery of xCC from the CC-POI in the RCS Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RCS content will not be deliverable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !235 Commit hash: b94382423f29fab6883ecaa81b0fa740409b2447 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i230708 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

### 7.13.Y	Generation of xCC over LI_X3

#### 7.13.Y.1	General

As described in TS 33.127 [5] clause 7.13, content for RCS may be present in the RCS Server, the HTTP Content Server or the File Localisation Server.

The interception of content at the RCS Server is covered in clause 7.13.Y.2. When interception at the RCS Server is not possible (e.g. when the RCS Server is provided by a third party), RCS Content may alternatively be intercepted in IMS as described in clause 7.12.6.

The interception of content at the HTTP Content Server is described in clause 7.13.Y.3.

The interception of content at the File Localisation Server is described in clause 7.13.Y.4.

#### 7.13.Y.2	Generation of xCC at the CC-POI in the RCS Server

The CC-POI present in the RCS Server shall send xCC over LI_X3 for any RCS event where content is available reporting for the events listed in TS 33.127 [5] clause 7.13.4.1.

When the RCS contents consist of a SIP message, the xCC payload shall consist of the RCS contents given as a SIP message as described in ETSI TS 103 221-2 [8] clause 5.4.10. The payload format shall be set to "SIP Message" (value 9).

When the RCS contents consist of an MSRP Message, the xCC payload shall consist of the RCS contents given as an MSRP Message as described in ETSI TS 103 221-2 [8] clause 5.4.14. The payload format shall be set to "MSRP Message" (value 13).

When the RCS contents consist of a MIME Entity, the xCC payload shall consist of the RCS contents given as a MIME encoded document as described in ETSI TS 103 221-2 [8] clause 5.4.16. The payload format shall be set to "MIME Message" (value 15).

#### 7.13.Y.3	Generation of xCC at the CC-POI in the HTTP Content Server

The interception of content at the HTTP Content Server is not described in the present document.

#### 7.13.Y.4	Generation of xCC at the CC-POI in the File Localisation Server

The interception of content at the File Localisation Server is not described in the present document.

#### 7.13.Y.5	Payload direction

The CC-POI shall set the payload direction to indicate the direction of the message containing the content being reported as xCC delivered to the MDF3 as described in ETSI TS 103 221-2 [8] clause 5.2.6.

### 7.13.Z	Generation of CC over LI_HI3

When xCC is received over LI_X3 from a CC-POI, the MDF3 shall deliver the CC over LI_HI3 to the LEMF according to the clause 5.5.3 of the present document without undue delay.

The MDF3 shall populate the threeGPP33128DefinedCC field with a CCPDU structure containing RCSCCPDU.

When the payload format of the xCC received over LI_X3 is "SIP Message" (value 9), the CC shall be mediated as RCSCCPDU.encapsulatedRCSPayload.sIP.

When the payload format of the xCC received over LI_X3 is "MSRP Message" (value 13), the CC shall be mediated as  RCSCCPDU.encapsulatedRCSPayload.mSRP.

When the payload format of the xCC received over LI_X3 is "MIME Message" (value 15), the CC shall be mediated as RCSCCPDU.encapsulatedRCSPayload.mIME.

The MDF3 shall populate the timeStamp field of the ETSI TS 102 232-1 [9] PSHeader structure of CC with the xCC timeStamp and the Payload Direction of the CCPayload structure to reflect the value received on xCC. The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

## **** END OF MAIN DOCUMENT CHANGES ****

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -514,8 +514,8 @@ CCPDU ::= CHOICE

514	514		    -- r16 is used in cCPayloadOID.

515	515		    nIDDCCPDU           [4] NIDDCCPDU,

516	516		    pTCCCPDU            [5] PTCCCPDU,

517		-

518		-	    iMSCCPDU            [6] IMSCCPDU

517	+	    iMSCCPDU            [6] IMSCCPDU,

518	+	    rCSCCPDU            [7] RCSCCPDU

519	519		}

520	520

521	521		-- ===========================

@@ -4382,6 +4382,10 @@ RCSCapabilityDiscovery ::= SEQUENCE

4382	4382		    location                      [5] Location OPTIONAL

4383	4383		}

4384	4384

4385	+	RCSCCPDU ::= CHOICE

4386	+	{

4387	+	    encapsulatedRCSPayload [1] EncapsulatedRCSPayload

4388	+	}

4385	4389

4386	4390		-- ==============

4387	4391		-- RCS Parameters

## **** END OF ALL CHANGES ****