3GPP TSG-SA3 Meeting #92-LI	s3i240065

# Sevilla, Spain, 30th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0227 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ProSe LI reporting at the UDM Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently no solution for ProSe LI exists in TS 33.127. This CR add capability to report target ProSe activity at the UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two new IRIs to clause 7.2.2.4, claify behaviour of the serving system xIRI generation for ProSe UE authinticating using a relay in a VPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for ProSe. CSPs will not be able to meet LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related Stage 3 found in CR0602 (s3i240005) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i240006 |  |  |  |  |  |  |  |  |

******* START OF CHANGES*******

******* START OF FIRST CHANGE*******

#### 7.2.2.4	IRI events

The IRI-POI present in the UDM shall generate xIRI, when the UDM detects the following specific events or information:

-	Serving system.

-	Subscriber record change.

-	Cancel location.

-	Location information request.

-	Location information result.

-	UE information response.

-	UE authentication response.

-	Start of interception with target already registered at the UDM.

-	ProSe target identifier deconcealment.

-	ProSe target authentication.

A serving system xIRI is generated when the IRI-POI present in the UDM detects the target UE registration or re-registration related notifications. The AMF Id or the MME Id, or the VPLMN Id (when the other two are not known) is used as the serving system identifier in a serving system xIRI. A serving system xIRI is also generated when the IRI-POI in the UDM detects that the target ProSe remote UE has utilized a ProSe relay with existing connection to a PLMN that is not the UEs HPLMN or an E-HPLMN to succeessfuly perform authentication.

NOTE:	The serving system xIRI may carry the information of one or more serving systems based on the target UE's network connectivity.

A subscriber record change xIRI is generated when the IRI-POI present in the UDM detects that the GPSI, or SUPI, or PEI associated to the target has changed. In addition, a subscriber record change xIRI is generated when the associated GPSI or, SUPI, or PEI for the target is de-provisioned. A subscriber record change xIRI is also generated when the target’s user service identifiers are modified (e.g. subscribed S-NSSAIs, subscribed CAG).

A cancel location xIRI is generated when the IRI-POI present in the UDM detects that a de-registration notification is sent, or received, by the UDM for the target. A cancel location xIRI is also generated when the IRI-POI present in the UDM detects that the UDM has sent a cancel location indicator to the AMF due to target de-registration.

A location information request xIRI is generated when the IRI-POI present in the UDM detects that the UDM received a query for the location information of the target UE from a different PLMN (e.g. inbound SMS routing) with a known PLMN Id.

A location information result xIRI is generated when the IRI-POI in the UDM detects that the UDM received a LocationInfoRequest from an NF service consumer (i.e. HSS) for the target and responds with a LocationInfoResult to the NF service consumer.

A UE information response xIRI is generated when the IRI-POI present in the UDM detects that the UDM received a ProvideUeInfo request for the target UE and returns a UeInfo response.

A UE authentication response xIRI is generated when the IRI-POI present in the UDM detects that the UDM received an authentication info request for the target UE from the HSS or AUSF and an authentication info result is sent.

A start of interception with already registered target xIRI is generated with the IRI-POI in the UDM detects that interception is activated on a identifier that has existing registration context information at the UDM.

A ProSe target identifier deconcealment xIRI is generated when the IRI-POI in the UDM detects that the UDM has responded to a request from the NF consumer (i.e. the 5G PKMF) to perform SUCI to SUPI deconcealment for a ProSe remote UE.

A ProSe target authentication record xIRI is generated when the IRI-POI present in the UDM detects that the UDM has responded to an authentication request from the AUSF for a ProSe remote UE.

******* END OF FIRST CHANGE*******

******* END OF ALL CHANGES*******