3GPP SA3#94-LI	s3i240475

# 9-12 June 2024, Amsterdam, Netherlands (NL)

Title:	Reply LS on enhancement to the protocol stack of IMS Data Channel

Response to:	S4-241373 LS on enhancement to the protocol stack of IMS Data Channel

Release:	Rel-18

Work Item:	5G_MEDIA_MTSI_ext

Source:	SA3-LI

To:	SA4

Cc:	SA3

Contact person:

Name: 	Tyler Hawbaker

E-mail Address: 	thawbaker@fbi.gov

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

SA3-LI thanks SA4 for the LS on enhancement to the protocol stack of IMS Data Channel. SA3-LI provides the following response:

Question 1: Would it be beneficial to be able to activate/deactivate IMS media plane encryption from lawful interception perspective?

Answer 1: The capability to activate/deactivate IMS media plane confidentiality protection is mandatory from a lawful interception perspective. In roaming cases, this disabling applies on a per roaming agreement basis, meaning that the entirety of the IMS traffic flow regardless of LI targeting is not confidentiality protected when the roaming agreement requires. Current S8/N9 Home routed roaming LI capabilities defined in TS 33.127/33.128 require media plane encryption [confidentiality protection] of IMS to be turned off due to the current IMS security architecture design limiting capability to provide hop-by-hop media protection. SA3-LI has responded to similar questions from GSMA NG/UPG (see LS out s3i240070, attached in this response) pointing to the continued requirement for IMS Data Channel.

From a general sense, SA3-LI maintains the requirement that, when present, media shall be delivered either in unencrypted format, or with the means and instructions to decrypt the encrypted media and communications related information. If current protocol stack design does not allow the latter, then the CSP shall have the means to disable such encryption.

SA3-LI has also concluded that while encryption includes both confidentiality and integrity protection, SA3-LI only requires confidentiality protection disabling mechanisms while preserving integrity protection.

# 2	Actions

To SA4

ACTION: SA3-LI kindly asks SA4 to take the answer into account when determining protocol stack enhancements.

# 3	Dates of next SA3-LI meetings

SA3#95-LI            29 October – 1 November 2024	Las Vegas, NV (US)

SA3#96-LI	 28 – 31 January 2025		Sophia Antipolis, France