SA3#95-LI	 S3i240623

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – MMS background with scope of LI -Part II

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS providing at a high level view of LI scope.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. This pCR gives complements the background, functional elements, and the interfaces with a high level view of LI scope.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

### 4.2.1	Background

***** New text ****

### 4.2.2	LI scope

For LI, the interfaces MM1, MM4 and the functional elements MMS Relay/Server (with the name MMS Relay-Proxy) and MMS User Agent are referenced in TS 33.128 [4]. All other interfaces and the functional elements depicted in figure 4.2.1.1-1 are out of scope even though the MM messages to and from the MMS User Agents may traverse through other interfaces to other functional entities (e.g. MM3 interface to/from External Email Server, MM7 interface from VAS applications).

Within the CSP's MMS domain, the MMS Relay/Server provides the IRI-POI and CC-POI functions. The IRI-POI and CC-POIs are provisioned with the target identifiers and deliver the xIRIs and the xCC to the MDF2 and MDF3 respectively. The MDF2 delivers the IRI messages to the LEMF over the LI_HI2 interface and MDF3 delivers the CC over the LI_HI3 interface to the LEMF.

The following aspects are applicable to MMS LI, partly, as specified in TS 33.127 [4] and TS 33.128 [4]:

- The target identities can be an E.164 number, an Email Address, GPSI (MSISDN, NAI), IMPI, IMPU, IMSI and SUPI (IMSI, NAI). See NOTE 1.

- The Email Address can only be used as a target non-local ID.

- The MM message originated from an MMS User Agent or destined to an MMS User Agent are routed through the MMS Relay/Server.

- The MM message originated from an Email Client or destined to an Email Client may not go through the MMS Relay/Server.

- An incoming MMS session can be forwarded by the receiving MMS User Agent, or by the receiving Email Client, or can be redirected by the receiving MMS Relay/Server that serves the MMS User Agent.

- MM messages can be stored at, uploaded to, retrieved from, or deleted from the MMBox for the MMS User Agent.

The originating, redirecting, or redirected-to MMS User Agent can be a target or target non-local ID in the immediate prior or next network. When MMS User Agent is roaming, the MMS LI is applicable only to the HPLMN.

NOTE 1:	As per TS 23.140 [18], only a phone number and an Email Address are the identities used in the MM messages. In the list, MSISDN, GPSI (MSISDN) and E.164 numbers represent the phone number.

Editor's Note: The shaded text can be removed if the CRs in S3i240614 to S3i240619 are approved.

The scope of LI based on the nature of the target is illustrated in figure 4.2.2-1 below:

Figure 4.2.2-1: Scope of LI within the MMS Relay/Server

NOTE 2:	The Email Server shown in figure 4.2.2-1 can be different for Email Client AA and Email Client BB.

NOTE 3:	MMS User Agent (A) and MMS User Agent (B) may be served by the same MMS Relay/Server.

When the target is identified with a phone number (GPSI (MSISDN), MSISDN or E.164 number) and is local to the MMS Relay/Server, the LI functions use the messages from the MM1 interface for generating the xIRIs.

When the target is identified with a phone number (GPSI (MSISDN), MSISDN or E.164number) and is non-local to MMS Relay/Server, the LI functions use the messages from the MM4 interface for generating the xIRIs.

When the target non-local ID is identified with an Email Address, the LI functions use the messages from MM1 interface transferred to or from the MM3 interface.

The Email Client interacts with the Email Server (shown as External Server in figure 4.2.2-1) to send or receive an email message. If an MM message does not pass through the MMS Relay/Server, then such an MM message cannot be intercepted. For example, as shown in the figure 4.2.2-1, an email message transferred between the two Email Clients does not hit the MMS Relay/Server at either end of the flow, and therefore, such email messages are not intercepted.

When either end of an email message is an MMS User Agent, then that MM message does pass through the MMS Relay/Server and hence, can be intercepted.

TS 33.128 refers to the OMA specification [19] for the MM1 message details and refer to TS 23.140 [18] for the MM4 message details. The MM1 messages may also be uploaded to, stored at, or retrieved from the MMBox (not shown in figure 4.2.2-1).

***** End of the new text ****