Prose LI – Announcing UE and Monitoring UE scnarios

Group A: Announcement UE is not roaming

Scenario 1: Monitoring UE is also non-roaming. PLMN of Announcement UE and Monitoring UE are the same.

Scenario 2:   Monitoring UE is also non-roaming. PLMN of Announcement UE and Monitoring UE are different.

Scenario 3:   Monitoring UE is roaming. PLMN of Announcement UE and VPLMN of monitoring UE are the same.

Scenario 4:   Monitoring UE is roaming. PLMN of Announcement UE and VPLMN of monitoring UE are different.

Scenario 5:   Monitoring UE is roaming. PLMN of Announcement UE and HPLMN of monitoring UE are the same.

Group B: Announcement UE is roaming

Scenario 1: Monitoring UE is non-roaming. VPLMN of Announcement UE and PLMN of Monitoring UE are the same.

Scenario 2:   Monitoring UE is non-roaming. VPLMN of Announcement UE and PLMN of Monitoring UE are different.

Scenario 3:   Monitoring UE is roaming. VPLMN of Announcement UE and VPLMN of monitoring UE are the same. HPLMN of Announcement UE and HPLMN of monitoring UE are the same.

Scenario 4:   Monitoring UE is roaming. VPLMN of Announcement UE and VPLMN of monitoring UE are the same. HPLMN of Announcement UE and HPLMN of monitoring UE are different.

Scenario 5:   Monitoring UE is roaming. VPLMN of Announcement UE and VPLMN of monitoring UE are different. HPLMN of Announcement UE and HPLMN of monitoring UE are the same.

Scenario 5:   Monitoring UE is roaming. VPLMN of Announcement UE and VPLMN of monitoring UE are different. HPLMN of Announcement UE and HPLMN of monitoring UE are different.

Group A: Announcement UE is not roaming.

Scenario 1: Monitoring UE is also not roaming

Announcement UE and Monitoring UE are in the same network. Both are targets.

| - Ann-UE gets the code from DDNMF. xIRI generated (DDNMF) for target Ann-UE. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF for authorization (w/ the code). - DDNMF responds to the Mon-UE with the authorization success. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE gets the code from DDNMF. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF (HPLMN) for authorization (w/ the code). xIRI generated (DDNMF) for target Mon-UE. - DDNMF (HPLMN) responds to the Mon-UE with the authorization success. xIRI generated (DDNMF) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 2: Monitoring UE is also not roaming.

Announcement UE and Monitoring UE are in the different networks. Both are targets.

| - Ann-UE gets the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). - DDNMF-2 interacts with the DDNMF-1 to confirm. - DDNMF-2 responds to the Mon-UE with the authorization success. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE gets the code from DDNMF-1. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). xIRI generated (DDNMF-2) for target Mon-UE. - DDNMF-2 interacts with the DDNMF-1 to confirm. xIRI generated (DDNMF-2)for target Mon-UE. - DDNMF-2 responds to the Mon-UE with the authorization success. xIRI generated (DDNMF-2) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 3: Monitoring UE is roaming.

VPLMN (Monitoring UE) is same as Announcement UE’s PLMN.

Both are targets (Mon-UE is a target in HPLMN and VPLMN).

| - Ann-UE gets the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). - DDNMF-2 interacts with the DDNMF-1 to confirm. - DDNMF-2 responds to the Mon-UE with the authorization success. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE gets the code from DDNMF-1. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). xIRI generated (DDNMF-2) for target Mon-UE. - DDNMF-2 interacts with the DDNMF-1 to confirm. xIRIs generated (DDNMF-2, DDNMF-1) for target Mon-UE. - DDNMF-2 responds to the Mon-UE with the authorization success. xIRI generated (DDNMF-2) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 4: Monitoring UE is roaming.

VPLMN is different from Announcement UE’s PLMN.

Both are targets (Mon-UE is a target in HPLMN and VPLMN).

| - Ann-UE gets the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). - DDNMF-2 interacts with the DDNMF-1 to confirm. - DDNMF-2 responds to the Mon-UE with the authorization success. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE gets the code from DDNMF-1. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). xIRI generated (DDNMF-2) for target Mon-UE. - DDNMF-2 interacts with the DDNMF-1 to confirm. xIRIs generated (DDNMF-2) for target Mon-UE. - DDNMF-2 responds to the Mon-UE with the authorization success. xIRI generated (DDNMF-2) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 5: Monitoring UE is roaming.

HPLMN (Mon-UE) and PLMN of Announcement UE are the same.

Both are targets (Mon-UE is a target in HPLMN and VPLMN).

| - Ann-UE gets the code from DDNMF. xIRI generated (DDNMF) for target Ann-UE. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF for authorization (w/ the code). - DDNMF responds to the Mon-UE with the authorization success. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE gets the code from DDNMF. - Ann-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF for authorization (w/ the code). xIRI generated (DDNMF) for target Mon-UE. - DDNMF responds to the Mon-UE with the authorization success. xIRI generated (DDNMF) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Group B: Announcement UE is roaming.

Scenario 1: Monitoring UE is not roaming

VPLMN (Ann-UE) and PLMN (Mon-UE) are the same.  Both are targets (Ann-UE is target in VPLMN and HPLMN)

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-2 for authorization (w/ the code). xIRI generated (DDNMF-2) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 2: Monitoring UE is not roaming

VPLMN (Ann-UE) and PLMN (Mon-UE) are different networks.  Both are targets (Ann-UE is target in VPLMN and HPLMN)

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). - DDNMF-3 interacts with the DDNMF-2 to confirm. - DDNMF-3 responds to Mon-UE with the authorization. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). xIRI generated (DDNMF-3) for target Mon-UE. - DDNMF-3 interacts with the DDNMF-2 to confirm. xIRI generated (DDNMF-3) for target Mon-UE. - DDDNMF-3 responds to the Mon-UE with the authorization. xIRI generated (DDNMF-3) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 3: Monitoring UE is roaming.

VPLMN of Mon-UE and Ann-UE are the same, HPLMN of Mon-UE and Ann-UE are the same.

Both are targets (Mon-UE, Ann-UE are  targets in HPLMN and VPLMN).

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-1 for authorization (w/ the code). - DDNMF-1 interacts with the DDNMF-2 to confirm. - DDNMF-1 responds to Mon-UE with the authorization. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-1 for authorization (w/ the code). xIRI generated (DDNMF-1) for target Mon-UE. - DDNMF-1 interacts with the DDNMF-2 to confirm. xIRI generated (DDNMF-1, DDNMF-2) for target Mon-UE. - DDNMF-1 responds to the Mon-UE with the authorization. xIRI generated (DDNMF-1) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 4: Monitoring UE is roaming.

VPLMN of Mon-UE and Ann-UE are the same, HPLMN of Mon-UE and Ann-UE are different.

Both are targets (Mon-UE, Ann-UE are  targets in HPLMN and VPLMN).

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). - DDNMF-3 interacts with the DDNMF-2 to confirm. - DDNMF-3 responds to Mon-UE with the authorization. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). xIRI generated (DDNMF-3) for target Mon-UE. - DDNMF-3 interacts with the DDNMF-2 to confirm. xIRI generated (DDNMF-3, DDNMF-2) for target Mon-UE. - DDNMF-3 responds to the Mon-UE with the authorization. xIRI generated (DDNMF-3) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 5: Monitoring UE is roaming.

HPLMN (Mon-UE) and HPLMN (Ann-UE) are the same. VPLMN (Mon-UE) and VPLMN (Ann-UE) are different.

Both are targets (Mon-UE and Ann-UE are targets in the respective HPLMN and VPLMN).

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-1 for authorization (w/ the code). - DDNMF-1 interacts with the DDNMF-2 to confirm. - DDNMF-1 responds to Mon-UE with the authorization. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-1 for authorization (w/ the code). xIRI generated (DDNMF-1) for target Mon-UE. - DDNMF-1 interacts with the DDNMF-2 to confirm. xIRI generated (DDNMF-1) for target Mon-UE. - DDNMF-1 responds to the Mon-UE with the authorization. xIRI generated (DDNMF-1) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |

Scenario 6: Monitoring UE is roaming.

HPLMN (Mon-UE) and HPLMN (Ann-UE) are different. VPLMN (Mon-UE) and VPLMN (Ann-UE) are different.

Both are targets (Mon-UE and Ann-UE are targets in the respective HPLMN and VPLMN).

| - Ann-UE requests for the code from DDNMF-1. xIRI generated (DDNMF-1) for target Ann-UE. - DDNMF-1 interacts with the DDNMF-2 for authorization. xIRI generated (DDNMF-1, DDNMF-2) for target Ann-UE. - DDNMF-1 responds to Ann-UE with the code. xIRI generated (DDNMF-1) for target Ann-UE. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). - DDNMF-3 interacts with the DDNMF-2 to confirm. - DDNMF-3 responds to Mon-UE with the authorization. - Mon-UE and Ann-UE have ProSe connection. | - Ann-UE requests for the code from DDNMF-1. - DDNMF-1 interacts with the DDNMF-2 for authorization. - DDNMF-1 responds to Ann-UE with the code. - ANN-UE announces to the Mon-UE with the code. - Mon-UE requests the DDNMF-3 for authorization (w/ the code). xIRI generated (DDNMF-3) for target Mon-UE. - DDNMF-3 interacts with the DDNMF-2 to confirm. xIRI generated (DDNMF-3) for target Mon-UE. - DDNMF-3 responds to the Mon-UE with the authorization. xIRI generated (DDNMF-3) for target Mon-UE. - Mon-UE and Ann-UE have ProSe connection. |
| --- | --- |