3GPP TSG-SA3 Meeting #96-LI	s3i250056

# Sophia-Antipolis, France, 28th Jan 2025 - 31st Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0704 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF enhancement for LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-01-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | SMF enhancement for LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.1; 6.2.3.7; Attachment TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/307 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/6d5aef1897cb38d69275a3d365e1e2131aa5dffd |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i250019 |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 6.2.3.7	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the SMF or the IRI-POI in the UPF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the SMF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.2.3.7-1.

Table 6.2.3.7-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| SMFPDUSessionEstablishment | BEGIN |
| SMFPDUSessionRelease | END |
| SMFPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedPDUSession | BEGIN |
| SMFUnsuccessfulProcedure | REPORT |
| SMFMAPDUSessionEstablishment | BEGIN |
| SMFMAPDUSessionRelease | END |
| SMFMAPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedMAPDUSession | BEGIN |
| SMFMAUnsuccessfulProcedure | REPORT |
| SMFPDUtoMAPDUSessionModification | CONTINUE |
| PDHeaderReport | REPORT |
| PDSummaryReport | REPORT |
| SMFRemoteUEReport | REPORT |
| SMFStartOfInterceptionWithAlreadyConnectedRemoteUE | REPORT |
| SMFUEToNetworkRelayWithRemoteUEsReport | REPORT |
| SMFStartOfInterceptionForUEToNetworkRelay | REPORT |

IRI messages associated with the same PDU Session shall be assigned the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field (see ETSI TS 102 232-7 [10] clause 15) of the LI_HI2 message shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record and the SMFStartOfInterceptionWithEstablishedMAPDUSession record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the SMFStartOfInterceptionWithEstablishedPDUSession record is specified in table 6.2.3.2.5-1, while the payload of the SMFStartOfInterceptionWithEstablishedMAPDUSession record is specified in table 6.2.3.4-1. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record for each of the established PDU sessions to the LEMF associated with the new warrant. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedMAPDUSession record for each of the established MA PDU sessions to the LEMF associated with the new warrant.

If the MDF2 did not receive a previous SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment for the same session from the IRI-POI, , the MDF2 shall set the value of the SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment to the time provided in the timestamp previously received in the header of the related SMFPDUSessionEstablishment or SMFMAPDUSessionEstablishment xIRI.

When the delivery of packet header information is authorised and approach 2 described in clause 6.2.3.9.1 is used, the MDF2 shall generate the IRI message and send it over LI_HI2 without undue delay when xCC is received over LI_MDF from the MDF3. The MDF2 shall generate packet header information reporting as described in clause 6.2.3.5.

END OF THIRD CHANGE

START OF CHANGE 1

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -304,7 +304,13 @@ XIRIEvent ::= CHOICE

304	304		    iMSHSSSubscriberRecordChange                        [174] IMSHSSSubscriberRecordChange,

305	305

306	306		    -- AMF events, see clause 6.2.2.2.14, continued from tag 147

307		-	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate

307	+	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate,

308	+

309	+	    -- SMF events, see clause 6.2.3.2 continued from tag 61

310	+	    sMFRemoteUEReport                                   [176] SMFRemoteUEReport,

311	+	    sMFStartOfInterceptionWithAlreadyConnectedRemoteUE  [177] SMFStartOfInterceptionWithAlreadyConnectedRemoteUE,

312	+	    sMFUEToNetworkRelayWithRemoteUEsReport              [178] SMFUEToNetworkRelayWithRemoteUEsReport,

313	+	    sMFStartOfInterceptionForUEToNetworkRelay           [179] SMFStartOfInterceptionForUEToNetworkRelay

308	314		}

309	315

310	316		-- ==============

@@ -592,7 +598,13 @@ IRIEvent ::= CHOICE

592	598		    iMSHSSSubscriberRecordChange                        [174] IMSHSSSubscriberRecordChange,

593	599

594	600		    -- AMF events, see clause 6.2.2.3, continued from tag 147

595		-	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate

601	+	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate,

602	+

603	+	    -- SMF events, see clause 6.2.3.7 continued from tag 61

604	+	    sMFRemoteUEReport                                   [176] SMFRemoteUEReport,

605	+	    sMFStartOfInterceptionWithAlreadyConnectedRemoteUE  [177] SMFStartOfInterceptionWithAlreadyConnectedRemoteUE,

606	+	    sMFUEToNetworkRelayWithRemoteUEsReport              [178] SMFUEToNetworkRelayWithRemoteUEsReport,

607	+	    sMFStartOfInterceptionForUEToNetworkRelay           [179] SMFStartOfInterceptionForUEToNetworkRelay

596	608		}

597	609

598	610		IRITargetIdentifier ::= SEQUENCE

@@ -2609,6 +2621,47 @@ SMFMAUnsuccessfulProcedure ::= SEQUENCE

2609	2621		    sMPDUDNRequest              [17] SMPDUDNRequest OPTIONAL

2610	2622		}

2611	2623

2624	+	SMFRemoteUEReport ::= SEQUENCE

2625	+	{

2626	+	    sUPI                        [1] SUPI,

2627	+	    gPSI                        [2] GPSI OPTIONAL,

2628	+	    pEI                         [3] PEI OPTIONAL,

2629	+	    pDUSessionID                [4] PDUSessionID,

2630	+	    remoteUEContextConnected    [5] RemoteUEContext OPTIONAL,

2631	+	    remoteUEContextDisconnected [6] RemoteUEContext OPTIONAL,

2632	+	    location                    [7] Location OPTIONAL

2633	+	}

2634	+

2635	+	SMFStartOfInterceptionWithAlreadyConnectedRemoteUE ::= SEQUENCE

2636	+	{

2637	+	    sUPI                     [1] SUPI,

2638	+	    gPSI                     [2] GPSI OPTIONAL,

2639	+	    pEI                      [3] PEI OPTIONAL,

2640	+	    pDUSessionID             [4] PDUSessionID,

2641	+	    remoteUEContextConnected [5] RemoteUEContextList OPTIONAL,

2642	+	    location                 [6] Location OPTIONAL

2643	+	}

2644	+

2645	+	SMFUEToNetworkRelayWithRemoteUEsReport ::= SEQUENCE

2646	+	{

2647	+	    uEToNetworkRelaySUPI         [1] SUPI,

2648	+	    uEToNetworkRelayGPSI         [2] GPSI OPTIONAL,

2649	+	    uEToNetworkRelayPEI          [3] PEI OPTIONAL,

2650	+	    pDUSessionID                 [4] PDUSessionID,

2651	+	    remoteUEsContextConnected    [5] RemoteUEContextList OPTIONAL,

2652	+	    remoteUEsContextDisconnected [6] RemoteUEContextList OPTIONAL,

2653	+	    location                     [7] Location OPTIONAL

2654	+	}

2655	+

2656	+	SMFStartOfInterceptionForUEToNetworkRelay ::= SEQUENCE

2657	+	{

2658	+	    uEToNetworkRelaySUPI      [1] SUPI,

2659	+	    uEToNetworkRelayGPSI      [2] GPSI OPTIONAL,

2660	+	    uEToNetworkRelayPEI       [3] PEI OPTIONAL,

2661	+	    pDUSessionID              [4] PDUSessionID,

2662	+	    remoteUEsContextConnected [5] RemoteUEContextList OPTIONAL,

2663	+	    location                  [6] Location OPTIONAL

2664	+	}

2612	2665

2613	2666		-- =================

2614	2667		-- 5G SMF parameters

@@ -2918,6 +2971,92 @@ EASServerAddress ::= SEQUENCE

2918	2971		-- See table 5.4.2.1 of TS 29.571 [17]

2919	2972		GEOSatelliteID ::= UTF8String

2920	2973

2974	+	RemoteUEContextList ::= SEQUENCE

2975	+	{

2976	+	    numberOfRemoteUEContexts [1] INTEGER,

2977	+	    remoteUEContexts         [2] SET OF RemoteUEContext

2978	+	}

2979	+

2980	+	RemoteUEContext ::= SEQUENCE

2981	+	{

2982	+	    remoteUEIDFormat       [1] RemoteUEIDFormat,

2983	+	    remoteUEIDType         [2] RemoteUEIDType,

2984	+	    remoteUEID             [3] RemoteUEID,

2985	+	    uDPPortRangeIndicator  [4] BOOLEAN,

2986	+	    tCPPortRangeIndicator  [5] BOOLEAN,

2987	+	    protocolUsedByRemoteUE [6] ProtocolUsedByRemoteUE,

2988	+	    addressInformation     [7] AddressInformation OPTIONAL,

2989	+	    hPLMNID                [8] PLMNID OPTIONAL

2990	+	}

2991	+

2992	+	ProtocolUsedByRemoteUE ::= ENUMERATED

2993	+	{

2994	+	    noIPInfo(1),

2995	+	    iPv4(2),

2996	+	    iPv6(3),

2997	+	    unstructured(4),

2998	+	    ethernet(5)

2999	+	}

3000	+

3001	+	RemoteUEIDFormat ::= ENUMERATED

3002	+	{

3003	+	    nAI(1),

3004	+	    sixtyFourBitString(2)

3005	+	}

3006	+

3007	+	RemoteUEIDType ::= ENUMERATED

3008	+	{

3009	+	    uPPRUKID(1),

3010	+	    cPPRUKID(2),

3011	+	    iMEI(3),

3012	+	    iMEISV(4)

3013	+	}

3014	+

3015	+	RemoteUEID ::= CHOICE

3016	+	{

3017	+	    uPPRUKIDNAI         [1] NAI,

3018	+	    uPPRUKID64BitString [2] BIT STRING (SIZE(64)),

3019	+	    cPPRUKIDNAI         [3] NAI,

3020	+	    cPPRUKID64BitString [4] BIT STRING (SIZE(64)),

3021	+	    iMEI                [5] IMEI,

3022	+	    iMEISV              [6] IMEISV

3023	+	}

3024	+

3025	+	AddressInformation ::= CHOICE

3026	+	{

3027	+	    iPv4Address                [1] IPv4Address,

3028	+	    iPv4AddressUDPPortRange    [2] IPv4AddressUDPPortRange,

3029	+	    iPv4AddressTCPPortRange    [3] IPv4AddressTCPPortRange,

3030	+	    iPv4AddressUDPTCPPortRange [4] IPv4AddressUDPTCPPortRange,

3031	+	    iPv6Address                [5] IPv6Address,

3032	+	    ethernetAddress            [6] MACAddress

3033	+	}

3034	+

3035	+	IPv4AddressUDPPortRange ::= SEQUENCE

3036	+	{

3037	+	    iPv4Address         [1] IPv4Address,

3038	+	    uDPPortRange        [2] PortRange

3039	+	}

3040	+

3041	+	IPv4AddressTCPPortRange ::= SEQUENCE

3042	+	{

3043	+	    iPv4Address         [1] IPv4Address,

3044	+	    tCPPortRange        [2] PortRange

3045	+	}

3046	+

3047	+	IPv4AddressUDPTCPPortRange ::= SEQUENCE

3048	+	{

3049	+	    iPv4Address         [1] IPv4Address,

3050	+	    uDPPortRange        [2] PortRange,

3051	+	    tCPPortRange        [2] PortRange

3052	+	}

3053	+

3054	+	PortRange ::= SEQUENCE

3055	+	{

3056	+	    portStart [1] INTEGER (0..65535),

3057	+	    portEnd   [2] INTEGER (0..65535)

3058	+	}

3059	+

2921	3060		-- ================================

2922	3061		-- PGW-C + SMF PDNConnection Events

2923	3062		-- ================================

END OF CHANGE 1

END OF LAST CHANGE