3GPP SA3#97-LI	S3i250140

# Washington DC; April 29-May 2, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.928 | CR | 0019 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TR 33.928 addition – LIPF provisioning logic for 5G Prose |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The 5G Prose related LI capabilities were introduced to TS 33.127 and TS 33.128. Those functions introduce an IRI-POI at the 5G DDNMF and hence, requires the provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LIPF provisioning logic is added to TR 33.928 in a new sub-clause under the additional services in 5GC. A typo was fixed to move the clause heading of 5.4.3.2 from a sentence to heading. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 33.928 will not be aligned to TS 33.128. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.4.3.2, 5.4.3.x (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

#### 5.4.3.1	The flow-chart

Figure 5.4.3.1-1 shows the LIPF logic in provisioning the LI functions for the 5GC for the service type of Data.

Figure 5.4.3.1-1: LIPF logic for the service type Data in 5GC

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs and the IRI-TFs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

Figure 5.4.3.1-2 shows the LIPF logic in provisioning the LI functions for additional data services in 5GC.

Figure 5.4.3.1-2: LIPF logic for the additional data services in 5GC

The details of LI provisioning for AKMA, NEF based services, ECS and 5G Media Streaming, 5G Prose are illustrated in clause 5.4.3.3.

NOTE:	Even though the figure 5.4.3.1-2 shows that LI provisioning for AKMA is part of LI provisioning for the service type Data, AKMA may have to be intercepted independently in order to support other services.

### ** Second Change **

#### 5.4.3.2	Interception

### ** Third Change **

##### 5.4.3.3.5	LI provisioning for 5G Prose

Using 5G Proximity-based services (5G Prose), devices can discover and communicate directly with the other near-by devices. Even though the user data path does not pass through network domain, some signalling level interactions do happen with the 5GC (e.g. authorization to use 5G Prose).

The 5G DDNMF (Direct Discovery Network Management Function) that handles the related functions provide the LI for 5G Prose.

Figure 5.4.3.3.5-1 shows the LIPF logic for provisioning the LI functions in 5G DDNMF.

Figure 5.4.3.3.5-1: LI provisioning at DDNMF for 5G Prose

Within the figure, GPSI collectively represents GPSIMSISDN and GPSINAI, and SUPI collectively represents SUPIIMSI and SUPINAI.

The table 5.4.3.3.5-1 provides the scope of NF domain providing the LI functions for 5G MS.

Table 5.4.3.3.5-1: Scope of NF domain providing the LI functions for 5G MS

| NF with the LI function | Not-roaming | Roaming |  |
| --- | --- | --- | --- |
|  |  | VPLMN | HPLMN |
| 5G DDNMF | IRI-POI | IRI-POI | IRI-POI |

NOTE 1:	The LIPF is not aware of the roaming situation of the target.

NOTE 2:	MDF2 which is also involved in providing the LI function is not shown in the tables above.

### ** End of all Changes **