3GPP SA3#97-LI	S3i250143

# Washington DC; April 29-May 2, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0285 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarity in reference to SIP and IMS Data Channel related text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2025-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IMS related clauses were initially written when only SIP messages were ruling the IMS signalling messages. When IMS Data Channel related LI functions were introduced to the IMS clause, the event notifications handled at the DCSF are not the SIP messages. To provide some clarity, tweaking to the wording in the sentences are provided. There are two NOTEs in a clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIP signalling is changed to IMS signalling messages. NOTEs are added to provide further clarity to the IMS signalling, IMS Signalling Function. The NOTEs are now numbered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec would provide inaccurate statements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1, 7.4.3.1, 7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

#### 7.4.2.1	Overview

The capabilities defined in this clause apply to the interception of IMS-based services. The target of interception can be a subscriber of the CSP, an inbound roamer or a non-local ID.

The network function involved in providing the interception of IMS-based services are determined based on the deployment option, the network configuration, LI service scope and the IMS session including the roaming scenarios. The IRI-POI functions are provided by the network functions that handle the IMS signalling messages (those network functions are referred to as IMS Signalling Functions) and the triggered CC-POI functions are provided by the network functions that handle the media (these network functions are referred to as IMS Media Functions). The CC-TF functions are also provided by the network functions that handle the IMS signalling messages (referred to as IMS Signalling Functions) and manage the IMS Media Functions. The network functions that provide the CC-TF functions can be different from the network functions that provide the IRI-POI functions.

NOTE 1:	The concepts presented above also apply to IMS Data Channel sessions.

NOTE 2: 	The term “IMS signalling messages” usually refers to SIP messages. However, when DCSF provides the IRI-POI/CC-TF functions (see figure 7.4-1), the term refers to HTTP messages. As a result, parameter used for a specific LI function may not always be a SIP parameter.

NOTE 3:   Based on NOTE 2, in this, and the subsequent clauses the term “IMS signalling message” refers to SIP messages as well as HTTP messages if and when the said Network Function that provides such signalling message is inclusive of DCSF. Therefore, the term “IMS Signalling Function” may include DCSF as well in some instances.

An architecture depicting the LI for IMS is depicted in figure 7.4-2 below.

Figure 7.4-2: IMS LI architecture

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF. The LIPF present in the ADMF provisions IRI-POI, CC-TF, MDF2 and MDF3 over the LI_X1 interfaces.

The CC-TF sends the CC intercept trigger to the CC-POI over LI_T3 interface. The IRI-POI generates the xIRI and delivers the same to the MDF2 over LI_X2 interface. The CC-POI generates the xCC and delivers the same to the MDF3 over LI_X3 interface.

The MDF2 generates IRI messages from the received xIRI and delivers those IRI messages to the LEMF over LI_HI2 interface. The MDF3 generates the CC from the received xCC and delivers that CC to the LEMF over LI_HI3 interface.

The network configuration and IMS service scenarios including the roaming scenarios determine the network functions that provide the IRI-POI, CC-TF and CC-POI functions. The network function that provides the IRI-POI or CC-TF is referred to as IMS Signalling Function in figure 7.4-2 and the network function that provides the CC-POI functions is referred to as IMS Media Function in figure 7.4-1.

NOTE 4:	The details of correlation between the xIRI and the xCC when IRI-POI and CC-TF are not co-located is not defined in the present document. The IRI-POI and CC-TF are logical functions and they may be handled by the same process when they are co-located in the same IMS Signalling Function.

### ** Next Change **

#### 7.4.3.1	General

The IRI-POI detects the IMS Signalling messages that are related to a target subscriber and then generates and delivers the related xIRI to the MDF2 over LI_X2.

The following IMS Network Functions (i.e. IMS Signalling Functions) that handle SIP signalling for IMS sessions may provide the IRI-POI functions:

-	S-CSCF.

-	E-CSCF.

-	P-CSCF.

-	IBCF.

-	MGCF.

-	Conference AS/MRFC.

-	Telephony AS.

-	PTC server.

-

The following IMS Network Function that handles the HTTP messages for IMS Data Channel may provide the IRI-POI functions:

- DCSF.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the IRI-POI functions. The Telephony AS is one of the IMS Network Functions that provides the IRI-POI for STIR/SHAKEN and RCD/eCNAM (see clause 7.14.2).

### ** Next Change **

#### 7.4.4.1	General

The CC-TF detects the IMS signalling messages that are related a target and then generates and sends a trigger to the CC-POI over the LI_T3 reference point.

The CC-POI based on the trigger detects the media to be intercepted, generates the xCC and delivers the same to the MDF3.

The following IMS Network Functions (i.e. IMS Media Functions and IMS Signalling Functions) may provide the CC-POI and CC-TF functions:

-	IMS-AGW with CC-TF in P-CSCF.

-	TrGW with CC-TF in IBCF.

-	IM-MGW with CC-TF in MGCF.

-	MRFP with CC-TF in AS/MRFC (see NOTE 3).

-	MRFP with CC-TF in Conference AS/MRFC (see NOTE 2).

-	PTC Server with CC-TF in PTC Server (see NOTE 1).

-	MF with CC-TF in IMS-AS.

-	DC-AS with CC-TF in DCSF.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the CC-TF/CC-POI functions.

NOTE 1:	The PTC Server provides the IRI-POI and CC-POI functions, accordingly, PTC Server itself is the CC-TF.

NOTE 2:	Conference AS, MRFC and MRFP together are referred to as Conference Server. Conference AS/MRFC provide the conference focus functions as defined in TS 24.147 [28].

NOTE 3:	When music tone or announcement is given to the calling user prior to answer on an incoming call to the target.

### ** End of all Changes **