3GPP TSG-SA3 Meeting #97-LI

# Washington, United States, 29th Apr 2025 - 2nd May 2025

Source:	Ministère Economie et Finances

Title:	Addition of an IRI-POI to PIN Server

Document for:	Information

Agenda Item:	7.2

Work Item / Release:	LI19

Abstract of the contribution: Add an IRI-POI to PIN Server for generating xIRIs regarding a target PIN Element management in the Personal IoT Network (PIN).

5G Personal IoT Networks (PIN) is described in 3GPP TS 23.542 [X] and is defined as a group of PIN Elements (PINE) that are able to communicate each other directly or via PIN Elements with Gateway Capability (PEGC), communicate with 5G network via at least one PEGC, and managed by at least one PIN Element with Management Capability (PEMC).

PIN Element is a UE or non-3GPP device that can communicate within a PIN (via PIN direct connection, via PEGC, or via PEGC and 5GC), or outside the PIN via a PEGC and 5GC.

PIN Element with Gateway Capability is a PIN Element with the ability to provide connectivity to and from the 5G network for other PIN Elements, or to provide relay for the communication between PIN Elements.

PIN Element with Management Capability is a PIN Element with capability to manage the PIN.

The PIN architecture consists of PIN client deployed in PIN Element (PINE, PEGC, PEMC) and PIN Server deployed in Data network. The PIN Server can be deployed by mobile operator or an authorized third party. When the PIN Server is deployed by third party, the interworking with 5GS need to be transferred via NEF. PIN Server may support the following functionalities:

-	Registration management of PINE, PEGC, PEMC.

-	PIN management and PIN status management.

-	Configuration and parameters provisioning to PEGC, PEMC, PINE.

-	Support for discovery of PIN, PINE and PEGC.

-	AF requested QoS control for PEGC and specific PINE traffic flow.

The overall 5G PIN Architecture is shown in Figure 6.2.1-1 of clause 6.2.1 of TS 23.542 shown hereafter.

The goal of this contribution is to add an IRI-POI to PIN Server for generating xIRIs regarding a target PIN Element management or a target PIN management.