3GPP TSG-SA3 Meeting #97-LI	s3i250171

# Washington, United States, 29th Apr 2025 - 2nd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0725 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 on Personal IoT Network (PIN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-04-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for 5G Personal IoT Network (PIN) management in Stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LI for 5G PIN management in Stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LI for 5G PIN management in Stage 3 would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.X (new), TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/324 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/ee0a32da2f68e4b569c5464561a7a607d8de26ec |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250110 |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

[8]	ETSI TS 103 221-2: "Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)".

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: "Numbering, addressing and identification ".

[20]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf.

[21]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[22]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[23]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[24]	3GPP TS 29.572: "Location Management Services; Stage 3".

[25]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[26]	IETF RFC 815: "IP datagram reassembly algorithms".

[27]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[28]	IETF RFC 793: "Transmission Control Protocol".

[29]	IETF RFC 768: "User Datagram Protocol".

[30]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[31]	IETF RFC 4960: "Stream Control Transmission Protocol".

[32]	IANA (www.iana.org): Assigned Internet Protocol Numbers, "Protocol Numbers".

[33]	IETF RFC 6437: "IPv6 Flow Label Specification".

[34]	IETF RFC 791: "Internet Protocol".

[35]	Open Geospatial Consortium OGC 05-010: "URNs of definitions in ogc namespace".

[36]	3GPP TS 33.107: "3G security; Lawful interception architecture and functions".

[37]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[38]	3GPP TS 36.413: "S1 Application Protocol (S1AP)".

[39]	OMA-TS-MMS_ENC-V1_3-20110913-A: "Multimedia Messaging Service Encapsulation Protocol".

[40]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[41]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IETF RFC 4566: "SDP: Session Description Protocol".

[44]	3GPP TS 24.193: "Stage 3: Access Traffic Steering, Switching and Splitting (ATSSS)".

[45]	3GPP TS 29.509: "5G System; Authentication Server Services; Stage 3".

[46]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[47]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[48]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[49]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[50]	3GPP TS 23.401 "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[51]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS), Stage 3".

[52]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[53]	3GPP TS 29.172 "Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface".

[54]	3GPP TS 29.171 "LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[55]	3GPP TS 24.379: "Mission Critical Push to Talk (MCPTT) call control; protocol specification".

[56]	OMA-TS-PoC-System_Description-V2_1-20110802-A: "OMA PoC System Description".

[57]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD); Stage 3".

[58]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[59]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs); Stage 3".

[60]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications".

[61]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[62]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[63]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[64]	3GPP TS 29.598: "5G System; Unstructured Data Storage Services; Stage3".

[65]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[66]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[67]	GSMA IR.88: "IR.88 LTE and EPC Roaming Guidelines".

[68]	GSMA NG.114 "IMS Profile for Voice, Video and Messaging over 5GS".

[69]	IETF RFC 8225: "PASSporT: Personal Assertion Token".

[70]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[71]	IETF RFC 8588: "Personal Assertion Token (PaSSporT) Extension for Signature-based Handling of Asserted information using toKENs (SHAKEN)".

[72]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[73]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data".

NOTE:	The above document cannot be formally referenced until it is published as an RFC.

[74]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP)and Session Description Protocol (SDP); Stage 3".

[75]	IANA Session Initiation Protocol (SIP) Parameters: https://www.iana.org/assignments/sip-parameters/sip-parameters.xhtml

[76]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[77]	3GPP TS 23.204: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[78]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[79]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[80]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[81]	IETF RFC 5438: "Instant Message Disposition Notification (IMDN)".

[82]	OMA-TS-CPM_System_Description-V2_2-20170926-C: "OMA Converged IP Messaging System Description".

[83]	Void.

[84]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa) ".

[85]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[86]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[87]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[88]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping".

[89]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[90]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[91]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[92]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[93]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[94]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification".

[95]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[96]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[97]	ETSI TS 103 280: "Lawful Interception (LI); Dictionary for common parameters".

[98]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[99]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[100]	3GPP TS 29.563: "5G System; Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[101]	3GPP TS 29.562: "5G System; Home Subscriber Server (HSS) Services; Stage 3".

[102]	3GPP TS 24.341 "Support of SMS over IP networks, Stage 3".

[103]	3GPP TS 38.473 "NG-RAN;F1 application protocol (F1AP)".

[104]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[105]	ITU-T Recommendation Q.763 (1999): "Specifications of Signalling System No.7; Formats and codes".

[106]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[107]	IETF RFC 6442: "Location Conveyance for the Session Initiation Protocol".

[108]	Void.

[109]	OMA-TS-CPM_Conv_Function: "OMA CPM Conversation Functions".

[110]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[111]	3GPP TS 32.299: " Telecommunication management; Charging management; Diameter charging applications".

[112]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[113]	3GPP TS 38.414: "NG-RAN; NG data transport".

[114]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[115]	IETF RFC 5322: "Internet Message Format".

[116]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[117]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[118]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[119]	W3C Recommendation: "XML Path Language (XPath)".

[120]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[121]	3GPP TR 33.928: "ADMF Logic for Provisioning Lawful Interception (LI) ".

[122]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System".

[123]	3GPP TS 23.038: "Alphabets and language-specific information".

[124]	ITU-T Recommendation X.680 (2021): "Information technology—Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[125]	IETF RFC 4282: "The Network Access Identifier".

[126]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[127]	IEEE "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf

[128]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN)".

[129]	3GPP TS 33.503: "Security aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[130]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[131]	3GPP TS 24.174: "Support of multi-device and multi-identity in the IP Multimedia Subsystem (IMS)".

[132]	OMA-TS-CPM_Message_Storage: "OMA CPM Message Storage".

[133]	3GPP TS 29.520: "Network Data Analytics Services".

[134]	3GPP2 C.S0015-A: "Short Message Service (SMS) for Wideband Spread Spectrum Systems".

[135]	ETSI TS 102 232-5: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 5: Service-specific details for IP Multimedia services".

[136]	ETSI TS 102 232-3: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 3: Service-specific details for internet access services".

[137]	3GPP TS 29.176: "IP Multimedia Subsystem (IMS); Media Function (MF); Services Stage 3".

[138]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3".

[139]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[140]	3GPP TS 24.554: " Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[141]	3GPP TS 29.555: "5G Direct Discovery Name Management Services; Stage 3".

[X]	3GPP TS 24.583: "Application layer support for Personal IoT Network (PINAPP); Stage 3".

END OF FIRST CHANGE

START OF SECOND CHANGE

## 7.X	LI at 5G PIN Server

### 7.X.1	Provisioning over LI_X1

#### 7.X.1.1	Provisioning of IRI-POI in 5G PIN Server

The IRI-POI present in the 5G PIN Server is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the 5G PIN Server shall support the target identifier types given in table 7.X.1.1-1.

Table 7.X.1.1-1: TargetIdentifier types for data analytics

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |
| pINClientID | 3GPP | TargetIdentifierExtension | See XSD schema |
| identityToken | 3GPP | TargetIdentifierExtension | See XSD schema |
| mACAddress | 3GPP | MACAddress | See ETSI TS 103 221-1 [7] |
| pINID | 3GPP | TargetIdentifierExtension | See XSD schema |

Table 7.X.1.1-2 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the 5G PIN Server.

If the IRI-POI in the 5G PIN Server receives an ActivateTask message and the ListOfServiceTypes parameter contains a ServiceType that is not supported, the IRI-POI in the 5G PIN Server shall reject the task with an appropriate error as described in ETSI TS 103 221-1 [7] clause 6.2.1.2.

Table 7.X.1.1-2: ActivateTask message for the IRI-POI in the 5G PIN Server

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| xID | XID assigned by LIPF. | M |
| targetIdentifiers | One of the target identifiers listed in the paragraph above. | M |
| deliveryType | Set to “X2Only”. | M |
| listOfDIDs | Delivery endpoints for LI_X2 for the IRI-POI in the 5G PIN Server. These delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to the task activation. | M |

#### 7.X.1.2	Provisioning of the MDF2

The MDF2 listed as the delivery endpoint over LI_X2 for xIRI generated by 5G PIN Server shall be provisioned over LI_X1 by the LIPF.

The target identities listed in clause 7.X.1.1 shall apply for the provisioning of MDF2.

Table 7.X.1.2-1 and table 7.X.1.2-2 show the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF2.

Table 7.X.1.2-1: ActivateTask message for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| xID | XID assigned by LIPF. | M |
| targetIdentifiers | One or more of the target identifiers listed in table 7.X.1.1-1. | M |
| deliveryType | Set to “X2Only”. (Ignored by the MDF2). | M |
| listOfDIDs | Delivery endpoints of LI_HI2. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| listOfMediationDetails | Sequence of Mediation Details (see table 7.X.1.2-2). | M |

Table 7.X.1.2-2: Mediation Details for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| lIID | Lawful Interception ID associated with the task. | M |
| deliveryType | Set to “HI2Only”. | M |
| listOfDIDs | Details of where to send the IRI for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask message. | C |
| serviceScoping | Service type set to “Data”. Other fields are dependent on the warrant. | M |

### 7.X.2	Generation of xIRI over LI_X2

#### 7.X.2.1	General

The IRI-POI present in the PIN Server shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.4, the details of which are described in the following clauses.

#### 7.X.2.2	PINAPP message

The IRI-POI in the PIN Server shall generate an xIRI containing a FiveGPINAPPMessage record when the IRI-POI present in the PIN Server detects that the PIN Server interacts using the PINAPP protocol with a PIN client for PIN application layer procedures (see clause 5.1 of TS 24.583 [X]). PIN client is deployed in PIN Element (PINE, PEGC, PEMC). The IRI-POI present in the PIN Server shall generate the xIRI for the following events:

-	PIN Server exchanges PINAPP protocol messages with a target PIN client relating to PIN application layer procedures.

-	PIN Server exchanges PINAPP protocol messages with any PIN client of a target PIN relating to PIN application layer procedures.

Table 7.X.2.2-1: Payload for FiveGPINAPPMessage record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| pINClientID | PINClientID | 0..1 | Identity assigned by the PIN Server to the PIN client, if available. | C |
| gPSI | GPSI | 0..1 | Identifies the GPSI of the PIN client, if available. | C |
| identityToken | IdentityToken | 0..1 | Represents the identity token of the PIN client, if available. | C |
| mACAddress | MACAddress | 0..1 | Identifies the MAC address of the PIN client, if available. | C |
| pINID | PINID | 0..1 | Identifies the PIN, if available. | C |
| pINServerID | PINServerID | 1 | Identifies the PIN server. | M |
| pINAPPMessage | XML Type | 1 | Contains the PINAPP protocol message encoded as a "pinapp-info" XML structure according to TS 24.583 [X] clause 6.2.4. The XMLType.namespace for this parameter shall be set to the namespace of the root element of the XML document, e.g. "urn:3GPP:ns:pinapp:2023". | M |
| NOTE: At least one among pINClientID, gPSI, identityToken and mACAddress shall be provided. |  |  |  |  |

#### 7.X.2.3	Start of interception with PIN client in PIN

The IRI-POI in the PIN Server shall generate an xIRI containing a FiveGStartOfInterceptionWithPINClientInPIN record when the IRI-POI present in the PIN Server detects the following events:

-	Interception is activated on a target PIN client which is already member of one or several PINs.

-	Interception is activated on a target PIN with PIN clients already made as members.

Table 7.X.2.3-1: Payload for FiveGStartOfInterceptionWithPINClientInPIN record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| pINClientInPINS | PINClientInPINS | 1 | Contains the PIN client profile of the target PIN client and the PIN profiles of the PINs the target PIN client is member of. | C |
| pINClientsInPIN | PINClientsInPIN | 1 | Contains the PIN profile of the target PIN and the PIN client profiles of the PIN clients which are members of the target PIN. | C |
| NOTE: Either pINClientInPINS or pINClientsInPIN shall be present. |  |  |  |  |

Table 7.X.2.3-2: Payload for PINClientInPINS parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| pINClientID | PINClientID | 1 | Identity assigned by the PIN Server to the target PIN client. | M |
| gPSI | GPSI | 0..1 | Identifies the GPSI of the target PIN client, if available. | C |
| identityToken | IdentityToken | 0..1 | Represents the identity token of the target PIN client, if available. | C |
| mACAddress | MACAddress | 0..1 | Identifies the MAC address of the target PIN client, if available. | C |
| pINServerID | PINServerID | 1 | Identifies the PIN server. | M |
| pINClientProfile | XML Type | 1 | Contains the PIN client profile information of the target PIN client. The PIN client profile is encoded as a "PIN-Client-Profile" XML structure according to TS 24.583 [X] clause 6.2.4. The XMLType.namespace for this parameter shall be set to the namespace of the root element of the XML document, e.g. "urn:3GPP:ns:pinapp:2023". | M |
| pINProfiles | SET OF PINProfile | 1..MAX | Contains the PIN profiles of the PINs the target PIN client is member of. Each PIN profile is encoded as a "PIN-Profile" XML structure according to TS 24.583 [X] clause 6.2.4. The XMLType.namespace for this parameter shall be set to the namespace of the root element of the XML document, e.g. "urn:3GPP:ns:pinapp:2023". | M |

Table 7.X.2.3-2: Payload for PINClientsInPIN parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| pINID | PINID | 1 | Identifies the target PIN. | M |
| pINServerID | PINServerID | 1 | Identifies the PIN server. | M |
| pINClientProfiles | SET OF PINClientProfile | 1..MAX | Contains the PIN client profiles of the PIN clients which are members of the target PIN. Each PIN client profile information is encoded as a "PIN-Client-Profile" XML structure according to TS 24.583 [X] clause 6.2.4. The XMLType.namespace for this parameter shall be set to the namespace of the root element of the XML document, e.g. "urn:3GPP:ns:pinapp:2023". | M |
| pINProfile | XML Type | 1 | Contains the PIN profile of the target PIN. The PIN profile is encoded as a "PIN-Profile" XML structure according to TS 24.583 [X] clause 6.2.4. The XMLType.namespace for this parameter shall be set to the namespace of the root element of the XML document, e.g. "urn:3GPP:ns:pinapp:2023". | M |

### 7.X.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the 5G PIN Server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the 5GMS AF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter shall be included and coded according to table 7.14.2.11-1 (see ETSI TS 102 232-1 [9] clause 5.2.10).

Table 7.15.3-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| FiveGPINAPPMessage | REPORT |
| FiveGStartOfInterceptionWithPINClientInPIN | REPORT |

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

MDF2 delivers the IRI to the LEMF with GPSI as the target identity if and only if GPSI is present in the xIRI.

END OF SECOND CHANGE

START OF THIRD CHANGE

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -322,8 +322,11 @@ XIRIEvent ::= CHOICE

322	322

323	323		    -- 5G ProSe Direct Communication events, see clause 7.17.2

324	324		    fiveGDDNMFProSeUNIDirectDiscovery                   [184] FiveGDDNMFProSeUNIDirectDiscovery,

325		-	    fiveGDDNMFProSeNNIDirectDiscovery                   [185] FiveGDDNMFProSeNNIDirectDiscovery

325	+	    fiveGDDNMFProSeNNIDirectDiscovery                   [185] FiveGDDNMFProSeNNIDirectDiscovery,

326	326

327	+	    -- 5G Personal IoT events, see clause 7.X.2

328	+	    fiveGPINAPPMessage                                  [186] FiveGPINAPPMessage,

329	+	    fiveGStartOfInterceptionWithPINClientInPIN          [187] FiveGStartOfInterceptionWithPINClientInPIN

327	330		}

328	331

329	332		-- ==============

@@ -629,7 +632,11 @@ IRIEvent ::= CHOICE

629	632

630	633		    -- 5G ProSe Direct Communication events, see clause 7.17.3

631	634		    fiveGDDNMFProSeUNIDirectDiscovery                   [184] FiveGDDNMFProSeUNIDirectDiscovery,

632		-	    fiveGDDNMFProSeNNIDirectDiscovery                   [185] FiveGDDNMFProSeNNIDirectDiscovery

635	+	    fiveGDDNMFProSeNNIDirectDiscovery                   [185] FiveGDDNMFProSeNNIDirectDiscovery,

636	+

637	+	    -- 5G Personal IoT events, see clause 7.X.3

638	+	    fiveGPINAPPMessage                                  [186] FiveGPINAPPMessage,

639	+	    fiveGStartOfInterceptionWithPINClientInPIN          [187] FiveGStartOfInterceptionWithPINClientInPIN

633	640		}

634	641

635	642		IRITargetIdentifier ::= SEQUENCE

@@ -5956,6 +5963,59 @@ FiveGProSeMessage ::= CHOICE

5956	5963		    fiveGProSeUsageInformationReportMessage [2] XMLType

5957	5964		}

5958	5965

5966	+	-- =========================

5967	+	-- 5G PIN Server definitions

5968	+	-- =========================

5969	+

5970	+	-- See clause 7.X.2.2 for details of this structure

5971	+	FiveGPINAPPMessage ::= SEQUENCE

5972	+	{

5973	+	    pINClientID               [1] PINClientID OPTIONAL,

5974	+	    gPSI                      [2] GPSI OPTIONAL,

5975	+	    identityToken             [3] IdentityToken OPTIONAL,

5976	+	    mACAddress                [4] MACAddress OPTIONAL,

5977	+	    pINID                     [5] PINID OPTIONAL,

5978	+	    pINServerID               [6] PINServerID,

5979	+	    pINAPPMessage             [7] XMLType

5980	+	}

5981	+

5982	+	-- See clause 7.X.2.3 for details of this structure

5983	+	FiveGStartOfInterceptionWithPINClientInPIN ::= CHOICE

5984	+	{

5985	+	    pINClientInPINS         [1] PINClientInPINS,

5986	+	    pINClientsInPIN         [2] PINClientsInPIN

5987	+	}

5988	+

5989	+	PINClientInPINS ::= SEQUENCE

5990	+	{

5991	+	    pINClientID             [1] PINClientID,

5992	+	    gPSI                    [2] GPSI OPTIONAL,

5993	+	    identityToken           [3] IdentityToken OPTIONAL,

5994	+	    mACAddress              [4] MACAddress OPTIONAL,

5995	+	    pINServerID             [5] PINServerID,

5996	+	    pINClientProfile        [6] XMLType,

5997	+	    pINProfiles             [7] SET OF PINProfile

5998	+	}

5999	+

6000	+	PINClientsInPIN ::= SEQUENCE

6001	+	{

6002	+	    pINID                   [1] PINID,

6003	+	    pINServerID             [2] PINServerID,

6004	+	    pINClientProfiles       [3] SET OF PINClientProfile,

6005	+	    pINProfile              [4] XMLType

6006	+	}

6007	+

6008	+	-- ========================

6009	+	-- 5G PIN Server parameters

6010	+	-- ========================

6011	+

6012	+	PINClientID ::= UTF8String

6013	+	IdentityToken ::= UTF8String

6014	+	PINID ::= UTF8String

6015	+	PINServerID ::= UTF8String

6016	+	PINClientProfile ::=XMLType

6017	+	PINProfile ::= XMLType

6018	+

5959	6019		-- ===================

5960	6020		-- 5G LALS definitions

5961	6021		-- ===================

END OF THIRD CHANGE