3GPP SA3LI#98	S3i250320

# 15-18 July 2025, Florence (Italy)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0294 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PTC clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2025-07-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text in the IMS clause which was added long before the new PTC specific clause conflicting with the text in PTC clause with respect to the scope of LI functions provided by the PTC server. The IMS clause says the PTC Server provides IRI-POI, CC-TF and CC-POI functions. The PTC clause says the PTC Server provides the IRI-POI and CC-POI functions. The text in PTC clause is consistent with the rest of the specification when the IRI-POI and CC-POI are provided in the same NF. A CC-TF in this case is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PTC related text in IMS clauses is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conflicting architecture concepts remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.1, 7.4.4.1, 7.4.6.1, 7.4.6.2, 7.4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

#### 7.4.3.1	General

The IRI-POI detects the IMS Signalling messages that are related to a target subscriber and then generates and delivers the related xIRI to the MDF2 over LI_X2.

The following IMS Network Functions (i.e. IMS Signalling Functions) that handle SIP signalling for IMS sessions may provide the IRI-POI functions:

-	S-CSCF.

-	E-CSCF.

-	P-CSCF.

-	IBCF.

-	MGCF.

-	Conference AS/MRFC.

-	Telephony AS.

The following IMS Network Function that handles the HTTP messages for IMS Data Channel may provide the IRI-POI functions:

-	DCSF.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the IRI-POI functions. The Telephony AS is one of the IMS Network Functions that provides the IRI-POI for STIR/SHAKEN and RCD/eCNAM (see clause 7.14.2).

### ** Next Change **

#### 7.4.4.1	General

The CC-TF detects the IMS signalling messages that are related a target and then generates and sends a trigger to the CC-POI over the LI_T3 reference point.

The CC-POI based on the trigger detects the media to be intercepted, generates the xCC and delivers the same to the MDF3.

The following IMS Network Functions (i.e. IMS Media Functions and IMS Signalling Functions) may provide the CC-POI and CC-TF functions:

-	IMS-AGW with CC-TF in P-CSCF.

-	TrGW with CC-TF in IBCF.

-	IM-MGW with CC-TF in MGCF.

-	MRFP with CC-TF in AS/MRFC (see NOTE 3).

-	MRFP with CC-TF in Conference AS/MRFC (see NOTE 2).

-	MF with CC-TF in IMS-AS.

-	DC-AS with CC-TF in DCSF.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the CC-TF/CC-POI functions.

NOTE 1:	Void.

NOTE 2:	Conference AS, MRFC and MRFP together are referred to as Conference Server. Conference AS/MRFC provide the conference focus functions as defined in TS 24.147 [28].

NOTE 3:	When music tone or announcement is given to the calling user prior to answer on an incoming call to the target.

### ** Next Change **

#### 7.4.6.1	General

The IMS Network Functions that provide the IRI-POI, CC-TF and CC-POI functions can vary based on the network topology and session scenarios such as:

-	Network topologies:

-	Non-roaming.

-	Roaming case with Local Break Out (LBO), VPLMN.

-	Roaming case with LBO, HPLMN.

-	Roaming case with Home Routed (HR), VPLMN.

-	Roaming case with HR, in HPLMN.

-	Session types:

-	Normal sessions.

-	Emergency sessions.

-	Redirected sessions.

-	IMS specific services such as conferencing, music, announcements.

-	SMS over IMS.

-	Target type:

-	Non-local ID target.

A deployment option within the CSP may also have a role in selection of the Network Functions. In the case of roaming case, the interception performed in the VPLMN and HPLMN are based on separate independent warrants.

More detailed description of these scenarios is given in TS 33.128 [15].

### ** Next Change **

#### 7.4.6.2	IMS Network Functions providing the IRI-POI

The IMS Network Functions that handle the target side of the session provide the IRI-POI functions except when the alternate option is used for the non-local ID target. When the alternate option is used for the non-local ID target, the IMS network function that handles the session-leg of the local served user connected directly to the non-local ID target.

Table 7.4.6.2-1 below identifies the IMS Network Functions in providing the IRI-POI functions in a non-roaming case for various session scenarios.

Table 7.4.6.2-1: IMS Network Functions providing the IRI-POI functions (non-roaming case)

| Session type/target type | Default | Alternative option |
| --- | --- | --- |
| Normal sessions | S-CSCF | P-CSCF |
| SMS over IMS | S-CSCF | P-CSCF |
| Emergency sessions | E-CSCF | P-CSCF (NOTE 1) |
| SMS over IMS to emergency services | E-CSCF | P-CSCF (NOTE1) |
| Redirected sessions: intra-PLMN | S-CSCF | P-CSCF |
| Redirected sessions: inter-PLMN (CS domain) | S-CSCF | MGCF |
| Redirected sessions: inter-PLMN (IMS domain) | S-CSCF | IBCF |
| Conference (NOTE 2) | Conf-AS/MRFC | - |
| Non-local ID in CS domain (NOTE 3, NOTE 3A) | MGCF | S-CSCF |
| Non-local ID in IMS domain (NOTE 3, NOTE 3A) | IBCF | S-CSCF |
| Non-local ID for SMS over IMS (NOTE 3) | S-CSCF | P-CSCF (NOTE 3A) |
| Data Channel Sessions | DCSF | - |

NOTE 1:	For originated emergency sessions (or SMS over IMS to emergency services centre) handled in the fixed networks, where S-CSCF is also part of an emergency session, the S-CSCF based IRI-POI as a deployment option may also be considered.

NOTE 2:	A conference ID can also be a target. Conf-AS stands for conference AS (see NOTE 2 in clause 7.4.4.1). When a normal session is extended to a conference session, the IMS signalling functions that provide the IRI-POI functions prior to the conference may continue to provide the IRI-POI functions in addition to the conference AS/MRFC.

NOTE 3:	Non-roaming means that the local served user is non-roaming.

NOTE 3A:	The default/alternate option used when the target is non-local ID is mutually independent of default/alternate option used when the target is local served user.

Table 7.4.6.2-2 below identifies the IMS Network Functions in providing the IRI-POI functions in a roaming case for various session scenarios.

Table 7.4.6.2-2: IMS Network Functions providing the IRI-POI functions (roaming case)

| Session type/target type | Local Breakout (LBO) |  |  |  | Home Routed (HR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPLMN |  | VPLMN |  | HPLMN |  | VPLMN |  |
|  | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option |
| Normal sessions | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| SMS over IMS | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Emergency sessions/SMS over IMS | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| SMS over IMS to emergency services | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| Redirected sessions | S-CSCF | See table 7.4.6.2-3 | - | - | S-CSCF | See table 7.4.6.2-3 | - | - |
| Conference (NOTE 2) | Conf-AS/MRFC | - | - | - | Conf-AS/MRFC | - | - | - |
| Non-local ID (E.164) in CS domain (NOTE 3A, NOTE 4, NOTE 4A) | MGCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | MGCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID in SIP/IMS domain (NOTE 3A, NOTE 4, NOTE 4A) | IBCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | IBCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID for SMS over IMS (NOTE 4) | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Data Channel Sessions | DCSF | - | n/a | n/a | DCSF | - | n/a | n/a |

NOTE 4:	For roaming, this means the local served user is roaming. Also, see NOTE 3.

NOTE 4A:	The default/alternate options used in the HPLMN and default/alternate options used in the VPLMN are mutually independent.

NOTE 4B:	This alternate option may be used only in the VPLMN for IMS sessions with home-routed media.

The interception capabilities for normal sessions as defined in tables 7.4.6.2-1 (non-roaming) and 7.4.6.2-2 (roaming) shall be used for the cases where the Conf-AS and the PTSC-Server are not under the control of CSP serving the warrant.

Table 7.4.6.2-3: Extension of table 7.4.6.2-2

| Session type/target type |  | Local Breakout (LBO) | Home Routed (HR) |
| --- | --- | --- | --- |
| Redirected sessions: Intra-PLMN | Redirected-to party non-roaming | P-CSCF | P-CSCF |
|  | Redirected-to party is roaming | IBCF | P-CSCF |
| Redirected sessions Inter-PLMN | Redirected-to party in CS domain | MGCF | MGCF |
|  | Redirected-to party in IMS domain | IBCF | IBCF |

Table 7.4.6.2-3 shows the IMS Network Functions that provide the IRI-POI functions in the HPLMN for redirected sessions in a roaming case when the alternate option is used to provide the IRI-POI functions for the normal case.

NOTE 5:	For the redirected do not answer related sessions, the IMS Network Functions that provide the IRI-POI functions prior to the redirection are as illustrated in table 7.4.6.2-2 (normal case) and after the redirection are as illustrated in table 7.4.6.2-3.

The IMS Network Functions that provide the IRI-POI for STIR/SHAKEN and RCD/eCNAM are listed in clause 7.14.2.

#### 7.4.6.3	IMS Network Functions providing the CC-TF and CC-POI functions

The IMS Network Functions that handle the target side (including the non-local ID target) of the session provide the CC-TF and CC-POI functions. For redirected scenarios, the IMS Network Functions that handle the redirected-to-user side of the session provide the CC-TF and CC-POI functions.

Table 7.4.6.3-1 provides the IMS Network Functions that provide the CC-TF functions when the CC-POI functions are provided by the IMS Media Functions as indicated (also see clause 7.4.4.1).

Table 7.4.6.3-1: Mapping between the IMS Network Functions providing the CC-TF and the CC-POI functions

| CC-POI | CC-TF |
| --- | --- |
| PGW (NOTE 1) | P-CSCF |
| PGW-U (NOTE 1) | P-CSCF |
| IMS-AGW | P-CSCF |
| MRFP | AS/MRFC |
| MRFP | Conference AS/MRFC |
| TrGW | IBCF |
| IM-MGW | MGCF |
| MF | IMS-AS |
| DC-AS | DCSF |

NOTE 1: 	This is defined in TS 33.107 [11] and outside the scope of the present document.

Table 7.4.6.3-2 below identifies the IMS Media Functions that provide the CC-POI functions in a non-roaming case for session scenarios (PGW and PGW-U based options are not shown in the table).

Table 7.4.6.3-2: IMS Media Functions providing the CC-POI functions (non-roaming case)

| Session type/target type | CC-POI |
| --- | --- |
| Normal sessions | IMS-AGW |
| Emergency sessions | IMS-AGW |
| Redirected sessions: intra-PLMN | IMS-AGW |
| Redirected sessions: inter-PLMN (CS domain) | IM-MGW |
| Redirected sessions: inter-PLMN (IMS-domain) | TrGW |
| Music, announcement | MRFP |
| Conference (NOTE 4) | MRFP |
| Non-local ID in CS domain (NOTE 2) | IM-MGW |
| Non-local ID in IMS domain (NOTE 2) | TrGW |
| Data Channel | MF |
| Data Channel (MF as UDP proxy) | DC-AS |

NOTE 2:	Non-roaming means that the local served user is non-roaming.

Table 7.4.6.3-3 below identifies the IMS Media Functions that provide the CC-POI functions in a roaming case for various session scenarios (PGW and PGW-U based options are not shown in the table).

Table 7.4.6.3-3: IMS Media Functions providing the CC-POI functions (roaming case)

| Session type/target type |  | Local Breakout (LBO) |  |  | Home Routed (HR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | HPLMN | VPLMN |  | HPLMN | VPLMN |
|  |  |  | Default | Alternate Option |  |  |
| Normal sessions |  | TrGW | IMS-AGW | - | IMS-AGW | N9HR/ S8HR |
| Emergency sessions |  | - | IMS-AGW | - | - | IMS-AGW |
| Redirected sessions: intra-PLMN | Redirected-to-party non-roaming | IMS-AGW | - | - | IMS-AGW | - |
|  | Redirected-to-party roaming | TrGW | - | - | IMS-AGW | - |
| Redirected sessions: inter-PLMN | Redirected-to-party in CS domain | IM-MGW | - | - | IM-MGW | - |
|  | Redirected-to-party in IMS domain | TrGW | - | - | TrGW | - |
| Conference (NOTE 4) |  | MRFP | - | - | MRFP | - |
| Music, announcement |  | MRFP | - | - | MRFP | - |
| Non-local ID in CS domain (NOTE 3) |  | IM-MGW | IMS-AGW | TrGW (NOTE 5) | IM-MGW | N9HR/ S8HR |
| Non-local ID in IMS domain (NOTE 3) |  | TrGW | IMS-AGW | TrGW (NOTE 5) | TrGW | N9HR/S8HR |

NOTE 3:	Roaming means that the local served user is roaming.

NOTE 4:	When a normal session is extended to a conference session, the IMS-AGW that provides the CC-POI functions prior to the conference may continue to provide the CC-POI functions as an alternate (applicable only when the delivery of CC for a held conference call is not required), or in addition, to the MRFP. In that case, the P-CSCF provides the CC-TF functions for the CC-POI in the IMS-AGW.

NOTE 5:	This is applicable only for IMS sessions with home-routed media with a TrGW present in the VPLMN.

### ** End of all Changes **