SA3LI#98	 S3i250334

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #5 to TR 33.929-6: RCS topology Part III – RCS servers in one third party domain, CSP RCS users in the same CSP domain

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Provides some RCS topologies when the RCS servers are in the same third party RCS domain. Here, the CSP RCS users are in the same CSP domain.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. This pCR gives provides the RCS network topologies for RCS when the RCS servers are in the same third party domain. The CSP RCS users are in the same CSP domain.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

### 4.2.1	General

### 4.2.2	Few conventions used

In all the illustrations, when the RCS user is a subscriber of a CSP domain, 5G radio and packet core is assumed. Within the drawings, (A) represents the ingress side and (B) represents the egress side. When ingress and egress have to be shown in multiple network domains, (A1), (A11), (B1), B (11), etc., are used.

The figure 4.2.2-1 shows some of the conventions used within the topology drawings (in a general way):

Figure 4.2.2-1: Few conventions used within the drawings

### 4.2.3	RCS servers in CSP domain

PROPOSAL

Incorporate the following to the TR 33.929-6.

***** New text ****

### 4.2.4	Third party RCS service providers

#### 4.2.4.1	General

In the case where the RCS service is provided by a third party RCS service provider, the RCS Server resides within that third RCS service provider domain. The RCS users may or may not be a subscriber of a CSP. In the former case (i.e. when the RCS user is a subscriber of CSP), the signalling and RCS media pass through the CSP domain whereas in the latter case, RCS signalling and RCS media pass through data network that is not related to any CSPs (e.g. Wi-Fi).

As in the case where the RCS Servers are in the CSP domain, there are two main cases when the RCS is provided by the third party RCS service provider:

- RCS Servers in one third party RCS service provider domain.

- RCS Servers in different third party RCS service provider domains.

When RCS user is in a CSP domain, there are two cases:

- RCS users in the same CSP domain.

- RCS users in different CSP domains.

It is possible that the third party RCS service provider may not have IMS NFs deployed. Instead, the SIP messages may pass through proprietary SIP message handling proxies. The illustrations shown in the present document, however, assume as if the third party RCS service provider domain also has the IMS NFs since the RCS Server is viewed as an IMS-based application in a general sense. However, the focus here (i.e. within the third party RCS service provider domain) is just on the RCS LI and applicability of LI for IMS other than the RCS (from a third party RCS service provider perspective) is outside the scope of the present document. Therefore, realism does not matter for the illustrations shown here.

When RCS user is in a CSP domain with third party RCS service provider being used for RCS, two scenarios are considered:

- CSP domain does not have IMS.

- CSP domain has IMS.

When the CSP domain does not have IMS, RCS signalling and RCS media pass through the CSP domain over the packet data connection. This is similar to the case where RCS users are not subscribers of a CSP.

When the CSP domain has IMS, the present document illustrates the following deployment option:

- RCS Signalling and RCS media pass through the CSP domain over the packet data connection. In this case CSP IMS is not on the signalling and media path of an RCS session.

When CSP domain has the IMS, another deployment option where the RCS Signalling and RCS media pass through the IMS present within the CSP domain is for further study.

To illustrate the point of signalling and media split (at the Ingress end) and combined (at the Egress end) within the third party RCS service provider domain, NFs named in the present document as Border GWs are shown at the Ingress and Egress end of the third party RCS service provider domain. The details how exactly such a split is done is outside the scope of present document.

Figure 4.2.4.1-1: Conceptual Border Gateways that splits/combines signalling and media

#### 4.2.4.2	One third party RCS service provider

##### 4.2.4.2.1	General

This clause illustrates the case when RCS Servers are within the same third party RCS service provider domain with various scenarios depending on where the RCS users are.

Scenario 1: RCS users are not in CSP domain.

Scenario 2: RCS users are in the same CSP domain that does not have IMS.

Scenario 3: RCS users are in the same CSP domain that has IMS.

Scenario 4: RCS users are in the different CSP domains that do not have IMS.

Scenario 5: RCS users are in the different CSP domains that have IMS.

Not all the scenarios listed above may be deployed in practice, The present document only provides the illustrations in case those scenarios are deployed.

##### 4.2.4.2.2	Scenario 1: RCS users are not in CSP domain

When RCS is provided by a third party RCS service provider, the RCS users are not necessarily to be the subscribers of any CSPs. The use-case for this can be RCS users are connected to the third party RCS service provider domain via Wi-Fi access.

The figure 4.2.4.2.2-1 illustrates RCS LI topology when RCS User (A) and RCS User (B) are targets for RCS in the third party RCS service provider domain.

Figure 4.2.4.2.2-1: Non-CSP RCS users, third party RCS service provider

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when RCS User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the RCS User (B) is a target for in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

##### 4.2.4.2.3	Scenario 2: RCS users are in the same CSP domain that does not have IMS

From an overall topology perspective, this case is similar to the topology shown in clause 4.2.4.2.2, One difference here is that RCS users being the subscribers of a CSP can also be outbound roaming.

Non-roaming case

RCS User (A) and RCS User (B) are the subscribers of the same CSP domain that does not have IMS. The CSP domain provides just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domain over the packet data connection.

The figure 4.2.4.2.3-1 illustrates the RCS LI for the non-roaming case when User (A) and User (B) are targets for RCS in the third party RCS service provider domain.

Figure 4.2.4.2.3-1: Non-roaming case, no IMS in CSP domain

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) in is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

Roaming LBO

RCS User (A) and RCS User (B) are the subscribers of the same home CSP that does not support IMS. RCS User (A) is roaming in CSP domain A1 (VPLMN) and RCS User (B) is roaming in CSP domain B1 (VPLMN). The CSP domains (VPLMNs) just provide the access when those users establish RCS sessions and as such both signalling and media do not pass through the HPLMN with LBO, instead pass through the VPLMNs over the packet data connection.

The figure 4.2.4.2.3-2 illustrates the RCS LI for the roaming case (with LBO) when User (A) and User (B) are targets for RCS in the HPLMN.

Figure 4.2.4.2.3-2: Roaming case (LBO), no IMS in CSP domain

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) in is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

Roaming HR

RCS User (A) and RCS User (B) are the subscribers (shown as User (A) and User (B)) of the same home CSP that does not support IMS. User (A) is roaming in CSP domain A1 (VPLMN) and User (B) is roaming in CSP domain B1 (VPLMN). The, CSP domains (VPLMN and HPLMN) just provide the access when those users establish RCS sessions and as such both signalling.

The figure 4.2.4.2.3-3 illustrates the RCS LI for the roaming case (with HR) when RCS User (A) and RCS User (B) are targets for RCS in the HPLMN.

Figure 4.2.4.2.3-3: Roaming case (HR), no IMS in CSP domain

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

##### 4.2.4.2.4	Scenario 3: RCS users are in the same CSP domain that has IMS

###### 4.2.4.2.4.1	General

In the illustrations shown here RCS users are the subscribers in the CSP domain that has IMS.

Illustrations include the two deployment options mentioned in clause 4.2.4.1 with non-roaming, roaming with LBO and roaming with HR for both deployment options. LI for other IMS-based services is also illustrated for a comparison purpose to grasp the scope of RCS LI.

###### 4.2.4.2.4.2	Non-roaming

Since the RCS Servers used for the RCS sessions are in third party RCS service provider domain, in this deployment option, CSP domain provides just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domain over the packet data connection.

The signalling and media for non-RCS related IMS sessions do not pass through the third party RCS service provider domain.

RCS LI

The figure 4.2.4.2.4.2-1 illustrates the CSP deployment option 1 when User (A) and User (B) are targets for RCS in the third party service provider domain. In the illustration, LI applies only for RCS.

Figure 4.2.4.2.4.2-1: Non-roaming case, CSP with IMS for RCS only

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

RCS LI and IMS LI

The figure 4.2.4.2.4.2-2 illustrates the CSP deployment option 1 when User (A) and User (B) are targets for RCS in the third party service provider domain. The User (A) and User (B) are targets for other IMS-based services (e.g. voice) in the CSP domain.

Figure 4.2.4.2.4.2-2: Non-roaming case, CSP with IMS - LI for RCS and other IMS based services

For RCS LI, the RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for in the third party RCS service provider domain. The RCS Servers provide both IRI-POI and CC-POI functions.

For LI of other IMS-based services, the S-CSCF (A1), P-CSCF (A1), IMS-AGW (A1) present in the CSP domain provide the LI functions when the User (A) is a target for other IMS-based services in the CSP domain and the S-CSCF (B1), P-CSCF (B1), IMS-AGW (B1) present in the CSP domain provide the LI functions when the User (B) is a target for other IMS-based services in the CSP domain. The S-CSCFs provide the IRI-POI functions, the P-CSCFs provide the CC-TF functions and the IMS-AGWs provide the CC-POIs functions.

###### 4.2.4.2.4.3	Roaming (LBO)

In these illustrations, User (A) and User (B) are the subscribers of the same home CSP. User (A) is roaming in CSP domain A2 (VPLMN) and User (B) is roaming in CSP domain B2 (VPLMN).

Since the RCS Servers used for the RCS sessions are in third party RCS service provider domain, in this deployment option, CSP domains just provide the access when those users establish RCS sessions and as such both signalling and media do not pass through the HPLMN, instead pass through the VPLMNs over the packet data connection.

The signalling and media for non-RCS related IMS sessions do not pass through the third party RCS service provider domain.

RCS LI

The figure 4.2.4.2.4.3-1 illustrates the CSP deployment option 1 when User (A) and User (B) roaming in CSP domain A2 (VPLMN) and CSP domain B2 (VPLMN) respectively, are targets for RCS in the third party service provider domain.

Figure 4.2.4.2.4.3-1: Roaming case (LBO), CSP with IMS – LI for RCS only

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

RCS LI and IMS LI

The figure 4.2.4.2.4.3-2 illustrates the CSP deployment option 1 when User (A) and User (B) roaming in CSP domain A2 (VPLMN) and CSP domain B2 (VPLMN) respectively, are targets for RCS in the third party service provider domain. The User (A) and User (B) are targets for other IMS-based services (e.g. voice) in the CSP domain (HPLMN) and the respective VPLMNs.

Figure 4.2.4.2.4.3-2: Roaming case (LBO), CSP with IMS – LI for RCS and other IMS-based services

For RCS LI, the RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain. The RCS servers provide both IRI-POI and CC-POI functions.

For LI of other IMS-based service in the HPLMN, the S-CSCF (A1), IBCF (A1), TrGW (A1) present in the CSP domain (HPLMN) provide the LI functions when the User (A) is a target for other IMS-based services in the CSP domain (HPLMN) and the S-CSCF (B1), IBCF (B1), TrGW (B1) present in the CSP domain (HPLMN) provide the LI functions when the User (B) is a target for other IMS-based services in the CSP domain (HPLMN). The S-CSCFs provide the IRI-POI functions, the IBCFs provide the CC-TF functions and the TrGWs provide the CC-POIs functions.

For LI of other IMS-based services in the VPLMNs, the P-CSCF (A2), IMS-AGW (A2) present in the CSP domain A2 (VPLMN) provide the LI functions when the User (A) is a target for other IMS-based services in the CSP domain A2 (VPLMN) and the P-CSCF (B2), IMS-AGW (B2) present in the CSP domain B2 (VPLMN) provide the LI functions when the User (B) is a target for other IMS-based services in the CSP domain B2 (VPLMN). The P-CSCFs provide the IRI-POI and CC-TF functions and the IMS-AGWs provide the CC-POIs functions.

###### 4.2.4.2.4.4	Roaming (HR)

In these illustrations, User (A) and User (B) are the subscribers of the same home CSP. User (A) is roaming in CSP domain A2 (VPLMN) and User (B) is roaming in CSP domain B2 (VPLMN).

Since the RCS Servers used for the RCS sessions are in third party RCS service provider domain, in this deployment option, CSPs provide just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domains (VPLMN and HPLMN) over the packet data connection.

The signalling and media for non-RCS related IMS sessions do not pass through the third party RCS service provider domain.

RCS LI

The figure 4.2.4.2.4.4-1 illustrates the CSP deployment option 1 when User (A) and User (B) roaming in CSP domain A2 (VPLMN) and CSP domain B2 (VPLMN) respectively, are targets for RCS in the third party service provider domain.

Figure 4.2.4.2.4.4-1: Roaming case (HR), CSP with IMS – LI for RCS only

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain. The RCS Servers provide both IRI-POI and CC-POI functions.

RCS LI and IMS LI

The figure 4.2.4.2.4.2.3-2 illustrates the CSP deployment option 1 when User (A) and User (B) roaming in CSP domain A2 (VPLMN) and CSP domain B2 (VPLMN) respectively, are targets for RCS in the third party service provider domain. The User (A) and User (B) are targets for other IMS-based services (e.g. voice) in the CSP domain (HPLMN) and the respective VPLMNs.

Figure 4.2.4.2.4.4-2: Roaming case (HR), CSP with IMS – LI for RCS and other IMS-based services

For RCS LI, the RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain. The RCS servers provide both IRI-POI and CC-POI functions.

For LI of other IMS-based services in the HPLMN, the S-CSCF (A1), P-CSCF(A1), IMS-AGW (A1) present in the CSP domain (HPLMN) provide the LI functions when the User (A) is a target for other IMS-based services in the CSP domain (HPLMN) and the S-CSCF (B1), P-CSCF (B1), IMS-AGW (B1) present in the CSP domain (HPLMN) provide the LI functions when the User (B) is a target for other IMS-based services in the CSP domain (HPLMN). The S-CSCFs provide the IRI-POI functions, the P-CSCFs provide the CC-TF functions and the IMS-AGWs provide the CC-POIs functions.

For LI of other IMS-based services (in the VPLMNs), the BBIFFs and LMISFs, introduced as a part of N9HR, provide the LI functions for inbound roaming User (A) is a target for other IMS-based services in CSP domain A2 (VPLMN) and for inbound roaming User (B) is a target for other IMS-based services in CSP domain B2 (VPLMN). The BBIFFs and LMISFs in the VPLMNs provide the IRI-POI and CC-POI functions.

***** End of the new text ****