SA3LI#98	 S3i250346

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #17 to TR 33.929-6: RCS LI – RCS topology Part XV – File transfer without a file localization function in the terminating RCS domain

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Provides some RCS topologies for the file transfer without a file localization function in the terminating RCS domain.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. This pCR gives provides the RCS network topologies for the file transfer scenario when the content servers are in different domains and no file localization function is used.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

### 4.2.1	General

### 4.2.2	Few conventions used

In all the illustrations, when the RCS user is a subscriber of a CSP domain, 5G radio and packet core is assumed. Within the drawings, (A) represents the ingress side and (B) represents the egress side. When ingress and egress have to be shown in multiple network domains, (A1), (A11), (B1), B (11), etc., are used.

The figure 4.2.2-1 shows some of the conventions used within the topology drawings (in a general way):

Figure 4.2.2-1: Few conventions used within the drawings

### 4.2.3	RCS servers in CSP domain

### 4.2.4	Third party RCS service providers

### 4.2.5	Hybrid cases

### 4.2.6	RCS interworking with SMS

### 4.2.7	RCS interworking with MMS

### 4.2.8	File transfer via content server

#### 4.2.8.1	General

#### 4.2.8.2	File transfer between two users in one CSP domain

#### 4.2.8.3	File transfer between two users in one third party RCS service provider domain

#### 4.2.8.4	File transfer between two users in two CSP domains with a File Localization Function

#### 4.2.8.5	File transfer between two users in two third party domains with a File Localization Function

###### PROPOSAL

Propose the following text to TR 33.929-6:

***** New text ****

#### 4.2.8.6	File transfer between two users in two CSP domains without a File Localization Function

The figure 4.2.8.6-1 illustrates the topology for the case when two RCS Users and hence, the two RCS servers are in the different CSP domains. In this illustration, a File Localization Function is not present in the terminating CSP domain. In this case, User (A) is a target for RCS at RCS server (A) and User (B) is a target for RCS at RCS Server (B).

Figure 4.2.8.6-1: File transfer between users in the different CSP domains without a File Localization Function

The RCS Server (A) provides the LI functions when User (A) is a target for RCS and the RCS Server (B) providers the LI functions when User (B) is a target for RCS. For file transfer related functions, the RCS servers provide the IRI-POI functions.

When the file is uploaded or retrieved, HTTP Content Server provides the LI functions for file upload/retrieval related events based on the trigger received from the IRI-TF and CC-TF present in the RCS Server (A). Those triggers include the location of the file (an URL) stored at the HTTP Content Server. Therefore, the LI functions provided for this case are purely based on the URL.

The HTTP Content Server may also provide the LI functions when the identity of the RCS User (A) received from RCS User (A) during file upload related authentication procedure is target for RCS.

Target non-local ID

The figure 4.2.8.6-2 illustrates the topology for the case when two RCS Users and hence, the two RCS servers are in the different CSP domains. In this illustration, a File Localization Function is not present in the terminating CSP domain. In this case User (A) is a target non-local ID for RCS at RCS server (B) and User (B) is a target non-local ID for RCS at RCS Server (A).

Figure 4.2.8.6-2: File transfer between users in the different CSP domains without a File Localization Function (target non-local ID)

The RCS Server (A) provides the LI functions when User (B) is a target for RCS in CSP domain A and the RCS Server (B) providers the LI functions when User (A) is a target for RCS in CSP domain B. For file transfer related functions, the RCS servers provide the IRI-POI functions.

When the file is uploaded or retrieved, HTTP Content Server provides the LI functions for file upload/retrieval related events based on the trigger received from the IRI-TF and CC-TF present in the RCS Server (A). Those triggers include the location of the file (an URL) stored at the HTTP Content Server. Therefore, the LI functions provided for this case are purely based on the URL.

#### 4.2.8.7	File transfer between two users in two third party domains without a File Localization Function

When the RCS server is in third party RCS service provider domain, all the topologies illustrated in clause 4.2.4 are applicable. Hybrid cases illustrated in clause 4.2.5 are applicable as well.

Only one of those topology cases is used for the illustration here.

The figure 4.2.8.7-1 illustrates the topology for the case when two RCS Users use different third party RCS service providers. In this illustration, the two RCS servers are in the different third party RCS service provider domains. Furthermore, in this illustration, a File Localization Function is not present in the terminating third party RCS service provider domain. In this case, User (A), a subscriber of CSP domain A1 is a target for RCS in third party RCS service provider domain A and User (B) a subscriber of CSP domain B1 is a target for RCS in third party RCS service provider B.

Figure 4.2.8.7-1: File transfer between users in the different third party domains without a File Localization Function

The RCS Server (A) provides the LI functions when User (A) is a target for RCS in third party RCS service provider domain A and the RCS Server (B) providers the LI functions when User (B) is a target for RCS in third party RCS service provider domain B. For file transfer related functions, the RCS servers provide the IRI-POI functions.

When the file is uploaded or retrieved, HTTP Content Server provides the LI functions for file upload/retrieval related events based on the trigger received from the IRI-TF and CC-TF present in the RCS Server (A). Those triggers include the location of the file (an URL) stored at the HTTP Content Server. Therefore, the LI functions provided for this case are purely based on the URL.

The HTTP Content Server may also provide the LI functions when the identity of the RCS User (A) received from RCS User (A) during file upload related authentication procedure is target for RCS.

Target non-local ID

The figure 4.2.8.7-2 illustrates the topology for the case when two RCS Users use different third party RCS service providers. In this illustration, the two RCS servers are in the different third party RCS service provider domains. Furthermore, in this illustration, a File Localization Function is not present in the third party RCS service provider domain B. In this case, User (A), a subscriber of CSP domain A1 is a target non-local ID for RCS in third party RCS service provider domain B and User (B) a subscriber of CSP domain B1 is a target non-local ID for RCS in third party RCS service provider B.

Figure 4.2.8.7-2: File transfer between users in the different third party domains without a File Localization Function (target non-local ID)

The RCS Server (A) provides the LI functions when User (B) is a target for RCS in third party RCS service provider domain A and the RCS Server (B) providers the LI functions when User (A) is a target for RCS in third party RCS service provider domain B. For file transfer related functions, the RCS servers provide the IRI-POI functions.

When the file is uploaded or retrieved, HTTP Content Server provides the LI functions for file upload/retrieval related events based on the trigger received from the IRI-TF and CC-TF present in the RCS Server (A). Those triggers include the location of the file (an URL) stored at the HTTP Content Server. Therefore, the LI functions provided for this case are purely based on the URL.

***** End of new text ****