SA3LI#98	 S3i250347

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #18 to TR 33.929-6: RCS LI – File transfer procedure with the steps

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Illustrates file transfer related steps.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. File transfer related topology views were already introduced. This pCR gives the file transfer procedure related steps.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

PROPOSAL

Incorporate the following to the TR 33.929-6.

***** New text ****

## 4.3	Steps of file transfer via content server

### 4.3.1	Background

#### 4.3.1.1	General

The subsequent clauses illustrate the steps involved during the file transfer via content server procedure between the two RCS users.

To transfer a file the sender uploads the file to a server (referred to as the HTTP Content Server), receives the URL where the file was stored from the HTTP Content Server and sends the URL to recipient using a SIP message through the RCS Servers.

Using the received URL, the recipient downloads the file from the HTTP Content Server. When the recipient is a different domain, the file may be retrieved and stored at a local server referred to as s File Localization Function. The recipient downloads the file from the File Localization Function.

The transfer of the SIP message carrying the URL may happen independent of whether, or not, the RCS users are in an RCS session.

The illustrations shown in clause 4.2 provide various topology scenarios for the RCS LI. The illustrations shown in clause 4.2.8 provide a few scenarios that include the HTTP Content Server and File Localization Function. The illustrations shown in the following clauses mask out those topology details to avoid repetition in the illustrations. Focus here is more on the file-transfer related steps.

In all the illustration, it is assumed that the HTTP Content Server is in the same domain as the originating RCS Server while the File Localization Function is in the same domain as the terminating RCS Server. When the two users are in the same domain (and hence, the two servers), the need for a File Localization Function does not arise.

Furthermore, it is assumed that the RCS users are authenticated when a file is uploaded or retrieved for downloading. The HTTP Content Server would authenticate the File Localization Function before the latter can retrieve the file.

#### 4.3.1.2	File transfer in the same domain

The figure 4.3.1.2-1 illustrates the steps involved during the file transfer via content server procedure when two RCS Users and hence, the two RCS Servers are in the same domain. Since the two RCS users are served by the same RCS domain, the File Localization Function is not used here. In the illustration the RCS User (A) sends the file to RCS User (B).

Figure 4.3.1.2-1: File transfer RCS servers in the same domain

The figure 4.3.1.2-1 shows the following file-transfer steps:

- RCS User (A) uploads the file to the HTTP Content Server. The authentication procedure to perform RCS User (A) authorization to upload the file steps happens here.

- HTTP Content Server stores the file at the URL shown as URL-FO.

- HTTP Content Server provides the location where the file is stored (URL-FO) to the RCS User (A).

- RCS User (A) sends the URL-FO to the RCS Server (A) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (A) forwards the URL-FO to the RCS Server (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (B) forwards the URL-FO to the RCS User (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- Using URL-FO, the RCS User (B) reaches the HTTP Content Server to download the file. The authentication procedure to perform RCS User (B) authorization to download the file happens here.

- File is downloaded to RCS User (B).

- SIP response to 6.

- SIP response to 5.

- SIP response to 4.

As illustrated in clause 4.2.8 with various topology scenarios, the RCS Server (A) and RCS Server (B) may be in CSP domain or in a third party RCS service provider domain. In the latter case, the RCS users may also not belong to any CSP domain (see clause 4.2.4).

#### 4.3.1.3	File transfer with the file localization function

The figure 4.3.1.3-1 illustrates the steps involved during the file transfer via content server procedure when the two RCS users are in different RCS domains. Here, a File Localization Function is present at the recipient's end. In the illustration, RCS User (A) sends the file to RCS User (B).

Figure 4.3.1.3-1: File transfer with a file localization function

The figure 4.3.1.3-1 shows the following file-transfer steps:

- RCS User (A) uploads the file to the HTTP Content Server. The authentication procedure to perform RCS User (A) authorization to upload the file happens here.

- HTTP Content Server stores the file at the URL shown as URL-FO.

- HTTP Content Server provides the location where the file is stored (URL-FO) to the RCS User (A).

- RCS User (A) sends the URL-FO to the RCS Server (A) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (A) forwards the URL-FO to the RCS Server (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (B) provides the URL-FO to the File Localization Function to retrieve the file HTTP Content Server.

- Using URL-FO, the File Localization Function reaches the HTTP Content Server to retrieve the file. The authentication procedure to perform File Localization Function authorization to retrieve the file happens here.

- File is retrieved by the File Localization Function.

- File Localization Function stores the retrieved file at the URL shown as URL-FT.

- File Localization Function provides the location where the file is stored (URL-FT) to the RCS Server (B).

- RCS Server (B) forwards the URL-FT to the RCS User (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- Using URL-FT, the RCS User (B) reaches the File Localization Function to download the file. The authentication procedure to perform RCS User (B) authorization to download the file happens here.

- File is downloaded to RCS User (B).

- SIP response to 11.

- SIP response to 5.

- SIP response to 4.

As illustrated in clause 4.2.8 with various topology scenarios, the RCS Server (A) and RCS Server (B) may be in CSP domains or in third party RCS service provider domains. In the latter case, the RCS users may also not belong to any CSP domain. A hybrid case (one RCS Server in CSP domain and the other in a third party RCS service provider domain) is also possible.

#### 4.3.1.4	File transfer without the file localization function

The figure 4.3.1.4-1 illustrates the steps involved during the file transfer via content server procedure when the two RCS users are in different RCS domains. Here, a File Localization Function is not present at the recipient's end. In the illustration, RCS User (A) sends the file to RCS User (B).

Figure 4.3.1.4-1: File transfer with a file localization function

- RCS User (A) uploads the file to the HTTP Content Server. The authentication procedure to perform RCS User (A) authorization to upload the file steps happens here.

- HTTP Content Server stores the file at the URL shown as URL-FO.

- HTTP Content Server provides the location where the file is stored (URL-FO) to the RCS User (A).

- RCS User (A) sends the URL-FO to the RCS Server (A) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (A) forwards the URL-FO to the RCS Server (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- RCS Server (B) forwards the URL-FO to the RCS User (B) in a SIP message (note that the signalling passes through the IMS signalling functions).

- Using URL-FO, the RCS User (B) reaches the HTTP Content Server to download the file. No authentication procedure here, since RCS User (B) is in a different domain.

- File is downloaded to RCS User (B).

- SIP response to 6.

- SIP response to 5.

- SIP response to 4.

As illustrated in clause 4.2.8 with various topology scenarios, the RCS Server (A) and RCS Server (B) may be in CSP domains or in third party RCS service provider domains. In the latter case, the RCS users may also not belong to any CSP domain. A hybrid case (one RCS Server in CSP domain and the other in a third party RCS service provider domain) is also possible.

***** End of the new text ****