3GPP TSG-SA3 Meeting #100-LI	s3i260026

# Sophia-Antipolis, France, 27th Jan 2026 - 30th Jan 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0311 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conferencing IRI Events Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conference specific events to be detected and reported by the Conference AS/MRFC are missing and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper adds the missing conference events based on 3GPP TS 33.107, Stage 2 conference IRI events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI for Conferencing will not be supported making it difficult for network operators to meet LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.2, 7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There are no schema changes for this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***START OF CHANGES***

***START OF FIRST CHANGE***

#### 7.4.3.2	IRI events

The IRI-POI present in the IMS Signalling Function (other than the Conference AS/MRFC) generates the following xIRI:

-	Encapsulated SIP message.

-	CC unavailable in serving PLMN.

-	Start of interception with an established IMS session.

-	Data channel setup message.

-	Data channel termination message.

-	Start of interception with an established IMS data channel.

-	IRI Only RTP Packet.

The encapsulated SIP message xIRI is generated and delivered to the MDF2 when the IRI-POI in the IMS Signalling Function detects that a SIP message is received from, or sent to, a target or processed on behalf of a target at the IMS Signalling Function.

The CC unavailable in PLMN xIRI is generated and delivered to the MDF2 for the session scenarios where access to the target media is not available to the CSP (see clause 7.4.7.1).

The start of interception with an established IMS session xIRI is generated when an interception is activated on an established IMS session. To support the possibility of generating such an xIRI, the IMS Signalling Function shall store and maintain the session related information including the media information for the life of all IMS sessions.

The data channel setup message xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that the DCSF has received a DC call event from the IMS AS in the form of a DC control request and has responded with a media instruction set (see TS 23.228 [13], clause AC.7.1).

NOTE 1:	Data channel setup invokes procedures within the HSS. The reporting of these events is defined in clause 7.2.3 of the present document.

The data channel termination message xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that either UE has triggered a SDP renegotiation to release the application data channel or the application has been closed (see TS 23.228 [13] clause AC.7.6).

NOTE 2:	LI for IMS DC across international NNI is not considered in the present document.

NOTE 3:	Roaming cases for IMS data channel are not considered in the present document.

The start of interception with established IMS data channel xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that an interception has been activated for a target with an already established IMS data channel session.

The IRI Only RTP Packet xIRI is generated and delivered when authorized IRI is present in the RTP Stream (e.g. when the delivery of Post Dialled Digits is required).

The IRI-POI present in the IMS Signalling Function containing the Conference AS/MRFC generates the following xIRI:

-	Start of Conference.

-	Party Join.

-	Party Leave.

-	Conference Bearer Modification.

-	Start of Intercept on an Active Conference.

-	End of Conference.

-	Creation of Conference.

-	Update of Conference.

***END OF FIRST CHANGE***

***START OF SECOND CHANGE***

### 7.4.4	CC-TF and CC-POI

#### 7.4.4.1	General

The CC-TF (other than for the Conference AS/MRFC) detects the IMS signalling messages that are related to a target and then generates and sends a trigger to the CC-POI over the LI_T3 reference point. The CC-TF for the Conference AS/MRFC detects the conference events that are related to a target or target conference and then generates and sends a trigger to the CC-POI over the LI_T3 reference point.

The CC-POI based on the trigger detects the media to be intercepted, generates the xCC and delivers the same to the MDF3.

The following IMS Network Functions (i.e. IMS Media Functions and IMS Signalling Functions) may provide the CC-POI and CC-TF functions:

-	IMS-AGW with CC-TF in P-CSCF.

-	TrGW with CC-TF in IBCF.

-	IM-MGW with CC-TF in MGCF.

-	MRFP with CC-TF in AS/MRFC (see NOTE 3).

-	MRFP with CC-TF in Conference AS/MRFC (see NOTE 2).

-	MF with CC-TF in IMS-AS.

-	DC-AS with CC-TF in DCSF.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the CC-TF/CC-POI functions.

NOTE 1:	Void.

NOTE 2:	Conference AS, MRFC and MRFP together are referred to as Conference Server. Conference AS/MRFC provide the conference focus functions as defined in TS 24.147 [28].

NOTE 3:	When music tone or announcement is given to the calling user prior to answer on an incoming call to the target.

#### 7.4.4.2	CC intercept trigger

The CC-TF shall send CC intercept trigger to the CC-POI over LI_T3. The CC intercept trigger, at the minimum, shall consist of the following:

-	Correlation Identifier.

-	Media Identifier (e.g. SDP information).

The Correlation Identifier is used to correlate the xCC with the corresponding xIRI and is delivered from the CC-POI over the LI_X3 interface to the MDF3.

The Media Identifier is used to identify the media packets that have to be intercepted and/or delivered to the MDF3.

#### 7.4.4.3	Common CC parameters

For the delivery of intercepted media packets, the following information shall be passed from the CC-POI to the MDF3 in addition to the intercepted media packets:

-	Target identity.

-	Correlation identifier.

-	Time stamp.

-	Direction (indicates media is from or to the target or not applicable for conferencing, e.g., when the conference itself is the target).

***END OF SECOND CHANGE***

***END OF ALL CHANGES***