3GPP TSG-SA3 Meeting #100bis-e 	S3-202391

# e-meeting, 12 - 16 October 2020													        Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	pCR – Addressing EN on UAS registration IE in Sol#1

# Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

Approve this contribution to add a solution in TR33.854

# 2	References

[1]

# 3	Rationale

The contribution address in one EN on UAS registration IE in solution 1.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

## 6.1	Solution #1: UAS Authentication and Authorization

### 6.1.1	Solution overview

This solution address the key issue #1.

This solution assumes each UAV or UAVC is provisioned with a PLMN UE ID (SUPI) and the corresponding credential so that it can be authenticated (primary authentication) by the PLMN as a normal UE. In addition, UAV or UAVC is provisioned with a UAS ID and corresponding credentials to perform UAS authentication and authorization (UAA) with USS/UTM.

The UAA is mandatory for UAA or UAVC and is based on EAP framework, where AMF is taking the role of the transparent Authenticator.

### 6.1.2	Solution details

The call flow of this solution is shown in the figure below.

Figure 6.X.2-1: UAA procedure

- UAV (or UAVC) sends registration request to AMF. It may indicate that this is a registration for UAS.

Note: a new IE or an extension of an existing IE can be used to indicate UAA is requested. The IE can be defined in stage 3 and in coordination with CT.

- AMF initiates Primary authentication as a normal UE

- After successful Primary authentication, AMF checks whether UAV (or UAVC) requires UAA. This may be based on the subscription information retrieved from UDM in step 2

- UAA starts with EAP message exchanges.

  - AMF may optionally request UAS ID from UE.

  - UAV (or UAVC) responses with UAS ID. It may indicate whether this is a UAV or UAVC.

  - AMF sends UAA requests with UAS-ID and UAV or UAVC indicator in the EAP message. In addition, UAA request contains GPSI for USS/UTM to identify the UAV. GPSI shall be bound to UAS-ID.

  - USS/UTM response with EAP messages accordingly

  - EAP messages may continue based on the EAP method used.

  - …

Note: the EAP authentication method used by UTM is out of scope of 3GPP

- Based on the EAP authentication outcome, USS/UTM sends the results to AMF.

- AMF stores the results, together with SUPI (converted from GPSI), UAS-ID, and UAV/UAVC indicator

- AMF sends UAS registration complete message to UE

Editor's note:  Whether the UUA steps are executed within or outside the Registration procedure is FFS and in coordination with SA2

Editor's note:  Which core network function(s) (AMF, and/or others) and messaging will be used in the UAV authentication and authorization by USS/UTM procedure is FFS and in coordination with SA2

Editor's note:  What is provided to the UE following successful UAV authentication and authorization is FFS

Editor's note:  How authorization revocation is supported should be marked as FFS

### 6.1.3	Solution evaluation

TBC

***	END OF CHANGES	***