3GPP TSG-SA3 Meeting #102e 	S3-210189

# e-meeting, 18 – 29 January 2021

# Source:	CATT

# Title:	Add new key issue in TR 33.866

# Document for:	Approval

Agenda Item:	5.16

1	Decision/action requested

It is proposed to add one new key issue in TR 33.866. SA3 is kindly requested to approve this contribution.

2	References

[1]	3GPP TR 33.866, v0.2.0

3	Rationale

This pCR proposes to add one new key issue for the security of eNA.

4	Detailed proposal

******	FIRST OF CHANGE *********

## 5.X Key Issue #X The security of Data Management Framework related interfcace

5.X.1 Key issue details

As defined in TS 23.900-91[1], data management function is decoupled from analysis functions for the NWDAF in Rel-17. The NWDAF is decomposed by moving Data Collection, including the task of identifying the Data Source, to the Data Management Framework. New NF (e.g. DCCF, DRF) is specified. If the DCCF and DRF are implemented as standalone NFs, the new interfaces are introduced.

In Rel-17, Data Collection Coordination Functionality (DCCF) is used to coordinate collection of data from one or more NF(s) based on data collection requests from one or more Consumer NF(s). The NWDAF would request Data from the Data Management Framework, and if the data is not collected already, the Data Management Framework would request the data from a data source. In other words, a Data Source would independently send data to the Data Management Framework that sent a request for the NWDAF(s).

DRF functionality shall be used to store data.

As concluded in clause 8 in TR 23.900-91[1], Data Collection Coordination Functionality (DCCF) and Data Repository Functionality (DRF) and the related interfaces (e.g. data collection, data request) are to be standardized.

### 5.X.2	Security threats

An attacker may eavesdrop or manipulate or replay the communication or initiate the MITM attacks on the interface without confidentiality, integrity and replay protection,

### 5.X.3 Potential security requirements

Confidentiality protection, integrity protection and replay-protection shall be supported on the new interfaces.

*********END OF CHANGES*********