3GPP TSG-SA3 Meeting #102-e 	S3-210346

# e-meeting, 18th - 29th January 2021												Revision of S3-20xxxx

# Source:	Ericsson

# Title:	High-level conclusions for KI#1 (Credentials owned by an external entity)

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to add the following high-level conclusions for Key Issue #1 (Credentials owned by an external entity).

# 2	References

[1]	3GPP TR 33.857: "Study on enhanced security support for Non-Public Networks"

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

# 3	Rationale

The rationale is provided in the companion contribution S3-210340.

# 4	Detailed proposal

******BEGIN CHANGES*****

# 7	Conclusions

## 7.1		Conclusions to Key Issue #1 "Credentials owned by an external entity"

The primary authentication procedure in TS 33.501 [2] is re-used, with the following changes: the AUSF does not perform the authentication itself but relays the authentication procedure between the external entity and the UE. A new interface is introduced between the AUSF and the external entity, via an intermediate function that provides AAA interworking, as proposed in solutions #1, #2, #5 and #7. The new interface via the intermediate function is used for sending both EAP-messages and other data, e.g. keys.

A solution for the support of a legacy AAA server that supports any key-generating EAP-method will be specified during normative work. The MSK will be used as the root key for 5G key hierarchy as described in solutions #1, #5 and #7. A deployment recommendation needs to be included in the specifications: it is strongly recommended that the same credentials that are used for authentication towards a 5G SNPN are not used for authentication towards a non-5G network.

******END OF CHANGES*****